======================================================================= Evalplot

Evalplot

PROGRAM EVALPLOT                                                   Evalplot

VERSION 75-1 (AUGUST 1975)                                         Evalplot

VERSION 76-1 (JULY 1976)                                           Evalplot

VERSION 77-1 (APRIL 1977)                                          Evalplot

VERSION 78-1 (JULY 1978)                                           Evalplot

VERSION 79-1 (FEBRUARY 1979)                                       Evalplot

VERSION 80-1 (JULY 1980)    *IBM VERSION                           Evalplot

VERSION 80-2 (DECEMBER 1980)                                       Evalplot

VERSION 81-1 (MARCH 1981)                                          Evalplot

VERSION 81-2 (AUGUST 1981)  *IMPROVED ZOOM CAPABILITY              Evalplot

VERSION 82-1 (JANUARY 1982) *IMPROVED COMPUTER COMPATIBILITY       Evalplot

VERSION 83-1 (JANUARY 1983) *ELIMINATED COMPUTER DEPENDENT CODING. Evalplot

VERSION 83-2 (OCTOBER 1983) *ADDED PLOTTING OF HISTOGRAM DATA.     Evalplot

VERSION 84-1 (DECEMBER 1984)*ADDED PLOTS OF LEGENDRE COEFFICENTS   Evalplot

AS A FUNCTION OF ENERGY.              Evalplot

*ADDED SMALL PLOTTING MODE.            Evalplot

VERSION 85-1 (AUGUST 1985)  *FORTRAN-77/H VERSION                  Evalplot

VERSION 86-1 (JANUARY 1986) *ENDF/B-VI FORMAT                      Evalplot

VERSION 88-1 (JULY 1988)    *MAJOR REVISION TO MAKE CODE EASILY    Evalplot

INTERFACEABLE TO ALMOST ANY PLOTTER.  Evalplot

*WARNING...INPUT PARAMETERS FROM BEEN  Evalplot

CHANGED (SEE, DESCRIPTION BELOW)      Evalplot

*COMPUTER INDEPENDENT SOFTWARE         Evalplot

CHARACTERS.                           Evalplot

*COLOR PLOTS.                          Evalplot

*MT NUMBER DEFINITIONS FROM DATA FILE  Evalplot

READ BY PROGRAM                       Evalplot

*FORTRAN-77 REQUIRED (FORTRAN-H NO     Evalplot

SUPPORTED BY THIS PROGRAM).           Evalplot

*OPTION...INTERNALLY DEFINE ALL I/O    Evalplot

FILE NAMES (SEE, SUBROUTINE FILEIO    Evalplot

FOR DETAILS).                         Evalplot

*IMPROVED BASED ON USER COMMENTS.      Evalplot

VERSION 89-1 (JANUARY 1989) *PSYCHOANALYZED BY PROGRAM FREUD TO    Evalplot

INSURE PROGRAM WILL NOT DO ANYTHING   Evalplot

CRAZY.                                Evalplot

*UPDATED TO USE NEW PROGRAM CONVERT    Evalplot

KEYWORDS.                             Evalplot

*ADDED LIVERMORE CIVIC COMPILER        Evalplot

CONVENTIONS.                          Evalplot

*FORTRAN-77/FORTRAN-H COMPATIBLE       Evalplot

*SPECIAL ENDF/B MATERIAL DEFINITIONS   Evalplot

(ZA.LT.1000) FROM DATA FILE READ      Evalplot

BY PROGRAM.                           Evalplot

VERSION 89-2 (MARCH 1989)   *ADDED ENDF/B-V AND VI MT              Evalplot

DEFINITIONS. PROGRAM WILL DETERMINE   Evalplot

ENDF/B FORMAT BASED ON MF=1,          Evalplot

MT=451 AND USE ASPPROPRIATE MT        Evalplot

DEFINITIONS. IF NO MF=1, MT=451       Evalplot

PROGRAM WILL USE ENDF/B-V             Evalplot

MT DEFINITIONS.                       Evalplot

VERSION 89-3 (JUNE 1989)    *3 CHARACTER FONTS                     Evalplot

VERSION 92-1 (JANUARY 1992) *COMPLETE REWRITE OF CODE              Evalplot

*ADDED PHOTON DATA, MF=23 AND 27       Evalplot

*ADDED INCIDENT CHARGED PARTICLES      Evalplot

(IDENTIFIED IN PLOT TITLES)           Evalplot

*ADDED FORTRAN SAVE OPTION.            Evalplot

*UPDATED BASED ON USER COMMENTS        Evalplot

*ADDED RETRIEVAL BY UP TO 100          Evalplot

MAT/MF/MT OR ZA/MF/MT RANGES          Evalplot

*WARNING...INPUT PARAMETER FORMAT      Evalplot

HAS BEEN CHANGED...SEE DESCRIPTION    Evalplot

BELOW.                                Evalplot

VERSION 92-2 (FEBRUARY 1992)*ADDED PHOTON SPECTRA, MF=15.          Evalplot

*ADDED MULTIPLICATION OF DISTRIBUTIONS Evalplot

IN MF=5 AND 15 BY PROBABILITY=YIELD.  Evalplot

*INCREASED PAGE SIZE TO 12000 POINTS   Evalplot

VERSION 92-3 (MAY 1992)     *CORRECTED DESCRIPTION OF INPUT        Evalplot

PARAMETERS AND EXAMPLE PROBLEMS.      Evalplot

*CORRECTED FOR ENDF/B-VI DEFINITION OF Evalplot

TEMPERATURE FROM MF=1/MT=451.         Evalplot

*CORRECTED LOGIC SO THAT EACH REQUEST  Evalplot

IS TREATED SEPARATELY TO CREATE A     Evalplot

PLOT, UNLESS REQUESTS ARE CHAINED     Evalplot

TOGETHER.                             Evalplot

*ADDED VARIABLE CHARACTER SIZE INPUT.  Evalplot

VERSION 93-1 (MARCH 1993)   *INCREASED PAGE SIZE FROM 12000        Evalplot

TO 210000                             Evalplot

*INCREASED THE NUMBER OF ENERGIES      Evalplot

VS. LEGENDRE COEFFICIENTS FROM        Evalplot

167 TO 7000                           Evalplot

*UPDATED FOR ON SCREEN GRAPHICS        Evalplot

USING THE LAHEY FORTRAN COMPILER.     Evalplot

VERSION 94-1 (JANUARY 1994) *VARIABLE ENDF/B DATA FILENAMES        Evalplot

TO ALLOW ACCESS TO FILE STRUCTURES    Evalplot

(WARNING - INPUT PARAMETER FORMAT     Evalplot

HAS BEEN CHANGED)                     Evalplot

*CLOSE ALL FILES BEFORE TERMINATING    Evalplot

(SEE, SUBROUTINE ENDIT)               Evalplot

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Evalplot

*IMPROVED COMPUTER INDEPENDENCE        Evalplot

*ALL DOUBLE PRECISION                  Evalplot

*UNIFORM TREATMENT OF ENDF/B I/O       Evalplot

*IMPROVED OUTPUT PRECISION             Evalplot

*DEFINED SCRATCH FILE NAMES            Evalplot

*ALL DOUBLE PRECISION                  Evalplot

VERSION 97-1 (APRIL 1997)   *INCREASED PAGE SIZE FROM 210000       Evalplot

TO 480,000                            Evalplot

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Evalplot

POINT READ FOR MORE DIGITS            Evalplot

*UPDATED TEST FOR ENDF/B FORMAT        Evalplot

VERSION BASED ON RECENT FORMAT CHANGE Evalplot

*GENERAL IMPROVEMENTS BASED ON         Evalplot

USER FEEDBACK                         Evalplot

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF=10, ACTIVATION CROSS         Evalplot

SECTION PLOTS.                        Evalplot

*INCREASED DIMENSIONS TO HANDLE MORE   Evalplot

SECTIONS - UP TO 1,000                Evalplot

*GENERAL IMPROVEMENTS BASED ON         Evalplot

USER FEEDBACK                         Evalplot

VERS. 2002-1 (Nov. 2002)    *OPTIONAL INPUT PARAMETERTS            Evalplot

*OPTIONAL BLACK OR WHITE BACKGROUND    Evalplot

*COLOR POSTSCRIPT FILES                Evalplot

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Evalplot

*INCREASED PAGE SIZE TO 600,000        Evalplot

*INCREASED THE NUMBER OF ENERGIES      Evalplot

VS. LEGENDRE COEFFICIENTS FROM        Evalplot

7000 TO 20000                         Evalplot

Evalplot

OWNED, MAINTAINED AND DISTRIBUTED BY                               Evalplot

------------------------------------                               Evalplot

THE NUCLEAR DATA SECTION                                           Evalplot

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Evalplot

P.O. BOX 100                                                       Evalplot

A-1400, VIENNA, AUSTRIA                                            Evalplot

EUROPE                                                             Evalplot

Evalplot

ORIGINALLY WRITTEN BY                                              Evalplot

------------------------------------                               Evalplot

DERMOTT E. CULLEN                                                  Evalplot

UNIVERSITY OF CALIFORNIA                                           Evalplot

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Evalplot

L-159                                                              Evalplot

P.O. BOX 808                                                       Evalplot

LIVERMORE, CA 94550                                                Evalplot

U.S.A.                                                             Evalplot

TELEPHONE  925-423-7359                                            Evalplot

E. MAIL    CULLEN1@LLNL.GOV                                        Evalplot

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Evalplot

Evalplot

AUTHORS MESSAGE                                                    Evalplot

---------------                                                    Evalplot

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Evalplot

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Evalplot

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Evalplot

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Evalplot

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Evalplot

Evalplot

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Evalplot

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Evalplot

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Evalplot

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Evalplot

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Evalplot

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Evalplot

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Evalplot

COMPUTER.                                                          Evalplot

Evalplot

PURPOSE                                                            Evalplot

-------                                                            Evalplot

THIS PROGRAM IS DESIGNED TO READ EVALUATED DATA FROM THE ENDF/B    Evalplot

FORMAT AND TO PLOT THE DATA. THE USER MAY SELECT CROSS SECTIONS,   Evalplot

PARAMETERS (E.G. NU-BAR, MU-BAR, ETC.), ANGULAR DISTRIBUTIONS      Evalplot

AND/OR ENERGY DISTRIBUTIONS TO BE PLOTTED.                         Evalplot

Evalplot

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Evalplot

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Evalplot

DISK OR ANY OTHER MEDIUM.                                          Evalplot

Evalplot

ON WHAT COMPUTERS WILL THE PROGRAM RUN                             Evalplot

------------------------------------------------------------------ Evalplot

THE PROGRAM HAS BEEN IMPLEMENTED ON A WIDE VARIETY OF COMPUTERS    Evalplot

FROM THE ONE EXTREME OF LARGE MAINFRAME CRAY AND IBM COMPUTERS     Evalplot

TO THE OTHER EXTREME OF SUN TERMINALS AND IBM PERSONAL COMPUTERS.  Evalplot

THE PROGRAM IS DESIGNED TO RUN ON VIRTUALLY ANY COMPUTER. FOR      Evalplot

SPECIAL CONSIDERATIONS SEE THE SECTIONS BELOW ON,                  Evalplot

(1) COMPUTER DEPENDENT CODING                                      Evalplot

(2) PLOTTER/GRAPHICS TERMINAL INTERFACE                            Evalplot

Evalplot

ON WHAT PLOTTERS WILL THE PROGRAM RUN                              Evalplot

------------------------------------------------------------------ Evalplot

THE PLOTTER MAY USE UNITS OF INCHES, CENTIMETERS, MILLIMETERS,     Evalplot

VIRTUALLY ANYTHING. INTERNALLY THE PROGRAM WILL DEFINE PLOTS IN    Evalplot

APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. AS PART OF THE        Evalplot

INPUT THE USER DEFINES THE ACTUAL SIZE OF THE PLOT IN THE UNITS    Evalplot

(I.E., INCHES, CENTIMETERS, MILLIMETERS, WHATEVER) OF THE REAL     Evalplot

PLOT. THE PLOT IS TRANSFORMED TO THE SIZE OF THE LOCAL PLOTTER     Evalplot

AND OUTPUT. USING THIS CONVENTION THIS PROGRAM SHOULD BE EASY      Evalplot

TO INTERFACE TO VIRTUALLY ANY PLOTTER OR GRAPHICS TERMINAL.        Evalplot

Evalplot

GRAPHICS INTERFACE                                                 Evalplot

------------------------------------------------------------------ Evalplot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE GRAPHICS INTERFACE WHICH   Evalplot

REQUIRES ONLY 3 SUBROUTINES...PLOTS, PLOT AND PEN (DESCRIBED IN    Evalplot

DETAIL BELOW). ALL CHARACTERS AND SYMBOLS ARE DRAWN USING TABLES   Evalplot

OF PEN STROKES (SUPPLIED WITH THIS PROGRAM). USING THIS METHOD     Evalplot

THE PROGRAM SHOULD BE SIMPLE TO INTERFACE TO VIRTUALLY ANY PLOTTER Evalplot

OR GRAPHICS TERMINAL AND THE APPEARANCE AND LAYOUT OF THE PLOTS    Evalplot

SHOULD BE INDEPENDENT OF WHICH PLOTTER IS USED.                    Evalplot

Evalplot

PROGRAM IDENTIFICATION                                             Evalplot

----------------------                                             Evalplot

AS DISTRIBUTED THE FIRST FRAME OF PLOTTED OUTPUT WILL DOCUMENT     Evalplot

THE PROGRAM NAME, VERSION AND INSTALLATION. THIS INFORMATION IS    Evalplot

STORED AS DATA IN THE ARRAY VERSES NEAR THE BEGINNING OF           Evalplot

SUBROUTINE FRAME1. IF YOU WISH TO CUSTOMIZE THE OUTPUT TO IDENTIFY Evalplot

YOUR INSTALLATION CHANGE THE LAST TWO LINES OF THE ARRAY VERSES.   Evalplot

Evalplot

SIZE OF PLOTS                                                      Evalplot

-------------                                                      Evalplot

THE PROGRAM HAS A BUILT-IN DEFAULT SIZE TO MAKE EACH PLOT 13.50    Evalplot

BY 10.24 INCHES. THIS SIZE WAS SELECTED ASSUMING THAT THE          Evalplot

RESOLUTION OF THE PLOTTER IS 1024 RASTER POINTS PER INCH. THE      Evalplot

USER MAY CHANGE THE SIZE OF THE PLOT BY SPECIFYING ANY REQUIRED    Evalplot

SIZE ON THE FIRST INPUT LINE. IN PARTICULAR FOR USE ON ANY PLOTTER Evalplot

THAT USES CENTIMETERS INSTEAD OF INCHES THE USER MAY MERELY        Evalplot

SPECIFY THE REQUIRED SIZE OF THE PLOT IN CENTIMETERS (E.G., TO     Evalplot

OBTAIN A 13.50 BY 10.24 INCH PLOT, THE USER NEED ONLY SPECIFY      Evalplot

34.3 BY 26 ON THE FIRST INPUT LINE...ASSUMING 2.54 CENTIMETERS PER Evalplot

INCH, OR 343 BY 260 FOR MILLIMETERS..ASSUMING 25.4 MILLIMETERS     Evalplot

PER INCH).                                                         Evalplot

Evalplot

CHARACTER SIZE                                                     Evalplot

--------------                                                     Evalplot

THE PLOT HAS A BUILT-IN CHARACTER SIZE WHICH HAS BEEN DEFINED FOR  Evalplot

COMPATIBILITY WITH THE BUILT-IN PLOT SIZE. IF THE USER SPECIFIES   Evalplot

BY INPUT A DIFFERENT PLOT SIZE, THE PROGRAM WILL AUOTMATICALLY     Evalplot

SCALE THE SIZE OF ALL CHARACTERS BY THE RATIO OF THE Y SIZE OF THE Evalplot

PLOT SPECIFIED BY THE USER TO THE BUILT-IN Y SIZE OF PLOTS (E.G.,  Evalplot

FOR PLOTS WHICH ARE ONLY 5.12 HIGH (Y DIRECTION) ALL CHARACTERS    Evalplot

WILL BE SCALED TO BE ONLY 1/2 THE CHARACTER SIZE ON PLOTS WHICH    Evalplot

ARE 10.24 HIGH (10.24 = THE BUILT-IN SIZE). NOTE, CHANGES IN THE   Evalplot

X SIZE OF THE PLOT WILL NOT HAVE ANY EFFECT ON THE CHARACTER SIZE  Evalplot

(E.G., FOR A LONG PLOT, 30 BY 10.24 THE CHARACTER SIZE WILL BE THE Evalplot

THE SAME AS ON A 13.50 BY 10.24 PLOT).                             Evalplot

Evalplot

PLOT PER FRAME                                                     Evalplot

--------------                                                     Evalplot

BY INPUT THE USER CAN SPECIFY NOT ONLY THE ACTUAL SIZE OF THE      Evalplot

LOCAL PLOTTER, BUT ALSO HOW MANY PLOTS SHOULD APPEAR ON EACH       Evalplot

FRAME. THIS IS DONE BY SPECIFYING THE LAYOUT OF A FRAME IN TERMS   Evalplot

OF THE NUMBER OF PLOTS IN THE X AND Y DIRECTION. FOR EXAMPLE BY    Evalplot

SPECIFYING THAT EACH FRAME BE DIVIDED INTO 3 PLOTS IN THE X        Evalplot

DIRECTION AND 2 PLOTS IN THE Y DIRECTION, EACH FRAME WILL CONTAIN  Evalplot

UP TO 6 PLOTS (3 X 2). INTERNALLY EACH PLOT WILL BE GENERATED TO   Evalplot

STANDARD A4 SIZE, AS DESCRIBED ABOVE, AND THEN ON OUTPUT SCALED    Evalplot

TO THE NUMBER OF PLOTS PER FRAME SPECIFIED BY THE USER INPUT.      Evalplot

Evalplot

ENDF/B FORMAT                                                      Evalplot

-------------                                                      Evalplot

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Evalplot

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Evalplot

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Evalplot

Evalplot

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Evalplot

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Evalplot

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Evalplot

NUMBERS (COLUMNS 76-80) ARE IGNORED. FORMAT OF SECTION MT=452, 455 Evalplot

OF MF=1, AND ALL SECTIONS OF MF=3, 4 AND 5 MUST BE CORRECT. ALL    Evalplot

OTHER SECTION OF DATA ARE SKIPPED AND AS SUCH THE OPERATION OF     Evalplot

THIS PROGRAM IS INSENSITIVE TO THE CORRECTNESS OR INCORRECTNESS    Evalplot

OF ALL OTHER SECTIONS.                                             Evalplot

Evalplot

INTERPOLATION LAW                                                  Evalplot

-----------------                                                  Evalplot

EACH TABLE OF DATA MAY USE EITHER COMPLETELY HISTOGRAM OR          Evalplot

COMPLETELY LINEAR INTERPOLATION LAW (THE TWO INTERPOLATION LAWS    Evalplot

CANNOT BE MIXED TOGETHER IN ONE TABLE). EITHER OF THESE TWO        Evalplot

REPRESENTATIONS WILL BE STORED IN CORE IN LINEARLY INTERPOLABLE    Evalplot

FORM. IF THIS PROGRAM FINDS ANY DATA THAT USES ANY OTHER           Evalplot

INTERPOLATION LAW IT WILL PRINT AN ERROR MESSAGE AND PLOT THE      Evalplot

TABLE AS IF IT WERE LINEARLY INTERPOLABLE. THE ONLY ERROR THAT     Evalplot

WILL RESULT IN THE PLOT WILL BE IN THE CURVE FOLLOWED BETWEEN      Evalplot

TABULATED POINTS. PROGRAM LINEAR (UCRL-50400, VOL. 17, PART A)     Evalplot

MAY BE USED TO CONVERT CROSS SECTIONS TO LINEARLY INTERPOLABLE     Evalplot

FORM. PROGRAM LEGEND CAN BE USED FOR ANGULAR DISTRIBUTIONS AND     Evalplot

PROGRAM ENERGY CAN BE USED FOR SECONDARY ENERGY DISTRIBUTIONS.     Evalplot

Evalplot

REACTION INDEX                                                     Evalplot

--------------                                                     Evalplot

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Evalplot

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Evalplot

Evalplot

PAGE SIZE                                                          Evalplot

---------                                                          Evalplot

ONLY ONE PAGE OF DATA = 600000 DATA POINTS - IS KEPT IN CORE AT    Evalplot

ANY GIVEN TIME. IF THERE IS MORE THAN THIS MANY POINTS THEY WILL   Evalplot

BE KEPT ON A SCRATCH FILE AND LOADED INTO CORE AS NEEDED.          Evalplot

Evalplot

TO CHANGE THE PAGE SIZE,                                           Evalplot

Evalplot

1) CHANGE  600000 TO THE NEW PAGE SIZE                             Evalplot

2) CHANGE 1200000 TO TWO TIMES THE NEW PAGE SIZE                   Evalplot

Evalplot

SECTION SIZE                                                       Evalplot

------------                                                       Evalplot

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Evalplot

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Evalplot

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Evalplot

Evalplot

THE ONLY EXCEPTION TO THIS RULE IS THAT EACH TABLE OF DATA WHICH   Evalplot

USES A HISTOGRAM INTERPOLATION LAW CANNOT EXCEED HALF THE SIZE     Evalplot

OF THE IN CORE PAGE (PRESENTLY 600000/2=300000) WHICH IS ADEQUATE  Evalplot

FOR ALMOST ALL HISTOGRAM (E.G. MULTIGROUP) REPRESENTATIONS OF      Evalplot

A SINGLE TABLE (E.G. REACTION).                                    Evalplot

Evalplot

WHAT DATA CAN BE PLOTTED                                           Evalplot

------------------------                                           Evalplot

THIS CODE CAN PLOT VIRTUALLY ANY NEUTRON OR PHOTON CROSS SECTIONS  Evalplot

(MF=3 OR 23) AND ANY TABULATED ANGULAR OR ENERGY DISTRIBUTIONS OR  Evalplot

LEGENDRE COEFFICIENTS. WHAT IS ACTUALLY PLOTTED DEPENDS ON WHAT    Evalplot

DATA IS SELECTED BY THE USER.                                      Evalplot

Evalplot

SELECTION OF DATA                                                  Evalplot

-----------------                                                  Evalplot

DATA TO BE PLOTTED IS SPECIFIED BY INPUTTING UP TO 100 MAT/MF/MT   Evalplot

RANGES OR UP TO 100 ZA/MF/MT RANGES. IN ADDITION FOR EACH RANGE    Evalplot

THE USER MAY SPECIFY AN X RANGE (USUALLY ENERGY) AND THE TYPE OF   Evalplot

DATA TO BE PLOTTED (SEE: THE DESCRIPTION OF TYPES, BELOW).         Evalplot

Evalplot

THE X RANGE FOR MF = 1, 3, 23 AND 27 AND MF = 4 LEGENDRE           Evalplot

COEFFICIENTS WILL BE USED AS THE X LIMITS OF THE PLOTS, E.G.,      Evalplot

PLOT ENERGY DEPENDENT CROSS SECTIONS BETWEEN 1 AND 20 MEV.         Evalplot

Evalplot

THE X RANGE FOR MF = 4 AND 5 WILL BE USED TO ONLY SELECT ANGULAR   Evalplot

AND ENERGY DISTRIBUTION FOR WHICH THE INCIDENT NEUTRON ENERGY      Evalplot

IS IN THE X RANGE. E.G., ONLY PLOT ANGULAR DISTRIBUTIONS WHERE     Evalplot

THE INCIDENT NEUTRON ENERGY IS 1 TO 20 MEV.                        Evalplot

Evalplot

INTERACTIVE VS. BATCH MODE                                         Evalplot

--------------------------                                         Evalplot

VERSION 92-1 AND LATER VERSIONS OF THIS CODE ONLY USE A BATCH      Evalplot

MODE WHERE ALL REQUESTS ARE READ AND PROCESSED. EARLIER VERSIONS   Evalplot

OF THIS CODE HAD BOTH AN INTERACTIVE MODE (WHERE REQUESTS WHERE    Evalplot

READ AND EXECUTED ONE AT A TIME) AND A BATCH MODE. INTERACTIVE     Evalplot

MODE HAS BEEN DROPPED AND WILL NOT TO REINTRODUCED UNLESS THE      Evalplot

AUTHOR IS INFORMED BY USERS THAT THEY WERE USING THE INTERACTIVE   Evalplot

MODE.                                                              Evalplot

Evalplot

PLOT LAYOUT                                                        Evalplot

-----------                                                        Evalplot

VERSION 92-1 AND LATER VERSIONS OF THIS CODE WILL PLOT ALL         Evalplot

CURVES ON A SINGLE PLOT. EARLIER VERSIONS OF THIS CODE ALLOWED     Evalplot

THE OPTION TO HAVE,                                                Evalplot

MULTIPLE PLOTS - INDIVIDUAL SCALING                                Evalplot

MULTIPLE PLOTS - COMMON SCALING                                    Evalplot

SINGLE PLOT                                                        Evalplot

MULTILE PLOTS PER PLOT HAVE BEEN DROPPED AND WILL NOT BE           Evalplot

REINTRODUCED UNLESS IT IS DEMONSTRATED TO THE AUTHOR THAT THEY     Evalplot

ARE OF PRACTICAL USE IN SOME APPLICATION.                          Evalplot

Evalplot

PROCESSING OF DATA                                                 Evalplot

------------------                                                 Evalplot

IN THE CASE OF NEUTRON AND PHOTON CROSS SECTIONS (MF=3 OR 23)      Evalplot

AND PARAMETERS (MF=1 OR 27) ALL DATA IN A FILE (MF) IS READ        Evalplot

GROUPED TOGETHER BY TYPE (AS EXPLAINED BELOW) AND PLOTTED.         Evalplot

Evalplot

IN THE CASE OF ANGULAR AND ENERGY DISTRIBUTIONS (MF=4 OR 5) ONLY   Evalplot

ONE SECTION OF DATA AT A TIME IS READ AND PLOTTED.                 Evalplot

Evalplot

TYPES OF DATA (MF=1, 3, 23 AND 27 ONLY)                            Evalplot

---------------------------------------                            Evalplot

THESE DATA ARE DIVIDED INTO UP TO 18 TYPES AND EACH TYPE OF        Evalplot

DATA IS GROUPED TOGETHER AND PLOTTED (IF THE DATA IS ACTUALLY      Evalplot

PRESENT).                                                          Evalplot

Evalplot

WHAT TYPE OF DATA IS ACTUALLY PLOTTED CAN BE CONTROLLED BY USER    Evalplot

INPUT EITHER BASED ON SELECTED MAT/MF/MT OR ZA/MF/MT RANGES OR     Evalplot

BY EXPLICITLY SELECTING ONLY ONE TYPE OF DATA IS TO BE PLOTTED     Evalplot

(SEE THE DESCRIPTION OF INPUT BELOW).                              Evalplot

Evalplot

SIMPLE REQUESTS                                                    Evalplot

---------------                                                    Evalplot

GENERALLY EACH MAT/MF/MT OR ZA/MF/MT REQUESTED IS TREATED          Evalplot

SEPERATELY AND THE SPECIFIED DATA IS GROUPED BY TYPE AND PLOTTED.  Evalplot

FOR EXAMPLE, THE USER MAY SPECIFY USING ONE REQUEST THAT ALL       Evalplot

TYPES OF DATA BE PLOTTED OVER THE ENTIRE ENERGY RANGE AND USE      Evalplot

A SECOND REQUEST TO SPECIFY THAT ONE PARTICULAR TYPE OF DATA       Evalplot

BE PLOTTED OVER A SPECIFIC ENERGY RANGE.                           Evalplot

Evalplot

CHAINED REQUESTS                                                   Evalplot

----------------                                                   Evalplot

REQUESTS MAY ALSO BE CHAINED TOGETHER (SEE, THE DESCRIPTION OF     Evalplot

INPUT BELOW), WHERE A NUMBER OF REQUESTS MAY BE USED TO SELECT     Evalplot

DATA, BUT ONLY THE LAST REQUEST IN A CHAIN WILL CAUSE ALL SELECTED Evalplot

DATA TO BE PLOTTED. CHAINED REQUESTED ARE INDICATED ON INPUT BY    Evalplot

A SERIES OF REQUESTS FOR DATA TYPE = -1, EXCEPT FOR THE LAST       Evalplot

REQUEST OF THE CHAIN, WHICH MUST SPECIFY A TYPE DATA = 0 (ALL)     Evalplot

OR A POSITIVE NUMBER. UNLIKE SIMPLE REQUESTS, WHERE EACH WILL      Evalplot

PRODUCE ONE OR MORE PLOTS, WITH CHAINED REQUESTS THE ENTIRE        Evalplot

SERIES OF CHAINED REQUESTS WILL BE TREATED AS A SINGLE REQUEST     Evalplot

AND WILL PRODUCE ONE OR MORE PLOTS.                                Evalplot

Evalplot

FOR EXAMPLE, DATA TYPE = 1 WILL NORMALLY INCLUDE,                  Evalplot

MT =   1 - TOTAL                                                   Evalplot

=   2 - ELASTIC                                                 Evalplot

=   4 - TOTAL INELASTIC                                         Evalplot

=  18 - FISSION                                                 Evalplot

= 102 - CAPTURE                                                 Evalplot

IF YOU WISH TO EXCLUDE TOTAL INELASTIC FROM A PLOT YOU NEED ONLY   Evalplot

SPECIFY TWO CHAINED REQUESTS THE FIRST TO SELECT MT = 1 THROUGH    Evalplot

2 (TO INCLUDE TOTAL AND ELASTIC) AND A SECOND TO INCLUDE MT = 18   Evalplot

THROUGH 102. THE FIRST REQUEST SHOULD SPECIFY DATA TYPE = -1 AND   Evalplot

SECOND 1 (THIS WILL CHAIN THE 2 REQUESTS TOGETHER, SO THAT MT =1   Evalplot

THROUGH 2, AND MT = 18 THROUGH 102 ALL APPEAR ON THE SAME PLOT).   Evalplot

SINCE MT = 4 (TOTAL INELASTIC) IS NOT REQUESTED IT WILL NOT BE     Evalplot

PLOTTED.                                                           Evalplot

Evalplot

DEFINITION OF 18 DATA TYPES                                        Evalplot

------------------------------------------------------------------ Evalplot

NEUTRONS (MF = 3)                                                  Evalplot

-----------------                                                  Evalplot

(1) TOTAL, ELASTIC, CAPTURE, FISSION AND TOTAL INELASTIC           Evalplot

(2) (N,2N), (N,3N) AND (N,N' CHARGED PARTICLE)                     Evalplot

(3) (N,CHARGED PARTICLE)                                           Evalplot

(4) PARTICLE PRODUCTION (PROTON, DEUTERON, ETC.) AND DAMAGE        Evalplot

(5) TOTAL, FIRST, SECOND, ETC. CHANCE FISSION.                     Evalplot

(6) TOTAL INELASTIC, INELASTIC DISCRETE LEVELS AND CONTINUUM       Evalplot

(7) (N,P) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(8) (N,D) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(9) (N,T) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(10) (N,HE-3) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)          Evalplot

(11) (N,ALPHA) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)         Evalplot

(12) PARAMETERS MU-BAR, XI AND GAMMA                               Evalplot

(13) NU-BAR - TOTAL, PROMPT AND DELAYED                            Evalplot

Evalplot

PHOTONS  (MF=23 AND 27)                                            Evalplot

-----------------------                                            Evalplot

(14) TOTAL, COHERENT, INCOHERENT, TOTAL PHOTOELECTRIC, TOTAL       Evalplot

PAIR PRODUCTION                                               Evalplot

(15) TOTAL AND SUBSHELL PHOTOELECTRIC                              Evalplot

(16) TOTAL, NUCLEAR AND ELECTRON PAIR PRODUCTION                   Evalplot

(17) COHERENT FORM FACTOR AND INCOHERENT SCATTERING FUNCTION       Evalplot

(18) REAL AND IMAGINARY SCATTERING FACTORS                         Evalplot

Evalplot

IDENTIFICATION OF DATA                                             Evalplot

----------------------                                             Evalplot

ALL PLOTS IDENTIFY THE TARGET, E.G., U-238 AND UNITS OF THE X AND  Evalplot

Y AXIS, E.G., X = ENERGY (MEV) OR COSINE (LAB), ETC., Y = CROSS    Evalplot

SECTION (BARNS) OR PROBABILITY/COSINE, ETC.                        Evalplot

Evalplot

FOR TYPES OF DATA (MF=1, 3, 23 AND 27) DIFFERENT REACTIONS (MT)    Evalplot

ARE GROUPED TOGETHER TO APPEAR ON THE SAME PLOT. THE TITLE AT      Evalplot

THE TOP OF THE PLOT WILL IDENTIFY THE TYPE OF DATA BEING PLOTTED   Evalplot

AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY EACH REACTION.    Evalplot

Evalplot

FOR ANGULAR AND ENERGY DISTRIBUTIONS (MF=4 OR 5) EACH PLOT WILL    Evalplot

CONTAIN DATA FOR A SINGLE REACTION (MT) AND DIFFERENT INCIDENT     Evalplot

NEUTRON ENERGIES. THE TITLE AT THE TOP OF THE PLOT WILL IDENTIFY   Evalplot

THE REACTION AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY      Evalplot

THE INCIDENT ENERGY.                                               Evalplot

Evalplot

FOR LEGENDRE COEFFICIENT THE DATA IN ENDF/B FORMAT WILL BE         Evalplot

INVERTED IN ORDER TO PRESENT EACH LEGENDRE COEFFICIENT VERSUS      Evalplot

INCIDENT ENERGY. THE TITLE AT THE TOP OF THE PLOT WILL IDENTIFY    Evalplot

THE REACTION AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY      Evalplot

THE LEGENDRE ORDER.                                                Evalplot

Evalplot

INPUT FILES                                                        Evalplot

-----------                                                        Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

2   INPUT LINES (BCD - 80 CHARACTERS/RECORD)                    Evalplot

9   MT DEFINITIONS (BCD - 80 CHARACTERS/RECORD)                 Evalplot

10   ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)                    Evalplot

12   SOFTWARE CHARACTERS (BCD - 80 CHARACTERS/RECORD)            Evalplot

Evalplot

OUTPUT FILES                                                       Evalplot

------------                                                       Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

3   OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                 Evalplot

16   PLOTTING UNIT                                               Evalplot

Evalplot

SCRATCH FILES                                                      Evalplot

-------------                                                      Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

11  SCRATCH FILE (BINARY - 960000 WORDS/RECORD = 2*PAGE SIZE)    Evalplot

Evalplot

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Evalplot

---------------------------------------------------------------    Evalplot

UNIT  FILE NAME                                                    Evalplot

----  ----------                                                   Evalplot

2   EVALPLOT.INP                                                 Evalplot

3   EVALPLOT.LST                                                 Evalplot

9   MT.DAT                                                       Evalplot

10   ENDFB.IN    (OR AS INPUT PARAMETER)                          Evalplot

11   (SCRATCH)                                                    Evalplot

12   PLOT.CHR                                                     Evalplot

16   (PLOTTING UNIT...USUALLY A DUMMY)                            Evalplot

Evalplot

INPUT PARAMETERS                                                   Evalplot

----------------                                                   Evalplot

LINE COLUMNS FORMAT  DESCRIPTION                                   Evalplot

---- ------- ------  -----------                                   Evalplot

1   1-11   E11.4  LOWER X LIMIT OF PLOTTER                      Evalplot

12-22   E11.4  UPPER X LIMIT OF PLOTTER                      Evalplot

23-33   E11.4  LOWER Y LIMIT OF PLOTTER                      Evalplot

34-44   E11.4  UPPER Y LIMIT OF PLOTTER                      Evalplot

45-55    I11   NUMBER OF PLOTS PER FRAME IN X DIRECTION      Evalplot

56-66    I11   NUMBER OF PLOTS PER FRAME IN Y DIRECTION      Evalplot

67-70    F4.1  CHARACTER SIZE MULTIPLIER                     Evalplot

= 0 OR 1    - NORMAL CHARACTER SIZE           Evalplot

= OTHERWISE - CHARACTERS SCALED BY THIS       Evalplot

FACTOR.                         Evalplot

2   1-60    A60   ENDF/B DATA FILENAME                          Evalplot

(LEAVE BLANK FOR STANDARD = ENDFB.IN)         Evalplot

3   1-11    I11   RETRIEVAL CRITERIA                            Evalplot

= 0 - MAT                                     Evalplot

= 1 - ZA                                      Evalplot

12-22    I11   TYPE OF GRID                                  Evalplot

= 0 - TICK MARKS ON BORDER                    Evalplot

= 1 - SOLID AT COARSE INTERVALS               Evalplot

= 2 - DASHED AT COARSE INTERVALS              Evalplot

= 3 - SOLID AT FINE INTERVALS                 Evalplot

= 4 - DASHED AT FINE INTERVALS                Evalplot

= 5 - SOLID COARSE/DASHED FINE GRID           Evalplot

23-33    I11   SHOULD BORDER BE PLOTTED ON EACH PLOT         Evalplot

= 0 - NO                                      Evalplot

= 1 - YES                                     Evalplot

34-44    I11   LINE THICKNESS                                Evalplot

= 0 -  5 = BORDER/CURVES/CHARACTERS           Evalplot

=-1 - -5 = BORDER/CURVES (NOT CHARACTERS)     Evalplot

NOTE, THE GRID IS NEVER THICK.     Evalplot

45-55    I11   SHOULD TEMPERATURE BE PLOTTED.                Evalplot

= 0 - YES                                     Evalplot

= 1 - NO                                      Evalplot

56-66   E11.4  ALLOWABLE RATIO OF PLOT Y RANGE MAXIMUM TO    Evalplot

MINIMUM - IF THIS RATIO IS EXCEEDED THE Y     Evalplot

RANGE MINIMUM WILL BE CHANGED TO THE Y RANGE  Evalplot

MAXIMUM TIMES THIS RATIO.                     Evalplot

IF THIS RATIO IS NOT POSITIVE, IT IS          Evalplot

INTERPRETED TO MEAN NO LIMIT ON Y RANGE.      Evalplot

67-70    I4    BACKGROUND COLOR                              Evalplot

= 0         = BLACK                           Evalplot

= OTHERWISE = WHITE                           Evalplot

4-N   1- 6    I6    LOWER MAT OR ZA LIMIT                         Evalplot

7- 8    I2    LOWER MF LIMIT                                Evalplot

9-11    I3    LOWER MT LIMIT                                Evalplot

11-22   E11.4  LOWER X LIMIT (USUALLY ENERGY) - EV           Evalplot

23-28    I6    UPPER MAT OR ZA LIMIT                         Evalplot

29-30    I2    UPPER MF LIMIT                                Evalplot

31-33    I3    UPPER MT LIMIT                                Evalplot

34-44   E11.4  UPPER X LIMIT (USUALLY ENERGY) - EV           Evalplot

45-55   I11    TYPE OF DATA TO RETRIEVE AND PLOT             Evalplot

=  -1  - CHAIN THIS REQUEST TO THE NEXT ONE   Evalplot

=   0  - ALL                                  Evalplot

= 1-18 - TYPE AS SPECIFIED ABOVE              Evalplot

Evalplot

THERE MAY BE UP 100 MAT/MF/MT OR ZA/MF/MT REQUEST RANGES. INPUT    Evalplot

MUST BE TERMINATED BY A BLANK LINE.                                Evalplot

Evalplot

IF X LIMITS ARE NOT SPECIFIED (I.E., LOWER AND UPPER X LIMIT = 0)  Evalplot

THIS WILL BE INTERPRETED TO MEAN NO LIMIT AND ALL DATA WILL BE     Evalplot

PLOTTED OVER THEIR ENTIRE ENERGY RANGE, I.E., YOU NEED NOT         Evalplot

KNOW AND SPECIFY THE ACTUAL ENERGY LIMITS OF THE DATA.             Evalplot

Evalplot

EXAMPLE DEFINITION OF PLOTTER                                      Evalplot

-----------------------------                                      Evalplot

THE FIRST INPUT LINE DEFINES THE DIMENSIONS OF THE PLOTTER BEING   Evalplot

USED IN ANY UNITS (INCHES, CENTIMETERS, MILLIMETERS, ANYTHING)     Evalplot

WHICH APPLY TO THE PLOTTER. IN ADDITION THE FIRST LINE DEFINES     Evalplot

HOW MANY PLOTS SHOULD APPEAR ON EACH FRAME. THE PLOTTING AREA      Evalplot

DEFINED ON THE FIRST INPUT LINE MAY BE SUBDIVIDED INTO ANY NUMBER  Evalplot

OF PLOTS IN THE X AND Y DIRECTION. FOR EXAMPLE, TO PRODUCE A       Evalplot

SERIES OF FRAMES EACH CONTAINING 3 PLOTS IN THE X DIRECTION AND    Evalplot

2 PLOTS IN THE Y DIRECTION (6 PLOTS PER FRAME) COLUMN 45-55 OF     Evalplot

THE FIRST INPUT LINE SHOULD BE 3 AND COLUMNS 56-66 SHOULD BE 2.    Evalplot

Evalplot

IF THE LOCAL PLOTTER USES DIMENSIONS OF INCHES IN ORDER TO OBTAIN  Evalplot

10 X 10 INCH FRAMES WITH 3 X 2 PLOTS PER FRAME THE FIRST INPUT     Evalplot

LINE SHOULD BE,                                                    Evalplot

Evalplot

0.0       10.0      0.0     10.0              3          2       Evalplot

Evalplot

IF THE LOCAL PLOTTER USES DIMENSION OF MILLIMETERS THE SAME        Evalplot

PHYSICAL SIZE PLOT MAY BE OBTAINED IF THE FIRST INPUT LINE IS,     Evalplot

Evalplot

0.0      254.0      0.0    254.0              3          2       Evalplot

Evalplot

FOR SIMPLICITY THE FOLLOWING EXAMPLE INPUTS WILL NOT DISCUSS THE   Evalplot

PHYSICAL DIMENSIONS OF THE PLOTTER AND THE FIRST INPUT LINE WILL   Evalplot

IN ALL CASES INDICATE 10 X 10 INCH PLOTS WITH ONLY 1 PLOT PER      Evalplot

FRAME.                                                             Evalplot

Evalplot

ALL OF THE FOLLOWING EXAMPLE WILL USE,                             Evalplot

1) A DASHED GRID          (SECOND LINE, COLS. 12-22 = 2)           Evalplot

2) NO BORDER              (SECOND LINE, COLS. 23-33 = 0)           Evalplot

3) LINE THICKNESS -2      (SECOND LINE, COLS. 34-44 =-2)           Evalplot

4) TEMPERATURE ON PLOTS   (SECOND LINE, COLS. 45-55 = 0)           Evalplot

5) NO Y RANGE LIMIT       (SECOND LINE, COLS. 56-66 = 0.0)         Evalplot

Evalplot

EXAMPLE INPUT NO. 1                                                Evalplot

-------------------                                                Evalplot

FOR ALL THORIUM AND URANIUM ISOTOPES PLOT NEUTRON CROSS SECTIONS   Evalplot

ENTIRE ENERGY RANGE. IN ADDITION PLOT TYPE 1 DATA, MAJOR NEUTRON   Evalplot

CROSS SECTIONS OVER THE ENERGY RANGE 1 EV TO 1 KEV. USE THE        Evalplot

STANDARD FILENAME (ENDFB.IN) FOR THE ENDF/B DATA. THE FOLLOWING    Evalplot

6 INPUT LINES ARE REQUIRED,                                        Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  0           90999 3999                     0                 Evalplot

90000 3  0 1.00000+ 090999 3999 1.00000+ 3          1                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

EXAMPLE INPUT NO. 2                                                Evalplot

-------------------                                                Evalplot

PLOT FE-56 ELASTIC AND INELASTIC ANGULAR DISTRIBUTIONS BETWEEN     Evalplot

1 AND 20 MEV. THE FOLLOWING 6 INPUT LINES ARE REQUIRED,            Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

26056 4  2 1.00000+ 626056 4  2 2.00000+ 7          0                 Evalplot

26056 4  4 1.00000+ 626056 4  4 2.00000+ 7          0                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

EXAMPLE INPUT NO. 3 (CHAINED INPUT)                                Evalplot

-----------------------------------                                Evalplot

FOR ALL THORIUM AND URANIUM ISOTOPES PLOT TOTAL, ELASTIC ,CAPTURE  Evalplot

AND FISSION, BUT NOT INELASTIC CROSS SECTIONS OVER THERE ENTIRE    Evalplot

ENERGY RANGE AND FROM 1 KEV TO 1 MEV. THE FOLLOWING 8 INPUT        Evalplot

LINES ARE REQUIRED,                                                Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  1           90999 3  2                    -1                 Evalplot

90000 3 18           90999 3102                     1                 Evalplot

90000 3  1 1.00000+ 390999 3  2 1.00000+ 6         -1                 Evalplot

90000 3 18 1.00000+ 390999 3102 1.00000+ 6          1                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

NOTE, THIS EXAMPLE INCLUDES 2 CHAINED REQUESTED - INPUT LINES 3    Evalplot

AND 4 SELECTING DATA AND PRODUCING A PLOT OVER THE ENTIRE ENERGY   Evalplot

RANGE AND INPUT LINES 5 AND 6 SELECTING THE SAME DATA AND          Evalplot

PRODUCING A PLOT FROM 1 KEV TO 1 MEV.                              Evalplot

Evalplot

ANY NUMBER OF REQUEST LINES MAY TO CHAINED TOGETHER TO SELECT      Evalplot

DATA. THE CHAIN ENDS WHERE THE TYPE OF DATA (COLS. 45-55) IS NOT   Evalplot

NEGATIVE AND THEN THE SELECTED DATA WILL BE PLOTTED.               Evalplot

Evalplot

EXAMPLE INPUT NO. 4                                                Evalplot

-------------------                                                Evalplot

FOR THE SAME EXAMPLE AS ABOVE, EXCEPT USE A DIFFERENT FILENAME     Evalplot

FOR THE ENDF/B DATA TO READ FROM A FILE TREE STRUCTURE. THE        Evalplot

FOLLOWING 8 INPUT LINES ARE REQUIRED,                              Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

EVALUATION/ENDFB6/THORIUM                                              Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  1           90999 3  2                    -1                 Evalplot

90000 3 18           90999 3102                     1                 Evalplot

90000 3  1 1.00000+ 390999 3  2 1.00000+ 6         -1                 Evalplot

90000 3 18 1.00000+ 390999 3102 1.00000+ 6          1                 Evalplot

(BLANK LINE MUST FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Evalplot

Evalplot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE INTERFACE INVOLVING       Evalplot

ONLY 6 SUBROUTINES,                                               Evalplot

Evalplot

STARPLOT  - INITIALIZE PLOTTER                                    Evalplot

NEXTPLOT  - CLEAR THE SCREEN FOR THE NEXT PLOT                    Evalplot

ENDPLOTS  - TERMINATE PLOTTING                                    Evalplot

Evalplot

PLOT(X,Y,IPEN)        - DRAW OR MOVE FROM LAST LOCATION TO (X,Y), Evalplot

END OF CURRENT PLOT OR END OF PLOTTING.   Evalplot

IPEN =   2 - DRAW                                           Evalplot

=   3 - MOVE                                           Evalplot

Evalplot

PEN(IPEN)             - SELECT COLOR.                             Evalplot

IPEN- COLOR = 1 TO N (N = ANY POSITIVE INTEGER)              Evalplot

Evalplot

BOXCOLOR(X,Y,IFILL,IBORDER) - FILL A RECTANGULAR BOX DEFINED      Evalplot

BY THE X AND Y CORNERS - X(1),      Evalplot

X(2), Y(1),Y(2)                     Evalplot

IFILL          - COLOR TO FILL BOX WITH              Evalplot

IBORDER  - COLOR OF BOX BORDER                 Evalplot

Evalplot

IN ORDER TO INTERFACE THIS PROGRAM FOR USE ON ANY PLOTTER WHICH    Evalplot

DOES NOT USE THE ABOVE CONVENTIONS IT IS MERELY NECESSARY FOR THE  Evalplot

THE USER TO WRITE 6 SUBROUTINES WITH THE NAMES PLOTS, PLOT AND PEN Evalplot

WITH THE SUBROUTINE ARGUMENTS DESCRIBED ABOVE AND TO THEN CALL THE Evalplot

LOCAL EQUIVALENT ROUTINES.                                         Evalplot

Evalplot

COLOR PLOTS                                                        Evalplot

------------------------------------------------------------------ Evalplot

TO SELECT PLOTTING COLORS SUBROUTINE PEN (DESCRIBED ABOVE) IS USED Evalplot

TO SELECT ONE OF THE AVAILABLE COLORS. IF YOU HAVE COLOR ON YOUR   Evalplot

PLOTTER YOU SHOULD PROVIDE A SUBROUTINE PEN TO SELECT COLORS.      Evalplot

Evalplot

BLACK AND WHITE PLOTS                                              Evalplot

------------------------------------------------------------------ Evalplot

WHEN PRODUCING BLACK AND WHITE PLOTS SUBROUTINE PEN NEED MERELY    Evalplot

BE A DUMMY SUBROUTINE TO IGNORE ANY ATTEMPT TO CHANGE COLORS,      Evalplot

Evalplot

SUBROUTINE PEN(IPEN)                                               Evalplot

RETURN                                                             Evalplot

END                                                                Evalplot

Evalplot

SIMILAR BOXCOLOR CAN BE A DUMMY                                    Evalplot

Evalplot

SUBROUTINE BOXCOLOR(X,Y,IFILL,IBORDER)                             Evalplot

RETURN                                                             Evalplot

END                                                                Evalplot

Evalplot

CHARACTER SET                                                      Evalplot

------------------------------------------------------------------ Evalplot

THIS PROGRAM USES COMPUTER AND PLOTTER DEVICE INDEPENDENT SOFTWARE Evalplot

CHARACTERS. THIS PROGRAM COMES WITH A FILE THAT DEFINES THE PEN    Evalplot

STROKES REQUIRED TO DRAW ALL CHARACTERS ON AN IBM KEYBOARD (UPPER  Evalplot

AND LOWER CASE CHARACTERS, NUMBERS, ETC.) PLUS AN ALTERNATE SET OF Evalplot

ALL UPPER AND LOWER CASE GREEK CHARACTERS AND ADDITIONAL SPECIAL   Evalplot

SYMBOLS.                                                           Evalplot

Evalplot

THE SOFTWARE CHARACTER TABLE CONTAINS X AND Y AND PEN POSITIONS TO Evalplot

DRAW EACH CHARACTER. IF YOU WISH TO DRAW ANY ADDITIONAL CHARACTERS Evalplot

OR TO MODIFY THE FONT OF THE EXISTING CHARACTERS YOU NEED ONLY     Evalplot

MODIFY THIS TABLE.                                                 Evalplot

Evalplot

ADDITIONAL FONTS                                                   Evalplot

----------------                                                   Evalplot

THIS PROGRAM COMES WITH 3 COMPLETE SETS OF THE SAME CHARACTERS     Evalplot

USING DIFFERENT FONTS. FOR SPEED IN PLOTTING IT IS RECOMMENDED     Evalplot

THAT YOU USE THE SIMPLEX FONT. FOR FINISHED PLOTS SUITABLE FOR     Evalplot

PUBLICATION, BUT REQUIRING MORE TIME TO GENERATE A PLOT, IT IS     Evalplot

RECOMMENDED THAT YOU USE THE DUPLEX OR COMPLEX FONT - YOU CAN      Evalplot

EXPERIMENT WITH ANY OF THE 3 FONTS TO DETERMINE WHICH BEST MEETS   Evalplot

YOUR NEEDS.                                                        Evalplot

Evalplot

TO USE ANY ONE OF THE FONTS MERELY BY SURE THAT IT IS DEFINED AS   Evalplot

UNIT 12 FOR INPUT (IF USING STANDARD FILENAMES IT SHOULD BE        Evalplot

NAMED PLOT.CHR). SO THAT SWITCHING FONTS CAN BE SIMPLY DONE        Evalplot

MERELY BY COPYING THE FONT THAT YOU WANT TO THE UNIT 12 THAT       Evalplot

YOU ARE USING FOR INPUT.                                           Evalplot

Evalplot

CONTROL CHARACTERS                                                 Evalplot

------------------------------------------------------------------ Evalplot

IN THE SOFTWARE CHARACTER TABLE ALL CHARACTERS TO BE PLOTTED WILL  Evalplot

HAVE PEN POSITION = 2 (DRAW) OR = 3 (MOVE). IN ADDITION THE TABLE  Evalplot

CURRENTLY CONTAINS 4 CONTROL CHARACTERS,                           Evalplot

Evalplot

PEN POSITION = 0                                                   Evalplot

----------------                                                   Evalplot

SHIFT THE NEXT PRINTED CHARACTER BY X AND Y. 3 CONTROL CHARACTERS  Evalplot

ARE PRESENTLY INCLUDED IN THE SOFTWARE CHARACTER TABLE TO ALLOW    Evalplot

SHIFTING.                                                          Evalplot

Evalplot

{   = SHIFT UP (FOR SUPERSCRIPTS..............X= 0.0, Y= 0.5)      Evalplot

}   = SHIFT DOWN (FOR SUBSCRIPTS..............X= 0.0, Y=-0.5)      Evalplot

\   = SHIFT LEFT 1 CHARACTER (FOR BACKSPACE...X=-1.0, Y= 0.0)      Evalplot

Evalplot

PEN POSITION =-1                                                   Evalplot

----------------                                                   Evalplot

SELECT THE NEXT PRINTED CHARACTER FROM THE ALTERNATE CHARACTER     Evalplot

SET. AT PRESENT THIS CONTROL CHARACTER IS,                         Evalplot

Evalplot

|   = SWITCH TO ALTERNATE CHARACTER SET                            Evalplot

Evalplot

THESE 4 CONTROL CHARACTERS ARE ONLY DEFINED BY THE VALUE OF THE    Evalplot

PEN POSITION IN THE SOFTWARE CHARACTER TABLE (I.E., THEY ARE NOT   Evalplot

HARD WIRED INTO THIS PROGRAM). AS SUCH BY MODIFYING THE SOFTWARE   Evalplot

CHARACTER TABLE THE USER HAS THE OPTION OF DEFINING ANY CONTROL    Evalplot

CHARACTERS TO MEET SPECIFIC NEEDS.                                 Evalplot

Evalplot

THESE CHARACTERS MAY BE USED IN CHARACTER STRINGS TO PRODUCE       Evalplot

SPECIAL EFFECTS. FOR EXAMPLE, TO PLOT SUBSCRIPT 5, B, SUPERSCRIPT  Evalplot

10 USE THE STRING,                                                 Evalplot

Evalplot

}5B{1{0                                                            Evalplot

Evalplot

TO PLOT B, SUBSCRIPT 5 AND SUPERSCRIPT 10 WITH THE 5 DIRECTLY      Evalplot

BELOW THE 1 OF THE 10 WE CAN USE THE BACKSPACE CHARACTER TO        Evalplot

POSITION THE 1 DIRECTLY ABOVE THE 5 USING THE STRING,              Evalplot

Evalplot

B}5\{1{0                                                           Evalplot

Evalplot

TO PLOT UPPER CASE GREEK GAMMA FOLLOWED BY THE WORD TOTAL (I.E.,   Evalplot

RESONANCE TOTAL WIDTH) USE THE STRING.                             Evalplot

Evalplot

|G TOTAL                                                           Evalplot

Evalplot

NOTE, WHEN THESE CONTROL CHARACTERS ARE USED THEY ONLY EFFECT THE  Evalplot

NEXT 1 PRINTED CHARACTER (SEE, ABOVE EXAMPLE OF PLOTTING SUPER-    Evalplot

SCRIPT 10 WHERE THE SHIFT UP CONTROL CHARACTER WAS USED BEFORE THE Evalplot

1 AND THEN AGAIN BEFORE THE 0 AND THE BACKSPACE AND SHIFT UP       Evalplot

CONTROL CHARACTERS WERE USED IN COMBINATION).                      Evalplot

Evalplot

IF THESE 4 CONTROL CHARACTERS ARE NOT AVAILABLE ON YOUR COMPUTER   Evalplot

YOU CAN MODIFY THE SOFTWARE CHARACTER TABLE TO USE ANY OTHER 4     Evalplot

CHARACTERS THAT YOU DO NOT NORMALLY USE IN CHARACTER STRINGS (FOR  Evalplot

DETAILS SEE THE SOFTWARE CHARACTER TABLE).                         Evalplot

Evalplot

STANDARD/ALTERNATE CHARACTER SETS                                  Evalplot

------------------------------------------------------------------ Evalplot

THE SOFTWARE CHARACTER TABLE CONTAINS 2 SETS OF CHARACTERS WHICH   Evalplot

ARE A STANDARD SET (ALL CHARACTERS ON AN IBM KEYBOARD) AND AN      Evalplot

ALTERNATE SET (UPPER AND LOWER CASE GREEK CHARACTERS AND SPECIAL   Evalplot

CHARACTERS). TO DRAW A CHARACTER FROM THE ALTERNATE CHARACTER SET  Evalplot

PUT A RIGHT BRACKET CHARACTER (|) BEFORE A CHARACTER (SEE THE      Evalplot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Evalplot

CONTROL CHARACTER WILL ONLY EFFECT THE NEXT 1 PLOTTED CHARACTER.   Evalplot

Evalplot

SUB AND SUPER SCRIPTS                                              Evalplot

------------------------------------------------------------------ Evalplot

TO DRAW SUBSCRIPT PRECEED A CHARACTER BY }. TO DRAW SUPERSCRIPT    Evalplot

PRECEED A CHARACTER BY { (SEE THE ABOVE EXAMPLE AND THE SOFTWARE   Evalplot

CHARACTER TABLE FOR DETAILS). THESE CONTROL CHARACTER WILL ONLY    Evalplot

EFFECT THE NEXT 1 PLOTTED CHARACTER.                               Evalplot

Evalplot

BACKSPACING                                                        Evalplot

------------------------------------------------------------------ Evalplot

TO BACKSPACE ONE CHARACTER PRECEED A CHARACTER BY \ (SEE, THE      Evalplot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Evalplot

CONTROL CHARACTER WILL PERFORM A TRUE BACKSPACE AND WILL EFFECT    Evalplot

ALL FOLLOWING CHARACTERS IN THE SAME CHARACTER STRING.             Evalplot

Evalplot

PLOT DIMENSIONS                                                    Evalplot

---------------                                                    Evalplot

ARE DEFINED BY USER INPUT. INTERNALLY THE PROGRAM WILL CREATE A    Evalplot

PLOT IN APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. DURING        Evalplot

OUTPUT THE PLOT IS TRANSFORMED TO THE UNITS (INCHES, CENTIMETERS,  Evalplot

MILLIMETERS, WHATEVER) OF THE PLOTTER BEING USED AND OUTPUT.       Evalplot

Evalplot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Evalplot

======================================================================= Evalplot