======================================================================= Linear

Linear

PROGRAM LINEAR                                                     Linear

VERSION 74-1 (MAY 1974)                                            Linear

VERSION 75-1 (APRIL 1975)                                          Linear

VERSION 76-2 (OCTOBER 1976)                                        Linear

VERSION 77-1 (JANUARY 1977)                                        Linear

VERSION 78-1 (JULY 1978)                                           Linear

VERSION 79-1 (JULY 1979) CDC-7600 AND CRAY-1 VERSION.              Linear

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION.                 Linear

VERSION 80-2 (DECEMBER 1980)                                       Linear

VERSION 81-1 (MARCH 1981)                                          Linear

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY.       Linear

VERSION 83-1 (JANUARY 1983) *MAJOR RE-DESIGN.                      Linear

*PAGE SIZE INCREASED - 1002 TO 3006.   Linear

*ELIMINATED COMPUTER DEPENDENT CODING. Linear

*NEW, MORE COMPATIBLE I/O UNIT NUMBER. Linear

*ADDED OPTION TO KEEP ALL ORIGINAL     Linear

ENERGY POINTS FROM EVALUATION.        Linear

*ADDED STANDARD ALLOWABLE ERROR OPTION Linear

(CURRENTLY 0.1 PER-CENT).             Linear

VERSION 83-2 (OCTOBER 1983) IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 84-1 (APRIL 1984)   IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 84-2 (JUNE 1984)   *UPDATED FOR ENDF/B-VI FORMATS.         Linear

*SPECIAL I/O ROUTINES TO GUARANTEE      Linear

ACCURACY OF ENERGY.                    Linear

*DOUBLE PRECISION TREATMENT OF ENERGY   Linear

(REQUIRED FOR NARROW RESONANCES).      Linear

VERSION 85-1 (AUGUST 1985) *FORTRAN-77/H VERSION                   Linear

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMAT                       Linear

VERSION 87-1 (JANUARY 1987)*DOUBLE PRECISION TREATMENT OF CROSS    Linear

SECTION                                Linear

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Linear

FILE NAMES (SEE, SUBROUTINE FILEIO     Linear

FOR DETAILS).                          Linear

*IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Linear

INSURE PROGRAM WILL NOT DO ANYTHING    Linear

CRAZY.                                 Linear

*UPDATED TO USE NEW PROGRAM CONVERT     Linear

KEYWORDS.                              Linear

*ADDED LIVERMORE CIVIC COMPILER         Linear

CONVENTIONS.                           Linear

VERSION 90-1 (JUNE 1990)   *EXTENDED TO LINEARIZE PHOTON           Linear

INTERACTION DATA, MF=23 AND 27         Linear

*ADDED FORTRAN SAVE OPTION              Linear

*UPDATED BASED ON USER COMMENTS.        Linear

*NEW MORE CONSISTENT ENERGY OUTPUT      Linear

ROUTINE.                               Linear

*WARNING...INPUT PARAMETER FORMAT       Linear

HAS BEEN CHANGED...SEE DESCRIPTION     Linear

BELOW.                                 Linear

VERSION 91-1 (JULY 1991)   *ADDED INTERPOLATION LAW 6 - ONLY USED  Linear

FOR CHARGED PARTICLE CROSS SECTIONS    Linear

FOR COULOMB PENETRABILITIES.           Linear

VERSION 92-1 (JANUARY 1992)*ADDED NU-BAR (TOTAL, DELAYED, PROMPT)  Linear

POLYNOMIAL OR TABULATED ALL CONVERTED  Linear

TO LINEARLY INTERPOLABLE               Linear

*INCREASED PAGE SIZE FROM 3006 TO 5010  Linear

POINTS.                                Linear

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Linear

TO CALCULATIONS.                       Linear

*COMPLETELY CONSISTENT I/O AND ROUNDING Linear

ROUTINES - TO MINIMIZE COMPUTER        Linear

DEPENDENCE.                            Linear

VERSION 92-2 (JULY 1992)   *CORRECTED CONVERSION OF NU-BAR FROM    Linear

POLYNOMIAL TO TABULATED - COPY         Linear

SPONTANEOUS NU-BAR (BY DEFINITION      Linear

THE SPONTANEOUS NU-BAR IS NOT AN       Linear

ENERGY DEPENDENT QUANTITY).            Linear

VERSION 93-1 (MARCH 1993)  *UPDATED FOR USE WITH LAHEY COMPILER    Linear

ON IBM-PCS.                            Linear

*INCREASED PAGE SIZE FROM 5010 TO       Linear

30000 POINTS                           Linear

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Linear

TO ALLOW ACCESS TO FILE STRUCTURES     Linear

(WARNING - INPUT PARAMETER FORMAT      Linear

HAS BEEN CHANGED)                      Linear

*CLOSE ALL FILES BEFORE TERMINATING     Linear

(SEE, SUBROUTINE ENDIT)                Linear

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Linear

*IMPROVED COMPUTER INDEPENDENCE        Linear

*ALL DOUBLE PRECISION                  Linear

*ON SCREEN OUTPUT                      Linear

*UNIFORM TREATMENT OF ENDF/B I/O       Linear

*IMPROVED OUTPUT PRECISION             Linear

*DEFINED SCRATCH FILE NAMES            Linear

*ALWAYS INCLUDE THERMAL VALUE          Linear

*INCREASED PAGE SIZE FROM 30000 TO     Linear

60000 POINTS                          Linear

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Linear

POINT READ FOR MORE DIGITS            Linear

*UPDATED TEST FOR ENDF/B FORMAT        Linear

VERSION BASED ON RECENT FORMAT CHANGE Linear

*GENERAL IMPROVEMENTS BASED ON         Linear

USER FEEDBACK                         Linear

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Linear

MF=1, MT-451.                         Linear

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF = 9 AND 10 LINEARIZATION     Linear

*GENERAL IMPROVEMENTS BASED ON         Linear

USER FEEDBACK                         Linear

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Linear

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACK Linear

Linear

OWNED, MAINTAINED AND DISTRIBUTED BY                               Linear

------------------------------------                               Linear

THE NUCLEAR DATA SECTION                                           Linear

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Linear

P.O. BOX 100                                                       Linear

A-1400, VIENNA, AUSTRIA                                            Linear

EUROPE                                                             Linear

Linear

ORIGINALLY WRITTEN BY                                              Linear

------------------------------------                               Linear

DERMOTT E. CULLEN                                                  Linear

UNIVERSITY OF CALIFORNIA                                           Linear

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Linear

L-159                                                              Linear

P.O. BOX 808                                                       Linear

LIVERMORE, CA 94550                                                Linear

U.S.A.                                                             Linear

TELEPHONE  925-423-7359                                            Linear

E. MAIL    CULLEN1@LLNL.GOV                                        Linear

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Linear

Linear

AUTHORS MESSAGE                                                    Linear

---------------                                                    Linear

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Linear

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Linear

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Linear

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                  Linear

Linear

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Linear

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Linear

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Linear

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Linear

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Linear

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Linear

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Linear

COMPUTER.                                                          Linear

Linear

PURPOSE                                                            Linear

-------                                                            Linear

THIS PROGRAM IS DESIGNED TO CONVERT ENDF/B FILE 3, 23 AND 27 DATA  Linear

TO LINEAR-LINEAR INTERPOLABLE FORM. ANY SECTION THAT IS ALREADY    Linear

LINEAR-LINEAR INTERPOLABLE WILL BE THINNED.                        Linear

Linear

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY  Linear

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE    Linear

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                             Linear

Linear

ENDF/B FORMAT                                                      Linear

-------------                                                      Linear

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Linear

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Linear

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Linear

Linear

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Linear

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Linear

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Linear

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Linear

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451  Linear

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL   Linear

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Linear

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Linear

Linear

OUTPUT FORMAT                                                      Linear

-------------                                                      Linear

IN THIS VERSION OF LINEAR ALL ENERGIES WILL BE OUTPUT IN           Linear

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Linear

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Linear

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Linear

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Linear

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Linear

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Linear

DUE TO TRUNCATION OF ENERGIES TO 6 DIGITS DURING OUTPUT.           Linear

Linear

CONTENTS OF OUTPUT                                                 Linear

------------------                                                 Linear

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE LINEARIZED DATA        Linear

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO     Linear

INCLUDED.                                                          Linear

Linear

DOCUMENTATION                                                      Linear

-------------                                                      Linear

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Linear

BY THE ADDITION OF 3 COMMENT LINES AT THE END OF EACH HOLLERITH    Linear

SECTION IN THE FORM                                                Linear

Linear

***************** PROGRAM LINEAR (2004-1) ****************         Linear

FOR ALL DATA GREATER THAN 1.00000-10 IN ABSOLUTE VALUE             Linear

DATA LINEARIZED TO WITHIN AN ACCURACY OF  0.1  PER-CENT            Linear

Linear

THE ORDER OF SIMILAR COMMENTS (FROM RECENT, SIGMA1 AND GROUPIE)    Linear

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Linear

THE DATA BY THESE PROGRAMS.                                        Linear

Linear

THESE COMMENT LINES ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Linear

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Linear

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Linear

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Linear

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Linear

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Linear

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Linear

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Linear

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Linear

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Linear

Linear

REACTION INDEX                                                     Linear

--------------                                                     Linear

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Linear

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Linear

Linear

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Linear

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Linear

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Linear

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Linear

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Linear

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Linear

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Linear

Linear

SECTION SIZE                                                       Linear

------------                                                       Linear

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Linear

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Linear

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Linear

Linear

FOR ANY LINEARIZED SECTION THAT CONTAINS 60000 OR FEWER POINTS     Linear

THE ENTIRE OPERATION WILL BE PERFORMED IN CORE AND THE LINEARIZED  Linear

DATA WILL BE OUTPUT DIRECTLY TO THE ENDF/B FORMAT. FOR ANY SECTION Linear

THAT CONTAINS MORE POINTS THE DATA WILL BE LINEARIZED A PAGE AT A  Linear

TIME (1 PAGE = 60000 POINTS) AND OUTPUT TO SCRATCH. AFTER THE      Linear

ENTIRE SECTION HAS BEEN LINEARIZED THE DATA WILL BE READ BACK FROM Linear

SCRATCH AND OUTPUT TO THE ENDF/B FORMAT.                           Linear

Linear

SELECTION OF DATA                                                  Linear

-----------------                                                  Linear

THE PROGRAM SELECTS DATA TO BE LINEARIZED BASED EITHER ON EITHER   Linear

MAT (ENDF/B MAT NO.) OR ZA AS WELL AS MF AND MT NUMBERS. THIS      Linear

PROGRAM ALLOWS UP TO 100 MAT/MF/MT OR ZA/MF/MT RANGES TO BE        Linear

SPECIFIED BY INPUT PARAMETERS. THE PROGRAM WILL ASSUME THAT THE    Linear

ENDF/B TAPE IS IN MAT ORDER, REGARDLESS OF THE CRITERIA USED       Linear

TO RETRIEVE MATERIALS. IF RETRIEVAL IS BY MAT RANGE THE PROGRAM    Linear

WILL TERMINATE WHEN A MAT IS FOUND THAT IS ABOVE ALL REQUESTED     Linear

MAT RANGES. IF RETRIEVAL IS BY ZA RANGE THE PROGRAM WILL SEARCH    Linear

THE ENTIRE ENDF/B TAPE.                                            Linear

Linear

PROGRAM OPERATION                                                  Linear

-----------------                                                  Linear

EACH SECTION OF DATA IS CONSIDERED SEPARATELY. EACH SECTION OF     Linear

ENDF/B DATA TO LINEARIZE IS REPRESENTED BY A TABLE OF ENERGY       Linear

VS. CROSS SECTION AND ANY ONE OF FIVE ALLOWABLE INTERPOLATION LAWS Linear

BETWEEN ANY TWO TABULATED POINTS. THIS PROGRAM WILL REPLACE EACH   Linear

SECTION OF DATA CROSS SECTIONS BY A NEW TABLE OF ENERGY VS.        Linear

CROSS SECTION IN WHICH THE INTERPOLATION LAW IS ALWAYS LINEAR IN   Linear

ENERGY AND CROSS SECTION BETWEEN ANY TWO TABULATED POINTS.         Linear

Linear

DATA IS READ AND LINEARIZED A PAGE AT A TIME (ONE PAGE CONTAINS    Linear

60000 DATA POINTS). IF THE FINAL LINEARIZED SECTION CONTAINS TWO   Linear

PAGES OR LESS, DATA POINTS IT WILL BE ENTIRELY CORE RESIDENT       Linear

AFTER IT HAS BEEN LINEARIZED AND WILL BE WRITTEN DIRECTLY FROM     Linear

CORE TO THE OUTPUT TAPE. IF THE LINEARIZED SECTION IS LARGER THAN  Linear

TWO PAGES, AFTER EACH PAGE IS LINEARIZED IT WILL BE WRITTEN TO     Linear

SCRATCH. AFTER THE ENTIRE SECTION HAS BEEN LINEARIZED IT WILL      Linear

BE READ BACK FROM SCRATCH, TWO PAGES AT A TIME, AND WRITTEN TO     Linear

THE OUTPUT TAPE.                                                   Linear

Linear

KEEP EVALUATED DATA POINTS                                         Linear

--------------------------                                         Linear

SOMETIMES IT IS CONVENIENT TO KEEP ALL ENERGY POINTS WHICH WERE    Linear

PRESENT IN THE ORIGINAL EVALUATION AND TO MERELY SUPPLEMENT THESE  Linear

POINTS WITH ADDITIONAL ENERGY POINTS IN ORDER TO LINEARIZE THE     Linear

CROSS SECTIONS. FOR EXAMPLE, IT IS OFTEN CONVENIENT TO KEEP THE    Linear

THERMAL VALUE (AT 0.0253 EV) OR THE VALUE AT 14.1 MEV.             Linear

Linear

THE CURRENT VERSION OF THIS PROGRAM WILL ALLOW THE USER TO KEEP    Linear

ALL ORIGINAL EVALUATED DATA POINTS BY SPECIFYING 1 IN COLUMNS      Linear

34-44 OF THE FIRST INPUT LINE. THIS WILL TURN OFF THE BACKWARD     Linear

THINNING (SEE UCRL-50400, VOL. 17, PART A FOR EXPLANATION) AND     Linear

RESULT IN ALL ORIGINAL ENERGY POINTS BEING KEPT. CAUTION SHOULD    Linear

BE EXERCISED IN USING THIS OPTION SINCE IT CAN RESULT IN A         Linear

CONSIDERABLE INCREASE IN THE NUMBER OF DATA POINTS OUTPUT BY       Linear

THIS CODE.                                                         Linear

Linear

FOR ALL USERS WHO ARE NOT INTERESTED IN THIS OPTIONS NO CHANGES    Linear

ARE REQUIRED IN THE INPUT TO THIS PROGRAM, I. E. IF COLUMNS        Linear

34-44 ARE BLANK (AS FOR ALL PREVIOUS VERSIONS OF THIS CODE) THE    Linear

PROGRAM WILL OPERATE EXACTLY AS IT DID BEFORE.                     Linear

Linear

ALLOWABLE ERROR                                                    Linear

---------------                                                    Linear

ALLOWABLE ERROR MUST ALWAYS BE SPECIFIED IN THE INPUT TO THIS      Linear

PROGRAM AS A FRACTION, NOT A PER-CENT. FOR EXAMPLE, INPUT THE      Linear

ALLOWABLE FRACTIONAL ERROR 0.001 IN ORDER TO OBTAIN DATA THAT IS   Linear

ACCURATE TO WITHIN 0.1 PER-CENT.                                   Linear

Linear

THE CONVERSION OF THE DATA FROM THE GENERAL INTERPOLATION FORM TO  Linear

LINARLY INTERPOLABLE FORM CANNOT BE PERFORMED EXACTLY. HOWEVER, IT Linear

CAN BE PERFORMED TO VIRTUALLY ANY REQUIRED ACCURACY AND MOST       Linear

IMPORTANTLY CAN BE PERFORMED TO A TOLERANCE THAT IS SMALL COMPARED Linear

TO THE UNCERTAINTY IN THE CROSS SECTIONS THEMSELVES. AS SUCH THE   Linear

CONVERSION OF CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM CAN BE  Linear

PERFORMED WITH ESSENTIALLY NO LOSE OF INFORMATION.                 Linear

Linear

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Linear

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Linear

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Linear

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Linear

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Linear

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Linear

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Linear

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.     Linear

Linear

DEFAULT ALLOWABLE ERROR                                            Linear

-----------------------                                            Linear

IN ORDER TO INSURE CONVERGENCE OF THE LINEARIZING ALGORITHM THE    Linear

ALLOWABLE ERROR MUST BE POSITIVE. IF THE USER INPUTS AN ERROR      Linear

THAT IS NOT POSITIVE IT WILL AUTOMATICALLY BE SET TO THE DEFAULT   Linear

VALUE (CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT) AND         Linear

INDICATED AS SUCH IN THE OUTPUT LISTING.                           Linear

Linear

COULOMB PENETRABILITY (INTERPOLATION LAW = 6)                      Linear

--------------------------------------------                       Linear

INTRODUCED FOR ENDF/B-VI. THIS IS DEFINED AS,                      Linear

Linear

SIG(E) = C1*EXP(-C2/SQRT(E - T))                                   Linear

Linear

THIS PROGRAM ONLY CONSIDERS EXOTHERMIC REACTIONS - T = 0           Linear

Linear

SIG(E) = C1*EXP(-C2/SQRT(E))                                       Linear

Linear

WARNING...THIS INTERPOLATION LAW SHOULD ONLY BE USED FOR REACTIONS Linear

WHICH HAVE A POSITIVE Q-VALUE (EXOTHERMIC REACTIONS),    Linear

SINCE HERE WE ONLY CONSIDER T = 0.0 IN THE FORMALISM.    Linear

IN ALL OTHER CASES A WARNING MESSAGE WILL BE PRINTED.    Linear

Linear

INPUT FILES                                                        Linear

-----------                                                        Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                     Linear

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Linear

Linear

OUTPUT FILES                                                       Linear

------------                                                       Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Linear

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Linear

Linear

SCRATCH FILES                                                      Linear

-------------                                                      Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

12  SCRATCH FILE (BINARY - 180000 WORDS/RECORD                   Linear

Linear

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Linear

----------------------------------------------------               Linear

UNIT  FILE NAME                                                    Linear

----  ----------                                                   Linear

2   LINEAR.INP                                                   Linear

3   LINEAR.LST                                                   Linear

10   ENDFB.IN                                                     Linear

11   ENDFB.OUT                                                    Linear

12   (SCRATCH)                                                    Linear

Linear

Linear

INPUT PARAMETERS                                                   Linear

----------------                                                   Linear

FOR VERSIONS EARLIER THAN 90-1 THIS PROGRAM ONLY ALLOWED THE USER  Linear

TO SPECIFY BY INPUT PARAMETERS WHICH MATERIALS (MAT) TO PROCESS.   Linear

FOR EACH REQUESTED MATERIAL NEUTRON INTERACTION CROSS SECTIONS     Linear

(MF=3) WOULD BE LINEARIZED AND THE REMAINDER OF THE MATERIAL       Linear

WOULD BE COPIED.                                                   Linear

Linear

FOR VERSIONS 90-1 AND LATER THIS PROGRAM WILL ALLOW THE USER TO    Linear

TO SPECIFY BY INPUT PARAMETERS EXACTLY WHAT SECTIONS OF DATA       Linear

TO PROCESS. FOR EACH SECTION OF DATA, SPECIFIED BY MAT, MF, MT     Linear

RANGES, SECTIONS OF MF=3, 23 AND 27 WILL BE LINEARIZED AND ALL     Linear

OTHER REQUESTED SECTIONS WILL BE COPIED. ALL SECTIONS WHICH ARE    Linear

NOT EXPLICITLY REQUESTED WILL BE SKIPPED AND WILL NOT APPEAR ON    Linear

ENDF/B FILE OUTPUT BY THIS PROGRAM.                                Linear

Linear

WITH THIS NEW PROCEDURE YOU CAN MINIMIZE THE SIZE OF THE ENDF/B    Linear

FILE OUTPUT BY THIS PROGRAM, E.G., IF YOU ONLY WANT NEUTRON        Linear

CROSS SECTIONS FOR SUBSEQUENT PROCESSING YOU NEED ONLY REQUEST     Linear

ONLY MF=3 DATA.                                                    Linear

Linear

HOWEVER, YOU MUST UNDERSTAND THAT ONLY THOSE SECTIONS WHICH YOU    Linear

EXPLICITLY REQUEST WILL APPEAR ON THE ENDF/B FILE OUTPUT BY        Linear

THIS PROGRAM. FOR EXAMPLE, IF YOU WISH TO DOCUMENT EXACTLY         Linear

HOW YOU LINEARIZED THE DATA BY INCLUDING COMMENTS IN MF=1, MT=451  Linear

THEN YOU MUST EXPLICITLY REQUEST THAT MF=1, MT=451 BE PROCESSED    Linear

FOR EACH MATERIAL THAT YOU REQUEST. SIMILAR IF YOU WANT THE        Linear

ENTIRE EVALUATION YOU MUST REQUEST ALL MF AND MT TO BE OUTPUT.     Linear

Linear

LINE  COLS.  DESCRIPTION                                           Linear

----  -----  -----------                                           Linear

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                      Linear

12-22  MONITOR MODE SELECTOR                                 Linear

= 0 - NORMAL OPERATION                                Linear

= 1 - MONITOR PROGRESS OF LINEARIZING OF THE DATA.    Linear

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO   Linear

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF  Linear

POINTS ON SCRATCH AND THE LOWER AND UPPER       Linear

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE   Linear

USED IN ORDER TO MONITOR THE EXECUTION SPEED    Linear

OF LONG RUNNING JOBS).                          Linear

23-33  MINIMUM CROSS SECTION OF INTEREST (BARNS).            Linear

(IF 0.0 OR LESS IS INPUT THE PROGRAM WILL             Linear

USE 1.0E-10). ENERGY INTERVALS WILL NOT BE            Linear

SUB-DIVIDED IF THE ABSOLUTE VALUE OF THE CROSS        Linear

SECTION WITHIN THE INTERVAL IS LESS THAN THIS VALUE.  Linear

AN EXCEPTION TO THIS RULE IS NEAR THRESHOLDS ENERGY   Linear

INTERVALS WILL BE SUB-DIVIDED UNTIL CONVERGENCE       Linear

REGARDLESS OF THE MAGNITUDE OF THE CROSS SECTION.     Linear

34-44  KEEP ORIGINAL EVALUATED DATA POINTS.                  Linear

= 0 - NO.                                             Linear

= 1 - YES - ADDITIONAL POINTS MAY BE ADDED IN ORDER   Linear

TO LINEARIZE DATA, BUT ALL ORIGINAL       Linear

DATA POINTS WILL BE INCLUDED IN THE       Linear

RESULTS.                                  Linear

2   1-60  ENDF/B INPUT DATA FILENAME                            Linear

(STANDARD OPTION = ENDFB.IN)                          Linear

3   1-60  ENDF/B OUTPUT DATA FILENAME                           Linear

(STANDARD OPTION = ENDFB.OUT)                         Linear

4-N   1- 6  LOWER MAT OR ZA LIMIT                                 Linear

7- 8  LOWER MF LIMIT                                        Linear

9-11  LOWER MT LIMIT                                        Linear

12-17  UPPER MAT OR ZA LIMIT                                 Linear

18-19  UPPER MF LIMIT                                        Linear

20-22  UPPER MT LIMIT                                        Linear

UP TO 100 RANGES MAY BE SPECIFIED, ONLY ONE RANGE     Linear

PER LINE. THE LIST OF RANGES IS TERMINATED BY A       Linear

BLANK LINE. IF THE UPPER MAT LIMIT OF ANY REQUEST     Linear

IS LESS THAN THE LOW LIMIT IT WILL BE SET EQUAL TO    Linear

THE LOWER LIMIT. IF THE UPPER LIMIT IS STILL ZERO     Linear

IT WILL BE SET EQUAL TO 999999. IF THE UPPER MF OR    Linear

MT LIMIT IS ZERO IT WILL BE SET TO 99 OR 999          Linear

RESPECTIVELY.                                         Linear

VARY  1-11  ENERGY FOR ERROR LAW                                  Linear

12-22  ALLOWABLE FRACTIONAL ERROR FOR ERROR LAW.             Linear

THE ACCEPTABLE LINEARIZING ERROR MAY BE SPECIFIED TO  Linear

BE EITHER ENERGY INDEPENDENT (DEFINED BY A SINGLE     Linear

ERROR), OR ENERGY DEPENDENT (DEFINED BY UP TO 20      Linear

ENERGY, ERROR PAIRS). FOR THE ENERGY DEPENDENT CASE   Linear

LINEAR INTERPOLATION WILL BE USED TO DEFINE THE ERROR Linear

AT ENERGIES BETWEEN THOSE AT WHICH IT IS TABULATED.   Linear

IN ALL CASES THE ERROR LAW IS TERMINATED BY A BLANK   Linear

LINE. IF ONLY ONE ENERGY, ERROR PAIR IS GIVEN THE     Linear

THE LAW WILL BE CONSIDERED TO BE ENERGY INDEPENDENT.  Linear

IF MORE THAN ONE PAIR IS GIVEN IT WILL BE CONSIDERED  Linear

TO BE ENERGY DEPENDENT (NOTE, ENERGY INDEPENDENT      Linear

FORM WILL RUN FASTER THAN THE EQUIVALENT ENERGY       Linear

DEPENDENT FORM). FOR AN ENERGY DEPENDENT ERROR LAW    Linear

ALL ENERGIES MUST BE ASCENDING ENERGY ORDER. FOR      Linear

CONVERGENCE OF THE LINEARIZING ALGORITHM ALL ERRORS   Linear

MUST BE POSITIVE. IF AN ALLOWABLE ERROR IS NOT        Linear

POSITIVE IT WILL BE SET EQUAL TO THE STANDARD OPTION  Linear

(CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT).     Linear

IF THE FIRST ERROR LINE IS BLANK IT WILL TERMINATE    Linear

THE ERROR LAW AND THE ERROR WILL BE TREATED AS        Linear

ENERGY INDEPENDENT, EQUAL TO THE STANDARD OPTION      Linear

(CURRENTLY 0.1 PER-CENT). (SEE EXAMPLE INPUT 4).      Linear

Linear

EXAMPLE INPUT NO. 1                                                Linear

-------------------                                                Linear

RETRIEVE DATA BY ZA IN ORDER TO FIND ALL URANIUM ISOTOPES AND      Linear

THORIUM 232. RETRIEVE ALL NEUTRON INTERACTION CROSS SECTIONS       Linear

(MF=3). ALL ENERGY INTERVALS IN WHICH THE CROSS SECTION IS         Linear

AT LEAST 1 MICRO-BARN (1.0E-06 BARNS) WILL BE SUBDIVIDED.          Linear

BACKWARD THINNING WILL BE PERFORMED. FROM 0 TO 100 EV LINEARIZE    Linear

TO WITHIN 0.1 PER-CENT ACCURACY. FROM 100 EV TO 1 KEV VARY         Linear

ACCURACY BETWEEN 0.1 AND 1.0 PER-CENT. ABOVE 1 KEV USE 1           Linear

PER-CENT ACCURACY.                                                 Linear

Linear

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Linear

Linear

IN THIS CASE THE FOLLOWING 11 INPUT LINES ARE REQUIRED             Linear

Linear

1          0 1.00000- 6          0                            Linear

ENDFB.IN                                                               Linear

ENDFB.OUT                                                              Linear

92000 3  0 92999 3999                                                  Linear

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999) Linear

(END OF REQUEST LIST)                         Linear

0.00000+ 0 1.00000-03                                                  Linear

1.00000+ 2 1.00000-03                                                  Linear

1.00000+ 3 1.00000-02                                                  Linear

1.00000+ 9 1.00000-02                                                  Linear

(END OF ERROR LAW)                            Linear

Linear

EXAMPLE INPUT NO. 2                                                Linear

-------------------                                                Linear

SAME AS THE ABOVE CASE, EXCEPT LINEARIZE ALL DATA TO WITHIN THE    Linear

STANDARD ACCURACY (CURRENTLY 0.1 PER-CENT). IN ORDER TO USE THE    Linear

STANDARD ACCURACY YOU NEED NOT SPECIFY ANY ERROR LAW AT ALL. IN    Linear

THIS CASE INCLUDE THE HOLLERITH SECTION, MF=1, MT=451, FOR EACH    Linear

MATERIAL.                                                          Linear

Linear

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Linear

THEN USE STANDARD FILENAMES.                                       Linear

Linear

IN THIS CASE THE FOLLOWING 9 INPUT LINES ARE REQUIRED              Linear

Linear

1          0 1.00000- 6          0                            Linear

(USE DEFAULT FILENAME = ENDFB.IN)             Linear

(USE DEFAULT FILENAME = ENDFB.OUT)            Linear

92000 1451 92999 1451                                                  Linear

92000 3  0 92999 3999                                                  Linear

90232 1451     0 1451                                                  Linear

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999) Linear

(END OF REQUEST LIST)                         Linear

(0.1 PER-CENT ERROR, END OF ERROR LAW)        Linear

Linear

EXAMPLE INPUT NO. 3                                                Linear

-------------------                                                Linear

LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO WITHIN AN ACCURACY    Linear

OF 0.5 PER-CENT (0.005 AS A FRACTION). IN THIS CASE YOU NEED NOT   Linear

SPECIFY THE MAT, MF, MT RANGES.                                    Linear

Linear

READ THE ENDF/B DATA FROM \ENDFB6\ZA092238 AND WRITE THE ENDF/B    Linear

DATA TO \ENDFB6\LINEAR\ZA092238.                                   Linear

Linear

IN THIS CASE THE FOLLOWING 6 INPUT LINES ARE REQUIRED              Linear

Linear

(MAT, 1.0E-10 BARNS, THIN) Linear

\ENDFB6\ZA092238                                                       Linear

\ENDFB6\LINEAR\ZA092238                                                Linear

(RETRIEVE ALL DATA, END REQUEST LIST)         Linear

5.00000-03                                                  Linear

(END OF ERROR LAW)                            Linear

Linear

NOTE THAT IN THIS CASE IF THE INPUT HAD SPECIFIED AN EQUIVALENT    Linear

ENERGY DEPENDENT ERROR LAW BY GIVING A NUMBER OF ENERGY POINTS     Linear

AT EACH OF WHICH THE ERROR IS 0.5 PER-CENT THE PROGRAM WOULD TAKE  Linear

LONGER TO RUN (I.E., ONLY USE AN ENERGY DEPENDENT ERROR LAW WHEN   Linear

IT IS NECESSARY).                                                  Linear

Linear

EXAMPLE INPUT NO. 4                                                Linear

-------------------                                                Linear

IN ORDER TO LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO THE       Linear

STANDARD OPTION OF 0.1 PER-CENT IT IS ADEQUATE TO INPUT A SET      Linear

OF COMPLETELY BLANK LINES WHICH WILL AUTOMATICALLY INVOKE ALL      Linear

OF THE STANDARD OPTIONS.                                           Linear

Linear

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Linear

THEN USE STANDARD FILENAMES.                                       Linear

Linear

IN THIS CASE THE FOLLOWING THREE INPUT LINES ARE REQUIRED          Linear

Linear

(MAT, 1.0E-10 BARNS, THIN) Linear

(USE DEFAULT FILENAME = ENDFB.IN)             Linear

(USE DEFAULT FILENAME = ENDFB.OUT)            Linear

(RETRIEVE ALL DATA, END REQUEST LIST)         Linear

(0.1 PER-CENT ERROR, END OF ERROR LAW)        Linear

Linear

======================================================================= Linear