======================================================================= Mixer

Mixer

PROGRAM MIXER                                                      Mixer

Mixer

VERSION 76-1 (NOVEMBER 1976)                                       Mixer

VERSION 81-1 (APRIL 1981)  *IBM VERSION                            Mixer

VERSION 82-1 (AUGUST 1982) *COMPUTER INDEPENDENT VERSION           Mixer

VERSION 84-1 (JUNE 1984)   *SPECIAL I/O ROUTINES TO GUARANTEE      Mixer

ACCURACY OF ENERGY.                    Mixer

*DOUBLE PRECISION TREATMENT OF ENERGY   Mixer

(REQUIRED FOR NARROW RESONANCES).      Mixer

VERSION 86-1 (JANUARY 1986)*FORTRAN-77/H VERSION                   Mixer

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Mixer

FILE NAMES (SEE, SUBROUTINE FILIO1     Mixer

AND FILIO2 FOR DETAILS).               Mixer

*IMPROVED BASED ON USER COMMENTS.       Mixer

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Mixer

INSURE PROGRAM WILL NOT DO ANYTHING    Mixer

CRAZY.                                 Mixer

*UPDATED TO USE NEW PROGRAM CONVERT     Mixer

KEYWORDS.                              Mixer

*ADDED LIVERMORE CIVIC COMPILER         Mixer

CONVENTIONS.                           Mixer

VERSION 92-1 (JANUARY 1992)*UPDATED BASED ON USER COMMENTS         Mixer

*ADDED PHOTON CROSS SECTIONS            Mixer

*ADDED FORTRAN SAVE OPTION              Mixer

*OUTPUT IN ENDF/B-VI FORMAT             Mixer

*COMPLETELY CONSISTENT I/O ROUTINES -   Mixer

TO MINIMIZE COMPUTER DEPENDENCE.       Mixer

*NOTE, CHANGE IN INPUT PARAMETER        Mixer

FORMAT.                                Mixer

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Mixer

TO ALLOW ACCESS TO FILE STRUCTURES     Mixer

(WARNING - INPUT PARAMETER FORMAT      Mixer

HAS BEEN CHANGED)                      Mixer

*CLOSE ALL FILES BEFORE TERMINATING     Mixer

(SEE, SUBROUTINE ENDIT)                Mixer

*INCREASED INCORE PAGE SIZE FROM        Mixer

1002 TO 4008.                          Mixer

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Mixer

*IMPROVED COMPUTER INDEPENDENCE        Mixer

*ALL DOUBLE PRECISION                  Mixer

*ON SCREEN OUTPUT                      Mixer

*UNIFORM TREATMENT OF ENDF/B I/O       Mixer

*IMPROVED OUTPUT PRECISION             Mixer

*DEFINED SCRATCH FILE NAMES            Mixer

*INCREASED INCORE PAGE SIZE FROM       Mixer

4008 TO 12000.                        Mixer

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Mixer

POINT READ FOR MORE DIGITS            Mixer

*UPDATED TEST FOR ENDF/B FORMAT        Mixer

VERSION BASED ON RECENT FORMAT CHANGE Mixer

*GENERAL IMPROVEMENTS BASED ON         Mixer

USER FEEDBACK                         Mixer

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Mixer

MF=1, MT-451.                         Mixer

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Mixer

USER FEEDBACK                         Mixer

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Mixer

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Mixer

*INCREASED INCORE PAGE SIZE FROM       Mixer

12000 TO 60000.                       Mixer

Mixer

OWNED, MAINTAINED AND DISTRIBUTED BY                               Mixer

------------------------------------                               Mixer

THE NUCLEAR DATA SECTION                                           Mixer

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Mixer

P.O. BOX 100                                                       Mixer

A-1400, VIENNA, AUSTRIA                                            Mixer

EUROPE                                                             Mixer

Mixer

ORIGINALLY WRITTEN BY                                              Mixer

------------------------------------                               Mixer

DERMOTT E. CULLEN                                                  Mixer

UNIVERSITY OF CALIFORNIA                                           Mixer

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Mixer

L-159                                                              Mixer

P.O. BOX 808                                                       Mixer

LIVERMORE, CA 94550                                                Mixer

U.S.A.                                                             Mixer

TELEPHONE  925-423-7359                                            Mixer

E. MAIL    CULLEN1@LLNL.GOV                                        Mixer

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Mixer

Mixer

PURPOSE                                                            Mixer

-------                                                            Mixer

THIS PROGRAM IS DESIGNED TO CALCULATE THE ENERGY DEPENDENT CROSS   Mixer

SECTION FOR A COMPOSITE MIXTURE OF UP TO 10 DIFFERENT MATERIALS.   Mixer

Mixer

THE PRESENT VERSION WILL ONLY CALCULATE THE CROSS SECTION FOR ONE  Mixer

FINAL REACTION (ENDF/B SECTION), E.G. TOTAL CROSS SECTION, BUT NOT Mixer

ANY OTHER REACTION.                                                Mixer

Mixer

NOTE, THIS PROGRAM WILL NOT COMBINE ALL REACTIONS FOR A MIXTURE    Mixer

OF MATERIALS DURING A SINGLE RUN - ONLY ONE REACTION WILL BE       Mixer

CREATED PER RUN.                                                   Mixer

Mixer

EVALUATED DATA FORMAT                                              Mixer

---------------------                                              Mixer

THE CROSS SECTIONS ARE READ FROM THE ENDF/B FORMAT AND THE         Mixer

COMPOSITE CROSS SECTION IS CONVERTED TO AN EQUIVALENT BARNS/ATOM   Mixer

FORM AND OUTPUT IN THE ENDF/B FORMAT WITH AN EQUIVALENT ATOMIC     Mixer

WEIGHT. THE USER MUST SPECIFY THE COMPOSITION BY GIVING THE ZA,    Mixer

MT AND GRAMS/CC OF EACH CONSTITUENT. IN ADDITION THE USER MUST     Mixer

IDENTIFY THE COMPOSITE CROSS SECTION BY SPECIFYING THE ZA, MAT     Mixer

AND MT TO BE USED IN THE ENDF/B FORMATTED OUTPUT.                  Mixer

Mixer

SINCE ONLY THE CROSS SECTIONS IN FILE 3 AND 23 ARE USED, AND THE   Mixer

FORMAT FOR FILE 3/23 IS THE SAME IN ALL VERSIONS ON ENDF/B, THIS   Mixer

PROGRAM MAY BE USED WITH ANY VERSION OF ENDF/B DATA (I.E.,         Mixer

ENDF/B-I, II, III, IV, V OR VI). DURING A SINGLE RUN IT MAY EVEN   Mixer

BE USED TO READ AND COMBINE EVALUATIONS WHICH ARE IN DIFFERENT     Mixer

VERSIONS OF THE ENDF/B FORMAT.                                     Mixer

Mixer

ENDF/B FORMATTED OUTPUT WILL BE IN THE ENDF/B-VI FORMAT REGARDLESS Mixer

OF THE FORMAT OF THE INPUT ENDF/B DATA. THIS WILL ONLY EFFECT THE  Mixer

HOLLERITH SECTION (MF=1, MT=451). THE FORMAT OF CROSS SECTIONS     Mixer

(MF=3) IS THE SAME IN ALL VERSION OF THE ENDF/B FORMAT.            Mixer

Mixer

IN ORDER TO GUARANTEE PROPER OPERATION OF THIS PROGRAM THE DATA    Mixer

MUST BE PROPERLY CODED IN THE ENDF/B FORMAT. NO ERROR CHECKING IS  Mixer

PERFORMED. IT IS PARTICULARLY IMPORTANT THAT THE FOLLOWING DATA    Mixer

BE CORRECT                                                         Mixer

Mixer

(1) ZA, MF, MT - MUST BE CORRECT IN ORDER TO ALLOW PROGRAM TO      Mixer

SELECT THE APPROPRIATE SECTIONS TO BE COMBINED.                Mixer

(2) AWRE - ATOMIC WEIGHT RATIO MUST BE CORRECT TO ALLOW PROGRAM    Mixer

TO CONVERT THE USER SPECIFIED GRAMS/CC INTO ATOMS/CC FOR       Mixer

PROPER ATOM RATIO MIXING.                                      Mixer

(3) (ENERGIES, CROSS SECTIONS) - MUST BE CORRECT, LINEARLY         Mixer

========         Mixer

INTERPOLABLE, IN ASCENDING ENERGY ORDER OF (E, BARNS).         Mixer

============                                                   Mixer

Mixer

TO CONVERT ENDF/B FORMATTED DATA TO THE REQUIRED INPUT FORM    Mixer

THE FOLLOWING PROGRAMS MAY BE USED,                            Mixer

LINEAR - CONVERT TABULATED CROSS SECTIONS TO LINEARLY          Mixer

INTERPOLABLE FORM.                                    Mixer

RECENT - RECONSTRUCT RESONANCE CONTRIBUTION, ADD TO BACKGROUND Mixer

CROSS SECTION AND OUTPUT THE COMBINATION IN LINEARLY  Mixer

INTERPOLABLE FORM.                                    Mixer

SIGMA1 - DOPPLER BROADEN CROSS SECTIONS TO ANY TEMPERATURE AND Mixer

OUTPUT THE RESULT IN LINEARLY INTERPOLABLE FORM.      Mixer

Mixer

DOCUMENTATION                                                      Mixer

-------------                                                      Mixer

THE FACT THAT THIS PROGRAM HAS COMBINED THE DATA IS DOCUMENTED     Mixer

IN THE OUTPUT ENDF/B FORMAT IN THE HOLLERITH SECTION BY FIRST      Mixer

IDENTIFYING THE VERSION OF THIS PROGRAM THAT WAS USED, IN THE FORM Mixer

Mixer

********************( PROGRAM MIXER 2004-1) ********************** Mixer

Mixer

THIS IS FOLLOWED BY THE TWO LINE IDENTIFICATION INPUT BY THE USER. Mixer

THIS IS FOLLOWED BY COMPOSITION INPUT BY THE USER.                 Mixer

Mixer

NEUTRON OR PHOTON DATA                                             Mixer

----------------------                                             Mixer

THIS PROGRAM WILL ALLOW YOU TO PROCESS EITHER NEUTRON OR PHOTON    Mixer

CROSS SECTIONS - BUT YOU CANNOT MIX THE TWO TYPES TOGETHER. BY     Mixer

INPUT YOU CAN SPECIFY THE OUTPUT MF = 3 (NEUTRONS) OR 23 (PHOTONS) Mixer

WHATEVER TYPE YOU SPECIFIED FOR OUTPUT IS THE ONLY TYPE OF DATA    Mixer

WHICH WILL BE PROCESSED BY THIS PROGRAM.                           Mixer

Mixer

DEFINING THE COMPOSITION                                           Mixer

------------------------                                           Mixer

THE USER MAY SPECIFY UP TO 10 DIFFERENT SECTIONS OF DATA TO BE     Mixer

COMBINED, EACH SECTION IDENTIFIED BY ZA AND MT NUMBER. THE         Mixer

AMOUNT OF EACH MATERIAL IS SPECIFIED BY DEFINING THE NUMBER OF     Mixer

GRAMS/CC OF EACH MATERIAL IN THE COMPOSITE MIXTURE. THIS CAN BE    Mixer

DERIVED FROM THE VOLUME FRACTION SIMPLY BY MULTIPLYING THE STP     Mixer

DENSITY OF EACH MATERIAL BY ITS VOLUME FRACTION. NOTE, DO NOT      Mixer

INPUT ATOM FRACTIONS.                                              Mixer

Mixer

THE LIST OF SECTIONS TO BE COMBINED MAY BE SPECIFIED IN ANY        Mixer

ORDER, I.E. THEY NEED NOT BE IN ZA ORDER OR THE ORDER THAT THE     Mixer

EVALUATED DATA APPEARS ON THE ENDF/B FORMATTED TAPE.               Mixer

Mixer

IF ANY REQUESTED SECTION OF DATA IS NOT FOUND ON THE ORIGINAL      Mixer

ENDF/B FORMATTED FILE, THE PROGRAM WILL PRINT A LIST OF THE        Mixer

MISSING SECTIONS AND TERMINATE. IF ALL REQUESTED SECTIONS ARE      Mixer

FOUND THE PROGRAM WILL PRODUCE A COMPOSITE SECTION USING THE       Mixer

UNION OF ALL ENERGIES FOUND IN ANY SECTION. THE COMPOSITE SECTION  Mixer

WILL NOT BE THINNED.                                               Mixer

Mixer

PRIOR TO LATER USE IN ANY APPLICATION THE NUMBER OF ENERGY POINTS  Mixer

IN THE COMPOSITE CROSS SECTION MAY BE MINIMIZED BY USING PROGRAM   Mixer

LINEAR, UCRL-50400, VOL. 17, PART B TO THIN THE DATA.              Mixer

Mixer

ONLY LINEARLY INTERPOLABLE DATA                                    Mixer

-------------------------------                                    Mixer

THE CROSS SECTIONS TO BE COMBINED MUST BE IN LINEARLY INTERPOLABLE Mixer

TABULATED FORM (I. E., FILE 3 OR 23, INTERPOLATION LAW 2).         Mixer

Mixer

TO CONVERT TABULATED CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM  Mixer

SEE, PROGRAM LINEAR, UCRL-50400, VOL. 17, PART A.                  Mixer

Mixer

TO CONVERT RESONANCE PARAMETERS TO LINEARLY INTERPOLABLE FORM SEE, Mixer

PROGRAM RECENT, UCRL-50400, VOL. 17, PART C.                       Mixer

Mixer

TO DOPPLER BROADEN LINEARLY INTERPOLABLE DATA TO ANY TEMPERATURE   Mixer

SEE PROGRAM SIGMA1, UCRL-50400, VOL. 17, PART B.                   Mixer

Mixer

PAGING SYSTEM                                                      Mixer

-------------                                                      Mixer

THERE IS NO LIMIT TO THE THE NUMBER OF DATA POINTS IN EACH OF THE  Mixer

SECTIONS TO BE COMBINED, NOR IS THERE A LIMIT TO THE NUMBER OF     Mixer

DATA POINTS IN THE COMPOSITE MIXTURE CROSS SECTION.                Mixer

Mixer

ALL REQUIRED SECTIONS OF DATA ARE READ FROM THE ORIGINAL ENDF/B    Mixer

FORMATTED FILE. ANY SECTION OF 60000 OR FEWER POINTS WILL BE       Mixer

TOTALLY CORE RESIDENT. LARGER SECTIONS ARE LOADED INTO A PAGING    Mixer

SYSTEM USING A SCRATCH FILE WITH ONLY 60000 POINTS PER SECTION     Mixer

CORE RESIDENT AT ANY ONE TIME. SIMILARLY THE COMPOSITE SECTION     Mixer

WILL BE TOTALLY CORE RESIDENT IF IT CONTAINS 60000 OR FEWER POINTS Mixer

AND LARGER COMPOSITE SECTIONS WILL BE LOADED INTO A PAGING         Mixer

SYSTEM WHERE ONLY 60000 POINTS ARE CORE RESIDENT AT ANY TIME. SINC Mixer

A PAGING SYSTEM MAY BE USED BY ANY SECTION OF DATA THERE IS NO     Mixer

LIMIT TO THE SIZE OF EITHER THE ORIGINAL SECTIONS, NOR TO THE      Mixer

COMPOSITE SECTION, E.G. A SECTION MAY CONTAIN 100,000 ENERGIES     Mixer

AND CROSS SECTIONS TO DESCRIBE A GIVEN REACTION.                   Mixer

Mixer

PAGE SIZE                                                          Mixer

---------                                                          Mixer

THE PAGE SIZE USED IN THIS PROGRAM IS DEFINED BY THE PARAMETER     Mixer

NPAGE AND THE DIMENSIONS OF THE ARRAYS XTAB AND YTAB. IN ORDER     Mixer

TO ADAPT THIS PROGRAM FOR USE ON ANY COMPUTER THE PAGE SIZE MAY    Mixer

BE INCREASED OR DECREASED BUT THE FOLLOWING RULES MUST BE FOLLOWED Mixer

====             Mixer

Mixer

(1) NPAGE - MUST BE A MULTIPLE OF 3 IN ORDER TO ALLOW THE PROGRAM  Mixer

TO READ FULL CARDS OF ENDF/B DATA (3 POINTS PER LINE). FAILURE Mixer

TO FOLLOW THIS RULE CAN LEAD TO LOSS OF DATA AND/OR PROGRAM    Mixer

ERRORS DURING EXECUTION.                                       Mixer

(3) YTAB  - THE DIMENSION OF YTAB MUST BE (NPAGE,11).              Mixer

(4) XTAB  - THE DIMENSION OF XTAB MUST BE (NPAGE,11).              Mixer

Mixer

DOPPLER BROADENING                                                 Mixer

------------------                                                 Mixer

THE COMPOSITE CROSS SECTION OUTPUT FROM THIS PROGRAM SHOULD NOT    Mixer

BE DOPPLER BROADENED USING PROGRAM SIGMA1, OR THE EQUIVALENT. THE  Mixer

ATOMIC WEIGHT USED TO IDENTIFY THE COMPOSITE MIXTURE IS BASED ON   Mixer

THE ATOM FRACTION OF EACH CONSTITUENT AND CANNOT BE USED TO        Mixer

CHARACTERIZE THE BROADENING OF ANY GIVEN RESONANCE IN THE MIXTURE  Mixer

DUE TO THE CONTRIBUTION OF ONE CONSTITUENT. IN ORDER TO CONSIDER   Mixer

DOPPLER BROADENING FIRST USE PROGRAM SIGMA1 TO BROADEN THE CROSS   Mixer

SECTION FOR EACH OF THE CONSTITUENTS AND THEN COMBINE THE          Mixer

BROADENED DATA USING PROGRAM MIXER.                                Mixer

Mixer

EXAMPLE USE                                                        Mixer

-----------                                                        Mixer

THE OUTPUT FROM THIS PROGRAM HAS BEEN FOUND TO BE EXTREMELY        Mixer

USEFUL IN THE FOLLOWING APPLICATIONS...                            Mixer

Mixer

(1) CALCULATE A COMPOSITE TOTAL CROSS SECTON FOR LATER USE AS      Mixer

A WEIGHTING FUNCTION IN SELF-SHIELDING THE CROSS SECTIONS      Mixer

OF EACH CONSTITUENT OF THE MIXTURE SEPARATELY.                 Mixer

Mixer

PROGRAM GROUPIE CAN USE THE CALCULATED COMPOSITE TOTAL CROSS   Mixer

SECTION AS THE TOTAL CROSS SECTION FOR EACH CONSTITUENT OF     Mixer

THE MIXTURE IN ORDER TO CALCULATE  SELF-SHIELDED CROSS SECTION Mixer

FOR EACH CONSTITUENT OF THE MIXTURE.                           Mixer

Mixer

(2) CALCULATE COMPOSITE TOTAL AND FISSION CROSS SECTIONS IN        Mixer

ORDER TO CALCULATE THE TRANSMISSION AND SELF-INDICATION        Mixer

THROUGH COMPOSITE MATERIALS. GENERALLY IN THIS CASE THE        Mixer

TOTAL CROSS SECTION WILL BE CALCULATED FOR THE COMPOSITION     Mixer

OF THE SAMPLE AND THE FISSION CROSS SECTION WILL BE            Mixer

CALCULATED FOR THE COMPOSITION OF THE FISSION CHAMBER          Mixer

(WHICH GENERALLY WILL HAVE A DIFFERENT COMPOSITION THAN THE    Mixer

SAMPLE).                                                       Mixer

Mixer

PROGRAM VIRGIN CAN USE THE OUTPUT FROM THIS PROGRAM TO         Mixer

PERFORM TRANSMISSION AND SELF-INDICATION CALCULATIONS.         Mixer

PROGRAM VIRGIN WILL ANALYTICALLY CALCULATE THE UNCOLLIDED      Mixer

(I.E. VIRGIN) FLUX TRANSMITTED AND REACTION RATE DUE TO ANY    Mixer

TABULATED LINEARLY INTERPOLABLE INCIDENT SPECTRUM. RESULTS     Mixer

WILL BE PRESENTLY FOR UP TO 10 DIFFERENT SAMPLE THICKNESSES    Mixer

AND BINNED INTO ENERGY GROUPS IN ORDER TO SIMULATE AN          Mixer

EXPERIMENTAL MEASUREMENT.                                      Mixer

Mixer

(3) THE OUTPUT FROM THIS PROGRAM IS VERY USEFUL TO PLOT IN ORDER   Mixer

TO SEE THE IMPORTANCE OF SPECIFIC CROSS SECTION FEATURES IN    Mixer

THE COMPOSITE CROSS SECTION.                                   Mixer

Mixer

PROGRAM COMPLOT CAN BE USED TO PLOT THE OUTPUT FROM THIS       Mixer

PROGRAM AND IF REQUIRED EXAMINE ANY PARTICULAR ENERGY RANGE    Mixer

IN DETAIL. IN ORDER TO DO THIS THE (ZA, MT) EQUIVALENCE OPTION Mixer

OF PROGRAM COMPLOT SHOULD BE USED. TO COMPARE ANY CONSTITUENT  Mixer

CROSS SECTION TO THE COMPOSITE CROSS SECTION THE INPUT TO      Mixer

COMPLOT SHOULD EQUATE THE (ZA,MT) OF THE COMPOSITE TO THE      Mixer

(ZA,MT) OF ONE CONSTITUENT AND THE MULTIPLIER INPUT TO         Mixer

COMPLOT SHOULD BE THE ATOM FRACTION FOR THE CONSTITUENT (THE   Mixer

ATOM FRACTIONS ARE DEFINED IN THE OUTPUT LISTING FROM PROGRAM  Mixer

MIXER).                                                        Mixer

Mixer

INPUT FILES                                                        Mixer

-----------                                                        Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

2   INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                     Mixer

10   ORIGINAL EVALUATED DATA IN ENDF/B FORMAT                     Mixer

(BCD - 80 CHARACTERS/RECORD)                     Mixer

Mixer

OUTPUT FILES                                                       Mixer

------------                                                       Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

3   OUTPUT LISTING (BCD - 120 CHARACTERS/RECORD)                 Mixer

11   COMPOSITE EVALUATED DATA IN ENDF/B FORMAT                    Mixer

(BCD - 80 CHARACTERS/RECORD)                       Mixer

Mixer

SCRATCH FILES                                                      Mixer

-------------                                                      Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

12   SCRATCH FILE FOR EACH OF THE 10 SECTIONS WHICH               Mixer

13   WILL BE ADDED TOGETHER TO DEFINE THE FINAL                   Mixer

.   SECTION (BINARY - 60000 AND 480000 WORDS/RECORD)             Mixer

. .                                                              Mixer

. .                                                              Mixer

20 .                                                              Mixer

21 .                                                              Mixer

22   SCRATCH FILE FOR COMBINED SECTION.                           Mixer

(BINARY - 2004 WORDS/RECORD)                                 Mixer

Mixer

STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)            Mixer

----------------------------------------------------------------   Mixer

UNIT  FILE NAME                                                    Mixer

----  ----------                                                   Mixer

2   MIXER.INP                                                    Mixer

3   MIXER.LST                                                    Mixer

10   ENDFB.IN                                                     Mixer

11   ENDFB.OUT                                                    Mixer

12-22  (SCRATCH)                                                    Mixer

Mixer

INPUT CARDS                                                        Mixer

-----------                                                        Mixer

LINE  COLS.  FORMAT  NAME    DESCRIPTION                           Mixer

----  -----  ------  ------- ----------                            Mixer

1-2   1-66 16A4,A2  TITLE   TWO LINE TITLE DESCRIBING PROBLEM     Mixer

(THIS TITLE IS USED TO IDENTIFY THE   Mixer

OUTPUT LISTING AND IS ALSO WRITTEN    Mixer

IN MF=1, MT=451 (HOLLERITH SECTION)   Mixer

OF THE ENDF/B FORMATTED OUTPUT TO     Mixer

IDENTIFY THE COMPOSITE MIXTURE).      Mixer

3   1-60                  ENDF/B INPUT DATA FILENAME            Mixer

(STANDARD OPTION = ENDFB.IN)          Mixer

4   1-60                  ENDF/B OUTPUT DATA FILENAME           Mixer

(STANDARD OPTION = ENDFB.OUT)         Mixer

5   1-11    I11   IZAOUT  ZA IDENTIFICATION FOR COMBINATION     Mixer

5  12-17    I6    MATOUT  MAT IDENTIFICATION FOR COMBINATION    Mixer

5  18-19    I2    MFOUT   MF IDENTIFICATION FOR COMBINATION     Mixer

5  20-22    I3    MTOUT   MT IDENTIFICATION FOR COMBINATION     Mixer

6-N   1-11    I11   IZAGET  ZA (1000*Z+A) OF MATERIAL             Mixer

6-N  12-22    I11   MTGET   MT OF REACTION                        Mixer

6-N  23-33  E11.4   DENSE   DENSITY OF MATERIAL (GRAMS/CC)        Mixer

Mixer

THE SIXTH LINE IS REPEATED FOR EACH SECTION (FROM 2 TO 10).        Mixer

SINCE THE ENDF/B FORMATTED OUTPUT IS IN BARNS/ATOM FORM A MINIMUM  Mixer

OF TWO SECTIONS MUST BE COMBINED (I.E., IF ONLY ONE SECTION IS     Mixer

SPECIFIED THE OUTPUT WOULD BE IDENTICAL TO THE INPUT AND AS SUCH   Mixer

THE PROGRAM WILL CONSIDER THIS TO BE AN ERROR AND NOT PERFORM THE  Mixer

CALCULATION). THE LIST OF SECTIONS IS TERMINATED BY A BLANK LINE.  Mixer

Mixer

THE LIST OF SECTIONS TO BE COMBINED MAY BE SPECIFIED IN ANY        Mixer

ORDER, I.E. THEY NEED NOT BE IN ZA ORDER OR THE ORDER THAT THE     Mixer

EVALUATED DATA APPEARS ON THE ENDF/B FORMATTED TAPE.               Mixer

Mixer

EXAMPLE INPUT NO. 1                                                Mixer

-------------------                                                Mixer

CREATE THE TOTAL CROSS SECTION (MT=1) FOR STAINLESS STEEL AND      Mixer

IDENTIFY THE COMBINED MATERIAL WITH ZA=26800 AND MAT=4000,         Mixer

THE COMPOSITION BY VOLUME OF THE STEEL WILL BE...                  Mixer

Mixer

THE DATA FROM \ENDFB6\K300\LIBRARY.DAT AND WRITE DATA TO           Mixer

\MIXER\STEEL.DAT                                                   Mixer

Mixer

IRON      - 74.8 PER-CENT                                          Mixer

CHROMIUM  - 16.0                                                   Mixer

NICKEL    -  6.0                                                   Mixer

MANGANESE -  2.0                                                   Mixer

SILICON   -  1.0                                                   Mixer

CARBON    -  0.2                                                   Mixer

Mixer

THE INPUT MUST SPECIFY THE COMPOSITION BY GRAMS/CC. THIS IS        Mixer

DEFINED AS THE PRODUCT OF THE STANDARD DENSITY (GRAMS/CC)          Mixer

TIMES THE VOLUME FRACTION. NOTE, DO NOT USE ATOM FRACTIONS.        Mixer

FOR THIS EXAMPLE THE FOLLOWING 12 INPUT CARDS ARE REQUIRED....     Mixer

Mixer

STAINLESS STEEL. COMPOSITION BY PER-CENT VOLUME IS 74.8-IRON,      Mixer

16-CHROME, 6-NICKEL, 2-MANGANESE, 1-SILICON, 0.2-CARBON            Mixer

\ENDFB6\K300\LIBRARY.DAT                                           Mixer

\MIXER\STEEL.DAT                                                   Mixer

26800  4000 3  1                                              Mixer

26000          1 5.88676    (NOTE, GRAMS/CC INPUT FOR EACH    Mixer

24000          1 1.150448    CONSTITUENT, E.G. FOR IRON THE   Mixer

28000          1 0.533928    STP DENSITY IS 7.87 GRAMS/CC.    Mixer

25055          1 0.1486      THE INPUT VALUE OF 5.88676 IS    Mixer

14000          1 0.0233      0.748 X 7.87,I.E.  VOLUME        Mixer

6012          1 0.0044958   FRACTION TIMES STP DENSITY).     Mixer

(BLANK LINE TERMINATES INPUT LIST) Mixer

Mixer

EXAMPLE INPUT NO. 2                                                Mixer

-------------------                                                Mixer

THE SAME EXAMPLE AS THE ABOVE PROBLEM, ONLY USE THE STANDARD       Mixer

ENDF/B DATA FILENAMES - ENDFB.IN AND ENDFB.OUT (THIS CAN BE        Mixer

DONE BY LEAVING THE THIRD AND FOURTH INPUT LINES BLANK).           Mixer

FOR THIS EXAMPLE THE FOLLOWING 12 INPUT CARDS ARE REQUIRED....     Mixer

Mixer

STAINLESS STEEL. COMPOSITION BY PER-CENT VOLUME IS 74.8-IRON,      Mixer

16-CHROME, 6-NICKEL, 2-MANGANESE, 1-SILICON, 0.2-CARBON            Mixer

(NOTE - THIS LINE IS REALLY BLANK)                                 Mixer

(NOTE - THIS LINE IS REALLY BLANK)                                 Mixer

26800  4000 3  1                                              Mixer

26000          1 5.88676    (NOTE, GRAMS/CC INPUT FOR EACH    Mixer

24000          1 1.150448    CONSTITUENT, E.G. FOR IRON THE   Mixer

28000          1 0.533928    STP DENSITY IS 7.87 GRAMS/CC.    Mixer

25055          1 0.1486      THE INPUT VALUE OF 5.88676 IS    Mixer

14000          1 0.0233      0.748 X 7.87,I.E.  VOLUME        Mixer

6012          1 0.0044958   FRACTION TIMES STP DENSITY).     Mixer

(BLANK LINE TERMINATES INPUT LIST) Mixer

Mixer

======================================================================= Mixer