======================================================================= Sixpak

Sixpak

PROGRAM SIXPAK                                                     Sixpak

================================================================== Sixpak

VERSION 92-1 (JANUARY 1992)                                        Sixpak

VERSION 92-2 (FEBRUARY 1992)*INCREASED CORE ALLOCATION TO          Sixpak

ACCOMMODATE JEF AND EFF EVALUATIONS.  Sixpak

VERSION 92-3 (APRIL 1992)   *ADDED ADDITIONAL DATA TESTS.          Sixpak

VERSION 92-4 (SEPT. 1992)   *CORRECTED KALBACH-MANN CALCULATIONS.  Sixpak

*FOR PHOTON PRODUCTION OUTPUT MF=12    Sixpak

(MULTIPLICITY), MF=14 (ISOTROPIC      Sixpak

ANGULAR DISTRIBUTIONS) AND MF=15      Sixpak

(SPECTRA) - PREVIOUSLY ONLY MF=15.    Sixpak

*FIRST ORDER CORRECTIONS TRANSFORMING  Sixpak

CENTER-OF-MASS SPECTRA TO LAB SYSTEM  Sixpak

FOR OUTPUT IN MF=5                    Sixpak

*CORRECTED ISOTROPIC ANGULAR           Sixpak

DISTRIBUTION FLAG (LI)                Sixpak

VERSION 94-1 (JANUARY 1994) *VARIABLE ENDF/B INPUT DATA FILENAME   Sixpak

TO ALLOW ACCESS TO FILE STRUCTURES    Sixpak

(WARNING - INPUT PARAMETER FORMAT     Sixpak

HAS BEEN CHANGED)                     Sixpak

*CLOSE ALL FILES BEFORE TERMINATING    Sixpak

(SEE, SUBROUTINE ENDIT)               Sixpak

*INCREASED MAXIMUM TABLE SIZE FROM     Sixpak

2000 TO 6000.                         Sixpak

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Sixpak

*IMPROVED COMPUTER INDEPENDENCE        Sixpak

*ALL DOUBLE PRECISION                  Sixpak

*ON SCREEN OUTPUT                      Sixpak

*UNIFORM TREATMENT OF ENDF/B I/O       Sixpak

*IMPROVED OUTPUT PRECISION             Sixpak

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Sixpak

POINT READ FOR MORE DIGITS            Sixpak

*UPDATED TEST FOR ENDF/B FORMAT        Sixpak

VERSION BASED ON RECENT FORMAT CHANGE Sixpak

*GENERAL IMPROVEMENTS BASED ON         Sixpak

USER FEEDBACK                         Sixpak

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Sixpak

MF=1, MT-451.                         Sixpak

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Sixpak

USER FEEDBACK                         Sixpak

VERS. 2002-1 (JANUARY 2002) *CORRECTED ANGULAR DISTRIBUTION (MF=4) Sixpak

OUTPUT TO INSURE USED FIELDS ARE 0    Sixpak

(MAY 2002)     *OPTIONAL INPUT PARAMETERS             Sixpak

(NOV. 2002)    *EXTENDED TO ALLOW CHARGED PARTICLE    Sixpak

ANGULAR DISTRIBUTION IN MF=4 -        Sixpak

WARNING - STRICTLY SPEAKING THIS IS   Sixpak

NOT LEGAL, SINCE MF=4 IS SUPPOSED TO  Sixpak

BE USED ONLY FOR NEUTRON ANGULAR      Sixpak

DISTRIBUTIONS - BUT WHERE MT MAKES    Sixpak

IT OBVIOUS THAT THE OUTGOING PARTICLE Sixpak

IS NOT A NEUTRON HOPEFULLY IT WILL    Sixpak

NOT CAUSE A PROBLEM IF MF=4 IS USED   Sixpak

FOR CHARGED PARTICLES.                Sixpak

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Sixpak

*INCREASED MAXIMUM TABLE SIZE FROM     Sixpak

6,000 TO 12,000.                      Sixpak

*ADDED DUMMY A FOR ELEMENTS           Sixpak

*CORRECTED OUTPUT INTERPOLATON LAWS   Sixpak

Sixpak

OWNED, MAINTAINED AND DISTRIBUTED BY                               Sixpak

------------------------------------                               Sixpak

THE NUCLEAR DATA SECTION                                           Sixpak

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Sixpak

P.O. BOX 100                                                       Sixpak

A-1400, VIENNA, AUSTRIA                                            Sixpak

EUROPE                                                             Sixpak

Sixpak

ORIGINALLY WRITTEN BY                                              Sixpak

------------------------------------                               Sixpak

DERMOTT E. CULLEN                                                  Sixpak

UNIVERSITY OF CALIFORNIA                                           Sixpak

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Sixpak

L-159                                                              Sixpak

P.O. BOX 808                                                       Sixpak

LIVERMORE, CA 94550                                                Sixpak

U.S.A.                                                             Sixpak

TELEPHONE  925-423-7359                                            Sixpak

E. MAIL    CULLEN1@LLNL.GOV                                        Sixpak

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Sixpak

Sixpak

COLLABORATION                                                      Sixpak

================================================================== Sixpak

DEVELOPED IN COLLABORATION WITH,                                   Sixpak

Sixpak

*THE NATIONAL NUCLEAR DATA CENTER, BROOKHAVEN NATIONAL LAB         Sixpak

Sixpak

*THE NUCLEAR DATA SECTION, IAEA, VIENNA, AUSTRIA                   Sixpak

Sixpak

*CENTRO TECNICO AEROSPACIAL, SAO JOSE DOS CAMPOS, BRAZIL           Sixpak

Sixpak

AS A PART OF AN INTERNATIONAL PROJECT ON THE EXCHANGE OF           Sixpak

NUCLEAR DATA                                                       Sixpak

Sixpak

ACKNOWLEDGEMENT (VERSION 92-1)                                     Sixpak

================================================================== Sixpak

THE AUTHOR THANKS SOL PEARLSTEIN (BROOKHAVEN NATIONAL LAB) FOR     Sixpak

SIGNIFICANTLY CONTRIBUTING TOWARD IMPROVING THE ACCURACY AND       Sixpak

COMPUTER INDEPENDENCE OF THIS CODE - THANKS, SOL                   Sixpak

Sixpak

ACKNOWLEDGEMENT (VERSION 92-4)                                     Sixpak

================================================================== Sixpak

THE AUTHOR THANKS BOB MACFARLANE (LOS ALAMOS) FOR SUGGESTING HOW   Sixpak

TO PROPERLY OUTPUT THE PHOTON PRODUCTION DATA TO PUT IT INTO       Sixpak

EXACTLY THE FORM NEEDED FOR USE IN PROCESSING CODES.               Sixpak

Sixpak

THE AUTHOR THANKS CHRIS DEAN (WINFRITH) FOR POINTING OUT ERRORS    Sixpak

IN THE EARLIER TREATMENT OF THE KALBACH-MANN FORMALISM AND IN      Sixpak

THE DEFINITION OF THE ISOTROPIC ANGULAR DISTRIBUTION FLAG (LI).    Sixpak

Sixpak

AUTHORS MESSAGE                                                    Sixpak

================================================================== Sixpak

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENTATION   Sixpak

INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ ALL OF THESE        Sixpak

COMMENTS BEFORE IMPLEMENTING AND USING THESE CODES.                Sixpak

Sixpak

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Sixpak

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Sixpak

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Sixpak

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Sixpak

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Sixpak

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Sixpak

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Sixpak

COMPUTER.                                                          Sixpak

Sixpak

PURPOSE                                                            Sixpak

================================================================== Sixpak

1) CHECK ALL DOUBLE-DIFFERENTIAL DATA (MF=6)                       Sixpak

Sixpak

2) OUTPUT EQUIVALENT MF = 4, 5, 12, 14 AND 15 DATA.                Sixpak

Sixpak

DATA CHECKING                                                      Sixpak

================================================================== Sixpak

ALL OF THE ENDF/B-VI MF=6 DATA IS CHECKED - FOR DETAILS SEE BELOW. Sixpak

Sixpak

THE MF=6 DATA IS NOT CORRECTED AND OUTPUT IN THE ENDF/B FORMAT.    Sixpak

IT IS MERELY CHECKED. IF ERRORS ARE FOUND IT IS UP TO THE USER     Sixpak

TO TAKE CORRECTIVE ACTION ON THE MF=6 DATA.                        Sixpak

Sixpak

IN CONTRAST WHEN PROBLEMS ARE FOUND IN DATA WHICH WILL BE OUTPUT   Sixpak

IN THE ENDF/B FORMAT (MF=4, 5, 12, 14 AND 15), WHENEVER POSSIBLE   Sixpak

CORRECTIVE ACTION WILL BE TAKEN.                                   Sixpak

Sixpak

FURTHER CHECKS AND CORRECTIONS                                     Sixpak

================================================================== Sixpak

ONCE THE DATA HAS BEEN OUTPUT IN MF = 4, 5, 12, 14 AND 15 FORMATS  Sixpak

FURTHER CORRECTIVE ACTION CAN BE TAKEN AS FOLLOWS,                 Sixpak

Sixpak

PROGRAM LEGEND                                                     Sixpak

==============                                                     Sixpak

CAN BE USED TO CORRECT ANGULAR DISTRIBUTIONS WHICH ARE NEGATIVE,   Sixpak

TO CONVERT FROM LEGENDRE COEFFICIENTS TO TABULATED ANGULAR         Sixpak

DISTRIBUTIONS AND GENERALLY PERFORM MORE EXTENSIVE TESTS OF        Sixpak

ALL MF=4 DATA.                                                     Sixpak

Sixpak

PROGRAM EVALPLOT                                                   Sixpak

================                                                   Sixpak

VERSION 92-1 AND LATER VERSIONS CAN PLOT ALL OF THE MF=4, 5 AND 15 Sixpak

DATA OUTPUT BY THIS CODE. EARLIER VERSIONS CAN PLOT MF=4 AND 5.    Sixpak

GRAPHICS IS AN EXCELLENT WAY TO CHECK THIS DATA.                   Sixpak

Sixpak

PROGRAM PLOTTAB                                                    Sixpak

===============                                                    Sixpak

THIS IS A GENERAL PLOTTING PROGRAM AND THERE IS AN INTERFACE IN    Sixpak

THIS CODE TO PRODUCE OUTPUT FOR ANY MF=6 DATA IN THE PLOTTAB       Sixpak

INPUT FORMAT. THIS PROGRAM CAN BE USED TO CHECK ALL OF THE MF=6    Sixpak

DATA AS WELL AS THE EQUIVALENT MF=4, 5, 12, 14 AND 15 DATA - AS    Sixpak

WELL AS COMPARING THE ORIGINAL MF=6 AND EQUIVALENT DATA.           Sixpak

Sixpak

DATA OUTPUT                                                        Sixpak

================================================================== Sixpak

THE ENDF/B MF=4, 5, 12, 14 AND 15 FORMATS ONLY ALLOW FOR NEUTRONS  Sixpak

INCIDENTS                                                          Sixpak

Sixpak

THE ENDF/B MF=4 AND 5 FORMATS ONLY ALLOW FOR NEUTRONS OUTGOING.    Sixpak

Sixpak

THE ENDF/B MF=12, 14 AND 15 ONLY ALLOWS FOR PHOTONS OUTGOING.      Sixpak

Sixpak

THESE ARE THE ONLY COMBINATIONS OF DATA OUTPUT BY THIS CODE.       Sixpak

Sixpak

ALL OTHER COMBINATIONS OF INCIDENT AND OUTGOING PARTICLES ARE      Sixpak

CHECKED, BUT THE RESULTS CANNOT BE OUTPUT IN THE ENDF/B FORMAT.    Sixpak

HOWEVER, USING THE PLOTTAB INTERFACE BUILT INTO THIS CODE THIS     Sixpak

DATA CAN, AND HAS BEEN, OUTPUT AND CHECKED.                        Sixpak

Sixpak

THE NEUTRON DATA IN MF=4 CAN BE IN THE FORM OF EITHER TABULATED    Sixpak

ANGULAR DISTRIBUTIONS OR LEGENDRE COEFFICIENTS.                    Sixpak

Sixpak

THE NEUTRON (MF=5) OR PHOTON (MF=15) SPECTRA ARE BOTH IN EXACTLY   Sixpak

THE SAME FORMAT = ARBITRARY TABULATED FUNCTIONS - ENDF/B OPTION    Sixpak

LF=1.                                                              Sixpak

Sixpak

ENDF/B DATA OUTPUT ORDER                                           Sixpak

================================================================== Sixpak

ENDF/B DATA IS OUTPUT IN ASCENDING MAT, MF, MT ORDER. IN ORDER TO  Sixpak

ALLOW THIS PROGRAM TO PRODUCE ALL OUTPUT IN A SINGLE PASS THROUGH  Sixpak

THE MF=6 DATA, OUTPUT FOR EACH (MAT, MT) IS OUTPUT TO SEPERATE     Sixpak

FILES FOR MF=4, 5, 12, 14 AND 15.                                  Sixpak

Sixpak

FOR SUBSEQUENT USE THE ENDF/B FORMATTED DATA OUTPUT BY THIS CODE   Sixpak

CAN BE MERGED TOGETHER USING PROGRAM MERGER (CONTAIN THE AUTHOR    Sixpak

OF THIS CODE FOR A COPY OF MERGER), E.G., MERGE MF=12, 14 AND 15   Sixpak

DATA IN ORDER TO THEN CALCULATE PHOTON PRODUCTION DATA OR MF=4     Sixpak

AND 5 CAN BE MERGED TOGETHER TO CALCULATE NEUTRON TRANSFER - OR    Sixpak

ALL OF THEM CAN BE MERGED TOGETHER TO PERFORM NEUTRON AND PHOTON   Sixpak

CALCULATIONS.                                                      Sixpak

Sixpak

CORRELATED (MF=6) VS. UNCORRELATED (MF=4 AND 5) DATA               Sixpak

================================================================== Sixpak

THE ENDF/B DOUBLE DIFFERENTAL = CORRELATED - DATA IN MF=6          Sixpak

REPRESENTS DATA IN THE FORM,                                       Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

SIG(E)      = MF=3 CROSS SECTIONS                                  Sixpak

Y(E)        = YIELD (MULTIPLICITY)                                 Sixpak

G0(E,EP)    = ENERGY SPECTRUM                                      Sixpak

F(E,EP,COS) = ANGULAR DISTRIBUTION                                 Sixpak

Sixpak

IN A SITUATION WHERE YOU HAVE MONOENERGETIC AND MONODIRECTIONAL    Sixpak

NEUTRONS INCIDENT YOU WILL BE ABLE TO OBSERVE CORRELATION EFFECTS  Sixpak

IN THE NEUTRON SPECTRUM AND ANGULAR DISTRIBUTION.                  Sixpak

Sixpak

EVEN IN SITUATIONS WHERE YOU HAVE A NARROW SPECTRUM OF NEUTRONS    Sixpak

THAT ARE HIGHLY DIRECTIONALLY ORIENTED YOU MAY BE ABLE TO OBSERVE  Sixpak

THESE CORRELATION EFFECTS, E.G., A NARROW 14 MEV FUSION SOURCE     Sixpak

INCIDENT ON THE FIRST WALL OF A CTR DEVICE.                        Sixpak

Sixpak

FOR SUCH SITUATIONS USE OF THE CORRELATED (MF=6) DATA IS REQUIRED  Sixpak

IN CALCULATIONS.                                                   Sixpak

Sixpak

HOWEVER, IN MANY APPLICATIONS WHERE THERE IS A BROAD SPECTRUM OF   Sixpak

NEUTRONS AND THE NEUTRON FLUX IS NOT HIGHLY DIRECTIONALLY          Sixpak

ORIENTED, THE NEUTRON MULTIPLICATION, SPECTRUM AND ORIENTATION     Sixpak

CAN BE FAIRLY ACCURATELY CALCULATED WITHOUT CONSIDERING            Sixpak

CORRELATION EFFECTS.                                               Sixpak

Sixpak

THE UNCORRELATED DATA PRODUCED BY THIS CODE REPLACES THE           Sixpak

CORRELATED DATA,                                                   Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

BY THE UNCORRELATED DATA,                                          Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F0(E,COS)                       Sixpak

Sixpak

BY INTEGRATING G0(E,EP)*F(E,EP,COS) OVER SECONDARY ENERGY (EP)     Sixpak

TO DEFINE AN AVERAGE ANGULAR DISTRIBUTION, F0(E,COS).              Sixpak

Sixpak

WHAT IS LOST IN THIS PROCESS IS THE CORRELATION BETWEEN EP AND COS Sixpak

SO THAT IN A TRANSPORT CALCULATION ALL MOMENTS OF THE FLUX WILL    Sixpak

HAVE THE SAME SPECTRUM, G0(E,EP) AND EACH WILL BE EFFECTED BY THE  Sixpak

AVERAGE ANGULAR DISTRIBUTION.                                      Sixpak

Sixpak

FOR APPLICATIONS TO HIGH ENERGY FUSION APPLICATIONS CORRELATED     Sixpak

DATA SHOULD BE USED. HOWEVER, FOR LOWER ENERGY APPLICATIONS,       Sixpak

SUCH AS FISSION REACTORS, IT SHOULD BE ADEQUATE TO USE THE         Sixpak

UNCORRELATED DATA - IN THIS CASE THE MOST IMPORTANT EFFECT         Sixpak

WILL BE THE OVERALL NEUTRON MULTIPLICATION AND SPECTRUM.           Sixpak

Sixpak

AN IMPORTANT CONSIDERATION IN DESIGNING THIS PROGRAM IS THAT       Sixpak

MANY COMPUTER CODES - DATA PROCESSING AND TRANSPORT CODES -        Sixpak

CANNOT USE THE CORRELATED (MF=6) DATA - NOR ARE THEY INTENDED      Sixpak

FOR HIGH ENERGY USE. FOR THESE CODES THE UNCORRELATED DATA         Sixpak

PRODUCED BY THIS CODE SHOULD BE ADEQUATE TO MEET THEIR NEEDS.      Sixpak

Sixpak

WARNING - IT CANNOT BE STRESSED ENOUGH THAT THE OUTPUT OF THIS     Sixpak

CODE SHOULD ONLY BE USED FOR LOW ENERGY APPLICATIONS - FAILURE     Sixpak

TO HEED THIS WARNING CAN LEAD TO COMPLETELY UNRELIABLE RESULTS.    Sixpak

Sixpak

ENDF/B FORMAT                                                      Sixpak

================================================================== Sixpak

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Sixpak

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Sixpak

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Sixpak

Sixpak

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Sixpak

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Sixpak

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Sixpak

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Sixpak

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451  Sixpak

AND ALL SECTIONS OF MF=6 MUST BE CORRECT. THE PROGRAM SKIPS ALL    Sixpak

OTHER SECTIONS OF DATA AND AS SUCH IS INSENSITIVE TO THE FORMAT    Sixpak

OF ALL OTHER SECTIONS.                                             Sixpak

Sixpak

CONTENTS OF OUTPUT                                                 Sixpak

================================================================== Sixpak

5 ENDF/B FORMATTED OUTPUT FILES ARE PRODUCED FOR NEUTRON INCIDENT  Sixpak

DATA,                                                              Sixpak

Sixpak

1) ENDFB.MF4 - ANGULAR DISTRIBUTIONS AND LEGENDRE COEFFICIENTS     Sixpak

FOR NEUTRONS                                        Sixpak

2) ENDFB.MF5 - TABULATED NEUTRON ENERGY SPECTRA                    Sixpak

3) ENDFB.M12 - PHOTON EMISSION MULTIPLICITY                        Sixpak

4) ENDFB.M14 - PHOTON EMISSION ANGULAR DISTRIBUTIONS (ALWAYS       Sixpak

ISOTROPIC)                                          Sixpak

5) ENDFB.M15 - TABULATED PHOTON EMISSION SPECTRA                   Sixpak

Sixpak

EMITTED PARTICLE YIELD                                             Sixpak

================================================================== Sixpak

NEUTRONS                                                           Sixpak

========                                                           Sixpak

IN MF=6 THE YIELD FOR EACH REACTION IS THE ACTUAL MULTIPLICITY OF  Sixpak

THE REACTION, E.G., (N,2N) = 2. IN USING MF=4 AND 5 DATA THE       Sixpak

ENDF/B CONVENTION IS THAT THE MULTIPLICITY IS IMPLIED BY THE       Sixpak

MT NUMBER, E.G., MT=16 = (N,2N) = 2.                               Sixpak

Sixpak

THE ONLY EXCEPT IN ENDF/B-VI IS MT=201 = TOTAL NEUTRON PRODUCTION  Sixpak

WHERE AN ACTUAL ENERGY DEPENDENT YIELD IS INCLUDED IN MF=6.        Sixpak

HOWEVER, IN THIS CASE THE MF=3 CROSS SECTION INCLUDES THE          Sixpak

MULTIPLICITY (S. PEARLSTEIN, PRIVATE COMMUNICATION, JAN. 1992),    Sixpak

SIG(MT=201) = 2*SIG(N,2N)+3*SIG(N,3N).....ETC.                     Sixpak

Sixpak

SO THAT FOR ALL ENDF/B-VI DATA AS OF JANUARY 1992 THE MF=4 AND 5   Sixpak

DATA OUTPUT BY THIS CODE CAN BE USED IN CONJUNCTION WITH THE MF=3  Sixpak

CROSS SECTIONS - WITHOUT ANY REFERENCE TO THE MF=6 YIELD.          Sixpak

Sixpak

PHOTONS                                                            Sixpak

=======                                                            Sixpak

UNLIKE THE NEUTRONS WHERE WITH ONLY ONE EXCEPTION (MT=201) THE     Sixpak

MF=6 YIELD IS ENERGY INDEPENDENT, IN THE CASE OF PHOTON EMISSION   Sixpak

ALMOST ALL OF THE PHOTONS HAVE AN ENERGY DEPENDENT YIELD.          Sixpak

Sixpak

THIS PROGRAM WILL OUTPUT THE PHOTON MULTIPLICITY IN MF=12 AND      Sixpak

INDICATE THAT THERE IS A NORMALIZED DISTRIBUTION IN MF=15          Sixpak

(LF=1 IN MF=12).                                                   Sixpak

Sixpak

THIS PROGRAM WILL OUTPUT THE NORMALIZED PHOTON SPECTRA IN MF=15.   Sixpak

CONTINUOUS ENERGY SPECTRA AND DISCRETE PHOTONS WILL ALL BE OUTPUT  Sixpak

AS NORMALIZED SPECTRA.                                             Sixpak

Sixpak

THIS PROGRAM WILL ALSO OUTPUT MF=14 PHOTON ANGULAR DISTRIBUTION    Sixpak

DATA, ALWAYS USING THE ISOTROPIC FLAG TO MINIMIZE OUTPUT.          Sixpak

Sixpak

WARNING OF ENERGY DEPENDENT YIELD                                  Sixpak

=================================                                  Sixpak

THIS PROGRAM WILL PRINT A WARNING MESSAGE IF A SECTION OF DATA     Sixpak

BEING OUTPUT IN THE ENDF/B FORMAT HAS AN ENERGY DEPENDENT MF=6     Sixpak

YIELD AND THE EMITTED PARTICLE IS A NEUTRON - SINCE THE ENDF/B     Sixpak

CONVENTION IS THAT FOR EACH MT NUMBER THE MULTIPLICITY IS IMPLIED  Sixpak

WE DO NOT EXPECT AN ENERGY DEPENDENT MULTIPLICITY FOR NEUTRON      Sixpak

EMISSION.                                                          Sixpak

Sixpak

USING THE OUTPUT                                                   Sixpak

================================================================== Sixpak

NOTE, THAT IN USING THIS DATA, STARTING FROM THE RELATIONSHIP,     Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F0(E,COS)                       Sixpak

Sixpak

USING THE ENDF/B CONVENTION THAT THE MULTIPLICITY IS EITHER        Sixpak

IMPLIED BY THE MT NUMBER (E.G., MT=16 = N,2N - MULTIPLICITY = 2)   Sixpak

OR INCLUDED IN THE CROSS SECTION (E.G., MT=201 = TOTAL NEUTRON     Sixpak

PRODUCTION) ALL THE INFORMATION REQUIRED FOR A CALCULATION IS      Sixpak

AVAILABLE IN,                                                      Sixpak

Sixpak

MF=3       - SIG(E)                                                Sixpak

MF=4       - F0(E,COS) - FOR OUTGOING NEUTRONS                     Sixpak

MF=5       - G0(E,EP)  - FOR OUTGOING NEUTRONS                     Sixpak

MF=12      - Y(E)      - FOR OUTGOING PHOTONS                      Sixpak

MF=14      - F0(E,COS) - FOR OUTGOING PHOTONS (ALWAYS ISOTROPIC)   Sixpak

MF=15      - G0(E,EP)  - FOR OUTGOING PHOTONS                      Sixpak

Sixpak

DOCUMENTATION                                                      Sixpak

================================================================== Sixpak

ONLY SECTIONS OF MF=4, 5, 12, 14, 15 ARE OUTPUT ON A ENDF/B FILE.  Sixpak

THE ONLY DOCUMENTATION IS THE ENDF/B TAPE LABEL (FIRST RECORD OF   Sixpak

EACH FILE) WHICH IDENTIFIES THE DATA AS SIXPAK OUTPUT.             Sixpak

Sixpak

REACTION INDEX                                                     Sixpak

================================================================== Sixpak

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Sixpak

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Sixpak

Sixpak

SECTION SIZE                                                       Sixpak

================================================================== Sixpak

ALL OF THE DATA IN ENDF/B-VI, MF=6 ARE QUITE SMALL TABLES. AS SUCH Sixpak

THIS PROGRAM ONLY ALLOWS TABLES OF UP TO 12000 POINTS (12,000 X,   Sixpak

Y VALUES). THIS SIZE IS MORE THAN ADEQUATE TO HANDLE ALL OF THE    Sixpak

CURRENT ENDF/B-VI DATA, AND IT CAN BE EASILY INCREASED TO HANDLE   Sixpak

ANY NEWER DATA AS IT BECOMES AVAILABLE.                            Sixpak

Sixpak

PLEASE CONTACT THE AUTHOR IF YOU HAVE AN EVALUATION WHICH EXCEEDS  Sixpak

THIS LIMIT.                                                        Sixpak

Sixpak

SELECTION OF DATA                                                  Sixpak

================================================================== Sixpak

THE PROGRAM SELECTS DATA TO BE PROCESSED BASED ON MAT/MT RANGES    Sixpak

(MF=6 ASSUMED). THIS PROGRAM ALLOWS UP TO 100 MAT/MT RANGES TO BE  Sixpak

SPECIFIED BY INPUT PARAMETERS. THE PROGRAM WILL ASSUME THAT THE    Sixpak

ENDF/B TAPE IS IN MAT ORDER. THE PROGRAM WILL TERMINATE EXECUTION  Sixpak

WHEN A MAT IS FOUND THAT IS ABOVE ALL REQUESTED MAT RANGES.        Sixpak

Sixpak

PROGRAM OPERATION                                                  Sixpak

================================================================== Sixpak

EACH SECTION (MT) OF MF=6 DATA IS SUBDIVIDED INTO SUBSECTIONS -    Sixpak

ONE SUBSECTION FOR EACH EMITTED PARTICLE.                          Sixpak

Sixpak

EACH SUBSECTION OF DATA IS CONSIDERED SEPARATELY. EACH SUBSECTION  Sixpak

OF ENDF/B MF=6 DATA TO PROCESS IS IN THE FORM,                     Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

SIG(E)      = MF=3 CROSS SECTIONS                                  Sixpak

Y(E)        = YIELD (MULTIPLICITY)                                 Sixpak

G0(E,EP)    = ENERGY SPECTRUM                                      Sixpak

F(E,EP,COS) = ANGULAR DISTRIBUTION                                 Sixpak

Sixpak

G0(E,EP) = 1 WHEN INTEGRATED OVER EP (SECONDARY ENERGY)            Sixpak

G0(E,EP)*F(E,EP,COS) = 1 WHEN INTEGRATED OVER EP AND COS           Sixpak

Sixpak

THIS PROGRAM WILL DEFINE THE ZEROTH ORDER MOMENTS OF THE           Sixpak

ENERGY AND ANGULAR DISTRIBUTIONS,                                  Sixpak

Sixpak

G0(E,EP)  = G0(E,EP)*F(E,EP,COS) INTEGRATED OVER COS               Sixpak

F0(E,COS) = G0(E,EP)*F(E,EP,COS) INTEGRATED OVER EP                Sixpak

Sixpak

FOR NEUTRON INDUCED REACTIONS THE ENDF/B FORMATTED OUTPUT WILL BE  Sixpak

Sixpak

F0(E,COS)- IN ENDFB.MF4 FOR NEUTRONS OUT OF A REACTION             Sixpak

G0(E,EP) - IN ENDFB.MF5 FOR NEUTRONS OUT OF A REACTION             Sixpak

- IN ENDFB.M15 FOR PHOTONS OUT OF A REACTION              Sixpak

Sixpak

FOR NEUTRONS INCIDENT AND NEUTRONS EMITTED THIS DATA WILL BE       Sixpak

OUTPUT IN MF=4 AND 5 FORMATS.                                      Sixpak

Sixpak

FOR NEUTRONS INCIDENT AND PHOTONS EMITTED THIS DATA WILL BE        Sixpak

OUTPUT IN MF=15 FORMAT - THE SPECTRA ARE OUTPUT AND THE            Sixpak

ANGULAR DISTRIBUTION IS IGNORED.                                   Sixpak

Sixpak

ALL PHOTON EMISSION IN THE ENDF/B-VI LIBRARY AS OF JANUARY 1992    Sixpak

IS ISOTROPIC AND AS SUCH NO DISTRIBUTION OF PHOTON ANGULAR         Sixpak

DISTRIBUTIONS NEED BE OUTPUT - IT IS ALWAYS ISOTROPIC.             Sixpak

Sixpak

FOR ALL OTHER COMBINATIONS INCIDENT AND EMITTED PARTICLES          Sixpak

THERE WILL BE NO ENDF/B FORMATTED OUTPUT.                          Sixpak

Sixpak

VARIATIONS FROM ENDF/B MANUAL                                      Sixpak

================================================================== Sixpak

LAW=1, LANG=2 = KALBACH-MANN                                       Sixpak

============================                                       Sixpak

FOR THE DISTRIBUTIONS,                                             Sixpak

Sixpak

F(MU,E,EP) = G0(E,EP)*A*(COSH(MU*A)+R(E,EP)*SINH(MU*A))            Sixpak

Sixpak

G0(E,EP) = 1 - WHEN INTEGRATED OVER EP.                            Sixpak

Sixpak

A*(COSH(MU*A)+R(E,EP)*SINH(MU*A)) = 2 - WHEN INTEGRATD OVER MU     Sixpak

Sixpak

THIS MEANS AS DEFINED IN THE ENDF/B MANUAL THE DISTRIBUTIONS       Sixpak

ARE NORMALIZED TO 2, INSTEAD OF 1. IN ORDER TO OBTAIN CORRECTLY    Sixpak

NORMALIZED DISTRIBUTIONS THE DISTRIBUTION SHOULD BE DEFINED        Sixpak

TO INCLUDE A FACTOR OF 1/2 MULTIPLYING THE ANGULAR PART OF         Sixpak

THE DISTRIBUTION.                                                  Sixpak

Sixpak

F(MU,E,EP) = G0(E,EP)*0.5*A*(COSH(MU*A)+R(E,EP)*SINH(MU*A))        Sixpak

Sixpak

THIS IS THE FORM USED IN THIS CODE                                 Sixpak

Sixpak

LAW=1, ND NOT 0 = DISCRETE SECONDARY ENERGY DISTRIBUTION           Sixpak

========================================================           Sixpak

THE ENDF/B MANUAL SAYS THESE ARE FLAGGED WITH NEGATIVE ENERGIES.   Sixpak

IN ENDF/B-VI ALL OF THESE HAVE POSITIVE ENERGY. THIS CODE DOES     Sixpak

NOT CONSIDER THE ENDF/B-VI DATA TO BE IN ERROR.                    Sixpak

Sixpak

WITH THE CONVENTION ACTUALLY USED IN ENDF/B-VI ALL SECONDARY       Sixpak

ENERGIES SHOULD BE NON-NEGATIVE AND IN ASCENDING ENERGY ORDER      Sixpak

FOR EACH INCIDENT ENERGY.                                          Sixpak

Sixpak

FROM THE ENDF/B MANUAL IT IS NOT OBVIOUS WHAT G0(E,EP) SHOULD BE   Sixpak

FOR DISCRETE PHOTONS - PHYSICALLY THIS IS A DELTA FUNCTION. IN     Sixpak

ENDF/B-VI IT IS ENTERED AS 1.0 = INTERPRETING IT AS INTEGRATED     Sixpak

OVER SECONDARY ENERGY - IN WHICH CASE THE DELTA FUNCTION = 1.0.    Sixpak

Sixpak

LIMITATIONS                                                        Sixpak

================================================================== Sixpak

CHECKING DATA                                                      Sixpak

================================================================== Sixpak

THIS PROGRAM CHECKS ALL ENDF/B-VI MF=6 DATA. THE FOLLOWING CHECKS  Sixpak

ARE PERFORMED.                                                     Sixpak

Sixpak

PARAMETERS                                                         Sixpak

==========                                                         Sixpak

ALL PARAMETERS ARE CHECKED FOR CONSISTENCY. IF PARAMETERS ARE      Sixpak

NOT CONSISTENT THE PROGRAM MAY NOT BE ABLE TO PERFORM THE          Sixpak

FOLLOWING TESTS AND WILL MERELY SKIP A SECTION OF DATA.            Sixpak

Sixpak

INTERPOLATION LAWS                                                 Sixpak

==================                                                 Sixpak

ALL INTEGRATIONS ARE PERFORMED USING THE INTERPOLATION LAW GIVEN   Sixpak

FOR SECONDARY ENERGY AND/OR COSINE. INTEGRATIONS ARE NOT           Sixpak

PERFORMED OVER INCIDENT - ONLY INTEGRATION OVER SECONDARY ENERGY   Sixpak

AND/OR COSINE ARE PERFORMED AT EACH INCIDENT ENERGY. THEREFORE     Sixpak

THE INTERPOLATION LAW FOR INCIDENT ENERGY IS NOT USED BY THIS      Sixpak

CODE.                                                              Sixpak

Sixpak

ALL INTERPOLATION LAWS ARE CHECKED. ALL DATA ASSOCIATED WITH       Sixpak

INTERPOLATION LAWS ARE CHECKED, E.G., NO NON-NEGATIVE VALUES       Sixpak

REQUIRING LOG INTERPOLATION. IN ORDER TO PERFORM REQUIRED          Sixpak

INTEGRALS OVER COS AND EP IT IS IMPERATIVE THAT THE INTERPOLATION  Sixpak

LAWS BE COMPATIBLE WITH THE DATA.                                  Sixpak

Sixpak

ENDF/B-VI ALLOWS NEW INTERPOLATION LAWS FOR CORRESPONDING POINT    Sixpak

AND UNIT BASE TRANSFORMATION INTERPOLATION. NONE OF THESE NEW      Sixpak

INTERPOLATION LAWS ARE USED IN THE ENDF/B-VI LIBRARY AS OF         Sixpak

JANUARY 1992 TO INTERPOLATE IN SECONDARY ENERGY OR COSINE.         Sixpak

THEREFORE THIS PROGRAM CAN PERFORM ALL OF THE REQUIRED INTEGRALS   Sixpak

OVER SECONDARY ENERGY AND/OR COSINE USING ONLY THE OLDER           Sixpak

INTERPOLATION CODES. THIS PROGRAM ONLY PERFORMS INTEGRALS FOR      Sixpak

EACH INCIDENT ENERGY, SO THAT INTERPOLATION IN INCIDENT ENERGY     Sixpak

IS NOT PERFORMED BY THIS PROGRAM.                                  Sixpak

Sixpak

NEW INTERPOLATION SCHEMES ARE USED FOR INCIDENT ENERGY - FOR       Sixpak

EXAMPLE, CORRESPONDING POINT INTERPOLATION IS SPECIFIED TO ALLOW   Sixpak

INTERPOLATION IN G0(E,EP) TO SIMULATE CASES WHERE THE INPUT ENERGY Sixpak

LIMIT IS DEFINED BY E-EP = A DIAGONAL CURVE ACROSS (E,EP) SPACE.   Sixpak

THIS INTERPOLATION CODE CANNOT BE SPECIFIED IN THE MF=5 OUTPUT     Sixpak

OF THIS CODE - MF=5 ONLY ALLOWS THE OLDER INTERPOLATION LAWS       Sixpak

INT=1 THROUGH 5. THEREFORE THIS PROGRAM WILL USE THE CLOSEST       Sixpak

CORRESPONDING INTERPOLATION CODE FOR OUTPUT TO MF=5. FOR USE       Sixpak

WHERE THE OUTPUT OF THIS CODE = LOW ENERGY APPLICATIONS - THIS     Sixpak

SHOULD HAVE LITTLE EFFECT ON RESULTS.                              Sixpak

Sixpak

FOR CONSISTENCY WITH EARLIER VERSIONS OF ENDF/B IN CREATING THE    Sixpak

ENDF/B OUTPUT, IF ANY INPUT INTERPOLATION LAW IS NOT IN THE        Sixpak

RANGE 1-5, IT WILL FIRST BE TESTED TO SEE IF MOD(10) IT IS         Sixpak

IN THIS RANGE, FINALLY IF EVEN THIS DOESN'T WORK IT IS SET         Sixpak

EQUAL TO 2 (LINEARLY INTERPOLATION). THIS METHOD WILL EFFECTIVELY  Sixpak

REPLACE CORRESPONDING POINT AND UNIT BASE TRANSFORMATION BY THE    Sixpak

CLOSEST RELATED INTERPOLATION LAW 1 THROUGH 5 - AGAIN NOTE, AS     Sixpak

OF JANUARY 1992 NONE OF THESE NEW LAWS ARE USED IN ENDF/B-VI. IF   Sixpak

THIS MUST BE DONE FOR INTERPOLATION IN SECONDARY ENERGY OR COSINE  Sixpak

AN ERROR MESSAGE WILL BE PRINTED - SINCE THIS WOULD EFFECT THE     Sixpak

ACCURACY OF THE INTEGRALS PERFORMED BY THIS PROGRAM. IF THIS MUST  Sixpak

BE DONE FOR INCIDENT ENERGY NO MESSAGE IS PRINTED - SINCE THIS     Sixpak

WILL NOT EFFECT THE ACCURACY OF THE INTEGRALS PERFORMED BY THIS    Sixpak

PROGRAM.                                                           Sixpak

Sixpak

SPECTRA AND ANGULAR DISTRIBUTIONS                                  Sixpak

=================================                                  Sixpak

ALL SPECTRA AND ANGULAR DISTRIBUTIONS ARE CHECKED TO INSURE        Sixpak

THEY ARE NORMALIZED AND DO NOT INCLUDE ANY NEGATIVE VALUES.        Sixpak

Sixpak

LEGENDRE COEFFICIENTS                                              Sixpak

=====================                                              Sixpak

THE NORMALIZATION, F0, CANNOT BE NEGATIVE.                         Sixpak

Sixpak

LEGENDRE COEFFICIENTS IN NORMAL FORM ARE CHECKED TO INSURE         Sixpak

THEY ARE IN THE RANGE -1 TO +1 = THE LEGENDRE EXPANSION OF A       Sixpak

DELTA FUNCTION AT COS=+1 OR -1 - COEFFICIENTS SHOULD NOT           Sixpak

EXCEED WHAT YOU GET FROM A DELTA FUNCTION.                         Sixpak

Sixpak

ANGULAR DISTRIBUTIONS ARE CHECKED AT COS = -1, 0 AND +1.           Sixpak

Sixpak

CREATING ENDF/B OUTPUT                                             Sixpak

================================================================== Sixpak

THIS PROGRAM CAN CREATE EQUIVALENT MF =4, 5, 12, 14, 15 DATA FOR   Sixpak

ALL OF THE DATA INCLUDED IN ENDF/B-VI AS OF JANUARY 1992, EXCEPT   Sixpak

FOR 1 SECTION OF LAW=6 DATA (SEE DETAILS BELOW).                   Sixpak

Sixpak

THIS PROGRAM HAS NOT BEEN TESTED ON OTHER DATA LIBRARIES, E.G.,    Sixpak

JEF, JENDL, ETC.                                                   Sixpak

Sixpak

THE PROGRAM HAS THE FOLLOWING LIMITATION AS FAR AS CREATING        Sixpak

ENDF/B FORMATTED OUTPUT.                                           Sixpak

Sixpak

ISOTROPIC PHOTON EMISSION                                          Sixpak

=========================                                          Sixpak

FOR PHOTON EMISSION THE DISTRIBUTIONS ARE ASSUMED TO BE ISOTROPIC  Sixpak

AND ONLY THE MULTIPLICITY IS OUTPUT IN MF=12, ISOTROPIC ANGULAR    Sixpak

DISTRIBUTIONS IN MF=14 AND THE SPECTRA IN MF=15. ALL ENDF/B-VI     Sixpak

MF=6 DATA AS OF JANUARY 1992 INCLUDE ONLY ISOTROPIC PHOTON         Sixpak

EMISSION - SO THAT THIS IS NOT A LIMITATION ON TRANSLATING         Sixpak

ENDF/B-VI DATA.                                                    Sixpak

Sixpak

EITHER TABULATED OR LEGENDRE COEFFICIENTS                          Sixpak

=========================================                          Sixpak

FOR LAW=2 THE REPRESENTATION, EITHER TABULATED OR LEGENDRE         Sixpak

COEFFICIENTS, CAN BE SPECIFIED FOR EACH INCIDENT ENERGY.           Sixpak

Sixpak

IN ORDER TO OBTAIN CORRECT ENDF/B OUTPUT THE REPRESENTATION        Sixpak

MUST BE THE SAME FOR ALL INCIDENT ENERGIES = MF=4 DATA CAN ONLY    Sixpak

BE TABULATED OR LEGENDRE OVER THE ENTIRE ENERGY RANGE.             Sixpak

Sixpak

YIELD AND OUTPUT NORMALIZATION                                     Sixpak

==============================                                     Sixpak

THE YIELD INCLUDED WITH EACH SECTION OF DATA IS NOT USED FOR       Sixpak

OUTPUT FOR NEUTRONS, BUT IS INCLUDED IN THE OUTPUT FOR PHOTONS.    Sixpak

IN ALL CASES THE ANGULAR DISTRIBUTIONS AND SPECTRA OUTPUT ARE      Sixpak

NORMALIZED TO UNITY.                                               Sixpak

Sixpak

LAW=0                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=1                                                              Sixpak

=====                                                              Sixpak

FOR EACH INCIDENT ENERGY DISCRETE AND CONTINUOUS EMISSION SPECTRA  Sixpak

CANNOT BE MIXED TOGETHER - THEY MUST BE ALL EITHER DISCRETE OR     Sixpak

CONTINUOUS. IF DISCRETE EMISSION IS GIVEN ONLY 1 SECONDARY         Sixpak

ENERGY (NEP=1) MAY BE GIVEN = A NORMALIZED DISTRIBUTION FOR A      Sixpak

SINGLE DISCRETE EMISSION ENERGY. ALL OF THE ENDF/B-VI DATA AS      Sixpak

OF JANUARY 1992 CONFORM TO THESE LIMITATIONS.                      Sixpak

Sixpak

SINCE THE FLAG NA, TO INDICATE ISOTROPIC DISTRIBUTIONS, IS ONLY    Sixpak

GIVEN FOR EACH SECONDARY ENERGY (EP) THE PROGRAM CANNOT DECIDE     Sixpak

IN ADVANCE WHETHER OR NOT THE DISTRIBUTION WILL BE ISOTROPIC       Sixpak

AT ALL INCIDENT ENERGIES. THEREFORE ISOTROPIC DISTRIBUTIONS        Sixpak

WILL BE OUTPUT EITHER: LANG = 1 - AS 1 LEGENDRE COEFFICIENT = 0.0  Sixpak

OR LANG = NOT 1 - AS A 2 POINT ANGULAR DISTRIBUTION AT COS = -1.0  Sixpak

AND +1.0 WITH BOTH VALUES EQUAL TO 0.5 (A NORMALIZED ISOTROPIC     Sixpak

DISTRIBUTION).                                                     Sixpak

Sixpak

DISCRETE PHOTONS ARE OUTPUT IN MF=15 AS 3 POINT DISTRIBUTIONS      Sixpak

WITH SECONDARY ENERGY POINTS AT EP-DEP, EP, EP+DEP, WHERE          Sixpak

DEP=0.001*EP. THE VALUES AT EP-DEP AND EP+DEP ARE 0.0, AND         Sixpak

AT EP THE VALUE IS 1000.0/EP TO NORMALIZE THE DISTRIBUTION.        Sixpak

Sixpak

LAW=2                                                              Sixpak

=====                                                              Sixpak

NO LIMITATION ON REPRESENTATIONS.                                  Sixpak

Sixpak

LAW=3                                                              Sixpak

=====                                                              Sixpak

NO LIMITATION ON REPRESENTATIONS.                                  Sixpak

Sixpak

LAW=4                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=5                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=6                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - ENDF/B-VI ONLY INCLUDES 1 SECTION OF THIS TYPE OF DATA Sixpak

FOR (N,D) 2N,P.                                        Sixpak

Sixpak

LAW=7                                                              Sixpak

=====                                                              Sixpak

FOR EACH INCIDENT ENERGY THE REPRESENTATION MUST BE EITHER,        Sixpak

Sixpak

1) SQUARE = FOR EACH INCIDENT COSINE EXACTLY THE SAME SECONDARY    Sixpak

ENERGIES.                                              Sixpak

Sixpak

2) LINEAR = FOR EACH INCIDENT COSINE THE INTERPOLATION LAW         Sixpak

BETWEEN SECONDARY ENERGIES MUST BE LINEAR.             Sixpak

Sixpak

THESE 2 PRESENTATIONS ARE THE ONLY ONES PRESENTED IN ENDF/B-VI     Sixpak

AS OF JANUARY 1992 - SO THIS PROGRAM CAN TRANSLATED ALL LAW=7      Sixpak

DATA FOR ENDF/B-VI.                                                Sixpak

Sixpak

LABORATORY VS. CENTER-OF-MASS SYSTEM                               Sixpak

================================================================== Sixpak

IN MANY CASES PEOPLE ASSUME THAT FOR HEAVY (HIGH ATOMIC WEIGHT)    Sixpak

MATERIALS THE CENTER-OF-MASS AND LAB SYSTEMS ARE ALMOST IDENTICAL, Sixpak

SINCE IN THIS CASE THE CENTER-OF-MASS ENERGY WILL BE MUCH SMALLER  Sixpak

THAN THE INCIDENT ENERGY. FOR A PROCESS SUCH AS ELASTIC SCATTERING Sixpak

WHERE FOR HEAVY MATERIALS THE SECONDARY ENERGY, EP, WILL ALWAYS    Sixpak

BE A LARGE FRACTION OF THE INCIDENT ENERGY, THIS ASSUMPTION IS     Sixpak

VALID. HOWEVER, FOR THE TYPICAL REACTIONS INCLUDED IN MF=6 THIS    Sixpak

IS NOT ALWAYS TRUE - IN MANY OF THESE CASES THE SECONDARY ENERGY   Sixpak

CAN EXTEND ALL THE WAY DOWN TO ZERO, AND IN PARTICULAR IT CAN      Sixpak

BE SMALL COMPARED TO THE CENTER-OF-MASS ENERGY - WHICH MAKES THE   Sixpak

TRANSFORMATION FROM CENTER-OF-MASS TO LAB IMPORTANT. THEREFORE     Sixpak

GENERALLY TO TREAT MF=6 DATA WE MUST CONSIDER THIS TRANSFORMATION. Sixpak

Sixpak

THE FOLLOWING DISCUSSING ONLY APPLIES TO SPECTRA THAT MAY BE       Sixpak

OUTPUT IN MF=5 = ONLY DATA FOR NEUTRONS INCIDENT AND EMITTED -     Sixpak

IN PARTICULAR THE FOLLOWING DEFINITIONS ARE NOT GENERAL - THEY     Sixpak

ARE ONLY VALID FOR INCIDENT AND EMITTED NEUTRONS.                  Sixpak

Sixpak

DOUBLE DIFFERENTIAL DATA IN MF=6 MAY BE GIVEN IN EITHER THE LAB    Sixpak

OR C.M. SYSTEM. SIMILARLY ANGULAR DISTRIBUTIONS IN MF=4 MAY BE     Sixpak

GIVEN IN EITHER THE LAB OR C.M. SYSTEM. IN CONTRAST ENERGY         Sixpak

SPECTRA IN MF=5 CAN ONLY BE GIVEN IN THE LABORATORY SYSTEM.        Sixpak

Sixpak

THE ANGULAR DISTRIBUTIONS OUTPUT BY THIS CODE IN MF=4 ARE IN THE   Sixpak

SAME SYSTEM IN WHICH THEY ARE GIVEN IN MF=6 - EITHER LAB OR        Sixpak

CENTER-OF-MASS SYSTEM.                                             Sixpak

Sixpak

THE ENERGY SPECTRA OUTPUT BY THIS CODE IN MF=5 MUST BE IN THE LAB  Sixpak

SYSTEM - THIS IS THE ONLY ALLOWED FORM FOR MF=5 DATA.              Sixpak

Sixpak

FOR MF=6 SPECTRA GIVEN IN THE LAB SYSTEM THIS MERELY REQUIRES      Sixpak

COPYING THE GIVEN SPECTRA TO MF=5 OUTPUT.                          Sixpak

Sixpak

FOR MF=6 SPECTRA GIVEN IN THE CENTER-OF-MASS SYSTEM ONLY FIRST     Sixpak

ORDER CORRECTIONS IN THE SPECTRA AND USED AND THEY ARE THEN        Sixpak

OUTPUT IN MF=5 AS IN THE LAB SYSTEM - THE FIRST ORDER CORRECTIONS  Sixpak

ARE DESCRIBED BELOW.                                               Sixpak

Sixpak

DEFINING,                                                          Sixpak

MM      = CENTER OF MASS MOTION                                    Sixpak

CM      = OUTGOING (EMITTED) PARTICLE IN CENTER OF MASS            Sixpak

LAB     = OUTGOING (EMITTED) PARTICLE IN LAB                       Sixpak

THETA   = CM SCATTERING ANGLE RELATIVE TO INCIDENT DIRECTION       Sixpak

COS(CM) = COSINE OF THE CM SCATTERING ANGLE                        Sixpak

Sixpak

FOR NEUTRONS INCIDENT WITH AN ENERGY, E, AND THEREFORE A SPEED,    Sixpak

Sixpak

VN(E) = 2*SQRT(E)/MASS(IN)                                         Sixpak

Sixpak

THE CENTER-OF-MASS SPEED IS GIVEN BY,                              Sixpak

Sixpak

V(MM) = VN(E)/(1 + A)                                              Sixpak

Sixpak

AND THE CENTER OF MASS ENERGY BY,                                  Sixpak

Sixpak

E(MM) = 1/2*MASS(IN)*V(MM)**2                                      Sixpak

= 1/2*MASS(IN)*VN(E)**2/(1 + A)**2                           Sixpak

= E/(1 + A)**2                                               Sixpak

Sixpak

FOR DISTRIBUTIONS GIVEN IN MF=6 IN THE CM, THE SPEED, V(CM),       Sixpak

SHOULD BE VECTORIALLY ADDED TO THAT OF OUTGOING PARTICLES TO       Sixpak

DEFINE THE OUTGOING PARTICLES LAB VELOCITY, AND IN TURN IT'S       Sixpak

ENERGY,                                                            Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

V(LAB)*SIN(LAB) =         V(CM)*SIN(CM)                            Sixpak

Sixpak

V(LAB)**2 = V(MM)**2 + V(CM)**2 + 2*COS(CM)*V(MM)*V(CM)            Sixpak

Sixpak

EP(LAB)   = 0.5*MASS(OUT)*V(LAB)**2                                Sixpak

Sixpak

= E(MM) + EP(CM) + 2*COS(CM)*SQRT(E(MM)*EP(CM))          Sixpak

Sixpak

WE CAN ALSO DEFINE THE REVERSE TRANSFORMATION USING,               Sixpak

Sixpak

V(CM)*COS(CM) = V(LAB)*COS(LAB) - V(MM)                            Sixpak

V(CM)*SIN(CM) = V(LAB)*SIN(LAB)                                    Sixpak

Sixpak

V(CM)**2 = V(MM)**2 + V(LAB)**2 - 2*COS(LAB)*V(MM)*V(LAB)          Sixpak

Sixpak

EP(CM)   = 0.5*MASS(OUT)*V(CM)**2                                  Sixpak

Sixpak

= E(MM) + EP(LAB) - 2*COS(LAB)*SQRT(E(MM)*EP(LAB))       Sixpak

Sixpak

WE CAN DEFINE COS(LAB) FROM THE RELATIONSHIP,                      Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

Sixpak

COS(LAB)        =[V(MM) + V(CM)*COS(CM)]/V(LAB)                    Sixpak

Sixpak

[V(MM) + V(CM)*COS(CM)]                           Sixpak

COS(LAB)        =---------------------------------------------     Sixpak

SQRT[V(MM)**2+V(CM)**2+2*COS(CM)*V(MM)*V(CM)]     Sixpak

Sixpak

OR COS(CM) FROM THE RELATIONSHIP,                                  Sixpak

Sixpak

V(CM)*COS(CM)  = V(LAB)*COS(LAB) - V(MM)                           Sixpak

Sixpak

COS(CM)         =[V(LAB)*COS(LAB) - V(MM)]/V(CM)                   Sixpak

Sixpak

[V(LAB)*COS(LAB) - V(MM)]                         Sixpak

COS(CM)         =------------------------------------------------  Sixpak

SQRT[V(LAB)**2+V(CM)**2-2*COS(LAB)*V(LAB)*V(MM)]  Sixpak

Sixpak

THE JACOBIAN CAN BE DEFINED FROM,                                  Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

Sixpak

J = D[COS(CM)]/D[COS(LAB)] = V(LAB)/V(CM)                          Sixpak

= SQRT[EP(LAB)/EP(CM)]                  Sixpak

Sixpak

WITH THESE DEFINITIONS OF EP(LAB) AND COS(LAB) IN TERMS OF E(MM),  Sixpak

EP(CM) AND COS(CM) IT IS POSSIBLE TO PERFORM A POINT-BY-POINT      Sixpak

TRANSFORMATION OF DISTRIBUTIONS FROM THE CM TO LAB SYSTEM USING    Sixpak

THESE DEFINITIONS - OR IF WE WISHED WE COULD PERFORM THE REVERSE   Sixpak

TRANSFORMATION USING THE ABOVE RELATIONSHIPS AND THE IDENTITY,     Sixpak

Sixpak

F(E,EP(LAB),COS(LAB))*D(COS(LAB))=F(E,EP(CM),COS(CM))*D(COS(CM))   Sixpak

Sixpak

THIS IS NOT WHAT WILL BE DONE HERE, SINCE WE WILL ONLY BE          Sixpak

INTERESTED IN THE ZEROTH ORDER MOMENTS OF THESE DISTRIBUTIONS,     Sixpak

BUT WE WILL BE INTERESTED IN DEFINING THOSE MOMENTS IN THE         Sixpak

LAB SYSTEM IN TERMS OF MF=6 SPECTRA GIVEN IN THE CM SYSTEM USING,  Sixpak

Sixpak

F(E,EP(LAB),COS(LAB)) = F(E,EP(CM),COS(CM))*J                      Sixpak

Sixpak

THE LIMITS OF EP(LAB) ARE DEFINED BY SETTING COS(CM) = +1 OR -1,   Sixpak

Sixpak

EP(LAB)   = (SQRT(EP(CM)) + SQRT(E(MM)))**2  FOR COS(CM) = +1      Sixpak

= (SQRT(EP(CM)) - SQRT(E(MM)))**2  FOR COS(CM) = -1      Sixpak

Sixpak

IN THIS FORM WE CAN SEE THAT AS LONG AS THE SECONDARY ENERGY IN    Sixpak

THE CENTER-OF-MASS SYSTEM, EP(CM), IS MUCH LARGER THAN THE         Sixpak

ENERGY OF THE CENTER-OF-MASS, E(MM), THE CENTER-OF-MASS AND LAB    Sixpak

ENERGIES WILL BE ALMOST EQUAL - SIMILARLY FOR THE COSINE, IN       Sixpak

THIS CASE COS(LAB) AND COS(CM) WILL BE ALMOST EQUAL - HOWEVER,     Sixpak

FOR THE MF=6 DATA WE CANNOT ASSUME THAT THIS IS TRUE.              Sixpak

Sixpak

TO FIRST ORDER THE ANGULAR DEPENDENCE CAN BE IGNORED,              Sixpak

Sixpak

EP(LAB)   = E(MM) + EP(CM)                                         Sixpak

Sixpak

ALL THIS SAYS IS THAT TO FIRST ORDER THE EFFECT OF TRANSFORMING    Sixpak

FROM THE CM TO LAB SYSTEM IS TO INCREASE THE ENERGY OF THE         Sixpak

EMITTED PARTICLE IN THE CENTER-OF-MASS SYSTEM BY THE ENERGY OF     Sixpak

THE CENTER-OF-MASS TO DEFINE THE LAB ENERGY.                       Sixpak

Sixpak

NOT ONLY THE ENERGY, BUT ALSO THE SPECTRA MUST BE TRANSFORMED.     Sixpak

STARTING FROM THE DOUBLE DIFFERENTIAL DATA IN THE LAB SYSTEM,      Sixpak

F(E,EP,COS(LAB)), WE CAN DEFINE THE LAB SCALAR SPECTRUM AS,        Sixpak

Sixpak

G0(E,EP) = INTEGRAL F(E,EP,COS(LAB))*D(COS(LAB))                   Sixpak

Sixpak

THIS IS THE NORMAL CALCULATION DEFINED ABOVE AND USED FOR DATA     Sixpak

GIVEN IN THE LAB SYSTEM.                                           Sixpak

Sixpak

STARTING FROM DATA IN THE CENTER OF MASS SYSTEM F(E,EP,COS(CM)),   Sixpak

WE CAN USE THE RELATIONSHIP,                                       Sixpak

Sixpak

F(E,EP,COS(LAB))*D(COS(LAB)) = F(E,EP,COS(CM))*J*D(COS(LAB))       Sixpak

Sixpak

J         = SQRT(EP(LAB)/EP(CM)) - THE JACOBIAN                    Sixpak

Sixpak

= E(MM)/EP(CM) + 1 + 2*COS(CM)*SQRT(E(MM)/EP(CM))        Sixpak

Sixpak

AS IN THE CASE OF THE ENERGY, IN THIS FORM WE CAN SEE THAT AS      Sixpak

LONG AS THE SECONDARY ENERGY IN THE CENTER-OF-MASS SYSTEM,         Sixpak

EP(CM), IS LARGE COMPARED TO THE CENTER-OF-MASS ENERGY, E(MM),     Sixpak

THE JACOBIAN IS ESSENTIALLY UNITY AND THE CENTER-OF-MASS AND LAB   Sixpak

SPECTRA WILL BE VERY SIMILAR - AGAIN, GENERALLY WE CANNOT          Sixpak

ASSUME THAT THIS IS TRUE FOR THE MF=6 SPECTRA.                     Sixpak

Sixpak

THEREFORE WE CAN ALSO DEFINE THE LAB SCALAR SPECTRUM IN TERMS OF   Sixpak

THE CM SPECTRUM IN THE FORM,                                       Sixpak

Sixpak

G0(E,EP) = INTEGRAL F(E,EP,COS(CM))*J*D(COS(LAB))                  Sixpak

Sixpak

CONSISTENT WITH THE ABOVE ASSUMPTION THAT THE ANGULAR DEPENDENCE   Sixpak

OF EP(LAB) CAN BE IGNORED THE JACOBIAN WILL NOT BE USED IN         Sixpak

PERFORMING THESE INTEGRALS - IN WHICH CASE THE INTEGRAL REDUCES    Sixpak

TO EXACTLY THE SAME FORM AS IF THE DATA WERE IN THE LAB SYSTEM.    Sixpak

Sixpak

IT SHOULD BE NOTED THAT SINCE IN THIS CASE THE MF=4 ANGULAR        Sixpak

DISTRIBUTIONS ARE GIVEN IN THE CM SYSTEM AND WHEN USED IN ANY      Sixpak

APPLICATION THEY WILL BE TRANSFORMED TO THE LAB SYSTEM - WHEN      Sixpak

THIS IS DONE THE JACOBIAN WILL BE APPLIED.                         Sixpak

Sixpak

IN THIS CODE WHERE WE ARE MOSTLY CONCERNED WITH CONSERVING THE     Sixpak

NUMBER OF EMITTED PARTICLES AND AVERAGE ENERGIES THE NEUTRON       Sixpak

SPECTRA OUTPUT IN MF=5 WILL NOT BE COMPLETELY CONVERTED TO THE     Sixpak

LAB SYSTEM - ONLY FIRST ORDER CORRECTIONS WILL BE INCLUDED BY      Sixpak

INCREASING THE EMITTED PARTICLE ENERGY BY THE CENTER OF MASS       Sixpak

ENERGY, I.E., FOR A CENTER OF MASS SPECTRUM TABULATED AT CENTER    Sixpak

OF MASS ENERGIES EP(CM) THESE WILL ALL BE UNIFORMLY INCREASED      Sixpak

BY E(MM) TO ACCOUNT FOR THE CENTER OF MASS MOTION - THE SPECTRA    Sixpak

WILL NOT BE MODIFIED BY THE JACOBIAN FACTOR SQRT(EP(LAB)/EP(CM))   Sixpak

SINCE THIS WOULD REQUIRE A DETAILED TRANSFORMATION IN ENERGY AND   Sixpak

COS(THETA) SPACE - WHICH IS JUDGED NOT TO BE WORTH PERFORMING      Sixpak

WITHIN THE LIMITS OF WHERE THE OUTPUT FROM THIS CODE IS INTENDED   Sixpak

TO BE USED.                                                        Sixpak

Sixpak

SINCE THE ANGULAR DISTRIBUTION IS ALWAYS OUTPUT IN THE SAME        Sixpak

SYSTEM AS WHICH IT IS GIVEN IN MF=6, NO TRANSFORMATION IS          Sixpak

REQUIRED FOR THE MF=4 OUTPUT.                                      Sixpak

Sixpak

WHEN USED IN LOW ENERGY APPLICATIONS (E.G., FISSION REACTORS) THE  Sixpak

HIGH ENERGY SPECTRA PRESENTED IN MF=6 WILL BE MOSTLY IMPORTANT     Sixpak

SIMPLY IN CONSERVING PARTICLES, (E.G., AS IN (N,2N)) AND ENERGY    Sixpak

AND THE DETAILS OF THE CORRELATION AND GROSS ENERGY SPECTRA WILL   Sixpak

NOTE PLAY THAT IMPORTANT A ROLE. IN THIS CASE THE SPECTRA OUTPUT   Sixpak

BY THIS PROGRAM IN MF=5 SHOULD BE ADEQUATE.                        Sixpak

Sixpak

PLOTTAB FORMATTED OUTPUT                                           Sixpak

================================================================== Sixpak

THIS PROGRAM CONTAINS ROUTINES TO PRODUCE OUTPUT THAT CAN BE USED  Sixpak

AS INPUT TO THE PLOTTAB CODE TO OBTAIN GRAPHIC RESULTS.            Sixpak

Sixpak

THESE ROUTINES ARE DESIGNED ONLY FOR USE BY THE AUTHOR TO CHECK    Sixpak

THIS CODE. USERS ARE ASKED NOT TO ACTIVATE OR TRY TO USE THESE     Sixpak

ROUTINES. UNLESS YOU COMPLETELY UNDERSTAND THIS CODE THE RESULTS   Sixpak

CAN BE UNRELIABLE IF YOU ACTIVATE THESE ROUTINES.                  Sixpak

Sixpak

INPUT FILES                                                        Sixpak

================================================================== Sixpak

UNIT  DESCRIPTION                                                  Sixpak

----  -----------                                                  Sixpak

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                     Sixpak

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Sixpak

Sixpak

OUTPUT FILES                                                       Sixpak

================================================================== Sixpak

UNIT  DESCRIPTION                                                  Sixpak

----  -----------                                                  Sixpak

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Sixpak

11  ENDF/B DATA MF=4 (BCD - 80 CHARACTERS/RECORD)                Sixpak

12  ENDF/B DATA MF=5 (BCD - 80 CHARACTERS/RECORD)                Sixpak

14  ENDF/B DATA MF=15 (BCD - 80 CHARACTERS/RECORD)               Sixpak

17  ENDF/B DATA MF=12 (BCD - 80 CHARACTERS/RECORD)               Sixpak

18  ENDF/B DATA MF=14 (BCD - 80 CHARACTERS/RECORD)               Sixpak

15  PLOTTAB INPUT PARAMETERS (BCD - 80 CHARACTERS/RECORD)        Sixpak

16  PLOTTAB FORMATTED OUTPUT (BCD - 80 CHARACTERS/RECORD)        Sixpak

Sixpak

SCRATCH FILES                                                      Sixpak

================================================================== Sixpak

NONE                                                               Sixpak

Sixpak

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Sixpak

================================================================== Sixpak

UNIT  FILE NAME                                                    Sixpak

----  ----------                                                   Sixpak

2   SIXPAK.INP                                                   Sixpak

3   SIXPAK.LST                                                   Sixpak

10   ENDFB.IN                                                     Sixpak

11   ENDFB.MF4                                                    Sixpak

12   ENDFB.MF5                                                    Sixpak

14   ENDFB.M15                                                    Sixpak

17   ENDFB.M12                                                    Sixpak

18   ENDFB.M14                                                    Sixpak

15   PLOTTAB.INP                                                  Sixpak

16   PLOTTAB.CUR                                                  Sixpak

Sixpak

Sixpak

INPUT PARAMETERS                                                   Sixpak

================================================================== Sixpak

LINE  COLS.  DESCRIPTION                                           Sixpak

----  -----  -----------                                           Sixpak

1   1-60  ENDF/B INPUT DATA FILENAME                            Sixpak

(STANDARD OPTION = ENDFB.IN)                          Sixpak

2-N   1-6   MINIMUM MAT FOR REQUESTED RANGE                       Sixpak

9-11  MINIMUM MT FOR REQUESTED RANGE                        Sixpak

12-17  MAXIMUM MAT FOR REQUESTED RANGE                       Sixpak

20-22  MAXIMUM MT FOR REQUESTED RANGE                        Sixpak

Sixpak

LEAVE THE DEFINITION OF THE FILENAME BLANK - THE PROGRAM WILL      Sixpak

THEN USE THE STANDARD FILENAME (ENDFB.IN).                         Sixpak

Sixpak

UP TO 100 MAT/MT RANGES MAY BE SPECIFIED. THE LIST OF RANGES IS    Sixpak

TERMINATED BY A BLANK LINE. IF THE FIRST INPUT LINE IS COMPLETELY  Sixpak

BLANK ALL DATA WILL BE PROCESSED.                                  Sixpak

Sixpak

EXAMPLE INPUT NO. 1                                                Sixpak

-------------------                                                Sixpak

PROCESS ALL MF=6 DATA ON AN ENDF/B TAPE. USE THE STANDARD INPUT    Sixpak

DATA FILENAME ENDFB.IN IN THIS CASE THE USER CAN EITHER EXPLICITLY Sixpak

SPECIFY THE FILENAME AND MAT/MT RANGE BY THE FOLLOWING  2 INPUT    Sixpak

LINES,                                                             Sixpak

Sixpak

ENDFB.IN                                                           Sixpak

1    1  9999  999                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

OR BY INPUTTING 2 BLANK LINE = PROCESS EVERYTHING.                 Sixpak

Sixpak

EXAMPLE INPUT NO. 2                                                Sixpak

-------------------                                                Sixpak

PROCESS BE-9, MAT=425, MT=16. READ THE DATA FROM ENDFB6\BE9.       Sixpak

IN THIS CASE THE FOLLOWING 3 INPUT LINES ARE REQUIRED,             Sixpak

Sixpak

ENDFBB6\BE9                                                        Sixpak

425   16   425   16                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

EXAMPLE INPUT NO. 3                                                Sixpak

-------------------                                                Sixpak

PROCESS ALL MT=16 (N,2N) DATA. THIS CAN BE DONE BY SPECIFYING THE  Sixpak

MAXIMUM MAT RANGE = 1 TO 9999, AND MT=16 FOR THE MINIMUM AND       Sixpak

MAXIMUM MT RANGE. READ THE DATA FROM ENDFB6\K300. IN THIS CASE     Sixpak

CASE THE FOLLOWING 3 INPUT LINES ARE REQUIRED,                     Sixpak

Sixpak

ENDFB6\K300                                                        Sixpak

1   16  9999   16                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

======================================================================= Sixpak