|  | International Atomic Energy Agency Nuclear Data Services DOCUMENTATION SERIES OF THE IAEA NUCLEAR DATA SECTION __________________________________________________ |  |
| --- | --- | --- |

IAEA-NDS-39

Rev. 13, March 17, 2007

PREPRO 2007

2007 ENDF/B Pre-processing Codes

(ENDF/B-VII Tested)

Owned, Maintained and Distributed

by

The Nuclear Data Section

International Atomic Energy Agency

P.O. Box 100

A-1400, Vienna , Austria

Originally Written

by

Dermott E. Cullen

University of California

Lawrence Livermore National Laboratory

L-159

P.O. Box 808

Livermore, CA 94550, U.S.A.

tele: 925-423-7359

e.mail: cullen1@llnl.gov

website: http://home.cast.net/~redcullen1

Abstract: The codes are named "the Pre-processing" codes, because they are designed to pre-process ENDF formatted data, for later, further processing for use in applications. This is a modular set of computer codes, each of which reads and writes evaluated nuclear data in the ENDF format. Each code performs one or more independent operations on the data, as described below. These codes are designed to be computer independent, and are presently operational on every type of computer from large mainframe computer to small personal computers, such as IBM-PC and Power MAC. The codes are available on CD-ROM or diskettes from the IAEA Nuclear Data Section, free of charge upon request or can be downloaded from http://www-nds.iaea.org/ndspub/endf/prepro/

| Nuclear Data Section International Atomic Energy Agency P.O. Box 100 A-1400 Vienna Austria |  | e-mail: services@iaeand.iaea.org fax: (43-1) 26007 cable: INATOM VIENNA telex: 1-12645 telephone: (43-1) 2600-21710 |  |
| --- | --- | --- | --- |
|  | Online: TELNET or FTP: iaeand.iaea.org username: IAEANDS for interactive Nuclear Data Information System username: ANONYMOUS for FTP file transfer; Web: http://www-nds.iaea.org |  |  |

Note

The IAEA-NDS-reports should not be considered as formal publications.  When a nuclear data library is sent out by the IAEA Nuclear Data Section, it will be accompanied by an IAEA-NDS-report which should give the data user all necessary documentation on contents, format and origin of the data library.

IAEA-NDS-reports are updated whenever there is additional information of relevance to the users of the data library.

Neither the originator of the data libraries nor the IAEA assume any liability for their correctness or for any damages resulting from their use.

Citation guidelines

For citations care should be taken that credit is given to the author of the data library and/or to the data center which issued the data library.  The editor of the IAEA-NDS-report is usually not the author of the data library.

This computer code package should be cited as follows: D.E. Cullen, "PREPRO 2007: 2007 ENDF/B Pre-processing Codes”, report IAEA-NDS-39, Rev. 13, March 17, 2007

Nuclear Data Section Introduction

ENDF is the internationally agreed upon format for dissemination of evaluated nuclear data. The ENDF/B data library has now been through seven (VII) versions; the latest identified as ENDF/B-VII. Documentation for the current ENDF format and convention is available in ENDF-102, from the National Nuclear Data Center, Brookhaven National Laboratory

http://www.nndc.bnl.gov/nndcscr/documents/endf/endf102/

or the Nuclear Data Section of the IAEA

http://www-nds.iaea.org/ndspub/endf/prepro/DOCUMENT/DOCUMENT.HTM

The 2007 ENDF/B Pre-processing codes process nuclear data formatted in any version of the ENDF formats; ENDF/B-I through ENDF/B-VII evaluations. These codes can be used on virtually any computer: everything from large mainframe computers, to workstations, to IBM-PC (Windows or Linux) and Power MAC (OSX).

These codes are available free of charge on CD_ROM upon request from the Nuclear Data Section (see addresses on cover page) or downloaded from the Nuclear Data Section Web page

http://www-nds.iaea.org/ndspub/endf/prepro/

The present documentation (revision 13) completely supersedes all previous documentation of earlier versions of the ENDF/B Pre-processing data. It is strongly recommended that you use ONLY the latest 2007 version of the PREPRO codes. Failure to heed this warning can lead to completely erroneous results.

# Conditions for use of the codes

Any comments on the use of the codes, including difficulties encountered or any suggestions should be sent to the IAEA Nuclear Data Section. If any results obtained from using these codes are used or referenced in a publication, a copy of the publication should be sent to the IAEA Nuclear Data Section.

Dedication

Regardless of whose name appears on the cover of this report, most of the work involved in maintaining, testing and distributing the previous and current versions of the PREPRO codes, was done by Kevin McLaughlin, Nuclear Data Section, IAEA, Vienna. For over 20 years Kevin has played an invaluable role in updating and testing the PREPRO codes. After all of these years I am sorry to have to report that Kevin has now retired. I think I can speak for all present and past members of the Nuclear Data Section in saying that Kevin will be greatly missed both as a coworker and as a good friend.

Acknowledgement

I gratefully acknowledge the contribution of Andrej Trkov in continuing to propose interesting and useful extensions to these codes; keep those great ideas coming Andrej. I acknowledge the contribution of Ivan Sirakov in testing these codes and reporting all problems that he found; Ivan’s testing has greatly improved the reliability of these codes. I also acknowledge Jennie Manneschmidt, RSICC, Oak Ridge, for her review of the documentation and testing of the codes prior to their distribution through RSICC.

Table of Contents

## History and Terminology

Features of 2007 Version

Running Time

All ENDF Formats and Procedures

Consistent Handling of All ENDF Formatted Data

Computer Independence

MAC OSX Executables

Bigger, Faster, Improved Accuracy

On-Line Reports

Execution Timing

Features of All Versions

Code Documentation

Data Documentation

Obtaining the Codes

Your Feedback is IMPORTANT!!!

Implementing the Codes

What Computers do the codes run on?

The Most Up-to-Date Installation Instructions

Register as a User

Use of Codes

Read the Output Reports

Standard and Variable Filenames

Brief Description

Detailed Description

Verifying Implementation

Use of the Codes in Combination

Details of Compiling and Loading Codes

Parts of the Codes

Compiling/Loading

Graphics Codes

Postscript Output Files

On Screen Graphics

Comments from Codes

ACTIVATE

CONVERT

COMPLOT

DICTIN

EVALPLOT

FIXUP

GROUPIE

LEGEND

LINEAR

MERGER

MIXER

RECENT

RELABEL

SIGMA1

SIXPAK

VIRGIN

History and Terminology

Originally the Evaluated Nuclear Data File (ENDF) was divided into two different formats: ENDF/A which was designed to contain partial evaluations that might later be incorporated into complete evaluations, and ENDF/B which was designed to contain complete evaluations for use in applications. Originally these were physically two different formats, but circa 1970 both adopted the same format. Here I distinguish between the format, such as the current ENDF-6 format, and the data in the format, such as the current ENDF/B-VII data.

I try to distinguish between the ENDF-6 format, and the ENDF/B data that is coded into this format. The ENDF-6 format is now used universally to store evaluated nuclear data: in the ENDF/B-VII library in the United States, in JEFF in Western Europe, in JENDL in Japan, in CENDL in China, in BROND in Russia, etc. Here I will not be concerned with the differences between the contents of these data libraries. My only concern will be with the common ENDF-6 format, that all of these data libraries use. The PREPRO codes are designed to process evaluated data in any version of the ENDF format. The ENDF format has now been through six major versions, with the current format defined as ENDF-6.

Features of 2007 Version

Compared to earlier versions of these codes the 2007 version has the following features,

Running Time

It wasn't too many years ago that in order to process major ENDF/B evaluations we needed super, million dollar computers, and even then it could take days to process a large evaluation, such as U-238.

Need I say it: those days are gone forever. Today even small personal computers can be used to quickly process any ENDF evaluations. For example, on my IBM-PC, Pentium IV, 3.6 GHz computer [not a million dollar computer, a $ 1,000 computer (year 2007)], I can process U-238 in the time it takes me to go and get a cup of coffee - and with the next generation, it will require even less time.

So I am not going to list typical running times for the codes, for two reasons: 1) the running times have now become trivial, and 2) by the time you get a copy of this report any times I quote here will be outdated by the availability of newer, faster, and cheaper computers.

Bottom line: running time is no longer a major concern in processing ENDF data, and even small personal computers are now powerful enough to be used to process all ENDF evaluations.

All ENDF Formats and Procedures

These codes can automatically determine the ENDF format version that each evaluation is coded in and use the appropriate procedures. It should be particularly noted, that these codes now handle all ENDF formats and procedures through ENDF-6, and they have even been tested with all of the new ENDF/B-VII.0 evaluations that were approved by CSEWG in November 2006 and released for public use in December 2006

WARNING: The 2007 codes include extensions to handle all current ENDF formats and procedures, and corrections to problems that existed in earlier versions of these codes. As such the 2007 codes completely supersede all earlier versions and it is strongly recommended that all users of these codes only use the 2007 version of these codes. Failure to heed this warning can lead to completely erroneous results.

Consistent Handling of All ENDF Formatted Data

All of the codes now use exactly the same routines to handle all ENDF formatted input and output. This has resulted in a completely consistent interpretation of all ENDF formatted data by all of the codes, and has also allowed the precision of the ENDF output to be consistently extended in all codes. For 2007 the ENDF output is completely consistent for input into C and C++ codes, while still maintaining the accuracy of the data.

## Optional Input Parameters

All of the codes now allow input parameter files and ALL input parameters to be optional; all input parameters now have built-in default values. Of particular note is that allowable uncertainties are now optional input. This allows us to select what we consider the best choices, based on the most recent advances in the speed and size of computer.

Computer Independence

The only computer dependence in the 2007 codes is to define running time. Routines to define running time are supplied for most types of computers, and instructions are provided in this report to help you define a timing routine for any other type of computer.

MAC OSX Executables

Earlier versions of PREPRO supplied executables for MAC OS9. The current PREPRO supplies executable for MAC OSX (there is no executables for OS9). Under OSX the codes run much faster than under OS9. Under OSX the codes appear to the user very similar to how they appear on a UNIX or LINUX computer. The exception being the on-screen graphics codes COMPLOT and EVALPLOT, which use the UNIX X11 graphics library. On a MAC to execute these codes you must first start an X11 window and use this as if it were a UNIX or Linux Window.

Bigger, Faster, Improved Accuracy

In line with the enormous increase in computer sizes during the last few years, the 2007 versions are bigger, allowing more complicated problems to be run much more efficiently, and in general allowing each problem to be run much faster.

All of the codes now use double precision throughout, resulting in improved accuracy. Compared to the earlier versions that used a mixture of single and double precision, with modern compilers and hardware, using double precision throughout has also contributed to making the codes faster.

On-Line Reports

All of the codes now include an on-line report to your screen, and a report to an output file; the on-line report allows users to monitor the progress of each code as it executes. Earlier versions had no on-line report; as far as what the user saw, the code started and ran to conclusion without printing anything on-line. This made it impossible to monitor the progress of each code, and for long running problems often resulted in users terminating the codes before they completed execution, because it appeared that the codes weren't doing anything.

Execution Timing

The codes now include a timer, to print execution time at the end of processing each evaluation (MAT), and at the end of execution.

Features of All Versions

Code Documentation

These codes are designed to be self-documenting, in the sense that the latest documentation for each code is included as comments at the beginning of each code. Printed documentation, such as this report, is periodically published and consists mostly of a copy of the comment lines from the beginning of each code.

The user should be aware that the comment lines within the codes are continually updated to reflect the most recent status of the codes and these comments within the codes should always be considered to be the most recent documentation for the codes and may supersede published documentation, such as this document. Therefore users are advised to always read the documentation within the actual code that is being used.

Data Documentation

It is essential that the pedigree of the evaluated data be documented. This is the purpose of the comment lines at the beginning of each ENDF/B evaluation. The PREPRO codes are designed to document any operations that they perform on ENDF/B data. If one of these codes produces ENDF/B formatted output which in any way effects the actual evaluated data, what the code did is documented by adding additional comment lines at the end of the comment lines at the beginning of each evaluation, defining the code and input parameters that it used. The sequence of all such comments completely documents all of the operations that have been performed on the data. Code users are advised that it is very important to leave this documentation directly inside each evaluation, i.e., please do not modify these codes or the evaluations to remove this documentation.

Obtaining the Codes

These codes are available free of charge on CD ROM upon request from the Nuclear Data Section (see addresses on cover page) or downloaded from the Nuclear Data Section Web page

http://www-nds.iaea.org/ndspub/endf/prepro/

Your Feedback is IMPORTANT!!!

We are trying to develop a set of codes that are as computer independent as possible. In this effort your feedback is IMPORTANT!!! It is impossible for us to test these codes on all available computer/compiler combinations. Therefore your experience, on your specific computer/compiler can help us to improve the computer independence of these codes. It is also in your best interest to share your experience with us, since it will insure that future versions of these codes are as compatible as possible to meet your needs.

Please send all feedback via e. mail at,

mailto:services@iaeand.iaea.org

Implementing the Codes

What Computers do the codes run on?

The codes are designed to run on virtually any computer. The exceptions to this rule are the interactive graphics codes complot and evalplot, which are designed to produce on-screen graphics on UNIX workstations, IBM-PC, PowerMAC and VMS, i.e., not mainframe computers. However, even these codes can be used in their non-interactive mode, named comhard and evalhard (note the names to indicate hardcopy output), to produce Postscript formatted files that can be printed on any Postscript printer.

For use on IBM-PC running Windows or Linux, and on PowerMAC, the distribution includes executables, ready to use immediately. For use on a variety of UNIX based computers, the distribution includes a batch file for each type of computer, to compile and load all programs, and to then clean up by deleting everything not required to execute the programs. For other types of computers, see the section below on, Details of Compiling and Loading Codes

The Most Up-to-Date Installation Instructions

The most up-to-date installation instructions, documentation, and the codes, can be downloaded from the website,

http://www-nds.iaea.org/ndspub/endf/prepro/

Read the text and then select “Download Codes” or “Download Documentation”

We try to maintain these installation instructions as up-to-date as possible, based on user feedback. So if you have any problems or suggestions regarding installation please e.mail them to the Nuclear Data Section at,

mailto:services@iaeand.iaea.org

Register as a User

We try to maintain these codes and data as up-to-date as possible. So if you are using any of these codes it is important that you tell us about this, so that the Nuclear Data Section can put your name on the distribution list to inform you about the latest updates. This is a FREE!!! service which is  provided to users of these codes. We have tried to make this as easy as possible for you - PLEASE take a moment to e.mail to mailto:services@iaeand.iaea.org, and tell what codes you are using, and what type of computer(s) you are using - it’s as simple as that.

# Use of Codes

Read the Output Reports

MOST IMPORTANT! You cannot use these codes like a black box and assume that everything is perfect. Don't make the mistake of assuming that all ENDF/B data is perfect, or that these codes are perfect. It's up to you, the code user, to check and be sure that the data output by these codes is accurate and can be used in applications. If you don't, you are wasting your time, and will produce inaccurate results in your applications.

You can do this by reading the output reports produced by each code. These output reports will generally be quite small. They are intended to be used by you to quickly scan through them and look for WARNING or ERROR messages - these indicate problems with the ENDF/B data that you should check before using the data in any applications. You need not read each output report in detail; it is sufficient to merely search for the words WARNING or ERROR – these will always accompany important messages.

Checking these output reports doesn't take very much time, but failing to check them can cause you to waste an awful lot of time and can cause you headaches later, if you try to use data that a code has clearly indicated to be bad. If there are errors in the ENDF/B data, you are clearly in a “garbage in, garbage out” situation as far as the result you calculate in your applications. Caveat Emptor!

Standard and Variable Filenames

Currently all input files and input parameters are optional, and have built-in default values.

All of the codes have standard, built-in, filenames, that they will use by default, unless input parameters explicitly define other filenames.

The default filenames have been defined to make it easy for you to remember, and to be compatible with as many operating systems as possible, e.g., DOS, that only allows short filenames, and Unix, that allows longer filenames. The default filenames are all of the form NAME.EXT, where NAME identifies a program name, and EXT identifies the type of file. All default filenames use ONLY upper case characters. The basic filenames include,

1) ???.INP - The INPut parameters for each code, where ??? is the name of the code. For example, the input parameters for RECENT are in a file named RECENT.INP. This name cannot be changed by input. Currently these input files are optional; if they are not present default values are used for all input parameters.

2) ???.LST - The output LiSTing from each code, where ??? is the name of the code. For example, the output listing from RECENT is in a file named RECENT.LST. This name cannot be changed by input.

3) ???.IN - ENDF formatted data to be read (INput) be each code, where ??? is the name of the code. For example, the ENDF/B data read by RECENT are in a file named RECENT.IN. This name can be changed by input.

4) ???.OUT - ENDF formatted data written (OUTput) be each code, where ??? is the name of the code. For example, the ENDF/B data written by RECENT are in a file named RECENT.OUT. This name can be changed by input.

The above simple filename conventions will allow you to easily remember for each code, where the input parameters and output report are located, as well as where the ENDF/B data that is read and written by the code are located.

By input you can change the filenames of the ENDF formatted data files; data read and/or written.

If you input blank filenames the codes will use the default names (described above).

If you input anything else, the code will use the filenames you have defined. Variable filenames for each code can be up to 60 characters long. This allows you to specify directory structures, so that you can store your ENDF/B data in some rational way within a directory file structure.

For example if you store all of the ENDF/B-VI data files in a directory named ENDFB6, the following input filename used with linear will read a file named za092238 on an IBM-PC,

\ENDFB6\ORIGINAL\za092238

or on a Unix workstation,

/ENDFB6/ORIGINAL/za092238

Warning - generally on Unix workstations you will have to include the complete path to files. For example, the path to my files on my workstation may be /home/pd11/cullen, in which case my filename should be,

/home/pd11/cullen/ENDFB6/ORIGINAL/za092238

The ability to directly reference file structures is a very powerful facility that you should not overlook in organizing your ENDF/B data.

Brief Description

Linear            	- Linearize cross sections

Recent		- Reconstruct cross sections from resonance parameters

Sigma1		- Doppler broaden cross sections

Activate	         - Generate activation cross sections (MF=10) from MF=3 and 9 data

Legend		- Calculate/correct angular distributions

Sixpak		- Convert double differential data (MF=6) to single differential

Fixup		- Correct format and cross sections, define cross sections by summation

Dictin		- Create reaction dictionary (MF=1, MT=451)

Merger           - Retrieve and/or Merge evaluated data

Groupie		- Calculate group averages and multi-band parameters

Complot		- Plot comparisons of cross sections (MF=3, 23); Comhard for hardcopy

Evalplot		- Plot evaluated data (MF=3, 4, 5, 23, 27); Evalhard for hardcopy

Mixer		- Calculate mixtures of cross sections

Virgin		- Calculated transmitted uncollided (virgin) flux and reactions

Convert		- Convert codes for computer/precision/compiler

Relabel		- Relabel and sequence programs

Detailed Description

The codes can be used to: 1) extensively check and correct evaluated data prior to using them in applications, 2) pre-process the data into a form that will make subsequent use of the data much easier.

The normal sequence in which the codes are used is described below. WARNING  - this is the recommended sequence of codes that you should run to produce LEGAL ENDF formatted data, that conforms to ALL ENDF formats and conventions. Note in particular that if you do not run FIXUP and DICTIN at the end of this sequence the resulting ENDF data WILL NOT conform to all ENDF formats and conventions, and may cause problem if you subsequently try to use the data.

1) LINEAR - Linearize cross sections. ENDF format allows cross sections to be represented as tables of data points using a number of different interpolation laws between tabulated points; in order to obtain accurate results it is important to interpret the data using these interpolation laws. The interpolation laws are very useful during evaluation, but can present problems when they are used in applications. The subsequent use of the data can be greatly simplified and the accuracy of results improved by first linearizing all of the cross sections, i.e., replace the original tabulated data points and interpolation law by a new table where one can use linearly interpolation between tabulated points to within any required accuracy.

2) RECENT - Add the contribution of resonances to the cross sections. ENDF format allows cross sections to be represented as a contribution of resonance parameters and tabulated background corrections. This code will add the resonance contribution to the background cross sections in order to define the cross sections as linearly interpolable tables at 0 Kelvin (cold). Therefore subsequent codes need only deal with tabulated, linearly interpolable, 0 Kelvin cross sections.

3) SIGMA1 - Doppler broaden cross sections to any temperature of interest in applications. As in the case of LINEAR and RECENT all cross sections read and written by this code are tabulated, linearly interpolable. All subsequent codes need not explicitly consider temperature effects and need only deal with tabulated, linearly interpolable cross sections at a given temperature.

4) ACTIVATE – Combine neutron interaction cross sections (MF=3) and multipliers (MF=9) to create activation cross sections (MF=10). LINEAR and GROUPIE have been updated to process multipliers (MF=9) and activation cross sections (MF=10). The sequence of codes LINEAR, ACTIVATE, and GROUPIE allow you to produce group averaged activation cross sections.

5) LEGEND - Convert tabulated distributions and Legendre coefficients to linearly interpolable tables (similar to what LINEAR does for cross sections). Check all angular distributions and Legendre coefficients, in particular check for negative angular distributions and if found, correct the distributions to make them positive. Note, negative angular distributions can lead to numerical instabilities and unreliable results if they are used in applications.

6) SIXPAK - ENDF-6 format introduced double differential data (MF=6) into the ENDF/B system for the first time. If your application codes have not yet been updated to handle double differential data, you can use SIXPAK to obtain single differential (MF=4 and 5) approximations to double differential data. Earlier versions of SIXPAK only output results for outgoing (emitted) neutrons and photons, however currently SIXPAK will output angular distributions for discrete charged particle levels.

7) FIXUP - Define all cross sections to be consistently exactly equal to the sum of their parts, make format corrections, and a number of other tests and corrections to the data, BEFORE the data is actually used in applications. It is extremely important for use in applications to guarantee that the cross sections are exactly consistent. For example, the total cross section MUST to defined as equal to the sum of its parts at all energies that appear in one or more of the contributing parts. In addition it should be mentioned that the total will be equal to the sum of its parts at all energies (not just the energies at which the total is tabulated), only if all of the cross sections are linearly interpolable; this illustrates the importance of the steps described above in processing data through each of these codes. Note, if FIXUP's option to output all cross sections on a uniform energy grid is used, the FIXUP output is compatible for use as NJOY input.

8) DICTIN - Update the section index in MF=1, MT=451. This step need only be run if the subsequent codes that use the data refer to this index. If you are unsure whether or not this is the case, it is always best to include this step, since relative to the other codes described above this step requires very little running time.

After this sequence of codes has been run the results will be evaluated data that has been carefully checked for consistency and has been reduced to a form that can be used more easily and reliably in subsequent applications.

In addition to the codes mentioned above, this package includes a number of useful utility codes including,

1) MERGER - Retrieve and/or combine evaluated data. This code can be used to create a single file of data in the ENDF format from a number of different files, each of which is in the ENDF format. It can also be used to retrieve specific evaluated data from a larger ENDF/B library in order to simplify and optimize the subsequent use of the data in applications, e.g., if you have an entire ENDF/B library, but will only be using five evaluations for your applications, you can first use this code to create a mini-library containing only the five evaluations that you need for your application.

2) GROUPIE - Calculates self-shielded, multigroup cross sections and multiband parameters. This code can be used as a simple and very economical means of obtaining multigroup cross sections, in the ENDF format, which can be used in many applications where only multigroup cross sections are required, e.g., dosimetry. For comparing data using COMPLOT this code can be used to reduce evaluations that have many resonances, to a form in which integral differences through the resonance region can be more easily seen.

3) COMPLOT – Plot a comparison of cross sections from two different evaluations. This code can be used to compare cross sections, for each reaction, to define exactly how two evaluations differ. This can be extremely important if one has already used a given evaluation in applications and wishes to quickly and inexpensively determine whether or not a newer evaluation can be expected to produce significantly different results when used in applications. It is also an excellent and simple means of documenting the differences between two evaluations, e.g., what's the difference between the ENDF/B-VI, Release 4 and 5, U-235 cross sections? See the above comments under GROUPIE for suggestions concerning comparing evaluations that have many resonances. This code can be used as a simple means of visually checking all of these cross section data types and is often very useful to help understand the results obtained when data is used in applications. In addition, the graphic Postscript output can serve as a part of the documentation for evaluations. Two versions of exactly the same code are provided: complot to produce on-screen graphics, and comhard to produce Postscript, hardcopy, output.

4) EVALPLOT - Plot cross sections, angular distributions, Legendre coefficients and/or energy distributions, for neutron interaction data, neutron induced photon production data, and photon interaction data. This code can be used as a simple means of visually checking all of these data types and is often very useful to help understand the results obtained when data is used in applications. In addition, the graphic Postscript output can serve as a part of the documentation for evaluations. Two versions of exactly the same code are provided: evalplot to produce on-screen graphics, and evalhard to produce Postscript, hardcopy, output.

5) MIXER - Can be used to define the cross sections for a combination of materials, e.g., stainless steel. This code can be used in combination with COMPLOT to see which energy ranges are important for each material and each constituent of a material. This code can also be used to define the correct total cross section for use in transmission calculations (see, VIRGIN), as well as in self-shielding calculations (see, GROUPIE), in order to avoid the approximations normally incoherent in the Bonderenko method of self-shielding. Since ENDF/B-VI has moved in the direction of representing separate isotopes for each element, this code is particularly useful if your applications only requires a natural mixture of isotopes, e.g., use MIXER to combine isotopes into the natural element.

6) VIRGIN - Can be used to perform exact uncollided (virgin) transmission calculations (exact, assuming the tabulated, linearly interpolable cross sections are exact - no other approximations are used). By using the data that has been prepared by a combination of LINEAR, RECENT, SIGMA1, MIXER, etc., this code can be used to simulate transmission through any given material, or layers of different materials, at any given temperature. The results include both transmitted flux and reaction rates (as measured in self-indication measurements) vs. material thickness. The results can be obtained either on a continuous energy basis, or they can be binned (energy integrated) to simulate any given experimental resolution.

In addition there are two utility codes that operate on the codes, rather than on ENDF/B data.

1) RELABEL - Is a file maintenance code used to maintain all of the codes in this package. This code will normally not be used by users, unless they plan to modify these codes.

2) CONVERT - Format and optimize codes for use at any given computer installation. This code is no longer required by the PREPRO, since the codes are now completely computer independent. It is still included in this package only because users have found it useful for other purposes. Generally this code was used only once to format all of the codes prior to their first use on any computer.

Verifying Implementation

This distribution comes with a file named VERIFY, which is designed to run all of the codes, one after another, with the final two steps being to run EVALPLOT and COMPLOT, so that you can see the final results. VERIFY is a simple text file; its contents are shown below,

linear

recent

sigma1

activate

legend

fixup

dictin

groupie

mixer

virgin

evalplot

complot

When executed as a batch file, this will run the codes in the order indicated. The distributed input parameters have been defined so that each code reads the ENDF formatted data file produced by the preceding code, and writes the ENDF formatted data file that will be read by the following code.

To verify implementation immediately after you have installed the codes, DO NOT change any input parameters for ANY codes, and execute VERIFY. It will take between 5 minutes and an hour (depending on the speed of your computer), to run all of the codes. When you get to the final two graphics codes, EVALPLOT and COMPLOT, you can be assured that all of the codes have run successfully.

COMPLOT will compare the cross sections calculated by you on your computer to a standard set of results distributed with PREPRO 2007. In both cases cross sections are calculated by each code to within an accuracy of 1 %. Therefore when COMPLOT compares the results you may find differences of about 1 %. This difference is o.k., and merely indicates the differences due to precision to which the cross sections have been calculated. Subsequently, for use in your applications you can feel free to modify the input parameters for each code to meet the precision that you require.

WARNING – for UNIX users - some UNIX systems now include diction as a system command. In order to avoid this conflict, in PREPRO 2007 the code previously named diction has been renamed dictin.

Use of the Codes in Combination

Almost any computer will allow you to submit a batch job, in which case you can perform any number of operations one after the other, as is done in the above verification. These computers can utilize this facility to run any number of these codes in combination, minimize the total amount of disk space used, and most important, optimize the use of your time.

In order to run any number of codes one after the other, all you need is the facility to: 1) start a program, 2) rename a file, 3) delete a file, if you want to minimize disk space.

For example, if I want to run the sequence of codes, LINEAR, RECENT, SIGMA1, ACTIVATE, LEGEND, FIXUP and DICTIN and only keep the original data read by LINEAR and the final results output by DICTIN, I can use the standard ENDF filenames for the data read and written by each code, and submit the following batch file on an IBM-PC,

linear

rename LINEAR.OUT RECENT.IN

recent

delete RECENT.IN

rename RECENT.OUT SIGMA1.IN

sigma1

delete SIGMA1.IN

rename SIGMA1.OUT ACTIVTE.IN

activate

delete ACTIVATE.IN

rename ACTIVATE.OUT LEGEND.IN

legend

delete LEGEND.IN

rename LEGEND.OUT FIXUP.IN

fixup

delete FIXUP.IN

rename FIXUP.OUT DICTIN.IN

dictin

delete DICTIN.IN

Note, when each code finishes the above batch deck renames the ENDF formatted data output by the code to the filename of the ENDF formatted data input to the next code. When the next code finishes, the ENDF formatted data input to it is deleted (we no longer need it), and the cycle starts for the next code. More efficiently you could have defined ENDF input and output file names in the input parameter files for each code to link them together, e.g., instead of copying LINEAR.OUT to RECENT.IN, you could have defined the input file to RECENT to be named LINEAR.OUT.

The result will be the original data read by LINEAR is still in the file named LINEAR.IN, and the final result is in the file named DICTIN.OUT. All other intermediate files have been deleted.

On any other system, such as Unix, the names delete and rename may be different, but the basic idea remains the same.

An alternative to the above approach is to use the facility of the codes to read and write files from any file structure. For example, assume I have a directory named ENDFB6, and within this directory I have three sub-directories: ORIGINAL, TMP, and K300 (data Doppler broadened to 300 Kelvin). What I can do is define LINEAR input parameters to read a file from ENDFB6/ORIGINAL, define input parameters to RECENT, SIGMA1, ACTIVATE, LEGEND and FIXUP to produce ENDF output in ENDFB6/TMP, and have each code read the output from the preceding code. Finally I can define DICTIN input parameters to write the ENDF output into ENDFB6/K300, with its final filename. In this case if I do not worry about deleting the intermediate files, the batch input need only be the names of the codes to run, i.e.,

linear

recent

sigma1

activate

legend

fixup

dictin

Using a batch approach can save you a great deal of your precious time. You don't have to sit there and babysit your terminal in order to start each code as the preceding one finishes. You can use batch jobs to combine code executions, and go off to work (or play) until the sequence of codes finishes. If you then want to be sure that everything ran correctly, you can read the output reports from each code, i.e., see the ???.LST from each code, e.g., for RECENT see RECENT.LST – it is HIGHLY Recommended that you always read these files.

Details of Compiling and Loading Codes

For use on IBM-PC running Windows or Linux, and on PowerMAC, the distribution includes executables, ready to immediately use. For use on a variety of UNIX based computers, the distribution includes a batch file for each type of computer, to compile and load all programs, and to then clean up by deleting everything not required to execute the programs. Only for other types of computers need you be concerned with the details concerning compiling and loading the codes, which are described here.

Parts of the Codes

The codes have now been divided into a number of parts that should be combined when compiling and loading; see, example compile/load instructions below. The parts are,

1) The basic code

2) An include file to define code storage

3) Routines to allow all codes to now uniformly treat all ENDF formatted input and output (endfio.f)

4) Routines to allow scratch files to be defined either with or without file names,

scratcha.f = with file name

scratchb.f = without file name

Most compilers/computers allow scratch files to be defined without scratch file names, so use either scratcha.f or scratchb.f. However, some compilers/system combinations get confused when there are multiple scratch files without file names, e.g., Lahey on IBM-PC (use scratcha.f), and some compilers do not allow scratch files with file names, e.g., ABSOFT on IBM-PC and Power MAC (use scratchb.f).

4) A timer, to define the execution time for each code. The standard timer routine (timer.f) distributed with the codes uses the standard Unix routine ETIME; on some computers you will have to consult the on-line manual to see how to link to ETIME, e.g., HP.

If you are not using a Unix based computer, you will have to supply your own timing routine. It is recommended that you use the distributed version of timer.f, and add a function ETIME, that defines the execution time on your computer - see, the timing routines included for a variety of UNIX computers

If you do define a non-standard timer, try to define EXECUTION - NOT WALL CLOCK time - on some computers this isn't possible, e.g., IBM-PC running DOS - in which case use whatever you can.

If you can't figure out how to define running time, or you don't want the codes to print running time, instead of using the distributed timer.f, define and use the following dummy routine,

SUBROUTINE TIMER

RETURN

END

If you do define a non-standard timer, PLEASE send us a copy, identifying what computer/compiler you are using - over a period of time we intend to build up a library of timer routines for as many different computers as possible - which we will then distribute with the codes = future versions will be more compatible to meet your needs.

5) A graphics interface, for complot and evalplot.

Compiling/Loading

This section applies to all of the codes, except the graphics codes, complot and evalplot; see, below under graphics codes. Below is an example of how to compile/load the codes on a Unix based computer. For this example I illustrate how to create executables on a SUN workstation; timing routines are provided for most types of computers. Note,

1) No special libraries are used by these codes, so that compile/load instructions are very simple.

2) How the pieces are combined.

3) Use the HIGHEST LEVEL OPTIMIZATION available on your computer - this can make a BIG difference in running time.

4) SUN.f is the timing routine to use on a SUN workstation. Similar timing routines are provided for most types of computers.

f77 -o linear  -O linear.f  endfio.f scratchb.f timer.f SUN.f

f77 -o recent  -O recent.f  endfio.f scratchb.f timer.f SUN.f

f77 -o sigma1  -O sigma1.f  endfio.f scratchb.f timer.f SUN.f

f77 -o fixup   -O fixup.f   endfio.f scratchb.f timer.f SUN.f

f77 -o legend  -O legend.f  endfio.f scratchb.f timer.f SUN.f

f77 -o sixpak  -O sixpak.f  endfio.f scratchb.f timer.f SUN.f

f77 -o mixer   -O mixer.f   endfio.f scratchb.f timer.f SUN.f

f77 -o merger  -O merger.f  endfio.f scratchb.f timer.f SUN.f

f77 -o dictin  -O dictin.f  endfio.f scratchb.f timer.f SUN.f

f77 -o virgin  -O virgin.f  endfio.f scratchb.f timer.f SUN.f

f77 -o groupie -O groupie.f endfio.f scratchb.f timer.f SUN.f

f77 -o relabel -O relabel.f

f77 -o convert -O convert.f

Graphics Codes

The graphics codes - complot and evalplot - can be used to produce either,

1) Postscript output files for printed hardcopy, using executables named comhard and evalhard.

2) On screen graphics, using executables named complot and evalplot.

The two executables, complot and comhard, are exactly the same code, loaded with different graphics interfaces; both executables use the same input and output files, COMPLOT.INP and COMPLOT.LST. Similarly, the two executables, evalplot and evalhard, are exactly the same code, loaded with different graphics interfaces; both executables use the same input and output files, EVALPLOT.INP and EVALPLOT.LST.

Postscript Output Files

The Postscript graphics interface should be completely computer independent, and as such should run on any computer.

It will create a series of output files - none of which are sent to your printer during execution of the code.

Output for each plot is saved on disk, so when the code ends all of the plot files will still be on disk, and you can then send them to your printer, and/or, save them for later use.

WARNING - the codes always use the same file names, PLOT0001.ps, PLOT0002.ps, etc. So that running a code again will overwrite any files that you previously created. If you want to save files, moved them or rename them before running a code again.

To use this method to create these Postscript files use hardsave.f with the codes.

For Postscript graphics, no special libraries are used, and an example of how to compile/load the codes on a Unix based computer is shown below - this is very similar to the compile instructions shown above, with the addition of hardsave.f,

f77 -o comhard  -O complot.f  endfio.f scratchb.f timer.f hardsave.f SUN.f

f77 -o evalhard -O evalplot.f endfio.f scratchb.f timer.f hardsave.f SUN.f

Note, that here the executables are given the names for the hardcopy versions of the codes, comhard and evalhard.

On Screen Graphics

For on screen graphics the codes are loaded with screen.f, in contrast to the hardcopy version of the codes, described above, for Postscript graphics that are loaded with  hardsave.f.

Example Makefiles are included for a variety of Unix systems.

On screen graphics is VERY computer dependent, so on Unix computers you may have to modify the Unix Makefile - this should only involve finding out where the X11 graphics library is on your computer, and setting the correct path in the Makefile.

If you do have to modify the Makefile, please send me a copy of the modified file, identifying your computer, so that we can build up a library of Makefiles to be distributed with the codes then future versions will be compatible with your needs.

The codes are distributed with graphics interfaces for,

1) Unix, MAC OSX, and openVMS systems, using the X11 graphics library (screen.f, nodash.c, dash.c)

2) If you are using any other system, you will have to supply your own graphics interface - see, screen.f for a description of the simple interface used by these codes.

## Interacting with Graphics

When you are using evalplot there is no true on-screen interaction with the plots. If you wish to view different data over different energy ranges your only option is to change your input parameters in the file EVALPLOT.INP.

When you are using complot you can interact with the on-screen plots. Once a plot is displayed on your screen if you would like to see a portion of the energy range of the plot in greater detail, you can do this by using your mouse to zoom in by indicating the lower and upper energy limits of the energy range you would like to see. As soon as you select the range the next zoomed plot will appear on your screen, with the same data as on the previous plot, but only over the energy range that you have selected. WARNING – complot only generates plots when the two evaluations differ by more than the allowable uncertainty you define by input in the file COMPLOT.INP. This also applies when you interact with the plots. Therefore, if you use your mouse to select an energy range over which the two evaluations do not differ by more than your allowable uncertainty a zoomed plot will not be produced, but the results of the comparison will be reported in the output file COMPLOT.LST, and complot will proceed to its next comparison.

Comments from Codes

These codes are designed to be self-documenting, in the sense that the most up-to-date documentation is included as comments at the beginning of each code. Periodically documentation, such as this report, is published. But the user is warned that the comments in the codes are continuously updated and it is these comments within the codes that should be considered to be the most up-to-date documentation, and the user should read these comments before, and while, using these codes.

The following section contains a listing of the comments from the codes as of the publication date of this report. The comments are listed for each code alphabetically according to the name of the code, including,

ACTIVATE

CONVERT

COMPLOT

DICTIN

EVALPLOT

FIXUP

GROUPIE

LEGEND

LINEAR

MERGER

MIXER

RECENT

RELABEL

SIGMA1

SIXPAK

VIRGIN

======================================================================= Activate

Activate

PROGRAM ACTIVATE                                                   Activate

VERS. 2000-1 (APRIL 2000)   *INITIAL VERSION.                      Activate

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Activate

VERS. 2004-1 (JAN. 2004)    *CORRECTED ERROR - FIRST RECORD AFTER  Activate

MF=10 WAS MISSING.                    Activate

*ADDED INCLUDE TO DEFINE COMMON        Activate

*INCREASED MAX. POINTS FROM 100,000    Activate

TO 1,000,000.                         Activate

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII        Activate

Activate

Acknowledgement 2004                                               Activate

--------------------                                               Activate

Currently almost all improvements to this code are based upon      Activate

feedback from code users who report problems. This feedback        Activate

benefits ALL users of this code, and ALL users are encouraged      Activate

to report problems.                                                Activate

Activate

Improvements on the 2004 version of this code based on user        Activate

feedback including,                                                Activate

1) Andrej Trkov - reported that the first record of the section    Activate

after an inserted MF=10 was missing.             Activate

Activate

OWNED, MAINTAINED AND DISTRIBUTED BY                               Activate

------------------------------------                               Activate

THE NUCLEAR DATA SECTION                                           Activate

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Activate

P.O. BOX 100                                                       Activate

A-1400, VIENNA, AUSTRIA                                            Activate

EUROPE                                                             Activate

Activate

ORIGINALLY WRITTEN BY                                              Activate

------------------------------------                               Activate

DERMOTT E. CULLEN                                                  Activate

UNIVERSITY OF CALIFORNIA                                           Activate

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Activate

L-159                                                              Activate

P.O. BOX 808                                                       Activate

LIVERMORE, CA 94550                                                Activate

U.S.A.                                                             Activate

TELEPHONE  925-423-7359                                            Activate

E. MAIL    CULLEN1@LLNL.GOV                                        Activate

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Activate

Activate

AUTHORS MESSAGE                                                    Activate

---------------                                                    Activate

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Activate

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Activate

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Activate

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                  Activate

Activate

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Activate

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Activate

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Activate

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Activate

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Activate

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Activate

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Activate

COMPUTER.                                                          Activate

Activate

PURPOSE                                                            Activate

-------                                                            Activate

THIS PROGRAM IS DESIGNED TO CREATE FILE 10 ACTIVATION CROSS        Activate

SECTIONS BY COMBINING FILE 3 CROSS SECTIONS AND FILE 9 MULTIPLIERS Activate

Activate

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY  Activate

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE    Activate

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                             Activate

Activate

ASSUMPTIONS                                                        Activate

-----------                                                        Activate

IT IS ASSUMED THAT THE FILE 3 AND 9 DATA HAVE BEEN LINEARIZED      Activate

BEFORE THIS CODE IS USED - FILE 3 AND 9 DATA CAN BE LINEARIZED     Activate

USING PROGRAM LINEAR.                                              Activate

Activate

IT IS ASSUMED THAT THE FILE 9 MULTIPLIERS ARE FAIRLY SMOOTH VERSUS Activate

ENERGY, AND THAT THE ACTIVATION CROSS SECTIONS FOR FILE 10 CAN BE  Activate

DEFINED AT EXACTLY THE SAME ENERGIES AS THE FILE 3 CROSS SECTIONS, Activate

AND THAT THESE NEED MERELY BE MULTIPLIED BY THE FILE 9 TO DEFINE   Activate

THE FILE 10 ACTIVATION CROSS SECTIONS.                             Activate

Activate

ENDF/B FORMAT                                                      Activate

-------------                                                      Activate

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Activate

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Activate

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Activate

Activate

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Activate

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Activate

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Activate

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Activate

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451  Activate

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL   Activate

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Activate

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Activate

Activate

OUTPUT FORMAT                                                      Activate

-------------                                                      Activate

ALL ENERGIES WILL BE OUTPUT IN F (INSTEAD OF E) FORMAT IN ORDER    Activate

TO ALLOW ENERGIES TO BE WRITTEN WITH UP TO 9 DIGITS OF ACCURACY.   Activate

COMPARISON OF THE NORMAL ENDF/B CONVENTION OF 6 DIGITS TO THE 9    Activate

DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE TO USE    Activate

THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA DUE TO     Activate

TRUNCATION OF ENERGIES TO 6 DIGITS DURING OUTPUT.                  Activate

Activate

CONTENTS OF OUTPUT                                                 Activate

------------------                                                 Activate

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE PROCESSED DATA, E.G.,  Activate

ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO INCLUDED.                Activate

Activate

DOCUMENTATION                                                      Activate

-------------                                                      Activate

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Activate

BY THE ADDITION OF 3 COMMENT LINES AT THE END OF EACH HOLLERITH    Activate

SECTION IN THE FORM                                                Activate

Activate

***************** PROGRAM ACTIVATE (2007-1) ****************       Activate

FILE 10 ACTIVATION CROSS SECTIONS HAVE BEEN DEFINED BY COMBINING   Activate

FILE 3 CROSS SECTIONS AND FILE 9 MULTIPLIERS. FILE 9 DELETED.      Activate

Activate

THE ORDER OF SIMILAR COMMENTS (FROM RECENT, SIGMA1 AND GROUPIE)    Activate

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Activate

THE DATA BY THESE PROGRAMS.                                        Activate

Activate

THESE COMMENT LINES ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Activate

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Activate

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Activate

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Activate

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Activate

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Activate

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Activate

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Activate

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Activate

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Activate

Activate

REACTION INDEX                                                     Activate

--------------                                                     Activate

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Activate

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Activate

Activate

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Activate

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Activate

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Activate

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Activate

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Activate

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Activate

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Activate

Activate

SECTION SIZE                                                       Activate

------------                                                       Activate

SECTIONS OF MF=9 MULTIPLIERS ARE LIMITED TO A MAXIMUM OF 10,000    Activate

ENERGY POINTS.                                                     Activate

Activate

THERE IS NO LIMIT ON THE NUMBER OF ENERGY POINTS IN MF=3 AND 10    Activate

TABLES.                                                            Activate

Activate

SELECTION OF DATA                                                  Activate

-----------------                                                  Activate

THE PROGRAM PROCESSES ALL ENDF/B DATA ON A SERIES OF ENDF/B TAPES. Activate

Activate

PROGRAM OPERATION                                                  Activate

-----------------                                                  Activate

PASS #1                                                            Activate

-------                                                            Activate

THE ENTIRE MAT IS COPIED TO A SCRATCH FILE IN THE ENDF/B ASCII     Activate

FORMAT AND WHILE COPYING IT TO SCRATCH MF=3, 9, AND 10 ARE ALSO    Activate

COPIED TO SEPERATE SCRATCH FILES, I.E., THERE ARE A TOTAL OF 4     Activate

SCRATCH FILES - SEE THEIR DEFINITIONS BELOW.                       Activate

Activate

PASS #2                                                            Activate

-------                                                            Activate

IF NO MF=9 MULTIPLIERS ARE FOUND DURING PASS #1, THE ENTIRE MAT    Activate

IS COPIED FROM SCRATCH TO THE OUTPUT FILE, WITHOUT ANY CHECKS.     Activate

Activate

IF MF=9 MULTIPLIERS ARE FOUND THEY ARE USED WITH MF=3 CROSS        Activate

SECTIONS TO CREATE MF=10 ACTIVATION CROSS SECTIONS.                Activate

Activate

FOR ANY SECTION OF MF=10 DATA FOR WHICH NO MF=9 MULTIPLIERS ARE    Activate

FOUND, THE ORIGINAL MF=10 IS OUTPUT.                               Activate

Activate

FOR CONSISTENCY ALL MF=9 MULTIPLIERS ARE DELETED, I.E., THEY ARE   Activate

NOT INCLUDED IN THE OUTPUT.                                        Activate

Activate

KEEP EVALUATED DATA POINTS                                         Activate

--------------------------                                         Activate

THE FILE 10 OUTPUT WILL BE AT EXACTLY THE SAME ENERGY POINTS AS    Activate

THE FILE 3 CROSS SECTIONS USED TO DEFINE THE FILE 10 ACTIVATION    Activate

CROSS SECTIONS.                                                    Activate

Activate

INPUT FILES                                                        Activate

-----------                                                        Activate

UNIT  DESCRIPTION                                                  Activate

----  -----------                                                  Activate

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                     Activate

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Activate

Activate

OUTPUT FILES                                                       Activate

------------                                                       Activate

UNIT  DESCRIPTION                                                  Activate

----  -----------                                                  Activate

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Activate

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Activate

Activate

SCRATCH FILES                                                      Activate

-------------                                                      Activate

UNIT  DESCRIPTION                                                  Activate

----  -----------                                                  Activate

12  SCRATCH FILE FOR ALL MAT (BCD - 80 CHARACTERS/RECORD)        Activate

14  SCRATCH FILE FOR MF=3 DATA (BCD - 80 CHARACTERS/RECORD)      Activate

15  SCRATCH FILE FOR MF=9 DATA (BCD - 80 CHARACTERS/RECORD)      Activate

16  SCRATCH FILE FOR MF=10 DATA (BCD - 80 CHARACTERS/RECORD)     Activate

Activate

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Activate

----------------------------------------------------               Activate

UNIT  FILE NAME                                                    Activate

----  ----------                                                   Activate

2   ACTIVATE.INP                                                 Activate

3   ACTIVATE.LST                                                 Activate

10   ENDFB.IN                                                     Activate

11   ENDFB.OUT                                                    Activate

12   (SCRATCH)                                                    Activate

14   (SCRATCH)                                                    Activate

15   (SCRATCH)                                                    Activate

Activate

INPUT PARAMETERS                                                   Activate

----------------                                                   Activate

Activate

LINE  COLS.  DESCRIPTION                                           Activate

----  -----  -----------                                           Activate

1   1-60  ENDF/B INPUT DATA FILENAME                            Activate

(STANDARD OPTION = ENDFB.IN)                          Activate

2   1-60  ENDF/B OUTPUT DATA FILENAME                           Activate

(STANDARD OPTION = ENDFB.OUT)                         Activate

Activate

ANY NUMBER OF PAIRS OF INPUT LINES MAY BE USED, TO PROCESS ANY     Activate

NUMBER OF ENDF/B TAPES, ONE AFTER ANOTHER.                         Activate

Activate

EXAMPLE INPUT NO. 1                                                Activate

-------------------                                                Activate

PROCESS ENDF/B TAPES NAMED, TAPE121, TAPE122, AND TAPE123, AND     Activate

NAME THE OUTPUT FILES ACTIVATE121, ACTIVATE122, AND ACTIVATE123.   Activate

Activate

IN THIS CASE THE FOLLOWING 6 INPUT LINES ARE REQUIRED              Activate

Activate

TAPE121                                                                Activate

ACTIVATE121                                                            Activate

TAPE122                                                                Activate

ACTIVATE122                                                            Activate

TAPE123                                                                Activate

ACTIVATE123                                                            Activate

Activate

EXAMPLE INPUT NO. 2                                                Activate

-------------------                                                Activate

SAME AS THE ABOVE CASE, EXCEPT THAT IN THIS CASE THE ORIGINAL      Activate

3 TAPES ARE IN A DIRECTORY NAMED \ENDFB6\ORIGINAL, AND THE         Activate

RESULTS WILL BE WRITTEN INTO A DIRECTORY NAMED \ENDFB6\ACTIVATE.   Activate

Activate

IN THIS CASE THE FOLLOWING 6 INPUT LINES ARE REQUIRED              Activate

Activate

\ENDFB6\ORIGINAL\TAPE121                                               Activate

\ENDFB6\ACTIVATE\ACTIVATE121                                           Activate

\ENDFB6\ORIGINAL\TAPE122                                               Activate

\ENDFB6\ACTIVATE\ACTIVATE122                                           Activate

\ENDFB6\ORIGINAL\TAPE123                                               Activate

\ENDFB6\ACTIVATE\ACTIVATE123                                           Activate

Activate

======================================================================= Activate

======================================================================= Complot

Complot

PROGRAM COMPLOT                                                    Complot

===============                                                    Complot

VERSION 83-1 (FEBRUARY, 1983)                                      Complot

VERSION 83-2 (MAY, 1983)                                           Complot

VERSION 83-3 (DECEMBER, 1983) *MAJOR MODIFICATION.                 Complot

*ADDED SELECTION OF PLOTS BY MAT OR  Complot

ZA/MT/ENERGY RANGE (EV).            Complot

*ADDED VARIABLE AXIS UNITS (PROGRAM  Complot

CONTROLLED..X=MILLI-EV, EV, KEV,    Complot

MEV...Y=MILLI-BARNS, BARNS).        Complot

VERSION 84-1 (APRIL, 1984)    *ADDED SELECTION BY REACTION/ENERGY  Complot

RANGE.                              Complot

*ADDED IDENTIFY DATA POINTS OPTION   Complot

(SMALL BOX DRAWN AROUND EACH CROSS  Complot

SECTION AND RATIO POINT).          Complot

*IMPROVED NON-IBM GRAPHICS INTERFACE Complot

(ALL CHARACTER POSITIONING NOW      Complot

BASED ON CHARACTER, NOT RASTER,    Complot

SIZE).                             Complot

VERSION 85-1 (APRIL, 1985)    *SPECIAL I/O ROUTINES TO GUARANTEE   Complot

ACCURACY OF ENERGY.                 Complot

*DOUBLE PRECISION TREATMENT OF       Complot

ENERGY (REQUIRED FOR NARROW         Complot

RESONANCES).                        Complot

*ADDED (ZA,MT) EQUIVALENCE OPTION.   Complot

*ADDED SMALL PLOT OPTION.            Complot

VERSION 85-2 (AUGUST, 1985)   *FORTRAN-77/H VERSION                Complot

VERSION 86-1 (JANUARY, 1986)  *ENERGY DEPENDENT SCATTERING RADIUS  Complot

VERSION 86-2 (DECEMBER, 1986) *DOUBLE PRECISION PLOT SCALING       Complot

(REQUIRED FOR NARROW ENERGY RANGES) Complot

VERSION 88-1 (JULY 1988)      *MAJOR REVISION TO MAKE CODE EASILY  Complot

INTERFACEABLE TO ALMOST ANY PLOTTER Complot

*WARNING..INPUT PARAMETERS FROM BEEN Complot

CHANGED (SEE, DESCRIPTION BELOW)    Complot

*COMPUTER INDEPENDENT SOFTWARE       Complot

CHARACTERS.                         Complot

*COLOR PLOTS.                        Complot

*MT NUMBER DEFINITIONS FROM DATA     Complot

FILE READ BY PROGRAM                Complot

*FORTRAN-77 REQUIRED (FORTRAN-H NO   Complot

SUPPORTED BY THIS PROGRAM).         Complot

*OPTION...INTERNALLY DEFINE ALL I/O  Complot

FILE NAMES (SEE, SUBROUTINE FILEIO  Complot

FOR DETAILS).                       Complot

*IMPROVED BASED ON USER COMMENTS.    Complot

VERSION 88-2 (OCTOBER 1988)   *IMPROVED BASED ON USER COMMENTS.    Complot

*ADDED LIVERMORE CIVIC COMPILER      Complot

CONVENTIONS.                        Complot

*UPDATED TO USE NEW PROGRAM CONVERT  Complot

KEYWORDS.                           Complot

VERSION 89-1 (JANUARY 1989)   *PSYCHOANALYZED BY PROGRAM FREUD TO  Complot

INSURE PROGRAM WILL NOT DO ANYTHING Complot

CRAZY.                              Complot

*FORTRAN-77/FORTRAN-H COMPATIBLE     Complot

*SPECIAL ENDF/B MATERIAL DEFINITIONS Complot

(ZA.LT.1000) FROM DATA FILE READ    Complot

BY PROGRAM.                         Complot

VERSION 89-2 (MARCH 1989)     *ADDED ENDF/B-V AND VI MT            Complot

DEFINITIONS. PROGRAM WILL DETERMINE Complot

ENDF/B FORMAT BASED ON MF=1,        Complot

MT=451 AND USE AS PPROPRIATE MT     Complot

DEFINITIONS. IF NO MF=1, MT=451     Complot

PROGRAM WILL USE ENDF/B-VI          Complot

MT DEFINITIONS.                     Complot

VERSION 90-1 (AUGUST 1990)    *A NEW PROGRAM                       Complot

*ADDED INTERACTIVE MOUSE INPUT       Complot

*ADDED 3 CHARACTER FONTS             Complot

*ADDED PHOTON DATA, MF=23 AND 27     Complot

*ADDED FORTRAN SAVE OPTION.          Complot

*ADDED MAXIMUM RATIO RANGE WHEN      Complot

PLOTTING RATIOS.                    Complot

*ADDED GRID TYPES                    Complot

*ADDED VARIABLE LINE THICKNESS       Complot

*WARNING...INPUT PARAMETER FORMAT    Complot

HAS BEEN CHANGED...SEE DESCRIPTION  Complot

BELOW.                              Complot

VERSION 92-1 (JANUARY 1992)   *ADDED INCIDENT CHARGED PARTICLES    Complot

(IDENTIFIED IN PLOT TITLES)         Complot

*ADDED COMPLETELY COMPATIBLE I/O     Complot

FOR READING FLOATING POINT NUMBERS. Complot

VERSION 92-2 (MAY 1992)       *CORRECTED DESCRIPTION OF INPUT      Complot

PARAMETERS AND EXAMPLE PROBLEMS.    Complot

*ADDED VARIABLE CHARACTER SIZE INPUT Complot

VERSION 93-1 (MARCH 1993)     *UPDATE FOR ON SCREEN GRAPHIC        Complot

OUTPUT USING THE LAHEY COMPILER     Complot

*ADDED NU-BAR (TOTAL, DELAYED,       Complot

PROMPT).                            Complot

VERSION 94-1 (JANUARY 1994)   *VARIABLE ENDF/B DATA FILENAMES      Complot

TO ALLOW ACCESS TO FILE STRUCTURES  Complot

(WARNING - INPUT PARAMETER FORMAT   Complot

HAS BEEN CHANGED)                   Complot

*CLOSE ALL FILES BEFORE TERMINATING  Complot

(SEE, SUBROUTINE ENDIT)             Complot

VERSION 95-1 (MARCH 1995)     *CORRECTED CROSS SECTION             Complot

MULTIPLIER FOR EQUIVALENCES         Complot

*CORRECTED RATIO SCALING, FOR        Complot

MAXIMUM RATIO LESS THAN 1.0         Complot

VERSION 96-1 (JANUARY 1996)   *COMPLETE RE-WRITE                   Complot

*IMPROVED COMPUTER INDEPENDENCE      Complot

*ALL DOUBLE PRECISION                Complot

*UNIFORM TREATMENT OF ENDF/B I/O     Complot

*IMPROVED OUTPUT PRECISION           Complot

*DEFINED SCRATCH FILE NAMES          Complot

*INCREASED PAGE SIZE FROM 24000      Complot

TO 48000 POINTS                     Complot

VERSION 97-1 (APRIL 1997)     *INCREASED PAGE SIZE FROM 48000      Complot

TO 480000 POINTS                    Complot

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Complot

POINT READ FOR MORE DIGITS            Complot

*UPDATED TEST FOR ENDF/B FORMAT        Complot

VERSION BASED ON RECENT FORMAT CHANGE Complot

*GENERAL IMPROVEMENTS BASED ON         Complot

USER FEEDBACK                         Complot

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Complot

USER FEEDBACK                         Complot

VERS. 2002-1 (MAY 2002)     *INPUT PARAMETERS OPTIONAL             Complot

*CONTROL MINIMUM RATIO RANGE BY INPUT  Complot

*OPTIONAL BLACK OR WHITE BACKGROUND    Complot

VERS. 2004-1 (SEPT. 2004)   *ADDED INCLUDE FOR COMMON              Complot

*INCREASED PAGE SIZE FROM 480000       Complot

TO 600000 POINTS                      Complot

*ADDED NEW REICH-MOORE TO FILE2 TO     Complot

ALLOW IDENTIFICATION OF RESOLVED AND  Complot

ANY FOLLOWING UNRESOLVED RESONANCE    Complot

REGIONS.                              Complot

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Complot

*INCREASED MAXLOAD TO 600,000 FROM     Complot

12,000                                Complot

Complot

OWNED, MAINTAINED AND DISTRIBUTED BY                               Complot

------------------------------------                               Complot

THE NUCLEAR DATA SECTION                                           Complot

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Complot

P.O. BOX 100                                                       Complot

A-1400, VIENNA, AUSTRIA                                            Complot

EUROPE                                                             Complot

Complot

ORIGINALLY WRITTEN BY                                              Complot

------------------------------------                               Complot

DERMOTT E. CULLEN                                                  Complot

UNIVERSITY OF CALIFORNIA                                           Complot

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Complot

L-159                                                              Complot

P.O. BOX 808                                                       Complot

LIVERMORE, CA 94550                                                Complot

U.S.A.                                                             Complot

TELEPHONE  925-423-7359                                            Complot

E. MAIL    CULLEN1@LLNL.GOV                                        Complot

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Complot

Complot

Complot

AUTHORS MESSAGE                                                    Complot

---------------                                                    Complot

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENTATION   Complot

ALL RECENT IMPROVEMENTS. PLEASE READ ALL OF THESE COMMENTS BEFORE, Complot

PARTICULARLY THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.     Complot

Complot

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Complot

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Complot

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Complot

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Complot

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Complot

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Complot

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Complot

COMPUTER.                                                          Complot

Complot

PURPOSE                                                            Complot

-------                                                            Complot

COMPARE ENDF/B FORMATTED DATA FROM TWO SEPARATE INPUT TAPES.       Complot

REACTIONS ARE CONSIDERED TO BE COMPARABLE IF THEY HAVE THE SAME    Complot

(ZA,MF,MT). RESULTS ARE PRESENTED IN GRAPHICAL FORM.               Complot

Complot

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Complot

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Complot

DISK OR ANY OTHER MEDIUM.                                          Complot

Complot

ON WHAT COMPUTERS WILL THE PROGRAM RUN                             Complot

------------------------------------------------------------------ Complot

THE PROGRAM HAS BEEN IMPLEMENTED ON A VARIETY OF COMPUTERS FROM    Complot

CRAY AND IBM MAINFRAME TO SUN WORKSTATIONS TO AN IBM-AT PC. THE    Complot

PROGRAM IS SMALL ENOUGH TO RUN ON VIRTUALLY ANY COMPUTER.          Complot

Complot

THE PROGRAM USES A SIMPLE CALCOMP LIKE GRAPHICS INTERFACE          Complot

(DESCRIBED BELOW) AND ALLOWS THE USER SPECIFY THE PHYSICAL SIZE    Complot

OF THE PLOTTER BEING USED, BY INPUT PARAMETERS. USING THESE        Complot

CONVENTIONS THIS PROGRAM CAN BE EASILY INTERFACED TO VIRTUALLY     Complot

ANY PLOTTER.                                                       Complot

Complot

FOR SPECIAL CONSIDERATIONS SEE THE SECTIONS BELOW ON,              Complot

(1) COMPUTER DEPENDENT CODING                                      Complot

(2) PLOTTER/GRAPHICS TERMINAL INTERFACE                            Complot

Complot

GRAPHICS INTERFACE                                                 Complot

------------------------------------------------------------------ Complot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE GRAPHICS INTERFACE WHICH   Complot

REQUIRES ONLY 3 SUBROUTINES...PLOTS, PLOT AND PEN (DESCRIBED IN    Complot

DETAIL BELOW). ALL CHARACTERS AND SYMBOLS ARE DRAWN USING TABLES   Complot

OF PEN STROKES (SUPPLIED WITH THIS PROGRAM). USING THIS METHOD     Complot

THE PROGRAM SHOULD BE SIMPLE TO INTERFACE TO VIRTUALLY ANY PLOTTER Complot

OR GRAPHICS TERMINAL AND THE APPEARANCE AND LAYOUT OF THE PLOTS    Complot

SHOULD BE INDEPENDENT OF WHICH PLOTTER IS USED.                    Complot

Complot

ON WHAT PLOTTERS WILL THE PROGRAM RUN                              Complot

------------------------------------------------------------------ Complot

THE PLOTTER MAY USE UNITS OF INCHES, CENTIMETERS, MILLIMETERS,     Complot

VIRTUALLY ANYTHING. INTERNALLY THE PROGRAM WILL DEFINE PLOTS IN    Complot

APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. AS PART OF THE        Complot

INPUT THE USER DEFINES THE ACTUAL SIZE OF THE PLOT IN THE UNITS    Complot

(I.E., INCHES, CENTIMETERS, MILLIMETERS, WHATEVER) OF THE REAL     Complot

PLOT. THE PLOT IS TRANSFORMED TO THE SIZE OF THE LOCAL PLOTTER     Complot

AND OUTPUT. USING THIS CONVENTION THIS PROGRAM SHOULD BE EASY      Complot

TO INTERFACE TO VIRTUALLY ANY PLOTTER OR GRAPHICS TERMINAL.        Complot

Complot

PROGRAM IDENTIFICATION                                             Complot

----------------------                                             Complot

AS DISTRIBUTED THE FIRST FRAME OF PLOTTED OUTPUT WILL DOCUMENT     Complot

THE PROGRAM NAME, VERSION AND INSTALLATION. THIS INFORMATION IS    Complot

STORED AS DATA IN THE ARRAY VERSES NEAR THE BEGINNING OF           Complot

SUBROUTINE FRAME1. IF YOU WISH TO CUSTOMIZE THE OUTPUT TO IDENTIFY Complot

YOUR INSTALLATION CHANGE THE LAST TWO LINES OF THE ARRAY (VERSES). Complot

Complot

ENDF/B FORMAT                                                      Complot

-------------                                                      Complot

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Complot

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Complot

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Complot

Complot

BOTH SETS OF EVALUATED DATA MUST BE IN THE ENDF/B FORMAT. ONLY     Complot

SECTIONS OF FILE 2 (RESONANCE PARAMETERS) AND FILES 3, 23 AND 27   Complot

(TABULATED DATA) WILL BE READ AND ALL OTHER SECTIONS WILL BE       Complot

SKIPPED. IN FILE 2 THE ONLY IMPORTANT INFORMATION IS THE ENERGY    Complot

LIMITS OF THE RESOLVED AND UNRESOLVED RESONANCE REGION WHICH IS    Complot

LOCATED IN THE SAME FIELDS IN ALL VERSIONS OF THE ENDF/B FORMAT.   Complot

SIMILARLY THE FORMAT OF FILES 3, 23 AND 27 IS THE SAME IN ALL      Complot

VERSIONS OF ENDF/B. THEREFORE THIS PROGRAM CAN BE USED WITH DATA   Complot

IN ANY ENDF/B FORMAT (I.E. ENDF/B-I, II, III, IV, V OR VI).        Complot

Complot

CROSS SECTION INTERPOLATION                                        Complot

---------------------------                                        Complot

CROSS SECTIONS MUST BE IN EITHER HISTOGRAM (I.E., INTERPOLATION    Complot

LAW 1) OR LINEARLY INTERPOLABLE (I.E. INTERPOLATION LAW 2) FORM.   Complot

IF THEY ARE NOT A WARNING MESSAGE WILL BE PRINTED AND EXECUTION    Complot

WILL BE TERMINATED. SEE INSTRUCTIONS BELOW ON HOW TO CONVERT       Complot

DATA TO HISTOGRAM OR LINEARLY INTERPOLABLE FORM.                   Complot

Complot

REACTION INDEX                                                     Complot

--------------                                                     Complot

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Complot

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Complot

Complot

SECTION SIZE                                                       Complot

------------                                                       Complot

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Complot

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Complot

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Complot

Complot

DATA SELECTION                                                     Complot

--------------                                                     Complot

THE USER MAY SPECIFYING THE DATA TO BE COMPARED BY INPUTTING UP    Complot

TO 100 MAT/MT/ENERGY OR ZA/MT/ENERGY RANGES. IF THE UPPER LIMIT    Complot

OF THE MAT OR ZA RANGE IS LESS THAN THE LOWER LIMIT IT WILL BE SET Complot

EQUAL TO THE LOWER LIMIT (I.E. THIS INDICATE ONLY COMPARE ONE      Complot

MAT OR ZA). IF THE UPPER LIMIT IS STILL ZERO IT WILL BE SET TO     Complot

9999 (NO LIMIT). IF THE UPPER MF OR MT LIMIT IS ZERO IT WILL BE    Complot

SET TO 99 OR 999, RESPECTIVELY (NO LIMIT). IF THE UPPER ENERGY     Complot

LIMIT IS ZERO IT WILL BE SET TO A LARGE NUMBER (NO LIMIT).         Complot

Complot

THE LIST OF RANGES MUST BE TERMINATED BY A BLANK LINE (I.E. ZERO   Complot

LOWER AND UPPER MAT/MF/MT OR ZA/MF/MT LIMITS).                     Complot

Complot

IF THE FIRST RANGE LINE IS BLANK THIS LINE WILL TERMINATE THE      Complot

LIST OF REQUESTS (I.E. A SECOND BLANK LINE NEED NOT BE INPUT)      Complot

AND ALL PHYSICALLY COMPARABLE DATA WILL BE PLOTTED.                Complot

Complot

WHICH REACTIONS WILL BE PLOTTED                                    Complot

-------------------------------                                    Complot

THOSE REACTIONS WITH THE SAME (ZA, MF, MT) WILL BE COMPARED, BUT   Complot

ONLY THOSE DATA WHICH DIFFER BY A USER SPECIFIED ALLOWABLE         Complot

DIFFERENCE WILL BE PLOTTED. IN ORDER TO FORCE ALL COMPARABLE       Complot

REACTIONS TO BE PLOTTED THE USER NEED ONLY SPECIFY AN ALLOWABLE    Complot

DIFFERENCE OF ZERO.                                                Complot

Complot

EQUIVALENT REACTIONS                                               Complot

--------------------                                               Complot

IN ORDER TO COMPARE REACTIONS WHICH HAVE DIFFERENT ZA, MF OR MT    Complot

THE USER IS ALLOWED TO SPECIFY AN EQUIVALENCE LIST OF UP TO        Complot

100 (ZA,MF,MT) COMBINATIONS ON THE MASTER FILE WHICH ARE TO BE     Complot

EQUATED TO DIFFERENT (ZA,MF,MT) ON THE SECOND FILE. THIS OPTION    Complot

MAY BE USED TO COMPARE SIMILAR REACTIONS FROM DIFFERENT MATERIALS  Complot

(E.G. IRON AND NICKEL INELASTIC SCATTERING) OR DIFFERENT REACTIONS Complot

FROM THE SAME OR DIFFERENT MATERIALS (E.G. U-235 CAPTURE AND       Complot

FISSION - IN WHICH CASE THE RATIO WILL BE THE CAPTURE TO FISSION   Complot

RATIO) OR THE SAME REACTION IN DIFFERENT VERSIONS OF THE ENDF/B    Complot

FORMAT WHICH MAY BE ASSIGNED DIFFERENT MT NUMBERS, E.G., THE       Complot

PHOTOELECTRIC CROSS SECTION IS MT=602 IN ENDF/B-V AND EARLIER      Complot

VERSIONS OF ENDF/B, BUT IS MT=522 IN ENDF/B-VI.                    Complot

Complot

IN THESE EQUIVALENCE LISTS A ZERO FIELD IMPLIES ALL. FOR EXAMPLE,  Complot

TO EQUATE MT=522 FROM ONE FILE TO MT=602 ON THE OTHER, FOR ALL     Complot

MATERIALS, ONE NEED ONLY SPECIFY ZA=0, MF=23, MT=522 EQUIVALENT    Complot

TO ZA=0, MF=23 AND MT=602.                                         Complot

Complot

PLOT FORMATS                                                       Complot

------------                                                       Complot

THE TWO CROSS SECTIONS ARE CONSIDERED TO BE A STANDARD (THE FIRST  Complot

CROSS SECTION) AND A CROSS SECTION TO BE COMPARED TO THE STANDARD  Complot

(THE SECOND CROSS SECTION). THE OUTPUT FROM THIS PROGRAM IS A      Complot

SERIES OF PLOTS. EACH PLOT WILL CONTAIN THE STANDARD CROSS SECTION Complot

AND IN ADDITION THE USER MAY SPECIFY THAT EACH PLOT ALSO CONTAIN   Complot

THE SECOND CROSS SECTION AND/OR THE RATIO OF THE SECOND CROSS      Complot

SECTION TO THE FIRST CROSS SECTION.                                Complot

Complot

THE USER MAY SELECT ONE OF THE FOLLOWING FIVE PLOT FORMATS (THE    Complot

NUMBER PRECEDING THE OPTION IS THE VALUE OF THE PLOT MODE SELECTOR Complot

THAT THE USER SHOULD SPECIFY AS INPUT ON THE FIRST LINE).          Complot

Complot

(0) THE STANDARD CROSS SECTION (I.E. FIRST EVALUATION) AND THE     Complot

RATIO OF THE SECOND EVALUATION TO THE FIRST EVALUATION. THE    Complot

DATA WILL BE PRESENETED AS TWO SUB-PLOTS PER PLOT WITH THE     Complot

STANDARD CROSS SECTION IN THE UPPER HALF OF THE PLOT AND THE   Complot

RATIO IN THE LOWER HALF OF THE PLOT.                           Complot

Complot

(1) THE STANDARD CROSS SECTION (I.E. FIRST EVALUATION) AND THE     Complot

SECOND EVALUATION. THE DATA WILL BE PRESENTED AS TWO SUB-PLOTS Complot

PER PLOT WITH THE STANDARD CROSS SECTION ON THE UPPER HALF     Complot

OF THE PLOT AND THE SECOND CROSS SECTION IN THE LOWER HALF OF  Complot

THE PLOT.                                                      Complot

Complot

(2) THE STANDARD CROSS SECTION (I.E. FIRST EVALUATION) AND THE     Complot

SECOND EVALUATION. THE DATA WILL BE PRESENTED AS ONE PLOT      Complot

CONTAINING BOTH THE STANDARD AND SECOND CROSS SECTION. THE     Complot

STANDARD CROSS SECTION WILL BE PRESENTED AS A SOLID LINE AND   Complot

THE SECOND CROSS SECTION WILL BE PRESENTED AS A DASHED LINE.   Complot

Complot

(3) THE STANDARD CROSS SECTION, SECOND CROSS SECTION AND RATIO OF  Complot

THE SECOND CROSS SECTION TO THE FIRST CROSS SECTION. THE DATA  Complot

WILL BE PRESENTED AS THREE SUB-PLOTS PER PLOT WITH THE         Complot

STANDARD CROSS SECTION IN THE UPPER THIRD OF THE PLOT, THE     Complot

SECOND CROSS SECTION IN THE MIDDLE THIRD AND THE RATIO OF THE  Complot

TWO IN THE LOWER THIRD OF THE PLOT (RECOMMENDED OPTION).       Complot

Complot

(4) THE STANDARD CROSS SECTION, SECOND CROSS SECTION AND RATIO OF  Complot

THE SECOND CROSS SECTION TO THE FIRST CROSS SECTION. THE DATA  Complot

WILL BE PRESENTED AS TWO SUB-PLOTS PER PLOT WITH THE STANDARD  Complot

AND SECOND CROSS SECTION ON THE SAME SUB-PLOT IN THE UPPER     Complot

TWO THIRDS OF THE PLOT AND THE RATIO OF THE TWO IN THE LOWER   Complot

THIRD OF THE PLOT. THE STANDARD CROSS SECTION WILL BE          Complot

PRESENTED AS A SOLID LINE AND THE SECOND CROSS SECTION WILL BE Complot

PRESENTED AS A DASHED LINE.                                    Complot

Complot

ADDITIONAL PLOT FEATURES                                           Complot

------------------------                                           Complot

IN ADDITION TO THE CROSS SECTIONS AND/OR RATIO THE FOLLOWING       Complot

INFORMATIONS WILL BE INCLUDED ON EACH PLOT.                        Complot

Complot

(1) AN IDENTIFICATION FOR EACH SET OF CROSS SECTIONS (UP TO 30     Complot

CHARACTERS FOR EACH SET).                                      Complot

Complot

(2) THE MAXIMUM NEGATIVE AND POSITIVE PER-CENT DIFFERENCE BETWEEN  Complot

THE TWO CROSS SECTIONS.                                        Complot

Complot

(3) ARROWS INDICATING THE ENERGY AT WHICH THE MAXIMUM DIFFERENCES  Complot

(MINIMUM AND MAXIMUM RATIO) OCCUR.                             Complot

Complot

(4) THE ENERGY LIMITS OF THE RESOLVED AND UNRESOLVED RESONANCE     Complot

REGION (IF THEY FALL WITHIN THE ENERGY LIMITS OF THE PLOT).    Complot

Complot

RATIO DATA                                                         Complot

----------                                                         Complot

IF RATIO OUTPUT IS REQUESTED THE RATIO WILL BE DEFINED AT EACH     Complot

ENERGY THAT APPEARS IN EITHER EVALUATION. BETWEEN THESE ENERGIES   Complot

THE RATIO WILL BE PLOTTED ASSUMING LINEAR DEPENDENCE BETWEEN       Complot

TABULATED VALUES. FOR HISTOGRAM OR LINEARLY INTERPOLABLE CROSS     Complot

SECTIONS THIS REPRESENTATION WILL POINT OUT ALL EXTREMA OF THE     Complot

RATIO, BUT NOT NECESSARILY THE ENERGY DEPENDENCE BETWEEN TABULATED Complot

VALUES.                                                            Complot

Complot

IF THE EVALUATED DATA IS NOT IN EITHER HISTOGRAM OR LINRARLY       Complot

INTERPOLABLE FORM THE RATIO MAY NOT EVEN FIND ALL EXTREMA. FOR     Complot

EXAMPLE, IF ONE EVALUATION IS LINEARLY INTERPOLABLE AND THE        Complot

OTHER NON-LINEAR, BUT BOTH AGREE AT ALL TABULATED ENERGIES THE     Complot

RATIO WILL APPEAR TO BE EQUAL TO UNITY AT ALL ENERGIES, BUT IN     Complot

FACT THE CROSS SECTION BETWEEN TABULATED ENERGIES MAY BE QUITE     Complot

DIFFERENT USING LINEAR VS. NON-LINEAR INTERPOLATION. FOR THIS      Complot

REASON ONLY LINEARLY INTERPOLABLE OR HISTOGRAM DATA IS ALLOWED     Complot

AS INPUT TO THIS PROGRAM.                                          Complot

Complot

LINEAR INTERPOLABLE                                                Complot

-------------------                                                Complot

ALL CROSS SECTIONS MAY BE CONVERTED TO LINEARLY INTERPOLABLE FORM  Complot

BE USING PROGRAM LINEAR (UCRL-50400, VOL. 17, PART A).             Complot

Complot

HISTOGRAM                                                          Complot

---------                                                          Complot

ALL LINEARLY INTERPOLABLE CROSS SECTION MAY BE CONVERTED TO        Complot

HISTOGRAM (I.E. MULTIGROUP) FORM BY USING PROGRAM GROUPIE          Complot

(UCRL-50400, VOL. 17, PART D).                                     Complot

Complot

INPUT UNITS                                                        Complot

-----------                                                        Complot

UNIT  DESCRIPTION                                                  Complot

----  -----------                                                  Complot

2   INPUT LINE                                                   Complot

9   MT DEFINITIONS.                                              Complot

10   FIRST ENDF/B FORMATTED EVALUATION (STANDARD).                Complot

11   SECOND ENDF/B FORMATTED EVALUATION.                          Complot

17   SOFTWARE CHARACTERS.                                         Complot

18   SOFTWARE SYMBOLS AND LINE TYPES                              Complot

Complot

OUTPUT UNITS                                                       Complot

------------                                                       Complot

UNIT  DESCRIPTION                                                  Complot

----  -----------                                                  Complot

3   NORMAL OUTPUT REPORT.                                        Complot

16   PLOTTER UNIT                                                 Complot

Complot

SCRATCH UNITS                                                      Complot

-------------                                                      Complot

UNIT  DESCRIPTION                                                  Complot

----  -----------                                                  Complot

12   SCRATCH UNIT FOR FIRST EVALUATION                            Complot

13   SCRATCH UNIT FOR SECOND EVALUATION                           Complot

14   SCRATCH UNIT FOR RATIO (ONLY USED IF RATIOS REQUESTED).      Complot

Complot

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Complot

---------------------------------------------------------------    Complot

UNIT  FILE NAME                                                    Complot

----  ----------                                                   Complot

2   COMPLOT.INP                                                  Complot

3   COMPLOT.LST                                                  Complot

9   MT.DAT                                                       Complot

10   ENDFB.IN1    (OR AS READ FROM INPUT)                         Complot

11   ENDFB.IN2    (OR AS READ FROM INPUT)                         Complot

12-14  (SCRATCH)                                                    Complot

15   PLOT.CHR                                                     Complot

16   (PLOTTER UNIT...USUALLY A DUMMY)                             Complot

Complot

INPUT PARAMETERS                                                   Complot

------------------------------------------------------------------ Complot

LINE  COLUMNS  FORMAT  DESCRIPTION                                 Complot

----  -------  ------  -----------                                 Complot

1     1-11    E11.4  LOWER X LIMIT OF PLOTTER                    Complot

12-22    E11.4  UPPER X LIMIT OF PLOTTER                    Complot

23-33    E11.4  LOWER Y LIMIT OF PLOTTER                    Complot

34-44    E11.4  UPPER Y LIMIT OF PLOTTER                    Complot

45-55     I11   NUMBER OF PLOTS PER FRAME IN X DIRECTION    Complot

56-66     I11   NUMBER OF PLOTS PER FRAME IN Y DIRECTION    Complot

67-70     F4.1  CHARACTER SIZE MULTIPLIER                   Complot

= 0 TO 1    - NORMAL CHARACTER SIZE         Complot

= OTHERWISE - CHARACTERS SCALED BY THIS     Complot

FACTOR                        Complot

Complot

PLOT ORIENTATION IS BASED ON THE UPPER X    Complot

LIMIT                                       Complot

= .GT.0 - X HORIZONTAL/Y VERTICAL           Complot

= .LT.0 - Y HORIZONTAL/X VERTICAL           Complot

AFTER TESTING THE UPPER X LIMIT WILL BE     Complot

SET TO ITS ABSOLUTE VALUE.                  Complot

2     1-72     A60   FILENAME FOR FIRST ENDF/B DATA FILE         Complot

(LEAVE BLANK FOR ENDFB.IN1)                 Complot

3     1-72     A60   FILENAME FOR SECOND ENDF/B DATA FILE        Complot

(LEAVE BLANK FOR ENDFB.IN2)                 Complot

4     1-11     I11   RETRIEVAL MODE (0=MAT, 1=ZA)                Complot

12-22     I11   GRID (SPEED) OPTION.                        Complot

= 0 - TICK MARKS ON BORDER                  Complot

= 1 - SOLID AT COARSE INTERVALS             Complot

= 2 - DASHED AT COARSE INTERVALS            Complot

= 3 - SOLID AT COARSE AND FINE INTERVALS    Complot

= 4 - DASHED AT COARSE AND FINE INTERVALS   Complot

= 5 - SOLID COARSE/DASHED FINE INTERVALS    Complot

23-33     I11   SHOULD BORDER BE PLOTTED AROUND EACH PLOT   Complot

= 0 - NO                                    Complot

= 1 - YES                                   Complot

34-44     I11   LINE THICKNESS                              Complot

= 0 TO  5 - LINES AND CHARACTERS            Complot

=-1 TO -5 - ONLY LINES                      Complot

45-55     I11   OUTPUT MODE                                 Complot

=-1 - ONLY COMPARISON LISTING. NO PLOTS.    Complot

= 0 - CROSS SECTION OVER RATIO.             Complot

= 1 - CROSS SECTION OVER CROSS SECTION.     Complot

= 2 - TWO CROSS SECTIONS ON SAME PLOT.      Complot

= 3 - CROSS SECTION OVER CROSS SECTION OVER Complot

RATIO.                                Complot

= 4 - TWO CROSS SECTIONS ON SAME PLOT OVER  Complot

RATIO.                                Complot

56-66     I11   STARTING PLOT NUMBER                        Complot

= 0     - DO NOT NUMBER PLOTS               Complot

= .GT.0 - NUMBER PLOTS IN LOWER LEFT HAND   Complot

CORNER STARTING WITH INPUT NUMBER Complot

67-70     I41   BACKGROUND COLOR                            Complot

= 0         = BLACK                         Complot

= OTHERWISE = WHITE                         Complot

5    1-11    E11.4  ALLOWABLE FRACTIONAL DIFFERENCE. USED WHEN  Complot

PLOTTING RATIOS. ANY REACTION WHERE THE     Complot

TWO EVALUATIONS DIFFER BY MORE THAN THE     Complot

ALLOWABLE DIFFERENCE WILL BE PLOTTED. IF    Complot

ZERO IS INPUT THE STANDARD ALLOWABLE        Complot

DIFFERENCE OF 0.001 (0.1 PER-CENT) WILL BE  Complot

USED.                                       Complot

12-22    E11.4  MAXIMUM ALLOWABLE RATIO. IF RATIOS ARE      Complot

PLOTTED THEY WILL BE IN THE RANGE RATMAX    Complot

TO 1/RATMAX. IF 0.0 IS INPUT THERE WILL     Complot

BE NO LIMIT ON THE RANGE OF THE RATIOS.     Complot

THIS OPTION MAY BE USED TO IGNORE LARGE     Complot

DIFFERENCES OVER VERY NARROW ENERGY RANGES  Complot

(WHICH MAY BE UNIMPORTANT) AND ALLOW ONE    Complot

TO SEE IMPORTANT, BUT SMALLER DIFFERENCES,  Complot

OVER EXTENDED ENERGY RANGES.                Complot

6     1-40     40A1  IDENTIFICATION FOR UPPER EVALUATIONS        Complot

7     1-40     40A1  IDENTIFICATION FOR LOWER EVALUATIONS        Complot

(IDENTIFICATIONS SHOULD BE LEFT ADJUSTED    Complot

TO START IN COLUMN 1).                      Complot

8-N     1- 6      I6   LOWER MAT OR ZA LIMIT (SEE SELECTION MODE,  Complot

INPUT LINE 1, COLUMNS 1-11).                Complot

7- 8      I2   LOWER MF LIMIT                              Complot

9-11      I3   LOWER MT LIMIT                              Complot

12-22    E11.4  LOWER ENERGY LIMIT                          Complot

23-28      I6   UPPER MAT OR ZA LIMIT (SEE SELECTION MODE,  Complot

INPUT LINE 1, COLUMNS 1-11).                Complot

29-30      I2   UPPER MF LIMIT                              Complot

31-33      I3   UPPER MT LIMIT                              Complot

34-44    E11.4  UPPER ENERGY LIMIT                          Complot

45-55     I11   IDENTIFY EVALUATED DATA POINTS OPTION.      Complot

= 0 - DO NOT IDENTIFY DATA POINTS.          Complot

= 1 - IDENTIFY DATA POINTS (BY DRAWING A    Complot

SMALL BOX AROUND EACH POINT).         Complot

56-66     I11   INTERACTIVE INPUT FLAG                      Complot

= 0 - NO INTERACTIVE INPUT ALLOWED          Complot

= 1 - INTERACTIVE INPUT ALLOWED             Complot

*SETTING THIS OPTION =1 WILL TURN ON THE     Complot

MOUSE AFTER EACH PLOT AND ALLOW YOU TO      Complot

INTERACTIVELY SPECIFY PLOT LIMITS.          Complot

*IF YOU DO NOT WISH TO INTERACT WITH A PLOT  Complot

OR IF YOU HAVE NO INTERACTIVE CAPABILITY    Complot

THIS OPTION SHOULD BE SET = 0.              Complot

Complot

*WARNING...DATA POINTS IDENTIFIED OPTION IS  Complot

NOT RECOMMENDED FOR PLOTS CONTAINING MANY   Complot

(I.E. THOUSANDS) OF DATA POINTS SINCE IT    Complot

WILL MERELY INCREASE THE RUNNING TIME OF    Complot

THE PROGRAM AND STILL NOT ALLOW ONE TO      Complot

ACCURATELY SEE DATA POINTS.                 Complot

Complot

*UP TO 100 MAT OR ZA RANGES ARE ALLOWED.     Complot

THE LIST IS TERMINATED BY A BLANK LINE.     Complot

IF THE UPPER LIMIT IS LESS THAN THE LOWER   Complot

LIMIT IT WILL BE SET EQUAL TO THE LOWER     Complot

LIMIT. IF THE FIRST RANGE LINE IS BLANK     Complot

ALL DATA WILL BE RETRIEVED. IF THE UPPER    Complot

MT LIMIT IS ZERO IT WILL BE SET EQUAL TO    Complot

999 (NO LIMIT). IF THE UPPER ENERGY LIMIT   Complot

IS ZERO IT WILL BE INTREPRETED TO MEAN NO   Complot

LIMIT. IF THE FIRST RANGE LINE SPECIFIES    Complot

ZERO LOWER AND UPPER MAT OR ZA RANGE IT     Complot

WILL TERMINATE THE LIST BE RANGE LINES      Complot

(A SECOND BLANK LINE NEED NOT BE INPUT)     Complot

AND THE ENTIRE RANGE OF MATS WILL BE        Complot

COMPARED FOR THE SPECIFIED MT AND ENERGY    Complot

RANGES.                                     Complot

Complot

N+1-M                    EQUIVALENCES                                Complot

1- 6      I6   MASTER ZA.                                  Complot

7- 8      I2   MASTER MF.                                  Complot

9-11      I3   MASTER MT.                                  Complot

12-17      I6   EQUIVALENT ZA FROM SECOND FILE.             Complot

18-19      I2   EQUIVALENT MF FROM SECOND FILE.             Complot

20-22      I3   EQUIVALENT MT FROM SECOND FILE.             Complot

23-33    E11.4  MULTIPLICATION FACTOR. ANY EQUATED ZA,MF,   Complot

MT DATA WILL BE MULTIPLIED BY THIS FACTOR.  Complot

*THIS OPTION MAY BE USED TO RE-NORMALIZE     Complot

THE SECOND CROSS SECTION OR IF COMPARING    Complot

ONE CONSTITUENT OF A MIXTURE TO THE MIXED   Complot

CROSS SECTION THIS MAY BE USED TO CONVERT   Complot

THE SECOND CROSS SECTION TO BARNS PER MIXED Complot

ATOM BY USING A MULTIPLICATION FACTOR WHICH Complot

IS EQUAL TO THE NUMBER OF ATOMS OF THE ONE  Complot

CONSTITUENT PER ATOM OF THE MIXTURE.        Complot

= 0.0 - ON INPUT WILL BE INTERPRETED AS 1.0 Complot

(WITH THIS CONVENTION THE USER NEED ONLY    Complot

INPUT MULTIPLICATION FACTORS IF THEY ARE    Complot

NOT 1.0).                                   Complot

*UP TO 100 MAT OR ZA EQUIVALENCES ARE        Complot

ALLOWED.                                    Complot

*THE LIST IS TERMINATED BY A BLANK LINE.     Complot

*A ZERO INPUT FIELD IMPLIES ALL. TO EQUATE   Complot

A GIVEN MT NUMBER TO ANOTHER MT NUMBER YOU  Complot

NEED MERELY SPECIFY ZA=0 ON INPUT.          Complot

*NOTE, IN ALL CASES THE TITLE AT TOP OF PLOT Complot

WILL ONLY INDENTIFY MASTER (ZA,MF,MT). THE  Complot

USER INPUT TITLES MUST BE USED TO IDENTIFY  Complot

THE SECOND REACTION (SEE, EXAMPLE INPUT 4   Complot

BELOW).                                     Complot

Complot

Complot

EXAMPLE DEFINITION OF PLOTTER                                      Complot

-----------------------------                                      Complot

THE FIRST INPUT LINE DEFINES THE DIMENSIONS OF THE PLOTTER BEING   Complot

USED IN ANY UNITS (INCHES, CENTIMETERS, MILLIMETERS, ANYTHING)     Complot

WHICH APPLY TO THE PLOTTER. IN ADDITION THE FIRST LINE DEFINES     Complot

HOW MANY PLOTS SHOULD APPEAR ON EACH FRAME. THE PLOTTING AREA      Complot

DEFINED ON THE FIRST INPUT LINE MAY BE SUBDIVIDED INTO ANY NUMBER  Complot

OF PLOTS IN THE X AND Y DIRECTION. FOR EXAMPLE, TO PRODUCE A       Complot

SERIES OF FRAMES EACH CONTAINING 3 PLOTS IN THE X DIRECTION AND    Complot

2 PLOTS IN THE Y DIRECTION (6 PLOTS PER FRAME) COLUMN 45-55 OF     Complot

THE FIRST INPUT LINE SHOULD BE 3 AND COLUMNS 56-66 SHOULD BE 2.    Complot

Complot

IF THE LOCAL PLOTTER USES DIMENSIONS OF INCHES IN ORDER TO OBTAIN  Complot

10 X 10 INCH FRAMES WITH 3 X 2 PLOTS PER FRAME THE FIRST INPUT     Complot

LINE SHOULD BE,                                                    Complot

Complot

0.0       10.0      0.0     10.0              3          2       Complot

Complot

IF THE LOCAL PLOTTER USES DIMENSION OF MILLIMETERS THE SAME        Complot

PHYSICAL SIZE PLOT MAY BE OBTAINED IF THE FIRST INPUT LINE IS,     Complot

Complot

0.0      254.0      0.0    254.0              3          2       Complot

Complot

FOR SIMPLICITY THE FOLLOWING EXAMPLE INPUTS WILL NOT DISCUSS THE   Complot

PHYSICAL DIMENSIONS OF THE PLOTTER AND THE FIRST INPUT LINE WILL   Complot

IN ALL CASES INDICATE 10 X 10 INCH PLOTS WITH ONLY 1 PLOT PER      Complot

FRAME.                                                             Complot

Complot

IN THE FOLLOWING EXAMPLES IN ALL CASES THESE OPTIONS WILL BE USED, Complot

1) DASHED GRID       - COLUMNS 12-22 OF SECOND INPUT LINE =  1     Complot

2) NO BORDER         - COLUMNS 23-33 OF SECOND INPUT LINE =  0     Complot

3) LINE THICKNESS    - COLUMNS 34-44 OF SECOND INPUT LINE = -2     Complot

4) OUTPUT MODE       - COLUMNS 45-55 OF SECOND INPUT LINE =  3     Complot

5) FIRST PLOT NUMBER - COLUMNS 56-66 OF SECOND INPUT LINE =  1     Complot

Complot

EXAMPLE INPUT 1                                                    Complot

---------------                                                    Complot

RETRIEVE MATS 1023, 1056 AND 1065 THROUGH 1072, MT = 1 AND 2       Complot

(TOTAL AND ELASTIC) FROM THE FIRST INPUT FILE AND COMPARE TO       Complot

ANY SECTION FROM THE SECOND FILE THAT HAS THE SAME ZA/MF/MT. ONLY  Complot

COMPARE DATA OVER THE ENERGY RANGE 0.1 EV TO 1 KEV. IDENTIFY       Complot

THE TWO SETS OF DATA AS ENDF/B-V AND ENDF/B-IV, RESPECTIVELY.      Complot

ONLY PLOT THOSE REACTIONS WHICH DIFFER AT ONE OR MORE ENERGIES     Complot

BY MORE THAN 1 PER-CENT (NOTE, 1 PER-CENT = 0.01 AS INPUT          Complot

FRACTION). NO EQUIVALENT REACTIONS ARE SPECIFIED. FILERNAMES       Complot

ARE STANDARD (THSE CAN EITHER BE EXPLICITLY INCLUDED, OR SIMPLY    Complot

LEFT BLANK).                                                       Complot

Complot

THE FOLLOWING 12 INPUT LINES ARE REQUIRED.                         Complot

Complot

0.0       10.0      0.0     10.0              3          2      Complot

ENDFB.IN1                                                             Complot

ENDFB.IN2                                                             Complot

0          1        0       -2              3          1      Complot

0.01        0.0                                                  Complot

ENDF/B-V DATA (STANDARD)                                              Complot

ENDF/B-IV DATA                                                        Complot

1023 3  1 0.1            3  2 1000.0                0                 Complot

1056 3  1 0.1            3  2 1000.0                0                 Complot

1065 3  1 0.1       1072 3  2 1000.0                0                 Complot

(TERMINATES REQUEST LIST)        Complot

(TERMINATES EQUIVALENCE LIST)    Complot

Complot

EXAMPLE INPUT 2                                                    Complot

---------------                                                    Complot

TO USE ALL OF THE SAME OPTIONS AS SPECIFIED IN EXAMPLE INPUT 1,    Complot

EXCEPT TO RETRIEVE U-235, U-238 AND PU-239 THROUGH PU-242 THE      Complot

FOLLOWING 12 INPUT LINES ARE REQUIRED.                             Complot

Complot

0.0       10.0      0.0     10.0              3          2      Complot

ENDFB.IN1                                                             Complot

ENDFB.IN2                                                             Complot

1          1        0       -2              3          1      Complot

0.01        0.0                                                  Complot

ENDF/B-V DATA (STANDARD)                                              Complot

ENDF/B-IV DATA                                                        Complot

92235 3  1 0.1            3  2 1000.0                0                 Complot

92238 3  1 0.1            3  2 1000.0                0                 Complot

94239 3  1 0.1      94242 3  2 1000.0                0                 Complot

(TERMINATES REQUEST LIST)        Complot

(TERMINATES EQUIVALENCE LIST)    Complot

EXAMPLE INPUT 3                                                    Complot

---------------                                                    Complot

TO USE ALL OF THE SAME OPTIONS AS SPECIFIED IN EXAMPLE INPUT 1,    Complot

EXCEPT TO RETRIEVE AND COMPARE ALL MATS THE FOLLOWING 10 INPUT     Complot

LINES ARE REQUIRED.                                                Complot

Complot

0.0       10.0      0.0     10.0              3          2       Complot

ENDFB.IN1                                                             Complot

ENDFB.IN2                                                             Complot

0          1        0       -2              3          1       Complot

0.01        0.0                                                   Complot

ENDF/B-V DATA (STANDARD)                                               Complot

ENDF/B-IV DATA                                                         Complot

1 1  1 0.0       999999999 0.0                   0                 Complot

(TERMINATES REQUEST LIST)        Complot

(TERMINATES EQUIVALENCE LIST)    Complot

NOTE, ZERO LOWER AND UPPER      Complot

MAT LIMITS INDICATES NO LIMIT.  Complot

Complot

EXAMPLE INPUT 4                                                    Complot

---------------                                                    Complot

RETRIEVE U-235 AND EQUATE THE FISSION CROSS SECTION (MT=18) ON     Complot

THE MASTER FILE TO CAPTURE (MT=102) ON THE SECOND FILE. PLOT       Complot

THE CAPTURE, FISSION AND CAPTURE TO FISSION RATIO OVER THE ENERGY  Complot

RANGE 0.0253 EV TO 1 KEV. THE FOLLOWING 11 INPUT LINES ARE         Complot

REQUIRED.                                                          Complot

Complot

0.0       10.0      0.0     10.0              3          2      Complot

ENDFB.IN1                                                             Complot

ENDFB.IN2                                                             Complot

1          1        0       -2              3          1      Complot

0.01        0.0                                                  Complot

FISSION                                                               Complot

CAPTURE                                                               Complot

92235 3 18 0.0253   92235 3 18 1000.0                0                 Complot

(TERMINATES REQUEST LIST)        Complot

92235 3 18 92235 3102                 (MULTIPLICATION OF 1.0 INFERRED) Complot

(TERMINATES EQUIVALENCE LIST)    Complot

Complot

EXAMPLE INPUT 5                                                    Complot

---------------                                                    Complot

IN DIFFERENT VERSIONS OF THE ENDF/B FORMAT DIFFERENT MT NUMBERS    Complot

ARE ASSIGNED TO THE SAME REACTION. FOR EXAMPLE, IN ENDF/B-V AND    Complot

EARLIER VERSIONS OF ENDF/B THE PHOTOELECTRIC CROSS SECTION IS      Complot

MT=602, WHILE IN ENDF/B-VI IT IS MT=522. IN ORDER TO COMPARE       Complot

ASSUMING THAT THE MASTER IS ENDF/B-VI AND THE OTHER ENDF/B FILE    Complot

IS ENDF/B-V (OR EARLIER) YOU MAY EQUATE MT=522 TO 602.             Complot

Complot

WHEN COMPARING PHOTOELECTRIC CROSS SECTIONS WE EXPECT THERE TO BE  Complot

LARGE DIFFERENCES NEAR EDGES, SINCE IT IS UNLIKELY THAT TWO        Complot

INDEPENDENT EVALUATIONS USE EXACTLY THE SAME EDGE ENERGIES. FROM   Complot

A PRACTICAL VIEWPOINT THESE DIFFERENCES ARE NOT IMPORTANT IF THEY  Complot

ONLY OCCUR OVER NARROW ENERGY RANGES NEAR ENERGIES. HOWEVER THESE  Complot

LARGE DIFFERENCES MAY MAKE IT DIFFICULT TO SEE DIFFERENCES OVER    Complot

OTHER ENERGY RANGES, WHICH MAY BE IMPORTANT. IN ORDER TO BE ABLE   Complot

TO SEE IMPORTANT DIFFERENCES IN THE FOLLOWING COMPARISON WE WILL   Complot

CONSTRAIN THE PLOTTED RATIO TO THE RANGE ABOUT 0.9 TO 1.1 IN       Complot

ORDER TO BE ABLE TO SEE DIFFERENCES OF UP TO 10 PER-CENT. WE WILL  Complot

DO THIS BY SPECIFYING A MAXIMUM RATIO OF 1.1, WHICH WILL IN TURN   Complot

DEFINE A MINIMUM RATIO OF 1/1.1, OR ABOUT 0.9.                     Complot

Complot

IN ORDER TO COMPARE THE PHOTOELECTRIC CROSS SECTION FOR ALL        Complot

MATERIALS THE FOLLOWING 11 INPUT LINES ARE REQUIRED.               Complot

Complot

0.0       10.0      0.0     10.0              3          2       Complot

ENDFB.IN1                                                             Complot

ENDFB.IN2                                                             Complot

0          1        0       -2              3          1       Complot

0.01        1.1                                                   Complot

ENDF/B-VI                                                              Complot

ENDF/B-V                                                               Complot

023522           999923522                       0                  Complot

(TERMINATES REQUEST LIST)        Complot

023522     023602                  (MULTIPLICATION OF 1.0 INFERRED) Complot

(TERMINATES EQUIVALENCE LIST)    Complot

Complot

EXAMPLE INPUT 6                                                    Complot

---------------                                                    Complot

THE SAME EXAMPLE AS ABOVE, EXCEPT THAT DIFFERENT FILENAMES WILL    Complot

BE USED TO READ THE DATA FROM A FILE TREE STRUCTURE. THE FOLLOWING Complot

11 INPUT LINES ARE REQUIRED.                                       Complot

Complot

0.0       10.0      0.0     10.0              3          2       Complot

/Evaluated/ENDFB6/PHOTON.IN                                           Complot

/Evaluated/ENDFB5/PHOTON.IN                                           Complot

0          1        0       -2              3          1       Complot

0.01        1.1                                                   Complot

ENDF/B-VI                                                              Complot

ENDF/B-V                                                               Complot

023522           999923522                       0                  Complot

(TERMINATES REQUEST LIST)        Complot

023522     023602                  (MULTIPLICATION OF 1.0 INFERRED) Complot

(TERMINATES EQUIVALENCE LIST)    Complot

Complot

EXAMPLE INPUT 7                                                    Complot

---------------                                                    Complot

THE OUTPUT FOR ALL OF THE ABOVE EXAMPLES ARE ORIENTED WITH X       Complot

HORIZONTAL AND Y VERTICAL. TO CHANGE THE ORIENTATION OF THE PLOTS  Complot

YOU NEED MERELY SPECIFY A NEGATIVE UPPER X LIMIT OF THE SIZE OF    Complot

THE PLOTS ON THE FIRST INPUT LINE.                                 Complot

Complot

THE FOLLOWING EXAMPLE IS EXACTLY THE SAME AS THE ABOVE EXAMPLE,    Complot

EXCEPT THAT THE ORIENTATION OF THE PLOTS HAS BEEN CHANGED. THE     Complot

FOLLOWING 11 INPUT LINES ARE REQUIRED.                             Complot

Complot

0.0      -10.0      0.0     10.0              3          2       Complot

/Evaluated/ENDFB6/PHOTON.IN                                           Complot

/Evaluated/ENDFB5/PHOTON.IN                                           Complot

0          1        0       -2              3          1       Complot

0.01        1.1                                                   Complot

ENDF/B-VI                                                              Complot

ENDF/B-V                                                               Complot

023522           999923522                       0                  Complot

(TERMINATES REQUEST LIST)        Complot

023522     023602                  (MULTIPLICATION OF 1.0 INFERRED) Complot

(TERMINATES EQUIVALENCE LIST)    Complot

Complot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Complot

Complot

NON-INTERACTIVE                                                   Complot

------------------------------------------------------------------ Complot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE INTERFACE INVOLVING       Complot

ONLY 5 SUBROUTINES,                                               Complot

Complot

STARPLOT     - INITIALIZE PLOTTER                                 Complot

NEXTPLOT     - CLEAR SCREEN FOR NEXT PLOT                         Complot

ENDPLOTS     - TERMINATE PLOTTING                                 Complot

Complot

PLOT(X,Y,IPEN)        - DRAW OR MOVE FROM LAST LOCATION TO (X,Y), Complot

END OF CURRENT PLOT OR END OF PLOTTING.   Complot

IPEN =   2 - DRAW                                           Complot

=   3 - MOVE                                           Complot

Complot

PEN(IPEN)             - SELECT COLOR.                             Complot

IPEN- COLOR = 1 TO N (N = ANY POSITIVE INTEGER)               Complot

Complot

BOXCOLOR(X,Y,IFILL,IBORDER) - FILL A RECTANGLE WITH COLOR         Complot

X,Y                = DEFINE THE CORNERS OF THE BOX       Complot

IFILL          = COLOR TO FILL BOX WITH              Complot

IBORDER  = COLOR OF BORDER OF BOX              Complot

Complot

INTERACTIVE                                                       Complot

------------------------------------------------------------------ Complot

THIS PROGRAM INCLUDES AN INTERACTIVE INTERFACE FOR USE WITH A     Complot

MOUSE. THE INTERFACE INVOLVES 2 SUBROUTINE,                       Complot

Complot

INTERACT(MYACTION)             - WHETHER OR NOT INTERACTION       Complot

MYACTION              = 0 - NO (RETURNED BY INTERACT)    Complot

= 1 - YES (RETURNED BY INTERACT)   Complot

Complot

MOUSEY(IWAY,XI,YI,IWAY1,IWAY2) - READ POSITION OF MOUSE           Complot

IWAY    = 0 - NO INPUT                     Complot

= 1 - LEFT BUTTON                  Complot

= 2 - MIDDLE BUTTON                Complot

= 3 - RIGHT BUTTON                 Complot

= 4 - KEYBOARD INPUT               Complot

XI      = X POSITION IN LOCAL UNITS        Complot

YI      = Y POSITION IN LOCAL UNITS        Complot

IWAY1   = MINIMUM ALLOWABLE IWAY           Complot

IWAY2   = MAXIMUM ALLOWABLE IWAY           Complot

Complot

AS USED BY THIS PROGRAM IWAY1   = 1                                Complot

IWAY2   = 4                                Complot

KEYBOARD INPUT (IWAY=4) MEANS NO ZOOMED PLOT REQUESTED.            Complot

MOUSE INPUT (IWAY=1 TO 3) MEANS A ZOOMED PLOT IS REQUESTED.        Complot

MOUSEY WILL BE CALLED ONCE TO SEE IF A ZOOMED PLOT IS REQUESTED.   Complot

IF IT IS XI WILL BE USED TO DEFINE ONE X (E.G., ENERGY) LIMIT OF   Complot

THE ZOOMED PLOT. MOUSEY WILL THEN BE CALLED A SECOND TIME TO       Complot

DEFINE A SECOND XI TO DEFINE THE OTHER X LIMIT OF THE ZOOMED       Complot

PLOT.                                                              Complot

Complot

IF YOU DO NOT WANT INTERACTION YOU SHOULD INCLUDE THE FOLLOWING    Complot

SUBROUTINES IN YOUR GRAPHIC INTERFACE,                             Complot

Complot

SUBROUTINE INTERACT(MYACTION)                                      Complot

MYACTION=0                                                         Complot

RETURN                                                             Complot

END                                                                Complot

SUBROUTINE MOUSEY(IWAY,XI,YI,IWAY1,IWAY2)                          Complot

IWAY=4                                                             Complot

XI=0.0                                                             Complot

YI=0.0                                                             Complot

RETURN                                                             Complot

END                                                                Complot

Complot

ALTERNATIVE INTERACTIVE                                            Complot

------------------------------------------------------------------ Complot

IF YOU DO NOT HAVE A MOUSE BUT WOULD STILL LIKE TO INTERACTIVE     Complot

INPUT YOU CAN REPLACE SUBROUTINE ACTION IN THIS PROGRAM.           Complot

Complot

AS DISTRIBUTED SUBROUTINE ACTION USES A MOUSE TO DEFINE LOWER      Complot

AND UPPER ENERGY (OR X) LIMITS WHICH ARE USED TO PRODUCE THE       Complot

NEXT PLOT. A CALL TO ACTION IS OF THE FORM,                        Complot

Complot

CALL ACTION(KACTV,XACT1,XACT2)                                     Complot

Complot

KACTV   = 0 - NO INTERACTIVE INPUT                     Complot

= 1 - INTERACTIVE INPUT                        Complot

XACT1   = LOWER ENERGY LIMIT                           Complot

XACT2   = UPPER ENERGY LIMIT                           Complot

Complot

IF THERE IS NO INTERACTIVE INPUT THE PROGRAM WILL PROCEED TO THE   Complot

NEXT PLOT REQUESTED BY NON-INTERACTIVE INPUT.                      Complot

Complot

IF THERE IS INTERACTIVE INPUT THE PROGRAM WILL USE XACT1 AND       Complot

XACT2 TO DEFINE THE ENERGY LIMITS OF THE NEXT PLOT USING THE       Complot

SAME DATA AS APPEARED ON THE LAST PLOT. AS WITH NON-INTERACTIVE    Complot

INPUT, IF YOU SELECT AN ENERGY RANGE WHERE THE MAXIMUM DIFFERENCE  Complot

IS LESS THAN THAT SPECIFIED BY INPUT NO PLOT WILL BE PRODUCED      Complot

AND THE CODE WILL PROCEED TO THE NEXT PLOT REQUESTED BY            Complot

NON-INTERACTIVE INPUT.                                             Complot

Complot

YOU CAN REPLACE SUBROUTINE ACTION FOLLOWING THE ABOVE CONVENTIONS  Complot

TO ALLOW INTERACTION VIA DIRECT READ OF X LIMITS, LIGHTPEN OR      Complot

WHATEVER FACILITIES YOU HAVE AVAILABLE.                            Complot

Complot

INTERFACING                                                        Complot

------------------------------------------------------------------ Complot

IN ORDER TO INTERFACE THIS PROGRAM FOR USE ON ANY PLOTTER WHICH    Complot

DOES NOT USE THE ABOVE CONVENTIONS IT IS MERELY NECESSARY FOR THE  Complot

THE USER TO WRITE 5 SUBROUTINES DESCRIBED ABOVE AND TO THEN CALL   Complot

THE LOCAL EQUIVALENT ROUTINES.                                     Complot

Complot

COLOR PLOTS                                                        Complot

------------------------------------------------------------------ Complot

TO SELECT PLOTTING COLORS SUBROUTINE PEN (DESCRIBED ABOVE) IS USED Complot

TO SELECT ONE OF THE AVAILABLE COLORS. WHEN RUNNING ON A MAINFRAME Complot

USING AN IBM GRAPHICS TERMINAL OR ON AN IBM-PC USING A HEWLETT-    Complot

PACKARD PLOTTER THE GRAPHICS INTERFACE (DESCRIBED ABOVE) WILL      Complot

PRODUCE COLOR PLOTS.                                               Complot

Complot

BLACK AND WHITE PLOTS                                              Complot

------------------------------------------------------------------ Complot

WHEN PRODUCING BLACK AND WHITE HARDCOPY ON A MAINFRAME THE USER    Complot

SHOULD ADD A DUMMY SUBROUTINE PEN TO THE END OF THE PROGRAM TO     Complot

IGNORE ATTEMPTS TO CHANGE COLOR. ADD THE FOLLOWING SUBROUTINE,     Complot

Complot

SUBROUTINE PEN(IPEN)                                               Complot

RETURN                                                             Complot

END                                                                Complot

Complot

CHARACTER SET                                                      Complot

------------------------------------------------------------------ Complot

THIS PROGRAM USES COMPUTER AND PLOTTER DEVICE INDEPENDENT SOFTWARE Complot

CHARACTERS. THIS PROGRAM COMES WITH A FILE THAT DEFINES THE PEN    Complot

STROKES REQUIRED TO DRAW ALL CHARACTERS ON AN IBM KEYBOARD (UPPER  Complot

AND LOWER CASE CHARACTERS, NUMBERS, ETC.) PLUS AN ALTERNATE SET OF Complot

ALL UPPER AND LOWER CASE GREEK CHARACTERS AND ADDITIONAL SPECIAL   Complot

SYMBOLS.                                                           Complot

Complot

THE SOFTWARE CHARACTER TABLE CONTAINS X AND Y AND PEN POSITIONS TO Complot

DRAW EACH CHARACTER. IF YOU WISH TO DRAW ANY ADDITIONAL CHARACTERS Complot

OR TO MODIFY THE FONT OF THE EXISTING CHARACTERS YOU NEED ONLY     Complot

MODIFY THIS TABLE.                                                 Complot

Complot

CONTROL CHARACTERS                                                 Complot

------------------------------------------------------------------ Complot

IN THE SOFTWARE CHARACTER TABLE ALL CHARACTERS TO BE PLOTTED WILL  Complot

HAVE PEN POSITION = 2 (DRAW) OR = 3 (MOVE). IN ADDITION THE TABLE  Complot

CURRENTLY CONTAINS 4 CONTROL CHARACTERS,                           Complot

Complot

PEN POSITION = 0                                                   Complot

----------------                                                   Complot

SHIFT THE NEXT PRINTED CHARACTER BY X AND Y. 3 CONTROL CHARACTERS  Complot

ARE PRESENTLY INCLUDED IN THE SOFTWARE CHARACTER TABLE TO ALLOW    Complot

SHIFTING.                                                          Complot

Complot

{   = SHIFT UP (FOR SUPERSCRIPTS..............X= 0.0, Y= 0.5)      Complot

}   = SHIFT DOWN (FOR SUBSCRIPTS..............X= 0.0, Y=-0.5)      Complot

\   = SHIFT LEFT 1 CHARACTER (FOR BACKSPACE...X=-1.0, Y= 0.0)      Complot

Complot

PEN POSITION =-1                                                   Complot

----------------                                                   Complot

SELECT THE NEXT PRINTED CHARACTER FROM THE ALTERNATE CHARACTER     Complot

SET. AT PRESENT THIS CONTROL CHARACTER IS,                         Complot

Complot

]   = SWITCH TO ALTERNATE CHARACTER SET                            Complot

Complot

THESE 4 CONTROL CHARACTERS ARE ONLY DEFINED BY THE VALUE OF THE    Complot

PEN POSITION IN THE SOFTWARE CHARACTER TABLE (I.E., THEY ARE NOT   Complot

HARD WIRED INTO THIS PROGRAM). AS SUCH BY MODIFYING THE SOFTWARE   Complot

CHARACTER TABLE THE USER HAS THE OPTION OF DEFINING ANY CONTROL    Complot

CHARACTERS TO MEET SPECIFIC NEEDS.                                 Complot

Complot

THESE CHARACTERS MAY BE USED IN CHARACTER STRINGS TO PRODUCE       Complot

SPECIAL EFFECTS. FOR EXAMPLE, TO PLOT SUBSCRIPT 5, B, SUPERSCRIPT  Complot

10 USE THE STRING,                                                 Complot

Complot

}5B{1{0                                                            Complot

Complot

TO PLOT B, SUBSCRIPT 5 AND SUPERSCRIPT 10 WITH THE 5 DIRECTLY      Complot

BELOW THE 1 OF THE 10 WE CAN USE THE BACKSPACE CHARACTER TO        Complot

POSITION THE 1 DIRECTLY ABOVE THE 5 USING THE STRING,              Complot

Complot

B}5\{1{0                                                           Complot

Complot

TO PLOT UPPER CASE GREEK GAMMA FOLLOWED BY THE WORD TOTAL (I.E.,   Complot

RESONANCE TOTAL WIDTH) USE THE STRING.                             Complot

Complot

]G TOTAL                                                           Complot

Complot

NOTE, WHEN THESE CONTROL CHARACTERS ARE USED THEY ONLY EFFECT THE  Complot

NEXT 1 PRINTED CHARACTER (SEE, ABOVE EXAMPLE OF PLOTTING SUPER-    Complot

SCRIPT 10 WHERE THE SHIFT UP CONTROL CHARACTER WAS USED BEFORE THE Complot

1 AND THEN AGAIN BEFORE THE 0 AND THE BACKSPACE AND SHIFT UP       Complot

CONTROL CHARACTERS WERE USED IN COMBINATION).                      Complot

Complot

IF THESE 4 CONTROL CHARACTERS ARE NOT AVAILABLE ON YOUR COMPUTER   Complot

YOU CAN MODIFY THE SOFTWARE CHARACTER TABLE TO USE ANY OTHER 4     Complot

CHARACTERS THAT YOU DO NOT NORMALLY USE IN CHARACTER STRINGS (FOR  Complot

DETAILS SEE THE SOFTWARE CHARACTER TABLE).                         Complot

Complot

STANDARD/ALTERNATE CHARACTER SETS                                  Complot

------------------------------------------------------------------ Complot

THE SOFTWARE CHARACTER TABLE CONTAINS 2 SETS OF CHARACTERS WHICH   Complot

ARE A STANDARD SET (ALL CHARACTERS ON AN IBM KEYBOARD) AND AN      Complot

ALTERNATE SET (UPPER AND LOWER CASE GREEK CHARACTERS AND SPECIAL   Complot

CHARACTERS). TO DRAW A CHARACTER FROM THE ALTERNATE CHARACTER SET  Complot

PUT A RIGHT BRACKET CHARACTER (]) BEFORE A CHARACTER (SEE THE      Complot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Complot

CONTROL CHARACTER WILL ONLY EFFECT THE NEXT 1 PLOTTED CHARACTER.   Complot

Complot

SUB AND SUPER SCRIPTS                                              Complot

------------------------------------------------------------------ Complot

TO DRAW SUBSCRIPT PRECEED A CHARACTER BY }. TO DRAW SUPERSCRIPT    Complot

PRECEED A CHARACTER BY { (SEE THE ABOVE EXAMPLE AND THE SOFTWARE   Complot

CHARACTER TABLE FOR DETAILS). THESE CONTROL CHARACTER WILL ONLY    Complot

EFFECT THE NEXT 1 PLOTTED CHARACTER.                               Complot

Complot

BACKSPACING                                                        Complot

------------------------------------------------------------------ Complot

TO BACKSPACE ONE CHARACTER PRECEED A CHARACTER BY \ (SEE, THE      Complot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Complot

CONTROL CHARACTER WILL PERFORM A TRUE BACKSPACE AND WILL EFFECT    Complot

ALL FOLLOWING CHARACTERS IN THE SAME CHARACTER STRING.             Complot

Complot

PLOT DIMENSIONS                                                    Complot

---------------                                                    Complot

ARE DEFINED BY USER INPUT. INTERNALLY THE PROGRAM WILL CREATE A    Complot

PLOT IN APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. DURING        Complot

OUTPUT THE PLOT IS TRANSFORMED TO THE UNITS (INCHES, CENTIMETERS,  Complot

MILLIMETERS, WHATEVER) OF THE PLOTTER BEING USED AND OUTPUT.       Complot

Complot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Complot

======================================================================= Complot

======================================================================= Convert

Convert

PROGRAM CONVERT                                                    Convert

VERSION 75-1 (APRIL 1975)                                          Convert

VERSION 78-1 (JANUARY 1978)                                        Convert

VERSION 80-1 (AUGUST 1980)  IBM VERSION                            Convert

VERSION 80-2 (DECEMBER 1980)                                       Convert

VERSION 82-1 (JANUARY 1982)                                        Convert

VERSION 83-1 (JANUARY 1983)                                        Convert

VERSION 86-1 (JANUARY 1986)*NEW PROGRAM                            Convert

*FORTRAN-77/H VERSION                   Convert

*MULTIPLE INPUT OPTIONS                 Convert

VERSION 88-1 (AUGUST 1988) *OPTION...INTERNALLY DEFINE ALL I/O     Convert

FILE NAMES (SEE, SUBROUTINE FILEIO     Convert

FOR DETAILS).                          Convert

*IMPROVED BASED ON USER COMMENTS.       Convert

*ADDED NAMES OPTION TO TURN ON/OFF      Convert

STANDARD FILE NAMES.                   Convert

*ADDED REWIND OPTION TO TURN ON/OFF     Convert

REWIND AT START OF PROGRAMS.           Convert

*DELETED HARWELL AND JAERI OPTIONS      Convert

(PREVIOUSLY ONLY REQUIRED FOR GRAPHIC  Convert

INTERFACE. NO LONGER REQUIRED).        Convert

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Convert

INSURE PROGRAM WILL NOT DO ANYTHING    Convert

CRAZY.                                 Convert

*IMPROVED BASED ON USER COMMENTS.       Convert

*ADDED LIVERMORE CIVIC COMPILER         Convert

CONVENTIONS.                           Convert

*UPDATED TO USE NEW PROGRAM CONVERT     Convert

KEYWORDS.                              Convert

*ADDED ENDFILE OPTION TO OPTIONALLY     Convert

ALLOW END OF FILE TO BE WRITTEN        Convert

VERSION 91-1 (JUNE 1991)   *ADDED FORTRAN SAVE OPTION              Convert

VERSION 92-1 (JANUARY 1992)*ADDED ACTION OPTION - TO CONTROL       Convert

INTERACTIVE INPUT TO CODES             Convert

*ADDED BLANK DELIMITED KEYWORD INPUT    Convert

(REPLACES EARLIER FIXED FIELD INPUT)   Convert

*WARNING...THE INPUT PARAMETER FORMAT   Convert

HAS BEEN GENERALIZED - FOR DETAILS     Convert

SEE BELOW.                             Convert

VERSION 94-1 (JANUARY 1994)*VARIABLE PROGRAM FILENAMES             Convert

TO ALLOW ACCESS TO FILE STRUCTURES     Convert

(WARNING - INPUT PARAMETER FORMAT      Convert

HAS BEEN CHANGED)                      Convert

*CLOSE ALL FILES BEFORE TERMINATING     Convert

(SEE, SUBROUTINE ENDIT)                Convert

*ADDED KEYWORD CLOSE.                   Convert

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Convert

*IMPROVED COMPUTER INDEPENDENCE        Convert

*ALL DOUBLE PRECISION                  Convert

*ON SCREEN OUTPUT                      Convert

VERSION 99-1 (MARCH 1999)   *GENERAL IMPROVEMENTS BASED ON         Convert

USER FEEDBACK                         Convert

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Convert

USER FEEDBACK                         Convert

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Convert

VERS. 2004-1 (MARCH 2004)   *GENERAL UPDATE                        Convert

VERS. 2007-1 (JAN.  2007)   *GENERAL UPDATE                        Convert

Convert

OWNED, MAINTAINED AND DISTRIBUTED BY                               Convert

------------------------------------                               Convert

THE NUCLEAR DATA SECTION                                           Convert

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Convert

P.O. BOX 100                                                       Convert

A-1400, VIENNA, AUSTRIA                                            Convert

EUROPE                                                             Convert

Convert

ORIGINALLY WRITTEN BY                                              Convert

------------------------------------                               Convert

DERMOTT E. CULLEN                                                  Convert

UNIVERSITY OF CALIFORNIA                                           Convert

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Convert

L-159                                                              Convert

P.O. BOX 808                                                       Convert

LIVERMORE, CA 94550                                                Convert

U.S.A.                                                             Convert

TELEPHONE  925-423-7359                                            Convert

E. MAIL    CULLEN1@LLNL.GOV                                        Convert

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Convert

Convert

AUTHORS MESSAGE                                                    Convert

---------------                                                    Convert

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENATION    Convert

FOR THIS PROGRAM INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ    Convert

ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY THE      Convert

COMMENTS CONCERNING COMPUTER DEPENDENT CODING.                     Convert

Convert

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Convert

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Convert

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Convert

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Convert

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Convert

IMPROVE THIS PROGRAM. IN PARTICULAR IF YOUR FORTRAN COMPILER, OR   Convert

COMPUTER HAS A SET OF REQUIREMENTS THAT ARE DIFFERENT FROM THOSE   Convert

OF CDC, CRAY OR IBM PLEASE NOTIFY THE AUTHOR AND THIS PROGRAM WILL Convert

BE MODIFIED TO CONSIDER YOUR COMPUTER SEPERATELY. HOWEVER, IN      Convert

ORDER TO PREVENT A PROLIFERATION OF CODING IT IS IMPERATIVE THAT   Convert

YOU IDENTIFY EXACTLY HOW YOUR FORTRAN COMPILER OR COMPUTER DIFFERS Convert

FROM THOSE ALREADY CONSIDERED BY THIS PROGRAM. HOPEFULLY,IN THIS   Convert

WAY FUTURE VERSIONS OF THIS PROGRAM WILL BE COMPLETELY COMPATIBLE  Convert

FOR USE ON YOUR COMPUTER.                                          Convert

Convert

PURPOSE                                                            Convert

-------                                                            Convert

THIS PROGRAM IS DESIGNED TO AUTOMATICALLY CONVERT FORTRAN PROGRAMS Convert

FOR USE ON ANY ONE OF A VARIETY OF,                                Convert

(1) COMPUTERS                                                      Convert

(2) COMPILERS                                                      Convert

(3) PRECISIONS (SINGLE OR DOUBLE PRECISION)                        Convert

(4) INSTALLATIONS                                                  Convert

(5) STANDARD OR NON-STANDARD FILE NAMES                            Convert

Convert

FORTRAN CODING CONVENTIONS                                         Convert

--------------------------                                         Convert

THIS PROGRAM MAY BE USED TO CONVERT ANY PROGRAM WHICH USES THE     Convert

FOLLOWING CONVENTIONS.                                             Convert

Convert

ALL FORTRAN STATEMENTS THAT DEPEND ON ANY COMBINATION OF COMPUTER, Convert

COMPILER, PRECISION AND/OR INSTALLATION AND STANDARD FILE NAMES    Convert

SHOULD BE PRECEDED AND FOLLOWED BY A COMMENT LINE THAT CONTAINS,   Convert

Convert

C***** DOUBLE ****** ACTIVATE DOUBLE PRECISION (DEFAULT)           Convert

C***** SINGLE ****** ACTIVATE SINGLE PRECISION                     Convert

C***** CHARACTER *** TREAT CHARACTER ARRAYS AS CHARACTERS(DEFAULT) Convert

C***** INTEGER ***** TREAT CHARACTER ARRAYS AS INTEGERS            Convert

C***** STOP ******** ACTIVATE STOP TO TERMINATE PROGRAM            Convert

C***** EXIT ******** ACTIVATE EXIT TO TERMINATE PROGRAM            Convert

C***** PROGRAM ***** ACTIVATE PROGRAM LINE AND CONTINUATIONS       Convert

C***** NAMES ******* ACTIVATE STANDARD FILENAMES                   Convert

C***** REWIND ****** ACTIVATE REWIND FILES AT START OF PROGRAM     Convert

C***** ENDFILE ***** ACTIVATE ENDFILE AT END OF PROGRAM            Convert

C***** CIVIC ******* ACTIVATE LIVERMORE CIVIC COMPILER CONVENTIONS Convert

C***** NOID ******** REMOVE LINE ID IN COLUMNS 73-80 (73-80=BLANK) Convert

C***** SAVE ******** SAVE VARIABLES BETWEEN SUBROUTINE CALLS       Convert

C***** ACTION ****** ACTIVATE INTERACTIVE INPUT TO CODES           Convert

C***** CLOSE ******* ACTIVATE CLOSE ALL FILES BEFORE TERMINATING   Convert

Convert

IF THE USER DOES NOT SELECT,                                       Convert

(1) DOUBLE OR SINGLE      - THE PROGRAM WILL ACTIVATE DOUBLE       Convert

(2) CHARACTER OR INTEGER  - THE PROGRAM WILL ACTIVATE CHARACTER    Convert

(3) STOP OR EXIT          - THE PROGRAM WILL ACTIVATE STOP         Convert

Convert

IF THE USER SELECTS,                                               Convert

(1) DOUBLE AND SINGLE     - THE PROGRAM WILL ACTIVATE DOUBLE       Convert

(2) CHARACTER AND INTEGER - THE PROGRAM WILL ACTIVATE CHARACTER    Convert

(3) STOP AND EXIT         - THE PROGRAM WILL ACTIVATE STOP         Convert

Convert

IF THE USER DOES NOT SELECT PROGRAM, NAMES, REWIND, ENDFILE,       Convert

CIVIC, NOID, SAVE OR ACTION THESE OPTIONS WILL BE TURNED OFF.      Convert

Convert

WHERE CODING IS COMPUTER OR COMPILER DEPENDENT CODING WILL BE      Convert

PRESENT FOR ALL POSSIBLE OPTIONS. THIS PROGRAM WILL ALLOW THE      Convert

THE USER TO CONVERT PROGRAMS FOR USE WITH ANY COMBINATION OF       Convert

OPTIONS. FOR EXAMPLES OF HOW THIS CONVENTION IS USED SEE THE       Convert

LISTING OF THIS PROGRAM AND THE COMMENTS BELOW ON COMPUTER         Convert

DEPENDENT CODING.                                                  Convert

Convert

INPUT LINES                                                        Convert

-----------                                                        Convert

LINE   COLS.  DESCRIPTION                                        Convert

----   -----  --------------------------------------             Convert

1   1-72   BLANK DELIMITED KEYWORDS                           Convert

2   1-60   ENDF/B INPUT DATA FILENAME                         Convert

(STANDARD OPTION = ENDFB.IN)                       Convert

3   1-60   ENDF/B OUTPUT DATA FILENAME                        Convert

(STANDARD OPTION = ENDFB.OUT)                      Convert

Convert

*THE FIRST INPUT LINE IS 72 CHARACTERS.                             Convert

*KEYWORDS MAY BE LOCATED ANYWHERE WITHIN THESE 72 CHARACTERS        Convert

*THERE MAY BE ANY NUMBER OF KEYWORDS INPUT                          Convert

*EACH KEYWORD MUST BE BLANK DELIMITED, E.G., DOUBLE CHARACTER       Convert

IS LEGAL INPUT - DOUBLECHARACTER IS NOT LEGAL INPUT.               Convert

*THERE MUST BE ONE OR MORE BLANKS BETWEEN KEYWORDS                  Convert

Convert

*NOTE, THIS NEW INPUT PARAMETER FORMAT (VERSION 92-1) IS COMPLETELY Convert

COMPATIBLE WITH THE OLDER FIXED FIELD FORMAT. SO THAT IF YOU HAVE  Convert

INPUT THAT YOU HAVE USED IN THE PAST YOU CAN CONTINUE TO USE IT.   Convert

Convert

LEGAL KEYWORDS INCLUDE,                                            Convert

Convert

DOUBLE        ACTIVATE DOUBLE PRECISION (DEFAULT)                  Convert

SINGLE        ACTIVATE SINGLE PRECISION                            Convert

CHARACTER     TREAT CHARACTER ARRAYS AS CHARACTERS(DEFAULT)        Convert

INTEGER       TREAT CHARACTER ARRAYS AS INTEGERS                   Convert

PROGRAM       ACTIVATE PROGRAM LINE AND CONTINUATIONS              Convert

NAMES         ACTIVATE STANDARD FILENAMES                          Convert

REWIND        ACTIVATE REWIND FILES AT START OF PROGRAM            Convert

ENDFILE       ACTIVATE ENDFILE AT END OF PROGRAM                   Convert

CIVIC         ACTIVATE LIVERMORE CIVIC COMPILER CONVENTIONS        Convert

NOID          REMOVE LINE ID IN COLUMNS 73-80 (73-80=BLANK)        Convert

SAVE          SAVE VARIABLES BETWEEN SUBROUTINE CALLS              Convert

ACTION        ACTIVATE INTERACTIVE INPUT FOR CODES                 Convert

CLOSE         ACTIVATE CLOSE ALL FILES BEFORE TERMINATING          Convert

Convert

EXAMPLE INPUT NO. 1                                                Convert

-------------------                                                Convert

TO USE A PROGRAM IN SINGLE PRECISION, USE THE STANDARD FILE NAMES, Convert

REWIND ALL UNITS AT THE START OF THE PROGRAM AND TREAT CHARACTER   Convert

ARRAYS AS CHARACTER (FORTRAN-77 CONVENTION).                       Convert

Convert

READ  \PREPRO93\RECENT\RECENT.OLD AND                              Convert

WRITE \PREPRO93\RECENT\RECENT.NEW                                  Convert

Convert

THE FOLLOWING 3 INPUT LINES ARE REQUIRED,                          Convert

Convert

SINGLE NAMES REWIND CHARACTER                                      Convert

\PREPRO93\RECENT\RECENT.OLD                                        Convert

\PREPRO93\RECENT\RECENT.NEW                                        Convert

Convert

NOTE, SINCE CHARACTER IS THE STANDARD OPTION THE KEYWORD CHARACTER Convert

NEED NOT APPEAR ON THE ABOVE INPUT LINE.                           Convert

Convert

EXAMPLE INPUT NO. 2                                                Convert

-------------------                                                Convert

TO USE A PROGRAM IN DOUBLE PRECISION AND TREAT ALL CHARACTER       Convert

ARRAYS AS INTEGER (FORTRAN-H CONVENTION).                          Convert

Convert

USE THE STANDARD FILENAMES TO READ = CONVERT.IN AND WRITE =        Convert

CONVERT.OUT (THIS CAN BE DONE BY LEAVING THE SECOND AND THIRD      Convert

INPUT LINES BLANK).                                                Convert

Convert

THE FOLLOWING 3 INPUT LINES ARE REQUIRED,                          Convert

Convert

DOUBLE  INTEGER                                                    Convert

(NOTE, THIS IS A BLANK LINE)                                       Convert

(NOTE, THIS IS A BLANK LINE)                                       Convert

Convert

NOTE, SINCE DOUBLE IS THE STANDARD OPTION THE KEYWORD DOUBLE       Convert

NEED NOT APPEAR ON THE ABOVE INPUT LINE.                           Convert

Convert

EXAMPLE INPUT NO. 3                                                Convert

-------------------                                                Convert

TO ACTIVATE THE PROGRAM LINE, USE DOUBLE PRECISION AND TREAT ALL   Convert

CHARACTER ARRAYS AS CHARACTER.                                     Convert

Convert

Convert

READ  \PREPRO93\RECENT\RECENT.OLD AND                              Convert

WRITE THE STANDARD FILENAME = CONVERT.OUT (LEAVE THE THIRD INPUT   Convert

LINE BLANK).                                                       Convert

Convert

THE FOLLOWING 3 INPUT LINES ARE REQUIRED,                          Convert

Convert

PROGRAM                                                            Convert

\PREPRO93\RECENT\RECENT.OLD                                        Convert

(NOTE, THIS IS A BLANK LINE)                                       Convert

Convert

NOTE, SINCE DOUBLE, CHARACTER AND EXIT ARE THE STANDARD OPTIONS    Convert

THEY NEED NOT APPEAR ON THE ABOVE INPUT LINE AND IN THIS EXAMPLE   Convert

HAVE BEEN OMITTED.                                                 Convert

Convert

WARNING                                                            Convert

-------                                                            Convert

(1) THE PROGRAM WILL ALWAYS ACTIVATE DOUBLE OR SINGLE, CHARACTER   Convert

OR INTEGER (AS DESCRIBED ABOVE).                                   Convert

Convert

(2) CODING IN THE PROGRAM FOR ANY KEYWORDS THAT ARE NOT ACTIVATED  Convert

WILL BE CONVERTED TO COMMENT LINES AND AS SUCH WILL EFFECTIVELY    Convert

DISAPPEAR FROM THE PROGRAM. THEREFORE IF THE KEYWORDS PROGRAM,     Convert

NAMES, REWIND, ENDFILE, CIVIC OR NOID ARE NOT INPUT BY THE USER    Convert

THESE OPTIONS WILL BE TURNED OFF AND ANY CODING USING THESE        Convert

KEYWORDS WILL EFFECTIVELY DISAPPEAR FROM THE PROGRAM.              Convert

Convert

(3) THE SERIES OF CODES THAT ARE DESIGNED TO BE AUTOMATICALLY      Convert

TRANSLATED BY THIS PROGRAM REQUIRE THAT ALL CALCULATIONS BE        Convert

PERFORMED IN DOUBLE PRECISION ON SHORT WORD LENGTH COMPUTERS       Convert

(E.G., IBM COMPUTERS). THIS PROGRAM WILL ALLOW YOU TO SPECIFY      Convert

EITHER DOUBLE OR SINGLE PRECISION. HOWEVER, IF YOU SPECIFY         Convert

SINGLE PRECISION THIS PROGRAM WILL PRINT A WARNING MESSAGE THAT    Convert

THE CONVERTED PROGRAM SHOULD ONLY BE USED ON LONG WORD LENGTH      Convert

COMPUTERS (E.G., CDC COMPUTERS).                                   Convert

Convert

PROGRAM OPERATION                                                  Convert

-----------------                                                  Convert

THE PROGRAM WILL SEARCH FOR COMMENT LINES THAT START WITH C**      Convert

IN COLUMNS 1-3 FOLLOWED BY ANY ONE OF THE ALLOWED KEYWORDS         Convert

IF THE KEYWORD IS THE SAME AS ONE OF THE KEYWORDS INPUT BY         Convert

THE USER ALL LINES UP TO THE NEXT LINE WITH C** IN COLUMNS 1-3     Convert

FOLLOWED BY THE SAME KEYWORD WILL BE SET ACTIVE BY SETTING COLUMN  Convert

1 TO BLANK. IF THE KEYWORDS DIFFERS FROM THAT INPUT BY THE USER    Convert

ALL LINES UP TO THE NEXT LINE WITH C** IN COLUMNS 1-3 FOLLOWED BY  Convert

THE SAME KEYWORD WILL BE SET INACTIVE BY SETTING COLUMN 1 TO C.    Convert

Convert

KEYWORDS MAY NOT BE NESTED (I.E., THIS PROGRAM WILL ONLY OPERATE   Convert

PROPERLY IF KEYWORDS APPEAR IN PAIRS. ONCE A LINE IS FOUND THAT    Convert

CONTAINS A KEYWORD, THE NEXT LINE THAT CONTAINS A KEYWORD MUST     Convert

CONTAIN THE SAME KEYWORD).                                         Convert

Convert

PROGRAM LINE                                                       Convert

------------                                                       Convert

THE FORTRAN FILE MAY START WITH A PROGRAM LINE AND CONTINUATIONS.  Convert

FOR USE ON CDC-7600 OR CRAY-1 COMPUTERS THIS PROGRAM CAN ACTIVATE  Convert

THE PROGRAM LINE AND CONTINUATION LINES. FOR USE ON OTHER TYPES OF Convert

COMPUTERS THIS PROGRAM WILL AUTOMATICALLY DE-ACTIVATE THE PROGRAM  Convert

LINE AND CONTINUATION LINES. THIS CONVENTIONS HAS BEEN INTRODUCED  Convert

BECAUSE SOME CDC-7600 COMPILERS CONSIDER IT AN ERROR IF THE FIRST  Convert

LINE IS NOT A PROGRAM LINE. PRECEEDING COMMENT LINES ARE NOT       Convert

ALLOWED. THEREFORE THE NORMAL CONVENTION, DESCRIBED ABOVE, OF      Convert

USING PRECEDING AND FOLLOWING COMMENT LINES, CANNOT BE USED AT     Convert

THE BEGINNING OF THE PROGRAM.                                      Convert

Convert

COMMENT LINES                                                      Convert

-------------                                                      Convert

COMMENT LINES MAY APPEAR ON LINES BETWEEN PAIRS OF KEYWORD LINES   Convert

ONLY IF THE COMMENT LINES CONTAINS C-- IN COLUMS 1-3. ANY LINE     Convert

THAT CONTAINS ANYTHING ELSE IN COLUMNS 1-3 MAY BE ACTIVATED        Convert

BY THIS PROGRAM BY SETTING COLUMN 1 BLANK AND CAN LEAD TO ERRORS   Convert

DURING COMPILATION AND/OR EXECUTION.                               Convert

Convert

INPUT FILES                                                        Convert

-----------                                                        Convert

UNIT  DESCRIPTION                                                  Convert

----  -----------                                                  Convert

2   INPUT LINE (BCD - 80 CHARACTERS/RECORD)                      Convert

10   ORIGINAL PROGRAM (BCD - 80 CHARACTERS/RECORD)                Convert

Convert

OUTPUT FILES                                                       Convert

------------                                                       Convert

UNIT  DESCRIPTION                                                  Convert

----  -----------                                                  Convert

3   OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Convert

11   RE-FORMATTED PROGRAM (BCD - 80 CHARACTERS/RECORD)            Convert

Convert

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Convert

---------------------------------------------------------------    Convert

UNIT  FILE NAME                                                    Convert

----  ----------                                                   Convert

2   CONVERT.INP                                                  Convert

3   CONVERT.LST                                                  Convert

10   CONVERT.IN                                                   Convert

11   CONVERT.OUT                                                  Convert

Convert

======================================================================= Convert

======================================================================= Dictin

Dictin

PROGRAM DICTIN (Renamed from DICTION to eliminate conflict with    Dictin

UNIX diction command - 12/22/02)                   Dictin

==============                                                     Dictin

VERSION 81-1 (SEPTEMBER 1981)                                      Dictin

VERSION 82-1 (JANUARY 1982)                                        Dictin

VERSION 83-1 (JANUARY 1983)  *KEEP ORIGINAL MOD. NUMBER            Dictin

*NEW, MORE COMPATIBLE I/O UNITS.      Dictin

VERSION 84-1 (SEPTEMBER 1984)*UPDATED TO HANDLE ENDF/B-VI FORMAT.  Dictin

(PROGRAM WILL NOW WORK ON ALL        Dictin

VERSIONS OF THE ENDF/B FORMAT).     Dictin

VERSION 85-1 (AUGUST 1985)   *FORTRAN-77/H VERSION                 Dictin

VERSION 86-1 (JANUARY 1986)  *MAT ORDER CHECK.                     Dictin

*IF NO HOLLERITH SECTION COPY MAT.    Dictin

VERSION 88-1 (JULY 1988)     *OPTION...INTERNALLY DEFINE ALL I/O   Dictin

FILE NAMES (SEE, SUBROUTINE FILEIO   Dictin

FOR DETAILS).                        Dictin

*IMPROVED BASED ON USER COMMENTS.     Dictin

VERSION 89-1 (JANUARY 1989)  *PSYCHOANALYZED BY PROGRAM FREUD TO   Dictin

INSURE PROGRAM WILL NOT DO ANYTHING  Dictin

CRAZY.                               Dictin

*IMPROVED BASED ON USER COMMENTS.     Dictin

*ADDED LIVERMORE CIVIC COMPILER       Dictin

CONVENTIONS.                         Dictin

*UPDATED TO USE NEW PROGRAM CONVERT   Dictin

KEYWORDS.                            Dictin

VERSION 92-1 (JANUARY 1992)  *UPDATED BASED ON USER COMMENTS.      Dictin

*UP TO 6000 SECTIONS PER TAPE.        Dictin

*CHANGED DEFAULT MOD NUMBER FOR NEW   Dictin

SECTIONS FROM 0 TO 1                 Dictin

VERSION 94-1 (JANUARY 1994)  *VARIABLE ENDF/B DATA FILENAMES       Dictin

TO ALLOW ACCESS TO FILE STRUCTURES   Dictin

(WARNING - INPUT PARAMETER FORMAT    Dictin

HAS BEEN CHANGED)                    Dictin

*CLOSE ALL FILES BEFORE TERMINATING   Dictin

(SEE, SUBROUTINE ENDIT)              Dictin

*ADDED FORTRAN SAVE OPTION            Dictin

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Dictin

*IMPROVED COMPUTER INDEPENDENCE        Dictin

*ALL DOUBLE PRECISION                  Dictin

*ON SCREEN OUTPUT                      Dictin

*UNIFORM TREATMENT OF ENDF/B I/O       Dictin

*IMPROVED OUTPUT PRECISION             Dictin

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Dictin

POINT READ FOR MORE DIGITS            Dictin

*UPDATED TEST FOR ENDF/B FORMAT        Dictin

VERSION BASED ON RECENT FORMAT CHANGE Dictin

*GENERAL IMPROVEMENTS BASED ON         Dictin

USER FEEDBACK                         Dictin

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Dictin

USER FEEDBACK                         Dictin

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Dictin

*RENAMED dictin TO ELIMINATE CONFLICT  Dictin

WITH UNIX diction COMMAND.            Dictin

*ADDED DOCUMENTATION LINE TO COMMENTS. Dictin

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACK Dictin

*UP TO 100,000 SECTIONS PER TAPE.      Dictin

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Dictin

*UP TO 500,000 SECTIONS PER TAPE.      Dictin

Dictin

OWNED, MAINTAINED AND DISTRIBUTED BY                               Dictin

------------------------------------                               Dictin

THE NUCLEAR DATA SECTION                                           Dictin

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Dictin

P.O. BOX 100                                                       Dictin

A-1400, VIENNA, AUSTRIA                                            Dictin

EUROPE                                                             Dictin

Dictin

ORIGINALLY WRITTEN BY                                              Dictin

------------------------------------                               Dictin

DERMOTT E. CULLEN                                                  Dictin

UNIVERSITY OF CALIFORNIA                                           Dictin

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Dictin

L-159                                                              Dictin

P.O. BOX 808                                                       Dictin

LIVERMORE, CA 94550                                                Dictin

U.S.A.                                                             Dictin

TELEPHONE  925-423-7359                                            Dictin

E. MAIL    CULLEN1@LLNL.GOV                                        Dictin

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Dictin

Dictin

AUTHORS MESSAGE                                                    Dictin

---------------                                                    Dictin

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENATION    Dictin

FOR THIS PROGRAM INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ    Dictin

ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                       Dictin

Dictin

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Dictin

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Dictin

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Dictin

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Dictin

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Dictin

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Dictin

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Dictin

COMPUTER.                                                          Dictin

Dictin

PURPOSE                                                            Dictin

-------                                                            Dictin

THIS PROGRAM IS DESIGNED TO CREATE A REACTION INDEX FOR EACH       Dictin

MATERIAL ON AN ENDF/B FORMATTED TAPE AND TO INSERT THIS REACTION   Dictin

INDEX IN FILE 1, SECTION 451 OF EACH MATERIAL.                     Dictin

Dictin

IN THE DESCRIPTION THAT FOLLOWS FOR SIMPLICITY THE ENDF/B          Dictin

TERMINOLOGY---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL       Dictin

MEDIUM MAY BE TAPE, CARDS, DISK, OR ANY OTHER MEDIUM.              Dictin

Dictin

ENDF/B FORMAT                                                      Dictin

-------------                                                      Dictin

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Dictin

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Dictin

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Dictin

Dictin

THIS PROGRAM WILL AUTOMATICALLY DETERMINE WHICH VERSION OF THE     Dictin

ENDF/B FORMAT EACH MAT IS IN AND WILL THEN PROPERLY REPLACE THE    Dictin

REACTION INDEX FOR EACH MAT. DIFFERENT MATS ON THE SAME TAPE MAY   Dictin

EVEN BE IN DIFFERENT VERSIONS OF THE ENDF/B FORMAT.                Dictin

Dictin

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Dictin

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Dictin

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Dictin

NUMBERS (COLUMNS 76-80) NEED NOT BE PRESENT ON INPUT, BUT WILL BE  Dictin

CORRECTLY OUTPUT ON ALL LINES.                                     Dictin

Dictin

ENDF/B FORMAT VERSION                                              Dictin

---------------------                                              Dictin

THE ENDF/B FORMAT CAN BE DETERMINED FROM THE SECOND LINE OF        Dictin

THE HOLLERITH SECTION (MF=1, MT=451).                              Dictin

ENDF/B-IV = N1 - LINE COUNT (POSITIVE)                             Dictin

ENDFB/-V  = N1 = N2 =0                                             Dictin

ENDF/B-VI = N1 =0, N2= VERSION NUMBER (6 OR MORE)                  Dictin

Dictin

SECTION SIZE                                                       Dictin

------------                                                       Dictin

SINCE THIS PROGRAM ONLY READS THE DATA ONE LINE AT A TIME THERE    Dictin

IS NO LIMIT TO THE SIZE OF ANY GIVEN SECTION, E.G. THE TOTAL       Dictin

CROSS SECTION MAY BE DESCRIBED BY 200,000 DATA POINTS.             Dictin

Dictin

NUMBER OF SECTIONS PER TAPE                                        Dictin

---------------------------                                        Dictin

IT IS ASSUMED THAT THE ENDF/B TAPE CONTAINS 100,000 OR FEWER       Dictin

SECTIONS = 100,000 OR FEWER MAT,MF,MT COMBINATIONS. IF THIS LIMIT  Dictin

IS EXCEEDED THIS PROGRAM WILL TERMINATE EXECUTION. IF NEED BE THIS Dictin

LIMIT CAN EASILY BE CHANGED BY CHANGING THE DIMENSION STATEMENT    Dictin

BELOW AND RE-DEFINING THE VARIABLE MAXTAB IN THE BELOW DATA        Dictin

STATEMENT. ALTERNATIVELY THE ENDF/B TAPE MAY BE DIVIDED INTO A     Dictin

NUMBER SMALLER TAPES EACH CONTAINING 100,000 OR FEWER SECTIONS.    Dictin

EACH ENDF/B TAPE CAN THEN RUN THROUGH THIS PROGRAM AND THE OUTPUT  Dictin

FOR EACH ENDF/B TAPE CAN THEN BE RE-COMBINED (I.E., MERGED BACK    Dictin

TOGETHER).                                                         Dictin

Dictin

HOLLERITH SECTION                                                  Dictin

-----------------                                                  Dictin

IF ANY MATERIAL DOES NOT INITIALLY CONATIN A SECTION MF=1, MT=451  Dictin

A WARNING MESSAGE WILL BE PRINTED AND THE MATERIAL WILL BE COPIED. Dictin

Dictin

IF ANY MATERIAL INITIALLY CONTAINS A SECTION MF=1, MT=451 A NEW    Dictin

REACTION INDEX WILL BE CREATED AND INSERTED. THE INITIAL SECTION   Dictin

MF=1, MT=451 MAY OR MAY NOT CONTAIN A REACTION INDEX.              Dictin

Dictin

IF THE MATERIAL INITIALLY CONTAINS A REACTION INDEX IT WILL BE     Dictin

USED TO DEFINE THE MOD NUMBER FOR CORRESPONDING SECTIONS IN THE    Dictin

NEW REACTION INDEX (I.E. IF A SECTION FROM THE ORIGINAL REACTION   Dictin

INDEX HAS THE SAME MF/MT NUMBERS AS A SECTION IN THE NEW REACTION  Dictin

INDEX THE MOD NUMBER FROM THE ORIGINAL REACTION INDEX WILL BE USED Dictin

IN THE NEW REACTION INDEX). OTHERWISE THE MOD NUMBER IN THE NEW    Dictin

REACTION INDEX WILL BE SET EQUAL TO ZERO.                          Dictin

Dictin

PROGRAM OPERATION                                                  Dictin

-----------------                                                  Dictin

THE ENTIRE ENDF/B TAPE IS FIRST READ AND A DICTIONARY ENTRY IS     Dictin

CREATED FOR EACH SECTION OF THE TAPE. THE ENDF/B TAPE IS THEN      Dictin

REWOUND AND READ A SECOND TIME. DURING THIS SECOND PASS THE        Dictin

DICTIONARY OF EACH MAT IS REPLACED. THIS VERSION OF DICTIN         Dictin

DOES NOT USE SCRATCH FILES AND IS MORE EFFICIENT THAN EARLIER      Dictin

VERSIONS OF DICTIN.                                                Dictin

Dictin

INPUT LINES                                                        Dictin

-----------                                                        Dictin

LINE   COLS.  DESCRIPTION                                        Dictin

----   -----  ------------------------------                     Dictin

1   1-60   ENDF/B INPUT DATA FILENAME                         Dictin

(STANDARD OPTION = ENDFB.IN)                       Dictin

2   1-60   ENDF/B OUTPUT DATA FILENAME                        Dictin

(STANDARD OPTION = ENDFB.OUT)                      Dictin

Dictin

EXAMPLE INPUT NO. 1                                                Dictin

-------------------                                                Dictin

READ \ENDFB6\K300\ENDFB.IN AND WRITE \ENDFB\K300\ENDFB.OUT. THE    Dictin

FOLLOWING 2 INPUT LINES ARE REQUIRED,                              Dictin

Dictin

\ENDFB6\K300\ENDFB.IN                                                  Dictin

\ENDFB6\K300\ENDFB.OUT                                                 Dictin

Dictin

EXAMPLE INPUT NO. 2                                                Dictin

-------------------                                                Dictin

USE THE DEFAULT FILENAMES TO READ ENDFB.IN AND WRITE ENDFB.OUT.    Dictin

2 BLANK INPUT LINES ARE REQUIRED                                   Dictin

Dictin

INPUT FILES                                                        Dictin

-----------                                                        Dictin

UNIT  DESCRIPTION                                                  Dictin

----  -----------                                                  Dictin

2  INPUT PARAMETERS (BCD - 80 CHARACTERS/RECORD)                Dictin

10  ORIGINAL TAPE OF ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)    Dictin

Dictin

OUTPUT FILES                                                       Dictin

------------                                                       Dictin

UNIT  DESCRIPTION                                                  Dictin

----  -----------                                                  Dictin

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Dictin

11  FINAL TAPE OF ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)       Dictin

Dictin

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Dictin

---------------------------------------------------------------    Dictin

UNIT  FILE NAME                                                    Dictin

----  ----------                                                   Dictin

2   DICTIN.INP                                                   Dictin

3   DICTIN.LST                                                   Dictin

10   ENDFB.IN                                                     Dictin

11   ENDFB.OUT                                                    Dictin

Dictin

======================================================================= Dictin

======================================================================= Evalplot

Evalplot

PROGRAM EVALPLOT                                                   Evalplot

VERSION 75-1 (AUGUST 1975)                                         Evalplot

VERSION 76-1 (JULY 1976)                                           Evalplot

VERSION 77-1 (APRIL 1977)                                          Evalplot

VERSION 78-1 (JULY 1978)                                           Evalplot

VERSION 79-1 (FEBRUARY 1979)                                       Evalplot

VERSION 80-1 (JULY 1980)    *IBM VERSION                           Evalplot

VERSION 80-2 (DECEMBER 1980)                                       Evalplot

VERSION 81-1 (MARCH 1981)                                          Evalplot

VERSION 81-2 (AUGUST 1981)  *IMPROVED ZOOM CAPABILITY              Evalplot

VERSION 82-1 (JANUARY 1982) *IMPROVED COMPUTER COMPATIBILITY       Evalplot

VERSION 83-1 (JANUARY 1983) *ELIMINATED COMPUTER DEPENDENT CODING. Evalplot

VERSION 83-2 (OCTOBER 1983) *ADDED PLOTTING OF HISTOGRAM DATA.     Evalplot

VERSION 84-1 (DECEMBER 1984)*ADDED PLOTS OF LEGENDRE COEFFICENTS   Evalplot

AS A FUNCTION OF ENERGY.              Evalplot

*ADDED SMALL PLOTTING MODE.            Evalplot

VERSION 85-1 (AUGUST 1985)  *FORTRAN-77/H VERSION                  Evalplot

VERSION 86-1 (JANUARY 1986) *ENDF/B-VI FORMAT                      Evalplot

VERSION 88-1 (JULY 1988)    *MAJOR REVISION TO MAKE CODE EASILY    Evalplot

INTERFACEABLE TO ALMOST ANY PLOTTER.  Evalplot

*WARNING...INPUT PARAMETERS FROM BEEN  Evalplot

CHANGED (SEE, DESCRIPTION BELOW)      Evalplot

*COMPUTER INDEPENDENT SOFTWARE         Evalplot

CHARACTERS.                           Evalplot

*COLOR PLOTS.                          Evalplot

*MT NUMBER DEFINITIONS FROM DATA FILE  Evalplot

READ BY PROGRAM                       Evalplot

*FORTRAN-77 REQUIRED (FORTRAN-H NO     Evalplot

SUPPORTED BY THIS PROGRAM).           Evalplot

*OPTION...INTERNALLY DEFINE ALL I/O    Evalplot

FILE NAMES (SEE, SUBROUTINE FILEIO    Evalplot

FOR DETAILS).                         Evalplot

*IMPROVED BASED ON USER COMMENTS.      Evalplot

VERSION 89-1 (JANUARY 1989) *PSYCHOANALYZED BY PROGRAM FREUD TO    Evalplot

INSURE PROGRAM WILL NOT DO ANYTHING   Evalplot

CRAZY.                                Evalplot

*UPDATED TO USE NEW PROGRAM CONVERT    Evalplot

KEYWORDS.                             Evalplot

*ADDED LIVERMORE CIVIC COMPILER        Evalplot

CONVENTIONS.                          Evalplot

*FORTRAN-77/FORTRAN-H COMPATIBLE       Evalplot

*SPECIAL ENDF/B MATERIAL DEFINITIONS   Evalplot

(ZA.LT.1000) FROM DATA FILE READ      Evalplot

BY PROGRAM.                           Evalplot

VERSION 89-2 (MARCH 1989)   *ADDED ENDF/B-V AND VI MT              Evalplot

DEFINITIONS. PROGRAM WILL DETERMINE   Evalplot

ENDF/B FORMAT BASED ON MF=1,          Evalplot

MT=451 AND USE ASPPROPRIATE MT        Evalplot

DEFINITIONS. IF NO MF=1, MT=451       Evalplot

PROGRAM WILL USE ENDF/B-V             Evalplot

MT DEFINITIONS.                       Evalplot

VERSION 89-3 (JUNE 1989)    *3 CHARACTER FONTS                     Evalplot

VERSION 92-1 (JANUARY 1992) *COMPLETE REWRITE OF CODE              Evalplot

*ADDED PHOTON DATA, MF=23 AND 27       Evalplot

*ADDED INCIDENT CHARGED PARTICLES      Evalplot

(IDENTIFIED IN PLOT TITLES)           Evalplot

*ADDED FORTRAN SAVE OPTION.            Evalplot

*UPDATED BASED ON USER COMMENTS        Evalplot

*ADDED RETRIEVAL BY UP TO 100          Evalplot

MAT/MF/MT OR ZA/MF/MT RANGES          Evalplot

*WARNING...INPUT PARAMETER FORMAT      Evalplot

HAS BEEN CHANGED...SEE DESCRIPTION    Evalplot

BELOW.                                Evalplot

VERSION 92-2 (FEBRUARY 1992)*ADDED PHOTON SPECTRA, MF=15.          Evalplot

*ADDED MULTIPLICATION OF DISTRIBUTIONS Evalplot

IN MF=5 AND 15 BY PROBABILITY=YIELD.  Evalplot

*INCREASED PAGE SIZE TO 12000 POINTS   Evalplot

VERSION 92-3 (MAY 1992)     *CORRECTED DESCRIPTION OF INPUT        Evalplot

PARAMETERS AND EXAMPLE PROBLEMS.      Evalplot

*CORRECTED FOR ENDF/B-VI DEFINITION OF Evalplot

TEMPERATURE FROM MF=1/MT=451.         Evalplot

*CORRECTED LOGIC SO THAT EACH REQUEST  Evalplot

IS TREATED SEPARATELY TO CREATE A     Evalplot

PLOT, UNLESS REQUESTS ARE CHAINED     Evalplot

TOGETHER.                             Evalplot

*ADDED VARIABLE CHARACTER SIZE INPUT.  Evalplot

VERSION 93-1 (MARCH 1993)   *INCREASED PAGE SIZE FROM 12000        Evalplot

TO 210000                             Evalplot

*INCREASED THE NUMBER OF ENERGIES      Evalplot

VS. LEGENDRE COEFFICIENTS FROM        Evalplot

167 TO 7000                           Evalplot

*UPDATED FOR ON SCREEN GRAPHICS        Evalplot

USING THE LAHEY FORTRAN COMPILER.     Evalplot

VERSION 94-1 (JANUARY 1994) *VARIABLE ENDF/B DATA FILENAMES        Evalplot

TO ALLOW ACCESS TO FILE STRUCTURES    Evalplot

(WARNING - INPUT PARAMETER FORMAT     Evalplot

HAS BEEN CHANGED)                     Evalplot

*CLOSE ALL FILES BEFORE TERMINATING    Evalplot

(SEE, SUBROUTINE ENDIT)               Evalplot

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Evalplot

*IMPROVED COMPUTER INDEPENDENCE        Evalplot

*ALL DOUBLE PRECISION                  Evalplot

*UNIFORM TREATMENT OF ENDF/B I/O       Evalplot

*IMPROVED OUTPUT PRECISION             Evalplot

*DEFINED SCRATCH FILE NAMES            Evalplot

*ALL DOUBLE PRECISION                  Evalplot

VERSION 97-1 (APRIL 1997)   *INCREASED PAGE SIZE FROM 210000       Evalplot

TO 480,000                            Evalplot

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Evalplot

POINT READ FOR MORE DIGITS            Evalplot

*UPDATED TEST FOR ENDF/B FORMAT        Evalplot

VERSION BASED ON RECENT FORMAT CHANGE Evalplot

*GENERAL IMPROVEMENTS BASED ON         Evalplot

USER FEEDBACK                         Evalplot

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF=10, ACTIVATION CROSS         Evalplot

SECTION PLOTS.                        Evalplot

*INCREASED DIMENSIONS TO HANDLE MORE   Evalplot

SECTIONS - UP TO 1,000                Evalplot

*GENERAL IMPROVEMENTS BASED ON         Evalplot

USER FEEDBACK                         Evalplot

VERS. 2002-1 (Nov. 2002)    *OPTIONAL INPUT PARAMETERTS            Evalplot

*OPTIONAL BLACK OR WHITE BACKGROUND    Evalplot

*COLOR POSTSCRIPT FILES                Evalplot

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Evalplot

*INCREASED PAGE SIZE TO 600,000        Evalplot

*INCREASED THE NUMBER OF ENERGIES      Evalplot

VS. LEGENDRE COEFFICIENTS FROM        Evalplot

7000 TO 20000                         Evalplot

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Evalplot

*INCREASED PAGE SIZE TO 2,400,000      Evalplot

FROM 600,000.                         Evalplot

VS. LEGENDRE COEFFICIENTS TO          Evalplot

80,000 FROM 20,000 (MUST BE 1/30      Evalplot

PAGE SIZE).                           Evalplot

*ADDEED (N,REMAINDER) TO FIRST PLOT.   Evalplot

Evalplot

OWNED, MAINTAINED AND DISTRIBUTED BY                               Evalplot

------------------------------------                               Evalplot

THE NUCLEAR DATA SECTION                                           Evalplot

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Evalplot

P.O. BOX 100                                                       Evalplot

A-1400, VIENNA, AUSTRIA                                            Evalplot

EUROPE                                                             Evalplot

Evalplot

ORIGINALLY WRITTEN BY                                              Evalplot

------------------------------------                               Evalplot

DERMOTT E. CULLEN                                                  Evalplot

UNIVERSITY OF CALIFORNIA                                           Evalplot

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Evalplot

L-159                                                              Evalplot

P.O. BOX 808                                                       Evalplot

LIVERMORE, CA 94550                                                Evalplot

U.S.A.                                                             Evalplot

TELEPHONE  925-423-7359                                            Evalplot

E. MAIL    CULLEN1@LLNL.GOV                                        Evalplot

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Evalplot

Evalplot

AUTHORS MESSAGE                                                    Evalplot

---------------                                                    Evalplot

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Evalplot

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Evalplot

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Evalplot

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Evalplot

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Evalplot

Evalplot

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Evalplot

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Evalplot

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Evalplot

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Evalplot

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Evalplot

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Evalplot

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Evalplot

COMPUTER.                                                          Evalplot

Evalplot

PURPOSE                                                            Evalplot

-------                                                            Evalplot

THIS PROGRAM IS DESIGNED TO READ EVALUATED DATA FROM THE ENDF/B    Evalplot

FORMAT AND TO PLOT THE DATA. THE USER MAY SELECT CROSS SECTIONS,   Evalplot

PARAMETERS (E.G. NU-BAR, MU-BAR, ETC.), ANGULAR DISTRIBUTIONS      Evalplot

AND/OR ENERGY DISTRIBUTIONS TO BE PLOTTED.                         Evalplot

Evalplot

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Evalplot

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Evalplot

DISK OR ANY OTHER MEDIUM.                                          Evalplot

Evalplot

ON WHAT COMPUTERS WILL THE PROGRAM RUN                             Evalplot

------------------------------------------------------------------ Evalplot

THE PROGRAM HAS BEEN IMPLEMENTED ON A WIDE VARIETY OF COMPUTERS    Evalplot

FROM THE ONE EXTREME OF LARGE MAINFRAME CRAY AND IBM COMPUTERS     Evalplot

TO THE OTHER EXTREME OF SUN TERMINALS AND IBM PERSONAL COMPUTERS.  Evalplot

THE PROGRAM IS DESIGNED TO RUN ON VIRTUALLY ANY COMPUTER. FOR      Evalplot

SPECIAL CONSIDERATIONS SEE THE SECTIONS BELOW ON,                  Evalplot

(1) COMPUTER DEPENDENT CODING                                      Evalplot

(2) PLOTTER/GRAPHICS TERMINAL INTERFACE                            Evalplot

Evalplot

ON WHAT PLOTTERS WILL THE PROGRAM RUN                              Evalplot

------------------------------------------------------------------ Evalplot

THE PLOTTER MAY USE UNITS OF INCHES, CENTIMETERS, MILLIMETERS,     Evalplot

VIRTUALLY ANYTHING. INTERNALLY THE PROGRAM WILL DEFINE PLOTS IN    Evalplot

APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. AS PART OF THE        Evalplot

INPUT THE USER DEFINES THE ACTUAL SIZE OF THE PLOT IN THE UNITS    Evalplot

(I.E., INCHES, CENTIMETERS, MILLIMETERS, WHATEVER) OF THE REAL     Evalplot

PLOT. THE PLOT IS TRANSFORMED TO THE SIZE OF THE LOCAL PLOTTER     Evalplot

AND OUTPUT. USING THIS CONVENTION THIS PROGRAM SHOULD BE EASY      Evalplot

TO INTERFACE TO VIRTUALLY ANY PLOTTER OR GRAPHICS TERMINAL.        Evalplot

Evalplot

GRAPHICS INTERFACE                                                 Evalplot

------------------------------------------------------------------ Evalplot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE GRAPHICS INTERFACE WHICH   Evalplot

REQUIRES ONLY 3 SUBROUTINES...PLOTS, PLOT AND PEN (DESCRIBED IN    Evalplot

DETAIL BELOW). ALL CHARACTERS AND SYMBOLS ARE DRAWN USING TABLES   Evalplot

OF PEN STROKES (SUPPLIED WITH THIS PROGRAM). USING THIS METHOD     Evalplot

THE PROGRAM SHOULD BE SIMPLE TO INTERFACE TO VIRTUALLY ANY PLOTTER Evalplot

OR GRAPHICS TERMINAL AND THE APPEARANCE AND LAYOUT OF THE PLOTS    Evalplot

SHOULD BE INDEPENDENT OF WHICH PLOTTER IS USED.                    Evalplot

Evalplot

PROGRAM IDENTIFICATION                                             Evalplot

----------------------                                             Evalplot

AS DISTRIBUTED THE FIRST FRAME OF PLOTTED OUTPUT WILL DOCUMENT     Evalplot

THE PROGRAM NAME, VERSION AND INSTALLATION. THIS INFORMATION IS    Evalplot

STORED AS DATA IN THE ARRAY VERSES NEAR THE BEGINNING OF           Evalplot

SUBROUTINE FRAME1. IF YOU WISH TO CUSTOMIZE THE OUTPUT TO IDENTIFY Evalplot

YOUR INSTALLATION CHANGE THE LAST TWO LINES OF THE ARRAY VERSES.   Evalplot

Evalplot

SIZE OF PLOTS                                                      Evalplot

-------------                                                      Evalplot

THE PROGRAM HAS A BUILT-IN DEFAULT SIZE TO MAKE EACH PLOT 13.50    Evalplot

BY 10.24 INCHES. THIS SIZE WAS SELECTED ASSUMING THAT THE          Evalplot

RESOLUTION OF THE PLOTTER IS 1024 RASTER POINTS PER INCH. THE      Evalplot

USER MAY CHANGE THE SIZE OF THE PLOT BY SPECIFYING ANY REQUIRED    Evalplot

SIZE ON THE FIRST INPUT LINE. IN PARTICULAR FOR USE ON ANY PLOTTER Evalplot

THAT USES CENTIMETERS INSTEAD OF INCHES THE USER MAY MERELY        Evalplot

SPECIFY THE REQUIRED SIZE OF THE PLOT IN CENTIMETERS (E.G., TO     Evalplot

OBTAIN A 13.50 BY 10.24 INCH PLOT, THE USER NEED ONLY SPECIFY      Evalplot

34.3 BY 26 ON THE FIRST INPUT LINE...ASSUMING 2.54 CENTIMETERS PER Evalplot

INCH, OR 343 BY 260 FOR MILLIMETERS..ASSUMING 25.4 MILLIMETERS     Evalplot

PER INCH).                                                         Evalplot

Evalplot

CHARACTER SIZE                                                     Evalplot

--------------                                                     Evalplot

THE PLOT HAS A BUILT-IN CHARACTER SIZE WHICH HAS BEEN DEFINED FOR  Evalplot

COMPATIBILITY WITH THE BUILT-IN PLOT SIZE. IF THE USER SPECIFIES   Evalplot

BY INPUT A DIFFERENT PLOT SIZE, THE PROGRAM WILL AUOTMATICALLY     Evalplot

SCALE THE SIZE OF ALL CHARACTERS BY THE RATIO OF THE Y SIZE OF THE Evalplot

PLOT SPECIFIED BY THE USER TO THE BUILT-IN Y SIZE OF PLOTS (E.G.,  Evalplot

FOR PLOTS WHICH ARE ONLY 5.12 HIGH (Y DIRECTION) ALL CHARACTERS    Evalplot

WILL BE SCALED TO BE ONLY 1/2 THE CHARACTER SIZE ON PLOTS WHICH    Evalplot

ARE 10.24 HIGH (10.24 = THE BUILT-IN SIZE). NOTE, CHANGES IN THE   Evalplot

X SIZE OF THE PLOT WILL NOT HAVE ANY EFFECT ON THE CHARACTER SIZE  Evalplot

(E.G., FOR A LONG PLOT, 30 BY 10.24 THE CHARACTER SIZE WILL BE THE Evalplot

THE SAME AS ON A 13.50 BY 10.24 PLOT).                             Evalplot

Evalplot

PLOT PER FRAME                                                     Evalplot

--------------                                                     Evalplot

BY INPUT THE USER CAN SPECIFY NOT ONLY THE ACTUAL SIZE OF THE      Evalplot

LOCAL PLOTTER, BUT ALSO HOW MANY PLOTS SHOULD APPEAR ON EACH       Evalplot

FRAME. THIS IS DONE BY SPECIFYING THE LAYOUT OF A FRAME IN TERMS   Evalplot

OF THE NUMBER OF PLOTS IN THE X AND Y DIRECTION. FOR EXAMPLE BY    Evalplot

SPECIFYING THAT EACH FRAME BE DIVIDED INTO 3 PLOTS IN THE X        Evalplot

DIRECTION AND 2 PLOTS IN THE Y DIRECTION, EACH FRAME WILL CONTAIN  Evalplot

UP TO 6 PLOTS (3 X 2). INTERNALLY EACH PLOT WILL BE GENERATED TO   Evalplot

STANDARD A4 SIZE, AS DESCRIBED ABOVE, AND THEN ON OUTPUT SCALED    Evalplot

TO THE NUMBER OF PLOTS PER FRAME SPECIFIED BY THE USER INPUT.      Evalplot

Evalplot

ENDF/B FORMAT                                                      Evalplot

-------------                                                      Evalplot

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Evalplot

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Evalplot

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Evalplot

Evalplot

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Evalplot

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Evalplot

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Evalplot

NUMBERS (COLUMNS 76-80) ARE IGNORED. FORMAT OF SECTION MT=452, 455 Evalplot

OF MF=1, AND ALL SECTIONS OF MF=3, 4 AND 5 MUST BE CORRECT. ALL    Evalplot

OTHER SECTION OF DATA ARE SKIPPED AND AS SUCH THE OPERATION OF     Evalplot

THIS PROGRAM IS INSENSITIVE TO THE CORRECTNESS OR INCORRECTNESS    Evalplot

OF ALL OTHER SECTIONS.                                             Evalplot

Evalplot

INTERPOLATION LAW                                                  Evalplot

-----------------                                                  Evalplot

EACH TABLE OF DATA MAY USE EITHER COMPLETELY HISTOGRAM OR          Evalplot

COMPLETELY LINEAR INTERPOLATION LAW (THE TWO INTERPOLATION LAWS    Evalplot

CANNOT BE MIXED TOGETHER IN ONE TABLE). EITHER OF THESE TWO        Evalplot

REPRESENTATIONS WILL BE STORED IN CORE IN LINEARLY INTERPOLABLE    Evalplot

FORM. IF THIS PROGRAM FINDS ANY DATA THAT USES ANY OTHER           Evalplot

INTERPOLATION LAW IT WILL PRINT AN ERROR MESSAGE AND PLOT THE      Evalplot

TABLE AS IF IT WERE LINEARLY INTERPOLABLE. THE ONLY ERROR THAT     Evalplot

WILL RESULT IN THE PLOT WILL BE IN THE CURVE FOLLOWED BETWEEN      Evalplot

TABULATED POINTS. PROGRAM LINEAR (UCRL-50400, VOL. 17, PART A)     Evalplot

MAY BE USED TO CONVERT CROSS SECTIONS TO LINEARLY INTERPOLABLE     Evalplot

FORM. PROGRAM LEGEND CAN BE USED FOR ANGULAR DISTRIBUTIONS AND     Evalplot

PROGRAM ENERGY CAN BE USED FOR SECONDARY ENERGY DISTRIBUTIONS.     Evalplot

Evalplot

REACTION INDEX                                                     Evalplot

--------------                                                     Evalplot

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Evalplot

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Evalplot

Evalplot

PAGE SIZE                                                          Evalplot

---------                                                          Evalplot

ONLY ONE PAGE OF DATA = 600000 DATA POINTS - IS KEPT IN CORE AT    Evalplot

ANY GIVEN TIME. IF THERE IS MORE THAN THIS MANY POINTS THEY WILL   Evalplot

BE KEPT ON A SCRATCH FILE AND LOADED INTO CORE AS NEEDED.          Evalplot

Evalplot

TO CHANGE THE PAGE SIZE,                                           Evalplot

Evalplot

1) CHANGE  600000 TO THE NEW PAGE SIZE                             Evalplot

2) CHANGE 1200000 TO TWO TIMES THE NEW PAGE SIZE                   Evalplot

Evalplot

SECTION SIZE                                                       Evalplot

------------                                                       Evalplot

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Evalplot

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Evalplot

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Evalplot

Evalplot

THE ONLY EXCEPTION TO THIS RULE IS THAT EACH TABLE OF DATA WHICH   Evalplot

USES A HISTOGRAM INTERPOLATION LAW CANNOT EXCEED HALF THE SIZE     Evalplot

OF THE IN CORE PAGE (PRESENTLY 600000/2=300000) WHICH IS ADEQUATE  Evalplot

FOR ALMOST ALL HISTOGRAM (E.G. MULTIGROUP) REPRESENTATIONS OF      Evalplot

A SINGLE TABLE (E.G. REACTION).                                    Evalplot

Evalplot

WHAT DATA CAN BE PLOTTED                                           Evalplot

------------------------                                           Evalplot

THIS CODE CAN PLOT VIRTUALLY ANY NEUTRON OR PHOTON CROSS SECTIONS  Evalplot

(MF=3 OR 23) AND ANY TABULATED ANGULAR OR ENERGY DISTRIBUTIONS OR  Evalplot

LEGENDRE COEFFICIENTS. WHAT IS ACTUALLY PLOTTED DEPENDS ON WHAT    Evalplot

DATA IS SELECTED BY THE USER.                                      Evalplot

Evalplot

SELECTION OF DATA                                                  Evalplot

-----------------                                                  Evalplot

DATA TO BE PLOTTED IS SPECIFIED BY INPUTTING UP TO 100 MAT/MF/MT   Evalplot

RANGES OR UP TO 100 ZA/MF/MT RANGES. IN ADDITION FOR EACH RANGE    Evalplot

THE USER MAY SPECIFY AN X RANGE (USUALLY ENERGY) AND THE TYPE OF   Evalplot

DATA TO BE PLOTTED (SEE: THE DESCRIPTION OF TYPES, BELOW).         Evalplot

Evalplot

THE X RANGE FOR MF = 1, 3, 23 AND 27 AND MF = 4 LEGENDRE           Evalplot

COEFFICIENTS WILL BE USED AS THE X LIMITS OF THE PLOTS, E.G.,      Evalplot

PLOT ENERGY DEPENDENT CROSS SECTIONS BETWEEN 1 AND 20 MEV.         Evalplot

Evalplot

THE X RANGE FOR MF = 4 AND 5 WILL BE USED TO ONLY SELECT ANGULAR   Evalplot

AND ENERGY DISTRIBUTION FOR WHICH THE INCIDENT NEUTRON ENERGY      Evalplot

IS IN THE X RANGE. E.G., ONLY PLOT ANGULAR DISTRIBUTIONS WHERE     Evalplot

THE INCIDENT NEUTRON ENERGY IS 1 TO 20 MEV.                        Evalplot

Evalplot

INTERACTIVE VS. BATCH MODE                                         Evalplot

--------------------------                                         Evalplot

VERSION 92-1 AND LATER VERSIONS OF THIS CODE ONLY USE A BATCH      Evalplot

MODE WHERE ALL REQUESTS ARE READ AND PROCESSED. EARLIER VERSIONS   Evalplot

OF THIS CODE HAD BOTH AN INTERACTIVE MODE (WHERE REQUESTS WHERE    Evalplot

READ AND EXECUTED ONE AT A TIME) AND A BATCH MODE. INTERACTIVE     Evalplot

MODE HAS BEEN DROPPED AND WILL NOT TO REINTRODUCED UNLESS THE      Evalplot

AUTHOR IS INFORMED BY USERS THAT THEY WERE USING THE INTERACTIVE   Evalplot

MODE.                                                              Evalplot

Evalplot

PLOT LAYOUT                                                        Evalplot

-----------                                                        Evalplot

VERSION 92-1 AND LATER VERSIONS OF THIS CODE WILL PLOT ALL         Evalplot

CURVES ON A SINGLE PLOT. EARLIER VERSIONS OF THIS CODE ALLOWED     Evalplot

THE OPTION TO HAVE,                                                Evalplot

MULTIPLE PLOTS - INDIVIDUAL SCALING                                Evalplot

MULTIPLE PLOTS - COMMON SCALING                                    Evalplot

SINGLE PLOT                                                        Evalplot

MULTILE PLOTS PER PLOT HAVE BEEN DROPPED AND WILL NOT BE           Evalplot

REINTRODUCED UNLESS IT IS DEMONSTRATED TO THE AUTHOR THAT THEY     Evalplot

ARE OF PRACTICAL USE IN SOME APPLICATION.                          Evalplot

Evalplot

PROCESSING OF DATA                                                 Evalplot

------------------                                                 Evalplot

IN THE CASE OF NEUTRON AND PHOTON CROSS SECTIONS (MF=3 OR 23)      Evalplot

AND PARAMETERS (MF=1 OR 27) ALL DATA IN A FILE (MF) IS READ        Evalplot

GROUPED TOGETHER BY TYPE (AS EXPLAINED BELOW) AND PLOTTED.         Evalplot

Evalplot

IN THE CASE OF ANGULAR AND ENERGY DISTRIBUTIONS (MF=4 OR 5) ONLY   Evalplot

ONE SECTION OF DATA AT A TIME IS READ AND PLOTTED.                 Evalplot

Evalplot

TYPES OF DATA (MF=1, 3, 23 AND 27 ONLY)                            Evalplot

---------------------------------------                            Evalplot

THESE DATA ARE DIVIDED INTO UP TO 18 TYPES AND EACH TYPE OF        Evalplot

DATA IS GROUPED TOGETHER AND PLOTTED (IF THE DATA IS ACTUALLY      Evalplot

PRESENT).                                                          Evalplot

Evalplot

WHAT TYPE OF DATA IS ACTUALLY PLOTTED CAN BE CONTROLLED BY USER    Evalplot

INPUT EITHER BASED ON SELECTED MAT/MF/MT OR ZA/MF/MT RANGES OR     Evalplot

BY EXPLICITLY SELECTING ONLY ONE TYPE OF DATA IS TO BE PLOTTED     Evalplot

(SEE THE DESCRIPTION OF INPUT BELOW).                              Evalplot

Evalplot

SIMPLE REQUESTS                                                    Evalplot

---------------                                                    Evalplot

GENERALLY EACH MAT/MF/MT OR ZA/MF/MT REQUESTED IS TREATED          Evalplot

SEPERATELY AND THE SPECIFIED DATA IS GROUPED BY TYPE AND PLOTTED.  Evalplot

FOR EXAMPLE, THE USER MAY SPECIFY USING ONE REQUEST THAT ALL       Evalplot

TYPES OF DATA BE PLOTTED OVER THE ENTIRE ENERGY RANGE AND USE      Evalplot

A SECOND REQUEST TO SPECIFY THAT ONE PARTICULAR TYPE OF DATA       Evalplot

BE PLOTTED OVER A SPECIFIC ENERGY RANGE.                           Evalplot

Evalplot

CHAINED REQUESTS                                                   Evalplot

----------------                                                   Evalplot

REQUESTS MAY ALSO BE CHAINED TOGETHER (SEE, THE DESCRIPTION OF     Evalplot

INPUT BELOW), WHERE A NUMBER OF REQUESTS MAY BE USED TO SELECT     Evalplot

DATA, BUT ONLY THE LAST REQUEST IN A CHAIN WILL CAUSE ALL SELECTED Evalplot

DATA TO BE PLOTTED. CHAINED REQUESTED ARE INDICATED ON INPUT BY    Evalplot

A SERIES OF REQUESTS FOR DATA TYPE = -1, EXCEPT FOR THE LAST       Evalplot

REQUEST OF THE CHAIN, WHICH MUST SPECIFY A TYPE DATA = 0 (ALL)     Evalplot

OR A POSITIVE NUMBER. UNLIKE SIMPLE REQUESTS, WHERE EACH WILL      Evalplot

PRODUCE ONE OR MORE PLOTS, WITH CHAINED REQUESTS THE ENTIRE        Evalplot

SERIES OF CHAINED REQUESTS WILL BE TREATED AS A SINGLE REQUEST     Evalplot

AND WILL PRODUCE ONE OR MORE PLOTS.                                Evalplot

Evalplot

FOR EXAMPLE, DATA TYPE = 1 WILL NORMALLY INCLUDE,                  Evalplot

MT =   1 - TOTAL                                                   Evalplot

=   2 - ELASTIC                                                 Evalplot

=   4 - TOTAL INELASTIC                                         Evalplot

=   5 - (N,REMAINDER)                                           Evalplot

=  18 - FISSION                                                 Evalplot

= 102 - CAPTURE                                                 Evalplot

IF YOU WISH TO EXCLUDE TOTAL INELASTIC FROM A PLOT YOU NEED ONLY   Evalplot

SPECIFY TWO CHAINED REQUESTS THE FIRST TO SELECT MT = 1 THROUGH    Evalplot

2 (TO INCLUDE TOTAL AND ELASTIC) AND A SECOND TO INCLUDE MT = 18   Evalplot

THROUGH 102. THE FIRST REQUEST SHOULD SPECIFY DATA TYPE = -1 AND   Evalplot

SECOND 1 (THIS WILL CHAIN THE 2 REQUESTS TOGETHER, SO THAT MT =1   Evalplot

THROUGH 2, AND MT = 18 THROUGH 102 ALL APPEAR ON THE SAME PLOT).   Evalplot

SINCE MT = 4 (TOTAL INELASTIC) IS NOT REQUESTED IT WILL NOT BE     Evalplot

PLOTTED.                                                           Evalplot

Evalplot

DEFINITION OF 18 DATA TYPES                                        Evalplot

------------------------------------------------------------------ Evalplot

NEUTRONS (MF = 3)                                                  Evalplot

-----------------                                                  Evalplot

(1) TOTAL, ELASTIC, CAPTURE, FISSION, TOTAL INELASTIC, REMAINDER   Evalplot

(2) (N,2N), (N,3N) AND (N,N' CHARGED PARTICLE)                     Evalplot

(3) (N,CHARGED PARTICLE)                                           Evalplot

(4) PARTICLE PRODUCTION (PROTON, DEUTERON, ETC.) AND DAMAGE        Evalplot

(5) TOTAL, FIRST, SECOND, ETC. CHANCE FISSION.                     Evalplot

(6) TOTAL INELASTIC, INELASTIC DISCRETE LEVELS AND CONTINUUM       Evalplot

(7) (N,P) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(8) (N,D) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(9) (N,T) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)              Evalplot

(10) (N,HE-3) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)          Evalplot

(11) (N,ALPHA) TOTAL AND LEVELS (ONLY IF LEVELS ARE GIVEN)         Evalplot

(12) PARAMETERS MU-BAR, XI AND GAMMA                               Evalplot

(13) NU-BAR - TOTAL, PROMPT AND DELAYED                            Evalplot

Evalplot

PHOTONS  (MF=23 AND 27)                                            Evalplot

-----------------------                                            Evalplot

(14) TOTAL, COHERENT, INCOHERENT, TOTAL PHOTOELECTRIC, TOTAL       Evalplot

PAIR PRODUCTION                                               Evalplot

(15) TOTAL AND SUBSHELL PHOTOELECTRIC                              Evalplot

(16) TOTAL, NUCLEAR AND ELECTRON PAIR PRODUCTION                   Evalplot

(17) COHERENT FORM FACTOR AND INCOHERENT SCATTERING FUNCTION       Evalplot

(18) REAL AND IMAGINARY SCATTERING FACTORS                         Evalplot

Evalplot

IDENTIFICATION OF DATA                                             Evalplot

----------------------                                             Evalplot

ALL PLOTS IDENTIFY THE TARGET, E.G., U-238 AND UNITS OF THE X AND  Evalplot

Y AXIS, E.G., X = ENERGY (MEV) OR COSINE (LAB), ETC., Y = CROSS    Evalplot

SECTION (BARNS) OR PROBABILITY/COSINE, ETC.                        Evalplot

Evalplot

FOR TYPES OF DATA (MF=1, 3, 23 AND 27) DIFFERENT REACTIONS (MT)    Evalplot

ARE GROUPED TOGETHER TO APPEAR ON THE SAME PLOT. THE TITLE AT      Evalplot

THE TOP OF THE PLOT WILL IDENTIFY THE TYPE OF DATA BEING PLOTTED   Evalplot

AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY EACH REACTION.    Evalplot

Evalplot

FOR ANGULAR AND ENERGY DISTRIBUTIONS (MF=4 OR 5) EACH PLOT WILL    Evalplot

CONTAIN DATA FOR A SINGLE REACTION (MT) AND DIFFERENT INCIDENT     Evalplot

NEUTRON ENERGIES. THE TITLE AT THE TOP OF THE PLOT WILL IDENTIFY   Evalplot

THE REACTION AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY      Evalplot

THE INCIDENT ENERGY.                                               Evalplot

Evalplot

FOR LEGENDRE COEFFICIENT THE DATA IN ENDF/B FORMAT WILL BE         Evalplot

INVERTED IN ORDER TO PRESENT EACH LEGENDRE COEFFICIENT VERSUS      Evalplot

INCIDENT ENERGY. THE TITLE AT THE TOP OF THE PLOT WILL IDENTIFY    Evalplot

THE REACTION AND THE LEGEND BOX WITHIN THE PLOT WILL IDENTIFY      Evalplot

THE LEGENDRE ORDER.                                                Evalplot

Evalplot

INPUT FILES                                                        Evalplot

-----------                                                        Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

2   INPUT LINES (BCD - 80 CHARACTERS/RECORD)                    Evalplot

9   MT DEFINITIONS (BCD - 80 CHARACTERS/RECORD)                 Evalplot

10   ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)                    Evalplot

12   SOFTWARE CHARACTERS (BCD - 80 CHARACTERS/RECORD)            Evalplot

Evalplot

OUTPUT FILES                                                       Evalplot

------------                                                       Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

3   OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                 Evalplot

16   PLOTTING UNIT                                               Evalplot

Evalplot

SCRATCH FILES                                                      Evalplot

-------------                                                      Evalplot

UNIT  DESCRIPTION                                                  Evalplot

----  -----------                                                  Evalplot

11  SCRATCH FILE (BINARY - 960000 WORDS/RECORD = 2*PAGE SIZE)    Evalplot

Evalplot

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Evalplot

---------------------------------------------------------------    Evalplot

UNIT  FILE NAME                                                    Evalplot

----  ----------                                                   Evalplot

2   EVALPLOT.INP                                                 Evalplot

3   EVALPLOT.LST                                                 Evalplot

9   MT.DAT                                                       Evalplot

10   ENDFB.IN    (OR AS INPUT PARAMETER)                          Evalplot

11   (SCRATCH)                                                    Evalplot

12   PLOT.CHR                                                     Evalplot

16   (PLOTTING UNIT...USUALLY A DUMMY)                            Evalplot

Evalplot

INPUT PARAMETERS                                                   Evalplot

----------------                                                   Evalplot

LINE COLUMNS FORMAT  DESCRIPTION                                   Evalplot

---- ------- ------  -----------                                   Evalplot

1   1-11   E11.4  LOWER X LIMIT OF PLOTTER                      Evalplot

12-22   E11.4  UPPER X LIMIT OF PLOTTER                      Evalplot

23-33   E11.4  LOWER Y LIMIT OF PLOTTER                      Evalplot

34-44   E11.4  UPPER Y LIMIT OF PLOTTER                      Evalplot

45-55    I11   NUMBER OF PLOTS PER FRAME IN X DIRECTION      Evalplot

56-66    I11   NUMBER OF PLOTS PER FRAME IN Y DIRECTION      Evalplot

67-70    F4.1  CHARACTER SIZE MULTIPLIER                     Evalplot

= 0 OR 1    - NORMAL CHARACTER SIZE           Evalplot

= OTHERWISE - CHARACTERS SCALED BY THIS       Evalplot

FACTOR.                         Evalplot

2   1-60    A60   ENDF/B DATA FILENAME                          Evalplot

(LEAVE BLANK FOR STANDARD = ENDFB.IN)         Evalplot

3   1-11    I11   RETRIEVAL CRITERIA                            Evalplot

= 0 - MAT                                     Evalplot

= 1 - ZA                                      Evalplot

12-22    I11   TYPE OF GRID                                  Evalplot

= 0 - TICK MARKS ON BORDER                    Evalplot

= 1 - SOLID AT COARSE INTERVALS               Evalplot

= 2 - DASHED AT COARSE INTERVALS              Evalplot

= 3 - SOLID AT FINE INTERVALS                 Evalplot

= 4 - DASHED AT FINE INTERVALS                Evalplot

= 5 - SOLID COARSE/DASHED FINE GRID           Evalplot

23-33    I11   SHOULD BORDER BE PLOTTED ON EACH PLOT         Evalplot

= 0 - NO                                      Evalplot

= 1 - YES                                     Evalplot

34-44    I11   LINE THICKNESS                                Evalplot

= 0 -  5 = BORDER/CURVES/CHARACTERS           Evalplot

=-1 - -5 = BORDER/CURVES (NOT CHARACTERS)     Evalplot

NOTE, THE GRID IS NEVER THICK.     Evalplot

45-55    I11   SHOULD TEMPERATURE BE PLOTTED.                Evalplot

= 0 - YES                                     Evalplot

= 1 - NO                                      Evalplot

56-66   E11.4  ALLOWABLE RATIO OF PLOT Y RANGE MAXIMUM TO    Evalplot

MINIMUM - IF THIS RATIO IS EXCEEDED THE Y     Evalplot

RANGE MINIMUM WILL BE CHANGED TO THE Y RANGE  Evalplot

MAXIMUM TIMES THIS RATIO.                     Evalplot

IF THIS RATIO IS NOT POSITIVE, IT IS          Evalplot

INTERPRETED TO MEAN NO LIMIT ON Y RANGE.      Evalplot

67-70    I4    BACKGROUND COLOR                              Evalplot

= 0         = BLACK                           Evalplot

= OTHERWISE = WHITE                           Evalplot

4-N   1- 6    I6    LOWER MAT OR ZA LIMIT                         Evalplot

7- 8    I2    LOWER MF LIMIT                                Evalplot

9-11    I3    LOWER MT LIMIT                                Evalplot

11-22   E11.4  LOWER X LIMIT (USUALLY ENERGY) - EV           Evalplot

23-28    I6    UPPER MAT OR ZA LIMIT                         Evalplot

29-30    I2    UPPER MF LIMIT                                Evalplot

31-33    I3    UPPER MT LIMIT                                Evalplot

34-44   E11.4  UPPER X LIMIT (USUALLY ENERGY) - EV           Evalplot

45-55   I11    TYPE OF DATA TO RETRIEVE AND PLOT             Evalplot

=  -1  - CHAIN THIS REQUEST TO THE NEXT ONE   Evalplot

=   0  - ALL                                  Evalplot

= 1-18 - TYPE AS SPECIFIED ABOVE              Evalplot

Evalplot

THERE MAY BE UP 100 MAT/MF/MT OR ZA/MF/MT REQUEST RANGES. INPUT    Evalplot

MUST BE TERMINATED BY A BLANK LINE.                                Evalplot

Evalplot

IF X LIMITS ARE NOT SPECIFIED (I.E., LOWER AND UPPER X LIMIT = 0)  Evalplot

THIS WILL BE INTERPRETED TO MEAN NO LIMIT AND ALL DATA WILL BE     Evalplot

PLOTTED OVER THEIR ENTIRE ENERGY RANGE, I.E., YOU NEED NOT         Evalplot

KNOW AND SPECIFY THE ACTUAL ENERGY LIMITS OF THE DATA.             Evalplot

Evalplot

EXAMPLE DEFINITION OF PLOTTER                                      Evalplot

-----------------------------                                      Evalplot

THE FIRST INPUT LINE DEFINES THE DIMENSIONS OF THE PLOTTER BEING   Evalplot

USED IN ANY UNITS (INCHES, CENTIMETERS, MILLIMETERS, ANYTHING)     Evalplot

WHICH APPLY TO THE PLOTTER. IN ADDITION THE FIRST LINE DEFINES     Evalplot

HOW MANY PLOTS SHOULD APPEAR ON EACH FRAME. THE PLOTTING AREA      Evalplot

DEFINED ON THE FIRST INPUT LINE MAY BE SUBDIVIDED INTO ANY NUMBER  Evalplot

OF PLOTS IN THE X AND Y DIRECTION. FOR EXAMPLE, TO PRODUCE A       Evalplot

SERIES OF FRAMES EACH CONTAINING 3 PLOTS IN THE X DIRECTION AND    Evalplot

2 PLOTS IN THE Y DIRECTION (6 PLOTS PER FRAME) COLUMN 45-55 OF     Evalplot

THE FIRST INPUT LINE SHOULD BE 3 AND COLUMNS 56-66 SHOULD BE 2.    Evalplot

Evalplot

IF THE LOCAL PLOTTER USES DIMENSIONS OF INCHES IN ORDER TO OBTAIN  Evalplot

10 X 10 INCH FRAMES WITH 3 X 2 PLOTS PER FRAME THE FIRST INPUT     Evalplot

LINE SHOULD BE,                                                    Evalplot

Evalplot

0.0       10.0      0.0     10.0              3          2       Evalplot

Evalplot

IF THE LOCAL PLOTTER USES DIMENSION OF MILLIMETERS THE SAME        Evalplot

PHYSICAL SIZE PLOT MAY BE OBTAINED IF THE FIRST INPUT LINE IS,     Evalplot

Evalplot

0.0      254.0      0.0    254.0              3          2       Evalplot

Evalplot

FOR SIMPLICITY THE FOLLOWING EXAMPLE INPUTS WILL NOT DISCUSS THE   Evalplot

PHYSICAL DIMENSIONS OF THE PLOTTER AND THE FIRST INPUT LINE WILL   Evalplot

IN ALL CASES INDICATE 10 X 10 INCH PLOTS WITH ONLY 1 PLOT PER      Evalplot

FRAME.                                                             Evalplot

Evalplot

ALL OF THE FOLLOWING EXAMPLE WILL USE,                             Evalplot

1) A DASHED GRID          (SECOND LINE, COLS. 12-22 = 2)           Evalplot

2) NO BORDER              (SECOND LINE, COLS. 23-33 = 0)           Evalplot

3) LINE THICKNESS -2      (SECOND LINE, COLS. 34-44 =-2)           Evalplot

4) TEMPERATURE ON PLOTS   (SECOND LINE, COLS. 45-55 = 0)           Evalplot

5) NO Y RANGE LIMIT       (SECOND LINE, COLS. 56-66 = 0.0)         Evalplot

Evalplot

EXAMPLE INPUT NO. 1                                                Evalplot

-------------------                                                Evalplot

FOR ALL THORIUM AND URANIUM ISOTOPES PLOT NEUTRON CROSS SECTIONS   Evalplot

ENTIRE ENERGY RANGE. IN ADDITION PLOT TYPE 1 DATA, MAJOR NEUTRON   Evalplot

CROSS SECTIONS OVER THE ENERGY RANGE 1 EV TO 1 KEV. USE THE        Evalplot

STANDARD FILENAME (ENDFB.IN) FOR THE ENDF/B DATA. THE FOLLOWING    Evalplot

6 INPUT LINES ARE REQUIRED,                                        Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  0           90999 3999                     0                 Evalplot

90000 3  0 1.00000+ 090999 3999 1.00000+ 3          1                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

EXAMPLE INPUT NO. 2                                                Evalplot

-------------------                                                Evalplot

PLOT FE-56 ELASTIC AND INELASTIC ANGULAR DISTRIBUTIONS BETWEEN     Evalplot

1 AND 20 MEV. THE FOLLOWING 6 INPUT LINES ARE REQUIRED,            Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

26056 4  2 1.00000+ 626056 4  2 2.00000+ 7          0                 Evalplot

26056 4  4 1.00000+ 626056 4  4 2.00000+ 7          0                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

EXAMPLE INPUT NO. 3 (CHAINED INPUT)                                Evalplot

-----------------------------------                                Evalplot

FOR ALL THORIUM AND URANIUM ISOTOPES PLOT TOTAL, ELASTIC ,CAPTURE  Evalplot

AND FISSION, BUT NOT INELASTIC CROSS SECTIONS OVER THERE ENTIRE    Evalplot

ENERGY RANGE AND FROM 1 KEV TO 1 MEV. THE FOLLOWING 8 INPUT        Evalplot

LINES ARE REQUIRED,                                                Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

ENDFB.IN                                                               Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  1           90999 3  2                    -1                 Evalplot

90000 3 18           90999 3102                     1                 Evalplot

90000 3  1 1.00000+ 390999 3  2 1.00000+ 6         -1                 Evalplot

90000 3 18 1.00000+ 390999 3102 1.00000+ 6          1                 Evalplot

(BLANK LINE MUSE FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

NOTE, THIS EXAMPLE INCLUDES 2 CHAINED REQUESTED - INPUT LINES 3    Evalplot

AND 4 SELECTING DATA AND PRODUCING A PLOT OVER THE ENTIRE ENERGY   Evalplot

RANGE AND INPUT LINES 5 AND 6 SELECTING THE SAME DATA AND          Evalplot

PRODUCING A PLOT FROM 1 KEV TO 1 MEV.                              Evalplot

Evalplot

ANY NUMBER OF REQUEST LINES MAY TO CHAINED TOGETHER TO SELECT      Evalplot

DATA. THE CHAIN ENDS WHERE THE TYPE OF DATA (COLS. 45-55) IS NOT   Evalplot

NEGATIVE AND THEN THE SELECTED DATA WILL BE PLOTTED.               Evalplot

Evalplot

EXAMPLE INPUT NO. 4                                                Evalplot

-------------------                                                Evalplot

FOR THE SAME EXAMPLE AS ABOVE, EXCEPT USE A DIFFERENT FILENAME     Evalplot

FOR THE ENDF/B DATA TO READ FROM A FILE TREE STRUCTURE. THE        Evalplot

FOLLOWING 8 INPUT LINES ARE REQUIRED,                              Evalplot

Evalplot

0.0       10.0       0.0       10.0               3          2      Evalplot

EVALUATION/ENDFB6/THORIUM                                              Evalplot

1          2         0         -2          0 0.0             Evalplot

90000 3  1           90999 3  2                    -1                 Evalplot

90000 3 18           90999 3102                     1                 Evalplot

90000 3  1 1.00000+ 390999 3  2 1.00000+ 6         -1                 Evalplot

90000 3 18 1.00000+ 390999 3102 1.00000+ 6          1                 Evalplot

(BLANK LINE MUST FOLLOW LAST REQUEST)                                  Evalplot

Evalplot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Evalplot

Evalplot

THIS PROGRAM USES A SIMPLE CALCOMP LIKE INTERFACE INVOLVING       Evalplot

ONLY 6 SUBROUTINES,                                               Evalplot

Evalplot

STARPLOT  - INITIALIZE PLOTTER                                    Evalplot

NEXTPLOT  - CLEAR THE SCREEN FOR THE NEXT PLOT                    Evalplot

ENDPLOTS  - TERMINATE PLOTTING                                    Evalplot

Evalplot

PLOT(X,Y,IPEN)        - DRAW OR MOVE FROM LAST LOCATION TO (X,Y), Evalplot

END OF CURRENT PLOT OR END OF PLOTTING.   Evalplot

IPEN =   2 - DRAW                                           Evalplot

=   3 - MOVE                                           Evalplot

Evalplot

PEN(IPEN)             - SELECT COLOR.                             Evalplot

IPEN- COLOR = 1 TO N (N = ANY POSITIVE INTEGER)              Evalplot

Evalplot

BOXCOLOR(X,Y,IFILL,IBORDER) - FILL A RECTANGULAR BOX DEFINED      Evalplot

BY THE X AND Y CORNERS - X(1),      Evalplot

X(2), Y(1),Y(2)                     Evalplot

IFILL          - COLOR TO FILL BOX WITH              Evalplot

IBORDER  - COLOR OF BOX BORDER                 Evalplot

Evalplot

IN ORDER TO INTERFACE THIS PROGRAM FOR USE ON ANY PLOTTER WHICH    Evalplot

DOES NOT USE THE ABOVE CONVENTIONS IT IS MERELY NECESSARY FOR THE  Evalplot

THE USER TO WRITE 6 SUBROUTINES WITH THE NAMES PLOTS, PLOT AND PEN Evalplot

WITH THE SUBROUTINE ARGUMENTS DESCRIBED ABOVE AND TO THEN CALL THE Evalplot

LOCAL EQUIVALENT ROUTINES.                                         Evalplot

Evalplot

COLOR PLOTS                                                        Evalplot

------------------------------------------------------------------ Evalplot

TO SELECT PLOTTING COLORS SUBROUTINE PEN (DESCRIBED ABOVE) IS USED Evalplot

TO SELECT ONE OF THE AVAILABLE COLORS. IF YOU HAVE COLOR ON YOUR   Evalplot

PLOTTER YOU SHOULD PROVIDE A SUBROUTINE PEN TO SELECT COLORS.      Evalplot

Evalplot

BLACK AND WHITE PLOTS                                              Evalplot

------------------------------------------------------------------ Evalplot

WHEN PRODUCING BLACK AND WHITE PLOTS SUBROUTINE PEN NEED MERELY    Evalplot

BE A DUMMY SUBROUTINE TO IGNORE ANY ATTEMPT TO CHANGE COLORS,      Evalplot

Evalplot

SUBROUTINE PEN(IPEN)                                               Evalplot

RETURN                                                             Evalplot

END                                                                Evalplot

Evalplot

SIMILAR BOXCOLOR CAN BE A DUMMY                                    Evalplot

Evalplot

SUBROUTINE BOXCOLOR(X,Y,IFILL,IBORDER)                             Evalplot

RETURN                                                             Evalplot

END                                                                Evalplot

Evalplot

CHARACTER SET                                                      Evalplot

------------------------------------------------------------------ Evalplot

THIS PROGRAM USES COMPUTER AND PLOTTER DEVICE INDEPENDENT SOFTWARE Evalplot

CHARACTERS. THIS PROGRAM COMES WITH A FILE THAT DEFINES THE PEN    Evalplot

STROKES REQUIRED TO DRAW ALL CHARACTERS ON AN IBM KEYBOARD (UPPER  Evalplot

AND LOWER CASE CHARACTERS, NUMBERS, ETC.) PLUS AN ALTERNATE SET OF Evalplot

ALL UPPER AND LOWER CASE GREEK CHARACTERS AND ADDITIONAL SPECIAL   Evalplot

SYMBOLS.                                                           Evalplot

Evalplot

THE SOFTWARE CHARACTER TABLE CONTAINS X AND Y AND PEN POSITIONS TO Evalplot

DRAW EACH CHARACTER. IF YOU WISH TO DRAW ANY ADDITIONAL CHARACTERS Evalplot

OR TO MODIFY THE FONT OF THE EXISTING CHARACTERS YOU NEED ONLY     Evalplot

MODIFY THIS TABLE.                                                 Evalplot

Evalplot

ADDITIONAL FONTS                                                   Evalplot

----------------                                                   Evalplot

THIS PROGRAM COMES WITH 3 COMPLETE SETS OF THE SAME CHARACTERS     Evalplot

USING DIFFERENT FONTS. FOR SPEED IN PLOTTING IT IS RECOMMENDED     Evalplot

THAT YOU USE THE SIMPLEX FONT. FOR FINISHED PLOTS SUITABLE FOR     Evalplot

PUBLICATION, BUT REQUIRING MORE TIME TO GENERATE A PLOT, IT IS     Evalplot

RECOMMENDED THAT YOU USE THE DUPLEX OR COMPLEX FONT - YOU CAN      Evalplot

EXPERIMENT WITH ANY OF THE 3 FONTS TO DETERMINE WHICH BEST MEETS   Evalplot

YOUR NEEDS.                                                        Evalplot

Evalplot

TO USE ANY ONE OF THE FONTS MERELY BY SURE THAT IT IS DEFINED AS   Evalplot

UNIT 12 FOR INPUT (IF USING STANDARD FILENAMES IT SHOULD BE        Evalplot

NAMED PLOT.CHR). SO THAT SWITCHING FONTS CAN BE SIMPLY DONE        Evalplot

MERELY BY COPYING THE FONT THAT YOU WANT TO THE UNIT 12 THAT       Evalplot

YOU ARE USING FOR INPUT.                                           Evalplot

Evalplot

CONTROL CHARACTERS                                                 Evalplot

------------------------------------------------------------------ Evalplot

IN THE SOFTWARE CHARACTER TABLE ALL CHARACTERS TO BE PLOTTED WILL  Evalplot

HAVE PEN POSITION = 2 (DRAW) OR = 3 (MOVE). IN ADDITION THE TABLE  Evalplot

CURRENTLY CONTAINS 4 CONTROL CHARACTERS,                           Evalplot

Evalplot

PEN POSITION = 0                                                   Evalplot

----------------                                                   Evalplot

SHIFT THE NEXT PRINTED CHARACTER BY X AND Y. 3 CONTROL CHARACTERS  Evalplot

ARE PRESENTLY INCLUDED IN THE SOFTWARE CHARACTER TABLE TO ALLOW    Evalplot

SHIFTING.                                                          Evalplot

Evalplot

{   = SHIFT UP (FOR SUPERSCRIPTS..............X= 0.0, Y= 0.5)      Evalplot

}   = SHIFT DOWN (FOR SUBSCRIPTS..............X= 0.0, Y=-0.5)      Evalplot

\   = SHIFT LEFT 1 CHARACTER (FOR BACKSPACE...X=-1.0, Y= 0.0)      Evalplot

Evalplot

PEN POSITION =-1                                                   Evalplot

----------------                                                   Evalplot

SELECT THE NEXT PRINTED CHARACTER FROM THE ALTERNATE CHARACTER     Evalplot

SET. AT PRESENT THIS CONTROL CHARACTER IS,                         Evalplot

Evalplot

|   = SWITCH TO ALTERNATE CHARACTER SET                            Evalplot

Evalplot

THESE 4 CONTROL CHARACTERS ARE ONLY DEFINED BY THE VALUE OF THE    Evalplot

PEN POSITION IN THE SOFTWARE CHARACTER TABLE (I.E., THEY ARE NOT   Evalplot

HARD WIRED INTO THIS PROGRAM). AS SUCH BY MODIFYING THE SOFTWARE   Evalplot

CHARACTER TABLE THE USER HAS THE OPTION OF DEFINING ANY CONTROL    Evalplot

CHARACTERS TO MEET SPECIFIC NEEDS.                                 Evalplot

Evalplot

THESE CHARACTERS MAY BE USED IN CHARACTER STRINGS TO PRODUCE       Evalplot

SPECIAL EFFECTS. FOR EXAMPLE, TO PLOT SUBSCRIPT 5, B, SUPERSCRIPT  Evalplot

10 USE THE STRING,                                                 Evalplot

Evalplot

}5B{1{0                                                            Evalplot

Evalplot

TO PLOT B, SUBSCRIPT 5 AND SUPERSCRIPT 10 WITH THE 5 DIRECTLY      Evalplot

BELOW THE 1 OF THE 10 WE CAN USE THE BACKSPACE CHARACTER TO        Evalplot

POSITION THE 1 DIRECTLY ABOVE THE 5 USING THE STRING,              Evalplot

Evalplot

B}5\{1{0                                                           Evalplot

Evalplot

TO PLOT UPPER CASE GREEK GAMMA FOLLOWED BY THE WORD TOTAL (I.E.,   Evalplot

RESONANCE TOTAL WIDTH) USE THE STRING.                             Evalplot

Evalplot

|G TOTAL                                                           Evalplot

Evalplot

NOTE, WHEN THESE CONTROL CHARACTERS ARE USED THEY ONLY EFFECT THE  Evalplot

NEXT 1 PRINTED CHARACTER (SEE, ABOVE EXAMPLE OF PLOTTING SUPER-    Evalplot

SCRIPT 10 WHERE THE SHIFT UP CONTROL CHARACTER WAS USED BEFORE THE Evalplot

1 AND THEN AGAIN BEFORE THE 0 AND THE BACKSPACE AND SHIFT UP       Evalplot

CONTROL CHARACTERS WERE USED IN COMBINATION).                      Evalplot

Evalplot

IF THESE 4 CONTROL CHARACTERS ARE NOT AVAILABLE ON YOUR COMPUTER   Evalplot

YOU CAN MODIFY THE SOFTWARE CHARACTER TABLE TO USE ANY OTHER 4     Evalplot

CHARACTERS THAT YOU DO NOT NORMALLY USE IN CHARACTER STRINGS (FOR  Evalplot

DETAILS SEE THE SOFTWARE CHARACTER TABLE).                         Evalplot

Evalplot

STANDARD/ALTERNATE CHARACTER SETS                                  Evalplot

------------------------------------------------------------------ Evalplot

THE SOFTWARE CHARACTER TABLE CONTAINS 2 SETS OF CHARACTERS WHICH   Evalplot

ARE A STANDARD SET (ALL CHARACTERS ON AN IBM KEYBOARD) AND AN      Evalplot

ALTERNATE SET (UPPER AND LOWER CASE GREEK CHARACTERS AND SPECIAL   Evalplot

CHARACTERS). TO DRAW A CHARACTER FROM THE ALTERNATE CHARACTER SET  Evalplot

PUT A RIGHT BRACKET CHARACTER (|) BEFORE A CHARACTER (SEE THE      Evalplot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Evalplot

CONTROL CHARACTER WILL ONLY EFFECT THE NEXT 1 PLOTTED CHARACTER.   Evalplot

Evalplot

SUB AND SUPER SCRIPTS                                              Evalplot

------------------------------------------------------------------ Evalplot

TO DRAW SUBSCRIPT PRECEED A CHARACTER BY }. TO DRAW SUPERSCRIPT    Evalplot

PRECEED A CHARACTER BY { (SEE THE ABOVE EXAMPLE AND THE SOFTWARE   Evalplot

CHARACTER TABLE FOR DETAILS). THESE CONTROL CHARACTER WILL ONLY    Evalplot

EFFECT THE NEXT 1 PLOTTED CHARACTER.                               Evalplot

Evalplot

BACKSPACING                                                        Evalplot

------------------------------------------------------------------ Evalplot

TO BACKSPACE ONE CHARACTER PRECEED A CHARACTER BY \ (SEE, THE      Evalplot

ABOVE EXAMPLE AND THE SOFTWARE CHARACTER TABLE FOR DETAILS). THIS  Evalplot

CONTROL CHARACTER WILL PERFORM A TRUE BACKSPACE AND WILL EFFECT    Evalplot

ALL FOLLOWING CHARACTERS IN THE SAME CHARACTER STRING.             Evalplot

Evalplot

PLOT DIMENSIONS                                                    Evalplot

---------------                                                    Evalplot

ARE DEFINED BY USER INPUT. INTERNALLY THE PROGRAM WILL CREATE A    Evalplot

PLOT IN APPROXIMATELY A4 OR 8-1/2 BY 11 INCH FORMAT. DURING        Evalplot

OUTPUT THE PLOT IS TRANSFORMED TO THE UNITS (INCHES, CENTIMETERS,  Evalplot

MILLIMETERS, WHATEVER) OF THE PLOTTER BEING USED AND OUTPUT.       Evalplot

Evalplot

===== PLOTTER/GRAPHICS TERMINAL INTERFACE ============================= Evalplot

======================================================================= Evalplot

======================================================================= Fixup

Fixup

PROGRAM FIXUP                                                      Fixup

VERSION 84-1 (NOVEMBER 1984)                                       Fixup

VERSION 86-1 (JANUARY 1986) *IMPROVED BASED ON USER COMMENTS       Fixup

*FORTRAN-77/H VERSION                  Fixup

VERSION 86-2 (JUNE 1986)    *ALLOW CREATION OF SECTIONS OF CROSS   Fixup

SECTIONS WHICH ARE NOT PRESENT IN     Fixup

THE ORIGINAL EVALUATION               Fixup

VERSION 88-1 (JULY 1988)    *OPTION...INTERNALLY DEFINE ALL I/O    Fixup

FILE NAMES (SEE, SUBROUTINE FILEIO    Fixup

FOR DETAILS).                         Fixup

*IMPROVED BASED ON USER COMMENTS.      Fixup

VERSION 89-1 (JANUARY 1989) *PSYCHOANALYZED BY PROGRAM FREUD TO    Fixup

INSURE PROGRAM WILL NOT DO ANYTHING   Fixup

CRAZY.                                Fixup

*UPDATED TO USE NEW PROGRAM CONVERT    Fixup

KEYWORDS.                             Fixup

*ADDED LIVERMORE CIVIC COMPILER        Fixup

CONVENTIONS.                          Fixup

VERSION 89-2 (MARCH 1989)   *ADDED ENDF/B-VI SUMMATION RULES AND   Fixup

DEFINED MF AND MT NUMBERS. PROGRAM    Fixup

WILL NOW USE MF=1, MT=451 TO DEFINE   Fixup

THE ENDF/B FORMAT OF THE DATA (E.G.,  Fixup

ENDF/B-VI OR EARLIER) AND USE THE     Fixup

CORRECT SUMMATION RULES FOR EACH      Fixup

VERSION OF THE ENDF/B FORMAT. IF      Fixup

MF=1, MT=451 IS NOT PRESENT PROGRAM   Fixup

WILL USE ENDF/B-VI SUMMATION          Fixup

CONVENTIONS AS A DEFAULT.             Fixup

VERSION 90-1 (JUNE 1990)    *UPDATED BASED ON USER COMMENTS        Fixup

*ADDED PHOTON INTERACTION, MF=23       Fixup

VERSION 91-1 (JUNE 1991)    *ADDED FORTRAN SAVE OPTION             Fixup

*NEW MORE CONSISTENT ENERGY OUTPUT     Fixup

ROUTINE                               Fixup

VERSION 92-1 (JANUARY 1992) *ADDED OPTION TO CALCULATE RATIOS,     Fixup

E.G., CAPTURE/FISSION AND PRODUCTS,   Fixup

NU-BAR*FISSION - AND OUTPUT THE       Fixup

RESULTS IN THE ENDF/B FORMAT (SEE,    Fixup

BELOW - CREATING RATIOS AND PRODUCTS) Fixup

*ALLOW TOTAL NU-BAR (MF=1, MT=452) TO  Fixup

BE USED IN DEFINING RATIOS OR         Fixup

PRODUCTS.                             Fixup

*ALLOW ALL CROSS SECTIONS TO BE PUT    Fixup

ON A UNIFORM ENERGY GRID.             Fixup

*NOTE, CHANGE IN INPUT FORMAT FOR      Fixup

RANGES OF MT NUMBERS                  Fixup

*COMPLETELY CONSISTENT I/O ROUTINES -  Fixup

TO MINIMIZE COMPUTER DEPENDENCE.      Fixup

VERSION 93-1 (JULY 1993)    *CORRECTED ALGORITHM TO CREATE UNIFORM Fixup

ENERGY GRID.                          Fixup

VERSION 94-1 (JANUARY 1993) *VARIABLE ENDF/B DATA FILENAMES        Fixup

TO ALLOW ACCESS TO FILE STRUCTURES    Fixup

(WARNING - INPUT PARAMETER FORMAT     Fixup

HAS BEEN CHANGED)                     Fixup

*INCREASED PAGE SIZE FROM 1002 TO      Fixup

12000 DATA POINTS.                    Fixup

*CLOSE ALL FILES BEFORE TERMINATING    Fixup

(SEE, SUBROUTINE ENDIT)               Fixup

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Fixup

*IMPROVED COMPUTER INDEPENDENCE        Fixup

*ALL DOUBLE PRECISION                  Fixup

*ON SCREEN OUTPUT                      Fixup

*UNIFORM TREATMENT OF ENDF/B I/O       Fixup

*IMPROVED OUTPUT PRECISION             Fixup

*DEFINED SCRATCH FILE NAMES            Fixup

*INCREASED PAGE SIZE FROM 12000 TO     Fixup

36000 DATA POINTS.                    Fixup

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Fixup

POINT READ FOR MORE DIGITS            Fixup

*UPDATED TEST FOR ENDF/B FORMAT        Fixup

VERSION BASED ON RECENT FORMAT CHANGE Fixup

*GENERAL IMPROVEMENTS BASED ON         Fixup

USER FEEDBACK                         Fixup

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Fixup

MF=1, MT-451.                         Fixup

*FIXED CREATION OF SECTIONS            Fixup

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Fixup

USER FEEDBACK                         Fixup

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Fixup

*SUMMATION RULES ARE DEFINED BASED     Fixup

ON CONTENTS OF TABLES.                Fixup

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACK Fixup

*INCREASED PAGE SIZE FROM 36000 TO     Fixup

60000 DATA POINTS.                    Fixup

VERS. 2005-1 (JAN. 2005)    *UPDATED MT CREATION TO ALLOW MAT =0   Fixup

INDICATING CREATE FOR ALL MATS.       Fixup

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII        Fixup

*INCREASED PAGE SIZE FROM 60,000 TO    Fixup

600,000 DATA POINTS.                  Fixup

Fixup

OWNED, MAINTAINED AND DISTRIBUTED BY                               Fixup

------------------------------------                               Fixup

THE NUCLEAR DATA SECTION                                           Fixup

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Fixup

P.O. BOX 100                                                       Fixup

A-1400, VIENNA, AUSTRIA                                            Fixup

EUROPE                                                             Fixup

Fixup

ORIGINALLY WRITTEN BY                                              Fixup

------------------------------------                               Fixup

DERMOTT E. CULLEN                                                  Fixup

UNIVERSITY OF CALIFORNIA                                           Fixup

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Fixup

L-159                                                              Fixup

P.O. BOX 808                                                       Fixup

LIVERMORE, CA 94550                                                Fixup

U.S.A.                                                             Fixup

TELEPHONE  925-423-7359                                            Fixup

E. MAIL    CULLEN1@LLNL.GOV                                        Fixup

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Fixup

Fixup

PURPOSE                                                            Fixup

=======                                                            Fixup

THIS PROGRAM IS DESIGNED TO READ EVALUATED DATA IN THE ENDF/B      Fixup

FORMAT, PERFORM CORRECTIONS AND OUTPUT THE RESULT IN THE ENDF/B    Fixup

FORMAT. TWO TYPES OF CORRECTIONS ARE POSSIBLE (1) AUTOMATIC AND    Fixup

(2) OPTIONAL (BASED ON USER INPUT) CORRECTIONS.                    Fixup

Fixup

ONE OF THE MOST IMPORTANT FUNCTIONS OF THIS PROGRAM IS TO          Fixup

RE-DEFINE ALL REDUNDANT CROSS SECTIONS (E.G. TOTAL) TO BE EXACTLY  Fixup

EQUAL TO THE SUM OF ITS PARTS. THIS PROCEDURE ELIMINATES THE       Fixup

PROBLEM WITH MANY ENDF/B EVALUATIONS, WHERE DUE TO THE USE OF      Fixup

NON-LINEAR INTERPOLATION LAWS THE TOTAL MAY BE EQUAL TO THE SUM    Fixup

OF ITS PARTS AT ALL TABULATED ENERGIES, BUT BASED ON THE           Fixup

INTERPOLATION LAWS IT CAN BE QUITE DIFFERENT AT ENERGIES BETWEEN   Fixup

TABULATED ENERGIES.                                                Fixup

Fixup

AUTOMATIC CHECKS/CORRECTIONS                                       Fixup

============================                                       Fixup

(1) CHECK THAT MAT/MF/MT DOES NOT CHANGE UNLESS A MEND/FEND/SEND   Fixup

LINE IS READ. IF MAT/MF/MT CHANGES A WARNING MESSAGE IS        Fixup

PRINTED BUT NO CORRECTIVE ACTION IS TAKEN.                     Fixup

(2) ALL LINES WITHIN A GIVEN MAT WILL BE SEQUENTIALLY NUMBERED     Fixup

ON OUTPUT.                                                     Fixup

Fixup

OPTIONAL CHECKS/CORRECTIONS                                        Fixup

===========================                                        Fixup

THE FOLLOWING NUMBERS CORRESPOND TO THE INPUT DATA OPTION COLUMNS  Fixup

(SEE THE DESCRIPTION OF THE INPUT BELOW)                           Fixup

Fixup

(1) CORRECT ZA AND AWR IN ALL SECTIONS. CHECK TO INSURE THAT THE   Fixup

C1 AND C2 VALUES (ZA AND AWR) ARE THE SAME IN ALL SECTIONS.    Fixup

THE C1 AND C2 OF THE FIRST SECTION READ ARE ASSUMED TO BE      Fixup

CORRECT AND ARE USED FOR COMPARISON. IF THE C1 AND/OR C2 OF    Fixup

THE FIRST SECTION ARE NOT POSITIVE AN ERROR MESSAGE IS OUTPUT  Fixup

AND THE MATERIAL IS COPIED WITHOUT CHANGE.                     Fixup

NOTE....TO CHANGE THE ZA AND/OR AWR OF ANY MATERIAL IT IS      Fixup

MERELY NECESSARY TO CHANGE THE ZA AND/OR AWR IN THE FIRST      Fixup

SECTION OF THE MATERIAL AND USE THIS OPTION TO AUTOMATICALLY   Fixup

CHANGE ALL OTHER SECTIONS.                                     Fixup

(2) CORRECT CROSS SECTION (MF=3) THRESHOLDS. THE Q-VALUE AND AWR   Fixup

ARE USED TO DERIVE THE REACTION THRESHOLD USING THE RELATION,  Fixup

Fixup

E-THRESHOLD = -(Q-VALUE)*(AWRE+1.0)/AWRE                       Fixup

Fixup

IF THE THRESHOLD IS POSITIVE THE CROSS SECTION IS CHECKED TO   Fixup

INSURE THAT THE FIRST TABULATED POINT IS AT THE THRESHOLD AND  Fixup

HAS A ZERO CROSS SECTION. IF NOT, THE CROSS SECTION WILL BE    Fixup

CHANGED.                                                       Fixup

(A) IF THE FIRST TABULATED POINT IS ABOVE THE THRESHOLD AND    Fixup

HAS A ZERO CROSS SECTION, THE POINT IS DELETED AND A POINT Fixup

IS INSERTED AT THE THRESHOLD.                              Fixup

(B) IF THE FIRST TABULATED POINT IS ABOVE THE THRESHOLD AND    Fixup

HAS A NON-ZERO CROSS SECTION, A POINT WITH ZERO CROSS      Fixup

SECTION IS INSERTED AT THE THRESHOLD.                      Fixup

(C) IF THE FIRST TABULATED POINT IS BELOW THE THRESHOLD AND    Fixup

HAS A NON-ZERO CROSS SECTION, ALL POINTS BELOW THE         Fixup

THRESHOLD ARE DELETED AND A POINT WITH ZERO CROSS SECTION  Fixup

IS INSERTED AT THE THRESHOLD.                              Fixup

(3) EXTEND ALL CROSS SECTIONS (MF=3) TO 20 MEV. IF THE TABULATED   Fixup

CROSS SECTION ENDS BELOW 20 MEV IT WILL BE EXTENDED TO 20 MEV  Fixup

AS EITHER ZERO (IMOPS(3)=1) OR CONSTANT (IMOPS(3)=2) EQUAL     Fixup

TO THE LAST TABULATED VALUE.                                   Fixup

(4) ALLOW REACTION (MF=3, ANY MT) DELETION. ALL SPECIFIED          Fixup

REACTIONS WILL BE DELETED WHEN THE DATA IS READ FROM THE       Fixup

INPUT ENDF/B DATA FILE AND WILL NOT BE IN THE OUTPUT ENDF/B    Fixup

DATA FILE. WARNING DELETED REACTIONS MAY NOT BE USED TO DEFINE Fixup

ANY RECONSTRUCTED REACTIONS (I.E. REACTIONS DEFINED BY SUMMING Fixup

OTHER REACTIONS). SINCE DELETED REACTIONS ARE DELETED DURING   Fixup

READING IT IS AS IF THEY NEVER EXISTED AND IF ANY DELETED      Fixup

REACTION IS REQUIRED LATER TO DEFINE ANY SUM AN ERROR WILL     Fixup

RESULT. THE USER MAY SPECIFY THAT THE DELETION RULES ARE TO BE Fixup

READ FROM INPUT (IMOPS(4)=1) OR THAT THE BUILT IN SUMMATION    Fixup

RULES ARE TO BE USED (MOPS(4)=2). AT THE PRESENT TIME THE      Fixup

BUILT-IN DELETION RULES ARE THAT NO SECTIONS SHOULD BE DELETED Fixup

(THE USER MAY OVERRIDE THIS CONVENTION BY INPUT).              Fixup

(5) ALLOW REACTION (MF=3, ANY MT) RECONSTRUCTION BY SUMMING OTHER  Fixup

REACTIONS. IN ORDER TO OPTIMIZE THE RUNNING TIME OF THIS       Fixup

PROGRAM CARE SHOULD BE EXERCISED TO MINIMIZE THE NUMBER OF     Fixup

TIMES THAT EACH CONTRIBUTING CROSS SECTION MUST BE USED.       Fixup

THE USED MAY SPECIFY THAT THE SUMMATION RULES ARE TO BE READ   Fixup

AS INPUT (IMOPS(5)=1) OR THAT THE BUILT IN SUMMATION RULES     Fixup

ARE TO BE USED (IMOPS(5)=2). THE BUILT IN SUMMATION RULES ARE  Fixup

DESIGNED TO USE ENDF/B CONVENTIONS AND TO MINIMIZE THE NUMBER  Fixup

OF TIMES THAT EACH CROSS SECTION IS USED.                      Fixup

(6) INSURE THAT ALL CROSS SECTIONS ARE NON-NEGATIVE (I.E. ARE      Fixup

ZERO OR POSITIVE). DURING READING ALL NEGATIVE CROSS SECTIONS  Fixup

WILL BE SET EQUAL TO ZERO AND TREATED AS SUCH DURING ALL       Fixup

SUBSEQUENT SUMMATIONS AND ENDF/B OUTPUT.                       Fixup

NOTE...THIS OPTION SHOULD NEVER BE USED WITH DATA CONTAINING   Fixup

BACKGROUND CROSS SECTIONS WHICH MAY BE NEGATIVE. ONLY AFTER    Fixup

THE RESONANCE CONTRIBUTION HAS BEEN ADDED TO THE BACKGROUND    Fixup

TO DEFINE THE ACTUAL CROSS SECTION IS IT VALID TO ELIMINATE    Fixup

NEGATIVE CROSS SECTIONS.                                       Fixup

NOTE...THIS OPTION MAY BE USED TO DELETE NEGATIVE ELASTIC      Fixup

CROSS SECTIONS THAT MAY RESULT FROM RECONSTRUCTING CROSS       Fixup

SECTIONS FROM SINGLE LEVEL BREIT-WIGNER PARAMETERS. IF THE     Fixup

TOTAL CROSS SECTION IS THEN RECONSTRUCTED USING THE CORRECTED  Fixup

ELASTIC CROSS SECTION THE TOTAL WILL BE POSITIVE DUE TO THE    Fixup

CONTRIBUTIONS OF CAPTURE AND FISSION (THUS AVOIDING NUMERICAL  Fixup

INSTABILITY PROBLEMS DURING SELF-SHIELDING CALCULATIONS).      Fixup

(7) WITHIN EACH SECTION OF CROSS SECTIONS DELETE ENERGIES THAT     Fixup

ARE NOT IN ASCENDING ENERGY ORDER (ENERGY REPETITION IS O.K.)  Fixup

(8) WITHIN EACH SECTION OF CROSS SECTIONS ELIMINATE DUPLICATE      Fixup

POINTS (SUCCESSIVE POINTS WITH THE SAME ENERGY-CROSS SECTION). Fixup

(9) TEST THAT ALL SECTIONS ARE IN ASCENDING MAT/MF/MT ORDER.       Fixup

IF NOT, NO CORRECTIVE ACTION WILL BE TAKEN, ONLY AN ERROR      Fixup

MESSAGE WILL BE OUTPUT.                                        Fixup

(10) CHECK MF/MT FOR EACH SECTION TO INSURE THAT THEY ARE DEFINED   Fixup

IN THE ENDF/B FORMAR MANUAL. IF THEY ARE NOT DEFINED AN ERROR  Fixup

MESSAGE IS PRINTED, BUT NO CORRECTIVE ACTION IS TAKEN.         Fixup

(11) ALLOW SECTIONS WHICH ARE NOT PRESENT IN THE ORIGINAL (INPUT)   Fixup

EVALUATION TO BE CREATED. NORMALLY THIS PROGRAM WILL ONLY      Fixup

RECONSTRUCT AND OUTPUT SECTIONS IF THE SECTION IS PRESENT      Fixup

IN THE ORIGINAL EVALUATION. THIS PROCEDURE IS FOLLOWED BECAUSE Fixup

NORMALLY THE PROGRAM DOES NOT KNOW HOW TO DEFINE THE CONTENTS  Fixup

OF THE FIRST TWO LINES OF THE SECTION (E.G., Q-VALUE,          Fixup

TEMPERATURE, INITIAL AND FINAL STATES). THIS OPTION MAY BE     Fixup

USED TO ALLOW THE PROGRAM TO READ AND SAVE A TABLE DEFINING    Fixup

THE CONTENTS OF THE FIRST TWO LINES OF EACH SECTION TO BE      Fixup

CREATED.                                                       Fixup

NOTE...IF A SECTION IS PRESENT ANY COMMAND TO CREATE IT WILL   Fixup

BE IGNORED.                                                    Fixup

(12)ALLOW ENERGY POINTS TO BE INSERTED. THE PROGRAM CAN READ UP    Fixup

TO 50, ENERGIES, MAT, MT AND USE LINEAR INTERPOLATION TO       Fixup

INSERT ENERGY POINTS INTO TABLES AS THEY ARE READ, E.G.,       Fixup

INSERT AN ENERGY POINT AT THERMAL ENERGY (0.0253 EV). IF       Fixup

AN MAT AND/OR MT IS ZERO THIS IMPLIES = ALL - INSERT THE       Fixup

ENERGY IN ALL TABLES.                                          Fixup

(13)PUT ALLOW CROSS SECTIONS ON A UNIFORM ENERGY GRID = EACH       Fixup

SECTION (MT) OF CROSS SECTIONS WILL INCLUDE ALL ENERGIES       Fixup

WHICH APPEAR IN AT LEAST ONE SECTION OF DATA. PARAMETERS       Fixup

(MT=251 THROUGH 255) ARE NOT INCLUDED IN THE UNIFORM ENERGY    Fixup

GRID.                                                          Fixup

(14)DELETE SECTION IF CROSS SECTION = 0 AT ALL ENERGIES. THIS      Fixup

SOUNDS LIKE AN ABSURD OPTION, BUT IS REQUIRED BECAUSE SUCH     Fixup

SECTIONS EXIST IN ENDF/B-VI.                                   Fixup

Fixup

CREATING RATIOS AND PRODUCTS                                       Fixup

============================                                       Fixup

IN ORDER TO CREATE RATIOS AND PRODUCTS = NEW MT NUMBERS, YOU MUST  Fixup

DO TWO THINGS,                                                     Fixup

Fixup

1) DEFINE EACH NEW MT NUMBER AS A RATIO OR PRODUCT OF TWO MT       Fixup

NUMBERS.                                                        Fixup

Fixup

2) USE THE CREATE MT NUMBER OPTION AND INPUT THE FIRST TWO LINES   Fixup

OF THE SECTION                                                  Fixup

Fixup

WARNING - UNLESS YOU DO BOTH OF THESE YOU WILL NOT OBTAIN OUTPUT   Fixup

IN THE ENDF/B FORMAT.                                              Fixup

Fixup

TWO SPECIAL MT NUMBERS HAVE BEEN DEFINED BY CSEWG INVOLVING        Fixup

RATIOS AND PRODUCTS,                                               Fixup

Fixup

ALPHA (MT=254)= CAPTURE (MT=102)/FISSION (MT=18)                   Fixup

Fixup

ETA (MT=255)  = NU-BAR (MT=452)*FISSION (MT=18)/ABSORPTION (MT=27) Fixup

Fixup

ABSORPTION (MT=27) = FISSION (MT=18) + SUM (MT=102 THROUGH 116)    Fixup

Fixup

AS YET THERE IS NO STANDARD DEFINITION OF MT NUMBERS FOR RATIO     Fixup

OR PRODUCT DATA. YOU ARE FREE TO USE ANY MT NUMBERS NORMALLY NOT   Fixup

USED IN THE ENDF/B. HOWEVER, IT WILL THEN BE YOUR RESPONSIBILITY   Fixup

TO PROPERLY INTERPRET THE RESULTS, I.E., NOBODY ELSE WILL HAVE     Fixup

ANY IDEA HOW TO INTERPRET A TABLE OF DATA ASSOCIATED WITH THE MT   Fixup

NUMBERS YOU HAVE USED.                                             Fixup

Fixup

THIS PROGRAM CAN BE ONLY DIRECTLY DEFINE RATIOS AND PRODUCTS       Fixup

USING TWO MT NUMBERS = BINARY OPERATIONS, E.G., DEFINE THE CAPTURE Fixup

TO FISSION RATIO, OR DEFINE THE PRODUCT NU-BAR*FISSION.            Fixup

Fixup

THIS PROGRAM CANNOT DIRECTLY DEFINE RATIO OR PRODUCT OF A SUM OF   Fixup

SECTIONS TO THE SUM OF ANOTHER SET OF SECTIONS. HOWEVER, THIS CAN  Fixup

BE DONE INDIRECTLY BY FIRST DEFINING A DUMMY MT NUMBER (ANY MT     Fixup

NUMBER NOT NORMALLY USED IN ENDF/B) TO BE A SUM OF SECTIONS AND    Fixup

A SECOND DUMMY MT NUMBER TO BE A SECOND SUM OF SECTIONS. YOU CAN   Fixup

THEN DEFINE RATIO OR PRODUCT YOU REQUIRE TO BE THE RATIO OF THESE  Fixup

TWO DUMMY MT NUMBERS.                                              Fixup

Fixup

FOR EXAMPLE, TO DEFINE ETA,                                        Fixup

1) FIRST DEFINE (MT=27) = (MT=27) + (SUM OF MT=102 THROUGH 116)    Fixup

2) NEXT DEFINE (MT=333) = (MT=452)*(MT=18)                         Fixup

3) LAST DEFINE (MT=255) = (MT=333)/(MT=27)                         Fixup

DO NOT FORGET TO TURN ON THE CREATE SECTION OPTION (ON THE FIRST   Fixup

INPUT LINE) AND INPUT THE FIRST TWO LINES OF SECTION MT=255 -      Fixup

OTHERWISE YOU WILL NOT GET ANY ENDF/B FORMATTED OUTPUT.            Fixup

Fixup

THE ONLY SPECIAL CONVENTIONS USED BY THIS PROGRAM IN CALCULATING   Fixup

RATIOS ARE WHEN THE DENOMINATOR OF THE RATIO IS ZERO. IN THIS      Fixup

CASE IF THE NUMERATOR IS ALSO ZERO THE RATIO IS DEFINED TO BE ONE. Fixup

IN THIS CASE IF THE NUMERATOR IS NOT ZERO THE RATIO IS DEFINED     Fixup

TO BE ZERO.                                                        Fixup

Fixup

ENDF/B FORMAT                                                      Fixup

=============                                                      Fixup

THIS PROGRAM MAY BE USED WITH DATA IN ANY VERSION OF THE ENDF/B    Fixup

FORMAT (I.E. ENDF/B-I, II, III, IV, V OR VI FORMAT). SINCE A       Fixup

PAGING SYSTEM IS USED STORE CROSS SECTION TABLES ON SCRATCH FILES  Fixup

THERE IS NO LIMIT TO THE SIZE OF TABLES (E.G. THE TOTAL CROSS      Fixup

SECTION MAY BE REPRESENTED BY 200,000 TABULATED POINTS).           Fixup

Fixup

WARNING                                                            Fixup

=======                                                            Fixup

(1) FOR EACH SECTION OF CROSS SECTIONS (I.E. EACH MT, MF=3) IN     Fixup

THE ORIGINAL EVALUATION (I.E. ENDF/B DATA READ) ONE SECTION    Fixup

OF DATA WILL BE OUTPUT, UNLESS THE SECTION HAS BEEN DELETED.   Fixup

THIS INCLUDES ANY SECTIONS WHICH ARE NOT PRESENT IN THE        Fixup

ORIGINAL EVALUATION, BUT THE USER INDICATES (BY INPUT) SHOULD  Fixup

BE CREATED.                                                    Fixup

Fixup

THE PROGRAM WILL NOT OUTPUT ANY SECTION RECONSTRUCTED BY       Fixup

SUMMATION UNLESS THE CORRESPONDING SECTION (MT NUMBER) IS      Fixup

PRESENT IN THE ORIGINAL EVALUATION OR USER INPUT INDICATES     Fixup

SHOULD BE CREATED AND OUTPUT. THIS IS (A) BECAUSE THE          Fixup

PROGRAM CANNOT DEFINE THE PARAMETERS TO APPEAR ON THE FIRST    Fixup

TWO LINES OF THE SECTION, (B) TO AVOID OUTPUTTING TOO MUCH     Fixup

DATA WHICH THE USER MAY NOT BE INTERESTED IN.                  Fixup

Fixup

(2) FOR ANY SECTIONS THAT DO NOT APPEAR IN THE ORIGINAL DATA THE   Fixup

USER MAY SPECIFY THAT THEY BE DEFINED BY SUMMATION. ANY SUCH   Fixup

SECTION MAY BE USED BE DEFINE SUBSEQUENT SUMS, BUT THE SECTION Fixup

ITSELF WILL NOT BE OUTPUT (E.G. GENERALLY MT=27 AND 101 ARE    Fixup

NOT PRESENT IN EVALUATIONS. HOWEVER, THE BUILT-IN SUMMATION    Fixup

RULES OF THIS PROGRAM USES THE ENDF/B SUMMATION RULES TO       Fixup

DEFINE MT=27 AND 101, WHICH IN TURN ARE USED TO DEFINE THE     Fixup

NON-ELASTIC CROSS SECTION, MT=3. SECTIONS MT=27 AND 101 ARE    Fixup

NOT OUTPUT).                                                   Fixup

Fixup

(3) ALL DATA IN FILE 3 AND 23 MUST BE LINEARLY INTERPOLABLE. IF    Fixup

THE DATA IS NOT LINEARLY INTERPOLABLE THIS PROGRAM WILL        Fixup

TERMINATE.                                                     Fixup

Fixup

PROGRAM OPERATION                                                  Fixup

=================                                                  Fixup

ALL MAT NUMBER ON AN ENDF/B TAPE ARE PROCESSED. EACH MAT IS        Fixup

TREATED SEPARATELY. WITHIN EACH MAT, EACH SECTION BEFORE MF=3      Fixup

IS READ, CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND OUTPUT.    Fixup

WHEN MF=3 IS LOCATED ALL CROSS SECTIONS ARE READ, SECTIONS TO BE   Fixup

DELETED ARE DELETED, SECTIONS WHICH ARE NOT PRESENTED AND USER     Fixup

INPUT INDICATES SHOULD BE CREATED ARE CREATE, SECTIONS TO BE KEPT  Fixup

ARE CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND WRITTEN TO A    Fixup

SCRATCH FILE. NEXT, IF THE USER SPECIFIES THAT THEY SHOULD,        Fixup

SECTIONS ARE RECONSTRUCTED. FINALLY ALL CROSS SECTIONS (OLD AND    Fixup

NEW) ARE OUTPUT. WITHIN THE SAME MAT, EACH SECTION AFTER MF=3 IS   Fixup

READ, CHECKED/CORRECTED (BASED ON INPUT OPTIONS) AND OUTPUT.       Fixup

Fixup

MF=3                                                               Fixup

====                                                               Fixup

THE TREATMENT OF THE CROSS SECTIONS REQUIRES UP TO 4 PASSES FOR    Fixup

CROSS SECTIONS. IN THE PROGRAM THEY CORRESPOND TO SUBROUTINES      Fixup

PASS1, PASS2, PASS3 AND PASS4. THE ORIGINAL AND FINAL ENDF/B DATA  Fixup

FILES, 5 SCRATCH FILES AND 3 IN CORE ARRAYS ARE USED. OPERATIONS   Fixup

PERFORMED DURING EACH PASS ARE,                                    Fixup

Fixup

PASS1                                                              Fixup

=====                                                              Fixup

READ ALL CROSS SECTIONS FROM ITAPE. DELETED ANY SECTIONS. CREATE   Fixup

ANY SECTIONS. CHECK/CORRECT THEM AND WRITE THEM TO SCRATCH FILE.   Fixup

DATA IS READ INTO ARRAY A, TRANSFERRED TO ARRAY C (AFTER EDITING)  Fixup

AND OUTPUT TO ISCRC FROM ARRAY C.                                  Fixup

ITAPE  - UNIT ORIGINAL ENDF/B DATA IS READ FROM.                   Fixup

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS WRITTEN ON.              Fixup

TABA   - ARRAY INTO WHICH ORIGINAL DATA IS READ.                   Fixup

TABC   - ARRAY INTO WHICH EDITED DATA IS TRANSFERRED TO AND        Fixup

FROM WHICH IT IS WRITTEN TO ISCRC.                        Fixup

Fixup

PASS2                                                              Fixup

=====                                                              Fixup

IF A UNIFORM ENERGY GRID IS REQUESTED IT IS CREATED DURING THIS    Fixup

PASS. FIRST ALL OF THE CROSS SECTIONS FROM PASS1 ARE READ AND A    Fixup

UNIFORM ENERGY GRID IS CREATED = ALL ENERGIES THAT ARE INCLUDED    Fixup

IN AT LEAST ONE SECTION (MT) OF CROSS SECTIONS.                    Fixup

ISCRA  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.              Fixup

ISCRB  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.              Fixup

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS READ FROM.               Fixup

TABA   - ARRAY CONTAINING UNIFORM ENERGY GRID.                     Fixup

TABB   - ARRAY CONTAINING UNIFORM ENERGY GRID.                     Fixup

TABC   - ARRAY CONTAINING EDITED DATA.                             Fixup

Fixup

THE UNIFORM ENERGY GRID ENDS UP ON ISCRB. NEXT EACH SECTION OF     Fixup

CROSS SECTIONS FROM PASS1 IS READ FROM ISCRC, INTERPOLATED TO      Fixup

THE UNIFORM ENERGY GRID AND OUTPUT TO ISCRA. FINALLY ISCRA AND     Fixup

ISCRC ARE SWITCH, SO THAT AT THE END OF THIS PASS THE DATA WILL    Fixup

AGAIN BE ON ISCRC (EXACTLY AS AT THE END OF PASS1), WITH UPDATED   Fixup

POINT COUNTS.                                                      Fixup

ISCRA  - SCRATCH UNIT THAT UNIFORM ENERGY GRID DATA IS WRITTEN ON. Fixup

ISCRB  - SCRATCH UNIT CONTAINING UNIFORM ENERGY GRID.              Fixup

ISCRC  - SCRATCH UNIT THAT EDITED DATA IS READ FROM.               Fixup

TABA   - ARRAY CONTAINING UNIFORM ENERGY GRID DATA.                Fixup

TABB   - ARRAY CONTAINING UNIFORM ENERGY GRID.                     Fixup

TABC   - ARRAY CONTAINING EDITED DATA.                             Fixup

Fixup

PASS3                                                              Fixup

=====                                                              Fixup

SUMMATION CROSS SECTIONS ARE DEFINED BY READING DATA FROM ISCRC    Fixup

AND MERGING THEM ONTO ISCRA. THE FIRST SECTION THAT CONTRIBUTES    Fixup

TO A SUM IS MERELY COPIED FROM C TO A. IF MORE SECTIONS WILL       Fixup

CONTRIBUTE TO THE SUM THE DATA IN A IS TRANSFERRED TO B, A         Fixup

SECTION OF DATA FROM C IS ADDED TO THE DATA IN B AND STORED IN     Fixup

A. THE CYLE OF ADDED C AND B TO A, FOLLOWED BY MOVING A TO B       Fixup

IS CONTINUED UNTIL ALL CONTRIBUTING SECTIONS HAVE BEEN ADDED.      Fixup

THE SUM IS THEN COPIED FROM A TO D. IF NEWLY CONSTRUCTED SECTION   Fixup

IS REQUIRED FOR ANY LATER SUMMUATIONS IT IS ALSO COPIED TO E.      Fixup

THE CYCLE OF ADDED SECTIONS FROM C AND B TO A IS REPEATED FOR      Fixup

EACH REQUIRED SUMMATION REACTION. IN ADDITION TO SECTIONS FROM     Fixup

C, AFTER THE FIRST SUMMATION SECTIONS MAY ALSO BE ADDED TO A       Fixup

FROM E (THE CONTRIBUTION OF NEW RECONSTRUCTED CROSS SECTIONS).     Fixup

WHEN ALL REQUIRED SECTIONS HAVE BEEN RECONSTRUCTED THE NEW         Fixup

SECTIONS WILL BE ON E AND THE ORIGINAL SECTIONS ON C.              Fixup

ISCRC  - SCRATCH FILE FROM WHICH ORIGINAL DATA IS READ.            Fixup

ISCRA  - SCRATCH FILE ONTO WHICH SUM FOR ONE SECTION IS WRITTEN.   Fixup

ISCRD  - SCRATCH FILE ONTO WHICH ALL SUM CROSS SECTIONS ARE        Fixup

WRITTEN.                                                  Fixup

ISCRE  - SCRATCH FILE ONTO WHICH ALL SUM CROSS SECTIONS WHICH      Fixup

ARE REQUIRED FOR LATER SUMS ARE WRITTEN.                  Fixup

ISCRB  - UTILITY SCRATCH FILE USED TO CREATE SUM CROSS SECTIONS.   Fixup

TABA   - ARRAY INTO WHICH SUMS ARE WRITTEN.                        Fixup

TABB   - ARRAY INTO WHICH PARTIAL SUMS ARE WRITTEN.                Fixup

TABC   - ARRAY INTO WHICH ORIGINAL DATA IS READ.                   Fixup

Fixup

PASS4                                                              Fixup

=====                                                              Fixup

CROSS SECTIONS ARE READ FROM ISCRC (ORIGINAL) AND ISCRD (NEW)      Fixup

AND ARE WRITTEN IN THE ENDF/B FORMAT ON OTAPE. THE BEGINNING OF    Fixup

EACH SECTION OF ORIGINAL DATA IS READ FROM ISCRC (TO DEFINE        Fixup

SECTION HEADER INFORMATION). IF THIS MT HAS NOT BEEN RECOSTRUCTED  Fixup

ON ISCRD THE ORIGINAL SECTION IS OUTPUT. IF THE SECTION HAS BEEN   Fixup

RECONSTRUCTED THE ORIGINAL SECTION IS SKIPPED AND THE NEW SECTION  Fixup

IS OUTPUT.                                                         Fixup

OTAPE  - OUTPUT DATA IN THE ENDF/B FORMAT.                         Fixup

ISCRC  - SCRATCH FILE FROM WHICH ORIGINAL DATA IS READ.            Fixup

ISCRD  - SCRATCH FILE FROM WHICH NEW DATA IS READ.                 Fixup

TABC   - ARRAY INTO WHICH CROSS SECTIONS ARE READ FROM SCRATCH     Fixup

AND WRITTEN TO OTAPE                                      Fixup

Fixup

I/O FILE DEFINITIONS                                               Fixup

====================                                               Fixup

UNIT   DESCRIPTION                                                 Fixup

====   ===========                                                 Fixup

2    INPUT PARAMETERS.                                           Fixup

3    OUTPUT REPORT.                                              Fixup

10    ORIGINAL DATA IN THE ENDF/B FORMAT.                         Fixup

11    FINAL DATA IN THE ENDF/B FORMAT.                            Fixup

12    SCRATCH FILE                                                Fixup

14    SCRATCH FILE                                                Fixup

15    SCRATCH FILE                                                Fixup

16    SCRATCH FILE                                                Fixup

17    SCRATCH FILE                                                Fixup

Fixup

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Fixup

===============================================================    Fixup

UNIT  FILE NAME   FORMAT                                           Fixup

====  ==========  ======                                           Fixup

2   FIXUP.INP    BCD                                             Fixup

3   FIXUP.LST    BCD                                             Fixup

10   ENDFB.IN     BCD                                             Fixup

11   ENDFB.OUT    BCD                                             Fixup

12-17  (SCRATCH)   BINARY                                           Fixup

Fixup

INPUT LINES                                                        Fixup

===========                                                        Fixup

LINE  COLUMNS  FORMAT  DESCRIPTION                                 Fixup

====  =======  ======  ===========                                 Fixup

1    1-14     14I1   INPUT OPTIONS AS DESCRIBED ABOVE.           Fixup

EACH COLUMN OF THE INPUT LINE CONTROLS      Fixup

ONE OF THE TESTS/CORRECTIONS DESCRIBED      Fixup

ABOVE. TESTS/CORRECTION 1-14 (NOT ALL       Fixup

IMPLEMENTED YET) CORRESPOND TO COLUMNS      Fixup

1-14 OF THIS INPUT LINE AND ARE TREATED     Fixup

AS FOLLOWS,                                 Fixup

= 0 - DO NOT PERFORM TEST/CORRECTION.       Fixup

= 1 - PERFORM TEST/CORRECTION.              Fixup

FOR MT EXCLUSION FROM THRESHOLD TESTS       Fixup

(COLUMN 2), DELETION (COLUMN 4), OR         Fixup

SUMMATION (COLUMN 5) THE INPUT OPTION       Fixup

MAY BE,                                     Fixup

= 1 - READ RULES FROM INPUT                 Fixup

= 2 - USE BUILT-IN RULES                    Fixup

2    1-60      A60   ENDF/B INPUT DATA FILENAME                  Fixup

(STANDARD OPTION = ENDFB.IN)                Fixup

3    1-60      A60   ENDF/B OUTPUT DATA FILENAME                 Fixup

(STANDARD OPTION = ENDFB.OUT)               Fixup

4-M    1-5      FREE   CHARACTER (S,D,T,R,*) FOLLOWED BY BLANK OR  Fixup

FORM   MT NUMBER                                   Fixup

- THE ALLOWED CHARACTERS ARE,               Fixup

- S OR BLANK = SUM (OR DIFFERENCES)         Fixup

- D = DELETE                                Fixup

- T = NO THRESHOLD ENERGY CORRECTIONS       Fixup

- R = RATIO                                 Fixup

- * = PRODUCT                               Fixup

6-72      FREE   UP TO 10 LOWER AND UPPER MT RANGES WHICH    Fixup

FORM   WILL BE USED TO DEFINE THE RECONSTRUCTED    Fixup

CROSS SECTION OR TO DEFINE MT RANGES WHICH  Fixup

ARE EXCLUDED FROM THRESHOLD TESTS.          Fixup

Fixup

EACH MT NUMBER IS DEFINED BY A CONTINUOUS   Fixup

STRING OF DIGITS, POSSIBILITY PRECEEDED BY  Fixup

A - (MINUS SIGN). EACH MT NUMBER MUST BE    Fixup

BLANK OR OTHERWISE (NOT A DIGIT) DELIMITED. Fixup

Fixup

COLUMNS 6-72 MAY CONTAIN STRINGS OF DIGITS  Fixup

THE FIRST DIGIT STRING OF EACH PAIR MAY BE  Fixup

PRECEEDED BY A - (MINUS SIGN).              Fixup

Fixup

EACH LINE WILL BE INTERPRETED AS FOLLOWS,   Fixup

Fixup

*SUMMATION (OR DIFFERENCES)                  Fixup

--------------------------                  Fixup

COLUMNS 1-5 = S OR BLANK FOLLOWED BY THE    Fixup

MT NUMBER TO BE DEFINED BY SUMMATION        Fixup

Fixup

COLUMNS 6-72 = UP TO 10 MT RANGE (PAIRS OF  Fixup

MT NUMBERS) TO BE USED TO DEFINED THE SUM.  Fixup

IF THE FIRST MT NUMBER OF A PAIR IS         Fixup

NEGATIVE THE RANGE OF MT NUMBERS IS         Fixup

SUBTRACTED - AT LEAST ONE RANGE MUST BE     Fixup

SPECIFIED.                                  Fixup

Fixup

*DELETIONS                                   Fixup

---------                                   Fixup

COLUMNS 1-5 = D FOLLOWED BY BLANKS          Fixup

Fixup

COLUMNS 6-72 CONTAIN UP TO 10 MT RANGE      Fixup

(PAIRS OF MT NUMBERS), EACH RANGE DEFINING  Fixup

A RANGE OF MT NUMBERS TO BE DELETED - AT    Fixup

LEAST ONE RANGE MUST BE SPECIFIED.          Fixup

Fixup

*EXCLUSION FROM THRESHOLD TESTS              Fixup

------------------------------              Fixup

COLUMNS 1=5 = T FOLLOWED BY BLANKS          Fixup

Fixup

COLUMNS 6-72 CONTAIN UP TO 10 MT RANGE      Fixup

(PAIRS OF MT NUMBERS), EACH RANGE DEFINING  Fixup

A RANGE OF MT NUMBERS WHOSE THRESHOLD       Fixup

ENERGY WILL NOT BE CHECKED - AT LEAST ONE   Fixup

RANGE MUST BE SPECIFIED.                    Fixup

Fixup

*RATIO                                       Fixup

-----                                       Fixup

COLUMNS 1-5 = R FOLLOWED BY THE MT NUMBER   Fixup

TO BE DEFINED BY A RATIO                    Fixup

Fixup

COLUMNS 6-72 CONTAINS 2 MT NUMBERS TO BE    Fixup

USED TO DEFINE THE RATIO.                   Fixup

Fixup

*PRODUCT                                     Fixup

-----                                       Fixup

COLUMNS 1-5 = * FOLLOWED BY THE MT NUMBER   Fixup

TO BE DEFINED BY A PRODUCT                  Fixup

Fixup

COLUMNS 6-72 CONTAINS 2 MT NUMBERS TO BE    Fixup

USED TO DEFINE THE PRODUCT.                 Fixup

Fixup

CONVENTIONS                                 Fixup

-----------                                 Fixup

*UP TO 20 DELETIONS AND 20 SUMMATIONS OR     Fixup

RATIOS OR PRODUCTS MAY BE SPECIFIED.        Fixup

*ONLY 1 EXCLUSION FROM THRESHOLD TESTS       Fixup

MAY BE SPECIFIED (THE 1 LINE MAY CONTAIN    Fixup

UP TO 10 MT RANGES TO EXCLUDE FROM TESTS).  Fixup

*INPUT IS TERMINATED BY INPUTTNG 0 OR        Fixup

BLANK IN COLUMNS 1-72 (I.E. THE LAST        Fixup

INPUT LINE MUST BE BLANK).                  Fixup

*THE UPPER LIMIT OF EACH RANGE MUST BE AT    Fixup

LEAST AS BIG AS THE LOWER LIMIT (IN         Fixup

ABSOLUTE VALUE).                            Fixup

*FOR RECONSTRUCTION POSITIVE MT RANGES WILL  Fixup

BE ADDED TO THE SUM AND NEGATIVE MT RANGES  Fixup

WILL BE SUBTRACTED.                         Fixup

*IF INPUT OPTION 2 (FIRST INPUT LINE) IS     Fixup

0 THRESHOLD EXCLUSION IS NOT ALLOWED.       Fixup

*IF INPUT OPTION 4 (FIRST INPUT LINE) IS     Fixup

0 DELETIONS ARE NOT ALLOWED.                Fixup

*IF INPUT OPTION 5 (FIRST INPUT LINE) IS     Fixup

0 SUMMATIONS AND RATIOS ARE NOT ALLOWED.    Fixup

N-K                    IF THE USER SPECIFIES THAT SECTIONS WHICH   Fixup

ARE NOT PRESENT IN THE ORIGINAL EVALUATION  Fixup

MAY BE CREATED, TWO LINES MUST BE INPUT FOR Fixup

EACH SECTION TO BE CREATED. THE TWO LINES   Fixup

DEFINE (C1, C2, L1 AND L2) FOR EACH OF THE  Fixup

FIRST TWO LINES OF THE SECTION TO BE        Fixup

CREATED. THE FIRST LINE ALSO DEFINES (MAT   Fixup

AND MT). (N1, N2) ARE ALWAYS ZERO ON THE    Fixup

FIRST LINE AND WILL BE CALCULATED BY THE    Fixup

PROGRAM FOR THE SECOND LINE.                Fixup

FIRST    1-11     E11.4  ZA OF SECTION TO BE CREATED                 Fixup

LINE    12-22     E11.4  AWRE OF SECTION TO BE CREATED               Fixup

23-33      I11   L1 OF SECTION TO BE CREATED                 Fixup

34-44      I11   L2 OF SECTION TO BE CREATED                 Fixup

45-48      I4    MAT OF SECTION TO BE CREATED                Fixup

49-51      I3    MT OF SECTION TO BE CREATED                 Fixup

SECOND    1-11     E11.4  C1 OF SECTION TO BE CREATED                 Fixup

LINE    12-22     E11.4  C2 OF SECTION TO BE CREATED                 Fixup

23-33      I11   L1 OF SECTION TO BE CREATED                 Fixup

34-44      I11   L2 OF SECTION TO BE CREATED                 Fixup

*PAIRS OF LINES MAY BE IN ANY MAT/MT ORDER   Fixup

(E.G., THEY NEED NOT BE IN ASCENDING        Fixup

MAT/MT ORDER).                             Fixup

*UP TO 50 PAIRS OF LINES MAY BE USED TO      Fixup

DEFINE SECTIONS TO BE CREATED. THE LIST     Fixup

IS TERMINATED WHEN THE FIRST LINE OF A      Fixup

PAIR CONTAINS A ZERO (OR BLANK) MAT AND/OR  Fixup

MT.                                         Fixup

M-N                    IF THE USER SPECIFIES THAT ENERGIES WHICH   Fixup

ARE NOT PRESENT IN THE ORIGINAL EVALUATION  Fixup

MAY BE INSERTED, ONE LINE MUST BE INPUT FOR Fixup

EACH ENERGY TO BE INSERTED.                 Fixup

1-11    E11.4   ENERGY TO BE INSERTED                       Fixup

12-15     I4     MAT IN WHICH TO INSERT ENERGY = 0 = ALL     Fixup

16-18     I3     MT IN WHICH TO INSERT ENERGY = 0 = ALL      Fixup

*UP TO 50 (ENERGY, MAT, MT) LINES MAY BE     Fixup

USED. THE LIST IS TERMINATED BY A BLANK     Fixup

LINE.                                       Fixup

*INPUT MAY BE IN ANY (ENERGY, MAT, MT)       Fixup

ORDER.                                      Fixup

*ENERGY POINTS CAN ONLY BE INSERTED WITHIN   Fixup

THE ORIGINAL ENERGY RANGE OF A SECTION -    Fixup

THIS OPTION CANNOT BE USED TO EXTEND THE    Fixup

CROSS SECTION EITHER BELOW OR ABOVE THE     Fixup

ORIGINAL TABULATED ENERGY RANGE.            Fixup

Fixup

EXAMPLE INPUT NO. 1                                                Fixup

===================                                                Fixup

(1) USE OPTIONS 1-11 (ALL OPTIONS, EXCEPT INSERT ENERGY POINTS)    Fixup

(2) DELETE MT=900 (FOR EXAMPLE PURPOSES ONLY)                      Fixup

(3) DEFINE THE FOLLOWING MT NUMBERS TO BE RECONSTRUCTED,           Fixup

(MT=  4) = THE SUM OF MT= 51 THROUGH  91                       Fixup

(MT=103) = THE SUM OF MT=700 THROUGH 718 (NOT 719)             Fixup

(MT=104) = THE SUM OF MT=720 THROUGH 738 (NOT 739)             Fixup

(MT=105) = THE SUM OF MT=740 THROUGH 758 (NOT 759)             Fixup

(MT=106) = THE SUM OF MT=760 THROUGH 778 (NOT 779)             Fixup

(MT=107) = THE SUM OF MT=780 THROUGH 798 (NOT 799)             Fixup

(MT=101) = THE SUM OF MT=102 THROUGH 114                       Fixup

(MT= 18) = (MT=19) + (MT=20 AND 21) + (MT=38)                  Fixup

(IF TOTAL FISSION, MT=18, IS NOT PRESENT, DEFINE    Fixup

IT BY SUMMING FIRST, SECOND, ETC. CHANCE - NOTE     Fixup

THAT THIS MUST BE DONE IN THIS ORDER, SINCE THE     Fixup

NEXT SUM INVOLVES USING MT=18.                      Fixup

(MT= 27) = THE SUM OF MT= 18 AND 101                           Fixup

(MT=101 RECONSTRUCTED ABOVE USED IN SUM).           Fixup

(MT=  3) = THE SUM OF (MT=4)+(MT=6-9)+(MT=16-17)+(MT=22-37)+   Fixup

(MT=41-45)                                          Fixup

(MT=4 AND 27 RECONSTRUCTED ABOVE USED IN SUM).      Fixup

(MT= 19) = (MT=18) - (MT=20 AND 21) - (MT=38)                  Fixup

(DEFINE FIRST CHANGE FISSION BY SUBTRACTION TO      Fixup

ALLOW RESONANCE CONTRIBUTION FROM MT=18 TO BE       Fixup

INCLUDED IN MT=19).                                 Fixup

(MT=  1) = THE SUM OF MT=2 AND 3                               Fixup

(MT=3 RECONSTRUCTED ABOVE USED IN SUM).             Fixup

(4) THRESHOLD ENERGIES OF THE FOLLOWING MT NUMBERS WILL NOT BE      Fixup

TESTED OR CORRECTED.                                           Fixup

MT=1, 4, 18, 19, 91, 103 THROUGH 114.                          Fixup

(5) DEFINE MT=254 TO BE THE CAPTURE TO FISSION RATIO (MT=102/18)    Fixup

(6) CREATE MAT=1300/MT=254 - NOTE, THIS IS NECESSARY IN ORDER TO    Fixup

HAVE THE CAPTURE TO FISSION RATIO OUTPUT IN THE ENDF/B FORMAT   Fixup

Fixup

NOTE, ON THE FOLLOWING INPUT LINES THE CHARACTERS = ( ) + , HAVE   Fixup

BEEN USED ONLY TO MAKE THE INPUT MORE READABLE - THESE CHARACTERS  Fixup

WILL BE SKIPPED BY THE PROGRAM IN READING INPUT - THE RESULTS      Fixup

WOULD BE THE SAME IF THESE CHARACTERS WERE OMITTED, AS LONG AS     Fixup

ALL OF THE MT NUMBERS ARE DELIMITED, I.E., THERE IS AT LEAST ONE   Fixup

NON-DIGITAL CHARACTER BETWEEN MT NUMBERS. NOTE, THAT - (MINUS      Fixup

SIGN) IS IMPORTANT AND IS USED DURING INPUT TO DEFINE MT RANGES    Fixup

WHICH SHOULD BE SUBTRACTED, E.,G., SEE THE DEFINITION OF MT=19.    Fixup

Fixup

READ FILE /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT     Fixup

Fixup

THE FOLLOWING 21 INPUT LINES ARE REQUIRED.                         Fixup

Fixup

11111111111                                                        Fixup

/ENDFB6/K300/LEAD.IN                                               Fixup

/ENDFB6/K300/LEAD.OUT                                              Fixup

D900                                                               Fixup

4=( 51, 91)                                                     Fixup

103=(700,718)                                                     Fixup

104=(720,738)                                                     Fixup

105=(740,758)                                                     Fixup

106=(760,778)                                                     Fixup

107=(780,798)                                                     Fixup

101=(102,114)                                                     Fixup

18=( 19, 19)+( 20, 21)+( 38, 38)                                 Fixup

27=( 18, 18)+(101,101)                                           Fixup

3=(  4,  4)+(  6,  9)+( 16, 17)+( 22, 37)+( 41, 45)             Fixup

19=( 18, 18)-( 20, 21)-( 38, 38)                                 Fixup

1=(  2,  3)                                                     Fixup

T    (  1,  1) (  4,  4) ( 18, 19) ( 91, 91) (103,114)             Fixup

R254=(102/ 18)                                                     Fixup

(BLANK LINE TO TERMINATE SUMMATION/DELETION RULES)    Fixup

2.00400+ 3 0.00000+ 0          0          01300254                Fixup

0.00000+ 0 0.00000+ 0          0          0                       Fixup

(BLANK LINE TO TERMINATE SECTION CREATION RULES)      Fixup

Fixup

NOTE, THE DELETION AND THRESHOLD EXCLUSION LINES MAY APPEAR IN     Fixup

IN ANY ORDER. HOWEVER, SUMMATION AND RATIO RULES MUST APPEAR IN    Fixup

THE ORDER IN WHICH YOU WANT THEM TO BE EXECUTED - E.G., THE        Fixup

ABOVE INPUT WILL FIRST RECONSTRUCT MT=4, WHICH CAN THEN BE USED    Fixup

TO CONTRIBUTE TO THE FOLLOWING SUM TO DEFINE MT=3, WHICH IN TURN   Fixup

CAN THEN BE USED TO CONTRIBUTE TO THE FOLLOWING SUM TO DEFINE      Fixup

MT=1. IF THE ORDER OF THE INPUT LINES IS CHANGED SUCH THAT MT=3    Fixup

IS RECONSTRUCTED BEFORE MT=4, THE ORIGINAL MT=4 WILL BE USED IN    Fixup

THE SUMMATION TO DEFINE MT=3. THE SAME RULES APPLY TO CALCULATING  Fixup

RATIOS, IF EITHER THE NUMERATOR OR DENOMINATOR IS TO BE DEFINED    Fixup

BY SUMMATION, THIS SHOULD BE DONE BEFORE DEFINING THE RATIO BY     Fixup

INPUT PARAMETERS.                                                  Fixup

Fixup

EXAMPLE INPUT NO. 2                                                Fixup

===================                                                Fixup

(1) USE OPTIONS 1-11 (ALL OPTIONS, EXCEPT INSERT ENERGY POINTS)    Fixup

(2) USE BUILT-IN TABLES FOR SUMMATION/DELETION/THRESHOLD EXCLUSION Fixup

(THIS ONLY REQUIRES COLUMNS 2, 4 AND 5 TO BE SET =2 ON THE     Fixup

FIRST INPUT LINE. THE BUILT-IN RULES EXACTLY CORRESPOND TO     Fixup

THE INPUT ABOVE UNDER EXAMPLE NO. 1, EXCEPT THAT NO MT NUMBERS Fixup

WILL BE DELETED.                                               Fixup

(3) IF NOT PRESENT, CREATE MAT=1300/MT=1                           Fixup

Fixup

USE THE STANDARD FILE NAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE    Fixup

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).           Fixup

Fixup

THE FOLLOWING 6 INPUT LINES ARE REQUIRED.                          Fixup

Fixup

12122111111                                                        Fixup

Fixup

Fixup

2.00400+ 3 0.00000+ 0          0          01300  1                Fixup

0.00000+ 0 0.00000+ 0          0          0                       Fixup

(BLANK LINE TO TERMINATE SECTION CREATION RULES)      Fixup

Fixup

EXAMPLE INPUT NO. 3                                                Fixup

===================                                                Fixup

(1) USE OPTIONS 1-10 (ALL OPTIONS PRESENTLY IMPLEMENTED, EXCEPT    Fixup

DO NOT ALLOW SECTION CREATION AND INSERT ENERGY POINTS).       Fixup

(2) USE BUILT-IN TABLES FOR SUMMATION/DELETION/THRESHOLD EXCLUSION Fixup

(THIS ONLY REQUIRES COLUMNS 2, 4 AND 5 TO BE SET =2 ON THE     Fixup

FIRST INPUT LINE. THE BUILT-IN RULES EXACTLY CORRESPOND TO     Fixup

THE INPUT ABOVE UNDER EXAMPLE NO. 1, EXCEPT THAT NO MT NUMBERS Fixup

WILL BE DELETED.                                               Fixup

(3) DO NOT CREATE ANY SECTIONS.                                    Fixup

Fixup

READ FILE /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT     Fixup

Fixup

THE FOLLOWING 3 INPUT LINES ARE REQUIRED.                          Fixup

Fixup

1212211111                                                         Fixup

/ENDFB6/K300/LEAD.IN                                               Fixup

/ENDFB6/K300/LEAD.OUT                                              Fixup

Fixup

EXAMPLE INPUT NO. 4                                                Fixup

===================                                                Fixup

SAME AS EXAMPLE NO. 3, ABOVE, EXCEPT INSERT AN ENERGY POINT AT     Fixup

THERMAL FOR ALL REACTIONS WHICH SPAN THE THERMAL ENERGY RANGE.     Fixup

Fixup

USE THE STANDARD FILE NAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE    Fixup

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).           Fixup

Fixup

THE FOLLOWING 5 INPUT LINES ARE REQUIRED.                          Fixup

Fixup

121221111101                                                       Fixup

Fixup

Fixup

2.53000- 2   0  0                                                 Fixup

(BLANK LINE TO TERMINATE ENERGY INSERTS)              Fixup

Fixup

WARNING                                                            Fixup

=======                                                            Fixup

ALTHOUGH THIS PROGRAM IS DESIGNED TO ALLOW REACTIONS TO BE DEFINED Fixup

BY ADDING OR SUBTRACTING REACTIONS THE USER SHOULD ALWAYS TRY TO   Fixup

DEFINE REACTIONS BY SUMMING TO AVOID NEGATIVE CROSS SECTIONS. FOR  Fixup

EXAMPLE, IT IS POSSIBLE TO CALCULATE MT=3 AND DEFINE MT=1 AS THE   Fixup

SUM OF MT=2 AND 3 (THE RECOMMENDED APPROACH AS USED IN THE ABOVE   Fixup

INPUT). ALTERATIVELY IT IS POSSIBLE TO CALCULATE MT=1 AND DEFINE   Fixup

MT=3 AS MT=1 MINUS MT=2 (THIS APPROACH IS NOT RECOMMENDED).        Fixup

Fixup

THE ONLY BUILT-IN SUMMATION RULE THAT USES SUBTRACTION IS THE      Fixup

CALCULATION OF THE FIRST CHANGE FISSION (MT=19) AS THE TOTAL       Fixup

FISSION (MT=18) MINUS THE SECOND, THIRD AND FOURTH CHANGE FISSION  Fixup

(MT=20, 21, 38). THIS HAS BEEN DONE TO ALLOW THE RESONANCE         Fixup

CONTRIBUTION, CALCULATED BY MANY CODES AND INCLUDED IN MT=18,      Fixup

TO BE CONSISTENTLY INCLUDED IN THE FIRST CHANCE FISSION.           Fixup

Fixup

======================================================================= Fixup

======================================================================= Groupie

Groupie

PROGRAM GROUPIE                                                    Groupie

VERSION 76-1 (NOVEMBER 1976)                                       Groupie

VERSION 79-1 (OCTOBER 1979) CDC-7600 AND CRAY-1 VERSION.           Groupie

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION                  Groupie

VERSION 81-1 (JANUARY 1981) EXTENSION TO 3000 GROUPS               Groupie

VERSION 81-2 (MARCH 1981) IMPROVED SPEED                           Groupie

VERSION 81-3 (AUGUST 1981) BUILT-IN 1/E WEIGHTING SPECTRUM         Groupie

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY        Groupie

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Groupie

*ELIMINATED COMPUTER DEPENDENT CODING.  Groupie

*NEW, MORE COMPATIBLE I/O UNIT NUMBERS. Groupie

*NEW MULTI-BAND LIBRARY BINARY FORMAT.  Groupie

VERSION 83-2 (OCTOBER 1983) ADDED OPTION TO ALLOW SIGMA-0 TO BE    Groupie

DEFINED EITHER AS MULTIPLES OF         Groupie

UNSHIELDED TOTAL CROSS SECTION IN EACH Groupie

GROUP, OR POWERS OF 10 IN ALL GROUPS.  Groupie

VERSION 84-1 (APRIL 1984)   ADDED MORE BUILT IN MULTIGROUP ENERGY  Groupie

STRUCTURES.                            Groupie

VERSION 85-1 (APRIL 1985)  *UPDATED FOR ENDF/B-VI FORMATS.         Groupie

*SPECIAL I/O ROUTINES TO GUARANTEE      Groupie

ACCURACY OF ENERGY.                    Groupie

*DOUBLE PRECISION TREATMENT OF ENERGY   Groupie

(REQUIRED FOR NARROW RESONANCES).      Groupie

*MINIMUM TOTAL CROSS SECTION TREATMENT  Groupie

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Groupie

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMAT                       Groupie

VERSION 86-2 (JUNE 1986)   *BUILT-IN MAXWELLIAN, 1/E AND FISSION   Groupie

WEIGHTING SPECTRUM.                    Groupie

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Groupie

FILE NAMES (SEE, SUBROUTINES FILIO1    Groupie

FILIO2 FOR DETAILS).                   Groupie

*IMPROVED BASED ON USER COMMENTS.       Groupie

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Groupie

INSURE PROGRAM WILL NOT DO ANYTHING    Groupie

CRAZY.                                 Groupie

*UPDATED TO USE NEW PROGRAM CONVERT     Groupie

KEYWORDS.                              Groupie

*ADDED LIVERMORE CIVIC COMPILER         Groupie

CONVENTIONS.                           Groupie

VERSION 91-1 (JUNE 1991)   *INCREASED PAGE SIZE FROM 1002 TO 5010  Groupie

POINTS                                 Groupie

*UPDATED BASED ON USER COMMENTS         Groupie

*ADDED FORTRAN SAVE OPTION              Groupie

*COMPLETELY CONSISTENT ROUTINE TO READ  Groupie

FLOATING POINT NUMBERS.                Groupie

VERSION 92-1 (JANUARY 1992)*ADDED RESONANCE INTEGRAL CALCULATION - Groupie

UNSHIELDED AND/OR SHIELDED - FOR       Groupie

DETAILS SEE BELOW                      Groupie

*INCREASED NUMBER OF ENERGY POINTS      Groupie

IN BUILT-IN SPECTRA - TO IMPROVE       Groupie

ACCURACY.                              Groupie

*ALLOW SELECTION OF ZA/MF/MT OR         Groupie

MAT/MF/MT RANGES - ALL DATA NOT        Groupie

SELECTED IS SKIPPED ON INPUT AND       Groupie

NOT WRITTEN AS OUTPUT.                 Groupie

*COMPLETELY CONSISTENT I/O ROUTINES -   Groupie

TO MINIMIZE COMPUTER DEPENDENCE.       Groupie

*NOTE, CHANGES IN INPUT PARAMETER       Groupie

FORMAT - FOR ZA/MF/MT OR MAT/MF/MT     Groupie

RANGES.                                Groupie

VERSION 92-2 (JUNE 1992)   *MULTIBAND PARAMETERS OUTOUT AS         Groupie

CHARACTER (RATHER THAN BINARY) FILE.   Groupie

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 5010 TO       Groupie

30000 POINTS                           Groupie

*ELIMINATED COMPUTER DEPENDENCE.        Groupie

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Groupie

TO ALLOW ACCESS TO FILE STRUCTURES     Groupie

(WARNING - INPUT PARAMETER FORMAT      Groupie

HAS BEEN CHANGED)                      Groupie

*CLOSE ALL FILES BEFORE TERMINATING     Groupie

(SEE, SUBROUTINE ENDIT)                Groupie

VERSION 95-1 (JANUARY 1994)*CORRECTED MAXWELLIAN WEIGHTING         Groupie

*CHANGING WEIGHTING SPECTRUM FROM       Groupie

0.1 TO 0.001 % UNCERTAINTY             Groupie

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Groupie

*IMPROVED COMPUTER INDEPENDENCE        Groupie

*ALL DOUBLE PRECISION                  Groupie

*ON SCREEN OUTPUT                      Groupie

*UNIFORM TREATMENT OF ENDF/B I/O       Groupie

*IMPROVED OUTPUT PRECISION             Groupie

*DEFINED SCRATCH FILE NAMES            Groupie

*UP TO 1000 GROUP MULTI-BAND           Groupie

CALCULATION (PREVIOUSLY 175)          Groupie

*MAXIMUM NUMBER OF GROUPS REDUCED      Groupie

FROM 3,000 TO 1,000                   Groupie

*UP TO 1000 MATERIALS                  Groupie

(PREVIOUSLY 100)                      Groupie

*CORRECTED USE OF MAXWELLIAN +         Groupie

1/E + FISSION SPECTRUM                Groupie

*ONLY 2 BAND VERSION DISTRIBUTED       Groupie

(CONTACT AUTHOR FOR DETAILS)          Groupie

*DEFINED SCRATCH FILE NAMES            Groupie

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Groupie

POINT READ FOR MORE DIGITS            Groupie

*UPDATED TEST FOR ENDF/B FORMAT        Groupie

VERSION BASED ON RECENT FORMAT CHANGE Groupie

*GENERAL IMPROVEMENTS BASED ON         Groupie

USER FEEDBACK                         Groupie

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Groupie

MF=1, MT-451.                         Groupie

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF=10, ACTIVATION CROSS SECTION Groupie

PROCESSING.                           Groupie

*GENERAL IMPROVEMENTS BASED ON         Groupie

USER FEEDBACK                         Groupie

VERS. 2002-1 (FEBRUARY 2002)*ADDED TART 700 GROUP STRUCTURE        Groupie

*ADDED VARIABLE SIGMA0 INPUT OPTION    Groupie

(MAY 2002)     *OPTIONAL INPUT PARAMETERS             Groupie

(NOV. 2002)    *ADDED SAND-II EXTENDED DOWN TO        Groupie

1.0E-5 EV.                            Groupie

(JUNE 2003)    *CORRECTED SAND-II 620 AND 640 GROUP   Groupie

ENERGY BOUNDARIES DEFINITIONS.        Groupie

VERS. 2004-1 (SEPT. 2004)  *INCREASED PAGE SIZE FROM 30000 TO      Groupie

120000 POINTS                          Groupie

*ADDED "OTHER" AS ADDITIONAL REACTION   Groupie

TO IMPROVE MULTI-BAND FITTING          Groupie

*ADDED ITERATION FOR "BEST" PARTIAL     Groupie

PARAMETERS.                            Groupie

*DO NOT SKIP LOW TOTAL ENERGY RANGES    Groupie

WHEN DEFINING AVERAGE CROSS SECTIONS - Groupie

THIS MAKES OUTPUT COMPATIBLE WITH      Groupie

ANY STANDARD AVERAGING PROCEDURE       Groupie

VERS. 2005-1 (JAN. 2005)   *ADDED OPTION TO CHANGE TEMPERATURE OF  Groupie

BUILT-IN STANDARD SPECTRUM.            Groupie

VERS. 2007-1 (JAN. 2007)   *CHECKED AGAINST ALL ENDF/B-VII.    OF  Groupie

*INCREASED PAGE SIZE FROM 120,000 TO    Groupie

600,000 POINTS                         Groupie

Groupie

OWNED, MAINTAINED AND DISTRIBUTED BY                               Groupie

------------------------------------                               Groupie

THE NUCLEAR DATA SECTION                                           Groupie

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Groupie

P.O. BOX 100                                                       Groupie

A-1400, VIENNA, AUSTRIA                                            Groupie

EUROPE                                                             Groupie

Groupie

ORIGINALLY WRITTEN BY                                              Groupie

------------------------------------                               Groupie

DERMOTT E. CULLEN                                                  Groupie

UNIVERSITY OF CALIFORNIA                                           Groupie

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Groupie

L-159                                                              Groupie

P.O. BOX 808                                                       Groupie

LIVERMORE, CA 94550                                                Groupie

U.S.A.                                                             Groupie

TELEPHONE  925-423-7359                                            Groupie

E. MAIL    CULLEN1@LLNL.GOV                                        Groupie

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Groupie

Groupie

AUTHORS MESSAGE                                                    Groupie

---------------                                                    Groupie

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Groupie

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Groupie

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Groupie

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Groupie

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Groupie

Groupie

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Groupie

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Groupie

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Groupie

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Groupie

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Groupie

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Groupie

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Groupie

COMPUTER.                                                          Groupie

Groupie

PURPOSE                                                            Groupie

-------                                                            Groupie

THIS PROGRAM IS DESIGNED TO CALCULATE ANY COMBINATION OF           Groupie

THE FOLLOWING QUANTITIES FROM LINEARLY INTERPOLABLE TABULATED      Groupie

CROSS SECTIONS IN THE ENDF/B FORMAT                                Groupie

Groupie

(1) UNSHIELDED GROUP AVERAGED CROSS SECTIONS                       Groupie

(2) BONDARENKO SELF-SHIELDED GROUP AVERAGED CROSS SECTIONS         Groupie

(3) MULTI-BAND PARAMETERS                                          Groupie

Groupie

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Groupie

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Groupie

DISK OR ANY OTHER MEDIUM.                                          Groupie

Groupie

ENDF/B FORMAT                                                      Groupie

-------------                                                      Groupie

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Groupie

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Groupie

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV OR V FORMAT).     Groupie

Groupie

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Groupie

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Groupie

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE  Groupie

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Groupie

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Groupie

AND ALL SECTIONS OF MF= 3 MUST BE CORRECT. THE PROGRAM COPIES ALL  Groupie

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Groupie

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Groupie

Groupie

ALL FILE 3 CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE    Groupie

LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B          Groupie

INTERPOLATION LAW 2). FILE 3 BACKGROUND CROSS SECTIONS MAY BE MADE Groupie

LINEARLY INTERPOLABLE USING PROGRAM LINEAR (UCRL-50400, VOL. 17,   Groupie

PART A). THE RESONANCE CONTRIBUTION MAY BE ADDED TO THE BACKGROUND Groupie

CROSS SECTIONS USING PROGRAM RECENT (UCRL-50400, VOL. 17, PART B). Groupie

IF THIS PROGRAM FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT       Groupie

LINEARLY INTERPOLABLE THIS PROGRAM WILL TERMINATE EXECUTION.       Groupie

Groupie

CONTENTS OF OUTPUT                                                 Groupie

------------------                                                 Groupie

IF ENDF/B FORMATTED OUTPUT IS REQUESTED ENTIRE EVALUATIONS ARE     Groupie

OUTPUT, NOT JUST THE MULTI-GROUPED FILE 3 CROSS SECTIONS, E.G.     Groupie

ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO INCLUDED.                Groupie

Groupie

DOCUMENTATION                                                      Groupie

-------------                                                      Groupie

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Groupie

BY THE ADDITION OF THREE COMMENT CARDS AT THE END OF EACH          Groupie

HOLLERITH SECTION TO DESCRIBE THE GROUP STRUCTURE AND WEIGHTING    Groupie

SPECTRUM, E.G.                                                     Groupie

Groupie

********************** PROGRAM GROUPIE (2007-1) ***************    Groupie

UNSHIELDED GROUP AVERAGES USING   69 GROUPS (WIMS)                 Groupie

MAXWELLIAN, 1/E AND FISSION WEIGHTING SPECTRUM                     Groupie

Groupie

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR, RECENT AND SIGMA1) Groupie

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Groupie

THE DATA.                                                          Groupie

Groupie

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Groupie

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Groupie

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Groupie

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Groupie

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Groupie

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Groupie

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Groupie

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Groupie

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Groupie

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Groupie

Groupie

REACTION INDEX                                                     Groupie

--------------                                                     Groupie

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Groupie

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Groupie

Groupie

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Groupie

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Groupie

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Groupie

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Groupie

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Groupie

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Groupie

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Groupie

Groupie

SECTION SIZE                                                       Groupie

------------                                                       Groupie

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Groupie

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Groupie

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Groupie

Groupie

SELECTION OF DATA                                                  Groupie

-----------------                                                  Groupie

THE PROGRAM SELECTS MATERIALS TO BE PROCESSED BASED EITHER ON      Groupie

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Groupie

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Groupie

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Groupie

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Groupie

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Groupie

Groupie

ENERGY ORDER AND UNITS                                             Groupie

----------------------                                             Groupie

ALL ENERGIES (FOR CROSS SECTIONS, WEIGHTING SPECTRUM OR GROUP      Groupie

BOUNDARIES) MUST BE IN UNITS OF EV AND MUST BE IN ASCENDING        Groupie

NUMERICAL ORDER.                                                   Groupie

Groupie

ENERGY GRID                                                        Groupie

-----------                                                        Groupie

ALTHOUGH ALL REACTIONS MUST TO LINEARLY INTERPOLABLE, THEY DO NOT  Groupie

ALL HAVE TO USE THE SAME ENERGY GRID. EACH REACTION CAN BE GIVEN   Groupie

BY AN INDEPENDENT ENERGY GRID. THIS PROGRAM WILL PROCEED FROM      Groupie

THE LOWEST TO HIGHEST ENERGY SELECTING EACH ENERGY INTERVAL OVER   Groupie

WHICH ALL DATA, FOR ANY GIVEN CALCULATION, ARE ALL LINEARLY        Groupie

INTERPOLABLE.                                                      Groupie

Groupie

GROUP STRUCTURE                                                    Groupie

---------------                                                    Groupie

THIS PROGRAM IS DESIGNED TO USE AN ARBITRARY ENERGY GROUP          Groupie

STRUCTURE WHERE THE ENERGIES ARE IN EV AND ARE IN INCREASING       Groupie

ENERGY ORDER. THE MAXIMUM NUMBER OF GROUPS IS 1000.                Groupie

Groupie

THE USER MAY INPUT AN ARBITRARY GROUP STRUCTURE OR THE USER MAY    Groupie

USE USE ONE OF THE SEVEN BUILT-IN GROUP STRUCTURES.                Groupie

(0) 175 GROUP (TART STRUCTURE)                                     Groupie

(1)  50 GROUP (ORNL STRUCTURE)                                     Groupie

(2) 126 GROUP (ORNL STRUCTURE)                                     Groupie

(3) 171 GROUP (ORNL STRUCTURE)                                     Groupie

(4) 620 GROUP (SAND-II STRUCTURE, UP TO 18 MEV)                    Groupie

(5) 640 GROUP (SAND-II STRUCTURE, UP TO 20 MEV)                    Groupie

(6)  69 GROUP (WIMS STRUCTURE)                                     Groupie

(7)  68 GROUP (GAM-I STRUCTURE)                                    Groupie

(8)  99 GROUP (GAM-II STRUCTURE)                                   Groupie

(9)  54 GROUP (MUFT STRUCTURE)                                     Groupie

(10)  28 GROUP (ABBN STRUCTURE)                                     Groupie

(11) 650 GROUP (TART STRUCTURE)                                     Groupie

(12) 700 GROUP (TART STRUCTURE)                                     Groupie

(13) 665 GROUP (SAND-II STRUCTURE, 1.0e-5 eV, UP TO 18 MEV)         Groupie

(14) 685 GROUP (SAND-II STRUCTURE, 1.0e-5 eV, UP TO 20 MEV)         Groupie

Groupie

GROUP AVERAGES                                                     Groupie

--------------                                                     Groupie

THIS PROGRAM DEFINES GROUP AVERAGED CROSS SECTIONS AS...           Groupie

Groupie

(INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)             Groupie

AVERAGE = -----------------------------------------                Groupie

(INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)                      Groupie

WHERE...                                                           Groupie

Groupie

AVERAGE  = GROUP AVERAGED CROSS SECTION                            Groupie

E1, E2   = ENERGY LIMITS OF THE GROUP                              Groupie

SIGMA(E) = ENERGY DEPENDENT CROSS SECTION FOR ANY GIVEN REACTION   Groupie

S(E)     = ENERGY DEPENDENT WEIGHTING SPECTRUM                     Groupie

WT(E)    = ENERGY DEPENDENT SELF-SHIELDING FACTOR.                 Groupie

Groupie

ENERGY DEPENDENT WEIGHTING SPECTRUM                                Groupie

-----------------------------------                                Groupie

THE ENERGY DEPENDENT WEIGHTING SPECTRUM IS GIVEN BY AN ARBITRARY   Groupie

TABULATED LINERLY INTERPOLABLE FUNCTION WHICH CAN BE DESCRIBED     Groupie

BY AN ARBITRARY NUMBER OF POINTS. THIS ALLOWS THE USER TO          Groupie

SPECIFY ANY DESIRED WEIGHTING SPECTRUM TO ANY GIVEN DEGREE OF      Groupie

ACCURACY. REMEMBER THAT THE PROGRAM WILL ASSUME THAT THE SPECTRUM  Groupie

IS LINEARLY INTERPOLABLE BETWEEN TABULATED POINTS. THEREFORE THE   Groupie

USER SHOULD USE ENOUGH POINTS TO INSURE AN ADEQUATE REPRESENTATION Groupie

OF THE SPECTRUM BETWEEN TABULATED DATA POINTS.                     Groupie

Groupie

THE PRESENT VERSION OF THE CODE HAS THREE BULIT-IN WEIGHTING       Groupie

SPECTRA,                                                           Groupie

Groupie

(1) CONSTANT                                                       Groupie

(2) 1/E                                                            Groupie

(3) MAXWELLIAN = E*EXP(-E/KT)/KT                (0.0 TO 4*KT)      Groupie

1/E        = C1/E                           (4*KT TO 67 KEV)   Groupie

FISSION    = C2*EXP(-E/WA)*SINH(SQRT(E*WB)) (ABOVE 67 KEV)     Groupie

Groupie

KT     = 0.253 EV (293 KELVIN)                                 Groupie

WA     = 9.65E+5                                               Groupie

WB     = 2.29E-6                                               Groupie

C1, C2 = DEFINED TO MAKE SPECTRUM CONTINUOUS                   Groupie

Groupie

FISSION SPECTRUM CONSTANTS FROM                                Groupie

A.F.HENRY, NUCLEAR REACTOR ANALYSIS, P. 11, MIT PRESS (1975)   Groupie

Groupie

UNSHIELDED GROUP AVERAGES                                          Groupie

-------------------------                                          Groupie

FOR UNSHIELDED AVERAGES THE SELF-SHIELDING FACTOR (WT(E)) IS SET   Groupie

TO UNITY. THIS PROGRAM ALLOWS UP TO 1000 GROUPS.                   Groupie

Groupie

SELF-SHIELDED GROUP AVERAGES                                       Groupie

----------------------------                                       Groupie

IF SELF-SHIELDED AVERAGES AND/OR MULTI-BAND PARAMETERS ARE         Groupie

CALCULATED THIS PROGRAM ALLOWS UP TO 1000 GROUPS. SELF-SHIELDED    Groupie

AVERAGES AND/OR MULTI-BAND PARAMETERS ARE CALCULATED FOR THE       Groupie

TOTAL, ELASTIC, CAPTURE AND FISSION.                               Groupie

Groupie

FOR THE TOTAL, ELASTIC, CAPTURE AND FISSION THE PROGRAM USES A     Groupie

WEIGHTING FUNCTION THAT IS A PRODUCT OF THE ENERGY DEPENDENT       Groupie

WEIGHTING SPECTRUM TIMES A BONDERENKO TYPE SELF-SHIELDING FACTOR.  Groupie

Groupie

WT(E) = S(E)/(TOTAL(E)+SIGMA0)**N                                  Groupie

Groupie

WHERE...                                                           Groupie

Groupie

S(E)     - ENERGY DEPENDENT WEIGHTING SPECTRUM (DEFINED BY         Groupie

TABULATED VALUES AND LINEAR INTERPOLATION BETWEEN       Groupie

TABULATED VALUES).                                      Groupie

TOTAL(E) - ENERGY DEPENDENT TOTAL CROSS SECTION FOR ONE MATERIAL   Groupie

(DEFINED BY TABULATED VALUES AND LINEAR INTERPOLATION   Groupie

BETWEEN TABULATED VALUES).                              Groupie

SIGMA0   - CROSS SECTION TO REPRESENT THE EFFECT OF ALL OTHER      Groupie

MATERIALS AND LEAKAGE (DEFINED WITHIN EACH GROUP TO BE  Groupie

A MULTIPLE OF THE UNSHIELDED TOTAL CROSS SECTION WITHIN Groupie

THAT GROUP OR POWERS OF 10 - INPUT OPTION).             Groupie

N        - A POSITIVE INTEGER (0, 1, 2 OR 3).                      Groupie

Groupie

THE PROGRAM WILL USE ONE ENERGY DEPENDENT WEIGHTING SPECTRUM S(E)  Groupie

AND 25 DIFFERENT BONDERENKO TYPE SELF-SHIELDING FACTORS (25 SIGMA0 Groupie

AND N COMBINATIONS) TO DEFINE 25 DIFFERENT AVERAGE CROSS SECTIONS, Groupie

FOR EACH REACTION, WITHIN EACH GROUP.                              Groupie

Groupie

THE 25 WEIGHTING FUNCTIONS USED ARE....                            Groupie

(1)   - UNSHIELDED CROSS SECTIONS (N=0)                            Groupie

(2-22)- PARTIALLY SHIELDED CROSS SECTIONS (N=1 ,VARIOUS SIGMA0)    Groupie

THE VALUES OF SIGMA0 USED WILL BE EITHER,                  Groupie

(A) THE VALUES OF SIGMA0 THAT ARE USED VARY FROM 1024      Groupie

TIMES THE UNSHIELDED TOTAL CROSS SECTIONS IN STEPS OF 1/2  Groupie

DOWN TO 1/1024 TIMES THE UNSHIELDED TOTAL CROSS SECTION    Groupie

(A RANGE OF OVER 1 MILLION, CENTERED ON THE UNSHIELDED     Groupie

TOTAL CROSS SECTION WITHIN EACH GROUP).                    Groupie

(B) THE SAME CONSTANT VALUES OF SIGMA0 IN EACH GROUP. THE  Groupie

VALUES OF SIGMA0 USED INCLUDE 40000, 20000, 10000, 7000,   Groupie

4000, 2000, 1000, 700, 400, 200, 100, 70, 40, 20, 10, 7,   Groupie

4, 2, 1, 0.7, 0.4 (A RANGE OF 100,000 SPANNING MORE THAN   Groupie

THE RANGE OF SIGMA0 VALUES THAT MAY BE ENCOUNTERED IN      Groupie

ACTUAL APPLICATIONS)                                       Groupie

(23)  - TOTALLY SHIELDED FLUX WEIGHTED CROSS SECTION               Groupie

(N=1, SIGMA0=0)                                            Groupie

(24)  - TOTALLY SHIELDED CURRENT WEIGHTED CROSS SECTION            Groupie

(N=2, SIGMA0=0)                                            Groupie

(25)  - TOTALLY SHIELDED COSINE SQUARED WEIGHTED CROSS SECTION     Groupie

(N=3, SIGMA0=0)                                            Groupie

Groupie

FOR ALL OTHER REACTIONS (EXCEPT TOTAL, ELASTIC, CAPTURE AND        Groupie

FISSION) THE PROGRAM WILL USE THE ENERGY DEPENDENT WEIGHTING       Groupie

SPECTRUM S(E) TO DEFINE THE UNSHIELDED (BONDERENKO N=0)            Groupie

AVERAGED CROSS SECTION WITHIN EACH GROUP.                          Groupie

Groupie

CALCULATION OF RESONANCE INTEGRALS                                 Groupie

----------------------------------                                 Groupie

IN A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH A         Groupie

CONSTANT CROSS SECTION THE SPECTRUM WILL BE 1/E AND THERE WILL     Groupie

BE NO SELF-SHIELDING.                                              Groupie

Groupie

IN THIS CASE IF THE CROSS SECTION VARIES WITH ENERGY THE           Groupie

SPECTRUM WILL STILL BE 1/E AND THE SELF-SHIELDING FACTOR WILL      Groupie

BE EXACTLY 1/SIG-TOT(E) - WHERE SIG-TOT(E) = SIG-EL(E), SINCE      Groupie

THERE IS ONLY SCATTERING.                                          Groupie

Groupie

IF WE HAVE AN INFINITELY DILUTE AMOUNT OF A MATERIAL UNIFORMLY     Groupie

MIXED WITH A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH   Groupie

A CONSTANT CROSS SECTION THE STANDARD DEFINITION OF THE RESONANCE  Groupie

INTEGRAL CAN BE USED TO DEFINE REACTION RATES FOR EACH REACTION.   Groupie

Groupie

THE RESONANCE INTEGRAL IS DEFINED AS,                              Groupie

Groupie

RI      = (INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)             Groupie

Groupie

WHERE NORMALLY,                                                    Groupie

S(E)    = 1/E                                                      Groupie

WT(E)   = 1    - NO SELF-SHIELDING                                 Groupie

Groupie

FROM THE ABOVE DEFINITION OF GROUP AVERAGED CROSS SECTIONS THE     Groupie

RESONANCE INTEGRAL IS,                                             Groupie

Groupie

RI      = AVERAGE * (INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)            Groupie

Groupie

FOR A 1/E SPECTRUM AND NO SELF-SHIELDING THIS REDUCES TO,          Groupie

Groupie

RI      = AVERAGE* LOG(E2/E1)                                      Groupie

Groupie

IN ANY OTHER SITUATION, INCLUDING ABSORPTION AND/OR ENERGY         Groupie

DEPENDENT CROSS SECTIONS, THE SPECTRUM WILL NOT BE 1/E -           Groupie

ABSORPTION WILL TEND TO DECREASE THE SPECTRUM PROGRESSIVELY        Groupie

MORE AT LOWER ENERGIES - ENERGY DEPENDENCE OF THE CROSS SECTION    Groupie

WILL LEAD TO SELF-SHIELDING.                                       Groupie

Groupie

HERE WE WILL NOT ATTEMPT TO PERFORM A DETAILED SPECTRUM            Groupie

CALCULATION TO ACCOUNT FOR ABSORPTION.                             Groupie

Groupie

HOWEVER, WE WILL EXTEND THE DEFINITION OF THE RESONANCE INTEGRAL   Groupie

TO ACCOUNT FOR SELF-SHIELDING EFFECTS BY ALLOWING FOR INCLUSION    Groupie

OF SELF-SHIELDING EFFECTS IN THE DEFINITION OF GROUP AVERAGES      Groupie

AND THEN DEFINING THE RESONANCE INTEGRAL AS,                       Groupie

Groupie

RI      = AVERAGE* LOG(E2/E1)                                      Groupie

Groupie

IN ORDER TO CALCULATE RESONANCE INTEGRALS YOU MUST FOLLOW THESE    Groupie

STEPS,                                                             Groupie

Groupie

1) SELECT A 1/E SPECTRUM - ON FIRST LINE OF INPUT PARAMETERS.      Groupie

2) SELECT THE ENERGY BOUNDARIES - NORMALLY ONLY 1 GROUP FROM       Groupie

0.5 EV UP TO 20 MEV - HOWEVER, YOU ARE FREE TO SELECT ANY       Groupie

ENERGY RANGE THAT YOU WISH - YOU MAY EVEN SELECT MORE THAN      Groupie

1 GROUP MERELY BY SPECIFYING MORE THAN 1 GROUP AS INPUT -       Groupie

THIS CAN BE USED TO DEFINE THE CONTRIBUTIONS TO THE RESONANCE   Groupie

INTEGRAL FROM INDIVIDUAL ENERGY RANGES.                         Groupie

3) SELECT THIS OPTION FOR THE UNSHIELDED AND/OR SHIELDED OUTPUT    Groupie

LISTING - ON THE SECOND LINE OF INPUT PARAMETERS.               Groupie

Groupie

WHEN THIS OPTION IS USED THE PROGRAM WILL CALCULATE GROUP AVERAGED Groupie

CROSS SECTIONS - AS DEFINED ABOVE - PRIOR TO OUTPUT THE RESULTS    Groupie

WILL MERELY BE MULTIPLIED BY THE WIDTH OF THE GROUP ASSUMING YOU   Groupie

HAVE SELECTED A 1/E SPECTRUM - THERE IS NO CHECK ON THIS - THE     Groupie

PROGRAM MERELY MULTIPLIES THE GROUP AVERAGED CROSS SECTIONS BY,    Groupie

Groupie

LOG(E2/E1) - WHERE E2 AND E1 ARE THE GROUP ENERGY BOUNDARIES.      Groupie

Groupie

WARNING - IT IS UP TO YOU TO INSURE THAT YOU FOLLOW EXACTLY THE    Groupie

STEPS OUTLINED ABOVE IF YOU WISH TO OBTAIN MEANINGFUL    Groupie

RESULTS.                                                 Groupie

Groupie

NOTE - OUTPUT IN THE ENDF/B FORMAT IS ALWAYS GROUP AVERAGED CROSS  Groupie

SECTIONS, REGARDLESS OF WHETHER YOU ASK FOR AVERAGED CROSS  Groupie

SECTIONS OR RESONANCE INTEGRALS - THIS IS BECAUSE DATA IN   Groupie

THE ENDF/B FORMAT IS EXPLICITLY DEFINED TO BE CROSS         Groupie

SECTIONS.                                                   Groupie

Groupie

RESONANCE INTEGRAL OUTPUT CAN ONLY BE OBTAINED IN THE       Groupie

LISTING FORMATS.                                            Groupie

Groupie

MINIMUM TOTAL CROSS SECTION TREATMENT                              Groupie

-------------------------------------                              Groupie

SINCE THE BONDARENKO SELF-SHIELDING DEPENDS ON 1/TOTAL CROSS       Groupie

SECTION, THE ALGORITHM WILL BECOME NUMERICALLY UNSTABLE IF THE     Groupie

TOTAL CROSS SECTION IS NEGATIVE (AS OCCURS IN MANY ENDF/B          Groupie

EVALUATIONS). IF THE TOTAL IS LESS THAN SOME MINIMUM ALLOWABLE     Groupie

VALUE (DEFINE BY OKMIN, PRESENTLY 1 MILLI-BARN) AN ERROR MESSAGE   Groupie

WILL BE PRINTED AND FOR THE SELF-SHIELDING CALCULATION ALL ENERGY  Groupie

INTERVALS IN WHICH THE TOTAL IS LESS THAN THE MINIMUM WILL BE      Groupie

IGNORED.                                                           Groupie

Groupie

NOTE, FOR THE UNSHIELDED CALCULATIONS ALL CROSS SECTIONS WILL BE   Groupie

CONSIDERED WHETHER THEY ARE POSITIVE OR NEGATIVE. THEREFORE IF     Groupie

THE TOTAL CROSS SECTION IS NEGATIVE OR LESS THAN THE MINIMUM       Groupie

VALUE THERE MAY BE AN INCONSISTENCY BETWEEN THE UNSHIELDED AND     Groupie

THE SELF-SHIELDED CROSS SECTIONS. IF THE TOTAL CROSS SECTION IS    Groupie

NEGATIVE AND SELF-SHIELDED CROSS SECTIONS ARE CALCULATED THE       Groupie

PROGRAM WILL PRINT AN ERROR MESSAGE INDICATING THAT THE SELF-      Groupie

SHIELDED RESULTS ARE UNRELIABLE AND SHOULD NOT BE USED. THEREFORE  Groupie

IN THIS CASE THE PROGRAM WILL NOT ATTEMPT TO MODIFY THE UNSHIELDED Groupie

RESULTS TO ELIMINATE THE EFFECT OF NEGATIVE CROSS SECTIONS, SINCE  Groupie

THE UNSHIELDED RESULTS ARE THE ONLY ONES WHICH TRULY REFLECT THE   Groupie

ACTUAL INPUT.                                                      Groupie

Groupie

RESOLVED RESONANCE REGION                                          Groupie

-------------------------                                          Groupie

IN THE RESOLVED RESONANCE REGION (ACTUALLY EVERYWHERE BUT IN THE   Groupie

UNRESOLVED RESONANCE REGION) THE CROSS SECTIONS OUTPUT BY LINEAR-  Groupie

RECENT-SIGMA1 WILL BE ACTUAL ENERGY DEPENDENT CROSS SECTIONS AND   Groupie

THE CALCULATIONS BY THIS PROGRAM WILL YIELD ACTUAL SHIELDED AND    Groupie

UNSHIELDED CROSS SECTIONS.                                         Groupie

Groupie

UNRESOLVED RESONANCE REGION                                        Groupie

---------------------------                                        Groupie

IN THE UNRESOLVED RESONANCE REGION PROGRAM RECENT USES THE         Groupie

UNRESOLVED RESONANCE PARAMETERS TO CALCULATE INFINITELY DILUTE     Groupie

AVERAGE CROSS SECTIONS. THIS PROGRAM WILL MERELY READ THIS         Groupie

INFINITELY DILUTE DATA AS IF IT WERE ENERGY DEPENDENT DATA AND     Groupie

GROUP AVERAGE IT. AS SUCH THIS PROGRAM WILL PRODUCE THE CORRECT    Groupie

UNSHIELDED CROSS SECTION IN THE UNRESOLVED RESONANCE REGION, BUT   Groupie

IT WILL NOT PRODUCE THE CORRECT SELF-SHIELDING EFFECTS.            Groupie

Groupie

ACCURACY OF RESULTS                                                Groupie

-------------------                                                Groupie

ALL INTEGRALS ARE PERFORMED ANALYTICALLY. THEREFORE NO ERROR IS    Groupie

INTRODUCED DUE TO THE USE OF TRAPAZOIDAL OR OTHER INTEGRATION      Groupie

SCHEME. THE TOTAL ERROR THAT CAN BE ASSIGNED TO THE RESULTING      Groupie

AVERAGES IS JUST THAT DUE TO THE ERROR IN THE CROSS SECTIONS       Groupie

AND ENERGY DEPENDENT WEIGHTING SPECTRUM. GENERALLY SINCE THE       Groupie

THE ENERGY DEPENDENT WEIGHTING SPECTRUM APPEARS IN BOTH THE        Groupie

NUMERATOR AND THE DENOMINATOR THE AVERAGES RAPIDLY BECOME          Groupie

INSENSITIVE TO THE WEIGHTING SPECTRUM AS MORE GROUPS ARE USED.     Groupie

SINCE THE WEIGHTING SPECTRUM IS LOADED IN THE PAGING SYSTEM THE    Groupie

USER CAN DESCRIBE THE SPECTRUM TO ANY REQUIRED ACCURACY USING      Groupie

ANY NUMBER OF ENERGY VS. SPECTRUM PAIRS.                           Groupie

Groupie

MULTI-BAND PARAMETERS                                              Groupie

---------------------                                              Groupie

MULTI-BAND PARAMETERS ARE CALCULATED FOR THE TOTAL, ELASTIC,       Groupie

CAPTURE AND FISSION REACTIONS. WITH THE NUMBER OF GROUPS THAT      Groupie

ARE NORMALLY USED (SEE BUILT IN GROUP STRUCTURES) ALL OTHER        Groupie

REACTIONS RESULT IN A NEGLIGABLE AMOUNT OF SELF-SHIELDING. AS      Groupie

SUCH THEIR EQUIVALENT BAND CROSS SECTION WILL MERELY BE THEIR      Groupie

UNSHIELDED VALUE WITHIN EACH BAND.                                 Groupie

Groupie

FOR ANY GIVEN EVALUATION, WITHIN ANY GIVEN GROUP THIS PROGRAM      Groupie

WILL GENERATE THE MINIMUM NUMBER OF BANDS REQUIRED WITHIN THAT     Groupie

GROUP. AS OUTPUT TO THE COMPUTER READABLE DISK FILE THE BAND       Groupie

PARAMETERS FOR EACH EVALUATION WILL BE FORMATTED TO HAVE THE       Groupie

SAME NUMBER OF BANDS IN ALL GROUPS (WITH ZERO WEIGHT FOR SOME      Groupie

BANDS WITHIN ANY GROUP). THE USER MAY DECIDE TO HAVE OUTPUT        Groupie

EITHER WITH THE MINIMUM NUMBER OF BANDS REQUIRED FOR EACH          Groupie

EVALUATION (E.G. 2 BANDS FOR HYDROGEN AND 4 BANDS FOR U-233) OR    Groupie

THE SAME NUMBER OF BANDS FOR ALL EVALUATIONS (E.G. 4 BANDS FOR     Groupie

BOTH HYDROGEN AND U-233).                                          Groupie

Groupie

FOR 2 OR FEWER BANDS THE PROGRAM USES AN ANALYTIC EXPRESSION       Groupie

TO DEFINE ALL MULTI-BAND PARAMETERS. FOR MORE THAN 2 BANDS THE     Groupie

PROGRAM PERFORMS A NON-LINEAR FIT TO SELECT THE MULTI-BAND         Groupie

PARAMETERS THAT MINIMIZE THE MAXIMUM FRACTIONAL ERROR AT ANY       Groupie

POINT ALONG THE ENTIRE SELF-SHIELDING CURVE. THE NUMBER OF BANDS   Groupie

REQUIRED WITHIN ANY GIVEN GROUP IS DEFINED BY INSURING THAT THE    Groupie

MULTI-BAND PARAMETERS CAN BE USED TO ACCURATELY DEFINE SELF-       Groupie

SHIELDED CROSS SECTIONS ALONG THE ENTIRE SELF-SHIELDING CURVE      Groupie

FROM SIGMA0 = 0 TO INFINITY. THE USER MAY DEFINE THE ACCURACY      Groupie

REQUIRED.                                                          Groupie

Groupie

ENDF/B FORMATTED UNSHIELDED AVERAGES                               Groupie

------------------------------------                               Groupie

UNSHIELDED MULTI-GROUP AVERAGED CROSS SECTIONS FOR ALL REACTIONS   Groupie

MAY BE OBTAINED IN THE ENDF/B FORTRAN IN EITHER HISTOGRAM          Groupie

(INTERPOLATION LAW 1) OR LINEARLY INTERPOLABLE (INTERPOLATION      Groupie

LAW 2) FORM. SEE INPUT BELOW FOR DETAILS.                          Groupie

Groupie

MIXTURES OF MATERIALS AND RESONANCE OVERLAP                        Groupie

-------------------------------------------                        Groupie

THE SELF-SHIELDED CROSS SECTIONS FOR THE INDIVIDUAL CONSTITUENTS   Groupie

OF ANY MIXTURE CAN BE CALCULATED BY THIS PROGRAM BY REALIZING THAT Groupie

THIS PROGRAM ESSENTIALLY ONLY USES THE TOTAL CROSS SECTION AS A    Groupie

WEIGHTING FUNCTION TO ACCOUNT FOR SELF-SHIELDING EFFECTS. FOR A    Groupie

MIXTURE IT IS THEREFORE ONLY NECESSARY TO USE THE TOTAL CROSS      Groupie

SECTION FOR THE MIXTURE IN PLACE OF THE ACTUAL TOTAL CROSS SECTION Groupie

FOR EACH CONSTITUENT AND TO RUN THIS PROGRAM. THIS CAN BE DONE BY  Groupie

FIRST RUNNING PROGRAM MIXER TO CALCULATE THE ENERGY DEPENDENT      Groupie

TOTAL CROSS SECTION FOR ANY COMPOSITE MIXTURE. NEXT, SUBSTITUTE    Groupie

THIS COMPOSITE TOTAL CROSS SECTION FOR THE ACTUAL TOTAL CROSS      Groupie

SECTION OF EACH CONSTITUENT (IN EACH ENDF/B FORMATTED EVALUATION). Groupie

FINALLY, RUN THIS PROGRAM TO CALCULATE THE SELF-SHIELDED CROSS     Groupie

SECTION FOR EACH CONSTITUENT, PROPERLY ACCOUNTING FOR RESONANCE    Groupie

OVERLAP BETWEEN THE RESONANCES OF ALL OF THE CONSTITUENTS OF THE   Groupie

MIXTURE. DURING THE SAME RUN THESE SELF-SHIELDED CROSS SECTIONS    Groupie

CAN IN TURN BE USED TO CALCULATE FULLY CORRELATED MULT-BAND        Groupie

Groupie

MULTI-BAND PARAMETER OUTPUT FORMAT                                 Groupie

----------------------------------                                 Groupie

FOR VERSIONS 92-2 AND LATER VERSIONS THE MULTI-BAND PARAMETERS     Groupie

ARE OUTPUT IN A SIMPLE CHARACTER FORMAT, THAT CAN BE TRANSFERRED   Groupie

AND USED ON VIRTUALLY ANY COMPUTER.                                Groupie

Groupie

THE BINARY FORMAT USED IN EARLIER VERSIONS OF THIS CODE IS NO      Groupie

LONGER USED.                                                       Groupie

Groupie

CONTACT THE AUTHOR IF YOU WOULD LIKE TO RECEIVE A SIMPLE PROGRAM   Groupie

TO READ THE CHARACTER FORMATTED MULTI-BAND PARAMETER FILE AND      Groupie

CREATE A BINARY, RANDOM ACCESS FILE FOR USE ON VIRTUALLY ANY       Groupie

COMPUTER.                                                          Groupie

Groupie

THE FORMAT OF THE CHARACTER FILE IS,                               Groupie

Groupie

RECORD   COLUMNS   FORMAT   DESCRIPTION                            Groupie

1       1-72     18A4    LIBRARY DESCRIPTION (AS READ)          Groupie

2       1-11      I11    MATERIAL ZA                            Groupie

12-22      I11    NUMBER GROUPS                          Groupie

23-33      I11    NUMBER OF BANDS                        Groupie

34-44     D11.4   TEMPERATURE (KELVIN)                   Groupie

45-55    1X,10A1  HOLLERITH DESCRIPTION OF ZA            Groupie

3       1-11     D11.4   ENERGY (EV) - GROUP BOUNDARY.          Groupie

12-22     D11.4   TOTAL      (FIRST BAND)                Groupie

23-33     D11.4   ELASTIC                                Groupie

34-44     D11.4   CAPTURE                                Groupie

35-55     D11.4   FISSION                                Groupie

4       1-11     -----   BLANK                                  Groupie

12-22     D11.4   TOTAL      (SECOND BAND)               Groupie

23-33     D11.4   ELASTIC                                Groupie

34-44     D11.4   CAPTURE                                Groupie

35-55     D11.4   FISSION                                Groupie

Groupie

LINES 3 AND 4 ARE REPEATED FOR EACH GROUP. THE LAST LINE FOR EACH  Groupie

MATERIAL (ZA) IS,                                                  Groupie

Groupie

N       1-11     D11.4   ENERGY (EV) - UPPER ENERGY LIMIT OF    Groupie

LAST GROUP.              Groupie

Groupie

FOR EXAMPLE, A 175 GROUP, 2 BAND FILE, FOR EACH MATERIAL WILL      Groupie

CONTAIN 352 LINES = 1 HEADER LINE, 175 * 2 LINES OF PARAMETERS,    Groupie

AND 1 FINAL LINE WITH THE UPPER ENERGY LIMIT   Groupie

OF THE LAST GROUP.                             Groupie

Groupie

INPUT FILES                                                        Groupie

-----------                                                        Groupie

UNIT  DESCRIPTION                                                  Groupie

----  -----------                                                  Groupie

2   INPUT DATA (BCD - 80 CHARACTERS/RECORD)                      Groupie

10   ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Groupie

Groupie

OUTPUT FILES                                                       Groupie

------------                                                       Groupie

UNIT  DESCRIPTION                                                  Groupie

----  -----------                                                  Groupie

31   MULTI-BAND PARAMETERS CHARACTER FILE - OPTIONAL              Groupie

(BCD - 80 CHARACTERS/RECORD)                                 Groupie

32   SELF-SHIELDED CROSS SECTION LISTING - OPTIONAL               Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

33   MULTI-BAND PARAMETER LISTING - OPTIONAL                      Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

34   UNSHIELDED CROSS SECTION LISTING - OPTION                    Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

3   OUTPUT REPORT (BCD - 80 CHARACTERS/RECORD)                   Groupie

11   MULTI-GROUP ENDF/B DATA - OPTIONAL                           Groupie

(BCD - 80 CHARACTERS/RECORD)                                 Groupie

Groupie

SCRATCH FILES                                                      Groupie

-------------                                                      Groupie

UNIT  FILENAME  DESCRIPTION                                        Groupie

----  --------  -----------                                        Groupie

8   ENERGY DEPENDENT WEIGHTING SPECTRUM                          Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

9   TOTAL CROSS SECTION                                          Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

12   ELASTIC CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

13   CAPTURE CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

14   FISSION CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

Groupie

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)   Groupie

----------------------------------------------------------------   Groupie

UNIT  FILE NAME                                                    Groupie

----  ----------                                                   Groupie

2   GROUPIE.INP                                                  Groupie

3   GROUPIE.LST                                                  Groupie

8   (SCRATCH)                                                    Groupie

9   (SCRATCH)                                                    Groupie

10   ENDFB.IN                                                     Groupie

11   ENDFB.OUT                                                    Groupie

12   (SCRATCH)                                                    Groupie

13   (SCRATCH)                                                    Groupie

14   (SCRATCH)                                                    Groupie

31   MULTBAND.TAB                                                 Groupie

32   SHIELD.LST                                                   Groupie

33   MULTBAND.LST                                                 Groupie

34   UNSHIELD.LST                                                 Groupie

Groupie

I/O UNITS USED                                                    Groupie

--------------                                                    Groupie

UNITS 2, 3 8, 9 AND 10 WILL ALWAYS BE USED.                       Groupie

UNITS 31 THROUGH 34 AND 11 ARE OPTIONALLY USED DEPENDING ON THE   Groupie

OUTPUT REQUESTED.                                                 Groupie

UNITS 12, 13 AND 14 WILL ONLY BE USED IF SELF-SHIELDED OR         Groupie

MULTIBAND OUTPUT IS REQUESTED.                                    Groupie

Groupie

INPUT CARDS                                                        Groupie

-----------                                                        Groupie

CARD  COLS.  FORMAT  DESCRIPTION                                   Groupie

----  -----  ------  -----------                                   Groupie

1    1-11    I11   SELECTION CRITERIA (0=MAT, 1=ZA)              Groupie

1   12-22    I11   NUMBER OF GROUPS.                             Groupie

=.GT.0 - ARBITRARY GROUP BOUNDARIES ARE READ  Groupie

FROM INPUT FILE (N GROUPS REQUIRE    Groupie

N+1 GROUP BOUNDARIES). CURRENT       Groupie

PROGRAM MAXIMUM IS 1000 GROUPS.      Groupie

BUILT-IN OPTIONS INCLUDE....         Groupie

=  0   - TART    175 GROUPS                   Groupie

= -1   - ORNL     50 GROUPS                   Groupie

= -2   - ORNL    126 GROUPS                   Groupie

= -3   - ORNL    171 GROUPS                   Groupie

= -4   - SAND-II 620 (665) GROUPS TO 18 MEV   Groupie

= -5   - SAND-II 640 (685) GROUPS TO 20 MEV   Groupie

= -6   - WIMS     69 GROUPS                   Groupie

= -7   - GAM-I    68 GROUPS                   Groupie

= -8   - GAM-II   99 GROUPS                   Groupie

= -9   - MUFT     54 GROUPS                   Groupie

=-10   - ABBN     28 GROUPS                   Groupie

=-11   - TART    650 GROUPS                   Groupie

=-12   - TART    700 GROUPS                   Groupie

=-13   - SAND-II 665 GROUPS TO 18 MEV         Groupie

=-14   - SAND-II 685 GROUPS TO 20 MEV         Groupie

1   23-33    I11   MULTI-BAND SELECTOR                           Groupie

=  0 - NO MULTI-BAND CALCULATIONS             Groupie

=  1 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)    Groupie

AND AV(1/TOT**2)                       Groupie

=  2 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)    Groupie

AND AV(1/(TOT+SIGMA0)) WHERE           Groupie

SIGMA0 = AV(TOT) IN EACH GROUP         Groupie

= 3-5- MULTI-BAND FIT. CONSERVE AV(TOT) AND   Groupie

MINIMIZE FRACTIONAL ERROR FOR ENTIRE   Groupie

SELF-SHIELDING CURVE (SIGMA0 = 0 TO    Groupie

INFINITY)                              Groupie

IF THE SELECTOR IS POSITIVE (1 TO 5) THE      Groupie

MINIMUM NUMBER OF BANDS WILL BE OUTPUT FOR    Groupie

EACH ISOTOPE INDEPENDENTLY. IF THE SELECTOR   Groupie

IS NEGATIVE (-1 TO -5) THE SAME NUMBER OF     Groupie

BANDS (ABS(SELECTOR)) WILL BE OUTPUT FOR      Groupie

ALL ISOTOPES.                                 Groupie

1   34-44    I11   NUMBER OF POINTS USED TO DESCRIBE ENERGY      Groupie

DEPENDENT WEIGHTING SPECTRUM S(E).            Groupie

= -2    - MAXWELLIAN - UP TO 0.1 EV           Groupie

1/E        - 0.1 EV TO 67 KEV       Groupie

FISSION    - ABOVE 67 KEV           Groupie

05/01/20-----------------ADDED OPTION TO ALLOW TEMPERATURE OF THE      Groupie

MAXWELLIAN TO BE CHANGED - SEE INPUT LINE 4,  Groupie

COLUMNS 55 - 66.                              Groupie

= -1    - 1/E                                 Groupie

= 0 OR 1- ENERGY INDEPENDENT (SO CALLED FLAT  Groupie

WEIGHTING SPECTRUM).                Groupie

= .GT.1 - READ THIS MANY POINTS FROM INPUT    Groupie

TO DESCRIBE WEIGHTING SPECTRUM.     Groupie

NO LIMIT TO THE NUMBER OF POINTS    Groupie

USED TO DESCRIBE WEIGHTING.         Groupie

1   45-55   D11.4  MULTI-BAND CONVERGENCE CRITERIA.              Groupie

ONLY USED FOR 3 OR MORE BANDS. THE NUMBER OF  Groupie

BANDS IN EACH GROUPS IS SELECTED TO INSURE    Groupie

THAT THE ENTIRE SELF-SHIELDING CURVE CAN BE   Groupie

REPRODUCED TO WITHIN THIS FRACTIONAL ERROR.   Groupie

= .LT. 0.0001 - USE STANDARD 0.001            Groupie

(0.1 PER-CENT)                Groupie

= .GE. 0.0001 - USE AS CONVERGENCE CRITERIA   Groupie

1   56-66    I11   SIGMA-0 DEFINITION SELECTOR.                  Groupie

< 0 - 21 VALUES OF SIGMA0 ARE READ INPUT AND  Groupie

INTERPRETED AS FIXED VALUES = SAME AS   Groupie

= 1 DESCRIPTION BELOW                   Groupie

INPUT VALUES MUST ALL BE,               Groupie

1) GREATER THAN 0                       Groupie

2) IN DESCENDING VALUE ORDER            Groupie

= 0 - SIGMA-0 WILL BE DEFINED AS A MULTIPLE   Groupie

OF THE UNSHIELDED TOTAL CROSS SECTION   Groupie

IN EACH GROUP (VALUES OF 1/1024 TO      Groupie

1024 IN STEPS OF A FACTOR OF 2 WILL     Groupie

BE USED AS THE MULTIPLIER).             Groupie

= 1 - SIGMA-0 WILL BE DEFINED AS THE SAME     Groupie

NUMBER OF BARNS IN EACH GROUP (VALUES   Groupie

40000 TO 0.4 BARNS WILL BE USED. WITHIN Groupie

EACH DECADE VALUES OF 10, 7, 4, 2, 1    Groupie

BARNS WILL BE USED).                    Groupie

2-4    1-66 6D11.4   IF SIGMA-0 DEFINITION SELECTOR < 0, THE NEXT  Groupie

4 LINES OF INPUT ARE THE 22 VALUES OF SIGMA0, Groupie

6 PER LINE.                                   Groupie

2    1-60    A60   ENDF/B INPUT DATA FILENAME                    Groupie

(STANDARD OPTION = ENDFB.IN)                  Groupie

3    1-60    A60   ENDF/B OUTPUT DATA FILENAME                   Groupie

(STANDARD OPTION = ENDFB.OUT)                 Groupie

Groupie

THE FOURTH INPUT CARD IS USED TO SELECT ALL DESIRED OUTPUT MODES.  Groupie

EACH OUTPUT DEVICE MAY BE TURNED OFF (0) OR ON (1). THEREFORE      Groupie

THEREFORE EACH OF THE FOLLOWING INPUT PARAMETERS MAY BE EITHER     Groupie

ZERO TO INDICATE NO OUTPUT OR NON-ZERO TO INDICATE OUTPUT.         Groupie

Groupie

4     1-11   I11   SELF-SHIELDED CROSS SECTION LISTING           Groupie

= 1 - CROSS SECTIONS                          Groupie

= 2 - RESONANCE INTEGRALS                     Groupie

4    12-22   I11   MULTI-BAND PARAMETER LISTING                  Groupie

4    23-33   I11   MULTI-BAND PARAMETERS COMPUTER READABLE       Groupie

4    34-44   I11   UNSHIELDED CROSS SECTIONS IN ENDF/B FORMAT    Groupie

= 1 - HISTOGRAM FORMAT (INTERPOLATION LAW 1)  Groupie

= 2 - LINEAR-LINEAR (INTERPOLATION LAW 2)     Groupie

4    45-55   I11   UNSHIELDED CROSS SECTIONS LISTING             Groupie

= 1 - CROSS SECTIONS                          Groupie

= 2 - RESONANCE INTEGRALS                     Groupie

05/01/20 - ADDED THE BELOW OPTION                                      Groupie

4    56-66   D11.4 IF THE STANDARD BUILT-IN SPECTRA IS USED,     Groupie

INPUT LINE 1, COLUMNS 34-44 = 2, THIS FIELD   Groupie

CAN BE USED TO OPTIONALLY CHANGE TEMPERATURE  Groupie

OF THE MAXWELLIAN.                            Groupie

INPUT IS IN EV (0.0253 EV = ROOM TEMPERATURE) Groupie

= 0 - USE DEFAULT 0.0253 EV, ROOM TEMPERATURE Groupie

> 0 - USE THIS AS THE TEMPERATURE             Groupie

RESTRICTION - TEMPERATURE CANNOT EXCEED       Groupie

1000 EV.                                      Groupie

Groupie

5     1-80   18A4  LIBRARY IDENTIFICATION. ANY TEXT THAT THE     Groupie

USER WISHES TO IDENTIFY THE MULTI-BAND        Groupie

PARAMETERS. THIS LIBRARY IDENTIFICATION IS    Groupie

WRITTEN INTO THE COMPUTER READABLE MULTI-BAND Groupie

DATA FILE.                                    Groupie

Groupie

6-N    1- 6    I6   LOWER MAT OR ZA LIMIT                         Groupie

7- 8    I2   LOWER MF LIMIT                                Groupie

9-11    I3   LOWER MT LIMIT                                Groupie

12-17   I11   UPPER MAT OR ZA LIMIT                         Groupie

18-19    I2   UPPER MF LIMIT                                Groupie

20-22    I3   UPPER MT LIMIT                                Groupie

UP TO 100 RANGES MAY BE SPECIFIED, ONE RANGE  Groupie

PER LINE. THE LIST OF RANGES IS TERMINATED    Groupie

BY A BLANK CARD. IF THE UPPER MAT OR ZA       Groupie

LIMIT IS LESS THAN THE LOWER LIMIT THE UPPER  Groupie

IS SET EQUAL TO THE LOWER LIMIT. IF THE UPPER Groupie

MF OR MT LIMIT IS ZERO IT WILL BE SET EQUAL   Groupie

TO ITS MAXIMUM VALUE, 99 OR 999, RESPECTIVELY Groupie

IF THE FIRST REQUEST LINE IS BLANK IT WILL    Groupie

TERMINATE THE LIST OF REQUESTS AND CAUSE ALL  Groupie

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).     Groupie

Groupie

VARY   1-66  6D11.4 ENERGY GROUP BOUNDARIES. ONLY REQUIRED IF     Groupie

THE NUMBER OF GROUPS INDICATED ON THE FIRST   Groupie

INPUT CARD IS POSITIVE. ALL ENERGIES MUST     Groupie

BE IN ASCENDING ENERGY IN EV. THE PRESENT     Groupie

LIMITS ARE 1 TO 1000 GROUPS. FOR N GROUPS     Groupie

N+1 BOUNDARIES WILL BE READ FROM THE          Groupie

INPUT FILE, E.G. IF THE FIRST INPUT CARD      Groupie

INDICATES 20 GROUPS, 21 ENERGY BOUNDARIES     Groupie

WILL BE READ FROM THE INPUT FILE.             Groupie

Groupie

VARY   1-66  6D11.4 ENERGY DEPENDENT WEIGHTING SPECTRUM. ONLY     Groupie

REQUIRED IF THE NUMBER OF POINTS INDICATED    Groupie

ON FIRST CARD IS MORE THAN ONE. DATA IS       Groupie

GIVEN IN (ENERGY, WEIGHT) PAIRS, UP TO 3      Groupie

PAIRS PER CARD, USING ANY NUMBER OF CARDS     Groupie

REQUIRED. ENERGIES MUST BE IN ASCENDING       Groupie

ORDER IN EV. THE SPECTRUM VALUES MUST BE      Groupie

NON-NEGATIVE. THE ENERGY RANGE OF SPECTRUM    Groupie

MUST AT LEAST SPAN THE ENERGY RANGE OF THE    Groupie

ENERGY GROUPS. SINCE SPECTRUM IS STORED IN    Groupie

PAGING SYSTEM THERE IS NO LIMIT TO NUMBER     Groupie

OF POINTS THAT CAN BE USED TO DESCRIBE THE    Groupie

WEIGHTING SPECTRUM.                           Groupie

Groupie

EXAMPLE INPUT NO. 1                                                Groupie

-------------------                                                Groupie

REQUEST DATA BY MAT AND PROCESS ALL DATA (ALL MAT BETWEEN 1 AND    Groupie

9999). USE THE TART 175 GROUP STRUCTURE, GENERATE 2 BAND           Groupie

PARAMETERS (THE FOR ALL ISOTOPES) TO 0.1 PER-CENT ACCURACY         Groupie

IN THE SELF-SHIELDING CURVE. OUTPUT ALL  LISTING, COMPUTER         Groupie

READABLE AND ENDF/B FORMAT GROUP AVERAGES.                         Groupie

Groupie

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Groupie

Groupie

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                          Groupie

Groupie

0          0         -2          0 1.00000-03          0      Groupie

ENDFB.IN                                                               Groupie

ENDFB.OUT                                                              Groupie

1          1          1          1          1                 Groupie

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY                Groupie

1 1  1  9999 0  0                                                  Groupie

(BLANK CARD TERMINATES REQUEST LIST)             Groupie

Groupie

EXAMPLE INPUT NO. 2                                                Groupie

-------------------                                                Groupie

THE SAME EXAMPLE 1, AS ABOVE, ONLY THE ENDF/B DATA WILL BE READ    Groupie

FROM \ENDFB6\SIGMA1\K300\ZA092238 (U-238 AT 300 KELVIN) AND        Groupie

WRITTEN TO \ENDFB6\GROUPIE\K300\ZA092238                           Groupie

Groupie

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                          Groupie

Groupie

0          0         -2          0 1.00000-03          0      Groupie

\ENDFB6\SIGMA1\K300\ZA092238                                           Groupie

\ENDFB6\GROUPIE\K300\ZA092238                                          Groupie

1          1          1          1          1                 Groupie

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY                Groupie

1 1  1  9999 0  0                                                  Groupie

(BLANK CARD TERMINATES REQUEST LIST)             Groupie

Groupie

EXAMPLE INPUT NO. 3                                                Groupie

-------------------                                                Groupie

PROCESS ALL DATA. USE 1/V WEIGHTING IN ORDER TO CALCULATE          Groupie

UNSHIELDED ONE GROUP CROSS SECTIONS OVER THE ENERGY RANGE 0.5 EV   Groupie

TO 1 MEV (NOTE THAT THE RESULTS ARE SIMPLY PROPORTIONAL TO THE     Groupie

RESONANCE INTEGRAL FOR EACH REACTION). OUTPUT UNSHIELDED LISTING.  Groupie

Groupie

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Groupie

THEN USE STANDARD FILENAMES.                                       Groupie

Groupie

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Groupie

Groupie

0          0          1         -1                     0      Groupie

(USE STANDARD FILENAME = ENDFB.IN)               Groupie

(USE STANDARD FILENAME = ENDFB.OUT)              Groupie

0          0          0          0          1                 Groupie

RESONANCE INTEGRAL CALCULATION (FROM 0.5 EV TO 1 MEV)                  Groupie

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Groupie

5.00000-01 1.00000+06                                                  Groupie

Groupie

======================================================================= Groupie

======================================================================= Legend

Legend

PROGRAM LEGEND                                                     Legend

VERSION 80-1 (SEPTEMBER 1980)                                      Legend

VERSION 84-1 (NOVEMBER 1984)                                       Legend

VERSION 86-1 (JANUARY 1986) *CORRECTED BASED ON USER COMMENTS      Legend

*FORTRAN-77/H VERSION                  Legend

VERSION 87-1 (JANUARY 1987) *CORRECTED BASED ON USER COMMENTS      Legend

VERSION 88-1 (JULY 1988)    *OPTION...INTERNALLY DEFINE ALL I/O    Legend

FILE NAMES (SEE, SUBROUTINE FILEIO    Legend

FOR DETAILS).                         Legend

*IMPROVED BASED ON USER COMMENTS.      Legend

VERSION 89-1 (JANUARY 1989) *PSYCHOANALYZED BY PROGRAM FREUD TO    Legend

INSURE PROGRAM WILL NOT DO ANYTHING   Legend

CRAZY.                                Legend

*UPDATED TO USE NEW PROGRAM CONVERT    Legend

KEYWORDS.                             Legend

*ADDED LIVERMORE CIVIC COMPILER        Legend

CONVENTIONS.                          Legend

VERSION 92-1 (JANUARY 1992) *FOR ANGULAR DISTRIBUTIONS CALCULATED  Legend

FROM LEGENDRE COEFFICIENTS, INTERVAL  Legend

HALF TO CONVERGENCE.                  Legend

*UPDATED BASED ON USER COMMENTS        Legend

*ADDED FORTRAN SAVE OPTION             Legend

*ADDED SELECTED OF DATA TO PROCESS     Legend

BY MAT/MF/MT/ENERGY RANGES.           Legend

*WARNING...THE INPUT PARAMETER FORMAT  Legend

HAS BEEN CHANGED - FOR DETAILS SEE    Legend

BELOW.                                Legend

VERSION 92-2 (SEPT. 1992)   *CORRECTED PROCESSING OF ISOTROPIC     Legend

ANGULAR DISTRIBUTIONS                 Legend

VERSION 94-1 (JANUARY 1994) *VARIABLE ENDF/B DATA FILENAMES        Legend

TO ALLOW ACCESS TO FILE STRUCTURES    Legend

(WARNING - INPUT PARAMETER FORMAT     Legend

HAS BEEN CHANGED)                     Legend

*CLOSE ALL FILES BEFORE TERMINATING    Legend

(SEE, SUBROUTINE ENDIT)               Legend

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Legend

*IMPROVED COMPUTER INDEPENDENCE        Legend

*ALL DOUBLE PRECISION                  Legend

*ON SCREEN OUTPUT                      Legend

*UNIFORM TREATMENT OF ENDF/B I/O       Legend

*IMPROVED OUTPUT PRECISION             Legend

*INCREASED MAX. POINTS FROM 5,000      Legend

TO 20,000.                            Legend

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Legend

POINT READ FOR MORE DIGITS            Legend

*UPDATED TEST FOR ENDF/B FORMAT        Legend

VERSION BASED ON RECENT FORMAT CHANGE Legend

*GENERAL IMPROVEMENTS BASED ON         Legend

USER FEEDBACK                         Legend

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Legend

USER FEEDBACK                         Legend

VERS. 2001-1 (MARCH 2001)   *UPDATED TO HANDLE COMBINATIONS OF     Legend

LEGENDRE COEFFICIENTS AT LOW ENERGY   Legend

AND TABULATED DATA AT HIGH ENERGY.    Legend

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Legend

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Legend

*ZERO ANGULAR DISTRIBUTIONS ARE O.K.   Legend

(PREVIOUSLY ZERO OR NEGATIVE WAS      Legend

TREATED AS AN ERROR - ZERO IS O.K.   Legend

FOR SOME REACTIONS OVER SOME COSINE  Legend

RANGES)                              Legend

VERS. 2006-1 (MARCH 2006)   *INCREASED MAXIMUM NUMBER OF LEGENDRE  Legend

COEFFICIENTS FROM 50 TO 500.          Legend

WARNING - THE RECURSION RELATIONSHIP  Legend

FOR LEGENDRE POLYNOMIALS BECOMES      Legend

UNSTABLE IN HIGHER ORDER POLYTNOMIALS Legend

EVEN USING DOUBLE PRECISION.          Legend

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B=VII.       Legend

*INCREASED MAX. POINTS FROM 60,000     Legend

TO 240,000.                           Legend

Legend

OWNED, MAINTAINED AND DISTRIBUTED BY                               Legend

------------------------------------                               Legend

THE NUCLEAR DATA SECTION                                           Legend

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Legend

P.O. BOX 100                                                       Legend

A-1400, VIENNA, AUSTRIA                                            Legend

EUROPE                                                             Legend

Legend

ORIGINALLY WRITTEN BY                                              Legend

------------------------------------                               Legend

DERMOTT E. CULLEN                                                  Legend

CURRENT ADDRESS                                                    Legend

UNIVERSITY OF CALIFORNIA                                           Legend

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Legend

L-159                                                              Legend

P.O. BOX 808                                                       Legend

LIVERMORE, CA 94550                                                Legend

U.S.A.                                                             Legend

TELEPHONE  925-423-7359                                            Legend

E. MAIL    CULLEN1@LLNL.GOV                                        Legend

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Legend

Legend

PURPOSE                                                            Legend

-------                                                            Legend

CALCULATE LINEARLY INTERPOLABLE TABULATED ANGULAR DISTRIBUTIONS    Legend

STARTING FROM DATA IN THE ENDF/B FORMAT. ANGULAR DISTRIBUTIONS     Legend

MAY BE DESCRIBED IN THE ENDF/B FORMAT IN ONE OF THREE WAYS.        Legend

FOR EACH OF THESE THREE FORMS THE USER MAY CHOOSE (SEE, INPUT      Legend

OPTIONS) TO EITHER COPY EACH TYPE OF DATA OR TO PROCESS IT AT      Legend

AS FOLLOWS,                                                        Legend

Legend

(1) ANGULAR DISTRIBUTION IS ISOTROPIC AT ALL ENERGIES (LTT=0)      Legend

-------------------------------------------------------------      Legend

IN THIS CASE THE INPUT DATA DOES NOT INCLUDE ANY ANGULAR           Legend

DISTRIBUTIONS. A SECTION MERELY CONTAINS A FLAG TO INDICATE        Legend

THE ANGULAR DISTRIBUTION IS ISOTROPIC AT ALL ENERGIES. IN THIS     Legend

CASE THE SECTION IS OUTPUT IN EXACTLY THE SAME FORM IN WHICH IT    Legend

WAS READ FROM THE INPUT.                                           Legend

Legend

(2) ANGULAR DISTRIBUTIONS GIVEN BY LEGENDRE COEFFICIENTS (LTT=1)   Legend

----------------------------------------------------------------   Legend

LEGENDRE COEFFICIENTS ARE GIVEN AT A SERIES OF ENERGIES. AN        Legend

INTERPOLATION LAW IS GIVEN BETWEEN ENERGIES. THE INTERPOLATION     Legend

LAW BETWEEN ENERGIES IS COPIED AS INPUT (I.E., NO ATTEMPT IS       Legend

MADE TO LINEARIZE THE VARIATION WITH ENERGY). FOR EACH ENERGY AT   Legend

WHICH LEGENDRE COEFFICIENTS ARE GIVEN A LINEARLY INTERPOLABLE      Legend

ANGULAR DISITRIBUTION IS RECONSTRUCTED IN THE SYSTEM IN WHICH THE  Legend

THE COEFFICIENTS ARE GIVEN (I.E., CM OR LAB - NO ATTEMPT IS MADE   Legend

TO CONVERT FROM ONE SYSTEM TO THE OTHER). A MAXIMUM OF 50 LEGENDRE Legend

COEFFICIENTS IS ALLOWED. REGARDLESS OF THE NUMBER OF COEFFICIENTS  Legend

INPUT THE PROGRAM WILL ONLY USE COEFFICIENTS UP TO THE LAST ORDER  Legend

AT WHICH THE COEFFICIENTS ARE NON-ZERO (E.G. IF COEFFICIENTS P1    Legend

THROUGH P12 ARE READ, BUT P9=P10=P11=P12=0.0, THE PROGRAM WILL     Legend

ONLY USE COEFFICIENTS UP TO P8). IF OVER 50 NON-ZERO COEFFICIENTS  Legend

ARE READ ONLY THE FIRST 50 WILL BE USED.                           Legend

Legend

(2) ANGULAR DISTRIBUTIONS IS TABULATED (LTT=2)                     Legend

----------------------------------------------------------------   Legend

ANGULAR DISTRIBUTIONS ARE GIVEN AT A SERIES OF ENERGIES. AN        Legend

INTERPOLATION LAW IS GIVEN BETWEEN ENERGIES AND A SECOND           Legend

INTERPOLATION LAW IS GIVEN AT EACH ENERGY TO INTERPOLATE BETWEEN   Legend

THE POINTS IN EACH TABULATED DISTRIBUTION. AT EACH ENERGY THE      Legend

ANGULAR DISTRIBUTION WILL BE CONVERTED TO LINEARLY INTERPOLABLE    Legend

FORM. THE INTERPOLATION BETWEEN ENERGIES IS OUTPUT EXACTLY AS      Legend

INPUT. THE INTERPOLATION LAW AT EACH ENERGY IS OUTPUT TO INDICATE  Legend

THE NOW LINEARLY INTERPOLABLE ANGULAR DISTRIBUTION.                Legend

Legend

(3) LEGENDRE COEFFICIENTS AND TABULATED (LTT=3)                    Legend

----------------------------------------------------------------   Legend

ENDF-102 SAYS THIS SHOULD BE LTT=4, BUT ALL OF THE EVALUATIONS     Legend

IN ENDF/B-VI, RELEASE 7, USE LTT=3? THIS CODE WILL TREAT THESE     Legend

AS LTT=4 - SEE BELOW.                                              Legend

Legend

(4) LEGENDRE COEFFICIENTS AND TABULATED (LTT=4)                    Legend

----------------------------------------------------------------   Legend

THIS IS A COMBINATION OF (1) AND (2) DESCRIBED ABOVE. THE          Legend

LEGENDRE DATA IS ALWAYS GIVEN FIRST, FOR LOWER ENERGIES,           Legend

FOLLOWED BY TABULATED ANGULAR DISTRIBUTIONS, FOR HIGHER ENERGIES.  Legend

Legend

THIS TYPE OF DATA CAN ONLY BE COPIED OR ALL CONVERTED TO           Legend

TABULATED (LTT=2).                                                 Legend

Legend

POINT VALUES - NORMALIZED VS. UNNORMALIZED                         Legend

------------------------------------------------------------------ Legend

THE VALUE OF AN ANGULAR DISTRIBUTION AT ANY COSINE WILL BE         Legend

CORRECTLY CALCULATED BY THIS CODE, BASED EITHER DIRECTLY ON THE    Legend

ANGULAR DISTRIBUTION, OR ON THE SUM OF THE CONTRIBUTING LEGENDRE   Legend

MOMENTS.                                                           Legend

Legend

ENDF/B ANGULAR DISTRIBUTIONS ARE BY DEFINITION NORMALIZED WHEN     Legend

INTEGRATED OVER COSINE. THEREFORE THIS CODE WILL NORMALIZE EACH    Legend

ANGULAR DISTRIBUTION BEFORE IT IS OUTPUT. THE OUTPUT REPORT FROM   Legend

THIS CODE WILL INDICATE THE NORMALIZATION FACTOR USED.             Legend

Legend

THE REASON THAT AN ANGULAR DISTRIBUTION MAY NOT BE NORMALIZED IS   Legend

DUE TO THE APPROXIMATION OF CREATING LINEARLY INTERPOLABLE         Legend

TABULATED ANGULAR DISTRIBUTIONS - THE MORE ACCURATELY THIS IS      Legend

DONE THE CLOSER THE NORMALIZATION FACTOR WILL BE TO UNITY. AS YOU  Legend

DECREASE THE ALLOWABLE ERROR THE NORMALIZED VALUES WILL APPROACH   Legend

THE CORRECT POINT VALUES CALCULATED BY THE CODE.                   Legend

Legend

SINCE THE DATA IS NORMALIZED PRIOR TO OUTPUT THE RESULTS IN THE    Legend

ENDF/B FORMAT MAY DIFFER SLIGHTLY FROM VALUES REFERRED TO BE ERROR Legend

MESSAGES, ETC. PRINTED BY THE CODE DURING EXECUTION. IN ALL CASES  Legend

THE VALUES PRINTED BY THE CODE IN ERROR MESSAGES, ETC. SHOULD BE   Legend

CONSIDERED TO BE THE CORRECT VALUES AND THE OUTPUT TABULATED       Legend

ANGULAR DISTRIBUTIONS APPROXIMATE DUE TO THE RE-NORMALIZATION -    Legend

TO RE-ITERATE, THE OUTPUT TABULATED VALUES ARE APPROXIMATE DUE     Legend

TO THE APPROXIMATIONS USED IN CONSTRUCTING LINEAR INTERPOLABLE     Legend

ANGULAR DISTRIBUTIONS TO WITHIN SOME ALLOWABLE TOLERANCE.          Legend

Legend

ELIMINATION OF NEGATIVE VALUES                                     Legend

------------------------------                                     Legend

THE RECONSTRUCTED ANGULAR DISTRIBUTION WILL BE TESTED AND IF IT    Legend

IS NEGATIVE AT ONE OR MORE COSINES AN ERROR MESSAGE WILL BE OUTPUT Legend

AND BASED ON THE INPUT OPTION SELECTED ONE OF THE FOLLOWING        Legend

CORRECTIVE ACTIONS WILL BE TAKEN (SEE, INPUT OPTIONS),             Legend

(1) NO CORRECTION                                                  Legend

(2) CHANGE INDIVIDUAL LEGENDRE COEFFICIENTS (EACH BY LESS THAN     Legend

1.0 PER-CENT) UNTIL THE RECONSTRUCTED ANGULAR DISTRIBUTION     Legend

IS POSITIVE (MINIMUM MORE THAN 1 MILLI-BARN). THE ALLOWABLE    Legend

PER-CENT CHANGE IN COEFFICIENTS AND MINIMUM CROSS SECTION CAN  Legend

BE CHANGED BY INPUT.                                           Legend

(3) CHANGE ALL LEGENDRE COEFFICIENTS TO FORCE DISTRIBUTION TO BE   Legend

POSITIVE (MINIMUM MORE THAN 1 MILLI-BARN). WITH THIS OPTION    Legend

THERE IS NO RESTRICTION ON THE AMOUNT THAT EACH COEFFICIENT    Legend

IS CHANGED AND AS SUCH THIS OPTION SHOULD BE USED WITH         Legend

CAUTION AND ONLY AS A LAST RESORT IF NO OTHER APPROACH CAN     Legend

BE USED TO MAKE THE DISTRIBUTION POSITIVE.                     Legend

Legend

OUTPUT                                                             Legend

------                                                             Legend

THE USER MAY REQUEST OUTPUT OF EITHER,                             Legend

(1) TABULATED VALUES - POSSIBLY CORRECTED TO ELIMINATE NEGATIVE    Legend

VALUES. THE TABULATED DISTRIBUTION WILL BE NORMALIZED BEFORE   Legend

OUTPUT.                                                        Legend

(2) LEGENDRE COEFFICIENTS - POSSIBLY CORRECTED TO ELIMINATE        Legend

NEGATIVE VALUES AND WITHOUT HIGHER ORDER ZERO COEFFICIENTS.    Legend

BY DEFINITION DISTRIBUTIONS DEFINED BY LEGENDRE COEFFICIENTS   Legend

ARE NORMALIZED TO UNITY.                                       Legend

Legend

(3) ANGULAR DISTRIBUTIONS GIVEN BY A TABULATION (LTT=2)            Legend

-------------------------------------------------------            Legend

TABULATED ANGULAR DISTRIBUTIONS ARE GIVEN AT A SERIES OF ENERGIES. Legend

AN INTERPOLATION LAW IS GIVEN BETWEEN ENERGIES. THE INTERPOLATION  Legend

LAW BETWEEN ENERGIES IS COPIED AS INPUT (I.E., NO ATTEMPT IS       Legend

MADE TO LINEARIZE THE VARIATION WITH ENERGY). FOR EACH ENERGY AT   Legend

AT WHICH TABULATED DATA ARE GIVEN A LINEARLY INTERPOLABLE ANGULAR  Legend

DISTRIBUTION IS CONSTRUCTED IN THE SYSTEM IN WHICH THE TABULATED   Legend

DATA ARE GIVEN (I.E., CM OR LAB - NO ATTEMPT IS MADE TO CONVERT    Legend

FROM ONE SYSTEM TO THE OTHER). A MAXIMUM OF 60000 POINTS IS ALLOWE Legend

TO REPRESENT THE ANGULAR DISTRIBUTION AT EACH ENERGY.              Legend

Legend

ELIMINATION OF NEGATIVE VALUES                                     Legend

------------------------------                                     Legend

THE RECONSTRUCTED ANGULAR DISTRIBUTION WILL BE TESTED AND IF IT    Legend

IS NEGATIVE AT ONE OR MORE COSINES AN ERROR MESSAGE WILL BE OUTPUT Legend

AND BASED ON THE INPUT OPTION SELECTED ONE OF THE FOLLOWING        Legend

CORRECTIVE ACTIONS WILL BE TAKEN (SEE, INPUT OPTIONS),             Legend

(1) NO CORRECTION                                                  Legend

(2) CHANGE ALL TABULATED VALUES TO FORCE DISTRIBUTION TO BE        Legend

POSITIVE (MINIMUM MORE THAN 1 MILLI-BARN). THE MINIMUM VALUE   Legend

MAY BE CHANGED BY INPUT. WITH THIS OPTION THERE IS NO          Legend

RESTRICTION ON THE AMOUNT THAT EACH VALUE IS CHANGED AND AS    Legend

SUCH THIS OPTION SHOULD BE USED WITH CAUTION AND ONLY AS A     Legend

LAST RESORT IF NO OTHER APPROACH CAN BE USED TO MAKE THE       Legend

DISTRIBUTION POSITIVE.                                         Legend

Legend

OUTPUT                                                             Legend

------                                                             Legend

THE OUTPUT WILL BE THE LINEARIZED ANGULAR DISTRIBUTION. THE        Legend

TABULATED DISTRIBUTION WILL BE NORMALIZED TO UNITY BEFORE OUTPUT.  Legend

Legend

CORRECTING NEGATIVE ANGULAR DISTRIBUTION                           Legend

----------------------------------------                           Legend

IF AN ANGULAR DISTRIBUTION IS NEGATIVE AN ERROR MESSAGE WILL BE    Legend

PRINTED AND THE USER MAY DECIDE (BASED ON INPUT OPTION) TO,        Legend

(1) NOT PERFORM ANY CORRECTIVE ACTION.                             Legend

(2) FOR TABULATED DISTRIBUTIONS - ADD THE SAME VALUE TO EACH POINT Legend

VALUE SUCH THAT WHEN THE DISTRIBUTION IS RE-NORMALIZED THE     Legend

MINIMUM VALUE IS 0.001 (1 MILLI-BARN). THE MINIMUM VALUE CAN   Legend

BE CHANGED BY INPUT. WARNING...EXCEPT FOR SELECTION OF THE     Legend

MINIMUM VALUE (BY INPUT) THE USER HAS NO CONTROL OVER HOW      Legend

MUCH THE DISTRIBUTION IS CHANGED. THEREFORE THIS OPTION SHOULD Legend

BE USED WITH CAUTION.                                          Legend

(3) FOR LEGENDRE COEFFICIENTS ONE OF TWO OPTIONS MAY BE SELECTED,  Legend

(A) CHANGE INDIVIDUAL COEFFICIENTS (NO ONE COEFFICIENT BY MORE     Legend

THAN 1 PER-CENT) TO MAKE THE DISTRIBUTION POSITIVE WITH A      Legend

MINIMUM VALUE OF 0.001 (1 MILLI-BARN). THE MAXIMUM PER-CENT    Legend

CHANGE IN EACH COEFFICIENT AND MINIMUM VALUE MAY BE CHANGED    Legend

BY INPUT. INPUT THE PROGRAM CANNOT MAKE THE DISTRIBUTION       Legend

POSITIVE BY CHANGING EACH COEFFICIENT BY UP TO THE MAXIMUM     Legend

ALLOWABLE AMOUNT, THE ORIGINAL ANGULAR DISTRIBUTION OR         Legend

COEFFICIENTS WILL BE OUTPUT. ONLY IN THE LATTER CASE SHOULD    Legend

ONE CONSIDER USING OPTION (B) DESCRIBED BELOW.                 Legend

(B) LOGICALLY ADD THE SAME VALUE TO EACH POINT VALUE SUCH THAT     Legend

WHEN THE DISTRIBUTION IS RE-NORMALIZED THE MINIMUM VALUE IS    Legend

0.001 (1 MILLI-BARN). THIS IS EQUIVALENT AT INCREASING P0      Legend

BY A CERTAIN AMOUNT AND RE-NORMALIZATION IS EQUIVALENT TO THEN Legend

DIVIDING EACH COEFFICIENT BY A CERTAIN AMOUNT. THEREFORE,      Legend

WHAT IS PHYSICALLY DONE BY THE PROGRAM IS TO DIVIDE EACH       Legend

COEFFICIENT BY THE SAME AMOUNT. WARNING..EXCEPT FOR SELECTION  Legend

OF THE MINIMUM VALUE (BY INPUT) THE USER HAS NO CONTROL OVER   Legend

HOW MUCH THE DISTRIBUTION IS CHANGED. THEREFORE THIS OPTION    Legend

SHOULD BE USED WITH CAUTION.                                   Legend

Legend

WARNING MESSAGES FROM PROGRAM                                      Legend

-----------------------------                                      Legend

THE WARNING MESSAGES PRINTED BY THIS PROGRAM SHOULD ONLY BE        Legend

CONSIDERED TO BE EXACTLY THAT..WARNINGS..NOT AN ABSOLUTE JUDGEMENT Legend

BY THIS PROGRAM THAT THERE IS SOMETHING WRONG WITH THE DATA. WHEN  Legend

WARNING MESSAGES ARE PRINTED EXAMINE THE DATA AND EITHER TAKE NO   Legend

ACTION (IF YOU FEEL THAT THE DATA IS O.K.) OR CORRECT THE DATA     Legend

(IF YOU FEEL THAT THE DATA IS INCORRECT AND YOU CAN CORRECT IT).   Legend

Legend

VALIDITY OF MODIFIED DATA                                          Legend

-------------------------                                          Legend

BEFORE BELIEVING AND USING DATA WHICH HAS BEEN MODIFIED (EITHER    Legend

TABULATED ANGULAR DISTRIBUTIONS OR LEGENDRE COEFFICIENTS) THE USER Legend

SHOULD INSURE THAT THE MODIFIED DATA IS PHYSICALLY MORE ACCEPTABLE Legend

THAN THE ORIGINAL DATA. IN ORDER TO DO THIS ONE OR MORE OF THE     Legend

FOLLOWING METHODS SHOULD BE USED,                                  Legend

Legend

(1) USE THE ENERGY VARIATION TESTS BUILT-IN TO THIS PROGRAM AND    Legend

EVALPLOT TO PLOT THE ENERGY DEPENDENCE OF THE LEGENDRE         Legend

COEFFICIENTS IN ORDER TO IDENTIFY AND CORRECT (BY HAND...NOT   Legend

BY THIS PROGRAM) ANY COEFFICIENTS WHICH HAVE UNREALISTIC       Legend

ENERGY AND L ORDER VARIATIONS. THIS SHOULD ALWAYS BE DONE      Legend

FIRST TO ELIMINATE MAJOR PROBLEMS BEFORE USING THIS PROGRAM    Legend

TO AUTOMATICALLY MAKE MINOR CORRECTIONS.                       Legend

(1) OUTPUT AND PLOT THE UNCORRECTED AND CORRECTED ANGULAR          Legend

DISTRIBUTIONS. COMPARE THE PLOTS TO INSURE THAT THE CORRECTED  Legend

DATA DOES NOT SERIOUSLY CHANGE THE ENERGY DEPENDENCE OF THE    Legend

ANGULAR DISTRIBUTION.                                          Legend

(2) IF PLOTTING CAPABILITY IS NOT AVAIALABLE, USE THE PRINTED OUT  Legend

OF THIS PROGRAM TO DETERMINE HOW MUCH THE TABULATED ANGULAR    Legend

DISTRIBUTION OR LEGENDRE COEFFICIENTS HAVE BEEN MODIFIED.      Legend

GENERALLY IF ONE COEFFICIENT HAS BEEN ONLY SLIGHTLY MODIFIED   Legend

THE DISTRIBUTION WILL BE ACCEPTABLE. HOWEVER IF MANY           Legend

COEFFICIENTS HAVE BEEN MODIFIED THE RESULT WILL NOT BE         Legend

RELIABLE.                                                      Legend

Legend

SEEING ANGULAR DISTRIBUTIONS AND LEGENDRE COEFFICIENTS             Legend

------------------------------------------------------             Legend

PROGRAM EVALPLOT CAN BE USED TO PLOT ANGULAR DISTRIBUTION AND      Legend

LEGENDRE COEFFICIENTS - WHEN IT COMES TO CHECKING THIS TYPE OF     Legend

DATA THERE IS NO SUBSTITUTE FOR PLOTS OF THE DATA TO MAKE THE      Legend

JOB EASY AND STRAIGHTFORWARD.                                      Legend

Legend

FOR LEGENDRE COEFFICIENTS EVALPLOT CAN BE USED TO SEE THE ENERGY   Legend

DEPENDENCE OF EACH COEFFICIENT - THIS IS AN EXTREMELY EASY AND     Legend

USEFUL WAY TO CHECK FOR ERRORS IN THE BASIC DATA.                  Legend

Legend

FOR ANGULAR DISTRIBUTION EVALPLOT CAN BE USED TO PLOT THEM AT      Legend

EACH ENERGY THAT THEY ARE TABULATED - THIS IS ALSO AN EASY AND     Legend

USEFUL WAY TO CHECK FOR ERRORS.                                    Legend

Legend

I/O UNIT DEFINITIONS                                               Legend

--------------------                                               Legend

UNIT  DESCRIPTION                                                  Legend

----  -----------                                                  Legend

2   INPUT CARDS                                                  Legend

3   OUTPUT REPORT                                                Legend

10   ORIGINAL DATA IN ENDF/B FORMAT                               Legend

11   FINAL DATA IN ENDF/B FORMAT                                  Legend

Legend

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Legend

---------------------------------------------------------------    Legend

UNIT  FILE NAME                                                    Legend

----  ----------                                                   Legend

2   LEGEND.INP                                                   Legend

3   LEGEND.LST                                                   Legend

10   ENDFB.IN                                                     Legend

11   ENDFB.OUT                                                    Legend

Legend

INPUT CARD                                                         Legend

----------                                                         Legend

CARD COLS.  FORMAT  DESCRIPTION                                        Legend

---- -----  ------ -----------                                         Legend

1    1-11   E11.4 FRACTIONAL THINNING CRITERIA                        Legend

12-22   I11   MAXIMUM NUMBER OF POINTS IN ANGULAR DISTRIBUTION    Legend

RECONSTRUCTED FROM LEGENDRE COEFFICIENTS (PRESENT   Legend

LIMITS ARE 11 TO 60000 POINTS)                      Legend

*THIS OPTION CAN BE USED TO RUN QUICK, BUT NOT       Legend

NECESSARILY SO ACCURATE CALCULATIONS - TO ROUGHLY   Legend

SEE WHAT THE ANGULAR DISTRIBUTIONS LOOK LIKE.       Legend

*IT IS RECOMMENDED THAT YOU USE 0 AS INPUT - IN      Legend

WHICH CASE THE PROGRAM WILL USE THE MAXIMUM         Legend

ALLOWABLE NUMBER OF POINTS = 60000.                 Legend

23-33   I11   TABULATED ANGULAR DISTRIBUTION TREATMENT            Legend

= 0 - COPY TABLES                                   Legend

= 1 - LINEARIZE TABLES (OUTPUT TABLES)              Legend

= 2 - LINEARIZE AND THIN TABLES (OUTPUT TABLES)     Legend

34-44   I11   LEGENDRE COEFFICIENT TREATMENT                      Legend

= 0 - COPY LEGENDRE COEFFICIENTS                    Legend

= 1 - RECONSTRUCT TABULATED ANGULAR DISTRIBUTION.   Legend

(OUTPUT TABLES).                              Legend

= 2 - RECONSTRUCT TABULATED ANGULAR DISTRIBUTION.   Legend

(OUTPUT LEGENDRE COEFFICIENTS).               Legend

45-55   I11   NEGATIVE ANGULAR DISTRIBUTION TREATMENT.            Legend

= 0 - NO CORRECTION                                 Legend

= 1 - TABULATE DATA - NO CORRECTION.                Legend

- LEGENDRE DATA - CHANGE COEFFICIENTS           Legend

(NONE BY MORE THAN 1.0 PER-CENT - CAN BE      Legend

CHANGED BY INPUT).                            Legend

= 2 - FORCE DISTRIBUTIONS TO BE POSITIVE            Legend

(TABULATED OR LEGENDRE DATA).                 Legend

56-66   I11   LEGENDRE COEFFICIENT VARIATION TEST FLAG.           Legend

= 0 - TEST TESTS.                                   Legend

= 1 - PERFORM TESTS,                                Legend

(A) LEGENDRE ORDER INCREASES WITH ENERGY.     Legend

(C) MONOTONIC VARIATION OF COEFFICIENTS       Legend

AS A FUNCTION OF ENERGY.                  Legend

(C) COEFFICIENTS DECREASE AS A FUNCTION OF    Legend

LEGENDRE ORDER.                           Legend

2   1-60   60A1  ENDF/B INPUT DATA FILENAME                          Legend

(STANDARD OPTION = ENDFB.IN)                        Legend

3   1-60   60A1  ENDF/B OUTPUT DATA FILENAME                         Legend

(STANDARD OPTION = ENDFB.OUT)                       Legend

4-N   1- 6    I6   LOWER MAT LIMIT                                     Legend

7- 8    I2   LOWER MF LIMIT                                      Legend

9-11    I3   LOWER MT LIMIT                                      Legend

12-17    I6   UPPER MAT LIMIT                                     Legend

18-19    I2   UPPER MF LIMIT                                      Legend

20-22    I3   UPPER MT LIMIT                                      Legend

23-33  E11.4  LOWER ENERGY LIMIT                                  Legend

34-44  E11.4  UPPER ENERGY LIMIT                                  Legend

45-55  E11.4  MINIMUM ALLOWABLE VALUE OF ANGULAR DISTRIBUTION     Legend

56-66  E11.4  ALLOWABLE FRACTION (NOT PER-CENT) CHANGE IN ANY     Legend

ONE LEGENDRE COEFFICIENT TO MAKE THE ANGULAR        Legend

DISTRIBUTION POSITIVE (AND AT LEAST EQUAL TO THE    Legend

INPUT MINIMUM ALLOWABLE VALUE).                     Legend

Legend

*UP TO 100 MAT/MT/E RANGES MAY BE INPUT, EACH SPECIFYING AN        Legend

ALLOWABLE MINIMUM SIGMA AND MAXIMUM CHANGE IN COEFFICIENTS.       Legend

*INPUT IS TERMINATED BY A BLANK CARD.                              Legend

*ALL MAY/MT/E RANGES NOT SPECIFIED BY INPUT WILL BE TREATED BY     Legend

ALLOWING A MINIMUM SIGMA OF 0.001 (1 MILLI-BARN) AND A CHANGE     Legend

IN EACH COEFFICIENT BY UP TO 0.01 (1 PER-CENT).                   Legend

*THESE MAT/MT/E RANGES ARE NOT USED TO CORRECT ALL ANGULAR         Legend

DISTRIBUTIONS WHERE SIGMA IS LESS THAN THE MINIMUM. THEY ARE      Legend

ONLY USED TO CORRECT DISTRIBUTION THAT ARE NEGATIVE AND TO        Legend

INSURE THAT THE CROSS SECTION AT THE COSINES WHERE THE ANGULAR    Legend

DISTRIBUTION ARE INITIALLY NEGATIVE ARE CORRECTED TO BE POSITIVE  Legend

AND AT LEAST AS LARGE AS THE MINIMUM ALLOWABLE SIGMA (SPECIFIED   Legend

BY INPUT).                                                        Legend

Legend

EXAMPLE INPUT NO. 1                                                Legend

-------------------                                                Legend

PROCESS BOTH LEGENDRE COEFFICIENTS AND TABULATED DATA TO OBTAIN    Legend

ANGULAR DISTRIBUTION WHICH ARE ACCURATE TO WITHIN 0.1 PER-CENT     Legend

AND OUTPUT UNCORRECTED TABULATED ANGULAR DISTRIBUTION USING        Legend

A MAXIMUM OF 501 POINTS IN EACH TABULATED ANGULAR DISTRIBUTION.    Legend

SINCE LEGENDRE COEFFICIENTS WILL NOT BE CORRECTED THE INPUT NEED   Legend

NOT SPECIFY MAT/MT/E RANGES.                                       Legend

Legend

READ /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT          Legend

Legend

THE FOLLOWING 4 INPUT LINES ARE REQUIRED,                          Legend

Legend

1.00000- 3        501          2          1          0                 Legend

/ENDFB6/K300/LEAD.IN                                                   Legend

/ENDFB6/K300/LEAD.OUT                                                  Legend

(BLANK CARD TERMINATED INPUT)                                      Legend

Legend

EXAMPLE INPUT NO. 2                                                Legend

-------------------                                                Legend

PROCESS BOTH LEGENDRE COEFFICIENTS AND TABULATED DATA TO OBTAIN    Legend

ANGULAR DISTRIBUTION WHICH ARE ACCURATE TO WITHIN 0.1 PER-CENT     Legend

AND OUTPUT CORRECTED TABULATED ANGULAR DISTRIBUTION (ONLY THOSE    Legend

RE-CONSTRUCTED FROM LEGENDRE COEFFICIENTS WILL BE CORRECTED).      Legend

FOR ALL MAT/MT/E CORRECT NEGATIVE ANGULAR DISTRIBUTION TO A VALUE  Legend

OF 0.01 (10 MILLI-BARNS) AND ALLOW LEGENDRE COEFFICIENTS TO BE     Legend

CHANGED BY UP TO 0.02 (2 PER-CENT).                                Legend

Legend

USE THE DEFAULT FILENAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE      Legend

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).           Legend

Legend

THE FOLLOWING 5 INPUT LINES ARE REQUIRED,                          Legend

Legend

1.00000- 3        501          2          1          1                 Legend

Legend

Legend

1 1  1  999999999 0.00000+ 0 3.00000+ 7 1.00000- 2 2.00000- 2      Legend

(BLANK CARD TERMINATED INPUT)                                      Legend

Legend

EXAMPLE INPUT NO. 3                                                Legend

-------------------                                                Legend

PROCESS BOTH LEGENDRE COEFFICIENTS AND TABULATED DATA TO OBTAIN    Legend

ANGULAR DISTRIBUTION WHICH ARE ACCURATE TO WITHIN 0.1 PER-CENT     Legend

AND OUTPUT CORRECTED LEGENDRE COEFFICIENTS AND UNCORRECTED         Legend

TABULATED ANGULAR DISTRIBUTIONS. FOR MAT=1800, MT=2 CORRECT        Legend

NEGATIVE ANGULAR DISTRIBUTIONS TO INSURE THE MINIMUM IS 0.01       Legend

(10 MILLI-BARNS) ALLOWING EACH LEGENDRE COEFFICIENT TO CHANGE BY   Legend

UP TO 0.02 (2 PER-CENT). ALL OTHER MAT/MT/E WILL BE CORRECTED      Legend

TO A MINIMUM OF 0.001 (1 MILLI-BARN) ALLOWING A 0.01 (1 PER-CENT)  Legend

CHANGE (BUILT-IN OPTION).                                          Legend

Legend

READ /ENDFB6/K300/LEAD.IN AND WRITE /ENDFB6/K300/LEAD.OUT          Legend

Legend

THE FOLLOWING 5 INPUT LINES ARE REQUIRED,                          Legend

Legend

1.00000- 3        501          2          2          1                 Legend

/ENDFB6/K300/LEAD.IN                                                   Legend

/ENDFB6/K300/LEAD.OUT                                                  Legend

1800 4  2  1800 4  2 0.00000+ 0 3.00000+ 7 1.00000- 2 2.00000- 2      Legend

(BLANK CARD TERMINATED INPUT)                                      Legend

Legend

EXAMPLE INPUT NO. 4                                                Legend

-------------------                                                Legend

TO COPY TABULATED ANGULAR DISTRIBUTION AND CONVERT LEGENDRE        Legend

COEFFICIENTS TO UNCORRECTED TABULAR DISTRIBUTIONS.                 Legend

Legend

USE THE DEFAULT FILENAMES ENDFB.IN AND ENDFB.OUT (THIS CAN BE      Legend

DONE BY LEAVING THE SECOND AND THIRD INPUT LINES BLANK).           Legend

Legend

THE FOLLOWING 4 INPUT LINES ARE REQUIRED,                          Legend

Legend

1.00000- 3        501          0          1          0                 Legend

Legend

Legend

(BLANK CARD TERMINATED INPUT)                                      Legend

Legend

======================================================================= Legend

======================================================================= Linear

Linear

PROGRAM LINEAR                                                     Linear

VERSION 74-1 (MAY 1974)                                            Linear

VERSION 75-1 (APRIL 1975)                                          Linear

VERSION 76-2 (OCTOBER 1976)                                        Linear

VERSION 77-1 (JANUARY 1977)                                        Linear

VERSION 78-1 (JULY 1978)                                           Linear

VERSION 79-1 (JULY 1979) CDC-7600 AND CRAY-1 VERSION.              Linear

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION.                 Linear

VERSION 80-2 (DECEMBER 1980)                                       Linear

VERSION 81-1 (MARCH 1981)                                          Linear

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY.       Linear

VERSION 83-1 (JANUARY 1983) *MAJOR RE-DESIGN.                      Linear

*PAGE SIZE INCREASED - 1002 TO 3006.   Linear

*ELIMINATED COMPUTER DEPENDENT CODING. Linear

*NEW, MORE COMPATIBLE I/O UNIT NUMBER. Linear

*ADDED OPTION TO KEEP ALL ORIGINAL     Linear

ENERGY POINTS FROM EVALUATION.        Linear

*ADDED STANDARD ALLOWABLE ERROR OPTION Linear

(CURRENTLY 0.1 PER-CENT).             Linear

VERSION 83-2 (OCTOBER 1983) IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 84-1 (APRIL 1984)   IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 84-2 (JUNE 1984)   *UPDATED FOR ENDF/B-VI FORMATS.         Linear

*SPECIAL I/O ROUTINES TO GUARANTEE      Linear

ACCURACY OF ENERGY.                    Linear

*DOUBLE PRECISION TREATMENT OF ENERGY   Linear

(REQUIRED FOR NARROW RESONANCES).      Linear

VERSION 85-1 (AUGUST 1985) *FORTRAN-77/H VERSION                   Linear

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMAT                       Linear

VERSION 87-1 (JANUARY 1987)*DOUBLE PRECISION TREATMENT OF CROSS    Linear

SECTION                                Linear

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Linear

FILE NAMES (SEE, SUBROUTINE FILEIO     Linear

FOR DETAILS).                          Linear

*IMPROVED BASED ON USER COMMENTS.       Linear

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Linear

INSURE PROGRAM WILL NOT DO ANYTHING    Linear

CRAZY.                                 Linear

*UPDATED TO USE NEW PROGRAM CONVERT     Linear

KEYWORDS.                              Linear

*ADDED LIVERMORE CIVIC COMPILER         Linear

CONVENTIONS.                           Linear

VERSION 90-1 (JUNE 1990)   *EXTENDED TO LINEARIZE PHOTON           Linear

INTERACTION DATA, MF=23 AND 27         Linear

*ADDED FORTRAN SAVE OPTION              Linear

*UPDATED BASED ON USER COMMENTS.        Linear

*NEW MORE CONSISTENT ENERGY OUTPUT      Linear

ROUTINE.                               Linear

*WARNING...INPUT PARAMETER FORMAT       Linear

HAS BEEN CHANGED...SEE DESCRIPTION     Linear

BELOW.                                 Linear

VERSION 91-1 (JULY 1991)   *ADDED INTERPOLATION LAW 6 - ONLY USED  Linear

FOR CHARGED PARTICLE CROSS SECTIONS    Linear

FOR COULOMB PENETRABILITIES.           Linear

VERSION 92-1 (JANUARY 1992)*ADDED NU-BAR (TOTAL, DELAYED, PROMPT)  Linear

POLYNOMIAL OR TABULATED ALL CONVERTED  Linear

TO LINEARLY INTERPOLABLE               Linear

*INCREASED PAGE SIZE FROM 3006 TO 5010  Linear

POINTS.                                Linear

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Linear

TO CALCULATIONS.                       Linear

*COMPLETELY CONSISTENT I/O AND ROUNDING Linear

ROUTINES - TO MINIMIZE COMPUTER        Linear

DEPENDENCE.                            Linear

VERSION 92-2 (JULY 1992)   *CORRECTED CONVERSION OF NU-BAR FROM    Linear

POLYNOMIAL TO TABULATED - COPY         Linear

SPONTANEOUS NU-BAR (BY DEFINITION      Linear

THE SPONTANEOUS NU-BAR IS NOT AN       Linear

ENERGY DEPENDENT QUANTITY).            Linear

VERSION 93-1 (MARCH 1993)  *UPDATED FOR USE WITH LAHEY COMPILER    Linear

ON IBM-PCS.                            Linear

*INCREASED PAGE SIZE FROM 5010 TO       Linear

30000 POINTS                           Linear

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Linear

TO ALLOW ACCESS TO FILE STRUCTURES     Linear

(WARNING - INPUT PARAMETER FORMAT      Linear

HAS BEEN CHANGED)                      Linear

*CLOSE ALL FILES BEFORE TERMINATING     Linear

(SEE, SUBROUTINE ENDIT)                Linear

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Linear

*IMPROVED COMPUTER INDEPENDENCE        Linear

*ALL DOUBLE PRECISION                  Linear

*ON SCREEN OUTPUT                      Linear

*UNIFORM TREATMENT OF ENDF/B I/O       Linear

*IMPROVED OUTPUT PRECISION             Linear

*DEFINED SCRATCH FILE NAMES            Linear

*ALWAYS INCLUDE THERMAL VALUE          Linear

*INCREASED PAGE SIZE FROM 30000 TO     Linear

60000 POINTS                          Linear

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Linear

POINT READ FOR MORE DIGITS            Linear

*UPDATED TEST FOR ENDF/B FORMAT        Linear

VERSION BASED ON RECENT FORMAT CHANGE Linear

*GENERAL IMPROVEMENTS BASED ON         Linear

USER FEEDBACK                         Linear

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Linear

MF=1, MT-451.                         Linear

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF = 9 AND 10 LINEARIZATION     Linear

*GENERAL IMPROVEMENTS BASED ON         Linear

USER FEEDBACK                         Linear

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Linear

VERS. 2004-1 (JAN. 2004)    *GENERAL UPDATE BASED ON USER FEEDBACK Linear

VERS. 2005-1 (JAN. 2005)    *ALWAYS KEEP ORIGINAL TABULATED        Linear

NU-BAR POINTS.                        Linear

VERS. 2006-1 (FEB. 2006)    *CORRECTED INT=6 NEAR THRESHOLD        Linear

*NO SUBDIVIDE BELOW MINIMUM XCMIN      Linear

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Linear

*INCREASED PAGE SIZE FROM 60,000 TO    Linear

600,000 POINTS                        Linear

Linear

OWNED, MAINTAINED AND DISTRIBUTED BY                               Linear

------------------------------------                               Linear

THE NUCLEAR DATA SECTION                                           Linear

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Linear

P.O. BOX 100                                                       Linear

A-1400, VIENNA, AUSTRIA                                            Linear

EUROPE                                                             Linear

Linear

ORIGINALLY WRITTEN BY                                              Linear

------------------------------------                               Linear

DERMOTT E. CULLEN                                                  Linear

UNIVERSITY OF CALIFORNIA                                           Linear

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Linear

L-159                                                              Linear

P.O. BOX 808                                                       Linear

LIVERMORE, CA 94550                                                Linear

U.S.A.                                                             Linear

TELEPHONE  925-423-7359                                            Linear

E. MAIL    CULLEN1@LLNL.GOV                                        Linear

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Linear

Linear

AUTHORS MESSAGE                                                    Linear

---------------                                                    Linear

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Linear

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Linear

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Linear

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION.                  Linear

Linear

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Linear

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Linear

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Linear

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Linear

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Linear

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Linear

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Linear

COMPUTER.                                                          Linear

Linear

PURPOSE                                                            Linear

-------                                                            Linear

THIS PROGRAM IS DESIGNED TO CONVERT ENDF/B FILE 3, 23 AND 27 DATA  Linear

TO LINEAR-LINEAR INTERPOLABLE FORM. ANY SECTION THAT IS ALREADY    Linear

LINEAR-LINEAR INTERPOLABLE WILL BE THINNED.                        Linear

Linear

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY  Linear

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE    Linear

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                             Linear

Linear

ENDF/B FORMAT                                                      Linear

-------------                                                      Linear

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Linear

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Linear

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Linear

Linear

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Linear

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Linear

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Linear

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Linear

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451  Linear

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL   Linear

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Linear

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Linear

Linear

OUTPUT FORMAT                                                      Linear

-------------                                                      Linear

IN THIS VERSION OF LINEAR ALL ENERGIES WILL BE OUTPUT IN           Linear

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Linear

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Linear

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Linear

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Linear

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Linear

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Linear

DUE TO TRUNCATION OF ENERGIES TO 6 DIGITS DURING OUTPUT.           Linear

Linear

CONTENTS OF OUTPUT                                                 Linear

------------------                                                 Linear

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE LINEARIZED DATA        Linear

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO     Linear

INCLUDED.                                                          Linear

Linear

DOCUMENTATION                                                      Linear

-------------                                                      Linear

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Linear

BY THE ADDITION OF 3 COMMENT LINES AT THE END OF EACH HOLLERITH    Linear

SECTION IN THE FORM                                                Linear

Linear

***************** PROGRAM LINEAR (2007-1) ****************         Linear

FOR ALL DATA GREATER THAN 1.00000-10 IN ABSOLUTE VALUE             Linear

DATA LINEARIZED TO WITHIN AN ACCURACY OF  0.1  PER-CENT            Linear

Linear

THE ORDER OF SIMILAR COMMENTS (FROM RECENT, SIGMA1 AND GROUPIE)    Linear

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Linear

THE DATA BY THESE PROGRAMS.                                        Linear

Linear

THESE COMMENT LINES ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Linear

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Linear

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Linear

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Linear

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Linear

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Linear

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Linear

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Linear

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Linear

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Linear

Linear

REACTION INDEX                                                     Linear

--------------                                                     Linear

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Linear

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Linear

Linear

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Linear

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Linear

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Linear

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Linear

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Linear

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Linear

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Linear

Linear

SECTION SIZE                                                       Linear

------------                                                       Linear

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Linear

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Linear

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Linear

Linear

FOR ANY LINEARIZED SECTION THAT CONTAINS 60000 OR FEWER POINTS     Linear

THE ENTIRE OPERATION WILL BE PERFORMED IN CORE AND THE LINEARIZED  Linear

DATA WILL BE OUTPUT DIRECTLY TO THE ENDF/B FORMAT. FOR ANY SECTION Linear

THAT CONTAINS MORE POINTS THE DATA WILL BE LINEARIZED A PAGE AT A  Linear

TIME (1 PAGE = 60000 POINTS) AND OUTPUT TO SCRATCH. AFTER THE      Linear

ENTIRE SECTION HAS BEEN LINEARIZED THE DATA WILL BE READ BACK FROM Linear

SCRATCH AND OUTPUT TO THE ENDF/B FORMAT.                           Linear

Linear

SELECTION OF DATA                                                  Linear

-----------------                                                  Linear

THE PROGRAM SELECTS DATA TO BE LINEARIZED BASED EITHER ON EITHER   Linear

MAT (ENDF/B MAT NO.) OR ZA AS WELL AS MF AND MT NUMBERS. THIS      Linear

PROGRAM ALLOWS UP TO 100 MAT/MF/MT OR ZA/MF/MT RANGES TO BE        Linear

SPECIFIED BY INPUT PARAMETERS. THE PROGRAM WILL ASSUME THAT THE    Linear

ENDF/B TAPE IS IN MAT ORDER, REGARDLESS OF THE CRITERIA USED       Linear

TO RETRIEVE MATERIALS. IF RETRIEVAL IS BY MAT RANGE THE PROGRAM    Linear

WILL TERMINATE WHEN A MAT IS FOUND THAT IS ABOVE ALL REQUESTED     Linear

MAT RANGES. IF RETRIEVAL IS BY ZA RANGE THE PROGRAM WILL SEARCH    Linear

THE ENTIRE ENDF/B TAPE.                                            Linear

Linear

PROGRAM OPERATION                                                  Linear

-----------------                                                  Linear

EACH SECTION OF DATA IS CONSIDERED SEPARATELY. EACH SECTION OF     Linear

ENDF/B DATA TO LINEARIZE IS REPRESENTED BY A TABLE OF ENERGY       Linear

VS. CROSS SECTION AND ANY ONE OF FIVE ALLOWABLE INTERPOLATION LAWS Linear

BETWEEN ANY TWO TABULATED POINTS. THIS PROGRAM WILL REPLACE EACH   Linear

SECTION OF DATA CROSS SECTIONS BY A NEW TABLE OF ENERGY VS.        Linear

CROSS SECTION IN WHICH THE INTERPOLATION LAW IS ALWAYS LINEAR IN   Linear

ENERGY AND CROSS SECTION BETWEEN ANY TWO TABULATED POINTS.         Linear

Linear

DATA IS READ AND LINEARIZED A PAGE AT A TIME (ONE PAGE CONTAINS    Linear

60000 DATA POINTS). IF THE FINAL LINEARIZED SECTION CONTAINS TWO   Linear

PAGES OR LESS, DATA POINTS IT WILL BE ENTIRELY CORE RESIDENT       Linear

AFTER IT HAS BEEN LINEARIZED AND WILL BE WRITTEN DIRECTLY FROM     Linear

CORE TO THE OUTPUT TAPE. IF THE LINEARIZED SECTION IS LARGER THAN  Linear

TWO PAGES, AFTER EACH PAGE IS LINEARIZED IT WILL BE WRITTEN TO     Linear

SCRATCH. AFTER THE ENTIRE SECTION HAS BEEN LINEARIZED IT WILL      Linear

BE READ BACK FROM SCRATCH, TWO PAGES AT A TIME, AND WRITTEN TO     Linear

THE OUTPUT TAPE.                                                   Linear

Linear

KEEP EVALUATED DATA POINTS                                         Linear

--------------------------                                         Linear

SOMETIMES IT IS CONVENIENT TO KEEP ALL ENERGY POINTS WHICH WERE    Linear

PRESENT IN THE ORIGINAL EVALUATION AND TO MERELY SUPPLEMENT THESE  Linear

POINTS WITH ADDITIONAL ENERGY POINTS IN ORDER TO LINEARIZE THE     Linear

CROSS SECTIONS. FOR EXAMPLE, IT IS OFTEN CONVENIENT TO KEEP THE    Linear

THERMAL VALUE (AT 0.0253 EV) OR THE VALUE AT 14.1 MEV.             Linear

Linear

THE CURRENT VERSION OF THIS PROGRAM WILL ALLOW THE USER TO KEEP    Linear

ALL ORIGINAL EVALUATED DATA POINTS BY SPECIFYING 1 IN COLUMNS      Linear

34-44 OF THE FIRST INPUT LINE. THIS WILL TURN OFF THE BACKWARD     Linear

THINNING (SEE UCRL-50400, VOL. 17, PART A FOR EXPLANATION) AND     Linear

RESULT IN ALL ORIGINAL ENERGY POINTS BEING KEPT. CAUTION SHOULD    Linear

BE EXERCISED IN USING THIS OPTION SINCE IT CAN RESULT IN A         Linear

CONSIDERABLE INCREASE IN THE NUMBER OF DATA POINTS OUTPUT BY       Linear

THIS CODE.                                                         Linear

Linear

FOR ALL USERS WHO ARE NOT INTERESTED IN THIS OPTIONS NO CHANGES    Linear

ARE REQUIRED IN THE INPUT TO THIS PROGRAM, I. E. IF COLUMNS        Linear

34-44 ARE BLANK (AS FOR ALL PREVIOUS VERSIONS OF THIS CODE) THE    Linear

PROGRAM WILL OPERATE EXACTLY AS IT DID BEFORE.                     Linear

Linear

ALLOWABLE ERROR                                                    Linear

---------------                                                    Linear

ALLOWABLE ERROR MUST ALWAYS BE SPECIFIED IN THE INPUT TO THIS      Linear

PROGRAM AS A FRACTION, NOT A PER-CENT. FOR EXAMPLE, INPUT THE      Linear

ALLOWABLE FRACTIONAL ERROR 0.001 IN ORDER TO OBTAIN DATA THAT IS   Linear

ACCURATE TO WITHIN 0.1 PER-CENT.                                   Linear

Linear

THE CONVERSION OF THE DATA FROM THE GENERAL INTERPOLATION FORM TO  Linear

LINARLY INTERPOLABLE FORM CANNOT BE PERFORMED EXACTLY. HOWEVER, IT Linear

CAN BE PERFORMED TO VIRTUALLY ANY REQUIRED ACCURACY AND MOST       Linear

IMPORTANTLY CAN BE PERFORMED TO A TOLERANCE THAT IS SMALL COMPARED Linear

TO THE UNCERTAINTY IN THE CROSS SECTIONS THEMSELVES. AS SUCH THE   Linear

CONVERSION OF CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM CAN BE  Linear

PERFORMED WITH ESSENTIALLY NO LOSE OF INFORMATION.                 Linear

Linear

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Linear

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Linear

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Linear

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Linear

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Linear

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Linear

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Linear

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.     Linear

Linear

DEFAULT ALLOWABLE ERROR                                            Linear

-----------------------                                            Linear

IN ORDER TO INSURE CONVERGENCE OF THE LINEARIZING ALGORITHM THE    Linear

ALLOWABLE ERROR MUST BE POSITIVE. IF THE USER INPUTS AN ERROR      Linear

THAT IS NOT POSITIVE IT WILL AUTOMATICALLY BE SET TO THE DEFAULT   Linear

VALUE (CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT) AND         Linear

INDICATED AS SUCH IN THE OUTPUT LISTING.                           Linear

Linear

COULOMB PENETRABILITY (INTERPOLATION LAW = 6)                      Linear

--------------------------------------------                       Linear

INTRODUCED FOR ENDF/B-VI. THIS IS DEFINED AS,                      Linear

Linear

SIG(E) = C1*EXP(-C2/SQRT(E - T))                                   Linear

Linear

THIS PROGRAM ONLY CONSIDERS EXOTHERMIC REACTIONS - T = 0           Linear

Linear

SIG(E) = C1*EXP(-C2/SQRT(E))                                       Linear

Linear

WARNING...THIS INTERPOLATION LAW SHOULD ONLY BE USED FOR REACTIONS Linear

WHICH HAVE A POSITIVE Q-VALUE (EXOTHERMIC REACTIONS),    Linear

SINCE HERE WE ONLY CONSIDER T = 0.0 IN THE FORMALISM.    Linear

IN ALL OTHER CASES A WARNING MESSAGE WILL BE PRINTED.    Linear

Linear

INPUT FILES                                                        Linear

-----------                                                        Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                     Linear

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Linear

Linear

OUTPUT FILES                                                       Linear

------------                                                       Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Linear

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Linear

Linear

SCRATCH FILES                                                      Linear

-------------                                                      Linear

UNIT  DESCRIPTION                                                  Linear

----  -----------                                                  Linear

12  SCRATCH FILE (BINARY - 180000 WORDS/RECORD                   Linear

Linear

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Linear

----------------------------------------------------               Linear

UNIT  FILE NAME                                                    Linear

----  ----------                                                   Linear

2   LINEAR.INP                                                   Linear

3   LINEAR.LST                                                   Linear

10   ENDFB.IN                                                     Linear

11   ENDFB.OUT                                                    Linear

12   (SCRATCH)                                                    Linear

Linear

Linear

INPUT PARAMETERS                                                   Linear

----------------                                                   Linear

FOR VERSIONS EARLIER THAN 90-1 THIS PROGRAM ONLY ALLOWED THE USER  Linear

TO SPECIFY BY INPUT PARAMETERS WHICH MATERIALS (MAT) TO PROCESS.   Linear

FOR EACH REQUESTED MATERIAL NEUTRON INTERACTION CROSS SECTIONS     Linear

(MF=3) WOULD BE LINEARIZED AND THE REMAINDER OF THE MATERIAL       Linear

WOULD BE COPIED.                                                   Linear

Linear

FOR VERSIONS 90-1 AND LATER THIS PROGRAM WILL ALLOW THE USER TO    Linear

TO SPECIFY BY INPUT PARAMETERS EXACTLY WHAT SECTIONS OF DATA       Linear

TO PROCESS. FOR EACH SECTION OF DATA, SPECIFIED BY MAT, MF, MT     Linear

RANGES, SECTIONS OF MF=3, 23 AND 27 WILL BE LINEARIZED AND ALL     Linear

OTHER REQUESTED SECTIONS WILL BE COPIED. ALL SECTIONS WHICH ARE    Linear

NOT EXPLICITLY REQUESTED WILL BE SKIPPED AND WILL NOT APPEAR ON    Linear

ENDF/B FILE OUTPUT BY THIS PROGRAM.                                Linear

Linear

WITH THIS NEW PROCEDURE YOU CAN MINIMIZE THE SIZE OF THE ENDF/B    Linear

FILE OUTPUT BY THIS PROGRAM, E.G., IF YOU ONLY WANT NEUTRON        Linear

CROSS SECTIONS FOR SUBSEQUENT PROCESSING YOU NEED ONLY REQUEST     Linear

ONLY MF=3 DATA.                                                    Linear

Linear

HOWEVER, YOU MUST UNDERSTAND THAT ONLY THOSE SECTIONS WHICH YOU    Linear

EXPLICITLY REQUEST WILL APPEAR ON THE ENDF/B FILE OUTPUT BY        Linear

THIS PROGRAM. FOR EXAMPLE, IF YOU WISH TO DOCUMENT EXACTLY         Linear

HOW YOU LINEARIZED THE DATA BY INCLUDING COMMENTS IN MF=1, MT=451  Linear

THEN YOU MUST EXPLICITLY REQUEST THAT MF=1, MT=451 BE PROCESSED    Linear

FOR EACH MATERIAL THAT YOU REQUEST. SIMILAR IF YOU WANT THE        Linear

ENTIRE EVALUATION YOU MUST REQUEST ALL MF AND MT TO BE OUTPUT.     Linear

Linear

LINE  COLS.  DESCRIPTION                                           Linear

----  -----  -----------                                           Linear

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                      Linear

12-22  MONITOR MODE SELECTOR                                 Linear

= 0 - NORMAL OPERATION                                Linear

= 1 - MONITOR PROGRESS OF LINEARIZING OF THE DATA.    Linear

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO   Linear

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF  Linear

POINTS ON SCRATCH AND THE LOWER AND UPPER       Linear

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE   Linear

USED IN ORDER TO MONITOR THE EXECUTION SPEED    Linear

OF LONG RUNNING JOBS).                          Linear

23-33  MINIMUM CROSS SECTION OF INTEREST (BARNS).            Linear

(IF 0.0 OR LESS IS INPUT THE PROGRAM WILL             Linear

USE 1.0E-10). ENERGY INTERVALS WILL NOT BE            Linear

SUB-DIVIDED IF THE ABSOLUTE VALUE OF THE CROSS        Linear

SECTION WITHIN THE INTERVAL IS LESS THAN THIS VALUE.  Linear

AN EXCEPTION TO THIS RULE IS NEAR THRESHOLDS ENERGY   Linear

INTERVALS WILL BE SUB-DIVIDED UNTIL CONVERGENCE       Linear

REGARDLESS OF THE MAGNITUDE OF THE CROSS SECTION.     Linear

34-44  KEEP ORIGINAL EVALUATED DATA POINTS.                  Linear

= 0 - NO.                                             Linear

= 1 - YES - ADDITIONAL POINTS MAY BE ADDED IN ORDER   Linear

TO LINEARIZE DATA, BUT ALL ORIGINAL       Linear

DATA POINTS WILL BE INCLUDED IN THE       Linear

RESULTS.                                  Linear

2   1-60  ENDF/B INPUT DATA FILENAME                            Linear

(STANDARD OPTION = ENDFB.IN)                          Linear

3   1-60  ENDF/B OUTPUT DATA FILENAME                           Linear

(STANDARD OPTION = ENDFB.OUT)                         Linear

4-N   1- 6  LOWER MAT OR ZA LIMIT                                 Linear

7- 8  LOWER MF LIMIT                                        Linear

9-11  LOWER MT LIMIT                                        Linear

12-17  UPPER MAT OR ZA LIMIT                                 Linear

18-19  UPPER MF LIMIT                                        Linear

20-22  UPPER MT LIMIT                                        Linear

UP TO 100 RANGES MAY BE SPECIFIED, ONLY ONE RANGE     Linear

PER LINE. THE LIST OF RANGES IS TERMINATED BY A       Linear

BLANK LINE. IF THE UPPER MAT LIMIT OF ANY REQUEST     Linear

IS LESS THAN THE LOW LIMIT IT WILL BE SET EQUAL TO    Linear

THE LOWER LIMIT. IF THE UPPER LIMIT IS STILL ZERO     Linear

IT WILL BE SET EQUAL TO 999999. IF THE UPPER MF OR    Linear

MT LIMIT IS ZERO IT WILL BE SET TO 99 OR 999          Linear

RESPECTIVELY.                                         Linear

VARY  1-11  ENERGY FOR ERROR LAW                                  Linear

12-22  ALLOWABLE FRACTIONAL ERROR FOR ERROR LAW.             Linear

THE ACCEPTABLE LINEARIZING ERROR MAY BE SPECIFIED TO  Linear

BE EITHER ENERGY INDEPENDENT (DEFINED BY A SINGLE     Linear

ERROR), OR ENERGY DEPENDENT (DEFINED BY UP TO 20      Linear

ENERGY, ERROR PAIRS). FOR THE ENERGY DEPENDENT CASE   Linear

LINEAR INTERPOLATION WILL BE USED TO DEFINE THE ERROR Linear

AT ENERGIES BETWEEN THOSE AT WHICH IT IS TABULATED.   Linear

IN ALL CASES THE ERROR LAW IS TERMINATED BY A BLANK   Linear

LINE. IF ONLY ONE ENERGY, ERROR PAIR IS GIVEN THE     Linear

THE LAW WILL BE CONSIDERED TO BE ENERGY INDEPENDENT.  Linear

IF MORE THAN ONE PAIR IS GIVEN IT WILL BE CONSIDERED  Linear

TO BE ENERGY DEPENDENT (NOTE, ENERGY INDEPENDENT      Linear

FORM WILL RUN FASTER THAN THE EQUIVALENT ENERGY       Linear

DEPENDENT FORM). FOR AN ENERGY DEPENDENT ERROR LAW    Linear

ALL ENERGIES MUST BE ASCENDING ENERGY ORDER. FOR      Linear

CONVERGENCE OF THE LINEARIZING ALGORITHM ALL ERRORS   Linear

MUST BE POSITIVE. IF AN ALLOWABLE ERROR IS NOT        Linear

POSITIVE IT WILL BE SET EQUAL TO THE STANDARD OPTION  Linear

(CURRENTLY 0.001, CORRESPONDING TO 0.1 PER-CENT).     Linear

IF THE FIRST ERROR LINE IS BLANK IT WILL TERMINATE    Linear

THE ERROR LAW AND THE ERROR WILL BE TREATED AS        Linear

ENERGY INDEPENDENT, EQUAL TO THE STANDARD OPTION      Linear

(CURRENTLY 0.1 PER-CENT). (SEE EXAMPLE INPUT 4).      Linear

Linear

EXAMPLE INPUT NO. 1                                                Linear

-------------------                                                Linear

RETRIEVE DATA BY ZA IN ORDER TO FIND ALL URANIUM ISOTOPES AND      Linear

THORIUM 232. RETRIEVE ALL NEUTRON INTERACTION CROSS SECTIONS       Linear

(MF=3). ALL ENERGY INTERVALS IN WHICH THE CROSS SECTION IS         Linear

AT LEAST 1 MICRO-BARN (1.0E-06 BARNS) WILL BE SUBDIVIDED.          Linear

BACKWARD THINNING WILL BE PERFORMED. FROM 0 TO 100 EV LINEARIZE    Linear

TO WITHIN 0.1 PER-CENT ACCURACY. FROM 100 EV TO 1 KEV VARY         Linear

ACCURACY BETWEEN 0.1 AND 1.0 PER-CENT. ABOVE 1 KEV USE 1           Linear

PER-CENT ACCURACY.                                                 Linear

Linear

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Linear

Linear

IN THIS CASE THE FOLLOWING 11 INPUT LINES ARE REQUIRED             Linear

Linear

1          0 1.00000- 6          0                            Linear

ENDFB.IN                                                               Linear

ENDFB.OUT                                                              Linear

92000 3  0 92999 3999                                                  Linear

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999) Linear

(END OF REQUEST LIST)                         Linear

0.00000+ 0 1.00000-03                                                  Linear

1.00000+ 2 1.00000-03                                                  Linear

1.00000+ 3 1.00000-02                                                  Linear

1.00000+ 9 1.00000-02                                                  Linear

(END OF ERROR LAW)                            Linear

Linear

EXAMPLE INPUT NO. 2                                                Linear

-------------------                                                Linear

SAME AS THE ABOVE CASE, EXCEPT LINEARIZE ALL DATA TO WITHIN THE    Linear

STANDARD ACCURACY (CURRENTLY 0.1 PER-CENT). IN ORDER TO USE THE    Linear

STANDARD ACCURACY YOU NEED NOT SPECIFY ANY ERROR LAW AT ALL. IN    Linear

THIS CASE INCLUDE THE HOLLERITH SECTION, MF=1, MT=451, FOR EACH    Linear

MATERIAL.                                                          Linear

Linear

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Linear

THEN USE STANDARD FILENAMES.                                       Linear

Linear

IN THIS CASE THE FOLLOWING 9 INPUT LINES ARE REQUIRED              Linear

Linear

1          0 1.00000- 6          0                            Linear

(USE DEFAULT FILENAME = ENDFB.IN)             Linear

(USE DEFAULT FILENAME = ENDFB.OUT)            Linear

92000 1451 92999 1451                                                  Linear

92000 3  0 92999 3999                                                  Linear

90232 1451     0 1451                                                  Linear

90232 3  0     0 3  0    (UPPER LIMIT AUTOMATICALLY SET TO 90232 3999) Linear

(END OF REQUEST LIST)                         Linear

(0.1 PER-CENT ERROR, END OF ERROR LAW)        Linear

Linear

EXAMPLE INPUT NO. 3                                                Linear

-------------------                                                Linear

LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO WITHIN AN ACCURACY    Linear

OF 0.5 PER-CENT (0.005 AS A FRACTION). IN THIS CASE YOU NEED NOT   Linear

SPECIFY THE MAT, MF, MT RANGES.                                    Linear

Linear

READ THE ENDF/B DATA FROM \ENDFB6\ZA092238 AND WRITE THE ENDF/B    Linear

DATA TO \ENDFB6\LINEAR\ZA092238.                                   Linear

Linear

IN THIS CASE THE FOLLOWING 6 INPUT LINES ARE REQUIRED              Linear

Linear

(MAT, 1.0E-10 BARNS, THIN) Linear

\ENDFB6\ZA092238                                                       Linear

\ENDFB6\LINEAR\ZA092238                                                Linear

(RETRIEVE ALL DATA, END REQUEST LIST)         Linear

5.00000-03                                                  Linear

(END OF ERROR LAW)                            Linear

Linear

NOTE THAT IN THIS CASE IF THE INPUT HAD SPECIFIED AN EQUIVALENT    Linear

ENERGY DEPENDENT ERROR LAW BY GIVING A NUMBER OF ENERGY POINTS     Linear

AT EACH OF WHICH THE ERROR IS 0.5 PER-CENT THE PROGRAM WOULD TAKE  Linear

LONGER TO RUN (I.E., ONLY USE AN ENERGY DEPENDENT ERROR LAW WHEN   Linear

IT IS NECESSARY).                                                  Linear

Linear

EXAMPLE INPUT NO. 4                                                Linear

-------------------                                                Linear

IN ORDER TO LINEARIZE ALL MATERIALS ON AN ENDF/B TAPE TO THE       Linear

STANDARD OPTION OF 0.1 PER-CENT IT IS ADEQUATE TO INPUT A SET      Linear

OF COMPLETELY BLANK LINES WHICH WILL AUTOMATICALLY INVOKE ALL      Linear

OF THE STANDARD OPTIONS.                                           Linear

Linear

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Linear

THEN USE STANDARD FILENAMES.                                       Linear

Linear

IN THIS CASE THE FOLLOWING THREE INPUT LINES ARE REQUIRED          Linear

Linear

(MAT, 1.0E-10 BARNS, THIN) Linear

(USE DEFAULT FILENAME = ENDFB.IN)             Linear

(USE DEFAULT FILENAME = ENDFB.OUT)            Linear

(RETRIEVE ALL DATA, END REQUEST LIST)         Linear

(0.1 PER-CENT ERROR, END OF ERROR LAW)        Linear

Linear

======================================================================= Linear

======================================================================= Merger

Merger

PROGRAM MERGER                                                     Merger

VERSION 80-1 (JANUARY 1980)                                        Merger

VERSION 80-2 (DECEMBER 1980)                                       Merger

VERSION 82-1 (JANUARY 1982)                                        Merger

VERSION 83-1 (JANUARY 1983)*NEW, MORE COMPATIBLE I/O UNIT NUMBERS. Merger

VERSION 85-1 (AUGUST 1985) *FORTRAN-77/H VERSION                   Merger

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMATS                      Merger

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Merger

FILE NAMES (SEE, SUBROUTINES FILIO1    Merger

AND FILIO2 FOR DETAILS).               Merger

*IMPROVED BASED ON USER COMMENTS.       Merger

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Merger

INSURE PROGRAM WILL NOT DO ANYTHING    Merger

CRAZY.                                 Merger

*UPDATED TO USE NEW PROGRAM CONVERT     Merger

KEYWORDS.                              Merger

*ADDED LIVERMORE CIVIC COMPILER         Merger

CONVENTIONS.                           Merger

VERSION 92-1 (JANUARY 1992)*UPDATED BASED ON USER COMMENTS         Merger

*ADDED FORTRAN SAVE OPTION              Merger

VERSION 92-2 (JULY 1992)   *ALLOW UP TO 99 ENDF/B DATA FILES.      Merger

(TO ALLOW MANAGEMENT OF THE ENTIRE     Merger

ENDF/B SYSTEM).                       Merger

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Merger

TO ALLOW ACCESS TO FILE STRUCTURES     Merger

(WARNING - INPUT PARAMETER FORMAT      Merger

HAS BEEN CHANGED)                      Merger

*ONLY SPECIFY FILENAMES - NO UNIT       Merger

NUMBERS ON INPUT (WARNING - INPUT      Merger

PARAMETERS FORMAT HAS BEEN CHANGED)    Merger

*CLOSE ALL FILES BEFORE TERMINATING     Merger

(SEE, SUBROUTINE ENDIT)                Merger

*REQUEST LOG DELETED                    Merger

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Merger

*IMPROVED COMPUTER INDEPENDENCE        Merger

*ALL DOUBLE PRECISION                  Merger

*ON SCREEN OUTPUT                      Merger

*UNIFORM TREATMENT OF ENDF/B I/O       Merger

*IMPROVED OUTPUT PRECISION             Merger

VERSION 99-1 (MARCH 1999)   *GENERAL IMPROVEMENTS BASED ON         Merger

USER FEEDBACK                         Merger

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Merger

USER FEEDBACK                         Merger

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Merger

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE TO DEFINE COMMON        Merger

*ADDED TEND LINE IF NO DATA RETRIEVED  Merger

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Merger

Merger

OWNED, MAINTAINED AND DISTRIBUTED BY                               Merger

------------------------------------                               Merger

THE NUCLEAR DATA SECTION                                           Merger

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Merger

P.O. BOX 100                                                       Merger

A-1400, VIENNA, AUSTRIA                                            Merger

EUROPE                                                             Merger

Merger

ORIGINALLY WRITTEN BY                                              Merger

------------------------------------                               Merger

DERMOTT E. CULLEN                                                  Merger

UNIVERSITY OF CALIFORNIA                                           Merger

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Merger

L-159                                                              Merger

P.O. BOX 808                                                       Merger

LIVERMORE, CA 94550                                                Merger

U.S.A.                                                             Merger

TELEPHONE  925-423-7359                                            Merger

E. MAIL    CULLEN1@LLNL.GOV                                        Merger

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Merger

Merger

AUTHORS MESSAGE                                                    Merger

---------------                                                    Merger

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENTATION   Merger

FOR THIS PROGRAM INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ    Merger

ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY THE      Merger

COMMENTS CONCERNING MACHINE DEPENDENT CODING.                      Merger

Merger

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Merger

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Merger

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Merger

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Merger

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Merger

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Merger

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Merger

COMPUTER.                                                          Merger

Merger

PURPOSE                                                            Merger

-------                                                            Merger

THIS PROGRAM IS DESIGNED TO SELECTIVELY RETRIEVE DATA OFF OF FROM  Merger

1 TO 10 ENDF/B DATA TAPES AND TO MERGE THE SELECTED DATA INTO A    Merger

SINGLE MAT/MF/MT ORDERED FINAL OUTPUT FILE.                        Merger

Merger

IN THE DISCUSSION THAT FOLLOWS FOR SIMPLICITY THE ENDF/B           Merger

TERMINOLOGY---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL       Merger

MEDIUM USED MAY BE TAPE, CARD, DISK OR ANY OTHER MEDIUM.           Merger

Merger

ENDF/B FORMAT                                                      Merger

-------------                                                      Merger

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Merger

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Merger

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV OR V FORMAT).     Merger

Merger

THE ONLY NUMERICAL DATA THAT THIS PROGRAM READS IS THE ZA FROM THE Merger

FIRST CARD OF EACH SECTION AND THE MAT/MF/MT FROM EACH CARD.       Merger

SEQUENCE NUMBERS ARE IGNORED ON INPUT AND ALL OTHER FIELDS ARE     Merger

READ AS HOLLERITH. AS SUCH THIS PROGRAM NEED NOT DISTINGUISH       Merger

BETWEEN DIFFERENT VERSIONS OF THE ENDF/B FORMAT.                   Merger

Merger

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Merger

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Merger

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE  Merger

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Merger

CORRECTLY OUTPUT ON ALL CARDS.                                     Merger

Merger

SECTION SIZE                                                       Merger

------------                                                       Merger

SINCE THIS PROGRAM ONLY READS THE DATA ONE CARD AT A TIME THERE    Merger

IS NO LIMIT TO THE SIZE OF ANY GIVEN SECTION, E.G. THE TOTAL       Merger

CROSS SECTION MAY BE DESCRIBED BY 200,000 DATA POINTS.             Merger

Merger

SELECTION OF DATA                                                  Merger

-----------------                                                  Merger

THE USER MAY CHOOSE TO MERGE ALL DATA OR THE USER MAY SPECIFY      Merger

THAT ONLY CERTAIN DATA SHOULD BE SELECTED. THE DATA TO BE          Merger

SELECTED IS DEFINED BY SPECIFYING UP TO 100 MAT/MF/MT OR           Merger

ZA/MF/MT RANGES. EACH RANGE IS DEFINED BY LOWER AND UPPER LIMITS   Merger

OF MAT/MF/MT OR ZA/MF/MT.                                          Merger

Merger

REQUEST LIMITS                                                     Merger

--------------                                                     Merger

IN ORDER TO SIMPLIFY THE INPUT OF SELECTION REQUESTS THE FOLLOWING Merger

CONVENTIONS HAVE BEEN INTRODUCED IN ORDER TO DEFINE THE UPPER      Merger

LIMITS OF REQUESTS IF THEY ARE NOT DEFINED BY INPUT (I.E., IF THEY Merger

ARE ZERO).                                                         Merger

Merger

(1) MAT OR ZA - IF THE UPPER LIMIT IS ZERO IT IS SET EQUAL TO THE  Merger

LOWER LIMIT.                                       Merger

(2) MF OR MT  - IF THE UPPER LIMIT IS ZERO IT IS SET EQUAL TO THE  Merger

MAXIMUM POSSIBLE VALUE, 99 OR 999 RESPECTIVELY.    Merger

Merger

WITH THESE CONVENTIONS AN ENTIRE EVALUATION MAY BE SELECTED BY     Merger

MERELY SPECIFYING THE LOWER LIMIT OF MAT OR ZA. THE UPPER MAT OR   Merger

ZA LIMIT WILL BE SET EQUAL TO THE LOWER LIMIT, THE LOWER LIMITS OF Merger

MF/MT WILL BE 0/0 AND THE UPPER LIMITS OF MF/MT WILL BE SET TO     Merger

99/999. THIS WILL CAUSE ALL SECTIONS OF A SINGLE EVALUATION TO BE  Merger

SELECTED.                                                          Merger

Merger

SATISFYING SELECTION CRITERIA                                      Merger

-----------------------------                                      Merger

IN ORDER FOR A SECTION TO MEET THE SELECTION CRITERIA SPECIFIED    Merger

BY ONE OF THE RETRIEVAL REQUESTS, EACH OF THE THREE FIELDS (       Merger

MAT/MF/MT OR ZA/MF/MT) MUST INDIVIDUALLY SATISFY THE CORRESPONDING Merger

LIMITS OF THE REQUEST. IT IS NOT SUFFICIENT THAT THE MAT OF A      Merger

SECTION LIE BETWEEN THE MINIMUM AND MAXIMUM MATS OF A REQUEST. THE Merger

MF AND MT WILL ALSO BE INDIVIDUALLY COMPARED TO THE MF AND MT      Merger

LIMITS OF THE REQUEST. FOR EXAMPLE, A SECTION WITH MAT/MF/MT=      Merger

2500/3/2 DOES NOT SATISFY A REQUEST THAT SPECIFIES A REQUEST USING Merger

THE RANGE 2000/3/1 THROUGH 3000/3/1. THIS REQUEST SPECIFIES ALL    Merger

MATERIALS WITH MAT BETWEEN 2000 AND 3000, BUT ONLY THOSE SECTIONS  Merger

WITH MF/MT=3/1. SIMILARLY A REQUEST FOR 2000/3/1 THROUGH 3000/99/  Merger

999 WILL NOT SELECT ANY SECTIONS WITH MF=1 OR 2, SINCE THE         Merger

REQUEST SPECIFIES ALL MATERIALS WITH MAT BETWEEN 2000 AND 3000,    Merger

BUT ONLY THOSE SECTIONS WITH MF= 3, OR MORE.                       Merger

Merger

DUPLICATE SECTIONS                                                 Merger

------------------                                                 Merger

IF TWO OR MORE SECTIONS WITH THE SAME MAT/MF/MT ARE FOUND EITHER   Merger

ON THE SAME OR DIFFERENT TAPES, THE SECTION FROM THE TAPE DEFINED  Merger

EARLIEST IN THE INPUT CARDS WILL BE COPIED TO THE FINAL TAPE AND   Merger

ALL OTHER SECTIONS WITH THE SAME MAT/MF/MT WILL BE SKIPPED. THE    Merger

OUTPUT REPORT WILL INDICATE WHICH SECTIONS WERE COPIED FROM WHICH  Merger

TAPES, AS WELL AS WHICH SECTIONS ARE DUPLICATE AND WERE SKIPPED.   Merger

Merger

REACTION INDEX                                                     Merger

--------------                                                     Merger

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Merger

FOR EACH MATERIAL THE PROGRAM WILL FOLLOW THE CONVENTIONS          Merger

DEFINED ABOVE AND ONLY COPY ONE SECTION MF=1, MT=451 AND SKIP      Merger

ALL OTHERS (IF MORE THAN ONE). THIS CONVENTION HAS BEEN ADOPTED    Merger

BECAUSE MOST USERS DO NOT REQUIRE A CORRECT REACTION INDEX FOR     Merger

THERE APPLICATIONS AND IT WAS NOT CONSIDERED WORTHWHILE TO INCLUDE Merger

THE OVERHEAD OF CONSTRUCTING A CORRECT REACTION INDEX IN THIS      Merger

PROGRAM. HOWEVER, IF YOU REQUIRE A REACTION INDEX FOR YOUR         Merger

APPLICATION AFTER RUNNING THIS PROGRAM YOU MAY USE PROGRAM         Merger

DICTIN TO CREATE ONE.                                              Merger

Merger

RETRIEVAL STATISTICS                                               Merger

--------------------                                               Merger

THERE WILL ALWAYS BE AN OUTPUT REPORT LISTING INDICATING WHICH     Merger

SECTIONS WHERE SELECTED, WHICH DUPLICATE SECTIONS WERE SKIPPED,    Merger

WHICH TAPE THE SECTION WAS ON, WHICH REQUEST (MAT/MF/MT OR         Merger

ZA/MF/MT RANGE) CAUSED THE SECTION TO BE SELECTED AND HOW MANY     Merger

CARDS WERE IN THE SECTION. IN ADDITION THE USER MAY OPTIONALLY     Merger

OBTAIN A FILE CONTAINING THE SAME INFORMATION. THIS FILE MAY BE    Merger

COMBINED WITH OTHER SIMILAR FILES OUTPUT BY THIS PROGRAM IN ORDER  Merger

TO ACCUMULATE RETRIEVAL STATISTICS OVER A PERIOD OF TIME. IF       Merger

SPECIFIED THIS FILE WILL CONTAIN THE FOLLOWING INFORMATION IN      Merger

6I7 FORMAT.                                                        Merger

Merger

(1) ZA                                                             Merger

(2) MAT                                                            Merger

(3) MF                                                             Merger

(4) MT                                                             Merger

(5) NUMBER OF CARDS IN SECTION                                     Merger

(6) REQUEST NUMBER THAT CAUSED SECTION TO BE SELECTED              Merger

Merger

INPUT FILES                                                        Merger

-----------                                                        Merger

UNIT  DESCRIPTION                                                  Merger

----  -----------                                                  Merger

2   INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                     Merger

VARY  FROM 1 TO 99 ENDF/B DATA FILES (BCD - 80 CHARACTERS/RECORD)  Merger

Merger

OUTPUT FILES                                                       Merger

------------                                                       Merger

UNIT  DESCRIPTION                                                  Merger

----  -----------                                                  Merger

3   OUTPUT REPORT LISTING (BCD - 120 CHARACTERS/RECORD)          Merger

10   MERGED ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)              Merger

Merger

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)   Merger

----------------------------------------------------------------   Merger

UNIT  FILE NAME   DESCRIPTION                                      Merger

----  ----------  -----------                                      Merger

2   MERGER.INP  INPUT PARAMETERS                                 Merger

3   MERGER.LST  OUTPUT LISTING                                   Merger

11   ENDFB.OUT   RETRIEVED ENDF/B DATA                            Merger

12   ENDFB.IN1   ENDF/B DATA TO READ...FILENAMES WILL BE DEFINED  Merger

13   ENDFB.IN2   IN THE ORDER ENDFB.IN1, ENDFB.IN2,...ENDFB.I99   Merger

14   ENDFB.IN3   CORRESPONDING TO THE FIRST, SECOND,...99-TH      Merger

15   ENDFB.IN4   ENDF/B DATA FILE TO READ.                        Merger

16   ENDFB.IN5                                                    Merger

17   ENDFB.IN6                                                    Merger

18   ENDFB.IN7                                                    Merger

.                                                          Merger

.                                                          Merger

110   ENDFB.I99                                                    Merger

Merger

INPUT CARDS                                                       Merger

-----------                                                       Merger

CARD  COLUMNS  FORMAT  DESCRIPTION                                Merger

----  -------  ------  -----------                                Merger

1     1-60    A60    FILENAME FOR MERGED OUTPUT.                Merger

(LEAVE BLANK FOR STANDARD = ENDFB.OUT)     Merger

2     1-66  16A4,A2  MERGED FILE LABEL                          Merger

IF BLANK - LABEL FROM FIRST FILE READ WILL Merger

BE OUTPUT                                  Merger

67-70    I4     MERGED FILE ENDF/B NUMBER                  Merger

IF ZERO - NUMBER OF FIRST FILE READ WILL   Merger

BE OUTPUT.                                 Merger

71-72    I2     RETRIEVAL CRITERIA                         Merger

= 0 - MAT/MF/MT RANGES                     Merger

= 1 - ZA/MF/MT RANGES                      Merger

3-N     1-60    A60    FILENAME FOR FILE TO RETRIEVE DATA FROM    Merger

(LEAVE BLANK FOR STANDARD..ENDFB.IN1,ETC.) Merger

TERMINATE LIST OF FILES WITH A LINE THAT   Merger

SAYS END OR end                            Merger

VARY    1- 6    I6     LOWER PRIMARY LIMIT (MAT OR ZA)            Merger

7- 8    I2     LOWER MF LIMIT                             Merger

9-11    I3     LOWER MT LIMIT                             Merger

12-17    I6     UPPER PRIMARY LIMIT (MAT OR ZA)            Merger

18-19    I2     UPPER MF LIMIT                             Merger

20-22    I3     UPPER MT LIMIT                             Merger

RANGES OF MAT/MF/MT OR ZA/MF/MT TO BE      Merger

RETRIEVED ARE SPECIFIED BY DEFINING        Merger

ONE RANGE (LOWER AND UPPER LIMITS) PER     Merger

CARD. THE USER MAY SPECIFY 0 TO 100        Merger

RANGES AND THE LIST OF REQUEST RANGES      Merger

IS TERMINATED BY A BLANK CARD. IF          Merger

THE FIRST CARD IS BLANK (0 REQUESTS)       Merger

ALL DATA WILL BE RETRIEVED. IF THE UPPER   Merger

PRIMARY CRITERIA (MAT OR ZA) IS LESS THAN  Merger

THE LOWER PRIMARY CRITERIA, THE UPPER      Merger

PRIMARY CRITERIA WILL BE SET EQUAL TO      Merger

THE LOWER PRIMARY CRITERIA. IF THE UPPER   Merger

MF OR MT LIMIT IS ZERO, OR BLANK, IT       Merger

WILL BE SET TO THE MAXIMUM POSSIBLE        Merger

VALUE, I.E. MF=99 OR MT=999 (SEE           Merger

EXAMPLE INPUT).                            Merger

Merger

EXAMPLE INPUT NO. 1                                               Merger

-------------------                                               Merger

MERGE ENDF/B DATA ONTO UNIT 10 FROM UNITS 11, 12, 13 AND 14.      Merger

RETRIEVE DATA BY MAT NUMBER. RETRIEVE MATS 1103, 1106, ALL MATS   Merger

BETWEEN 1204 AND 1215, MF=1, 3, 4 AND 5 OF MAT 1219 AND MF=3,     Merger

MT=1 OF MAT 1304. USE STANDARD FILENAMES.                         Merger

Merger

THE FOLLOWING 13 INPUT CARDS ARE REQUIRED.                        Merger

Merger

ENDFB.OUT                                                          Merger

EXAMPLE FILE LABEL FOR MERGER                                  0 0 Merger

ENDFB.IN1                                                          Merger

ENDFB.IN2                                                          Merger

ENDFB.IN3                                                          Merger

ENDFB.IN4                                                          Merger

END                                                                Merger

1103                      4317  (UPPER LIMIT SET TO 1103/99/999) Merger

1106                      4317  (UPPER LIMIT SET TO 1106/99/999) Merger

1204       1215           4317  (UPPER LIMIT SET TO 1215/99/999) Merger

1219 1     1219 1         4317  (UPPER LIMIT SET TO 1219/ 1/999) Merger

1219 3     1219 5         4317  (UPPER LIMIT SET TO 1219/ 5/999) Merger

1304 3  1  1304 3  1      4317  (UPPER LIMIT COMPLETELY DEFINED) Merger

(BLANK CARD TERMINATES REQUESTS) Merger

Merger

EXAMPLE INPUT NO. 2                                               Merger

-------------------                                               Merger

THE SAME AS EXAMPLE 1, EXCEPT SPECIFY FILENAMES                   Merger

Merger

\ENDFB6\MERGED.LIB                                                 Merger

EXAMPLE FILE LABEL FOR MERGER                                  0 0 Merger

ENDFB6.PART1                                                       Merger

ENDFB6.PART2                                                       Merger

ENDFB6.PART3                                                       Merger

ENDFB6.PART4                                                       Merger

END                                                                Merger

1103                      4317  (UPPER LIMIT SET TO 1103/99/999) Merger

1106                      4317  (UPPER LIMIT SET TO 1106/99/999) Merger

1204       1215           4317  (UPPER LIMIT SET TO 1215/99/999) Merger

1219 1     1219 1         4317  (UPPER LIMIT SET TO 1219/ 1/999) Merger

1219 3     1219 5         4317  (UPPER LIMIT SET TO 1219/ 5/999) Merger

1304 3  1  1304 3  1      4317  (UPPER LIMIT COMPLETELY DEFINED) Merger

(BLANK CARD TERMINATES REQUESTS) Merger

Merger

======================================================================= Merger

======================================================================= Mixer

Mixer

PROGRAM MIXER                                                      Mixer

Mixer

VERSION 76-1 (NOVEMBER 1976)                                       Mixer

VERSION 81-1 (APRIL 1981)  *IBM VERSION                            Mixer

VERSION 82-1 (AUGUST 1982) *COMPUTER INDEPENDENT VERSION           Mixer

VERSION 84-1 (JUNE 1984)   *SPECIAL I/O ROUTINES TO GUARANTEE      Mixer

ACCURACY OF ENERGY.                    Mixer

*DOUBLE PRECISION TREATMENT OF ENERGY   Mixer

(REQUIRED FOR NARROW RESONANCES).      Mixer

VERSION 86-1 (JANUARY 1986)*FORTRAN-77/H VERSION                   Mixer

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Mixer

FILE NAMES (SEE, SUBROUTINE FILIO1     Mixer

AND FILIO2 FOR DETAILS).               Mixer

*IMPROVED BASED ON USER COMMENTS.       Mixer

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Mixer

INSURE PROGRAM WILL NOT DO ANYTHING    Mixer

CRAZY.                                 Mixer

*UPDATED TO USE NEW PROGRAM CONVERT     Mixer

KEYWORDS.                              Mixer

*ADDED LIVERMORE CIVIC COMPILER         Mixer

CONVENTIONS.                           Mixer

VERSION 92-1 (JANUARY 1992)*UPDATED BASED ON USER COMMENTS         Mixer

*ADDED PHOTON CROSS SECTIONS            Mixer

*ADDED FORTRAN SAVE OPTION              Mixer

*OUTPUT IN ENDF/B-VI FORMAT             Mixer

*COMPLETELY CONSISTENT I/O ROUTINES -   Mixer

TO MINIMIZE COMPUTER DEPENDENCE.       Mixer

*NOTE, CHANGE IN INPUT PARAMETER        Mixer

FORMAT.                                Mixer

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Mixer

TO ALLOW ACCESS TO FILE STRUCTURES     Mixer

(WARNING - INPUT PARAMETER FORMAT      Mixer

HAS BEEN CHANGED)                      Mixer

*CLOSE ALL FILES BEFORE TERMINATING     Mixer

(SEE, SUBROUTINE ENDIT)                Mixer

*INCREASED INCORE PAGE SIZE FROM        Mixer

1002 TO 4008.                          Mixer

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Mixer

*IMPROVED COMPUTER INDEPENDENCE        Mixer

*ALL DOUBLE PRECISION                  Mixer

*ON SCREEN OUTPUT                      Mixer

*UNIFORM TREATMENT OF ENDF/B I/O       Mixer

*IMPROVED OUTPUT PRECISION             Mixer

*DEFINED SCRATCH FILE NAMES            Mixer

*INCREASED INCORE PAGE SIZE FROM       Mixer

4008 TO 12000.                        Mixer

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Mixer

POINT READ FOR MORE DIGITS            Mixer

*UPDATED TEST FOR ENDF/B FORMAT        Mixer

VERSION BASED ON RECENT FORMAT CHANGE Mixer

*GENERAL IMPROVEMENTS BASED ON         Mixer

USER FEEDBACK                         Mixer

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Mixer

MF=1, MT-451.                         Mixer

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Mixer

USER FEEDBACK                         Mixer

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Mixer

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Mixer

*INCREASED INCORE PAGE SIZE FROM       Mixer

12000 TO 60000.                       Mixer

VERS. 2005-1 (OCT. 2005)    *CORRECTED MERGE ERROR                 Mixer

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII        Mixer

*INCREASED INCORE PAGE SIZE FROM       Mixer

60,000 TO 240,000.                    Mixer

Mixer

OWNED, MAINTAINED AND DISTRIBUTED BY                               Mixer

------------------------------------                               Mixer

THE NUCLEAR DATA SECTION                                           Mixer

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Mixer

P.O. BOX 100                                                       Mixer

A-1400, VIENNA, AUSTRIA                                            Mixer

EUROPE                                                             Mixer

Mixer

ORIGINALLY WRITTEN BY                                              Mixer

------------------------------------                               Mixer

DERMOTT E. CULLEN                                                  Mixer

UNIVERSITY OF CALIFORNIA                                           Mixer

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Mixer

L-159                                                              Mixer

P.O. BOX 808                                                       Mixer

LIVERMORE, CA 94550                                                Mixer

U.S.A.                                                             Mixer

TELEPHONE  925-423-7359                                            Mixer

E. MAIL    CULLEN1@LLNL.GOV                                        Mixer

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Mixer

Mixer

PURPOSE                                                            Mixer

-------                                                            Mixer

THIS PROGRAM IS DESIGNED TO CALCULATE THE ENERGY DEPENDENT CROSS   Mixer

SECTION FOR A COMPOSITE MIXTURE OF UP TO 10 DIFFERENT MATERIALS.   Mixer

Mixer

THE PRESENT VERSION WILL ONLY CALCULATE THE CROSS SECTION FOR ONE  Mixer

FINAL REACTION (ENDF/B SECTION), E.G. TOTAL CROSS SECTION, BUT NOT Mixer

ANY OTHER REACTION.                                                Mixer

Mixer

NOTE, THIS PROGRAM WILL NOT COMBINE ALL REACTIONS FOR A MIXTURE    Mixer

OF MATERIALS DURING A SINGLE RUN - ONLY ONE REACTION WILL BE       Mixer

CREATED PER RUN.                                                   Mixer

Mixer

EVALUATED DATA FORMAT                                              Mixer

---------------------                                              Mixer

THE CROSS SECTIONS ARE READ FROM THE ENDF/B FORMAT AND THE         Mixer

COMPOSITE CROSS SECTION IS CONVERTED TO AN EQUIVALENT BARNS/ATOM   Mixer

FORM AND OUTPUT IN THE ENDF/B FORMAT WITH AN EQUIVALENT ATOMIC     Mixer

WEIGHT. THE USER MUST SPECIFY THE COMPOSITION BY GIVING THE ZA,    Mixer

MT AND GRAMS/CC OF EACH CONSTITUENT. IN ADDITION THE USER MUST     Mixer

IDENTIFY THE COMPOSITE CROSS SECTION BY SPECIFYING THE ZA, MAT     Mixer

AND MT TO BE USED IN THE ENDF/B FORMATTED OUTPUT.                  Mixer

Mixer

SINCE ONLY THE CROSS SECTIONS IN FILE 3 AND 23 ARE USED, AND THE   Mixer

FORMAT FOR FILE 3/23 IS THE SAME IN ALL VERSIONS ON ENDF/B, THIS   Mixer

PROGRAM MAY BE USED WITH ANY VERSION OF ENDF/B DATA (I.E.,         Mixer

ENDF/B-I, II, III, IV, V OR VI). DURING A SINGLE RUN IT MAY EVEN   Mixer

BE USED TO READ AND COMBINE EVALUATIONS WHICH ARE IN DIFFERENT     Mixer

VERSIONS OF THE ENDF/B FORMAT.                                     Mixer

Mixer

ENDF/B FORMATTED OUTPUT WILL BE IN THE ENDF/B-VI FORMAT REGARDLESS Mixer

OF THE FORMAT OF THE INPUT ENDF/B DATA. THIS WILL ONLY EFFECT THE  Mixer

HOLLERITH SECTION (MF=1, MT=451). THE FORMAT OF CROSS SECTIONS     Mixer

(MF=3) IS THE SAME IN ALL VERSION OF THE ENDF/B FORMAT.            Mixer

Mixer

IN ORDER TO GUARANTEE PROPER OPERATION OF THIS PROGRAM THE DATA    Mixer

MUST BE PROPERLY CODED IN THE ENDF/B FORMAT. NO ERROR CHECKING IS  Mixer

PERFORMED. IT IS PARTICULARLY IMPORTANT THAT THE FOLLOWING DATA    Mixer

BE CORRECT                                                         Mixer

Mixer

(1) ZA, MF, MT - MUST BE CORRECT IN ORDER TO ALLOW PROGRAM TO      Mixer

SELECT THE APPROPRIATE SECTIONS TO BE COMBINED.                Mixer

(2) AWRE - ATOMIC WEIGHT RATIO MUST BE CORRECT TO ALLOW PROGRAM    Mixer

TO CONVERT THE USER SPECIFIED GRAMS/CC INTO ATOMS/CC FOR       Mixer

PROPER ATOM RATIO MIXING.                                      Mixer

(3) (ENERGIES, CROSS SECTIONS) - MUST BE CORRECT, LINEARLY         Mixer

========         Mixer

INTERPOLABLE, IN ASCENDING ENERGY ORDER OF (E, BARNS).         Mixer

============                                                   Mixer

Mixer

TO CONVERT ENDF/B FORMATTED DATA TO THE REQUIRED INPUT FORM    Mixer

THE FOLLOWING PROGRAMS MAY BE USED,                            Mixer

LINEAR - CONVERT TABULATED CROSS SECTIONS TO LINEARLY          Mixer

INTERPOLABLE FORM.                                    Mixer

RECENT - RECONSTRUCT RESONANCE CONTRIBUTION, ADD TO BACKGROUND Mixer

CROSS SECTION AND OUTPUT THE COMBINATION IN LINEARLY  Mixer

INTERPOLABLE FORM.                                    Mixer

SIGMA1 - DOPPLER BROADEN CROSS SECTIONS TO ANY TEMPERATURE AND Mixer

OUTPUT THE RESULT IN LINEARLY INTERPOLABLE FORM.      Mixer

Mixer

DOCUMENTATION                                                      Mixer

-------------                                                      Mixer

THE FACT THAT THIS PROGRAM HAS COMBINED THE DATA IS DOCUMENTED     Mixer

IN THE OUTPUT ENDF/B FORMAT IN THE HOLLERITH SECTION BY FIRST      Mixer

IDENTIFYING THE VERSION OF THIS PROGRAM THAT WAS USED, IN THE FORM Mixer

Mixer

********************( PROGRAM MIXER 2007-1) ********************** Mixer

Mixer

THIS IS FOLLOWED BY THE TWO LINE IDENTIFICATION INPUT BY THE USER. Mixer

THIS IS FOLLOWED BY COMPOSITION INPUT BY THE USER.                 Mixer

Mixer

NEUTRON OR PHOTON DATA                                             Mixer

----------------------                                             Mixer

THIS PROGRAM WILL ALLOW YOU TO PROCESS EITHER NEUTRON OR PHOTON    Mixer

CROSS SECTIONS - BUT YOU CANNOT MIX THE TWO TYPES TOGETHER. BY     Mixer

INPUT YOU CAN SPECIFY THE OUTPUT MF = 3 (NEUTRONS) OR 23 (PHOTONS) Mixer

WHATEVER TYPE YOU SPECIFIED FOR OUTPUT IS THE ONLY TYPE OF DATA    Mixer

WHICH WILL BE PROCESSED BY THIS PROGRAM.                           Mixer

Mixer

DEFINING THE COMPOSITION                                           Mixer

------------------------                                           Mixer

THE USER MAY SPECIFY UP TO 10 DIFFERENT SECTIONS OF DATA TO BE     Mixer

COMBINED, EACH SECTION IDENTIFIED BY ZA AND MT NUMBER. THE         Mixer

AMOUNT OF EACH MATERIAL IS SPECIFIED BY DEFINING THE NUMBER OF     Mixer

GRAMS/CC OF EACH MATERIAL IN THE COMPOSITE MIXTURE. THIS CAN BE    Mixer

DERIVED FROM THE VOLUME FRACTION SIMPLY BY MULTIPLYING THE STP     Mixer

DENSITY OF EACH MATERIAL BY ITS VOLUME FRACTION. NOTE, DO NOT      Mixer

INPUT ATOM FRACTIONS.                                              Mixer

Mixer

THE LIST OF SECTIONS TO BE COMBINED MAY BE SPECIFIED IN ANY        Mixer

ORDER, I.E. THEY NEED NOT BE IN ZA ORDER OR THE ORDER THAT THE     Mixer

EVALUATED DATA APPEARS ON THE ENDF/B FORMATTED TAPE.               Mixer

Mixer

IF ANY REQUESTED SECTION OF DATA IS NOT FOUND ON THE ORIGINAL      Mixer

ENDF/B FORMATTED FILE, THE PROGRAM WILL PRINT A LIST OF THE        Mixer

MISSING SECTIONS AND TERMINATE. IF ALL REQUESTED SECTIONS ARE      Mixer

FOUND THE PROGRAM WILL PRODUCE A COMPOSITE SECTION USING THE       Mixer

UNION OF ALL ENERGIES FOUND IN ANY SECTION. THE COMPOSITE SECTION  Mixer

WILL NOT BE THINNED.                                               Mixer

Mixer

PRIOR TO LATER USE IN ANY APPLICATION THE NUMBER OF ENERGY POINTS  Mixer

IN THE COMPOSITE CROSS SECTION MAY BE MINIMIZED BY USING PROGRAM   Mixer

LINEAR, UCRL-50400, VOL. 17, PART B TO THIN THE DATA.              Mixer

Mixer

ONLY LINEARLY INTERPOLABLE DATA                                    Mixer

-------------------------------                                    Mixer

THE CROSS SECTIONS TO BE COMBINED MUST BE IN LINEARLY INTERPOLABLE Mixer

TABULATED FORM (I. E., FILE 3 OR 23, INTERPOLATION LAW 2).         Mixer

Mixer

TO CONVERT TABULATED CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM  Mixer

SEE, PROGRAM LINEAR, UCRL-50400, VOL. 17, PART A.                  Mixer

Mixer

TO CONVERT RESONANCE PARAMETERS TO LINEARLY INTERPOLABLE FORM SEE, Mixer

PROGRAM RECENT, UCRL-50400, VOL. 17, PART C.                       Mixer

Mixer

TO DOPPLER BROADEN LINEARLY INTERPOLABLE DATA TO ANY TEMPERATURE   Mixer

SEE PROGRAM SIGMA1, UCRL-50400, VOL. 17, PART B.                   Mixer

Mixer

PAGING SYSTEM                                                      Mixer

-------------                                                      Mixer

THERE IS NO LIMIT TO THE THE NUMBER OF DATA POINTS IN EACH OF THE  Mixer

SECTIONS TO BE COMBINED, NOR IS THERE A LIMIT TO THE NUMBER OF     Mixer

DATA POINTS IN THE COMPOSITE MIXTURE CROSS SECTION.                Mixer

Mixer

ALL REQUIRED SECTIONS OF DATA ARE READ FROM THE ORIGINAL ENDF/B    Mixer

FORMATTED FILE. ANY SECTION OF 60000 OR FEWER POINTS WILL BE       Mixer

TOTALLY CORE RESIDENT. LARGER SECTIONS ARE LOADED INTO A PAGING    Mixer

SYSTEM USING A SCRATCH FILE WITH ONLY 60000 POINTS PER SECTION     Mixer

CORE RESIDENT AT ANY ONE TIME. SIMILARLY THE COMPOSITE SECTION     Mixer

WILL BE TOTALLY CORE RESIDENT IF IT CONTAINS 60000 OR FEWER POINTS Mixer

AND LARGER COMPOSITE SECTIONS WILL BE LOADED INTO A PAGING         Mixer

SYSTEM WHERE ONLY 60000 POINTS ARE CORE RESIDENT AT ANY TIME. SINC Mixer

A PAGING SYSTEM MAY BE USED BY ANY SECTION OF DATA THERE IS NO     Mixer

LIMIT TO THE SIZE OF EITHER THE ORIGINAL SECTIONS, NOR TO THE      Mixer

COMPOSITE SECTION, E.G. A SECTION MAY CONTAIN 100,000 ENERGIES     Mixer

AND CROSS SECTIONS TO DESCRIBE A GIVEN REACTION.                   Mixer

Mixer

PAGE SIZE                                                          Mixer

---------                                                          Mixer

THE PAGE SIZE USED IN THIS PROGRAM IS DEFINED BY THE PARAMETER     Mixer

NPAGE AND THE DIMENSIONS OF THE ARRAYS XTAB AND YTAB. IN ORDER     Mixer

TO ADAPT THIS PROGRAM FOR USE ON ANY COMPUTER THE PAGE SIZE MAY    Mixer

BE INCREASED OR DECREASED BUT THE FOLLOWING RULES MUST BE FOLLOWED Mixer

====             Mixer

Mixer

(1) NPAGE - MUST BE A MULTIPLE OF 3 IN ORDER TO ALLOW THE PROGRAM  Mixer

TO READ FULL CARDS OF ENDF/B DATA (3 POINTS PER LINE). FAILURE Mixer

TO FOLLOW THIS RULE CAN LEAD TO LOSS OF DATA AND/OR PROGRAM    Mixer

ERRORS DURING EXECUTION.                                       Mixer

(3) YTAB  - THE DIMENSION OF YTAB MUST BE (NPAGE,11).              Mixer

(4) XTAB  - THE DIMENSION OF XTAB MUST BE (NPAGE,11).              Mixer

Mixer

DOPPLER BROADENING                                                 Mixer

------------------                                                 Mixer

THE COMPOSITE CROSS SECTION OUTPUT FROM THIS PROGRAM SHOULD NOT    Mixer

BE DOPPLER BROADENED USING PROGRAM SIGMA1, OR THE EQUIVALENT. THE  Mixer

ATOMIC WEIGHT USED TO IDENTIFY THE COMPOSITE MIXTURE IS BASED ON   Mixer

THE ATOM FRACTION OF EACH CONSTITUENT AND CANNOT BE USED TO        Mixer

CHARACTERIZE THE BROADENING OF ANY GIVEN RESONANCE IN THE MIXTURE  Mixer

DUE TO THE CONTRIBUTION OF ONE CONSTITUENT. IN ORDER TO CONSIDER   Mixer

DOPPLER BROADENING FIRST USE PROGRAM SIGMA1 TO BROADEN THE CROSS   Mixer

SECTION FOR EACH OF THE CONSTITUENTS AND THEN COMBINE THE          Mixer

BROADENED DATA USING PROGRAM MIXER.                                Mixer

Mixer

EXAMPLE USE                                                        Mixer

-----------                                                        Mixer

THE OUTPUT FROM THIS PROGRAM HAS BEEN FOUND TO BE EXTREMELY        Mixer

USEFUL IN THE FOLLOWING APPLICATIONS...                            Mixer

Mixer

(1) CALCULATE A COMPOSITE TOTAL CROSS SECTON FOR LATER USE AS      Mixer

A WEIGHTING FUNCTION IN SELF-SHIELDING THE CROSS SECTIONS      Mixer

OF EACH CONSTITUENT OF THE MIXTURE SEPARATELY.                 Mixer

Mixer

PROGRAM GROUPIE CAN USE THE CALCULATED COMPOSITE TOTAL CROSS   Mixer

SECTION AS THE TOTAL CROSS SECTION FOR EACH CONSTITUENT OF     Mixer

THE MIXTURE IN ORDER TO CALCULATE  SELF-SHIELDED CROSS SECTION Mixer

FOR EACH CONSTITUENT OF THE MIXTURE.                           Mixer

Mixer

(2) CALCULATE COMPOSITE TOTAL AND FISSION CROSS SECTIONS IN        Mixer

ORDER TO CALCULATE THE TRANSMISSION AND SELF-INDICATION        Mixer

THROUGH COMPOSITE MATERIALS. GENERALLY IN THIS CASE THE        Mixer

TOTAL CROSS SECTION WILL BE CALCULATED FOR THE COMPOSITION     Mixer

OF THE SAMPLE AND THE FISSION CROSS SECTION WILL BE            Mixer

CALCULATED FOR THE COMPOSITION OF THE FISSION CHAMBER          Mixer

(WHICH GENERALLY WILL HAVE A DIFFERENT COMPOSITION THAN THE    Mixer

SAMPLE).                                                       Mixer

Mixer

PROGRAM VIRGIN CAN USE THE OUTPUT FROM THIS PROGRAM TO         Mixer

PERFORM TRANSMISSION AND SELF-INDICATION CALCULATIONS.         Mixer

PROGRAM VIRGIN WILL ANALYTICALLY CALCULATE THE UNCOLLIDED      Mixer

(I.E. VIRGIN) FLUX TRANSMITTED AND REACTION RATE DUE TO ANY    Mixer

TABULATED LINEARLY INTERPOLABLE INCIDENT SPECTRUM. RESULTS     Mixer

WILL BE PRESENTLY FOR UP TO 10 DIFFERENT SAMPLE THICKNESSES    Mixer

AND BINNED INTO ENERGY GROUPS IN ORDER TO SIMULATE AN          Mixer

EXPERIMENTAL MEASUREMENT.                                      Mixer

Mixer

(3) THE OUTPUT FROM THIS PROGRAM IS VERY USEFUL TO PLOT IN ORDER   Mixer

TO SEE THE IMPORTANCE OF SPECIFIC CROSS SECTION FEATURES IN    Mixer

THE COMPOSITE CROSS SECTION.                                   Mixer

Mixer

PROGRAM COMPLOT CAN BE USED TO PLOT THE OUTPUT FROM THIS       Mixer

PROGRAM AND IF REQUIRED EXAMINE ANY PARTICULAR ENERGY RANGE    Mixer

IN DETAIL. IN ORDER TO DO THIS THE (ZA, MT) EQUIVALENCE OPTION Mixer

OF PROGRAM COMPLOT SHOULD BE USED. TO COMPARE ANY CONSTITUENT  Mixer

CROSS SECTION TO THE COMPOSITE CROSS SECTION THE INPUT TO      Mixer

COMPLOT SHOULD EQUATE THE (ZA,MT) OF THE COMPOSITE TO THE      Mixer

(ZA,MT) OF ONE CONSTITUENT AND THE MULTIPLIER INPUT TO         Mixer

COMPLOT SHOULD BE THE ATOM FRACTION FOR THE CONSTITUENT (THE   Mixer

ATOM FRACTIONS ARE DEFINED IN THE OUTPUT LISTING FROM PROGRAM  Mixer

MIXER).                                                        Mixer

Mixer

INPUT FILES                                                        Mixer

-----------                                                        Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

2   INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                     Mixer

10   ORIGINAL EVALUATED DATA IN ENDF/B FORMAT                     Mixer

(BCD - 80 CHARACTERS/RECORD)                     Mixer

Mixer

OUTPUT FILES                                                       Mixer

------------                                                       Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

3   OUTPUT LISTING (BCD - 120 CHARACTERS/RECORD)                 Mixer

11   COMPOSITE EVALUATED DATA IN ENDF/B FORMAT                    Mixer

(BCD - 80 CHARACTERS/RECORD)                       Mixer

Mixer

SCRATCH FILES                                                      Mixer

-------------                                                      Mixer

UNIT  DESCRIPTION                                                  Mixer

----  -----------                                                  Mixer

12   SCRATCH FILE FOR EACH OF THE 10 SECTIONS WHICH               Mixer

13   WILL BE ADDED TOGETHER TO DEFINE THE FINAL                   Mixer

.   SECTION (BINARY - 60000 AND 480000 WORDS/RECORD)             Mixer

. .                                                              Mixer

. .                                                              Mixer

20 .                                                              Mixer

21 .                                                              Mixer

22   SCRATCH FILE FOR COMBINED SECTION.                           Mixer

(BINARY - 2004 WORDS/RECORD)                                 Mixer

Mixer

STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)            Mixer

----------------------------------------------------------------   Mixer

UNIT  FILE NAME                                                    Mixer

----  ----------                                                   Mixer

2   MIXER.INP                                                    Mixer

3   MIXER.LST                                                    Mixer

10   ENDFB.IN                                                     Mixer

11   ENDFB.OUT                                                    Mixer

12-22  (SCRATCH)                                                    Mixer

Mixer

INPUT CARDS                                                        Mixer

-----------                                                        Mixer

LINE  COLS.  FORMAT  NAME    DESCRIPTION                           Mixer

----  -----  ------  ------- ----------                            Mixer

1-2   1-66 16A4,A2  TITLE   TWO LINE TITLE DESCRIBING PROBLEM     Mixer

(THIS TITLE IS USED TO IDENTIFY THE   Mixer

OUTPUT LISTING AND IS ALSO WRITTEN    Mixer

IN MF=1, MT=451 (HOLLERITH SECTION)   Mixer

OF THE ENDF/B FORMATTED OUTPUT TO     Mixer

IDENTIFY THE COMPOSITE MIXTURE).      Mixer

3   1-60                  ENDF/B INPUT DATA FILENAME            Mixer

(STANDARD OPTION = ENDFB.IN)          Mixer

4   1-60                  ENDF/B OUTPUT DATA FILENAME           Mixer

(STANDARD OPTION = ENDFB.OUT)         Mixer

5   1-11    I11   IZAOUT  ZA IDENTIFICATION FOR COMBINATION     Mixer

5  12-17    I6    MATOUT  MAT IDENTIFICATION FOR COMBINATION    Mixer

5  18-19    I2    MFOUT   MF IDENTIFICATION FOR COMBINATION     Mixer

5  20-22    I3    MTOUT   MT IDENTIFICATION FOR COMBINATION     Mixer

6-N   1-11    I11   IZAGET  ZA (1000*Z+A) OF MATERIAL             Mixer

6-N  12-22    I11   MTGET   MT OF REACTION                        Mixer

6-N  23-33  E11.4   DENSE   DENSITY OF MATERIAL (GRAMS/CC)        Mixer

Mixer

THE SIXTH LINE IS REPEATED FOR EACH SECTION (FROM 2 TO 10).        Mixer

SINCE THE ENDF/B FORMATTED OUTPUT IS IN BARNS/ATOM FORM A MINIMUM  Mixer

OF TWO SECTIONS MUST BE COMBINED (I.E., IF ONLY ONE SECTION IS     Mixer

SPECIFIED THE OUTPUT WOULD BE IDENTICAL TO THE INPUT AND AS SUCH   Mixer

THE PROGRAM WILL CONSIDER THIS TO BE AN ERROR AND NOT PERFORM THE  Mixer

CALCULATION). THE LIST OF SECTIONS IS TERMINATED BY A BLANK LINE.  Mixer

Mixer

THE LIST OF SECTIONS TO BE COMBINED MAY BE SPECIFIED IN ANY        Mixer

ORDER, I.E. THEY NEED NOT BE IN ZA ORDER OR THE ORDER THAT THE     Mixer

EVALUATED DATA APPEARS ON THE ENDF/B FORMATTED TAPE.               Mixer

Mixer

EXAMPLE INPUT NO. 1                                                Mixer

-------------------                                                Mixer

CREATE THE TOTAL CROSS SECTION (MT=1) FOR STAINLESS STEEL AND      Mixer

IDENTIFY THE COMBINED MATERIAL WITH ZA=26800 AND MAT=4000,         Mixer

THE COMPOSITION BY VOLUME OF THE STEEL WILL BE...                  Mixer

Mixer

THE DATA FROM \ENDFB6\K300\LIBRARY.DAT AND WRITE DATA TO           Mixer

\MIXER\STEEL.DAT                                                   Mixer

Mixer

IRON      - 74.8 PER-CENT                                          Mixer

CHROMIUM  - 16.0                                                   Mixer

NICKEL    -  6.0                                                   Mixer

MANGANESE -  2.0                                                   Mixer

SILICON   -  1.0                                                   Mixer

CARBON    -  0.2                                                   Mixer

Mixer

THE INPUT MUST SPECIFY THE COMPOSITION BY GRAMS/CC. THIS IS        Mixer

DEFINED AS THE PRODUCT OF THE STANDARD DENSITY (GRAMS/CC)          Mixer

TIMES THE VOLUME FRACTION. NOTE, DO NOT USE ATOM FRACTIONS.        Mixer

FOR THIS EXAMPLE THE FOLLOWING 12 INPUT CARDS ARE REQUIRED....     Mixer

Mixer

STAINLESS STEEL. COMPOSITION BY PER-CENT VOLUME IS 74.8-IRON,      Mixer

16-CHROME, 6-NICKEL, 2-MANGANESE, 1-SILICON, 0.2-CARBON            Mixer

\ENDFB6\K300\LIBRARY.DAT                                           Mixer

\MIXER\STEEL.DAT                                                   Mixer

26800  4000 3  1                                              Mixer

26000          1 5.88676    (NOTE, GRAMS/CC INPUT FOR EACH    Mixer

24000          1 1.150448    CONSTITUENT, E.G. FOR IRON THE   Mixer

28000          1 0.533928    STP DENSITY IS 7.87 GRAMS/CC.    Mixer

25055          1 0.1486      THE INPUT VALUE OF 5.88676 IS    Mixer

14000          1 0.0233      0.748 X 7.87,I.E.  VOLUME        Mixer

6012          1 0.0044958   FRACTION TIMES STP DENSITY).     Mixer

(BLANK LINE TERMINATES INPUT LIST) Mixer

Mixer

EXAMPLE INPUT NO. 2                                                Mixer

-------------------                                                Mixer

THE SAME EXAMPLE AS THE ABOVE PROBLEM, ONLY USE THE STANDARD       Mixer

ENDF/B DATA FILENAMES - ENDFB.IN AND ENDFB.OUT (THIS CAN BE        Mixer

DONE BY LEAVING THE THIRD AND FOURTH INPUT LINES BLANK).           Mixer

FOR THIS EXAMPLE THE FOLLOWING 12 INPUT CARDS ARE REQUIRED....     Mixer

Mixer

STAINLESS STEEL. COMPOSITION BY PER-CENT VOLUME IS 74.8-IRON,      Mixer

16-CHROME, 6-NICKEL, 2-MANGANESE, 1-SILICON, 0.2-CARBON            Mixer

(NOTE - THIS LINE IS REALLY BLANK)                                 Mixer

(NOTE - THIS LINE IS REALLY BLANK)                                 Mixer

26800  4000 3  1                                              Mixer

26000          1 5.88676    (NOTE, GRAMS/CC INPUT FOR EACH    Mixer

24000          1 1.150448    CONSTITUENT, E.G. FOR IRON THE   Mixer

28000          1 0.533928    STP DENSITY IS 7.87 GRAMS/CC.    Mixer

25055          1 0.1486      THE INPUT VALUE OF 5.88676 IS    Mixer

14000          1 0.0233      0.748 X 7.87,I.E.  VOLUME        Mixer

6012          1 0.0044958   FRACTION TIMES STP DENSITY).     Mixer

(BLANK LINE TERMINATES INPUT LIST) Mixer

Mixer

======================================================================= Mixer

======================================================================= Recent

Recent

PROGRAM RECENT                                                     Recent

VERSION 79-1 (OCTOBER 1979)  CDC-7600                              Recent

VERSION 80-1 (MAY 1980)      IBM, CDC AND CRAY VERSION             Recent

VERSION 80-2 (DECEMBER 1980) IMPROVED TREATMENT OF UNRESOLVED      Recent

REGION TO COMPUTE ALL REACTIONS AT    Recent

THE SAME TIME.                        Recent

VERSION 81-1 (MARCH 1981)  IMPROVED BASED ON USER COMMENTS.        Recent

VERSION 81-2 (AUGUST 1981) ADDED MONITOR MODE. ADDED SPEED OPTION  Recent

TO BYPASS BACKWARDS THINNING IF FILE 3  Recent

ALLOWABLE ERROR = 0.0 (NOTE THIS OPTION Recent

WILL RESULT IN ALL TABULATED POINTS     Recent

FROM THE EVALUATION BEING KEPT IN THE   Recent

OUTPUT FROM THIS PROGRAM).              Recent

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY.       Recent

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Recent

*PAGE SIZES INCREASED.                  Recent

*ELIMINATED COMPUTER DEPENDENT CODING.  Recent

*NEW, MORE COMPATIBLE I/O UNIT NUMBERS. Recent

*ADDED OPTION TO KEEP ALL RECONSTRUCTED Recent

AND BACKGROUND ENERGY POINTS.          Recent

*ADDED STANDARD ALLOWABLE ERROR OPTIONS Recent

(CURRENTLY 0.1 PER-CENT RECONSTRUCTION Recent

AND 0.0 PER-CENT THINNING).            Recent

VERSION 83-2 (OCTOBER 1983) IMPROVED BASED ON USER COMMENTS.       Recent

VERSION 84-1 (JANUARY 1984) IMPROVED INTERVAL HALFING CONVERGENCE. Recent

VERSION 85-1 (APRIL 1985)  *A BRAND NEW PROGRAM WHICH COMPLETELY   Recent

SUPERCEDES ALL PREVIOUS VERSIONS OF    Recent

THIS PROGRAM.                          Recent

*UPDATED FOR ENDF/B-VI FORMATS.         Recent

*ADDED GENERAL REICH-MOORE FORMALISM    Recent

(WITH TWO FISSION CHANNELS).           Recent

*DECREASED RUNNING TIME.                Recent

*SPECIAL I/O ROUTINES TO GUARANTEE      Recent

ACCURACY OF ENERGY.                    Recent

*DOUBLE PRECISION TREATMENT OF ENERGY   Recent

(REQUIRED FOR NARROW RESONANCES).      Recent

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Recent

VERSION 86-1 (JANUARY 1986)*ENERGY DEPENDENT SCATTERING RADIUS     Recent

VERSION 86-2 (JUNE 1986)   *IF FIRST CHANCE FISSION (MT=19)        Recent

BACKGROUND IS PRESENT ADD RESONANCE    Recent

CONTRIBUTION OF FISSION TO IT.         Recent

VERSION 86-3 (OCTOBER 1986)*MULTI-LEVEL OR REICH-MOORE..CORRECT    Recent

POTENTIAL SCATTERING CROSS SECTION FOR Recent

MISSING AND/OR FICTICIOUS (L,J)        Recent

SEQUENCES.                             Recent

VERSION 87-1 (JANUARY 1987)*IMPROVED COMBINING FILE 2+3            Recent

VERSION 87-2 (MARCH 1987)  *CORRECTED ADLER-ADLER CALCULATIONS.    Recent

VERSION 88-1 (JULY 1988)   *UPDATED REICH-MOORE ENDF/B-VI FORMAT   Recent

TO BE THE SAME AS REICH-MOORE FORMAT   Recent

IN EARLIER VERSIONS OF ENDF/B FORMAT.  Recent

*CHECK FOR PRELIMINARY ENDF/B-VI        Recent

REICH-MOORE FORMAT (NOW ABANDONED)     Recent

AND TERMINATE EXECUTION IF DATA IS     Recent

IN THIS FORMAT.                        Recent

*CALCULATE CHANNEL RADIUS OR SET IT     Recent

EQUAL TO THE SCATTERING RADIUS.        Recent

*IMPLEMENTED HYBRID R-FUNCTION WITH THE Recent

FOLLOWING RESTRICTIONS                 Recent

- ONLY INELASTIC COMPETITION (NO       Recent

CHARGED PARTICLES)                   Recent

- NO TABULATED FILE 2 BACKGROUND       Recent

- NO TABULATED OPTICAL MODEL PHASE     Recent

SHIFT                                Recent

*PROGRAM EXIT IF GENERAL R-MATRIX IN    Recent

THE EVALUATION (THIS FORMALISM WILL    Recent

BE IMPLEMENTED ONLY AFTER THE AUTHOR   Recent

RECEIVES REAL EVALUATIONS WHICH USE    Recent

THIS FORMALISM...UNTIL THEN IT IS      Recent

IMPOSSIBLE TO ADEQUATELY TEST THAT     Recent

THE CODING FOR THIS FORMALISM IS       Recent

CORRECT).                              Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 1002 TO 4008.                     Recent

*DOUBLE PRECISION RESONANCE REGION      Recent

LIMITS.                                Recent

*FILE 2 AND FILE 3 ENERGIES WHICH ARE   Recent

NEARLY EQUAL ARE TREATED AS EQUAL      Recent

(I.E., SAME TO ABOUT 9 DIGITS).        Recent

*CHECK FILE 3 BACKGROUND CROSS SECTIONS Recent

IN EDIT MODE.                          Recent

*OPTION...INTERNALLY DEFINE FILENAMES   Recent

(SEE SUBROUTINE FILEIO FOR DETAILS).   Recent

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Recent

INSURE PROGRAM WILL NOT DO ANYTHING    Recent

CRAZY.                                 Recent

*UPDATED TO USE NEW PROGRAM CONVERT     Recent

KEYWORDS.                              Recent

*CORRECTED MULTILEVEL, REICH-MOORE AND  Recent

HYBRID R-FUNCTION POTENTIAL SCATTER    Recent

TO ACCOUNT FOR REPEATED J-VALUES FOR   Recent

THE SAME TARGET SPIN AND L-VALUE.      Recent

*ADDED LIVERMORE CIVIC COMPILER         Recent

CONVENTIONS.                           Recent

*UPDATED TO USE NEW ENDF/B-VI           Recent

CONVENTION TO ALLOW UNRESOLVED         Recent

RESONANCE CONTRIBUTION TO ALREADY      Recent

BE INCLUDED IN THE FILE 3 CROSS        Recent

SECTIONS (INFINITELY DIULUTE           Recent

CONTRIBUTION).                         Recent

VERSION 90-1 (JUNE 1990)   *UPDATED BASED ON USER COMMENTS         Recent

*ADDED FORTRAN SAVE OPTION              Recent

*NEW MORE CONSISTENT ENERGY OUTPUT      Recent

ROUTINE                                Recent

VERSION 91-1 (JULY 1991)   *NEW UNIFORM TREATMENT OF ALL RESONANCE Recent

FORMALISMS (SEE, COMMENTS BELOW)       Recent

*NEW REICH-MOORE ALGORITHM              Recent

*MORE EXTENSIVE ERROR CHECKING AND      Recent

ERROR MESSAGE EXPLANATIONS             Recent

VERSION 92-1 (JANUARY 1992)*MAJOR RESTRUCTING TO IMPROVE ACCURACY  Recent

AND COMPUTER INDEPENDENCE.             Recent

*INCREASED ENERGY POINT PAGE SIZE FROM  Recent

1002 TO 4008.                          Recent

*NO MORE THAN 2 ENERGY POINTS WHERE     Recent

CROSS SECTION IS ZERO AT BEGINNING     Recent

OF A SECTION FOR EACH REACTION,E.G.,   Recent

THRESHOLD FISSION.                     Recent

*PROCESS ONLY A PORTION OF RESONANCE    Recent

REGION - SEE EXPLANATION BELOW         Recent

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Recent

TO CALCULATIONS.                       Recent

*COMPLETELY CONSISTENT I/O AND ROUNDING Recent

ROUTINES - TO MINIMIZE COMPUTER        Recent

DEPENDENCE.                            Recent

VERSION 93-1 (MARCH 1993)  *UPDATED REICH-MOORE TREATMENT TO USE   Recent

L DEPENDENT SCATTERING RADIUS (APL)    Recent

RATHER THAN SCATTERING RADIUS (AP)     Recent

(SEE, ENDF/B-VI FORMATS AND            Recent

PROCEDURES MANUAL, PAGE 2.6)          Recent

*INCREASED PAGE SIZE FROM 4008 TO       Recent

20040 DATA POINTS.                     Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 4008 TO 20040.                    Recent

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Recent

TO ALLOW ACCESS TO FILE STRUCTURES     Recent

(WARNING - INPUT PARAMETER FORMAT      Recent

HAS BEEN CHANGED).                     Recent

*CLOSE ALL FILES BEFORE TERMINATING     Recent

(SEE, SUBROUTINE ENDIT)                Recent

VERSION 94-2 (AUGUST 1994) *CORRECTED ADDJ FOR ENERGY DEPENDENT    Recent

(TABULATED) SCATTERING RADIUS CASE.    Recent

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Recent

*IMPROVED COMPUTER INDEPENDENCE        Recent

*ALL DOUBLE PRECISION                  Recent

*ON SCREEN OUTPUT                      Recent

*UNIFORM TREATMENT OF ENDF/B I/O       Recent

*IMPROVED OUTPUT PRECISION             Recent

*ALWAYS INCLUDE THERMAL VALUE          Recent

*DEFINED SCRATCH FILE NAMES            Recent

VERSION 97-1 (APRIL 1997)   *OPTIONAL MAKE NEGATIVE CROSS          Recent

SECTION = 0 FOR OUTPUT                Recent

*INCREASED PAGE SIZE FROM 20040 TO      Recent

120000 DATA POINTS.                    Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 20040 TO 120000.                  Recent

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Recent

POINT READ FOR MORE DIGITS            Recent

*UPDATED TEST FOR ENDF/B FORMAT        Recent

VERSION BASED ON RECENT FORMAT CHANGE Recent

*UPDATED CONSTANTS BASED ON CSEWG      Recent

SUBCOMMITTEE RECOMMENDATIONS          Recent

*GENERAL IMPROVEMENTS BASED ON         Recent

USER FEEDBACK                         Recent

VERSION 99-2 (JUNE 1999)    *IMPLEMENTED NEW REICH-MOORE FORMALISM Recent

TO ALLOW DEFINITION OF (L,J,S) FOR    Recent

EACH SEQUENCE.                        Recent

*ASSUME ENDF/B-VI, NOT V, IF MISSING   Recent

MF=1, MT-451.                         Recent

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Recent

USER FEEDBACK                         Recent

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Recent

(SEPT. 2002)   *OUTPUT RESONANCE WITH 9 DIGITS        Recent

*TO BE C AND C++ COMPATIBLE OUTPUT     Recent

VERS. 2004-1 (JAN. 2004)    *ADDED INCLUDE 'recent.h'              Recent

*MADE ENDF/B-VII READY                 Recent

*UPDATED FOR NEW REICH-MOORE LRF=7     Recent

PARAMETERS WITH COMPETITION           Recent

*ADDED COULOMB PENETRATION FACTORS FOR Recent

LRF=7 COMPETITIVE CHANNELS.           Recent

*EXTENDED DEFINITIONS OF PENETRATION   Recent

FACTOR, LEVEL SHIFT FACTOR, AND       Recent

POTENTIAL SCATTERING PHASE SHIFT      Recent

ABOVE L = 5 TO INFINITY.              Recent

*ADDED QUICK CALCULATION - IF THE      Recent

INPUT ALLOWABLE ERROR IS 1.0 OR MORE  Recent

(100 % OR MORE) THERE IS NO ITERATION Recent

TO CONVERGENCE - CROSS SECTION ARE    Recent

QUICKLY CALCULATED ONLY AT A FIXED    Recent

SET OF ENERGY POINTS, BASED ON THE    Recent

ENERGY AND WIDTH OF ALL RESONANCES.   Recent

THIS CAN BE USED TO QUICKLY "SEE"     Recent

NEW EVALUATIONS THAT MAY CONTAIN      Recent

ERRORS, THAT WOULD OTHERWISE CAUSE    Recent

THIS CODE TO RUN FOR AN EXCESSIVELY   Recent

LONG TIME.                            Recent

VERS. 2005-1 (JUNE 2005)    *ADDED ENERGY DEPENDENT SCATTERING     Recent

RADIUS FOR ALL RESONANCE TYPES        Recent

(EARLIER ONLY BREIT-WIGNER ALLOWED).  Recent

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Recent

*DECOUPLED PAGE SIZE FROM MAX. # OF    Recent

RESONANCES.                           Recent

*INCREASED PAGE SIZE FROM 120,000 TO   Recent

750,000 DATA POINTS.                  Recent

*KEPT MAX. # OF RESONANCE AT 120,000.  Recent

*CORRECTED ALL BACKGROUND = 0 CASE     Recent

Recent

OWNED, MAINTAINED AND DISTRIBUTED BY                               Recent

------------------------------------                               Recent

THE NUCLEAR DATA SECTION                                           Recent

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Recent

P.O. BOX 100                                                       Recent

A-1400, VIENNA, AUSTRIA                                            Recent

EUROPE                                                             Recent

Recent

ORIGINALLY WRITTEN BY                                              Recent

------------------------------------                               Recent

DERMOTT E. CULLEN                                                  Recent

UNIVERSITY OF CALIFORNIA                                           Recent

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Recent

L-159                                                              Recent

P.O. BOX 808                                                       Recent

LIVERMORE, CA 94550                                                Recent

U.S.A.                                                             Recent

TELEPHONE  925-423-7359                                            Recent

E. MAIL    CULLEN1@LLNL.GOV                                        Recent

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Recent

Recent

Acknowledgement (Version 2004-1)                                   Recent

================================================================== Recent

The author thanks Nancy Larson, ORNL, for providing her SAMRML     Recent

code for comparison to RECENT output for Reich-Moore evaluations,  Recent

in particular to verify results for the new LFR=7 evaluations. I   Recent

also thank her for providing guidance to help me understand and    Recent

implement this new teatment for Reich-Moore parameters.            Recent

Recent

ACKNOWLEDGEMENT (VERSION 92-1)                                     Recent

================================================================== Recent

THE AUTHOR THANKS SOL PEARLSTEIN (BROOKHAVEN NATIONAL LAB) FOR     Recent

SIGNIFICANTLY CONTRIBUTING TOWARD IMPROVING THE ACCURACY AND       Recent

COMPUTER INDEPENDENCE OF THIS CODE - THANKS, SOL                   Recent

================================================================== Recent

Recent

AUTHORS MESSAGE                                                    Recent

================================================================== Recent

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Recent

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Recent

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Recent

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Recent

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Recent

Recent

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Recent

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Recent

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Recent

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Recent

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Recent

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Recent

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Recent

COMPUTER.                                                          Recent

Recent

PURPOSE                                                            Recent

================================================================== Recent

THIS PROGRAM IS DESIGNED TO RECONSTRUCT THE RESONANCE CONTRIBUTION Recent

TO THE CROSS SECTION IN LINEARLY INTERPOLABLE FORM, ADD IN ANY     Recent

LINEARLY INTERPOLABLE BACKGROUND CROSS SECTION AND OUTPUT THE      Recent

RESULT IN THE ENDF/B FORMAT. THE CROSS SECTIONS OUTPUT BY THIS     Recent

PROGRAM WILL BE LINEARLY INTERPOLABLE OVER THE ENTIRE ENERGY RANGE Recent

Recent

THE RESONANCE CONTRIBUTION IS CALCULATED FOR TOTAL (MT=1),         Recent

ELASTIC (MT=2), CAPTURE (MT=102) AND FISSION (MT=18), ADDED        Recent

TO THE BACKGROUND (IF ANY) AND OUTPUT. IN ADDITION, IF THERE       Recent

IS A FIRST CHANCE FISSION (MT=19) BACKGROUND PRESENT THE RESONANCE Recent

CONTRIBUTION OF FISSION WILL BE ADDED TO THE BACKGROUND AND        Recent

OUTPUT. IF THERE IS NO FIRST CHANCE FISSION (MT=19) BACKGROUND     Recent

PRESENT THE PROGRAM WILL NOT OUTPUT MT=19.                         Recent

Recent

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Recent

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Recent

DISK OR ANY OTHER MEDIUM.                                          Recent

Recent

PROCESSING DATA IN THE ENDF/B-VI FORMAT                            Recent

================================================================== Recent

IT HAS NOW BEEN CONFIRMED (PRIVATE COMMUNICATION, CHARLES DUNFORD, Recent

APRIL, 1991) THAT THE PROPER PROCEDURE TO FOLLOW WHEN THERE ARE    Recent

MISSING OR DUPLICATE J VALUES IS TO IN ALL CASES ADD A SEQUENCE    Recent

WITH NO RESONANCES TO ACCOUNT FOR THE CONTRIBUTION OF THE SEQUENCE Recent

TO THE POTENTIAL SCATTERING CROSS SECTION.                         Recent

Recent

THIS IS THE PROCEDURE WHICH WAS FOLLOWED BY ALL VERSIONS OF RECENT Recent

SINCE 86-3 AND WILL CONTINUE TO BE THE PROCEDURE.                  Recent

Recent

INPUT ENDF/B FORMAT AND CONVENTIONS                                Recent

================================================================== Recent

ENDF/B FORMAT                                                      Recent

-------------                                                      Recent

THIS PROGRAM ONLY USES THE ENDF/B BCD OR LINE IMAGE FORMAT (AS     Recent

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Recent

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Recent

Recent

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Recent

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Recent

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Recent

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Recent

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Recent

AND ALL SECTIONS OF MF=2 AND 3 MUST BE CORRECT. THE PROGRAM COPIES Recent

ALL OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE  Recent

TO THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.         Recent

Recent

ENDF/B FORMAT VERSION                                              Recent

---------------------                                              Recent

THE FORMATS AND CONVENTIONS FOR READING AND INTERPRETING THE DATA  Recent

VARIES FROM ONE VERSION OF ENDF/B TO THE NEXT. HOWEVER, IF THE     Recent

HOLLERITH SECTION (MF=1, MT=451) IS PRESENT IT IS POSSIBLE FOR     Recent

THIS PROGRAM TO DISTINGUISH BETWEEN DATA IN THE ENDF/B-IV, V AND   Recent

VI FORMATS AND TO USE THE APPROPRIATE CONVENTIONS FOR EACH         Recent

ENDF/B VERSION (SEE, SUBROUTINE FILE1 FOR A DESCRIPTION OF HOW     Recent

THIS IS DONE). IF THE HOLLERITH SECTION IS NOT PRESENT THE         Recent

PROGRAM WILL ASSUME THE DATA IS IN THE ENDF/B-VI FORMAT AND USE    Recent

ALL CONVENTIONS APPROPRIATE TO ENDF/B-V. USERS ARE ENCOURAGED TO   Recent

INSURE THAT THE HOLLERITH SECTION (MF=1, MT=451) IS PRESENT IN     Recent

ALL EVALUATIONS.                                                   Recent

Recent

INPUT OF ENERGIES                                                  Recent

-----------------                                                  Recent

ALL ENERGIES ARE READ IN DOUBLE PRECISION (BY SPECIAL FORTRAN I/O  Recent

ROUTINES) AND ARE TREATED IN DOUBLE PRECISION IN ALL CALCULATIONS. Recent

Recent

OUTPUT ENDF/B FORMAT AND CONVENTIONS                               Recent

================================================================== Recent

CONTENTS OF OUTPUT                                                 Recent

------------------                                                 Recent

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE RECONSTRUCTED FILE     Recent

3 CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE        Recent

ALSO INCLUDED.                                                     Recent

Recent

DOCUMENTATION                                                      Recent

-------------                                                      Recent

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Recent

BY THE ADDITION OF COMMENT CARDS AT THE END OF EACH HOLLERITH      Recent

SECTION IN THE FORM                                                Recent

Recent

***************** RECENT (VERSION 2007-1) ***************          Recent

RESONANCE CONTRIBUTION RECONSTRUCTED TO WITHIN   0.100 PER-CENT    Recent

COMBINED DATA NOT THINNED (ALL RESONANCE + BACKGROUND DATA KEPT)   Recent

Recent

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR, SIGMA1 AND GROUPY) Recent

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Recent

THE DATA, INCLUDING WHICH VERSION OF EACH PROGRAM WAS USED.        Recent

Recent

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Recent

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Recent

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Recent

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Recent

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Recent

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Recent

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Recent

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Recent

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Recent

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Recent

Recent

REACTION INDEX                                                     Recent

--------------                                                     Recent

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Recent

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Recent

Recent

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Recent

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Recent

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Recent

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Recent

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Recent

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Recent

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Recent

Recent

OUTPUT FORMAT OF ENERGIES                                          Recent

-------------------------                                          Recent

IN THIS VERSION OF RECENT ALL FILE 3 ENERGIES WILL BE OUTPUT IN    Recent

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Recent

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Recent

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Recent

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Recent

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Recent

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Recent

JUST DUE TO TRANSLATION OF ENERGIES FROM THEIR INTERNAL (BINARY)   Recent

REPRESENTATION TO THE ENDF/B FORMAT.                               Recent

Recent

ACCURACY OF ENERGY                                                 Recent

------------------                                                 Recent

IN ORDER TO ALLOW ENERGIES TO BE ACCURATELY OUTPUT TO 9 DIGITS     Recent

ON SHORT WORD LENGTH COMPUTERS (E.G. IBM) ALL ENERGIES AND         Recent

ENERGY DEPENDENT TERMS ARE READ AND TREATED IN DOUBLE PRECISION.   Recent

Recent

OUTPUT OF RESONANCE PARAMETERS                                     Recent

------------------------------                                     Recent

A SPECIAL CONVENTION HAS BEEN INTRODUCED REGARDING RESONANCE       Recent

PARAMETERS. IN ORDER TO ALLOW THE USER TO DOPPLER BROADEN AND/OR   Recent

SELF-SHIELD CROSS SECTIONS THE RESONANCE PARAMETERS ARE ALSO       Recent

INCLUDED IN THE OUTPUT WITH THE EVALUATION. IN ORDER TO AVOID THE  Recent

POSSIBILITY OF ADDING THE RESONANCE CONTRIBUTION A SECOND TIME     Recent

TWO CONVENTIONS HAVE BEEN ADOPTED TO INDICATE THAT THE RESONANCE   Recent

CONTRIBUTION HAS ALREADY BEEN ADDED TO THE FILE 3 CROSS SECTIONS,  Recent

Recent

(1) WHEN THE DATA IS PROCESSED BY THIS PROGRAM LRP (IN MF=1,       Recent

MT=451) IS SET EQUAL TO 2. THIS IS A CONVENTION WHICH HAS BEEN     Recent

ADOPTED AS A STANDARD CONVENTION IN ENDF/B-VI, BUT IS ONLY TO BE   Recent

USED FOR PROCESSED DATA, AS OPPOSED TO THE ORIGINAL EVALUATIONS.   Recent

IN EVALUATIONS WHICH CONTAIN MF=1, MT=451 LRP CAN BE USED TO       Recent

DETERMINE IF THE MATERIAL HAS BEEN PROCESSED.                      Recent

Recent

(2) THE LRU FLAG IN EACH SECTION OF FILE 2 DATA IS CHANGED TO      Recent

LRU=LRU+3. FOR EXAMPLE WHEN READING AN ENDF/B EVALUATION LRU=0     Recent

(NO RESONANCES), =1 (RESOLVED) OR =2 (UNRESOLVED) INDICATES THAT   Recent

THE DATA IS IN THE ORIGINAL ENDF/B FORM. LRU=3 (NO RESONANCES),    Recent

=4 (RESOLVED) OR =5 (UNRESOLVED) INDICATES THAT THE RESONANCE      Recent

CONTRIBUTION HAS ALREADY BEEN ADDED TO THE FILE 3 DATA. THIS       Recent

SECOND CONVENTION HAS BEEN ADOPTED AS INSURANCE THAT THE RESONANCE Recent

CONTRIBUTION WILL NOT BE ADDED TWICE, EVEN FOR EVALUATIONS WHICH   Recent

DO NOT CONTAIN MF=1, MT=451 (EVALUATIONS WHICH CONTAIN MF=1,       Recent

MT=451 ARE COVERED BY CONVENTION (1), DESCRIBED ABOVE).            Recent

Recent

UNIFORM TREATMENT OF RESONANCE FORMALISMS                          Recent

================================================================== Recent

NORMALIZATION                                                      Recent

=============                                                      Recent

ALL OF THE RESONANCE FORMALISMS INCLUDE A FACTOR OF,               Recent

Recent

PI*(FRACTIONAL ABUNDANCE)/(K**2)                                   Recent

Recent

THIS FACTOR HAS BEEN REMOVED FROM THE CALCULATION OF EACH TYPE     Recent

OF RESONANCE FORMALISM AND IS APPLIED AS A FINAL NORMALIZATION     Recent

AFTER THE CALCULATION, ONLY ONE PLACE IN THIS PROGRAM.             Recent

Recent

FOR SIMPLICITY THIS TERM IS NOT INCLUDED IN THE FOLLOWING          Recent

DERIVATIONS - IN ALL CASES THE ACTUAL CROSS SECTION IS A PRODUCT   Recent

OF THE ABOVE FACTOR TIMES THE RESULTS PRESENTED BELOW.             Recent

Recent

SIMILARITIES                                                       Recent

============                                                       Recent

FOR THE RESOLVED RESONANCE REGION, EXCEPT FOR SINGLE LEVEL BREIT   Recent

WIGNER, PARAMETERS ALL OF THE FORMALISMS DEFINE THE CROSS SECTIONS Recent

IN AN EQUIVALENT FORM,                                             Recent

Recent

TOTAL    = 2*GJ*REAL(1 - U)                                        Recent

= 2*GJ*(1 - REAL(U))                                      Recent

ELASTIC  =   GJ*(1 - U)**2                                         Recent

=   GJ*((1 - 2*REAL(U)) + (REAL(U)**2 + IM(U)**2))        Recent

= 2*GJ*(1 - REAL(U)) - GJ*(1 - (REAL(U)**2 + IM(U)**2))   Recent

Recent

SINCE THE FIRST TERM IS THE TOTAL, THE SECOND TERM MUST BE         Recent

ABSORPTION. SO WE FIND,                                            Recent

Recent

ABSORPTION = GJ*(1 - (REAL(U)**2 + IM(U)**2))                      Recent

Recent

IN ALL CASES U IS DEFINED IN THE FORM,                             Recent

Recent

U        = EXP(-I*2*PS)*((1-X) - I*Y)                              Recent

Recent

WHERE (X) AND (Y) ARE RELATED TO THE SYMMETRIC AND ANTI-SYMMETRIC  Recent

CONTRIBUTIONS OF THE RESONANCES, RESPECTIVELY. ONLY THE DEFINITION Recent

OF (X) AND (Y) WILL BE DIFFERENT FOR EACH RESONANCE FORMALISM.     Recent

BELOW WE WILL SHOW THAT WHAT MIGHT APPEAR TO BE A STRANGE CHOICE   Recent

OF DEFINITION OF THE SIGN OF (X) AND(Y) HAS BEEN SELECTED SO THAT  Recent

FOR BREIT-WIGNER PARAMETERS (X) AND (Y) CORRESPOND EXACTLY TO THE  Recent

SYMMETRIC AND ANTI-SYMMETRIC CONTRIBUTION OF THE RESONANCES.       Recent

Recent

U        = (COS(2*PS) - I*SIN(2*PS))*((1-X) - I*Y)                 Recent

=   ((1-X)*COS(2*PS) - Y*SIN(2*PS))                       Recent

=-I*((1-X)*SIN(2*PS) + Y*COS(2*PS))                       Recent

Recent

REAL(U)  = ((1-X)*COS(2*PS) - Y*SIN(2*PS))                         Recent

IM(U)    =-((1-X)*SIN(2*PS) + Y*COS(2*PS))                         Recent

Recent

R(U)**2  =((1-X)*COS(2*PS))**2 + (Y*SIN(2*PS))**2                  Recent

-2*(1-X)*Y*COS(2*PS)*SIN(2*PS)                           Recent

I(U)**2  =((1-X)*SIN(2*PS))**2 + (Y*COS(2*PS))**2                  Recent

+2*(1-X)*Y*COS(2*PS)*SIN(2*PS)                           Recent

Recent

THE TERMS 2*(1-X)*Y*COS(2*PS)*SIN(2*PS) CANCEL AND UPON USING      Recent

THE IDENTITY COS(2*PS)**2 + SIN(2*PS)**2 = 1,                      Recent

Recent

SUM      = (1-X)**2 + (Y)**2                                       Recent

Recent

WE NOW HAVE ALL THE QUANTITIES THAT WE NEED TO DEFINE THE CROSS    Recent

SECTIONS,                                                          Recent

Recent

ELASTIC                                                            Recent

=======                                                            Recent

ELASTIC  =GJ*(1 - 2*REAL(U) + (REAL(U)**2 + IM(U)**2))             Recent

=GJ*(1 - 2*((1-X)*COS(2*PS)-Y*SIN(2*PS))+(1-X)**2+(Y)**2) Recent

Recent

THIS CAN BE WRITTEN AS A SUM OF 2 SQUARES,                         Recent

Recent

ELASTIC  =GJ*(COS(2*PS) - (1-X))**2 + (SIN(2*PS) + Y)**2)          Recent

Recent

=GJ*((COS(2*PS))**2 - 2*(1-X)*COS(2*PS) + (1-X)**2) +     Recent

(SIN(2*PS))**2 + 2*Y*SIN(2*PS)    + (Y)**2)          Recent

Recent

AGAIN USING THE IDENTITY COS(2*PS)**2 + SIN(2*PS)**2 = 1, WE CAN   Recent

SEE THAT THE DEFINITION AS THE SUM OF 2 SQUARES IS IDENTICAL TO    Recent

THE PRECEDING DEFINITION OF THE ELASTIC.                           Recent

Recent

ELASTIC  =GJ*(COS(2*PS) - (1-X))**2 + (SIN(2*PS) + Y)**2)          Recent

=GJ*((COS(2*PS)-1) + X)**2 + (SIN(2*PS) + Y)**2)          Recent

Recent

USING THE IDENTITY (1 - COS(2*PS))) = 2*SIN(PS)**2, WE OBTAIN      Recent

THE FINAL FORM FOR THE ELASTIC,                                    Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 - X)**2 + (SIN(2*PS) + Y)**2)            Recent

Recent

ABSORPTION                                                         Recent

==========                                                         Recent

ABSORPTION = GJ*(1 - (REAL(U)**2 + IM(U)**2))                      Recent

= GJ*(1 - ((1-X)**2   + (Y)**2)                         Recent

= GJ*(1 - (1 - 2*X + (X)**2 + (Y)**2)                   Recent

= GJ*(2*X - (X)**2 + (Y)**2)                            Recent

Recent

SINCE PHYSICALLY THE ABSORPTION CANNOT BE NEGATIVE WE CAN SEE      Recent

THAT (X) MUST BE POSITIVE AND 2*X MUST BE GREATER THAN             Recent

(X)**2 + (Y)**2, FOR ALL OF THE FORMALISMS.                        Recent

Recent

TOTAL                                                              Recent

=====                                                              Recent

IN THIS PROGRAM THE TOTAL CROSS SECTION IS ALWAYS DEFINED TO BE    Recent

THE SUM OF ITS PARTS - SO THE ABOVE DEFINITION IS NEVER EXPLICITLY Recent

USED. HOWEVER, WE CAN LEARN SOMETHING BY EXAMINING THE DEFINITION, Recent

Recent

TOTAL    = 2*GJ*REAL(1 - U)                                        Recent

= 2*GJ*(1 - (((1-X)*COS(2*PS) - Y*SIN(2*PS)))             Recent

= 2*GJ*((1 - COS(2*PS))*(1-X) - (1-X) + Y*SIN(2*PS))      Recent

= 2*GJ*(2*SIN(PS)**2*(1-X)    - (1-X) + Y*SIN(2*PS))      Recent

Recent

= 4*GJ*SIN(PS)**2 +                                       Recent

2*GJ*((X-1) - 2*X*SIN(PS)**2 +  Y*SIN(2*PS))            Recent

Recent

THE IMPORTANT POINT TO NOTE IS THAT THE DEFINITION OF THE TOTAL    Recent

DOES NOT EXPLICITLY CONTAIN ANY DEPENDENCE ON X**2 AND Y**2 -      Recent

THE LEVEL-LEVEL INTERFERENCE TERMS.                                Recent

Recent

THIS IMPLIES THAT IF A GIVEN SET OF RESONANCE PARAMETERS ARE USED  Recent

WITH THIS DEFINITION THEY WILL PRODUCE EXACTLY THE SAME TOTAL      Recent

CROSS SECTION - WHETHER WE CLAIM THE PARAMETERS HAVE BEEN          Recent

PRODUCED USING A SINGLE OR MULTI-LEVEL FIT. THIS RESULT COULD      Recent

BE VERY MISLEADING, IF THIS RESULT FOR THE TOTAL IS IMPLIED TO     Recent

MEAN THAT ONE INTERPRETATION OR THE OTHER WILL NOT HAVE ANY        Recent

EFFECT ON THE INDIVIDUAL CROSS SECTIONS.                           Recent

Recent

STARTING FROM EXACTLY THE SAME RESONANCE PARAMETERS, RELATIVE TO   Recent

THE RESULTS OBTAINED USING THE SINGLE LEVEL FORMULA, MULTI-LEVEL   Recent

RESULTS WILL TEND TO ALWAYS DECREASE THE ABSORPTION AND INCREASE   Recent

THE ELASTIC. THIS CAN BE IMMEDIATELY SEEN FROM OUR GENERAL         Recent

MULTI-LEVEL DEFINITION OF ABSORPTION,                              Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

Recent

THE SINGLE LEVEL ABSORPTION IS,                                    Recent

Recent

ABSORPTION =GJ*(2*X)                                               Recent

Recent

THE DIFFERENCE BETWEEN THE TWO IS -2*GJ*(X**2 + Y**2), SO THAT     Recent

REGARDLESS OF HOW WE DEFINE (X) AND (Y) THE INCLUSION OF THIS      Recent

TERM WILL ALWAYS DECREASE ABSORPTION. SINCE THE TOTAL CROSS        Recent

SECTION IS THE SAME IN BOTH CASE, THIS MEANS THAT THE ELASTIC      Recent

HAS BEEN INCREASED BY THIS AMOUNT.                                 Recent

Recent

AGAIN, THESE RESULTS ARE BASED ON STARTING FROM EXACTLY THE SAME   Recent

PARAMETERS - IN ANY ACTUAL CASE THE PARAMETERS BASED ON A SINGLE   Recent

OR MULTI-LEVEL FIT WILL BE QUITE DIFFERENT - THE POINT THAT WE     Recent

WANT TO STRESS HERE IS THAT YOU SHOULD NEVER USE PARAMETERS        Recent

WHICH HAVE BEEN DEFINED BY A FIT USING ONE FORMALISM - IN THE      Recent

EQUATIONS FOR A DIFFERENT FORMALISM - AND ASSUME THAT THE RESULTS  Recent

WILL BE CONSISTENT - AND NEVER USE THE TOTAL CROSS SECTION TO      Recent

SEE WHETHER OR NOT A SET OF SINGLE LEVEL PARAMETERS CAN BE USED    Recent

WITH A MULTI-LEVEL FORMALISM.                                      Recent

Recent

POTENTIAL CROSS SECTION                                            Recent

=======================                                            Recent

FAR FROM RESONANCES (X) AND (Y) WILL BE SMALL AND THE ELASTIC      Recent

CROSS SECTION REDUCES TO,                                          Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2)**2     + (SIN(2*PS))**2                 Recent

=GJ*4*(SIN(PS)**4         + SIN(2*PS)**2                   Recent

Recent

USING THE IDENTITY SIN(2*PS) = 2*SIN(PS)*COS(PS)                   Recent

Recent

=4*GJ*(SIN(PS)**4         + (SIN(PS)*COS(PS))**2)          Recent

=4*GJ*SIN(PS)**2*(SIN(PS)**2 + COS(PS)**2)                 Recent

=4*GJ*SIN(PS)**2                                           Recent

Recent

WHICH IS THE POTENTIAL CROSS SECTION. NOTE THAT THIS RESULT IS     Recent

INDEPENDENT OF THE FORMALISM USED, AS IT MUST PHYSICALLY BE,       Recent

AND AS SUCH ALTHOUGH AS YET WE HAVE NOT DEFINED IT, WE CAN         Recent

NOW SEE THAT IN ALL CASES (PS) MUST BE THE PHASE SHIFT AND FOR     Recent

CONSISTENCY IT MUST BE DEFINED USING EXACTLY THE SAME DEFINITION   Recent

IN ALL CASES.                                                      Recent

Recent

IN ADDITION SINCE PHYSICALLY FOR EACH L VALUE WE EXPECT TO OBTAIN  Recent

A POTENTIAL CROSS SECTION,                                         Recent

Recent

4*(2*L+1)*SIN(PS)**2                                               Recent

Recent

OBVIOUSLY FOR CONSISTENCY WE MUST HAVE,                            Recent

Recent

(2*L+1) = (SUM OVER J) GJ                                          Recent

Recent

ONLY IN THIS CASE WILL THE RESULTS BE CONSISTENT - THIS POINT WILL Recent

BE DISCUSSED IN DETAIL BELOW.                                      Recent

Recent

WHAT ARE THIS TERMS (X) AND (Y)                                    Recent

===============================                                    Recent

(X) AND (Y) CAN BE EASILY IDENTIFIED BY CONSIDERING THE SINGLE     Recent

AND MULTI-LEVEL BREIT WIGNER FORMALISMS. IN THESE CASES WE WILL    Recent

FIND THAT,                                                         Recent

Recent

X        = GAM(N)*GAM(T)/2/DEN                                     Recent

Y        = GAM(N)*(E-ER)/DEN                                       Recent

DEN      = ((E-ER)**2 + (GAM(T)/2)**2)                             Recent

Recent

EXTREME CARE HAS TO BE USED TO PROPERLY DEFINE (Y) SUCH THAT IT    Recent

IS NEGATIVE FOR E LESS THAN ER AND POSITIVE FOR E GREATER THAN     Recent

ER. I WILL MERELY MENTION THAT THE EQUATIONS FOR ALL FORMALISMS    Recent

IN ENDF-102 DO NOT CONSISTENTLY USE (E - ER) - IN SOME CASES       Recent

THIS IS WRITTEN AS (ER - E), WHICH CAN LEAD TO AN INCORRECT        Recent

SIGN IN THE DEFINITION OF THE (Y) THAT WE REQUIRE.                 Recent

Recent

THE INTERFERENCE TERMS CAN BE WRITTEN IN TERMS OF,                 Recent

1) LEVEL-SELF INTERFERENCE  = THE CONTRIBUTION OF EACH LEVEL       Recent

INTERFERRING WITH ITSELF             Recent

2) LEVEL-LEVEL INTERFERENCE = THE CONTRIBUTION OF EACH LEVEL       Recent

INTERFERRRING WITH ALL OTHER LEVELS  Recent

Recent

WE WILL REFER TO THESE TWO AS (L-S) AND (L-L),                     Recent

Recent

X**2     = (GAM(N)*(GAM(T)/2)**2/(DEN)**2      + (L-L)             Recent

= (GAM(N)**2*((GAM(T)/2)**2)/(DEN)**2 + (L-L)             Recent

Y**2     = (GAM(N))**2*((E-ER))**2/(DEN)**2    + (L-L)             Recent

Recent

X**2+Y**2= GAM(N)**2*DEN/(DEN)**2 = GAM(N)**2/DEN + (L-L)          Recent

Recent

TO SEE THE EFFECT OF INCLUDING MULTI-LEVEL INTERFERENCE WE CAN     Recent

CONSIDER OUR GENERAL EXPRESSION FOR ABSORPTION,                    Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

Recent

AND NOTE THAT FOR BOTH SINGLE AND MULTI-LEVEL BREIT WIGNER THE     Recent

ENDF-102 SAYS TO TREAT ABSORPTION IN A SINGLE LEVEL APPROXIMATION  Recent

I.E., IGNORE LEVEL-LEVEL INTERFERENCE. IF ALL INTERFERENCE IS      Recent

IGNORED THIS IS EQUIVALENT TO COMPLETELY IGNORING X**2 + Y**2 AND  Recent

DEFINING,                                                          Recent

Recent

ABSORPTION =GJ*2*X                                                 Recent

=2*GJ*GAM(N)*GAM(T)/DEN                                 Recent

Recent

WHICH IS INCORRECT - SINCE THIS SEEMS TO INDICATE EVERYTHING IS    Recent

ABSORBED. IN ORDER TO OBTAIN THE CORRECT EXPRESSION WE CANNOT      Recent

COMPLETELY IGNORE INTERFERENCE - WE CAN IGNORE LEVEL-LEVEL         Recent

INTERFERENCE, BUT WE MUST INCLUDE LEVEL-SELF INTERFERENCE,         Recent

Recent

X**2+Y**2= GAM(N)**2/DEN                                           Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

=GJ*GAM(N)*(GAM(T)-GAM(N))/DEN                          Recent

=GJ*GAM(N)*GAM(A)/DEN                                   Recent

Recent

SUMMARY                                                            Recent

=======                                                            Recent

AN IMPORTANT POINT TO NOTE IS THE DEFINITION OF (X) AND (Y)        Recent

WHICH IN ALL CASES WILL CORRESPOND TO THE SYMMETRIC AND            Recent

ANTI-SYMMETRIC CONTRIBUTION OF THE RESONANCES. IN PARTICULAR       Recent

DEFINING (U) IN TERMS OF (1-X) INSTEAD OF (X) IS EXTREMELY         Recent

IMPORTANT. NOTE, THAT THE DEFINITION OF THE ELASTIC AND            Recent

ABSORPTION ONLY INVOLVE (X), NOT (1-X). FAR FROM RESONANCES        Recent

(X) CAN BE EXTREMELY SMALL, THEREFORE (1-X) WILL BE VERY CLOSE     Recent

TO (1). IF THE CALCULATION PROCEEDS BY FIRST CALCULATING (1-X)     Recent

AND THEN DEFINING (X) BY SUBTRACTING (1), EXTREME ROUND-OFF        Recent

PROBLEMS CAN RESULT. THESE PROBLEMS CAN BE AVOIDED BY IN ALL       Recent

CASES DEFINING (X) DIRECTLY, WITHOUT ANY DIFFERENCES.              Recent

Recent

IN EACH FORMALISM THE DEFINITION OF (X) AND (Y) MAY BE DIFFERENT   Recent

BUT ONCE WE HAVE DEFINED (X) AND (Y) WE CAN IMMEDIATELY WRITE      Recent

THE CROSS SECTIONS USING A UNIFORM DEFINITION,                     Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 - X)**2 + (SIN(2*PS) + Y)**2)            Recent

Recent

ABSORPTION =-GJ*(2*X + (X)**2 + (Y)**2)                            Recent

Recent

AND DEFINE THE TOTAL AS THE SUM OF THESE 2 PARTS.                  Recent

Recent

RELATIONSHIP TO SINGLE LEVEL                                       Recent

============================                                       Recent

HOW DO THE SINGLE AND MULTI-LEVEL FORMALISMS COMPARE. TO SEE,      Recent

STARTING FROM OUR GENERAL DEFINITION OF THE ELASTIC IN THE FORM,   Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 + X)**2 + (SIN(2*PS) + Y)**2)            Recent

=GJ*(4*SIN(PS)**4 - 4*X*SIN(PS)**2 + X**2                  Recent

+ SIN(2*PS)**2 + 2*Y*SIN(2*PS)  + Y**2)                 Recent

Recent

=4*GJ*SIN(PS)**2 +                                         Recent

GJ*(X**2 + Y**2                                         Recent

-4*X*SIN(PS)**2                                      Recent

+2*Y*SIN(2*PS))                                      Recent

Recent

AND OUR SPECIFIC DEFINITIONS OF (X) AND (Y) FOR MULTI-LEVEL BREIT- Recent

WIGNER PARAMETERS,                                                 Recent

Recent

X        = GAM(N)*GAM(T)/2/DEN                                     Recent

Y        = GAM(N)*(E-ER)/DEN                                       Recent

DEN      = ((E-ER)**2 + (GAM(T)/2)**2)                             Recent

Recent

X**2+Y**2= GAM(N)**2/DEN + (L-L)                                   Recent

Recent

WE CAN RECOGNIZE X**2 AND Y**2 AS THE INTERFERENCE - (L-S) + (L-L) Recent

TERMS IN THE MULTI-LEVEL FORMALISM. IN ORDER TO OBTAIN THE SINGLE  Recent

LEVEL EQUATION WE CAN ASSUME THAT EACH LEVEL DOES NOT INTERFERE    Recent

WITH ANY OTHER LEVEL - THEREFORE THE (L-L) CONTRIBUTION IS ZERO.   Recent

Recent

ELASTIC =4*GJ*SIN(PS)**2 +                                         Recent

GJ*GAM(N)*(GAM(N)                                       Recent

-2*GAM(T)*SIN(PS)**2                         Recent

+2*(E-ER)*SIN(2*PS))/DEN                     Recent

Recent

WHICH IS THE FORM THAT IT APPEARS IN ENDF-102, EXCEPT FOR TWO      Recent

TYPOGRAPHICAL ERRORS IN THE SECOND TERM,                           Recent

Recent

-2*GAM(T)*SIN(PS)**2                                               Recent

Recent

WHICH IN ENDF-102 IS WRITTEN,                                      Recent

Recent

-2*(GAM(T)-GAM(N))*SIN(2*PS)**2                                    Recent

Recent

PROGRAM CONVENTIONS                                                Recent

================================================================== Recent

MINIMUM INPUT DATA                                                 Recent

------------------                                                 Recent

FOR EACH MATERIAL TO BE PROCESSED THE MINIMUM INPUT DATA ARE THE   Recent

RESONANCE PARAMETERS IN FILE 2. IF THERE ARE NO FILE 2 PARAMETERS  Recent

IN A GIVEN MATERIAL THE ENTIRE MATERIAL WILL SIMPLY BE COPIED.     Recent

NEITHER THE HOLLERITH SECTION (MF=1, MT=451) NOR THE BACKGROUND    Recent

CROSS SECTION (SECTIONS OF MF=3) NEED BE PRESENT FOR THIS PROGRAM  Recent

TO EXECUTE PROPERLY. HOWEVER, SINCE THE CONVENTIONS USED IN        Recent

INTERPRETING THE RESONANCE PARAMETERS DEPENDS ON ENDF/B VERSION    Recent

USERS ARE STRONGLY RECOMMENDED TO INSURE THAT MF=1, MT=451 IS      Recent

PRESENT IN EACH MATERIAL TO ALLOW THE PROGRAM TO DETERMINE THE     Recent

ENDF/B FORMAT VERSION.                                             Recent

Recent

RESONANCE PARAMETERS                                               Recent

--------------------                                               Recent

RESONANCE PARAMETERS MAY BE REPRESENTED USING ANY COMBINATION      Recent

OF THE REPRESENTATIONS ALLOWED IN ENDF/B,                          Recent

(1) RESOLVED DATA                                                  Recent

(A) SINGLE LEVEL BREIT-WIGNER                                  Recent

(B) MULTI-LEVEL BREIT-WIGNER                                   Recent

(C) ADLER-ADLER                                                Recent

(D) REICH-MOORE                                                Recent

(E) HYBRID R-FUNCTION                                          Recent

(2) UNRESOLVED DATA                                                Recent

(A) ALL PARAMETERS ENERGY INDEPENDENT                          Recent

(B) FISSION PARAMETERS ENERGY DEPENDENT                        Recent

(C) ALL PARAMETERS ENERGY DEPENDENT                            Recent

Recent

THE FOLLOWING RESOLVED DATA FORMALISMS ARE NOT TREATED BY THIS     Recent

VERSION OF THE CODE AND WILL ONLY BE IMPLEMENTED AFTER EVALUATIONS Recent

USING THESE FORMALISMS ARE AVAILABLE TO THE AUTHOR OF THIS CODE    Recent

FOR TESTING IN ORDER TO INSURE THAT THEY CAN BE HANDLED PROPERLY   Recent

(A) GENERAL R-MATRIX                                           Recent

Recent

CALCULATED CROSS SECTIONS                                          Recent

-------------------------                                          Recent

THIS PROGRAM WILL USE THE RESONANCE PARAMETERS TO CALCULATE THE    Recent

TOTAL, ELASTIC, CAPTURE AND POSSIBLY FISSION CROSS SECTIONS. THE   Recent

COMPETITIVE WIDTH WILL BE USED IN THESE CALCULATIONS, BUT THE      Recent

COMPETITIVE CROSS SECTION ITSELF WILL NOT BE CALCULATED. THE       Recent

ENDF/B CONVENTION IS THAT ALTHOUGH A COMPETITIVE WIDTH MAY BE      Recent

GIVEN, THE COMPETITIVE CROSS SECTION MUST BE SEPARATELY TABULATED  Recent

AS A SECTION OF FILE 3 DATA.                                       Recent

Recent

RESOLVED REGION                                                    Recent

---------------                                                    Recent

IN THE RESOLVED REGION THE RESOLVED PARAMETERS ARE USED TO         Recent

CALCULATE COLD (0 KELVIN), LINEARLY INTERPOLABLE, ENERGY DEPENDENT Recent

CROSS SECTIONS.                                                    Recent

Recent

SCATTERING RADIUS                                                  Recent

-----------------                                                  Recent

FOR SINGLE OR MULTI LEVEL BREIT-WIGNER PARAMETERS THE SCATTERING   Recent

RADIUS MAY BE SPECIFIED IN EITHER ENERGY INDEPENDENT (CONSTANT)    Recent

OR ENERGY DEPENDENT FORM (A TABLE OF ENERGY VS. RADIUS AND AN      Recent

ASSOCIATED INTERPOLATION LAW). IN ALL OTHER CASE ONLY AN ENERGY    Recent

INDEPENDENT SCATTERING RADIUS IS ALLOWED.                          Recent

Recent

FOR ANY ONE MATERIAL (I.E. MAT) IF ENERGY DEPENDENT SCATTERING     Recent

RADII ARE GIVEN THE TOTAL NUMBER OF INTERPOLATION REGIONS AND      Recent

TABULATED VALUES FOR THE ENTIRE MATERIAL CANNOT EXCEED,            Recent

200 - INTERPOLATION REGIONS                                        Recent

500 - TABULATED VALUES                                             Recent

IF THESE LIMITS ARE EXCEEDED THE PROGRAM WILL PRINT AN ERROR       Recent

MESSAGE AND TERMINATE.                                             Recent

Recent

IF YOU REQUIRE A LARGER NUMBER OF INTERPOLATION REGION AND/OR      Recent

TABULATED VALUES,                                                  Recent

(1) INTERPOLATION REGIONS - INCREASE THE DIMENSION OF NBTRHO AND   Recent

INTRHO IN COMMON/TABRHO/ THROUGHOUT THE PROGRAM AND CHANGE MAXSEC  Recent

IN SUBROUTINE RDAP (MAXSEC = MAXIMUM NUMBER OF INTERPOLATION       Recent

REGIONS).                                                          Recent

(2) TABULATED VALUES - INCREASE THE DIMENSION OF ERHOTB, RHOTAB    Recent

AND APTAB IN COMMON/TABRHO/ THROUGHOUT THE PROGRAM AND CHANGE      Recent

MAXRHO IN SUBROUTINE RDAP (MAXRHO = MAXIMUM NUMBER OF TABULATED    Recent

VALUES).                                                           Recent

Recent

RESOLVED REICH-MOORE AND MULTI-LEVEL BREIT-WIGNER PARAMETERS       Recent

------------------------------------------------------------       Recent

CROSS SECTIONS FOR REICH-MOORE PARAMETERS ARE CALCULATED ACCORDING Recent

TO THE EQUATION (1) - (8) OF SECTION D.1.3 OF ENDF-102. IN ORDER   Recent

TO CALCULATE CROSS SECTIONS FROM MULTI-LEVEL PARAMETERS IN A       Recent

REASONABLE AMOUNT OF TIME THIS PROGRAM EXPRESSES THE CROSS SECTION Recent

IN TERMS OF A SINGLE SUM OVER RESONANCES (SEE, ENDF-102, SECTION   Recent

D.1.2, EQUATIONS 6-7), RATHER THAN AS A DOUBLE SUM (SEE, ENDF-102  Recent

SECTION D.1.2, EQUATION 1-2). IN ORDER FOR THE ENDF-102 EQUATIONS  Recent

TO BE CORRECT THE PARAMETERS MUST MEET THE FOLLOWING CONDITIONS,   Recent

Recent

(1) FOR EACH L STATE ALL PHYSICALLY POSSIBLE J SEQUENCES MUST BE   Recent

PRESENT. ONLY IN THIS CASE WILL THE CONTRIBUTIONS OF THE       Recent

INDIVIDUAL J SEQUENCES ADD UP TO PRODUCE THE CORRECT POTENTIAL Recent

SCATTERING CONTRIBUTION FOR THE L STATE (SEE, ENDF-102,        Recent

SECTION D.1.2, EQUATIONS 6-7). IF ANY J SEQUENCE IS MISSING    Recent

THE PROGRAM WILL PRINT A WARNING AND ADD THE J SEQUENCE WITH   Recent

NO RESONANCE PARAMETERS IN ORDER TO ALLOW THE POTENTIAL        Recent

SCATTERING TO BE CALCULATED CORRECTLY (THIS IS EQUIVALENT TO   Recent

ASSUMING THAT THE EVALUATOR REALIZES THAT ALL J SEQUENCES MUST Recent

BE AND ARE PRESENT AND THAT THE EVALUATION STATES THAT THERE   Recent

ARE NO RESONANCES WITH CERTAIN PHYSICALLY POSSIBLE J VALUES... Recent

IN THIS CASE POTENTIAL CONTRIBUTION MUST STILL BE CONSIDERED). Recent

Recent

EXAMPLE                                                        Recent

=======                                                        Recent

AN EXAMPLE OF WHERE THIS OCCURS AND IS IMPORTANT TO CONSIDER   Recent

IS U-238 IN ENDF/B-IV AND V LIBRARIES WHERE FOR L=1 THERE IS   Recent

ONLY A J=1/2 SEQUENCE. NOT INCLUDING THE J=3/2 SEQUENCE LEADS  Recent

TO UNDERESTIMATING THE POTENTIAL SCATTERING AND PRODUCES       Recent

MINIMA IN THE ELASTIC CROSS SECTION WHICH ARE AN ORDER OF      Recent

MAGNITUDE LOWER THAN THE CROSS SECTIONS OBTAINED BE INCLUDING  Recent

THE J=3/2 SEQUENCE.                                            Recent

Recent

(2) FOR A GIVEN TARGET SPIN AND L VALUE THERE MAY BE 2 POSSIBLE    Recent

MEANS OF OBTAINING THE SAME J VALUE. WHEN THIS OCCURS IN       Recent

ORDER TO CALCULATE THE CORRECT POTENTIAL SCATTERING CROSS      Recent

SECTION IT IS IMPORTANT TO INCLUDE THE EFFECT OF BOTH          Recent

POSSIBLE J SEQUENCES, EVEN THOUGH FROM THE ENDF/B DATA IT IS   Recent

NOT POSSIBLE TO DETERMINE WHICH OF THE 2 POSSIBLE SEQUENCES    Recent

ANY GIVEN RESONANCE BELONGS TO. IN THIS CASE THIS PROGRAM      Recent

TREAT ALL RESONANCES WITH THE SAME J VALUE AS BELONGING TO     Recent

THE SAME J SEQUENCE (TO ALLOW INTERFERENCE) AND WILL ADD AN    Recent

ADDITIONAL J SEQUENCE WITH NO RESONANCES IN ORDER TO ALLOW     Recent

THE POTENTIAL CROSS SECTION TO BE CALCULATED CORRECTLY. WHEN   Recent

THIS OCCURS A WARNING MESSAGE IS PRINTED, BUT BASED ON THE     Recent

ENDF/B DATA THERE IS NOTHING WRONG WITH THE DATA AND THERE IS  Recent

NOTHING THAT THE USER CAN DO TO CORRECT OR IN ANY WAY MODIFY   Recent

THE DATA TO ELIMINATE THE PROBLEM.                             Recent

Recent

EXAMPLE                                                        Recent

=======                                                        Recent

FOR A TARGET SPIN =1 AND L=1 THE 2 RANGES OF PHYSICALLY        Recent

POSSIBLE J ARE 1/2, 3/2, 5/2 AND 1/2, 3/2. BY CHECKING THE     Recent

ENDF/B DATA IT IS POSSIBLE TO INSURE THAT THE 3 POSSIBLE       Recent

J VALUES (1/2, 3/2, 5/2) ARE PRESENT AND TO INCLUDE ALL 3      Recent

J SEQUENCES IN THE CALCULATIONS. HOWEVER, UNLESS ALL 5         Recent

POSSIBLE J SEQUENCES ARE INCLUDED THE STATISTICAL WEIGHTS      Recent

OF THE J SEQUENCES WILL NOT SUM UP TO 2*L+1 AND THE            Recent

POTENTIAL CROSS SECTION WILL BE UNDERESTIMATED. IN THIS        Recent

EXAMPLE THE SUM OF THE 3 J SEQUENCES 1/2, 3/2, 5/2 IS 2,       Recent

RATHER THAN 3 AS IT SHOULD BE FOR L=1, AND THE CONTRIBUTION    Recent

OF THE L=1 RESONANCES TO THE POTENTIAL SCATTERING CROSS        Recent

SECTION WILL ONLY BE 2/3 OF WHAT IT SHOULD BE, UNLESS THE      Recent

OTHER 2 J SEQUENCES (WITH DUPLICATE J VALUES) ARE INCLUDED     Recent

IN THE CALCULATION.                                            Recent

Recent

(3) EACH RESONANCE MUST HAVE AN ASSIGNED, PHYSICALLY POSSIBLE      Recent

J VALUE. PHYSICALLY IMPOSSIBLE OR AVERAGE J VALUES CANNOT BE   Recent

UNIQUELY INTERPRETED USING THE EQUATIONS IN ENDF-102 AND       Recent

THEIR USE WILL USUALLY RESULT IN PHYSICALLY UNRELIABLE CROSS   Recent

SECTIONS. THIS PROGRAM WILL CHECK ALL J VALUES AND IF ANY ARE  Recent

ARE FOUND TO BE PHYSICALLY IMPOSSIBLE (BASED ON TARGET SPIN    Recent

AND L VALUE) AN ERROR MESSAGE WILL BE PRINTED TO INDICATE THAT Recent

THE RECONSTRUCTED CROSS SECTIONS WILL BE UNRELIABLE AND THE    Recent

PROGRAM WILL CONTINUE. IN AN ATTEMPT TO CALCULATE THE CORRECT  Recent

POTENTIAL SCATTERING CROSS SECTION THIS PROGRAM WILL SUBTRACT  Recent

THE POTENTIAL SCATTERING CONTRIBUTION DUE TO ALL FICTICIOUS J  Recent

SEQUENCES AND ADD THE CONTRIBUTION OF ALL PHYSICALLY POSSIBLE  Recent

J SEQUENCES (AS DESCRIBED ABOVE).                              Recent

Recent

WARNING (LET THE USER BEWARE)                                  Recent

=============================                                  Recent

(A) IT CANNOT BE STRESSED ENOUGH THAT CROSS SECTIONS OBTAINED  Recent

USING PHYSICALLY IMPOSSIBLE J VALUES FOR REICH-MOORE AND   Recent

MULTI-LEVEL BREIT-WIGNER RESONANCE PARAMETERS WILL RESULT  Recent

IN UNRELIABLE CROSS SECTIONS. THE DECISION TO HAVE THIS    Recent

PROGRAM CONTINUE TO PROCESS WHEN THIS CONDITION IS FOUND   Recent

IS BASED ON AN ATTEMPT TO ALLOW THE USER TO AT LEAST HAVE  Recent

SOME RESULTS (HOWEVER BAD THEY MAY BE) IF THERE IS NO      Recent

OTHER EVALUATED DATA AVAILABLE.                            Recent

(B) EVEN THOUGH THE REICH-MOORE AND MULTI-LEVEL EQUATIONS ARE  Recent

DEFINED AS ABSOLUTE OR SQUARED CONTRIBUTIONS WHICH MUST    Recent

ALL BE PHYSICALLY POSSIBLE, ATTEMPTING TO CORRECT THE      Recent

POTENTIAL CROSS SECTION (AS DESCRIBED ABOVE) CAN LEAD TO   Recent

NEGATIVE ELASTIC CROSS SECTIONS. THIS IS BECAUSE BASED ON  Recent

THE INFORMATION AVAILABLE IN THE EVALUATION IT IS NOT      Recent

NOT POSSIBLE TO CORRECTLY ACCOUNT FOR THE INTERFERENCE     Recent

BETWEEN THE RESONANCE AND POTENTIAL CONTRIBUTIONS FOR EACH Recent

J SEQUENCE.                                                Recent

Recent

UNRESOLVED RESONANCE REGION                                        Recent

---------------------------                                        Recent

IN THE UNRESOLVED RESONANCE REGION THE UNRESOLVED PARAMETERS       Recent

ARE USED TO CALCULATE INFINITELY DILUTE AVERAGE CROSS SECTIONS.    Recent

NOTE, IT IS IMPORTANT TO UNDERSTAND THAT FROM THE DEFINITION OF    Recent

THE UNRESOLVED PARAMETERS IT IS NOT POSSIBLE TO UNIQUELY CALCULATE Recent

ENERGY DEPENDENT CROSS SECTIONS. ONLY AVERAGES OR DISTRIBUTIONS    Recent

MAY BE CALCULATED.                                                 Recent

Recent

UNRESOLVED INTERPOLATION                                           Recent

------------------------                                           Recent

IN THE UNRESOLVED RESONANCE REGION CROSS SECTIONS AT EACH ENERGY   Recent

ARE CALCULATED BY INTERPOLATING PARAMETERS. THIS IS THE CONVENTION Recent

USED IN ENDF/B-IV AND EARLIER VERSIONS OF ENDF/B. THE ENDF/B-V     Recent

CONVENTION OF INTERPOLATING CROSS SECTIONS, NOT PARAMETERS, HAS    Recent

BEEN ABANDONED AS IMPRACTICAL SINCE IT CAN LEAD TO THE SITUATION   Recent

WHERE EXACTLY THE SAME PHYSICAL DATA CAN LEAD TO DIFFERENT RESULTS Recent

DEPENDING ON WHICH OF THE THREE ENDF/B UNRESOLVED PARAMTER FORMATS Recent

IS USED. FOR EXAMPLE, GIVEN A SET OF ENERGY INDEPENDENT UNRESOLVED Recent

PARAMETERS IT IS POSSIBLE TO CODE THESE PARAMETERS IN EACH OF THE  Recent

THREE ENDF/B UNRESOLVED PARAMETER FORMATS. SINCE PHYSICALLY WE     Recent

ONLY HAVE ONE SET OF PARAMETERS WE WOULD EXPECT THE RESULTS TO BE  Recent

INDEPENDENT OF HOW THEY ARE REPRESENTED IN ENDF/B. UNFORTUNATELY   Recent

USING THE ENDF/B-V CONVENTION TO INTERPOLATE CROSS SECTIONS CAN    Recent

LEAD TO THREE COMPLETELY DIFFERENT RESULTS. IN CONTRAST USING THE  Recent

ENDF/B-IV AND EARLIER CONVENTION OF INTERPOLATING PARAMETERS LEADS Recent

TO COMPLETELY CONSISTENT RESULTS.                                  Recent

Recent

INTERNAL REPRESENTATION OF UNRESOLVED PARAMETERS                   Recent

------------------------------------------------                   Recent

ANY OF THE THREE POSSIBLE REPRESENTATIONS OF UNRESOLVED PARAMETERS Recent

CAN BE UNIQUELY REPRESENTED IN THE ALL PARAMETERS ENERGY DEPENDENT Recent

REPRESENTATIONS WITH THE APPROPRIATE (ENDF/B VERSION DEPENDENT)    Recent

INTERPOLATION LAW. THIS IS DONE BY THE PROGRAM WHILE READING THE   Recent

UNRESOLVED PARAMETERS AND ALL SUBSEQUENT CALCULATIONS NEED ONLY    Recent

CONSIDER THE ALL PARAMETERS ENERGY DEPENDENT REPRESENTATION.       Recent

Recent

RESONANCE RECONSTRUCTION STARTING ENERGY GRID                      Recent

---------------------------------------------                      Recent

AS IN ANY ITERATIVE METHOD THE WAY TO SPEED CONVERGENCE IS TO TRY  Recent

TO START CLOSE TO THE ANSWER. THIS PROGRAM ATTEMPTS TO DO THIS BY  Recent

STARTING FROM AN ENERGY GRID WHICH IS A GOOD APPROXIMATION TO A    Recent

SIMPLE BREIT-WIGNER LINE SHAPE,                                    Recent

Recent

SIGMA(X)=1.0/(1.0+X*X)                                             Recent

Recent

WHERE X IS THE DISTANCE FROM THE PEAK IN HALF-WIDTHS               Recent

Recent

SUBROUTINE SUBINT HAS A BUILT-IN TABLE OF NODES WHICH ARE THE      Recent

HALF-WIDTH MULTIPLES TO APPROXIMATE THE SIMPLE BREIT-LINE SHAPE    Recent

TO WITHIN 1 PER-CENT OVER THE ENTIRE INTERVAL 0 TO 500 HALF-WIDTHS Recent

Recent

BETWEEN ANY TWO RESOLVED RESONANCES THE STARTING GRID IS BASED ON  Recent

THE HALF-WIDTHS OF THE TWO RESONANCES. FROM THE LOWER ENERGY       Recent

RESONANCE UP TO THE MID-POINT BETWEEN THE RESONANCES (MID-POINT    Recent

IS DEFINED HERE AS AN EQUAL NUMBER OF HALF-WIDTHS FROM EACH        Recent

RESONANCE) THE HALF-WIDTH OF THE LOWER ENERGY RESONANCE IS USED.   Recent

FROM THE MID-POINT UP TO THE HIGHER ENERGY RESONANCE THE HALF-     Recent

WIDTH OF THE UPPER ENERGY RESONANCE IS USED.                       Recent

Recent

WITH THIS ALOGORITHM CLOSELY SPACED RESONANCES WILL HAVE ONLY      Recent

A FEW STARTING NODES PER RESONANCE (E.G. U-235). WIDELY SPACED     Recent

RESONANCES WILL HAVE MORE NODES PER RESONANCE (E.G. U-238). FOR    Recent

A MIX OF S, P, D ETC. RESONANCES THIS ALOGORITHM GUARANTEES AN     Recent

ADEQUTE DESCRIPTION OF THE PROFILE OF EVEN EXTREMELY NARROW        Recent

RESONANCES (WHICH MAY IMMEDIATELY CONVERGENCE TO THE ACCURACY      Recent

REQUESTED, THUS MINIMIZING ITERATION).                             Recent

Recent

BACKGROUND CROSS SECTIONS                                          Recent

-------------------------                                          Recent

THE PROGRAM WILL SEARCH FOR BACKGROUND CROSS SECTIONS FOR TOTAL    Recent

(MT=1), ELASTIC (MT=2), FISSION (MT=18), FIRST CHANCE FISSION      Recent

(MT=19) AND CAPTURE (MT=102).                                      Recent

Recent

(1) THE BACKGROUND CROSS SECTIONS (FILE 3) CAN BE PRESENT OR NOT   Recent

PRESENT FOR EACH REACTION.                                     Recent

(2) IF FOR A GIVEN REACTION THE BACKGROUND CROSS SECTION IS        Recent

PRESENT, IT WILL BE ADDED TO THE RESONANCE CONTRIBUTION AND    Recent

THE RESULT WILL BE OUTPUT.                                     Recent

(3) IF FOR A GIVEN REACTION THE BACKGROUND IS NOT PRESENT THE      Recent

PROGRAM WILL,                                                  Recent

(A) IF THE INPUT TO THE PROGRAM SPECIFIES NO OUTPUT FOR        Recent

REACTIONS WITH NO BACKGROUND THERE WILL BE NO OUTPUT.      Recent

(B) IF THE INPUT TO THE PROGRAM SPECIFIES OUTPUT FOR REACTIONS Recent

WITH NO BACKGROUND,                                        Recent

(I) THE RESONANCE CONTRIBUTION TO TOTAL, ELASTIC OR        Recent

CAPTURE WILL BE OUTPUT.                                Recent

(II) IF ALL FISSION RESONANCE PARAMETERS ARE ZERO THE      Recent

FISSION CROSS SECTION (MT=18) WILL NOT BE OUTPUT.     Recent

OTHERWISE THE RESONANCE CONTRIBUTION OF THE FISSION   Recent

(MT=18) WILL BE OUTPUT.                               Recent

(III) THERE WILL BE NO OUTPUT FOR FIRST CHANCE FISSION     Recent

(MT=19).                                             Recent

Recent

COMBINING RESONANCES AND BACKGROUND CROSS SECTIONS                 Recent

--------------------------------------------------                 Recent

IN ORDER TO BE COMBINED WITH THE RESONANCE CONTRIBUTION THE        Recent

BACKGROUND CROSS SECTIONS MUST BE GIVEN AT 0 KELVIN TEMPERATURE    Recent

AND MUST BE LINEARLY INTERPOLABLE. IF THESE CONDITIONS ARE MET     Recent

THE RESONANCE AND BACKGROUND CONTRIBUTIONS WILL BE ADDED TOGETHER  Recent

AND OUTPUT. IF THESE CONDITIONS ARE NOT MET THE BACKGROUND CROSS   Recent

SECTION WILL BE IGNORED AND ONLY THE RESONANCE CONTRIBUTION WILL   Recent

BE OUTPUT. IF THE BACKGROUND HAS NOT BEEN ADDED TO THE RESONANCE   Recent

CONTRIBUTION AFTER THIS PROGRAM FINISHES THE USER CAN MAKE THE     Recent

RESONANCE AND BACKGROUND CONTRIBUTIONS COMPATIBLE BY,              Recent

Recent

(1) IF THE BACKGROUND IS NOT LINEARLY INTERPOABLE, LINEARIZE THE   Recent

BACKGROUND (E.G., USE PROGRAM LINEAR).                         Recent

(2) IF THE BACKGROUND IS NOT GIVEN AT 0 KELVIN, DOPPLER BROADEN    Recent

THE RESONANCE (NOT BACKGROUND) CONTRIBUTION TO THE SAME        Recent

TEMPERATURE AS THE BACKGROUND (E.G., USE PROGRAM SIGMA1).      Recent

Recent

ONCE THE RESONANCE AND BACKGROUND CONTRIBUTIONS HAVE BEEN MADE     Recent

COMPATIBLE THEY CAN BE ADDED TOGETHER (E.G., USE PROGRAM MIXER).   Recent

Recent

THE RECONSTRUCTION OF THE RESONANCE CONTRIBUTION TO THE CROSS      Recent

SECTION CAN BE QUITE EXPENSIVE (IN TERMS OF COMPUTER TIME). SINCE  Recent

THE RECONSTRUCTION IS PERFORMED BEFORE THE BACKGROUND CROSS        Recent

SECTIONS ARE READ, THE ABOVE CONVENTIONS HAVE BEEN ADOPTED IN      Recent

ORDER TO AVOID LOSE OF COMPUTER TIME INVOLVED IN RECONSTRUCTING    Recent

THE RESONANCE CONTRIBUTION.                                        Recent

Recent

COMMON ENERGY GRID                                                 Recent

------------------                                                 Recent

THIS PROGRAM WILL RECONSTRUCT THE RESONANCE CONTRIBUTION TO THE    Recent

TOTAL, ELASTIC, FISSION AND CAPTURE CROSS SECTIONS ALL ON THE      Recent

SAME ENERGY GRID. EACH REACTION WILL THEN BE COMBINED WITH ITS     Recent

BACKGROUND CROSS SECTION (IF ANY) AND OUTPUT WITHOUT ANY FURTHER   Recent

THINNING. IF THERE ARE NO BACKGROUND CROSS SECTIONS, OR IF THE     Recent

BACKGROUND CROSS SECTION FOR ALL FOUR REACTIONS ARE GIVEN ON A     Recent

COMMON ENERGY GRID, THE OUTPUT FROM THIS PROGRAM WILL BE ON A      Recent

COMMON ENERGY GRID FOR ALL FOUR REACTIONS.                         Recent

Recent

THERMAL ENERGY                                                     Recent

--------------                                                     Recent

IF THE RESONANCE REGION SPANS THERMAL ENERGY (0.0253 EV) THIS      Recent

POINT IS ALWAYS INCLUDED IN THE COMMON ENERGY GRID USED FOR ALL    Recent

REACTIONS AND WILL ALWAYS APPEAR IN THE OUTPUT DATA.               Recent

Recent

SECTION SIZE                                                       Recent

------------                                                       Recent

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Recent

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Recent

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Recent

Recent

SELECTION OF DATA                                                  Recent

-----------------                                                  Recent

THE PROGRAM SELECTS MATERIALS TO BE PROCESSED BASED EITHER ON      Recent

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Recent

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Recent

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Recent

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Recent

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Recent

Recent

ALLOWABLE ERROR                                                    Recent

---------------                                                    Recent

THE RECONSTRUCTION OF LINEARLY INTERPOLABLE CROSS SECTIONS FROM    Recent

RESONANCE PARAMETERS CANNOT BE PERFORMED EXACTLY. HOWEVER IT CAN   Recent

BE PERFORMED TO VIRTUALLY ANY REQUIRED ACCURACY AND MOST           Recent

IMPORTANTLY CAN BE PERFORMED TO A TOLERANCE THAT IS SMALL COMPARED Recent

TO THE UNCERTAINTY IN THE CROSS SECTIONS THEMSELVES. AS SUCH THE   Recent

CONVERSION OF CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM CAN BE  Recent

PERFORMED WITH ESSENTIALLY NO LOSS OF INFORMATION.                 Recent

Recent

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Recent

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Recent

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Recent

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Recent

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Recent

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Recent

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Recent

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES,     Recent

E.G., 0.1 PER-CENT FROM 0 UP TO THE LOW EV RANGE AND A LESS        Recent

STRINGENT TOLERANCE AT HIGHER ENERGIES.                            Recent

Recent

DEFAULT ALLOWABLE ERROR                                            Recent

-----------------------                                            Recent

IN ORDER TO INSURE CONVERENCE OF THE RESONANCE RECONSTRUCTION THE  Recent

ALLOWABLE ERROR MUST BE POSITIVE. IF THE USER INPUTS AN ERROR FOR  Recent

RESONANCE RECONSTRUCTION THAT IS NOT POSITIVE IT WILL BE SET TO    Recent

THE DEFAULT VALUE (CURRENTLY 0.1 PER-CENT) AND INDICATED AS SUCH   Recent

IN THE OUTPUT LISTING.                                             Recent

Recent

INTERVAL HALVING ALGORITHM                                         Recent

-------------------------                                          Recent

THIS PROGRAM WILL START BY CALCULATING THE CROSS SECTIONS AT THE   Recent

ENERGIES CORRESPONDING TO THE PEAK OF EACH RESONANCE, AS WELL AS   Recent

A FIXED NUMBER OF HALF-WIDTHS ON EACH SIDE OF EACH RESONANCE.      Recent

STARTING FROM THIS BASIC GRID OF POINTS THE PROGRAM WILL CONTINUE  Recent

TO HALF EACH INTERVAL UNTIL THE CROSS SECTIONS FOR ALL REACTIONS   Recent

AT THE CENTER OF THE INTERVAL CAN BE DEFINED BY LINEAR             Recent

INTERPOLATION FROM THE ENDS OF THE INTERVAL TO WITHIN THE USER     Recent

SPECIFIED ACCURACY CRITERIA.                                       Recent

Recent

DISTANT RESONANCE TREATMENT                                        Recent

---------------------------                                        Recent

THE OPTION TO TREAT DISTANT RESONANCES, WHICH WAS AVAILABLE IN     Recent

EARLIER VERSIONS OF THIS PROGRAM, IS NO LONGER AVAILABLE, BECAUSE  Recent

IT WAS FOUND TO PRODUCE UNRELIABLE RESULTS. IN THIS VERSION OF     Recent

THE PROGRAM ALL RESONANCES ARE TREATED EXACTLY.                    Recent

Recent

PROGRAM OPERATION                                                  Recent

================================================================== Recent

EDIT MODE                                                          Recent

---------                                                          Recent

IT IS SUGGESTED THAT BEFORE RUNNING THIS PROGRAM TO RECONSTRUCT    Recent

CROSS SECTIONS FROM RESONANCE PARAMETERS (WHICH CAN BE QUITE       Recent

EXPENSIVE) THE USER FIRST RUN THE PROGRAM IN THE EDIT MODE (SEE,   Recent

DESCRIPTION OF INPUT PARAMETERS BELOW). IN THE EDIT MODE THE       Recent

PROGRAM WILL READ, LIST AND EXTENSIVELY CHECK THE CONSISTENCY OF   Recent

ALL RESONANCE PARAMETERS AND ENDF/B DEFINED RESONANCE FLAGS. THIS  Recent

IS A VERY INEXPENSIVE MEANS OF CHECKING ALL DATA BEFORE INVESTING  Recent

A LARGE AMOUNT OF MONEY IN RECONSTRUCTING CROSS SECTIONS. ANY AND  Recent

ALL DIGNOSTICS RECEIVED FROM THE EDIT WILL SUGGEST HOW TO CORRECT  Recent

THE EVALUATED DATA TO MAKE IT CONSISTENT BEFORE RECONSTRUCTING     Recent

CROSS SECTIONS. IN ORDER TO OBTAIN MEANINGFUL RESULTS FROM THE     Recent

RECONSTRUCTION ALL SUGGESTED CHANGES TO THE EVALUATION SHOULD BE   Recent

PERFORMED BEFORE TRYING RECONSTRUCTION (OTHERWISE THE RESULT OF    Recent

RECONSTRUCTION WILL NOT BE RELIABLE).                              Recent

Recent

RECONSTRUCTION MODE                                                Recent

-------------------                                                Recent

FOR EACH REQUESTED MATERIAL                                        Recent

---------------------------                                        Recent

IF SECTION MF=1, MT=451 IS PRESENT COMMENTS WILL BE ADD TO         Recent

DOCUMENT THAT THE MATERIAL HAS BEEN PROCESSED. MF=1, MT=451 WILL   Recent

ALSO BE USED TO DETERMINE THE VERSION OF THE ENDF/B FORMAT WHICH   Recent

WILL ALLOW THE PROGRAM TO USE THE APPROPRIATE CONVENTIONS.         Recent

Recent

ALL OF THE FILE 2 RESONANCE PARAMETERS ARE FIRST READ AND THE      Recent

LINEARLY INTERPOLABLE CONTRIBUTION OF THE RESONANCE PARAMETERS     Recent

TO THE TOTAL, ELASTIC, CAPTURE AND FISSION CROSS SECTIONS IS       Recent

CALCULATED SIMULTANEOUSLY USING A COMMON ENERGY GRID FOR ALL       Recent

FOUR REACTIONS.                                                    Recent

Recent

AFTER THE RESONANCE CONTRIBUTION HAS BEEN RECONSTRUCTED EACH OF    Recent

THE FIVE REACTIONS (MT=1, 2, 18, 19, 102) IS CONSIDERED SEPARATELY Recent

FOR COMBINATION WILL THE BACKGROUND CROSS SECTION, IF ANY, AS      Recent

DESCRIBED ABOVE.                                                   Recent

Recent

OUTPUT WILL INCLUDE THE ENTIRE EVALUATION, INCLUDING RESONANCES    Recent

PARAMETERS WITH LRU MODIFIED (AS DESCRIBED ABOVE) TO INDICATE      Recent

THAT THE RESONANCE CONTRIBUTION HAS ALREADY BEEN ADDED TO THE      Recent

FILE 3 CROSS SECTIONS.                                             Recent

Recent

THE CYCLE OF RECONSTRUCTING THE RESONANCE CONTRIBUTION AND ADDING  Recent

THE BACKGROUND WILL BE REPEATED FOR EACH MATERIAL REQUESTED.       Recent

Recent

PROCESS ONLY A PORTION OF RESONANCE REGION                         Recent

================================================================== Recent

MODERN EVALUATIONS MAY BE EXTREMELY LARGE AND IT MAY NOT BE        Recent

POSSIBLE TO PROCESS AN ENTIRE EVALUATION (I.E., ADD THE RESONANCE  Recent

CONTRIBUTION) DURING A SINGLE COMPUTER RUN.                        Recent

Recent

ALSO IN THE CASE WHERE YOU ARE ONLY INTERESTED IN THE CROSS        Recent

SECTIONS OVER A SMALL ENERGY RANGE, YOU MAY NOT WANT TO PROCESS    Recent

AN ENTIRE EVALUATION, E.G., IF YOU ONLY WANT TO KNOW WHAT THE      Recent

CROSS SECTIONS ARE NEAR THERMAL ENERGY, 0.0253 EV.                 Recent

Recent

IN ORDER TO ALLOW AN EVALUATION TO BE PROCESSED USING A NUMBER OF  Recent

SHORTER COMPUTER RUNS AN OPTION HAS BEEN ADDED TO THIS PROGRAM TO  Recent

ALLOW THE USER TO SPECIFY THE ENERGY RANGE TO BE PROCESSED.        Recent

Recent

USING THIS OPTION YOU MAY START AT THE LOWEST ENERGY (ZERO UP TO   Recent

SOME ENERGY) AND USE THE RESULTS OF THIS RUN AS INPUT TO THE       Recent

NEXT RUN, WHERE YOU CAN SPECIFY THE NEXT ENERGY RANGE. THIS        Recent

CYCLE CAN BE REPEATED UNTIL YOU HAVE PROCESSED THE ENTIRE          Recent

EVALUATION.                                                        Recent

Recent

WARNING - THIS OPTION SHOULD BE USED WITH EXTREME CARE - THIS      Recent

OPTION HAS BEEN RELUCTANTLY ADDED - RELUCTANTLY BECAUSE IT CAN     Recent

BE EXTREMELY DANGEROUS TO USE THIS OPTION UNLESS YOU CAREFULLY     Recent

CHECKED WHAT YOU ARE DOING.                                        Recent

Recent

THE OPTION SHOULD ONLY BE USED AS FOLLOWS,                         Recent

1) YOU MUST PROCESS USING ENERGY RANGES STARTING AT LOW ENERGY     Recent

AND WORKING YOUR WAY TOWARD HIGH ENERGY, E.G.,                  Recent

0.0   TO  3.0+3                                                Recent

3.0+3 TO 10.0+3                                                Recent

10.0+3 TO 80.0+3, ETC.                                          Recent

2) FOR THE LAST ENERGY RANGE THE LOWER ENERGY LIMIT MUST BE        Recent

NON-ZERO (WHERE TO START) AND THE UPPER ENERGY LIMIT MUST       Recent

BE ZERO (NO LIMIT)                                              Recent

80.0+3 TO  0.0                                                  Recent

Recent

IF YOU ARE ONLY INTERESTED IN THE CROSS SECTION OVER A NARROW      Recent

ENERGY INTERVAL AND DO NOT INTENT TO MAKE ANY OTHER USE OF THE     Recent

RESULTS, YOU CAN IGNORE THESE WARNINGS AND MERELY SPECIFY ANY      Recent

ENERGY INTERVAL OVER WHICH YOU WISH CALCULATIONS TO BE             Recent

PERFORMED.                                                         Recent

Recent

NORMALLY WHEN THIS PROGRAM PROCESSES AN EVALUATION IT WILL SET     Recent

FLAGS IN THE EVALUATION TO PREVENT THE SAME RESONANCE              Recent

CONTRIBUTION FROM BEING ADDED TO THE CROSS SECTION MORE THAN       Recent

ONCE, SHOULD YOU USE THE OUTPUT FROM THIS PROGRAM AS INPUT TO      Recent

THE PROGRAM.                                                       Recent

Recent

WHEN PROCESSING ONLY PORTIONS OF THE RESONANCE REGION THIS         Recent

PROGRAM CANNOT SET THESE FLAGS TO PROTECT AGAINST ADDING THE       Recent

RESONANCE CONTRIBUTION MORE THAN ONCE - WHICH MAKES USE OF         Recent

THIS OPTION EXTREMELY DANGEROUS.                                   Recent

Recent

ONLY YOU CAN CHECK TO MAKE SURE THAT YOU HAVE CORRECTLY            Recent

INCLUDED EACH ENERGY RANGE ONLY ONCE - SEE THE COMMENT LINES       Recent

AT THE END OF SECTION, MF=1, MT=451, FOR A COMPLETE RECORD         Recent

OF EACH RUN USING THIS PROGRAM. THIS SECTION WILL CONTAIN          Recent

LINES OF THE FORM                                                  Recent

Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  0.00000+ 0 TO  3.00000+ 3 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  3.00000+ 3 TO  1.00000+ 4 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  1.00000+ 4 TO  8.00000+ 4 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  8.00000+ 4 TO  2.00000+ 7 EV                         Recent

Recent

YOU SHOULD CHECK TO INSURE THAT THERE ARE NO OVERLAPPING ENERGY    Recent

RANGES OR MISSING ENERGY RANGES.                                   Recent

Recent

WHEN YOU INDICATE BY INPUT THAT YOU ARE ABOUT TO PROCESS THE       Recent

LAST ENERGY RANGE (SEE ABOVE, LOWER ENERGY LIMIT = NON-ZERO,       Recent

UPPER ENERGY LIMIT = ZERO), THIS PROGRAM WILL ASSUME THAT          Recent

YOU HAVE NOW COMPLETED ALL PROCESSING - AND ONLY THEN WILL         Recent

IT SET FLAGS IN THE EVALUATION TO PREVENT THE RESONANCE            Recent

CONTRIBUTION FROM BEING ADDED MORE THAN ONCE. FOR THIS REASON      Recent

YOU CANNOT PROCESS STARTING WITH ENERGY INTERVALS AT HIGH          Recent

ENERGY AND WORKING TOWARD LOW ENERGY - YOU MUST START AT LOW       Recent

ENERGY AND WORK TOWARD HIGH ENERGY.                                Recent

Recent

I/O FILES                                                          Recent

================================================================== Recent

INPUT FILES                                                        Recent

-----------                                                        Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

2   INPUT LINE (BCD - 80 CHARACTERS/RECORD)                      Recent

10   ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Recent

Recent

OUTPUT FILES                                                       Recent

------------                                                       Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

3   OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Recent

11   FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Recent

Recent

SCRATCH FILES                                                      Recent

-------------                                                      Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

12   SCRATCH FILE FOR DATA RECONSTRUCTED FROM RESONANCE           Recent

PARAMETERS (BINARY - 100200 WORDS/RECORD)                    Recent

14   SCRATCH FILE FOR COMBINED FILE 2 AND 3 DATA                  Recent

(BINARY - 40080 WORDS/RECORD)                                Recent

Recent

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Recent

================================================================== Recent

UNIT  FILE NAME                                                    Recent

----  ----------                                                   Recent

2   RECENT.INP                                                   Recent

3   RECENT.LST                                                   Recent

10   ENDFB.IN                                                     Recent

11   ENDFB.OUT                                                    Recent

12   (SCRATCH)                                                    Recent

14   (SCRATCH)                                                    Recent

Recent

INPUT CARDS                                                        Recent

================================================================== Recent

LINE  COLS.  FORMAT  DESCRIPTION                                   Recent

----  -----  ------  -----------                                   Recent

1    1-11    I11   RETRIEVAL CRITERIA (0=MAT, 1=ZA)              Recent

THIS OPTION DEFINED WHETHER COLUMNS 1-22 OF   Recent

SUBSEQUENT INPUT CARDS SHOULD BE INTERPRETED  Recent

TO BE MAT OR ZA RANGES.                       Recent

12-22   E11.4  FILE 2 MINIMUM ABSOLUTE CROSS SECTION         Recent

(IF 1.0E-10 OR LESS IS INPUT THE PROGRAM      Recent

WILL USE 1.0E-10)                             Recent

23-33    I11   TREATMENT OF REACTIONS FOR WHICH BACKGROUND   Recent

CROSS SECTION IS NOT GIVEN.                   Recent

= 0 - IGNOR (I.E. NO OUTPUT)                  Recent

= 1 - OUTPUT RESONANCE CONTRIBUTION.          Recent

THIS OPTION IS USEFUL WITH PARTIAL EVALUATION Recent

(E.G. ENDF/B-V DOSIMETRY LIBRARY) WHERE ONLY  Recent

ONE OR MORE OF THE REACTIONS ARE OF ACTUAL    Recent

INTEREST.                                     Recent

WARNING...THE USE OF THIS FIELD HAS BEEN      Recent

CHANGED. THIS FIELD WAS PREVIOUSLY USED TO    Recent

DEFINE THE PRECISION OF THE CALCULATION AND   Recent

OUTPUT. THE FORMER DEFINITION OF THIS FIELD   Recent

WAS...                                        Recent

MINIMUM ENERGY SPACING FLAG                   Recent

= 0 - 6 DIGIT MINIMUM ENERGY SPACING.         Recent

STANDARD 6 DIGIT E11.4 OUTPUT.          Recent

= 1 - 9 DIGIT MINIMUM ENERGY SPACING.         Recent

STANDARD 6 DIGIT E11.4 OUTPUT.          Recent

= 2 - 9 DIGIT MINIMUM ENERGY SPACING.         Recent

VARIABLE 9 DIGIT F FORMAT OUTPUT.       Recent

FROM EXPERIENCE IT HAS BEEN FOUND THAT        Recent

FAILURE TO SET THIS OPTION TO 2 CAN RESULT    Recent

IN LARGE ERRORS IN THE FINAL DATA. THEREFORE  Recent

INTERNALLY THIS OPTION IS SET TO 2.           Recent

34-44    I11   OPERATING MODE                                Recent

= 0 - CACULATE. MINIMUM OUTPUT LISTING        Recent

= 1 - CACULATE. LIST ALL RESONANCE PARAMETERS Recent

= 2 - EDIT MODE. NO CALCULATION. LIST ALL     Recent

RESONANCE PARAMETERS.                   Recent

NOTE, THE EDIT MODE (=2) IS THE SUGGESTED     Recent

MODE TO FIRST TEST THE CONSISTENCY OF THE     Recent

EVALUATED DATA, BEFORE RECONSTRUCTING CROSS   Recent

SECTIONS (SEE, COMMENTS ABOVE).               Recent

45-55    I11   NEGATIVE CROSS SECTIOIN TREATMENT             Recent

= 0 - O.K. - NO CHANGE                        Recent

= 1 - SET = 0                                 Recent

56-66    I11   MONITOR MODE SELECTOR                         Recent

= 0 - NORMAL OPERATION                        Recent

= 1 - MONITOR PROGRESS OF RECONSTRUCTION OF   Recent

FILE 2 DATA AND COMBINING FILE 2 AND    Recent

FILE 3 DATA. EACH TIME A PAGE OF DATA   Recent

POINTS IS WRITTEN TO A SCRATCH FILE     Recent

PRINT OUT THE TOTAL NUMBER OF POINTS    Recent

ON SCRATCH AND THE LOWER AND UPPER      Recent

ENERGY LIMITS OF THE PAGE (THIS OPTION  Recent

MAY BE USED IN ORDER TO MONITOR THE     Recent

EXECUTION SPEED OF LONG RUNNING JOBS).  Recent

2    1-60    A60   ENDF/B INPUT DATA FILENAME                    Recent

(STANDARD OPTION = ENDFB.IN)                  Recent

3    1-60    A60   ENDF/B OUTPUT DATA FILENAME                   Recent

(STANDARD OPTION = ENDFB.OUT)                 Recent

4-N    1-11    I11   MINIMUM MAT OR ZA (SEE COLS. 1-11, LINE 1)    Recent

12-22    I11   MAXIMUM MAT OR ZA (SEE COLS. 1-11, LINE 1)    Recent

UP TO 100 MAT OR ZA RANGES MAY BE SPECIFIED,  Recent

ONE RANGE PER LINE. THE LIST IS TERMINATED    Recent

BY A BLANK LINE. IF THE THE UPPER LIMIT OF    Recent

ANY REQUEST IS LESS THAN THE LOWER LIMIT THE  Recent

UPPER LIMIT WILL BE SET EQUAL TO THE LOWER    Recent

LIMIT. IF THE FIRST REQUEST LINE IS BLANK IT  Recent

WILL TERMINATE THE REQUEST LIST AND CAUSE ALL Recent

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).     Recent

23-33   E11.4  LOWER ENERGY LIMIT FOR PROCESSING.            Recent

34-44   E11.4  UPPER ENERGY LIMIT FOR PROCESSING.            Recent

*THE LOWER AND UPPER ENERGY LIMITS MUST BE     Recent

ZERO, OR BLANK, UNLESS YOU WISH TO ONLY       Recent

PROCESS A PORTION OF RESONANCE REGIONS.       Recent

*THESE ENERGY LIMITS ARE ONLY READ FROM THE    Recent

FIRST MAT/ZA REQUEST LINE                     Recent

*IF BOTH ARE ZERO (OR BLANK) THE ENTIRE        Recent

RESONANCE REGION FOR EACH MATERIAL WILL BE    Recent

PROCESSED                                     Recent

*IF LIMITS ARE INPUT ONLY THAT PORTION OF THE  Recent

RESONANCE REGION FOR EACH MATERIAL WHICH      Recent

LIES BETWEEN THESE LIMITS WILL BE PROCESSED   Recent

*SEE INSTRUCTIONS ABOVE BEFORE USING THIS      Recent

OPTION.                                       Recent

VARY   1-11   E11.4  ENERGY FOR FILE 2 ERROR LAW     (  SEE   )    Recent

12-22   E11.4  ERROR FOR FILE 2 ERROR LAW      (COMMENTS)    Recent

( BELOW  )    Recent

Recent

NOTE, THIS VERSION OF THE PROGRAM DOES NOT THIN THE COMBINED FILE  Recent

FILE 2 + 3 DATA. AS SUCH THE ERROR LAW FOR COMBINING FILE 2 + 3    Recent

WHICH WAS REQUIRED IN EARLIER VERSIONS OF THIS CODE ARE NO LONGER  Recent

REQUIRED.                                                          Recent

Recent

THE FILE 2 ERROR LAW MAY BE ENERGY INDEPENDENT (DEFINED BY A       Recent

SINGLE ERROR) OR ENERGY DEPENDENT (DEFINED BY UP TO 20 ENERGY,     Recent

ERROR PAIRS). FOR THE ENERGY DEPENDENT CASE LINEAR INTERPOLATION   Recent

WILL BE USED TO DEFINE THE ERROR AT ENERGIES BETWEEN THOSE AT      Recent

WHICH THE ERROR IS TABULATED. THE ERROR LAW IS TERMINATED BY A     Recent

BLANK LINE. IF ONLY ONE ENERGY, ERROR PAIR IS GIVEN THE LAW WILL   Recent

BE CONSIDERED TO BE ENERGY INDEPENDENT. IF MORE THAN ONE PAIR      Recent

IS GIVEN IT BE CONSIDERED TO BE ENERGY DEPENDENT (NOTE, THAT       Recent

FOR A CONSTANT ERROR THE ENERGY INDEPENDENT FORM WILL RUN FASTER.  Recent

HOWEVER, FOR SPECIFIC APPLICATIONS AN ENERGY DEPENDENT ERROR MAY   Recent

BY USED TO MAKE THE PROGRAM RUN CONSIDERABLE FASTER).              Recent

Recent

ALL ENERGIES MUST BE IN ASCENDING ENERGY ORDER. FOR CONVERGENCE    Recent

OF THE FILE 2 RECONSTRUCTION ALGORITHM ALL THE ERRORS MUST BE      Recent

POSITIVE. IF ERROR IS NOT POSITIVE IT WILL BE SET EQUAL TO THE     Recent

STANDARD OPTION (CURRENTLY 0.001, CORRRESPONDING TO 0.1 PER-CENT). Recent

IF THE FIRST LINE OF THE ERROR LAW IS BLANK IT WILL TERMINATE THE  Recent

ERROR LAW AND THE ERROR WILL BE TREATED AS ENERGY INDEPENDENT,     Recent

EQUAL TO THE STANDARD OPTION (CURRENTLY, 0.1 PER-CENT). SEE,       Recent

EXAMPLE INPUT 4.                                                   Recent

Recent

EXAMPLE INPUT NO. 1                                                Recent

-------------------                                                Recent

CONSIDER ALL URANIUM ISOTOPES AND TH-232. CONSIDER CROSS SECTIONS  Recent

WHICH ARE LARGER THAN 1.0E-8 BARNS IN ABSOLUTE VALUE. ONLY OUTPUT  Recent

REACTIONS FOR WHICH A BACKGROUND IS GIVEN. LIST ALL PARAMETERS AND Recent

CALCULATE CROSS SECTIONS. MONITOR THE EXECUTION PROGRESS OF THE    Recent

PROGRAM. BETWEEN 0 AND 100 EV USE 0.1 PER-CENT ACCURACY. BETWEEN   Recent

100 EV AND 1 KEV VARY THE ACCURACY FROM 0.1 TO 1 PER-CENT. ABOVE   Recent

1 KEV USE 1 PER-CENT ACCURACY.                                     Recent

Recent

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Recent

Recent

THE FOLLOWING 11 INPUT CARDS ARE REQUIRED.                         Recent

Recent

1 1.00000-08          0          1          0         1       Recent

ENDFB.IN                                                               Recent

ENDFB.OUT                                                              Recent

92000      92999                                                  Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

0.00000+ 0 1.00000-03                                                  Recent

1.00000+02 1.00000-03                                                  Recent

1.00000+03 1.00000-02                                                  Recent

1.00000+09 1.00000-02                                                  Recent

(END FILE 2 ERROR LAW)                     Recent

Recent

EXAMPLE INPUT NO. 2                                                Recent

-------------------                                                Recent

CONSIDER ALL URANIUM ISOTOPES AND TH-232. CONSIDER CROSS SECTIONS  Recent

WHICH ARE LARGER THAN 1.0E-8 BARNS IN ABSOLUTE VALUE. ONLY OUTPUT  Recent

REACTIONS FOR WHICH A BACKGROUND IS GIVEN. CROSS SECTIONS WILL BE  Recent

CALCULATED, BUT PARAMETERS WILL NOT BE LISTED. THE PROGRESS OF THE Recent

PROGRAM WILL NOT BE MONITORED. USE 0.1 PER-CENT ACCURACY FOR ALL   Recent

ENERGIES. SINCE 0.1 PER-CENT IS THE STANDARD OPTION FOR THE ERROR  Recent

LAW THE FIRST ERROR LAW LINE MAY BE LEFT BLANK.                    Recent

Recent

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Recent

THEN USE THE STANDARD FILENAMES.                                   Recent

Recent

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Recent

Recent

1 1.00000-08          0          0          0         0       Recent

Recent

Recent

92000      92999                                                  Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

(USE STANDARD OPTION FOR ERROR LAW)        Recent

Recent

EXAMPLE INPUT NO. 3                                                Recent

-------------------                                                Recent

THE SAME AS EXAMPLE INPUT NO. 2, ONLY IN THIS CASE ONLY CALCULATE  Recent

CROSS SECTIONS OVER THE ENERGY RANGE 0.01 TO 0.1 EV - ACROSS THE   Recent

THERMAL ENERGY RANGE. NOTE, THE ONLY DIFFERENCE BETWEEN THE INPUT  Recent

PARAMETERS IN THIS CASE AND IN EXAMPLE NO. 2, IS THAT ON THE       Recent

SECOND INPUT LINE WE HAVE ADDED THE ENERGY RANGE 0.01 TO 0.1 EV.   Recent

USE \PREPRO94\LINEAR\ENDFB.OUT AS INPUT AND ENDFB.OUT AS OUTPUT -  Recent

SINCE ENDFB.OUT IS THE STANDARD OUTPUT FILENAME THE NAME CAN BE    Recent

EITHER INCLUDED IN THE INPUT OR LEFT BLANK.                        Recent

Recent

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Recent

Recent

1 1.00000-08          0          0          0         0       Recent

\PREPRO94\LINEAR\ENDFB.OUT                                             Recent

ENDFB.OUT                                                              Recent

92000      92999 1.00000- 2 1.00000- 1                            Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

(USE STANDARD OPTION FOR ERROR LAW)        Recent

Recent

EXAMPLE INPUT NO. 4                                                Recent

-------------------                                                Recent

RECONSTRUCT ALL DATA. OUTPUT ALL REACTIONS, REGARDING OF WHETHER   Recent

OR NOT THERE IS A BACKGROUND CROSS SECTION. DO NOT MONITOR THE     Recent

PROGRESS OF THE PROGRAM. RECONSTRUCT CROSS SECTIONS TO 1 PER-CENT  Recent

ACCURACY. USE \ENDFB6\LINEAR\ZA092238 AS INPUT AND                 Recent

\ENDFB6\RECENT\ZA092238 AS OUTPUT.                                 Recent

Recent

THE FOLLOWING 6 INPUT CARDS ARE REQUIRED.                          Recent

Recent

0 0.0                 1          0          0         0       Recent

\ENDFB6\ZA092238                                                       Recent

\ENDFB6\RECENT\ZA092238                                                Recent

(RETRIEVE ALL DATA, END REQUEST LIST)            Recent

1.00000- 2                                                  Recent

(END FILE 2 ERROR LAW)                           Recent

Recent

EXAMPLE INPUT NO. 5                                                Recent

-------------------                                                Recent

RECONSTRUCT ALL DATA. ONLY OUTPUT REACTIONS FOR WHICH A BACKGROUND Recent

CROSS SECTION IS GIVEN. DO NOT MONITOR THE PROGRESS OF THE PROGRAM Recent

RECONSTRUCT CROSS SECTIONS TO 0.1 PER-CENT ACCURACY. USE ENDFB.IN  Recent

AS INPUT AND ENDFB.OUT AS OUTPUT.                                  Recent

Recent

THIS CORRESPONDS TO USING ALL OF THE STANDARD OPTONS BUILT-IN TO   Recent

THE PROGRAM AND ALL INPUT CARDS MAY BE BLANK.                      Recent

Recent

IN THIS CASE THE FOLLOWING 5 INPUT CARDS ARE REQUIRED.             Recent

(ZEROES ARE INDICATED ON THE FIRST LINE, BELOW, ONLY TO INDICATE   Recent

WHERE THE LINE IS. THE ACTUAL INPUT LINE CAN BE COMPLETELY BLANK). Recent

Recent

0 0.0                 0          0          0         0       Recent

(USE STANDARD INPUT FILENAME = ENDFB.IN)         Recent

(USE STANDARD OUTPUT FILENAME = ENDFB.OUT)       Recent

(RETRIEVE ALL DATA, END REQUEST LIST)            Recent

(0.1 ERROR, END FILE 2 ERROR LAW)                Recent

Recent

======================================================================= Recent

======================================================================= Relabel

Relabel

PROGRAM RELABEL                                                    Relabel

VERSION 69-1 (APRIL 1969)                                          Relabel

VERSION 73-1 (JUNE 1973)                                           Relabel

VERSION 77-1 (SEPTEMBER 1977)                                      Relabel

VERSION 80-1 (AUGUST 1980) IBM VERSION                             Relabel

VERSION 83-1 (JANUARY 1983) COMBINED STATEMENT NUMBER SEQUENCE     Relabel

AND LINE I.D. INTO ONE PROGRAM.        Relabel

VERSION 86-1 (JANUARY 1986) FORTRAN-77/H VERSION                   Relabel

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Relabel

FILE NAMES (SEE, SUBROUTINES FILIO1    Relabel

AND FILIO2 FOR DETAILS).               Relabel

*IMPROVED BASED ON USER COMMENTS.       Relabel

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Relabel

INSURE PROGRAM WILL NOT DO ANYTHING    Relabel

CRAZY.                                 Relabel

*UPDATED TO USE NEW PROGRAM CONVERT     Relabel

KEYWORDS.                              Relabel

*ADDED LIVERMORE CIVIC COMPILER         Relabel

CONVENTIONS.                           Relabel

VERSION 92-1 (JANUARY 1992)*ADDED FORTRAN SAVE OPTION              Relabel

VERSION 94-1 (JANUARY 1994)*COMPLETE RE-WRITE                      Relabel

*OUTPUT MINIMUM NON-BLANK LENGTH FOR    Relabel

EACH LINE - NO SEQUENCE NUMBERS.       Relabel

*INCREASED MAXIMUM NUMBER OF LABELS     Relabel

PER ROUTINE FROM 1000 TO 50,000        Relabel

*CAN NOW PROCESS UPPER OR LOWER CASE    Relabel

CODING.                                Relabel

*SKIP IMBEDDED BLANKS IN KEYWORDS.      Relabel

*ADDED WRITE(XX,XX,ERR=YYY,END=ZZZ)     Relabel

*ADDED OPEN(XX,ERR=YYY,END=ZZZ)         Relabel

*ADDED CLOSE(XX,ERR=YYY,END=ZZZ)        Relabel

*INTEGER INSTEAD OF CHARACTERS IS NO    Relabel

LONGER SUPPORTED - ALL CHARACTERS      Relabel

MUST BE IDENTIFIED AS CHARACTERS.      Relabel

*VARIABLE FILENAMES TO ALLOW ACCESS     Relabel

FILE STRUCTURES                        Relabel

(WARNING - INPUT PARAMETER FORMAT      Relabel

HAS BEEN CHANGED)                      Relabel

*CLOSE ALL FILES BEFORE TERMINATING     Relabel

(SEE, SUBROUTINE ENDIT)                Relabel

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Relabel

*IMPROVED COMPUTER INDEPENDENCE        Relabel

*ALL DOUBLE PRECISION                  Relabel

*ON SCREEN OUTPUT                      Relabel

*IMPROVED OUTPUT PRECISION             Relabel

*DEFINED SCRATCH FILE NAMES            Relabel

*INCREASED THE NUMBER OF LABELS        Relabel

IN A ROUTINE FOR 5,000 TO 50,000      Relabel

VERSION 99-1 (MARCH 1999)   *GENERAL IMPROVEMENTS BASED ON         Relabel

USER FEEDBACK                         Relabel

VERS. 2000-1 (FEBRUARY 2000)*UPDATED TO IGNORE ( AND ) IN QUOTES   Relabel

*GENERAL IMPROVEMENTS BASED ON         Relabel

USER FEEDBACK                         Relabel

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Relabel

*CORRECTED END=, ERR=, WHEN I/O UNIT   Relabel

NUMBER IS DIMENSIONED                 Relabel

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Relabel

*INCREASED THE NUMBER OF LABELS IN     Relabel

A ROUTINE FOR 50,000 TO 100,000       Relabel

Relabel

OWNED, MAINTAINED AND DISTRIBUTED BY                               Relabel

------------------------------------                               Relabel

THE NUCLEAR DATA SECTION                                           Relabel

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Relabel

P.O. BOX 100                                                       Relabel

A-1400, VIENNA, AUSTRIA                                            Relabel

EUROPE                                                             Relabel

Relabel

ORIGINALLY WRITTEN BY                                              Relabel

------------------------------------                               Relabel

DERMOTT E. CULLEN                                                  Relabel

UNIVERSITY OF CALIFORNIA                                           Relabel

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Relabel

L-159                                                              Relabel

P.O. BOX 808                                                       Relabel

LIVERMORE, CA 94550                                                Relabel

U.S.A.                                                             Relabel

TELEPHONE  925-423-7359                                            Relabel

E. MAIL    CULLEN1@LLNL.GOV                                        Relabel

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Relabel

Relabel

PURPOSE                                                            Relabel

-------                                                            Relabel

THIS PROGRAM IS DESIGNED TO RE-LABEL A FORTRAN PROGRAM SO THAT     Relabel

STATEMENT LABELS ARE IN INCREASING ORDER IN INCREMENTS OF 10       Relabel

WITHIN EACH ROUTINE.                                               Relabel

Relabel

THE FOLLOWING TYPES OF FORTRAN STATEMENTS ARE CONSIDERED,          Relabel

Relabel

GO TO NN                                                           Relabel

GO TO (NN,MM,.....,JJ,KK),LL (MULTI LINE O.K.)                     Relabel

DO NN                                                              Relabel

IF(......) NN,MM,...JJ,KK                                          Relabel

IF(......) GO TO NN                                                Relabel

IF(......) GO TO (NN,MM,.....,JJ,KK),LL (MULTI LINE O.K.)          Relabel

IF(......) READ(.....,END=NN,ERR=MM)                               Relabel

IF(......) WRITE(.....,END=NN,ERR=MM)                              Relabel

READ(......,END=NN,ERR=MM)                                         Relabel

OPEN(......,END=NN,ERR=MM)                                         Relabel

Relabel

GO TO STATEMENTS MAY APPEAR IN THE FORM 'GO TO' OR 'GOTO'. IF      Relabel

THERE IS ROOM ON THE LINE 'GOTO' WILL BE CONVERTED TO 'GO TO'.     Relabel

WHETHER OR NOT 'GOTO' IS CONVERTED TO 'GO TO' IT WILL BE TREATED   Relabel

AS IDENTICAL TO 'GO TO' FOR SUBSEQUENT PROCESSING AND RELACEMENT   Relabel

OF STATEMENT NUMBERS.                                              Relabel

Relabel

ALL OTHER STATEMENT TYPES ARE NOT CHANGED. IN PARTICULAR ALL I/O   Relabel

STATEMENTS AND ASSOCIATED FORMAT STATEMENTS ARE NOT CONVERTED.     Relabel

Relabel

WARNING                                                            Relabel

-------                                                            Relabel

THIS PROGRAM IS ONLY DESIGNED TO MAINTAIN ENDF/B PRE-PROCESSING    Relabel

PROGRAMS, WHICH ONLY USE A RESTRICTED SET OF FORTRAN STATEMENT     Relabel

TYPES THAT CAN BE USED ON A VARIETY OF DIFFERENT TYPES OF          Relabel

COMPUTERS. THIS PROGRAM IS NOT DESIGNED TO HANDLE ALL POSSIBLE     Relabel

TYPES OF FORTRAN STATEMENTS.                                       Relabel

Relabel

THE FORTRAN STATEMENTS DESCRIBED ABOVE AND TREATED BY THIS PROGRAM Relabel

DO NOT INCLUDE ALL POSSIBLE FORTRAN STATEMENTS. AS SUCH THIS       Relabel

PROGRAM IS NOT COMPLETELY GENERAL AND SHOULD ONLY BE USED WITH     Relabel

PROGRAMS THAT ONLY USE THE FORTRAN STATEMENTS DESCRIBED ABOVE.     Relabel

Relabel

FAILURE TO FOLLOW THESE INSTRUCTIONS CAN LEAD TO ERROR IN PROGRAMS Relabel

Relabel

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)   Relabel

----------------------------------------------------------------   Relabel

UNIT  FILE NAME    DESCRIPTION                                     Relabel

----  ----------   -----------                                     Relabel

2   RELABEL.INP  INPUT PARAMETERS                                Relabel

3   RELABEL.LST  OUTPUT REPORT                                   Relabel

10   RELABEL.IN   PROGRAM TO READ                                 Relabel

11   RELABEL.OUT  PROGRAM TO WRITE                                Relabel

12   (SCRATCH)                                                    Relabel

Relabel

INPUT CARDS                                                        Relabel

-----------                                                        Relabel

LINE  COLUMNS  DEFINITION                                          Relabel

----  -------  ----------                                          Relabel

1   1-60    INPUT PROGRAM FILENAME                              Relabel

(STANDARD OPTION = RELABEL.IN)                      Relabel

2   1-60    OUTPUT PROGRAM FILENAME                             Relabel

(STANDARD OPTION = RELABEL.OUT)                     Relabel

Relabel

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Relabel

THEN USE STANDARD FILENAMES.                                       Relabel

Relabel

EXAMPLE INPUT NO. 1                                                Relabel

-------------------                                                Relabel

TO READ \PREPRO94\RECENT\RECENT.FOR AND                            Relabel

WRITE   \PREPRO94\RECENT\RECENT.NEW THE FOLLOWING 2 INPUT LINES    Relabel

ARE REQUIRED,                                                      Relabel

Relabel

\PREPRO94\RECENT\RECENT.FOR                                        Relabel

Relabel

EXAMPLE INPUT NO. 2                                                Relabel

-------------------                                                Relabel

TO READ RELABEL.IN AND WRITE RELABEL.OUT THE FOLOWING 2 INPUT      Relabel

LINES ARE REQUIRED,                                                Relabel

Relabel

RELABEL.IN                                                         Relabel

RELABEL.OUT                                                        Relabel

Relabel

EXAMPLE INPUT NO. 3                                                Relabel

-------------------                                                Relabel

TO READ RELABEL.IN AND WRITE RELABEL.OUT, SINCE THESE ARE THE      Relabel

STANDARD OPTIONS THE 2 INPUT LINES CAN BE COMPLETELY BLANK.        Relabel

Relabel

======================================================================= Relabel

======================================================================= Sigma1

Sigma1

PROGRAM SIGMA1                                                     Sigma1

==============                                                     Sigma1

VERSION 73-1 (MARCH 1973)                                          Sigma1

VERSION 76-1 (FEBRUARY 1976)                                       Sigma1

VERSION 76-2 (OCTOBER 1976)                                        Sigma1

VERSION 77-1 (JANUARY 1977)                                        Sigma1

VERSION 78-1 (JULY 1978)                                           Sigma1

VERSION 79-1 (JULY 1979)    CDC-7600 AND CRAY-1 VERSION.           Sigma1

VERSION 80-1 (MAY 1980)     IBM, CDC AND CRAY VERSION              Sigma1

VERSION 80-2 (DECEMBER 1980)IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 81-1 (MARCH 1981)   DOUBLE PRECISION IBM VERSION           Sigma1

VERSION 81-2 (AUGUST 1981)  IMPROVED IBM SPEED AND STABILITY       Sigma1

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY        Sigma1

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Sigma1

*PAGE SIZE INCREASED - 1002 TO 2004.    Sigma1

*ELIMINATED COMPUTER DEPENDENT CODING.  Sigma1

*NEW, MORE COMPATIBLE I/O UNIT NUMBER.  Sigma1

*ADDED STANDARD ALLOWABLE ERROR OPTION  Sigma1

(CURRENTLY 0.1 PER-CENT).              Sigma1

*UNRESOLVED RESONANCE REGION COPIED.    Sigma1

*1/V EXTENSION OF CROSS SECTIONS        Sigma1

OUTSIDE OF TABULATED ENERGY RANGE AND  Sigma1

INTO UNRESOLVED ENERGY RANGE.          Sigma1

VERSION 83-2 (OCTOBER 1983)*IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 84-1 (APRIL 1984)  *IMPROVED NUMERICAL STABILITY.          Sigma1

*PARTIAL EVALUATION TREATMENT.          Sigma1

VERSION 85-1 (APRIL 1985)  *ITERATE TO CONVERGENCE (USING THE SAME Sigma1

ENERGY GRID FOR HOT CROSS SECTION AS   Sigma1

COLD CROSS SECTIONS WAS FOUND TO BE    Sigma1

INACCURATE).                           Sigma1

*NEW FASTER HIGH ENERGY BROADENING.     Sigma1

*UPDATED FOR ENDF/B-VI FORMATS.         Sigma1

*SPECIAL I/O ROUTINES TO GUARANTEE      Sigma1

ACCURACY OF ENERGY.                    Sigma1

*DOUBLE PRECISION TREATMENT OF ENERGY   Sigma1

(REQUIRED FOR NARROW RESONANCES).      Sigma1

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Sigma1

VERSION 86-1 (JANUARY 1986)*ENERGY DEPENDENT SCATTERING RADIUS     Sigma1

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Sigma1

FILE NAMES (SEE, SUBROUTINE FILEIO     Sigma1

FOR DETAILS).                          Sigma1

*IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Sigma1

INSURE PROGRAM WILL NOT DO ANYTHING    Sigma1

CRAZY.                                 Sigma1

*UPDATED TO USE NEW PROGRAM CONVERT     Sigma1

KEYWORDS.                              Sigma1

*ADDED LIVERMORE CIVIC COMPILER         Sigma1

CONVENTIONS.                           Sigma1

VERSION 90-1 (JUNE 1990)   *UPDATED BASED ON USER COMMENTS         Sigma1

*ADDED FORTRAN SAVE OPTION              Sigma1

*NEW MORE CONSISTENT ENERGY OUTPUT      Sigma1

ROUTINES                               Sigma1

VERSION 91-1 (JULY 1991)   *WARNING...INPUT PARAMETER FORMAT       Sigma1

HAS BEEN CHANGED - SEE BELOW FOR       Sigma1

DETAILS.                               Sigma1

*ADDED CHARGED PARTICLE PROJECTILES     Sigma1

*OUTPUT ENERGY RANGE IS ALWAYS AT       Sigma1

LEAST AS LARGE AS INPUT ENERGY RANGE.  Sigma1

*NO 1/V EXTENSION OF CROSS SECTIONS     Sigma1

FROM UNRESOLVED ENERGY RANGE.          Sigma1

VERSION 92-1 (JANUARY 1992)*INSURE MINIMUM AND MAXIMUM CROSS       Sigma1

SECTIONS ARE ALWAYS KEPT (NOT THINNED) Sigma1

*MT=19 (FIRST CHANCE FISSION) TREATED   Sigma1

THE SAME AS FISSION.                   Sigma1

*VARIABLE MINIMUM CROSS SECTION OF      Sigma1

INTEREST - TO ALLOW SMALL CROSS        Sigma1

SECTIONS NEAR THRESHOLDS TO BE         Sigma1

TREATED PROPERLY.                      Sigma1

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Sigma1

TO CALCULATIONS.                       Sigma1

*COMPLETELY CONSISTENT I/O AND ROUNDING Sigma1

ROUTINES - TO MINIMIZE COMPUTER        Sigma1

DEPENDENCE.                            Sigma1

VERSION 92-2 (JULY 1992)   *CORRECTED BUG ASSOCIATED WITH          Sigma1

THRESHOLD REACTIONS.                   Sigma1

*UNRESOLVED REGION COPIED WITHOUT       Sigma1

THINNING (IT SHOULD BE EXACTLY THE     Sigma1

SAME AT ALL TEMPERATURES).             Sigma1

*NO THINNING OF REACTIONS (MT) THAT     Sigma1

WERE NOT BROADENED.                    Sigma1

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 2004          Sigma1

TO 24000 ENERGY PONTS.                 Sigma1

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Sigma1

TO ALLOW ACCESS TO FILE STRUCTURES     Sigma1

(WARNING - INPUT PARAMETER FORMAT      Sigma1

HAS BEEN CHANGED)                      Sigma1

*CLOSE ALL FILES BEFORE TERMINATING     Sigma1

(SEE, SUBROUTINE ENDIT)                Sigma1

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Sigma1

*IMPROVED COMPUTER INDEPENDENCE        Sigma1

*ALL DOUBLE PRECISION                  Sigma1

*ON SCREEN OUTPUT                      Sigma1

*UNIFORM TREATMENT OF ENDF/B I/O       Sigma1

*IMPROVED OUTPUT PRECISION             Sigma1

*DEFINED SCRATCH FILE NAMES            Sigma1

*ALWAYS INCLUDE THERMAL VALUE          Sigma1

VERSION 97-1 (APRIL 1997)   *OPTIONALLY SET NEGATIVE CROSS         Sigma1

SECTIONS = 0 ON INPUT AND             Sigma1

OUTPUT.                               Sigma1

*INCREASED PAGE SIZE FROM 24000        Sigma1

TO 60000 ENERGY POINTS.               Sigma1

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Sigma1

POINT READ FOR MORE DIGITS            Sigma1

*UPDATED TEST FOR ENDF/B FORMAT        Sigma1

VERSION BASED ON RECENT FORMAT CHANGE Sigma1

*TREAT LOW ENERGY INITIAL CROSS        Sigma1

SECTIONS AS LOG-LOG INTERPOLABLE      Sigma1

*CONSTANT (RATHER THAN 1/V) EXTENSION  Sigma1

TO HIGHER ENERGY.                     Sigma1

*UPDATED CONSTANTS BASED ON CSEWG      Sigma1

SUBCOMMITTEE RECOMMENDATIONS          Sigma1

*GENERAL IMPROVEMENTS BASED ON         Sigma1

USER FEEDBACK                         Sigma1

VERSION 99-2 (JUNE 1999)    *EXTENDED RANGE OF INTEGRALS FROM 4    Sigma1

TO 5 UNITS ON EACH SIDE OF ENERGY     Sigma1

POINT TO ALLOW FOR LARGER VARIATION   Sigma1

IN THE LOCAL CROSS SECTION            Sigma1

*ASSUME ENDF/B-VI, NOT V, IF MISSING   Sigma1

MF=1, MT-451.                         Sigma1

VERSION 99-3 (OCTOBER 1999))*IMPROVED ERFC FUNCTION DEFINITION.    Sigma1

I THANK BOB MACFARLANE (LANL) FOR     Sigma1

SUPPLYING A MORE ACCURATE ERFC        Sigma1

FUNCTION.                             Sigma1

VERS. 2000-1 (FEBRUARY 2000)*CORRECTED LOW ENERGY INTERPOLATION    Sigma1

FOR NON-POSITIVE CROSS SECTIONS       Sigma1

*GENERAL IMPROVEMENTS BASED ON         Sigma1

USER FEEDBACK                         Sigma1

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Sigma1

VERS. 2004-1 (JAN. 2004)    *OPTIONALLY IGNORE UNRESOLVED REGION   Sigma1

*CORRECTED PROBLEM AT THE RESOLVED/    Sigma1

UNRESOLVED ENERGY BOUNDARY.           Sigma1

*CORRECTED HIGH ENERGY CONSTANT CROSS  Sigma1

SECTION EXTENSION.                    Sigma1

*TIGHTER CRITERIA FOR INITIAL ENERGY   Sigma1

POINT SPACING                         Sigma1

*TEMPERATURE DEPENDENT ENERGY POINT    Sigma1

SPACING.                              Sigma1

*ADDED NEW REICH-MOORE (LRF=7) TO      Sigma1

FILE2 TO ALLOW COPY TO FIND ANY       Sigma1

FOLLOWING UNRESOLVED PARAMETERS       Sigma1

VERS. 2005-1 (JUNE 2005)    *CORRECTED ERROR IN EHOT3 EQUIVALENCE  Sigma1

TO EHOT - THIS ONLY EFFECTS VERY BIG  Sigma1

OUTPUT FILES.                         Sigma1

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Sigma1

*INCREASED PAGE SIZE FROM 60,000       Sigma1

TO 360,000 ENERGY POINTS.             Sigma1

Sigma1

Acknowledgement 2004                                               Sigma1

--------------------                                               Sigma1

Currently almost all improvements to this code are based upon      Sigma1

feedback from code users who report problems. This feedback        Sigma1

benefits ALL users of this code, and ALL users are encouraged      Sigma1

to report problems.                                                Sigma1

Sigma1

Improvements on the 2004 version of this code based on user        Sigma1

feedback including,                                                Sigma1

1) Bret Beck  - reported a problem at the resolved/unresolved      Sigma1

energy boundary.                                   Sigma1

2) S. Ganesan - reported a problem for small temperature changes.  Sigma1

Sigma1

OWNED, MAINTAINED AND DISTRIBUTED BY                               Sigma1

------------------------------------                               Sigma1

THE NUCLEAR DATA SECTION                                           Sigma1

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Sigma1

P.O. BOX 100                                                       Sigma1

A-1400, VIENNA, AUSTRIA                                            Sigma1

EUROPE                                                             Sigma1

Sigma1

ORIGINALLY WRITTEN BY                                              Sigma1

------------------------------------                               Sigma1

DERMOTT E. CULLEN                                                  Sigma1

UNIVERSITY OF CALIFORNIA                                           Sigma1

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Sigma1

L-159                                                              Sigma1

P.O. BOX 808                                                       Sigma1

LIVERMORE, CA 94550                                                Sigma1

U.S.A.                                                             Sigma1

TELEPHONE  925-423-7359                                            Sigma1

E. MAIL    CULLEN1@LLNL.GOV                                        Sigma1

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Sigma1

Sigma1

AUTHORS MESSAGE                                                    Sigma1

---------------                                                    Sigma1

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Sigma1

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Sigma1

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Sigma1

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Sigma1

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Sigma1

Sigma1

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Sigma1

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Sigma1

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Sigma1

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Sigma1

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Sigma1

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Sigma1

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Sigma1

COMPUTER.                                                          Sigma1

Sigma1

PURPOSE                                                            Sigma1

-------                                                            Sigma1

THIS PROGRAM IS DESIGNED TO DOPPLER BROADEN NEUTRON INDUCED        Sigma1

CROSS SECTIONS. EACH SECTION OF CROSS SECTIONS (FILE 3) IS READ    Sigma1

FROM THE ENDF/B FORMAT. THE DATA IS DOPPLER BROADENED, THINNED     Sigma1

AND OUTPUT IN THE ENDF/B FORMAT.                                   Sigma1

Sigma1

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY  Sigma1

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE    Sigma1

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                             Sigma1

Sigma1

ENDF/B FORMAT                                                      Sigma1

-------------                                                      Sigma1

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Sigma1

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Sigma1

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II, III, IV OR V FORMAT).    Sigma1

Sigma1

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Sigma1

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Sigma1

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE  Sigma1

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Sigma1

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Sigma1

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL   Sigma1

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Sigma1

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Sigma1

Sigma1

ALL CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE TABULATED Sigma1

AND LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B      Sigma1

INTERPOLATION LAW 2). FILE 3 CROSS SECTIONS MAY BE MADE LINEARLY   Sigma1

INTERPOLABLE BY USING PROGRAM LINEAR (UCRL-50400, VOL.17, PART A). Sigma1

FILE 2 RESONANCE PARAMETERS MAY BE USED TO RECONSTRUCT ENERGY      Sigma1

DEPENDENT CROSS SECTIONS AND ADD IN FILE 3 BACKGROUND CROSS        Sigma1

SECTIONS TO DEFINE LINEARLY INTERPOLABLE CROSS SECTIONS BY USING   Sigma1

PROGRAM RECENT (UCRL-50400, VOL. 17, PART C). IF THIS PROGRAM      Sigma1

FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT LINEARLY INTERPOLABLE Sigma1

THIS PROGRAM WILL TERMINATE EXECUTION.                             Sigma1

Sigma1

UNRESOLVED RESONANCE REGION                                        Sigma1

---------------------------                                        Sigma1

IN THE UNRESOLVED RESONANCE REGION IT IS NOT POSSIBLE TO EXACTLY   Sigma1

DEFINE THE ENERGY DEPENDENCE OF THE CROSS SECTIONS. THE AVERAGE    Sigma1

WIDTHS AND SPACINGS GIVEN IN ENDF/B ARE ONLY ADEQUATE TO DEFINE    Sigma1

AVERAGE VALUES OF THE CROSS SECTIONS. THEREFORE ALL CROSS SECTIONS Sigma1

IN THE ENDF/B FORMAT FOR THE UNRESOLVED REGION ARE REALLY AVERAGE  Sigma1

VALUES WHICH CANNOT BE DOPPLER BROADENED USING THE SIGMA1 METHOD   Sigma1

(WHICH REQUIRES TABULATED, LINEARLY INTERPOLABLE, ENERGY DEPENDENT Sigma1

CROSS SECTIONS.                                                    Sigma1

Sigma1

THEREFORE,                                                         Sigma1

(1) ALL TABULATED POINTS WITHIN THE UNRESOLVED RESONANCE REGION    Sigma1

WILL BE COPIED, WITHOUT MODIFICATION OR BROADENING. ADOPTION OF    Sigma1

THIS CONVENTION WILL ALLOW SUBSEQUENT PROGRAMS TO PROPERLY DEFINE  Sigma1

SELF-SHIELDED, DOPPLER BROADENED CROSS SECTIONS IN THE UNRESOLVED  Sigma1

RESONANCE REGION.                                                  Sigma1

(2) CROSS SECTIONS WILL BE EXTENDED AS 1/V ABOVE THE UPPER ENERGY  Sigma1

LIMIT OF THE RESOLVED RESONANCE REGION AND BELOW THE LOWER ENERGY  Sigma1

LIMIT OF THE CONTINUUUM REGION (I.E. INTO THE UNRESOLVED           Sigma1

RESONANCE REGION). THIS CONVENTION WILL GUARANTEE A SMOOTH         Sigma1

BEHAVIOR CLOSE TO THE UNRESOLVED RESONANCE REGION BOUNDARIES.      Sigma1

Sigma1

OUTPUT FORMAT                                                      Sigma1

-------------                                                      Sigma1

IN THIS VERSION OF SIGMA1 ALL FILE 3 ENERGIES WILL BE OUTPUT IN    Sigma1

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Sigma1

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Sigma1

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Sigma1

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Sigma1

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Sigma1

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Sigma1

JUST DUE TO TRANSLATION OF THE ENERGIES TO THE ENDF/B FORMAT.      Sigma1

Sigma1

CONTENTS OF OUTPUT                                                 Sigma1

------------------                                                 Sigma1

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE BROADENED FILE 3       Sigma1

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO     Sigma1

INCLUDED.                                                          Sigma1

Sigma1

DOCUMENTATION                                                      Sigma1

-------------                                                      Sigma1

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Sigma1

BY THE ADDITION OF THREE COMMENTS CARDS AT THE END OF EACH         Sigma1

HOLLERITH SECTION IN THE FORM                                      Sigma1

Sigma1

***************** PROGRAM SIGMA1 (2007-1) ***************          Sigma1

DATA DOPPLER BROADENED TO 300.0   KELVIN AND                       Sigma1

DATA THINNED TO WITHIN AN ACCURACY OF  0.1 PER-CENT                Sigma1

Sigma1

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR,RECENT AND GROUPY)  Sigma1

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Sigma1

THE DATA.                                                          Sigma1

Sigma1

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Sigma1

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Sigma1

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Sigma1

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Sigma1

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Sigma1

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Sigma1

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Sigma1

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Sigma1

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Sigma1

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Sigma1

Sigma1

REACTION INDEX                                                     Sigma1

--------------                                                     Sigma1

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Sigma1

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Sigma1

Sigma1

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Sigma1

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Sigma1

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Sigma1

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Sigma1

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Sigma1

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Sigma1

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Sigma1

Sigma1

SECTION SIZE                                                       Sigma1

------------                                                       Sigma1

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Sigma1

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Sigma1

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Sigma1

Sigma1

SELECTION OF DATA                                                  Sigma1

-----------------                                                  Sigma1

THE PROGRAM SELECTS MATERIALS TO BE BROADENED BASED EITHER ON      Sigma1

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Sigma1

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Sigma1

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Sigma1

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Sigma1

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Sigma1

Sigma1

ENERGY GRID OF BROADENED DATA                                      Sigma1

-----------------------------                                      Sigma1

THE ENERGY GRID FOR THE DOPPLER BROADENED CROSS SECTIONS IS        Sigma1

SELECTED TO INSURE THAT THE BROADENED DATA IS LINEAR-LINEAR        Sigma1

INTERPOLABLE. AS SUCH THE ENERGY GRID FOR THE BROADENED DATA       Sigma1

MAY NOT BE THE SAME AS THE ENERGY GRID FOR THE ORIGINAL            Sigma1

UNBROADENED DATA. GENERALLY AFTER BROADENING THERE WILL BE         Sigma1

FEWER DATA POINTS IN THE RESONANCE REGION, BUT AT LOW ENERGY       Sigma1

THERE MAY BE MORE POINTS, DUE TO THE 1/V LOW ENERGY EFFECT         Sigma1

CREATED BY DOPPLER BROADENING.                                     Sigma1

Sigma1

EFFECTIVE TEMERATURE INCREASE                                      Sigma1

-----------------------------                                      Sigma1

IF THE ORIGINAL DATA IS NOT AT ZERO KELVIN THE PROGRAM WILL        Sigma1

BROADEN THE DATA BY THE EFFECTIVE TEMPERATURE DIFFENCE TO THE      Sigma1

FINAL TEMPERATURE. IF THE DATA IS ALREADY AT A TEMPERATURE THAT    Sigma1

IS HIGHER THAN THE FINAL TEMPERATURE DOPPLER BROADENING IS         Sigma1

NATURALLY NOT PERFORMED AND THE TEMPERATURE IN THE SECTION IS LEFT Sigma1

AT ITS ORIGINAL VALUE.                                             Sigma1

Sigma1

MULTIPLE FINAL TEMPERATURES                                        Sigma1

---------------------------                                        Sigma1

THE PRESENT VERSION ONLY DOPPLER BROADENS TO ONE FINAL TEMPERATURE Sigma1

(IF THERE IS SUFFICIENT INTEREST EXPRESSED BY USERS FUTURE         Sigma1

VERSION MAY BROADEN TO MULTIPLE TEMPERATURES. PLEASE               Sigma1

CONTACT THE AUTHOR IF YOU ARE INTERESTED IN A MULTIPLE             Sigma1

TEMPERATURE OPTION).                                               Sigma1

Sigma1

PROGRAM OPERATION                                                  Sigma1

-----------------                                                  Sigma1

EACH SECTION OF FILE 3 DATA IS CONSIDERED SEPERATELY. THE DATA     Sigma1

IS READ AND DOPPLER BROADENED A PAGE AT A TIME (ONE PAGE IS        Sigma1

60000 DATA POINTS). UP TO THREE PAGES OF DATA MAY BE IN THE CORE   Sigma1

AT ANY GIVEN TIME, THE PAGE BEING BROADENED, THE PAGE BELOW IT     Sigma1

IN ENERGY AND THE PAGE ABOVE IT IN ENERGY. AFTER A PAGE HAS BEEN   Sigma1

BROADENED IT IS THINNED, IF THE ENTIRE SECTION CONTAINS ONLY       Sigma1

ONE PAGE OR LESS, IT WILL STILL BE CORE RESIDENT AND WILL BE       Sigma1

WRITTEN DIRECTLY FROM CORE TO THE OUTPUT TAPE. IF THE BROADENED,   Sigma1

THINNED SECTION IS LARGER THAN A PAGE, AFTER A PAGE HAS BEEN       Sigma1

BROADENED AND THINNED IT IS WRITTEN TO A SCRATCH FILE. AFTER THE   Sigma1

ENTIRE SECTION HAS BEEN BROADENED AND THINNED THE DATA IS READ     Sigma1

FROM SCRATCH TO CORE, ONE PAGE AT A TIME, THE OUTPUT TO THE OUTPUT Sigma1

TAPE.                                                              Sigma1

Sigma1

ALLOWABLE ERROR                                                    Sigma1

---------------                                                    Sigma1

AFTER DOPPLER BROADENING THE CROSS SECTION IN THE RESONANCE REGION Sigma1

WILL GENERALLY BE MUCH SMOOTHER THAN THE UNBROADENED DATA AND CAN  Sigma1

BE REPRESENTED TO THE SAME ACCURACY BY A SMALLER NUMBER OF ENERGY  Sigma1

POINTS. THEREFORE AFTER DOPPLER BROADENING THE DATA CAN BE THINNED Sigma1

WITH ESSENTIALLY NO LOSE OF INFORMATION.                           Sigma1

Sigma1

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Sigma1

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Sigma1

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Sigma1

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Sigma1

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Sigma1

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Sigma1

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Sigma1

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.     Sigma1

Sigma1

INPUT FILES                                                        Sigma1

-----------                                                        Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

2  INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                     Sigma1

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Sigma1

Sigma1

OUTPUT FILES                                                       Sigma1

------------                                                       Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Sigma1

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Sigma1

Sigma1

SCRATCH FILES                                                      Sigma1

-------------                                                      Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

12  SCRATCH FILE FOR BROADENED DATA                              Sigma1

(BINARY - 180000 WORDS/RECORD - DOUBLE PRECISION/            Sigma1

42000 WORDS/RECORD - SINLGE PRECISION)            Sigma1

Sigma1

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Sigma1

----------------------------------------------------               Sigma1

UNIT  FILE NAME                                                    Sigma1

----  ----------                                                   Sigma1

2   SIGMA1.INP                                                   Sigma1

3   SIGMA1.LST                                                   Sigma1

10   ENDFB.IN                                                     Sigma1

11   ENDFB.OUT                                                    Sigma1

12   (SCRATCH)                                                    Sigma1

Sigma1

INPUT CARDS                                                        Sigma1

-----------                                                        Sigma1

CARD  COLS.  DESCRIPTION                                           Sigma1

----  -----  -----------                                           Sigma1

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                      Sigma1

12-22  MONITOR MODE SELECTOR                                 Sigma1

= 0 - NORMAL OPERATION                                Sigma1

= 1 - MONITOR PROGRESS OF DOPPLER BROADENING OF DATA. Sigma1

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO   Sigma1

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF  Sigma1

POINTS ON SCRATCH AND THE LOWER AND UPPER       Sigma1

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE   Sigma1

USED IN ORDER TO MONITOR THE EXECUTION SPEED    Sigma1

OF LONG RUNNING JOBS).                          Sigma1

23-33  KELVIN TEMPERATURE                                    Sigma1

34-44  MINIMUM CROSS SECTION OF INTEREST                     Sigma1

(DEFAULT VALUE = 1.0E-10 BARNS).                      Sigma1

45-55  NEGATIVE CROSS SECTION TREATMENT                      Sigma1

= 0 - O.K. - NO CHANGE                                Sigma1

= 1 - SET = 0                                         Sigma1

56-66  UNRESOLVED RESONANCE REGION TREATMENT                 Sigma1

= 0 - COPY (NO BROADENING)                            Sigma1

= 1 - IGNORE (BROADEN)                                Sigma1

2   1-60  ENDF/B INPUT DATA FILENAME                            Sigma1

(STANDARD OPTION = ENDFB.IN)                          Sigma1

3   1-60  ENDF/B OUTPUT DATA FILENAME                           Sigma1

(STANDARD OPTION = ENDFB.OUT)                         Sigma1

4-N   1-11  LOWER MAT OR ZA LIMIT                                 Sigma1

12-22  UPPER MAT OR ZA LIMIT                                 Sigma1

UP TO 100 MAT OR ZA RANGES MAY BE SPECIFIED, ONE      Sigma1

RANGE PER CARD. THE LIST OF RANGES IS TERMINATED BY   Sigma1

A BLANK CARD. IF THE UPPER LIMIT IS LESS THAN THE     Sigma1

LOWER LIMIT THE UPPER LIMIT WILL BE SET EQUAL TO THE  Sigma1

LOWER LIMIT. IF THE FIRST REQUEST CARD IS BLANK IT    Sigma1

WILL TERMINATE THE LIST OF REQUESTS AND CAUSE ALL     Sigma1

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).             Sigma1

VARY  1-11  ENERGY FOR ERROR LAW                                  Sigma1

12-22  ERROR FOR ERROR LAW                                   Sigma1

THE ACCEPTABLE LINEARIZING ERROR CAN BE GIVEN AS AN   Sigma1

ENERGY DEPENDENT FUNCTION SPECIFIED BY UP TO 20       Sigma1

(ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION         Sigma1

TABULATE POINTS. ENERGIES MUST BE IN ASCENDING ORDER. Sigma1

THE ERROR LAW IS TERMINATED BY A BLANK CARD. IF THE   Sigma1

FIRST ERROR LAW CARD IS BLANK IT WILL TERMINATE THE   Sigma1

ERROR LAW AND THE ERROR WILL BE TREATED AS ENERGY     Sigma1

INDEPENDENT, EQUAL TO ZERO, WHICH INDICATES THAT THE  Sigma1

BROADENED DATA SHOULD NOT BE THINNED.                 Sigma1

Sigma1

EXAMPLE INPUT NO. 1                                                Sigma1

-------------------                                                Sigma1

BROADEN ALL URANIUM ISOTOPES AND THORIUM-232 TO 300 KELVIN. FROM   Sigma1

0 TO 100 EV THIN OUTPUT DATA TO 0.1 PER-CENT ACCURACY. FROM 100 EV Sigma1

TO 1 KEV VARY THE ERROR BETWEEN 0.1 AND 1 PER-CENT. ABOVE 1 KEV    Sigma1

USE 1 PER-CENT ACCURACY.                                           Sigma1

Sigma1

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Sigma1

Sigma1

THE FOLLOWING 11 CARDS ARE REQUIRED                                Sigma1

Sigma1

1          0 3.00000+ 2                                       Sigma1

ENDFB.IN                                                               Sigma1

ENDFB.OUT                                                              Sigma1

92000      92999                                                  Sigma1

90232               (UPPER LIMIT WILL AUTOMATICALLY BE DEFINED)   Sigma1

(BLANK CARD INDICATES END OF REQUEST LIST)    Sigma1

0.00000+ 0 1.00000-03                                                  Sigma1

1.00000+ 2 1.00000-03                                                  Sigma1

1.00000+ 3 1.00000-02                                                  Sigma1

1.00000+ 9 1.00000-02                                                  Sigma1

(BLANK CARD INDICATES END OF ERROR LAW)       Sigma1

Sigma1

EXAMPLE INPUT NO. 2                                                Sigma1

-------------------                                                Sigma1

BROADEN ALL DATA TO 300 KELVIN AND DO NOT THIN THE BROADEN DATA.   Sigma1

ALL OF THE STANDARD OPTION MAY BE INVOKED MERELY BY SPECIFYING     Sigma1

THE KELVIN TEMPERATURE ON THE FIRST CARD. ALL OTHER FIELDS MAY     Sigma1

BE LEFT BLANK.                                                     Sigma1

Sigma1

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Sigma1

THEN USE STANDARD FILENAMES.                                       Sigma1

Sigma1

THE FOLLOWING 5 CARDS ARE REQUIRED                                 Sigma1

Sigma1

3.00000+ 2                                       Sigma1

(USE STANDARD FILENAME = ENDFB.IN)               Sigma1

(USE STANDARD FILENAME = ENDFB.OUT)              Sigma1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Sigma1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)       Sigma1

Sigma1

EXAMPLE INPUT NO. 3                                                Sigma1

-------------------                                                Sigma1

THE SAME AS ABOVE, ONLY DEFINE THE MINIMUM CROSS SECTION OF        Sigma1

INTEREST TO BE 1.0E-30 BARNS (INSTEAD OF THE DEFAULT VALUE OF      Sigma1

1.0E-10).                                                          Sigma1

Sigma1

READ ENDF/B DATA FROM \ENDFB6\RECENT\ZA092238 AND WRITE ENDF/B     Sigma1

DATA TO \ENDFB\SIGMA1\ZA092238                                     Sigma1

Sigma1

THE FOLLOWING 5 CARDS ARE REQUIRED                                 Sigma1

Sigma1

3.00000+ 2 1.00000-30                            Sigma1

\ENDFB6\RECENT\ZA092238                                                Sigma1

\ENDFB6\SIGMA1\ZA092238                                                Sigma1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Sigma1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)       Sigma1

Sigma1

======================================================================= Sigma1

======================================================================= Sixpak

Sixpak

PROGRAM SIXPAK                                                     Sixpak

================================================================== Sixpak

VERSION 92-1 (JANUARY 1992)                                        Sixpak

VERSION 92-2 (FEBRUARY 1992)*INCREASED CORE ALLOCATION TO          Sixpak

ACCOMMODATE JEF AND EFF EVALUATIONS.  Sixpak

VERSION 92-3 (APRIL 1992)   *ADDED ADDITIONAL DATA TESTS.          Sixpak

VERSION 92-4 (SEPT. 1992)   *CORRECTED KALBACH-MANN CALCULATIONS.  Sixpak

*FOR PHOTON PRODUCTION OUTPUT MF=12    Sixpak

(MULTIPLICITY), MF=14 (ISOTROPIC      Sixpak

ANGULAR DISTRIBUTIONS) AND MF=15      Sixpak

(SPECTRA) - PREVIOUSLY ONLY MF=15.    Sixpak

*FIRST ORDER CORRECTIONS TRANSFORMING  Sixpak

CENTER-OF-MASS SPECTRA TO LAB SYSTEM  Sixpak

FOR OUTPUT IN MF=5                    Sixpak

*CORRECTED ISOTROPIC ANGULAR           Sixpak

DISTRIBUTION FLAG (LI)                Sixpak

VERSION 94-1 (JANUARY 1994) *VARIABLE ENDF/B INPUT DATA FILENAME   Sixpak

TO ALLOW ACCESS TO FILE STRUCTURES    Sixpak

(WARNING - INPUT PARAMETER FORMAT     Sixpak

HAS BEEN CHANGED)                     Sixpak

*CLOSE ALL FILES BEFORE TERMINATING    Sixpak

(SEE, SUBROUTINE ENDIT)               Sixpak

*INCREASED MAXIMUM TABLE SIZE FROM     Sixpak

2000 TO 6000.                         Sixpak

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Sixpak

*IMPROVED COMPUTER INDEPENDENCE        Sixpak

*ALL DOUBLE PRECISION                  Sixpak

*ON SCREEN OUTPUT                      Sixpak

*UNIFORM TREATMENT OF ENDF/B I/O       Sixpak

*IMPROVED OUTPUT PRECISION             Sixpak

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Sixpak

POINT READ FOR MORE DIGITS            Sixpak

*UPDATED TEST FOR ENDF/B FORMAT        Sixpak

VERSION BASED ON RECENT FORMAT CHANGE Sixpak

*GENERAL IMPROVEMENTS BASED ON         Sixpak

USER FEEDBACK                         Sixpak

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Sixpak

MF=1, MT-451.                         Sixpak

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Sixpak

USER FEEDBACK                         Sixpak

VERS. 2002-1 (JANUARY 2002) *CORRECTED ANGULAR DISTRIBUTION (MF=4) Sixpak

OUTPUT TO INSURE USED FIELDS ARE 0    Sixpak

(MAY 2002)     *OPTIONAL INPUT PARAMETERS             Sixpak

(NOV. 2002)    *EXTENDED TO ALLOW CHARGED PARTICLE    Sixpak

ANGULAR DISTRIBUTION IN MF=4 -        Sixpak

WARNING - STRICTLY SPEAKING THIS IS   Sixpak

NOT LEGAL, SINCE MF=4 IS SUPPOSED TO  Sixpak

BE USED ONLY FOR NEUTRON ANGULAR      Sixpak

DISTRIBUTIONS - BUT WHERE MT MAKES    Sixpak

IT OBVIOUS THAT THE OUTGOING PARTICLE Sixpak

IS NOT A NEUTRON HOPEFULLY IT WILL    Sixpak

NOT CAUSE A PROBLEM IF MF=4 IS USED   Sixpak

FOR CHARGED PARTICLES.                Sixpak

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Sixpak

*INCREASED MAXIMUM TABLE SIZE FROM     Sixpak

6,000 TO 12,000.                      Sixpak

*ADDED DUMMY A FOR ELEMENTS            Sixpak

*CORRECTED OUTPUT INTERPOLATON LAWS    Sixpak

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Sixpak

*INCREASED MAXIMUM TABLE SIZE FROM     Sixpak

12,000 TO 120,000.                    Sixpak

Sixpak

OWNED, MAINTAINED AND DISTRIBUTED BY                               Sixpak

------------------------------------                               Sixpak

THE NUCLEAR DATA SECTION                                           Sixpak

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Sixpak

P.O. BOX 100                                                       Sixpak

A-1400, VIENNA, AUSTRIA                                            Sixpak

EUROPE                                                             Sixpak

Sixpak

ORIGINALLY WRITTEN BY                                              Sixpak

------------------------------------                               Sixpak

DERMOTT E. CULLEN                                                  Sixpak

UNIVERSITY OF CALIFORNIA                                           Sixpak

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Sixpak

L-159                                                              Sixpak

P.O. BOX 808                                                       Sixpak

LIVERMORE, CA 94550                                                Sixpak

U.S.A.                                                             Sixpak

TELEPHONE  925-423-7359                                            Sixpak

E. MAIL    CULLEN1@LLNL.GOV                                        Sixpak

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Sixpak

Sixpak

COLLABORATION                                                      Sixpak

================================================================== Sixpak

DEVELOPED IN COLLABORATION WITH,                                   Sixpak

Sixpak

*THE NATIONAL NUCLEAR DATA CENTER, BROOKHAVEN NATIONAL LAB         Sixpak

Sixpak

*THE NUCLEAR DATA SECTION, IAEA, VIENNA, AUSTRIA                   Sixpak

Sixpak

*CENTRO TECNICO AEROSPACIAL, SAO JOSE DOS CAMPOS, BRAZIL           Sixpak

Sixpak

AS A PART OF AN INTERNATIONAL PROJECT ON THE EXCHANGE OF           Sixpak

NUCLEAR DATA                                                       Sixpak

Sixpak

ACKNOWLEDGEMENT (VERSION 92-1)                                     Sixpak

================================================================== Sixpak

THE AUTHOR THANKS SOL PEARLSTEIN (BROOKHAVEN NATIONAL LAB) FOR     Sixpak

SIGNIFICANTLY CONTRIBUTING TOWARD IMPROVING THE ACCURACY AND       Sixpak

COMPUTER INDEPENDENCE OF THIS CODE - THANKS, SOL                   Sixpak

Sixpak

ACKNOWLEDGEMENT (VERSION 92-4)                                     Sixpak

================================================================== Sixpak

THE AUTHOR THANKS BOB MACFARLANE (LOS ALAMOS) FOR SUGGESTING HOW   Sixpak

TO PROPERLY OUTPUT THE PHOTON PRODUCTION DATA TO PUT IT INTO       Sixpak

EXACTLY THE FORM NEEDED FOR USE IN PROCESSING CODES.               Sixpak

Sixpak

THE AUTHOR THANKS CHRIS DEAN (WINFRITH) FOR POINTING OUT ERRORS    Sixpak

IN THE EARLIER TREATMENT OF THE KALBACH-MANN FORMALISM AND IN      Sixpak

THE DEFINITION OF THE ISOTROPIC ANGULAR DISTRIBUTION FLAG (LI).    Sixpak

Sixpak

AUTHORS MESSAGE                                                    Sixpak

================================================================== Sixpak

THE COMMENTS BELOW SHOULD BE CONSIDERED THE LATEST DOCUMENTATION   Sixpak

INCLUDING ALL RECENT IMPROVEMENTS. PLEASE READ ALL OF THESE        Sixpak

COMMENTS BEFORE IMPLEMENTING AND USING THESE CODES.                Sixpak

Sixpak

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Sixpak

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Sixpak

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Sixpak

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Sixpak

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Sixpak

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Sixpak

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Sixpak

COMPUTER.                                                          Sixpak

Sixpak

PURPOSE                                                            Sixpak

================================================================== Sixpak

1) CHECK ALL DOUBLE-DIFFERENTIAL DATA (MF=6)                       Sixpak

Sixpak

2) OUTPUT EQUIVALENT MF = 4, 5, 12, 14 AND 15 DATA.                Sixpak

Sixpak

DATA CHECKING                                                      Sixpak

================================================================== Sixpak

ALL OF THE ENDF/B-VI MF=6 DATA IS CHECKED - FOR DETAILS SEE BELOW. Sixpak

Sixpak

THE MF=6 DATA IS NOT CORRECTED AND OUTPUT IN THE ENDF/B FORMAT.    Sixpak

IT IS MERELY CHECKED. IF ERRORS ARE FOUND IT IS UP TO THE USER     Sixpak

TO TAKE CORRECTIVE ACTION ON THE MF=6 DATA.                        Sixpak

Sixpak

IN CONTRAST WHEN PROBLEMS ARE FOUND IN DATA WHICH WILL BE OUTPUT   Sixpak

IN THE ENDF/B FORMAT (MF=4, 5, 12, 14 AND 15), WHENEVER POSSIBLE   Sixpak

CORRECTIVE ACTION WILL BE TAKEN.                                   Sixpak

Sixpak

FURTHER CHECKS AND CORRECTIONS                                     Sixpak

================================================================== Sixpak

ONCE THE DATA HAS BEEN OUTPUT IN MF = 4, 5, 12, 14 AND 15 FORMATS  Sixpak

FURTHER CORRECTIVE ACTION CAN BE TAKEN AS FOLLOWS,                 Sixpak

Sixpak

PROGRAM LEGEND                                                     Sixpak

==============                                                     Sixpak

CAN BE USED TO CORRECT ANGULAR DISTRIBUTIONS WHICH ARE NEGATIVE,   Sixpak

TO CONVERT FROM LEGENDRE COEFFICIENTS TO TABULATED ANGULAR         Sixpak

DISTRIBUTIONS AND GENERALLY PERFORM MORE EXTENSIVE TESTS OF        Sixpak

ALL MF=4 DATA.                                                     Sixpak

Sixpak

PROGRAM EVALPLOT                                                   Sixpak

================                                                   Sixpak

VERSION 92-1 AND LATER VERSIONS CAN PLOT ALL OF THE MF=4, 5 AND 15 Sixpak

DATA OUTPUT BY THIS CODE. EARLIER VERSIONS CAN PLOT MF=4 AND 5.    Sixpak

GRAPHICS IS AN EXCELLENT WAY TO CHECK THIS DATA.                   Sixpak

Sixpak

PROGRAM PLOTTAB                                                    Sixpak

===============                                                    Sixpak

THIS IS A GENERAL PLOTTING PROGRAM AND THERE IS AN INTERFACE IN    Sixpak

THIS CODE TO PRODUCE OUTPUT FOR ANY MF=6 DATA IN THE PLOTTAB       Sixpak

INPUT FORMAT. THIS PROGRAM CAN BE USED TO CHECK ALL OF THE MF=6    Sixpak

DATA AS WELL AS THE EQUIVALENT MF=4, 5, 12, 14 AND 15 DATA - AS    Sixpak

WELL AS COMPARING THE ORIGINAL MF=6 AND EQUIVALENT DATA.           Sixpak

Sixpak

DATA OUTPUT                                                        Sixpak

================================================================== Sixpak

THE ENDF/B MF=4, 5, 12, 14 AND 15 FORMATS ONLY ALLOW FOR NEUTRONS  Sixpak

INCIDENTS                                                          Sixpak

Sixpak

THE ENDF/B MF=4 AND 5 FORMATS ONLY ALLOW FOR NEUTRONS OUTGOING.    Sixpak

Sixpak

THE ENDF/B MF=12, 14 AND 15 ONLY ALLOWS FOR PHOTONS OUTGOING.      Sixpak

Sixpak

THESE ARE THE ONLY COMBINATIONS OF DATA OUTPUT BY THIS CODE.       Sixpak

Sixpak

ALL OTHER COMBINATIONS OF INCIDENT AND OUTGOING PARTICLES ARE      Sixpak

CHECKED, BUT THE RESULTS CANNOT BE OUTPUT IN THE ENDF/B FORMAT.    Sixpak

HOWEVER, USING THE PLOTTAB INTERFACE BUILT INTO THIS CODE THIS     Sixpak

DATA CAN, AND HAS BEEN, OUTPUT AND CHECKED.                        Sixpak

Sixpak

THE NEUTRON DATA IN MF=4 CAN BE IN THE FORM OF EITHER TABULATED    Sixpak

ANGULAR DISTRIBUTIONS OR LEGENDRE COEFFICIENTS.                    Sixpak

Sixpak

THE NEUTRON (MF=5) OR PHOTON (MF=15) SPECTRA ARE BOTH IN EXACTLY   Sixpak

THE SAME FORMAT = ARBITRARY TABULATED FUNCTIONS - ENDF/B OPTION    Sixpak

LF=1.                                                              Sixpak

Sixpak

ENDF/B DATA OUTPUT ORDER                                           Sixpak

================================================================== Sixpak

ENDF/B DATA IS OUTPUT IN ASCENDING MAT, MF, MT ORDER. IN ORDER TO  Sixpak

ALLOW THIS PROGRAM TO PRODUCE ALL OUTPUT IN A SINGLE PASS THROUGH  Sixpak

THE MF=6 DATA, OUTPUT FOR EACH (MAT, MT) IS OUTPUT TO SEPERATE     Sixpak

FILES FOR MF=4, 5, 12, 14 AND 15.                                  Sixpak

Sixpak

FOR SUBSEQUENT USE THE ENDF/B FORMATTED DATA OUTPUT BY THIS CODE   Sixpak

CAN BE MERGED TOGETHER USING PROGRAM MERGER (CONTAIN THE AUTHOR    Sixpak

OF THIS CODE FOR A COPY OF MERGER), E.G., MERGE MF=12, 14 AND 15   Sixpak

DATA IN ORDER TO THEN CALCULATE PHOTON PRODUCTION DATA OR MF=4     Sixpak

AND 5 CAN BE MERGED TOGETHER TO CALCULATE NEUTRON TRANSFER - OR    Sixpak

ALL OF THEM CAN BE MERGED TOGETHER TO PERFORM NEUTRON AND PHOTON   Sixpak

CALCULATIONS.                                                      Sixpak

Sixpak

CORRELATED (MF=6) VS. UNCORRELATED (MF=4 AND 5) DATA               Sixpak

================================================================== Sixpak

THE ENDF/B DOUBLE DIFFERENTAL = CORRELATED - DATA IN MF=6          Sixpak

REPRESENTS DATA IN THE FORM,                                       Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

SIG(E)      = MF=3 CROSS SECTIONS                                  Sixpak

Y(E)        = YIELD (MULTIPLICITY)                                 Sixpak

G0(E,EP)    = ENERGY SPECTRUM                                      Sixpak

F(E,EP,COS) = ANGULAR DISTRIBUTION                                 Sixpak

Sixpak

IN A SITUATION WHERE YOU HAVE MONOENERGETIC AND MONODIRECTIONAL    Sixpak

NEUTRONS INCIDENT YOU WILL BE ABLE TO OBSERVE CORRELATION EFFECTS  Sixpak

IN THE NEUTRON SPECTRUM AND ANGULAR DISTRIBUTION.                  Sixpak

Sixpak

EVEN IN SITUATIONS WHERE YOU HAVE A NARROW SPECTRUM OF NEUTRONS    Sixpak

THAT ARE HIGHLY DIRECTIONALLY ORIENTED YOU MAY BE ABLE TO OBSERVE  Sixpak

THESE CORRELATION EFFECTS, E.G., A NARROW 14 MEV FUSION SOURCE     Sixpak

INCIDENT ON THE FIRST WALL OF A CTR DEVICE.                        Sixpak

Sixpak

FOR SUCH SITUATIONS USE OF THE CORRELATED (MF=6) DATA IS REQUIRED  Sixpak

IN CALCULATIONS.                                                   Sixpak

Sixpak

HOWEVER, IN MANY APPLICATIONS WHERE THERE IS A BROAD SPECTRUM OF   Sixpak

NEUTRONS AND THE NEUTRON FLUX IS NOT HIGHLY DIRECTIONALLY          Sixpak

ORIENTED, THE NEUTRON MULTIPLICATION, SPECTRUM AND ORIENTATION     Sixpak

CAN BE FAIRLY ACCURATELY CALCULATED WITHOUT CONSIDERING            Sixpak

CORRELATION EFFECTS.                                               Sixpak

Sixpak

THE UNCORRELATED DATA PRODUCED BY THIS CODE REPLACES THE           Sixpak

CORRELATED DATA,                                                   Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

BY THE UNCORRELATED DATA,                                          Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F0(E,COS)                       Sixpak

Sixpak

BY INTEGRATING G0(E,EP)*F(E,EP,COS) OVER SECONDARY ENERGY (EP)     Sixpak

TO DEFINE AN AVERAGE ANGULAR DISTRIBUTION, F0(E,COS).              Sixpak

Sixpak

WHAT IS LOST IN THIS PROCESS IS THE CORRELATION BETWEEN EP AND COS Sixpak

SO THAT IN A TRANSPORT CALCULATION ALL MOMENTS OF THE FLUX WILL    Sixpak

HAVE THE SAME SPECTRUM, G0(E,EP) AND EACH WILL BE EFFECTED BY THE  Sixpak

AVERAGE ANGULAR DISTRIBUTION.                                      Sixpak

Sixpak

FOR APPLICATIONS TO HIGH ENERGY FUSION APPLICATIONS CORRELATED     Sixpak

DATA SHOULD BE USED. HOWEVER, FOR LOWER ENERGY APPLICATIONS,       Sixpak

SUCH AS FISSION REACTORS, IT SHOULD BE ADEQUATE TO USE THE         Sixpak

UNCORRELATED DATA - IN THIS CASE THE MOST IMPORTANT EFFECT         Sixpak

WILL BE THE OVERALL NEUTRON MULTIPLICATION AND SPECTRUM.           Sixpak

Sixpak

AN IMPORTANT CONSIDERATION IN DESIGNING THIS PROGRAM IS THAT       Sixpak

MANY COMPUTER CODES - DATA PROCESSING AND TRANSPORT CODES -        Sixpak

CANNOT USE THE CORRELATED (MF=6) DATA - NOR ARE THEY INTENDED      Sixpak

FOR HIGH ENERGY USE. FOR THESE CODES THE UNCORRELATED DATA         Sixpak

PRODUCED BY THIS CODE SHOULD BE ADEQUATE TO MEET THEIR NEEDS.      Sixpak

Sixpak

WARNING - IT CANNOT BE STRESSED ENOUGH THAT THE OUTPUT OF THIS     Sixpak

CODE SHOULD ONLY BE USED FOR LOW ENERGY APPLICATIONS - FAILURE     Sixpak

TO HEED THIS WARNING CAN LEAD TO COMPLETELY UNRELIABLE RESULTS.    Sixpak

Sixpak

ENDF/B FORMAT                                                      Sixpak

================================================================== Sixpak

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Sixpak

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Sixpak

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Sixpak

Sixpak

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Sixpak

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Sixpak

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Sixpak

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Sixpak

CORRECTLY OUTPUT ON ALL LINES. THE FORMAT OF SECTION MF=1, MT=451  Sixpak

AND ALL SECTIONS OF MF=6 MUST BE CORRECT. THE PROGRAM SKIPS ALL    Sixpak

OTHER SECTIONS OF DATA AND AS SUCH IS INSENSITIVE TO THE FORMAT    Sixpak

OF ALL OTHER SECTIONS.                                             Sixpak

Sixpak

CONTENTS OF OUTPUT                                                 Sixpak

================================================================== Sixpak

5 ENDF/B FORMATTED OUTPUT FILES ARE PRODUCED FOR NEUTRON INCIDENT  Sixpak

DATA,                                                              Sixpak

Sixpak

1) ENDFB.MF4 - ANGULAR DISTRIBUTIONS AND LEGENDRE COEFFICIENTS     Sixpak

FOR NEUTRONS                                        Sixpak

2) ENDFB.MF5 - TABULATED NEUTRON ENERGY SPECTRA                    Sixpak

3) ENDFB.M12 - PHOTON EMISSION MULTIPLICITY                        Sixpak

4) ENDFB.M14 - PHOTON EMISSION ANGULAR DISTRIBUTIONS (ALWAYS       Sixpak

ISOTROPIC)                                          Sixpak

5) ENDFB.M15 - TABULATED PHOTON EMISSION SPECTRA                   Sixpak

Sixpak

EMITTED PARTICLE YIELD                                             Sixpak

================================================================== Sixpak

NEUTRONS                                                           Sixpak

========                                                           Sixpak

IN MF=6 THE YIELD FOR EACH REACTION IS THE ACTUAL MULTIPLICITY OF  Sixpak

THE REACTION, E.G., (N,2N) = 2. IN USING MF=4 AND 5 DATA THE       Sixpak

ENDF/B CONVENTION IS THAT THE MULTIPLICITY IS IMPLIED BY THE       Sixpak

MT NUMBER, E.G., MT=16 = (N,2N) = 2.                               Sixpak

Sixpak

THE ONLY EXCEPT IN ENDF/B-VI IS MT=201 = TOTAL NEUTRON PRODUCTION  Sixpak

WHERE AN ACTUAL ENERGY DEPENDENT YIELD IS INCLUDED IN MF=6.        Sixpak

HOWEVER, IN THIS CASE THE MF=3 CROSS SECTION INCLUDES THE          Sixpak

MULTIPLICITY (S. PEARLSTEIN, PRIVATE COMMUNICATION, JAN. 1992),    Sixpak

SIG(MT=201) = 2*SIG(N,2N)+3*SIG(N,3N).....ETC.                     Sixpak

Sixpak

SO THAT FOR ALL ENDF/B-VI DATA AS OF JANUARY 1992 THE MF=4 AND 5   Sixpak

DATA OUTPUT BY THIS CODE CAN BE USED IN CONJUNCTION WITH THE MF=3  Sixpak

CROSS SECTIONS - WITHOUT ANY REFERENCE TO THE MF=6 YIELD.          Sixpak

Sixpak

PHOTONS                                                            Sixpak

=======                                                            Sixpak

UNLIKE THE NEUTRONS WHERE WITH ONLY ONE EXCEPTION (MT=201) THE     Sixpak

MF=6 YIELD IS ENERGY INDEPENDENT, IN THE CASE OF PHOTON EMISSION   Sixpak

ALMOST ALL OF THE PHOTONS HAVE AN ENERGY DEPENDENT YIELD.          Sixpak

Sixpak

THIS PROGRAM WILL OUTPUT THE PHOTON MULTIPLICITY IN MF=12 AND      Sixpak

INDICATE THAT THERE IS A NORMALIZED DISTRIBUTION IN MF=15          Sixpak

(LF=1 IN MF=12).                                                   Sixpak

Sixpak

THIS PROGRAM WILL OUTPUT THE NORMALIZED PHOTON SPECTRA IN MF=15.   Sixpak

CONTINUOUS ENERGY SPECTRA AND DISCRETE PHOTONS WILL ALL BE OUTPUT  Sixpak

AS NORMALIZED SPECTRA.                                             Sixpak

Sixpak

THIS PROGRAM WILL ALSO OUTPUT MF=14 PHOTON ANGULAR DISTRIBUTION    Sixpak

DATA, ALWAYS USING THE ISOTROPIC FLAG TO MINIMIZE OUTPUT.          Sixpak

Sixpak

WARNING OF ENERGY DEPENDENT YIELD                                  Sixpak

=================================                                  Sixpak

THIS PROGRAM WILL PRINT A WARNING MESSAGE IF A SECTION OF DATA     Sixpak

BEING OUTPUT IN THE ENDF/B FORMAT HAS AN ENERGY DEPENDENT MF=6     Sixpak

YIELD AND THE EMITTED PARTICLE IS A NEUTRON - SINCE THE ENDF/B     Sixpak

CONVENTION IS THAT FOR EACH MT NUMBER THE MULTIPLICITY IS IMPLIED  Sixpak

WE DO NOT EXPECT AN ENERGY DEPENDENT MULTIPLICITY FOR NEUTRON      Sixpak

EMISSION.                                                          Sixpak

Sixpak

USING THE OUTPUT                                                   Sixpak

================================================================== Sixpak

NOTE, THAT IN USING THIS DATA, STARTING FROM THE RELATIONSHIP,     Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F0(E,COS)                       Sixpak

Sixpak

USING THE ENDF/B CONVENTION THAT THE MULTIPLICITY IS EITHER        Sixpak

IMPLIED BY THE MT NUMBER (E.G., MT=16 = N,2N - MULTIPLICITY = 2)   Sixpak

OR INCLUDED IN THE CROSS SECTION (E.G., MT=201 = TOTAL NEUTRON     Sixpak

PRODUCTION) ALL THE INFORMATION REQUIRED FOR A CALCULATION IS      Sixpak

AVAILABLE IN,                                                      Sixpak

Sixpak

MF=3       - SIG(E)                                                Sixpak

MF=4       - F0(E,COS) - FOR OUTGOING NEUTRONS                     Sixpak

MF=5       - G0(E,EP)  - FOR OUTGOING NEUTRONS                     Sixpak

MF=12      - Y(E)      - FOR OUTGOING PHOTONS                      Sixpak

MF=14      - F0(E,COS) - FOR OUTGOING PHOTONS (ALWAYS ISOTROPIC)   Sixpak

MF=15      - G0(E,EP)  - FOR OUTGOING PHOTONS                      Sixpak

Sixpak

DOCUMENTATION                                                      Sixpak

================================================================== Sixpak

ONLY SECTIONS OF MF=4, 5, 12, 14, 15 ARE OUTPUT ON A ENDF/B FILE.  Sixpak

THE ONLY DOCUMENTATION IS THE ENDF/B TAPE LABEL (FIRST RECORD OF   Sixpak

EACH FILE) WHICH IDENTIFIES THE DATA AS SIXPAK OUTPUT.             Sixpak

Sixpak

REACTION INDEX                                                     Sixpak

================================================================== Sixpak

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Sixpak

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Sixpak

Sixpak

SECTION SIZE                                                       Sixpak

================================================================== Sixpak

ALL OF THE DATA IN ENDF/B-VI, MF=6 ARE QUITE SMALL TABLES. AS SUCH Sixpak

THIS PROGRAM ONLY ALLOWS TABLES OF UP TO 12000 POINTS (12,000 X,   Sixpak

Y VALUES). THIS SIZE IS MORE THAN ADEQUATE TO HANDLE ALL OF THE    Sixpak

CURRENT ENDF/B-VI DATA, AND IT CAN BE EASILY INCREASED TO HANDLE   Sixpak

ANY NEWER DATA AS IT BECOMES AVAILABLE.                            Sixpak

Sixpak

PLEASE CONTACT THE AUTHOR IF YOU HAVE AN EVALUATION WHICH EXCEEDS  Sixpak

THIS LIMIT.                                                        Sixpak

Sixpak

SELECTION OF DATA                                                  Sixpak

================================================================== Sixpak

THE PROGRAM SELECTS DATA TO BE PROCESSED BASED ON MAT/MT RANGES    Sixpak

(MF=6 ASSUMED). THIS PROGRAM ALLOWS UP TO 100 MAT/MT RANGES TO BE  Sixpak

SPECIFIED BY INPUT PARAMETERS. THE PROGRAM WILL ASSUME THAT THE    Sixpak

ENDF/B TAPE IS IN MAT ORDER. THE PROGRAM WILL TERMINATE EXECUTION  Sixpak

WHEN A MAT IS FOUND THAT IS ABOVE ALL REQUESTED MAT RANGES.        Sixpak

Sixpak

PROGRAM OPERATION                                                  Sixpak

================================================================== Sixpak

EACH SECTION (MT) OF MF=6 DATA IS SUBDIVIDED INTO SUBSECTIONS -    Sixpak

ONE SUBSECTION FOR EACH EMITTED PARTICLE.                          Sixpak

Sixpak

EACH SUBSECTION OF DATA IS CONSIDERED SEPARATELY. EACH SUBSECTION  Sixpak

OF ENDF/B MF=6 DATA TO PROCESS IS IN THE FORM,                     Sixpak

Sixpak

F(E,EP,COS) = SIG(E)*Y(E)*G0(E,EP)*F(E,EP,COS)                     Sixpak

Sixpak

SIG(E)      = MF=3 CROSS SECTIONS                                  Sixpak

Y(E)        = YIELD (MULTIPLICITY)                                 Sixpak

G0(E,EP)    = ENERGY SPECTRUM                                      Sixpak

F(E,EP,COS) = ANGULAR DISTRIBUTION                                 Sixpak

Sixpak

G0(E,EP) = 1 WHEN INTEGRATED OVER EP (SECONDARY ENERGY)            Sixpak

G0(E,EP)*F(E,EP,COS) = 1 WHEN INTEGRATED OVER EP AND COS           Sixpak

Sixpak

THIS PROGRAM WILL DEFINE THE ZEROTH ORDER MOMENTS OF THE           Sixpak

ENERGY AND ANGULAR DISTRIBUTIONS,                                  Sixpak

Sixpak

G0(E,EP)  = G0(E,EP)*F(E,EP,COS) INTEGRATED OVER COS               Sixpak

F0(E,COS) = G0(E,EP)*F(E,EP,COS) INTEGRATED OVER EP                Sixpak

Sixpak

FOR NEUTRON INDUCED REACTIONS THE ENDF/B FORMATTED OUTPUT WILL BE  Sixpak

Sixpak

F0(E,COS)- IN ENDFB.MF4 FOR NEUTRONS OUT OF A REACTION             Sixpak

G0(E,EP) - IN ENDFB.MF5 FOR NEUTRONS OUT OF A REACTION             Sixpak

- IN ENDFB.M15 FOR PHOTONS OUT OF A REACTION              Sixpak

Sixpak

FOR NEUTRONS INCIDENT AND NEUTRONS EMITTED THIS DATA WILL BE       Sixpak

OUTPUT IN MF=4 AND 5 FORMATS.                                      Sixpak

Sixpak

FOR NEUTRONS INCIDENT AND PHOTONS EMITTED THIS DATA WILL BE        Sixpak

OUTPUT IN MF=15 FORMAT - THE SPECTRA ARE OUTPUT AND THE            Sixpak

ANGULAR DISTRIBUTION IS IGNORED.                                   Sixpak

Sixpak

ALL PHOTON EMISSION IN THE ENDF/B-VI LIBRARY AS OF JANUARY 1992    Sixpak

IS ISOTROPIC AND AS SUCH NO DISTRIBUTION OF PHOTON ANGULAR         Sixpak

DISTRIBUTIONS NEED BE OUTPUT - IT IS ALWAYS ISOTROPIC.             Sixpak

Sixpak

FOR ALL OTHER COMBINATIONS INCIDENT AND EMITTED PARTICLES          Sixpak

THERE WILL BE NO ENDF/B FORMATTED OUTPUT.                          Sixpak

Sixpak

VARIATIONS FROM ENDF/B MANUAL                                      Sixpak

================================================================== Sixpak

LAW=1, LANG=2 = KALBACH-MANN                                       Sixpak

============================                                       Sixpak

FOR THE DISTRIBUTIONS,                                             Sixpak

Sixpak

F(MU,E,EP) = G0(E,EP)*A*(COSH(MU*A)+R(E,EP)*SINH(MU*A))            Sixpak

Sixpak

G0(E,EP) = 1 - WHEN INTEGRATED OVER EP.                            Sixpak

Sixpak

A*(COSH(MU*A)+R(E,EP)*SINH(MU*A)) = 2 - WHEN INTEGRATD OVER MU     Sixpak

Sixpak

THIS MEANS AS DEFINED IN THE ENDF/B MANUAL THE DISTRIBUTIONS       Sixpak

ARE NORMALIZED TO 2, INSTEAD OF 1. IN ORDER TO OBTAIN CORRECTLY    Sixpak

NORMALIZED DISTRIBUTIONS THE DISTRIBUTION SHOULD BE DEFINED        Sixpak

TO INCLUDE A FACTOR OF 1/2 MULTIPLYING THE ANGULAR PART OF         Sixpak

THE DISTRIBUTION.                                                  Sixpak

Sixpak

F(MU,E,EP) = G0(E,EP)*0.5*A*(COSH(MU*A)+R(E,EP)*SINH(MU*A))        Sixpak

Sixpak

THIS IS THE FORM USED IN THIS CODE                                 Sixpak

Sixpak

LAW=1, ND NOT 0 = DISCRETE SECONDARY ENERGY DISTRIBUTION           Sixpak

========================================================           Sixpak

THE ENDF/B MANUAL SAYS THESE ARE FLAGGED WITH NEGATIVE ENERGIES.   Sixpak

IN ENDF/B-VI ALL OF THESE HAVE POSITIVE ENERGY. THIS CODE DOES     Sixpak

NOT CONSIDER THE ENDF/B-VI DATA TO BE IN ERROR.                    Sixpak

Sixpak

WITH THE CONVENTION ACTUALLY USED IN ENDF/B-VI ALL SECONDARY       Sixpak

ENERGIES SHOULD BE NON-NEGATIVE AND IN ASCENDING ENERGY ORDER      Sixpak

FOR EACH INCIDENT ENERGY.                                          Sixpak

Sixpak

FROM THE ENDF/B MANUAL IT IS NOT OBVIOUS WHAT G0(E,EP) SHOULD BE   Sixpak

FOR DISCRETE PHOTONS - PHYSICALLY THIS IS A DELTA FUNCTION. IN     Sixpak

ENDF/B-VI IT IS ENTERED AS 1.0 = INTERPRETING IT AS INTEGRATED     Sixpak

OVER SECONDARY ENERGY - IN WHICH CASE THE DELTA FUNCTION = 1.0.    Sixpak

Sixpak

LIMITATIONS                                                        Sixpak

================================================================== Sixpak

CHECKING DATA                                                      Sixpak

================================================================== Sixpak

THIS PROGRAM CHECKS ALL ENDF/B-VI MF=6 DATA. THE FOLLOWING CHECKS  Sixpak

ARE PERFORMED.                                                     Sixpak

Sixpak

PARAMETERS                                                         Sixpak

==========                                                         Sixpak

ALL PARAMETERS ARE CHECKED FOR CONSISTENCY. IF PARAMETERS ARE      Sixpak

NOT CONSISTENT THE PROGRAM MAY NOT BE ABLE TO PERFORM THE          Sixpak

FOLLOWING TESTS AND WILL MERELY SKIP A SECTION OF DATA.            Sixpak

Sixpak

INTERPOLATION LAWS                                                 Sixpak

==================                                                 Sixpak

ALL INTEGRATIONS ARE PERFORMED USING THE INTERPOLATION LAW GIVEN   Sixpak

FOR SECONDARY ENERGY AND/OR COSINE. INTEGRATIONS ARE NOT           Sixpak

PERFORMED OVER INCIDENT - ONLY INTEGRATION OVER SECONDARY ENERGY   Sixpak

AND/OR COSINE ARE PERFORMED AT EACH INCIDENT ENERGY. THEREFORE     Sixpak

THE INTERPOLATION LAW FOR INCIDENT ENERGY IS NOT USED BY THIS      Sixpak

CODE.                                                              Sixpak

Sixpak

ALL INTERPOLATION LAWS ARE CHECKED. ALL DATA ASSOCIATED WITH       Sixpak

INTERPOLATION LAWS ARE CHECKED, E.G., NO NON-NEGATIVE VALUES       Sixpak

REQUIRING LOG INTERPOLATION. IN ORDER TO PERFORM REQUIRED          Sixpak

INTEGRALS OVER COS AND EP IT IS IMPERATIVE THAT THE INTERPOLATION  Sixpak

LAWS BE COMPATIBLE WITH THE DATA.                                  Sixpak

Sixpak

ENDF/B-VI ALLOWS NEW INTERPOLATION LAWS FOR CORRESPONDING POINT    Sixpak

AND UNIT BASE TRANSFORMATION INTERPOLATION. NONE OF THESE NEW      Sixpak

INTERPOLATION LAWS ARE USED IN THE ENDF/B-VI LIBRARY AS OF         Sixpak

JANUARY 1992 TO INTERPOLATE IN SECONDARY ENERGY OR COSINE.         Sixpak

THEREFORE THIS PROGRAM CAN PERFORM ALL OF THE REQUIRED INTEGRALS   Sixpak

OVER SECONDARY ENERGY AND/OR COSINE USING ONLY THE OLDER           Sixpak

INTERPOLATION CODES. THIS PROGRAM ONLY PERFORMS INTEGRALS FOR      Sixpak

EACH INCIDENT ENERGY, SO THAT INTERPOLATION IN INCIDENT ENERGY     Sixpak

IS NOT PERFORMED BY THIS PROGRAM.                                  Sixpak

Sixpak

NEW INTERPOLATION SCHEMES ARE USED FOR INCIDENT ENERGY - FOR       Sixpak

EXAMPLE, CORRESPONDING POINT INTERPOLATION IS SPECIFIED TO ALLOW   Sixpak

INTERPOLATION IN G0(E,EP) TO SIMULATE CASES WHERE THE INPUT ENERGY Sixpak

LIMIT IS DEFINED BY E-EP = A DIAGONAL CURVE ACROSS (E,EP) SPACE.   Sixpak

THIS INTERPOLATION CODE CANNOT BE SPECIFIED IN THE MF=5 OUTPUT     Sixpak

OF THIS CODE - MF=5 ONLY ALLOWS THE OLDER INTERPOLATION LAWS       Sixpak

INT=1 THROUGH 5. THEREFORE THIS PROGRAM WILL USE THE CLOSEST       Sixpak

CORRESPONDING INTERPOLATION CODE FOR OUTPUT TO MF=5. FOR USE       Sixpak

WHERE THE OUTPUT OF THIS CODE = LOW ENERGY APPLICATIONS - THIS     Sixpak

SHOULD HAVE LITTLE EFFECT ON RESULTS.                              Sixpak

Sixpak

FOR CONSISTENCY WITH EARLIER VERSIONS OF ENDF/B IN CREATING THE    Sixpak

ENDF/B OUTPUT, IF ANY INPUT INTERPOLATION LAW IS NOT IN THE        Sixpak

RANGE 1-5, IT WILL FIRST BE TESTED TO SEE IF MOD(10) IT IS         Sixpak

IN THIS RANGE, FINALLY IF EVEN THIS DOESN'T WORK IT IS SET         Sixpak

EQUAL TO 2 (LINEARLY INTERPOLATION). THIS METHOD WILL EFFECTIVELY  Sixpak

REPLACE CORRESPONDING POINT AND UNIT BASE TRANSFORMATION BY THE    Sixpak

CLOSEST RELATED INTERPOLATION LAW 1 THROUGH 5 - AGAIN NOTE, AS     Sixpak

OF JANUARY 1992 NONE OF THESE NEW LAWS ARE USED IN ENDF/B-VI. IF   Sixpak

THIS MUST BE DONE FOR INTERPOLATION IN SECONDARY ENERGY OR COSINE  Sixpak

AN ERROR MESSAGE WILL BE PRINTED - SINCE THIS WOULD EFFECT THE     Sixpak

ACCURACY OF THE INTEGRALS PERFORMED BY THIS PROGRAM. IF THIS MUST  Sixpak

BE DONE FOR INCIDENT ENERGY NO MESSAGE IS PRINTED - SINCE THIS     Sixpak

WILL NOT EFFECT THE ACCURACY OF THE INTEGRALS PERFORMED BY THIS    Sixpak

PROGRAM.                                                           Sixpak

Sixpak

SPECTRA AND ANGULAR DISTRIBUTIONS                                  Sixpak

=================================                                  Sixpak

ALL SPECTRA AND ANGULAR DISTRIBUTIONS ARE CHECKED TO INSURE        Sixpak

THEY ARE NORMALIZED AND DO NOT INCLUDE ANY NEGATIVE VALUES.        Sixpak

Sixpak

LEGENDRE COEFFICIENTS                                              Sixpak

=====================                                              Sixpak

THE NORMALIZATION, F0, CANNOT BE NEGATIVE.                         Sixpak

Sixpak

LEGENDRE COEFFICIENTS IN NORMAL FORM ARE CHECKED TO INSURE         Sixpak

THEY ARE IN THE RANGE -1 TO +1 = THE LEGENDRE EXPANSION OF A       Sixpak

DELTA FUNCTION AT COS=+1 OR -1 - COEFFICIENTS SHOULD NOT           Sixpak

EXCEED WHAT YOU GET FROM A DELTA FUNCTION.                         Sixpak

Sixpak

ANGULAR DISTRIBUTIONS ARE CHECKED AT COS = -1, 0 AND +1.           Sixpak

Sixpak

CREATING ENDF/B OUTPUT                                             Sixpak

================================================================== Sixpak

THIS PROGRAM CAN CREATE EQUIVALENT MF =4, 5, 12, 14, 15 DATA FOR   Sixpak

ALL OF THE DATA INCLUDED IN ENDF/B-VI AS OF JANUARY 1992, EXCEPT   Sixpak

FOR 1 SECTION OF LAW=6 DATA (SEE DETAILS BELOW).                   Sixpak

Sixpak

THIS PROGRAM HAS NOT BEEN TESTED ON OTHER DATA LIBRARIES, E.G.,    Sixpak

JEF, JENDL, ETC.                                                   Sixpak

Sixpak

THE PROGRAM HAS THE FOLLOWING LIMITATION AS FAR AS CREATING        Sixpak

ENDF/B FORMATTED OUTPUT.                                           Sixpak

Sixpak

ISOTROPIC PHOTON EMISSION                                          Sixpak

=========================                                          Sixpak

FOR PHOTON EMISSION THE DISTRIBUTIONS ARE ASSUMED TO BE ISOTROPIC  Sixpak

AND ONLY THE MULTIPLICITY IS OUTPUT IN MF=12, ISOTROPIC ANGULAR    Sixpak

DISTRIBUTIONS IN MF=14 AND THE SPECTRA IN MF=15. ALL ENDF/B-VI     Sixpak

MF=6 DATA AS OF JANUARY 1992 INCLUDE ONLY ISOTROPIC PHOTON         Sixpak

EMISSION - SO THAT THIS IS NOT A LIMITATION ON TRANSLATING         Sixpak

ENDF/B-VI DATA.                                                    Sixpak

Sixpak

EITHER TABULATED OR LEGENDRE COEFFICIENTS                          Sixpak

=========================================                          Sixpak

FOR LAW=2 THE REPRESENTATION, EITHER TABULATED OR LEGENDRE         Sixpak

COEFFICIENTS, CAN BE SPECIFIED FOR EACH INCIDENT ENERGY.           Sixpak

Sixpak

IN ORDER TO OBTAIN CORRECT ENDF/B OUTPUT THE REPRESENTATION        Sixpak

MUST BE THE SAME FOR ALL INCIDENT ENERGIES = MF=4 DATA CAN ONLY    Sixpak

BE TABULATED OR LEGENDRE OVER THE ENTIRE ENERGY RANGE.             Sixpak

Sixpak

YIELD AND OUTPUT NORMALIZATION                                     Sixpak

==============================                                     Sixpak

THE YIELD INCLUDED WITH EACH SECTION OF DATA IS NOT USED FOR       Sixpak

OUTPUT FOR NEUTRONS, BUT IS INCLUDED IN THE OUTPUT FOR PHOTONS.    Sixpak

IN ALL CASES THE ANGULAR DISTRIBUTIONS AND SPECTRA OUTPUT ARE      Sixpak

NORMALIZED TO UNITY.                                               Sixpak

Sixpak

LAW=0                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=1                                                              Sixpak

=====                                                              Sixpak

FOR EACH INCIDENT ENERGY DISCRETE AND CONTINUOUS EMISSION SPECTRA  Sixpak

CANNOT BE MIXED TOGETHER - THEY MUST BE ALL EITHER DISCRETE OR     Sixpak

CONTINUOUS. IF DISCRETE EMISSION IS GIVEN ONLY 1 SECONDARY         Sixpak

ENERGY (NEP=1) MAY BE GIVEN = A NORMALIZED DISTRIBUTION FOR A      Sixpak

SINGLE DISCRETE EMISSION ENERGY. ALL OF THE ENDF/B-VI DATA AS      Sixpak

OF JANUARY 1992 CONFORM TO THESE LIMITATIONS.                      Sixpak

Sixpak

SINCE THE FLAG NA, TO INDICATE ISOTROPIC DISTRIBUTIONS, IS ONLY    Sixpak

GIVEN FOR EACH SECONDARY ENERGY (EP) THE PROGRAM CANNOT DECIDE     Sixpak

IN ADVANCE WHETHER OR NOT THE DISTRIBUTION WILL BE ISOTROPIC       Sixpak

AT ALL INCIDENT ENERGIES. THEREFORE ISOTROPIC DISTRIBUTIONS        Sixpak

WILL BE OUTPUT EITHER: LANG = 1 - AS 1 LEGENDRE COEFFICIENT = 0.0  Sixpak

OR LANG = NOT 1 - AS A 2 POINT ANGULAR DISTRIBUTION AT COS = -1.0  Sixpak

AND +1.0 WITH BOTH VALUES EQUAL TO 0.5 (A NORMALIZED ISOTROPIC     Sixpak

DISTRIBUTION).                                                     Sixpak

Sixpak

DISCRETE PHOTONS ARE OUTPUT IN MF=15 AS 3 POINT DISTRIBUTIONS      Sixpak

WITH SECONDARY ENERGY POINTS AT EP-DEP, EP, EP+DEP, WHERE          Sixpak

DEP=0.001*EP. THE VALUES AT EP-DEP AND EP+DEP ARE 0.0, AND         Sixpak

AT EP THE VALUE IS 1000.0/EP TO NORMALIZE THE DISTRIBUTION.        Sixpak

Sixpak

LAW=2                                                              Sixpak

=====                                                              Sixpak

NO LIMITATION ON REPRESENTATIONS.                                  Sixpak

Sixpak

LAW=3                                                              Sixpak

=====                                                              Sixpak

NO LIMITATION ON REPRESENTATIONS.                                  Sixpak

Sixpak

LAW=4                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=5                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - INCIDENT NEUTRON - EMITTED PHOTON OR NEUTRON           Sixpak

REACTIONS ARE NOT EXPECTED.                            Sixpak

Sixpak

LAW=6                                                              Sixpak

=====                                                              Sixpak

NO OUTPUT - ENDF/B-VI ONLY INCLUDES 1 SECTION OF THIS TYPE OF DATA Sixpak

FOR (N,D) 2N,P.                                        Sixpak

Sixpak

LAW=7                                                              Sixpak

=====                                                              Sixpak

FOR EACH INCIDENT ENERGY THE REPRESENTATION MUST BE EITHER,        Sixpak

Sixpak

1) SQUARE = FOR EACH INCIDENT COSINE EXACTLY THE SAME SECONDARY    Sixpak

ENERGIES.                                              Sixpak

Sixpak

2) LINEAR = FOR EACH INCIDENT COSINE THE INTERPOLATION LAW         Sixpak

BETWEEN SECONDARY ENERGIES MUST BE LINEAR.             Sixpak

Sixpak

THESE 2 PRESENTATIONS ARE THE ONLY ONES PRESENTED IN ENDF/B-VI     Sixpak

AS OF JANUARY 1992 - SO THIS PROGRAM CAN TRANSLATED ALL LAW=7      Sixpak

DATA FOR ENDF/B-VI.                                                Sixpak

Sixpak

LABORATORY VS. CENTER-OF-MASS SYSTEM                               Sixpak

================================================================== Sixpak

IN MANY CASES PEOPLE ASSUME THAT FOR HEAVY (HIGH ATOMIC WEIGHT)    Sixpak

MATERIALS THE CENTER-OF-MASS AND LAB SYSTEMS ARE ALMOST IDENTICAL, Sixpak

SINCE IN THIS CASE THE CENTER-OF-MASS ENERGY WILL BE MUCH SMALLER  Sixpak

THAN THE INCIDENT ENERGY. FOR A PROCESS SUCH AS ELASTIC SCATTERING Sixpak

WHERE FOR HEAVY MATERIALS THE SECONDARY ENERGY, EP, WILL ALWAYS    Sixpak

BE A LARGE FRACTION OF THE INCIDENT ENERGY, THIS ASSUMPTION IS     Sixpak

VALID. HOWEVER, FOR THE TYPICAL REACTIONS INCLUDED IN MF=6 THIS    Sixpak

IS NOT ALWAYS TRUE - IN MANY OF THESE CASES THE SECONDARY ENERGY   Sixpak

CAN EXTEND ALL THE WAY DOWN TO ZERO, AND IN PARTICULAR IT CAN      Sixpak

BE SMALL COMPARED TO THE CENTER-OF-MASS ENERGY - WHICH MAKES THE   Sixpak

TRANSFORMATION FROM CENTER-OF-MASS TO LAB IMPORTANT. THEREFORE     Sixpak

GENERALLY TO TREAT MF=6 DATA WE MUST CONSIDER THIS TRANSFORMATION. Sixpak

Sixpak

THE FOLLOWING DISCUSSING ONLY APPLIES TO SPECTRA THAT MAY BE       Sixpak

OUTPUT IN MF=5 = ONLY DATA FOR NEUTRONS INCIDENT AND EMITTED -     Sixpak

IN PARTICULAR THE FOLLOWING DEFINITIONS ARE NOT GENERAL - THEY     Sixpak

ARE ONLY VALID FOR INCIDENT AND EMITTED NEUTRONS.                  Sixpak

Sixpak

DOUBLE DIFFERENTIAL DATA IN MF=6 MAY BE GIVEN IN EITHER THE LAB    Sixpak

OR C.M. SYSTEM. SIMILARLY ANGULAR DISTRIBUTIONS IN MF=4 MAY BE     Sixpak

GIVEN IN EITHER THE LAB OR C.M. SYSTEM. IN CONTRAST ENERGY         Sixpak

SPECTRA IN MF=5 CAN ONLY BE GIVEN IN THE LABORATORY SYSTEM.        Sixpak

Sixpak

THE ANGULAR DISTRIBUTIONS OUTPUT BY THIS CODE IN MF=4 ARE IN THE   Sixpak

SAME SYSTEM IN WHICH THEY ARE GIVEN IN MF=6 - EITHER LAB OR        Sixpak

CENTER-OF-MASS SYSTEM.                                             Sixpak

Sixpak

THE ENERGY SPECTRA OUTPUT BY THIS CODE IN MF=5 MUST BE IN THE LAB  Sixpak

SYSTEM - THIS IS THE ONLY ALLOWED FORM FOR MF=5 DATA.              Sixpak

Sixpak

FOR MF=6 SPECTRA GIVEN IN THE LAB SYSTEM THIS MERELY REQUIRES      Sixpak

COPYING THE GIVEN SPECTRA TO MF=5 OUTPUT.                          Sixpak

Sixpak

FOR MF=6 SPECTRA GIVEN IN THE CENTER-OF-MASS SYSTEM ONLY FIRST     Sixpak

ORDER CORRECTIONS IN THE SPECTRA AND USED AND THEY ARE THEN        Sixpak

OUTPUT IN MF=5 AS IN THE LAB SYSTEM - THE FIRST ORDER CORRECTIONS  Sixpak

ARE DESCRIBED BELOW.                                               Sixpak

Sixpak

DEFINING,                                                          Sixpak

MM      = CENTER OF MASS MOTION                                    Sixpak

CM      = OUTGOING (EMITTED) PARTICLE IN CENTER OF MASS            Sixpak

LAB     = OUTGOING (EMITTED) PARTICLE IN LAB                       Sixpak

THETA   = CM SCATTERING ANGLE RELATIVE TO INCIDENT DIRECTION       Sixpak

COS(CM) = COSINE OF THE CM SCATTERING ANGLE                        Sixpak

Sixpak

FOR NEUTRONS INCIDENT WITH AN ENERGY, E, AND THEREFORE A SPEED,    Sixpak

Sixpak

VN(E) = 2*SQRT(E)/MASS(IN)                                         Sixpak

Sixpak

THE CENTER-OF-MASS SPEED IS GIVEN BY,                              Sixpak

Sixpak

V(MM) = VN(E)/(1 + A)                                              Sixpak

Sixpak

AND THE CENTER OF MASS ENERGY BY,                                  Sixpak

Sixpak

E(MM) = 1/2*MASS(IN)*V(MM)**2                                      Sixpak

= 1/2*MASS(IN)*VN(E)**2/(1 + A)**2                           Sixpak

= E/(1 + A)**2                                               Sixpak

Sixpak

FOR DISTRIBUTIONS GIVEN IN MF=6 IN THE CM, THE SPEED, V(CM),       Sixpak

SHOULD BE VECTORIALLY ADDED TO THAT OF OUTGOING PARTICLES TO       Sixpak

DEFINE THE OUTGOING PARTICLES LAB VELOCITY, AND IN TURN IT'S       Sixpak

ENERGY,                                                            Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

V(LAB)*SIN(LAB) =         V(CM)*SIN(CM)                            Sixpak

Sixpak

V(LAB)**2 = V(MM)**2 + V(CM)**2 + 2*COS(CM)*V(MM)*V(CM)            Sixpak

Sixpak

EP(LAB)   = 0.5*MASS(OUT)*V(LAB)**2                                Sixpak

Sixpak

= E(MM) + EP(CM) + 2*COS(CM)*SQRT(E(MM)*EP(CM))          Sixpak

Sixpak

WE CAN ALSO DEFINE THE REVERSE TRANSFORMATION USING,               Sixpak

Sixpak

V(CM)*COS(CM) = V(LAB)*COS(LAB) - V(MM)                            Sixpak

V(CM)*SIN(CM) = V(LAB)*SIN(LAB)                                    Sixpak

Sixpak

V(CM)**2 = V(MM)**2 + V(LAB)**2 - 2*COS(LAB)*V(MM)*V(LAB)          Sixpak

Sixpak

EP(CM)   = 0.5*MASS(OUT)*V(CM)**2                                  Sixpak

Sixpak

= E(MM) + EP(LAB) - 2*COS(LAB)*SQRT(E(MM)*EP(LAB))       Sixpak

Sixpak

WE CAN DEFINE COS(LAB) FROM THE RELATIONSHIP,                      Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

Sixpak

COS(LAB)        =[V(MM) + V(CM)*COS(CM)]/V(LAB)                    Sixpak

Sixpak

[V(MM) + V(CM)*COS(CM)]                           Sixpak

COS(LAB)        =---------------------------------------------     Sixpak

SQRT[V(MM)**2+V(CM)**2+2*COS(CM)*V(MM)*V(CM)]     Sixpak

Sixpak

OR COS(CM) FROM THE RELATIONSHIP,                                  Sixpak

Sixpak

V(CM)*COS(CM)  = V(LAB)*COS(LAB) - V(MM)                           Sixpak

Sixpak

COS(CM)         =[V(LAB)*COS(LAB) - V(MM)]/V(CM)                   Sixpak

Sixpak

[V(LAB)*COS(LAB) - V(MM)]                         Sixpak

COS(CM)         =------------------------------------------------  Sixpak

SQRT[V(LAB)**2+V(CM)**2-2*COS(LAB)*V(LAB)*V(MM)]  Sixpak

Sixpak

THE JACOBIAN CAN BE DEFINED FROM,                                  Sixpak

Sixpak

V(LAB)*COS(LAB) = V(MM) + V(CM)*COS(CM)                            Sixpak

Sixpak

J = D[COS(CM)]/D[COS(LAB)] = V(LAB)/V(CM)                          Sixpak

= SQRT[EP(LAB)/EP(CM)]                  Sixpak

Sixpak

WITH THESE DEFINITIONS OF EP(LAB) AND COS(LAB) IN TERMS OF E(MM),  Sixpak

EP(CM) AND COS(CM) IT IS POSSIBLE TO PERFORM A POINT-BY-POINT      Sixpak

TRANSFORMATION OF DISTRIBUTIONS FROM THE CM TO LAB SYSTEM USING    Sixpak

THESE DEFINITIONS - OR IF WE WISHED WE COULD PERFORM THE REVERSE   Sixpak

TRANSFORMATION USING THE ABOVE RELATIONSHIPS AND THE IDENTITY,     Sixpak

Sixpak

F(E,EP(LAB),COS(LAB))*D(COS(LAB))=F(E,EP(CM),COS(CM))*D(COS(CM))   Sixpak

Sixpak

THIS IS NOT WHAT WILL BE DONE HERE, SINCE WE WILL ONLY BE          Sixpak

INTERESTED IN THE ZEROTH ORDER MOMENTS OF THESE DISTRIBUTIONS,     Sixpak

BUT WE WILL BE INTERESTED IN DEFINING THOSE MOMENTS IN THE         Sixpak

LAB SYSTEM IN TERMS OF MF=6 SPECTRA GIVEN IN THE CM SYSTEM USING,  Sixpak

Sixpak

F(E,EP(LAB),COS(LAB)) = F(E,EP(CM),COS(CM))*J                      Sixpak

Sixpak

THE LIMITS OF EP(LAB) ARE DEFINED BY SETTING COS(CM) = +1 OR -1,   Sixpak

Sixpak

EP(LAB)   = (SQRT(EP(CM)) + SQRT(E(MM)))**2  FOR COS(CM) = +1      Sixpak

= (SQRT(EP(CM)) - SQRT(E(MM)))**2  FOR COS(CM) = -1      Sixpak

Sixpak

IN THIS FORM WE CAN SEE THAT AS LONG AS THE SECONDARY ENERGY IN    Sixpak

THE CENTER-OF-MASS SYSTEM, EP(CM), IS MUCH LARGER THAN THE         Sixpak

ENERGY OF THE CENTER-OF-MASS, E(MM), THE CENTER-OF-MASS AND LAB    Sixpak

ENERGIES WILL BE ALMOST EQUAL - SIMILARLY FOR THE COSINE, IN       Sixpak

THIS CASE COS(LAB) AND COS(CM) WILL BE ALMOST EQUAL - HOWEVER,     Sixpak

FOR THE MF=6 DATA WE CANNOT ASSUME THAT THIS IS TRUE.              Sixpak

Sixpak

TO FIRST ORDER THE ANGULAR DEPENDENCE CAN BE IGNORED,              Sixpak

Sixpak

EP(LAB)   = E(MM) + EP(CM)                                         Sixpak

Sixpak

ALL THIS SAYS IS THAT TO FIRST ORDER THE EFFECT OF TRANSFORMING    Sixpak

FROM THE CM TO LAB SYSTEM IS TO INCREASE THE ENERGY OF THE         Sixpak

EMITTED PARTICLE IN THE CENTER-OF-MASS SYSTEM BY THE ENERGY OF     Sixpak

THE CENTER-OF-MASS TO DEFINE THE LAB ENERGY.                       Sixpak

Sixpak

NOT ONLY THE ENERGY, BUT ALSO THE SPECTRA MUST BE TRANSFORMED.     Sixpak

STARTING FROM THE DOUBLE DIFFERENTIAL DATA IN THE LAB SYSTEM,      Sixpak

F(E,EP,COS(LAB)), WE CAN DEFINE THE LAB SCALAR SPECTRUM AS,        Sixpak

Sixpak

G0(E,EP) = INTEGRAL F(E,EP,COS(LAB))*D(COS(LAB))                   Sixpak

Sixpak

THIS IS THE NORMAL CALCULATION DEFINED ABOVE AND USED FOR DATA     Sixpak

GIVEN IN THE LAB SYSTEM.                                           Sixpak

Sixpak

STARTING FROM DATA IN THE CENTER OF MASS SYSTEM F(E,EP,COS(CM)),   Sixpak

WE CAN USE THE RELATIONSHIP,                                       Sixpak

Sixpak

F(E,EP,COS(LAB))*D(COS(LAB)) = F(E,EP,COS(CM))*J*D(COS(LAB))       Sixpak

Sixpak

J         = SQRT(EP(LAB)/EP(CM)) - THE JACOBIAN                    Sixpak

Sixpak

= E(MM)/EP(CM) + 1 + 2*COS(CM)*SQRT(E(MM)/EP(CM))        Sixpak

Sixpak

AS IN THE CASE OF THE ENERGY, IN THIS FORM WE CAN SEE THAT AS      Sixpak

LONG AS THE SECONDARY ENERGY IN THE CENTER-OF-MASS SYSTEM,         Sixpak

EP(CM), IS LARGE COMPARED TO THE CENTER-OF-MASS ENERGY, E(MM),     Sixpak

THE JACOBIAN IS ESSENTIALLY UNITY AND THE CENTER-OF-MASS AND LAB   Sixpak

SPECTRA WILL BE VERY SIMILAR - AGAIN, GENERALLY WE CANNOT          Sixpak

ASSUME THAT THIS IS TRUE FOR THE MF=6 SPECTRA.                     Sixpak

Sixpak

THEREFORE WE CAN ALSO DEFINE THE LAB SCALAR SPECTRUM IN TERMS OF   Sixpak

THE CM SPECTRUM IN THE FORM,                                       Sixpak

Sixpak

G0(E,EP) = INTEGRAL F(E,EP,COS(CM))*J*D(COS(LAB))                  Sixpak

Sixpak

CONSISTENT WITH THE ABOVE ASSUMPTION THAT THE ANGULAR DEPENDENCE   Sixpak

OF EP(LAB) CAN BE IGNORED THE JACOBIAN WILL NOT BE USED IN         Sixpak

PERFORMING THESE INTEGRALS - IN WHICH CASE THE INTEGRAL REDUCES    Sixpak

TO EXACTLY THE SAME FORM AS IF THE DATA WERE IN THE LAB SYSTEM.    Sixpak

Sixpak

IT SHOULD BE NOTED THAT SINCE IN THIS CASE THE MF=4 ANGULAR        Sixpak

DISTRIBUTIONS ARE GIVEN IN THE CM SYSTEM AND WHEN USED IN ANY      Sixpak

APPLICATION THEY WILL BE TRANSFORMED TO THE LAB SYSTEM - WHEN      Sixpak

THIS IS DONE THE JACOBIAN WILL BE APPLIED.                         Sixpak

Sixpak

IN THIS CODE WHERE WE ARE MOSTLY CONCERNED WITH CONSERVING THE     Sixpak

NUMBER OF EMITTED PARTICLES AND AVERAGE ENERGIES THE NEUTRON       Sixpak

SPECTRA OUTPUT IN MF=5 WILL NOT BE COMPLETELY CONVERTED TO THE     Sixpak

LAB SYSTEM - ONLY FIRST ORDER CORRECTIONS WILL BE INCLUDED BY      Sixpak

INCREASING THE EMITTED PARTICLE ENERGY BY THE CENTER OF MASS       Sixpak

ENERGY, I.E., FOR A CENTER OF MASS SPECTRUM TABULATED AT CENTER    Sixpak

OF MASS ENERGIES EP(CM) THESE WILL ALL BE UNIFORMLY INCREASED      Sixpak

BY E(MM) TO ACCOUNT FOR THE CENTER OF MASS MOTION - THE SPECTRA    Sixpak

WILL NOT BE MODIFIED BY THE JACOBIAN FACTOR SQRT(EP(LAB)/EP(CM))   Sixpak

SINCE THIS WOULD REQUIRE A DETAILED TRANSFORMATION IN ENERGY AND   Sixpak

COS(THETA) SPACE - WHICH IS JUDGED NOT TO BE WORTH PERFORMING      Sixpak

WITHIN THE LIMITS OF WHERE THE OUTPUT FROM THIS CODE IS INTENDED   Sixpak

TO BE USED.                                                        Sixpak

Sixpak

SINCE THE ANGULAR DISTRIBUTION IS ALWAYS OUTPUT IN THE SAME        Sixpak

SYSTEM AS WHICH IT IS GIVEN IN MF=6, NO TRANSFORMATION IS          Sixpak

REQUIRED FOR THE MF=4 OUTPUT.                                      Sixpak

Sixpak

WHEN USED IN LOW ENERGY APPLICATIONS (E.G., FISSION REACTORS) THE  Sixpak

HIGH ENERGY SPECTRA PRESENTED IN MF=6 WILL BE MOSTLY IMPORTANT     Sixpak

SIMPLY IN CONSERVING PARTICLES, (E.G., AS IN (N,2N)) AND ENERGY    Sixpak

AND THE DETAILS OF THE CORRELATION AND GROSS ENERGY SPECTRA WILL   Sixpak

NOTE PLAY THAT IMPORTANT A ROLE. IN THIS CASE THE SPECTRA OUTPUT   Sixpak

BY THIS PROGRAM IN MF=5 SHOULD BE ADEQUATE.                        Sixpak

Sixpak

PLOTTAB FORMATTED OUTPUT                                           Sixpak

================================================================== Sixpak

THIS PROGRAM CONTAINS ROUTINES TO PRODUCE OUTPUT THAT CAN BE USED  Sixpak

AS INPUT TO THE PLOTTAB CODE TO OBTAIN GRAPHIC RESULTS.            Sixpak

Sixpak

THESE ROUTINES ARE DESIGNED ONLY FOR USE BY THE AUTHOR TO CHECK    Sixpak

THIS CODE. USERS ARE ASKED NOT TO ACTIVATE OR TRY TO USE THESE     Sixpak

ROUTINES. UNLESS YOU COMPLETELY UNDERSTAND THIS CODE THE RESULTS   Sixpak

CAN BE UNRELIABLE IF YOU ACTIVATE THESE ROUTINES.                  Sixpak

Sixpak

INPUT FILES                                                        Sixpak

================================================================== Sixpak

UNIT  DESCRIPTION                                                  Sixpak

----  -----------                                                  Sixpak

2  INPUT LINES (BCD - 80 CHARACTERS/RECORD)                     Sixpak

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Sixpak

Sixpak

OUTPUT FILES                                                       Sixpak

================================================================== Sixpak

UNIT  DESCRIPTION                                                  Sixpak

----  -----------                                                  Sixpak

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Sixpak

11  ENDF/B DATA MF=4 (BCD - 80 CHARACTERS/RECORD)                Sixpak

12  ENDF/B DATA MF=5 (BCD - 80 CHARACTERS/RECORD)                Sixpak

14  ENDF/B DATA MF=15 (BCD - 80 CHARACTERS/RECORD)               Sixpak

17  ENDF/B DATA MF=12 (BCD - 80 CHARACTERS/RECORD)               Sixpak

18  ENDF/B DATA MF=14 (BCD - 80 CHARACTERS/RECORD)               Sixpak

15  PLOTTAB INPUT PARAMETERS (BCD - 80 CHARACTERS/RECORD)        Sixpak

16  PLOTTAB FORMATTED OUTPUT (BCD - 80 CHARACTERS/RECORD)        Sixpak

Sixpak

SCRATCH FILES                                                      Sixpak

================================================================== Sixpak

NONE                                                               Sixpak

Sixpak

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILIO2)    Sixpak

================================================================== Sixpak

UNIT  FILE NAME                                                    Sixpak

----  ----------                                                   Sixpak

2   SIXPAK.INP                                                   Sixpak

3   SIXPAK.LST                                                   Sixpak

10   ENDFB.IN                                                     Sixpak

11   ENDFB.MF4                                                    Sixpak

12   ENDFB.MF5                                                    Sixpak

14   ENDFB.M15                                                    Sixpak

17   ENDFB.M12                                                    Sixpak

18   ENDFB.M14                                                    Sixpak

15   PLOTTAB.INP                                                  Sixpak

16   PLOTTAB.CUR                                                  Sixpak

Sixpak

Sixpak

INPUT PARAMETERS                                                   Sixpak

================================================================== Sixpak

LINE  COLS.  DESCRIPTION                                           Sixpak

----  -----  -----------                                           Sixpak

1   1-60  ENDF/B INPUT DATA FILENAME                            Sixpak

(STANDARD OPTION = ENDFB.IN)                          Sixpak

2-N   1-6   MINIMUM MAT FOR REQUESTED RANGE                       Sixpak

9-11  MINIMUM MT FOR REQUESTED RANGE                        Sixpak

12-17  MAXIMUM MAT FOR REQUESTED RANGE                       Sixpak

20-22  MAXIMUM MT FOR REQUESTED RANGE                        Sixpak

Sixpak

LEAVE THE DEFINITION OF THE FILENAME BLANK - THE PROGRAM WILL      Sixpak

THEN USE THE STANDARD FILENAME (ENDFB.IN).                         Sixpak

Sixpak

UP TO 100 MAT/MT RANGES MAY BE SPECIFIED. THE LIST OF RANGES IS    Sixpak

TERMINATED BY A BLANK LINE. IF THE FIRST INPUT LINE IS COMPLETELY  Sixpak

BLANK ALL DATA WILL BE PROCESSED.                                  Sixpak

Sixpak

EXAMPLE INPUT NO. 1                                                Sixpak

-------------------                                                Sixpak

PROCESS ALL MF=6 DATA ON AN ENDF/B TAPE. USE THE STANDARD INPUT    Sixpak

DATA FILENAME ENDFB.IN IN THIS CASE THE USER CAN EITHER EXPLICITLY Sixpak

SPECIFY THE FILENAME AND MAT/MT RANGE BY THE FOLLOWING  2 INPUT    Sixpak

LINES,                                                             Sixpak

Sixpak

ENDFB.IN                                                           Sixpak

1    1  9999  999                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

OR BY INPUTTING 2 BLANK LINE = PROCESS EVERYTHING.                 Sixpak

Sixpak

EXAMPLE INPUT NO. 2                                                Sixpak

-------------------                                                Sixpak

PROCESS BE-9, MAT=425, MT=16. READ THE DATA FROM ENDFB6\BE9.       Sixpak

IN THIS CASE THE FOLLOWING 3 INPUT LINES ARE REQUIRED,             Sixpak

Sixpak

ENDFBB6\BE9                                                        Sixpak

425   16   425   16                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

EXAMPLE INPUT NO. 3                                                Sixpak

-------------------                                                Sixpak

PROCESS ALL MT=16 (N,2N) DATA. THIS CAN BE DONE BY SPECIFYING THE  Sixpak

MAXIMUM MAT RANGE = 1 TO 9999, AND MT=16 FOR THE MINIMUM AND       Sixpak

MAXIMUM MT RANGE. READ THE DATA FROM ENDFB6\K300. IN THIS CASE     Sixpak

CASE THE FOLLOWING 3 INPUT LINES ARE REQUIRED,                     Sixpak

Sixpak

ENDFB6\K300                                                        Sixpak

1   16  9999   16                                             Sixpak

(BLANK LINE, TERMINATES REQUEST LIST)       Sixpak

Sixpak

======================================================================= Sixpak

======================================================================= Virgin

Virgin

PROGRAM VIRGIN                                                     Virgin

VERSION 76-1 (NOVEMBER 1976)                                       Virgin

VERSION 84-1 (JUNE 1984)   *DOUBLE PRECISION ENERGY                Virgin

VERSION 86-1 (JANUARY 1986)*FORTRAN-77/H VERSION                   Virgin

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Virgin

FILE NAMES (SEE, SUBROUTINE FILEIO     Virgin

FOR DETAILS).                          Virgin

*IMPROVED BASED ON USER COMMENTS.       Virgin

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Virgin

INSURE PROGRAM WILL NOT DO ANYTHING    Virgin

CRAZY.                                 Virgin

*UPDATED TO USE NEW PROGRAM CONVERT     Virgin

KEYWORDS.                              Virgin

*ADDED LIVERMORE CIVIC COMPILER         Virgin

CONVENTIONS.                           Virgin

VERSION 92-1 (JANUARY 1992)*COMPLETE RE-WRITE                      Virgin

*OUTPUT IN PLOTTAB FORMAT               Virgin

*UP TO 2000 THICKNESSES                 Virgin

*INCREASED INCORE PAGE SIZE TO 6000     Virgin

CROSS SECTION POINTS                   Virgin

*ADDED PHOTON CALCULATIONS              Virgin

*ADDED BLACKBODY SPECTRUM               Virgin

*ADDED MULTIPLE LAYERS                  Virgin

*ADDED SPATIALLY DEPENDENT DENSITY      Virgin

*ADDED FORTRAN SAVE OPTION              Virgin

*COMPLETELY CONSISTENT I/O ROUTINES -   Virgin

TO MINIMIZE COMPUTER DEPENDENCE.       Virgin

VERSION 92-2 (MAY 1992)    *CORRECTED TO HANDLE MULTIGROUP CROSS   Virgin

SECTIONS AS INPUT IN ENDF/B FORMAT.    Virgin

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Virgin

*IMPROVED COMPUTER INDEPENDENCE        Virgin

*ALL DOUBLE PRECISION                  Virgin

*ON SCREEN OUTPUT                      Virgin

*UNIFORM TREATMENT OF ENDF/B I/O       Virgin

*IMPROVED OUTPUT PRECISION             Virgin

*DEFINED SCRATCH FILE NAMES            Virgin

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Virgin

POINT READ FOR MORE DIGITS            Virgin

*UPDATED TEST FOR ENDF/B FORMAT        Virgin

VERSION BASED ON RECENT FORMAT CHANGE Virgin

*GENERAL IMPROVEMENTS BASED ON         Virgin

USER FEEDBACK                         Virgin

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Virgin

USER FEEDBACK                         Virgin

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Virgin

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Virgin

*UP TO 2000 THICKNESSES                Virgin

*INCREASED INCORE PAGE SIZE TO 60,000  Virgin

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Virgin

*INCREASED INCORE PAGE SIZE TO         Virgin

240,000 FROM 60,000.                  Virgin

Virgin

OWNED, MAINTAINED AND DISTRIBUTED BY                               Virgin

------------------------------------                               Virgin

THE NUCLEAR DATA SECTION                                           Virgin

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Virgin

P.O. BOX 100                                                       Virgin

A-1400, VIENNA, AUSTRIA                                            Virgin

EUROPE                                                             Virgin

Virgin

ORIGINALLY WRITTEN BY                                              Virgin

------------------------------------                               Virgin

DERMOTT E. CULLEN                                                  Virgin

UNIVERSITY OF CALIFORNIA                                           Virgin

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Virgin

L-159                                                              Virgin

P.O. BOX 808                                                       Virgin

LIVERMORE, CA 94550                                                Virgin

U.S.A.                                                             Virgin

TELEPHONE  925-423-7359                                            Virgin

E. MAIL    CULLEN1@LLNL.GOV                                        Virgin

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Virgin

Virgin

PURPOSE                                                            Virgin

-------                                                            Virgin

THIS PROGRAM IS DESIGNED TO CALCULATE UNCOLLIDED (I.E. VIRGIN)     Virgin

FLUX AND REACTIONS DUE TO TRANSMISSION OF A MONODIRECTIONAL        Virgin

BEAM OF NEUTRONS THROUGH ANY THICKNESS OF MATERIAL. IN ORDER       Virgin

TO SIMULATE AN EXPERIMENTAL MEASUREMENT THE RESULTS ARE GIVEN      Virgin

AS INTEGRALS OVER ENERGY TALLY GROUPS (AS OPPOSED TO POINTWISE     Virgin

IN ENERGY). BY TAKING THE RATIO OF REACTIONS TO FLUX IN EACH       Virgin

GROUP AN EQUIVALENT SPATIALLY DEPENDENT GROUP AVERAGED CROSS       Virgin

SECTION IS CALCULATED BY THE PROGRAM.                              Virgin

Virgin

EVALUATED DATA                                                     Virgin

--------------                                                     Virgin

THE EVALUATED DATA MUST BE IN THE ENDF/B FORMAT. HOWEVER IT        Virgin

MUST BE LINEAR-LINEAR INTERPOLABLE IN ENERGY-CROSS SECTION         Virgin

BETWEEN TABULATED POINTS. SINCE ONLY CROSS SECTIONS (FILE 3 OR 23) Virgin

ARE USED, THIS PROGRAM WILL WORK ON ANY VERSION OF ENDF/B          Virgin

(I.E. ENDF/B-I, II, III, IV, V OR VI).                             Virgin

Virgin

RELATED COMPUTER CODES                                             Virgin

----------------------                                             Virgin

IN ORDER TO CONVERT ENDF/B DATA TO THE FORM REQUIRED BY THIS CODE  Virgin

THE FOLLOWING COMPUTER CODES MAY BE USED,                          Virgin

Virgin

LINEAR  - CONVERT FROM GENERAL ENDF/B INTERPOLATION TO LINEAR-     Virgin

LINEAR INTERPOLATION.                                    Virgin

RECENT  - ADD THE RESONANCE CONTRIBUTION TO TABULATED BACKGROUND   Virgin

CROSS SECTIONS TO OBTAIN LINEAR-LINEAR INTERPOLABLE      Virgin

RESULTS.                                                 Virgin

SIGMA1  - DOPPLER BROADEN CROSS SECTION TO OBTAIN LINEAR-LINEAR    Virgin

INTERPOLABLE RESULTS.                                    Virgin

MIXER   - MIX INDIVIDUAL MATERIALS TOGETHER TO DEFINE COMPOSITE    Virgin

MIXTURES, E.G., COMBINE MATERIALS TO DEFINE STAINLESS    Virgin

STELL.                                                   Virgin

Virgin

IN ORDER TO PLOT THE OUTPUT RESULTS OF THIS CODE USE PROGRAM       Virgin

PLOTTAB.                                                           Virgin

Virgin

COPIES OF ANY OR ALL OF THESE CODES MAY BE OBTAINED FROM D.E.      Virgin

CULLEN AT THE ABOVE ADDRESS.                                       Virgin

Virgin

OUTPUT FORMAT                                                      Virgin

-------------                                                      Virgin

FOR ALL VERSIONS OF THIS PROGRAM PRIOR TO VERSION 92-1 OUTPUT WAS  Virgin

IN TABULAR FORM.                                                   Virgin

Virgin

FOR VERSION 92-1 AND LATER VERSIONS OF THIS CODE ALL OUTPUT IS IN  Virgin

THE PROGRAM PLOTTAB FORMAT TO ALLOW RESULTS TO BE EASILY PLOTTED.  Virgin

FOR A COPY OF PROGRAM PLOTTAB CONTACT D.E. CULLEN AT THE ABOVE     Virgin

ADDRESS.                                                           Virgin

Virgin

TALLY GROUPS                                                       Virgin

------------                                                       Virgin

THE TALLY GROUP STRUCTURE MAY BE ANY SET OF MONONTONICALLY         Virgin

INCREASING ENERGY BOUNDARIES. THERE MAY BE UP TO 2000 TALLY        Virgin

GROUPS. BY USING THE INPUT PARAMETERS THE USER MAY SPECIFY ANY     Virgin

ARBITRARY TALLY GROUP STRUCTURE OR SELECT ONE OF THE FOLLOWING     Virgin

BUILT-IN GROUP STRUCTURES.                                         Virgin

Virgin

(1) TART 175 GROUPS (INPUT  0)                                     Virgin

(2) ORNL  50 GROUPS (INPUT -1)                                     Virgin

(3) ORNL 126 GROUPS (INPUT -2)                                     Virgin

(4) ORNL 171 GROUPS (INPUT -3)                                     Virgin

(5) SAND-II 620 GROUPS...UP TO 18 MEV (INPUT -4)                   Virgin

(6) SAND-II 640 GROUPS...UP TO 20 MEV (INPUT -5)                   Virgin

(7) WIMS 69 GROUPS (INPUT -6)                                      Virgin

(8) GAM-I 68 GROUPS (INPUT -7)                                     Virgin

(9) GAM-II 99 GROUPS (INPUT -8)                                    Virgin

(10) MUFT 54 GROUPS (INPUT -9)                                      Virgin

(11) ABBN 28 GROUPS (INPUT -10)                                     Virgin

Virgin

INCIDENT SPECTRUM                                                  Virgin

-----------------                                                  Virgin

THE INCIDENT SPECTRUM MAY BE ANY TABULATED FUNCTION THAT IS        Virgin

GIVEN BY A SET OF POINTS THAT IS MONOTONICALLY INCREASING IN       Virgin

ENERGY AND LINEAR-LINEAR INTERPOLABLE IN ENERGY-SPECTRUM           Virgin

BETWEEN TABULATED POINTS. THERE IS NO LIMIT TO THE NUMBER OF       Virgin

POINTS USED TO DESCRIBE THE SPECTRUM. THERE ARE FIVE BUILT-IN      Virgin

OPTIONS FOR THE SPECTRUM.                                          Virgin

Virgin

(1) CONSTANT...ENERGY INDEPENDENT (INPUT 0)                        Virgin

(2) 1/E (INPUT 1)                                                  Virgin

(3) BLACKBODY - PHOTON SPECTRUM                                    Virgin

(4) BLACKBODY - ENERGY SPECTRUM (E TIMES THE PHOTON SPECTRUM)      Virgin

(5) TRANSMITTED SPECTRUM FROM PREVIOUS CASE                        Virgin

Virgin

NORMALIZATION OF SPECTRUM                                          Virgin

-------------------------                                          Virgin

ANY INCIDENT SPECTRUM, EITHER READ AS INPUT OR ONE OF THE          Virgin

BUILT-IN SPECTRA, WILL BE NORMALIZED TO UNITY WHEN INTEGRATED      Virgin

OVER THEIR ENTIRE ENERGY RANGE.                                    Virgin

Virgin

TRANSMITTED SPECTRA WILL NOT BE RE-NORMALIZED, SINCE IT ALREADY    Virgin

INCLUDES THE NORMALIZATION OF THE INCIDENT SPECTRUM.               Virgin

Virgin

NOTE, INCIDENT SPECTRA IS NORMALIZED TO UNITY OVER THEIR ENTIRE    Virgin

ENERGY RANGE - NOT OVER THE ENERGY RANGE OF THE GROUPS. IF THE     Virgin

ENERGY RANGE OF THE GROUPS IS LESS THAN THAT OF THE SPECTRUM       Virgin

ONLY THAT PORTION OF THE SPECTRUM WILL BE USED AND THIS WILL       Virgin

NOT BE RE-NORMALIZED TO UNITY.                                     Virgin

Virgin

COMPOSITION OF A LAYER                                             Virgin

----------------------                                             Virgin

YOU MAY RUN PROBLEMS INVOLVING                                     Virgin

1) A LAYER OF UNIFORM DENSITY - DENSITY FOR ATTENUATION IS THAT    Virgin

OF THE TOTAL. DENSITY FOR REACTIONS IS THAT OF THE REACTION.    Virgin

2) A LAYER OF UNIFORM DENSITY - DENSITY IS THE SUM OF THE TOTAL    Virgin

AND REACTION DENSITIES - THE SUM OF THE CROSS SECTIONS IS       Virgin

USED FOR ATTENUATION AND REACTIONS.                             Virgin

3) A LAYER OF VARYING DENSITY BASED ON A UNIFORM TOTAL DENSITY     Virgin

PLUS A VARIATION BETWEEN 0 AND A MAXIMUM BASED ON THE           Virgin

REACTION DENSITY - 0 AT 0 THICKNESS AND MAXIMUM AT MAXIMUM      Virgin

THICKNESS. IN THIS CASE THE AVERAGE REACTION DENSITY IS EQUAL   Virgin

TO THE INPUT REACTION DENSITY. THE VARIATION IN REACTION        Virgin

DENSITY CAN BE LINEAR, SQUARE OR CUBIC.                         Virgin

4) A LAYER OF VARYING DENSITY BASED ON A TOTAL DENSITY WHICH       Virgin

VARYING FROM MAXIMUM AT 0 THICKNESS TO 0 AT MAXIMUM THICKNESS   Virgin

PLUS A REACTION DENSITY WHICH VARIES FROM 0 AT 0 THICKNESS      Virgin

TO MAXIMUM AT MAXIMUM THICKNESS. IN THIS CASE THE AVERAGE       Virgin

DENSITY OF THE TOTAL AND REACTION WILL BOTH BE EQUAL TO THE     Virgin

INPUT TOTAL AND REACTION DENSITIES. THE VARIATION IN TOTAL      Virgin

AND REACTION DENSITY CAN BE LINEAR, SQUARE OR CUBIC.            Virgin

Virgin

IN THE FIRST CASE THE TWO REQUESTED CROSS SECTIONS ARE CONSIDERED  Virgin

TO BE INDEPENDENT - THE TOTAL CROSS SECTION IS USED TO CALCULATE   Virgin

ATTENUATION AND THE REACTION CROSS SECTION IS USED TO CALCULATE    Virgin

REACTIONS, E.G., TRANSMISSION THROUGH NATURAL URANIUM (THE TOTAL   Virgin

CROSS SECTION SHOULD BE THAT OF NATURAL URANIUM) AND REACTIONS     Virgin

IN A U-235 DETECTOR (THE REACTION CROSS SECTION SHOULD BE THAT OF  Virgin

U-235).                                                            Virgin

Virgin

IN THE OTHER THREE CASES THE TWO REQUESTED CROSS SECTIONS ARE      Virgin

TREATED AS TWO CONSTITUENTS OF A MIXTURE OF TWO MATERIALS AND      Virgin

THE TWO CROSS SECTIONS ARE USED BOTH TO DEFINE A TOTAL CROSS       Virgin

SECTION FOR ATTENUATION AND A REACTION CROSS SECTION TO DEFINE     Virgin

REACTIONS. IN THESE CASES THE MIXTURE WILL VARY CONTINUOUSLY,      Virgin

E.G., IN CASE 4) HALF WAY THROUGH THE LAYER THE COMPOSITION WILL   Virgin

BE 1/2 THE MATERIAL DEFINED BY THE TOTAL AND 1/2 THE MATERIAL      Virgin

BASED ON THE REACTION. IN THESE CASES RATHER THAN THINKING OF      Virgin

THE TWO CROSS SECTIONS AS A TOTAL AND REACTION CROSS SECTION,      Virgin

IT IS BETTER TO THINK OF THEM AS THE TOTAL CROSS SECTIONS FOR      Virgin

MATERIALS A AND B AND THE CALCULATED REACTIONS WILL BE BASED       Virgin

ON THESE TWO TOTAL CROSS SECTIONS.                                 Virgin

Virgin

MULTIPLE LAYERS                                                    Virgin

---------------                                                    Virgin

THIS CODE MAY BE USED TO RUN EITHER A NUMBER OF INDEPENDENT        Virgin

PROBLEMS, EACH INVOLVING TRANSMISSION THROUGH A SINGLE LAYER OF    Virgin

MATERIAL, OR TRANSMISSION THROUGH A NUMBER OF LAYERS ONE AFTER     Virgin

THE OTHER.                                                         Virgin

Virgin

IN THE CASE OF MULTIPLE LAYERS, ONE LAYER AFTER ANOTHER, THE       Virgin

TRANSMITTED ENERGY DEPENDENT SPECTRUM IS USED AS THE INCIDENT      Virgin

SPECTRUM FOR THE NEXT LAYER. THERE IS NO LIMIT TO THE NUMBER       Virgin

OF LAYERS WHICH MAY BE USED - EACH LAYER IS TREATED AS A           Virgin

COMPLETELY INDEPENDENT PROBLEM WITH A DEFINED INCIDENT SOURCE,     Virgin

AND AS SUCH THE CYCLE OF TRANSMISSION THROUGH EACH LAYER AND       Virgin

USING THE TRANSMITTED SPECTRUM AS THE INCIDENT SPECTRUM FOR THE    Virgin

NEXT LAYER MAY BE REPEATED ANY NUMBER OF TIMES.                    Virgin

Virgin

REMEMBER - THE INCIDENT SPECTRUM IS ASSUMED TO BE LINEARLY         Virgin

INTERPOLABLE IN ENERGY AND SPECTRUM BETWEEN THE ENERGIES AT        Virgin

WHICH IT IS TABULATED. THE TRANSMITTED SPECTRUM WILL BE TABULATED  Virgin

AT THE UNION OF ALL ENERGIES OF THE INCIDENT SPECTRUM AND CROSS    Virgin

SECTIONS (TOTAL AND REACTION). IN ORDER TO INSURE THE ACCURACY     Virgin

OF THE RESULT WHEN PERFORMING MULTIPLE LAYER CALCULATION BE SURE   Virgin

TO SPECIFY THE INCIDENT SPECTRUM ON THE FIRST LAYER TO SUFFICIENT  Virgin

DETAIL (ENOUGH ENERGY POINTS CLOSELY SPACED TOGETHER) IN ORDER TO  Virgin

ALLOW THE TRANSMITTED SPECTRUM TO BE ACCURATELY REPRESENTED BY     Virgin

LINEAR INTERPOLATION BETWEEN SUCCESSIVE ENERGY POINTS - THERE IS   Virgin

NO LIMIT TO THE NUMBER OF POINTS ALLOWED IN THE INCIDENT SPECTRUM, Virgin

SO IF YOU ARE IN DOUBT, SIMPLY USE MORE ENERGY POINTS TO SPECIFY   Virgin

THE INCIDENT SPECTRUM.                                             Virgin

Virgin

RESULT OUTPUT UNITS                                                Virgin

-------------------                                                Virgin

FLUX      = EXACTLY AS CALCULATED                                  Virgin

REACTIONS = 1/CM OR 1/GRAM                                         Virgin

AVERAGE   = 1/CM - MACROSCOPIC UNITS                               Virgin

CROSS                                                              Virgin

SECTION                                                            Virgin

Virgin

THICKNESS AND DENSITY                                              Virgin

---------------------                                              Virgin

THE UNCOLLIDED CALCULATION ONLY DEPENDS ON THE PRODUCT OF          Virgin

THICKNESS AND DENSITY (I.E. GRAMS PER CM SQUARED). THIS FACT       Virgin

MAY BE USED TO SIMPLIFY INPUT BY ALLOWING THE THICKNESS AND        Virgin

DENSITY TO BE GIVEN EITHER AS CM AND GRAMS/CC RESPECTIVELY         Virgin

OR ELSE TO GIVE THICKNESS IN GRAMS/(CM*CM) AND INPUT A             Virgin

DENSITY OF 1.0 - OR IN ANY OTHER CONVENIENT UNITS AS LONG AS       Virgin

THE PRODUCT OF THICKNESS AND DENSITY IS IN THE CORRECT GRAMS       Virgin

PER CENTIMETER SQUARED.                                            Virgin

Virgin

GRAMS/(CM*CM) ARE RELATED TO ATOMS/BARN THROUGH THE RELATIONSHIP   Virgin

Virgin

GRAMS/(CM*CM)=(ATOMS/BARN)*(GRAMS/MOLE)*(MOLE/ATOM)                Virgin

Virgin

OR...                                                              Virgin

Virgin

GRAMS/(CM*CM)=(ATOMS/BARN)*(ATOMIC WEIGHT)/0.602                   Virgin

Virgin

CROSS SECTIONS AT A SPACE POINT AND OPTICAL THICKNESS              Virgin

-----------------------------------------------------              Virgin

THIS PROGRAM ALLOWS LAYERS OF EITHER UNIFORM DENSITY OR            Virgin

CONTINUOUSLY VARYING DENSITY. THE DENSITY CAN BE ONE OF THE        Virgin

FOLLOWING FORMS,                                                   Virgin

1) C                   = UNIFORM DENSITY                           Virgin

2) C*2*(X/T)           = LINEAR VARIATION FROM 0 TO C              Virgin

3) C*(2-2*(X/T))       = LINEAR VARIATION FROM C TO 0              Virgin

4) C*3*(X/T)**2        = SQUARE VARIATION FROM 0 TO C              Virgin

5) C*(3-3*(X/T)**2)/2  = SQUARE VARIATION FROM C TO 0              Virgin

6) C*4*(X/T)**3        = CUBIC VARIATION FROM 0 TO C               Virgin

7) C*(4-4*(X/T)**3)/3  = CUBIC VARIATION FROM C TO 0               Virgin

Virgin

IN ORDER TO CALCULATE REACTIONS AT A POINT THE MICROSCOPIC         Virgin

REACTION CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES.   Virgin

Virgin

IN ORDER TO CALCULATE TRANSMISSION WE MUST DEFINE THE OPTICAL      Virgin

PATH LENGTH WHICH MAY BE DEFINED BY INTEGRATING EACH OF THE        Virgin

ABOVE DENSITY FORMS TO FIND,                                       Virgin

1) C*X                                                             Virgin

2) C*X*(X/T)                                                       Virgin

3) C*X*(2-(X/T))                                                   Virgin

4) C*X*(X/T)**2                                                    Virgin

5) C*X*(3-(X/T)**2)/2                                              Virgin

6) C*X*(X/T)**3                                                    Virgin

7) C*X*(4-(X/T)**3))/3                                             Virgin

Virgin

IN ORDER TO CALCULATE TRANSMISSION TO A POINT THE MICROSCOPIC      Virgin

TOTAL CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES       Virgin

TO DEFINE THE OPTICAL PATH LENGTH.                                 Virgin

Virgin

THE VARIATION OF THE DENSITY THROUGH THE LAYER MAY BE DEFINED      Virgin

BY SETTING X = 0 OR X = T TO FIND,                                 Virgin

X = 0      X = T                                                Virgin

-----      -----                                                Virgin

1) C          C                                                    Virgin

2) 0          2*C                                                  Virgin

3) 2*C        0                                                    Virgin

4) 0          3*C                                                  Virgin

5) 3*C/2      0                                                    Virgin

6) 0          4*C                                                  Virgin

7) 4*C/3      0                                                    Virgin

Virgin

THE OPTICAL PATH THROUGH A LAYER OF THICKNESS T MAY BE DEFINED     Virgin

FROM THE ABOVE EXPRESSIONS BY SETTING X=T TO FIND THAT IN ALL      Virgin

CASES THE ANSWER WILL BY C*T. THE CONSTANTS IN THE ABOVE           Virgin

EXPRESSIONS HAVE BEEN INTRODUCED IN ORDER TO FORCE THIS RESULT.    Virgin

WITH THESE FACTORS THE OPTICAL PATH LENGTH THROUGH THE LAYER       Virgin

WILL EXACTLY CORRESPOND TO AN AVERAGE DENSITY CORRESPONDING TO     Virgin

THAT INPUT FOR THE TOTAL AND/OR REACTION, I.E., C CORRESPONDS      Virgin

TO THE INPUT DENSITY.                                              Virgin

Virgin

NOTE - FOR THE SAME OPTICAL PATH LENGTHS THROUGH THE LAYER THE     Virgin

TRANSMISSION WILL BE EXACTLY THE SAME. HOWEVER, VARYING THE        Virgin

DENSITY WILL ALLOW YOU TO MODIFY THE REACTION RATES AT SPECIFIC    Virgin

DEPTHS INTO THE LAYER.                                             Virgin

Virgin

COMPUTATION OF INTEGRALS                                           Virgin

------------------------                                           Virgin

STARTING FROM TOTAL CROSS SECTIONS, REACTION CROSS SECTIONS AND    Virgin

A SOURCE SPECTRUM ALL OF WHICH ARE GIVEN IN TABULAR FORM WITH      Virgin

LINEAR INTERPOLATION BETWEEN TABULATED POINTS ALL REQUIRED         Virgin

INTEGRALS CAN BE DEFINED BY ANALYTICAL EXPRESSIONS INVOLVING       Virgin

NOTHING MORE COMPLICATED THAN EXPONENTIALS. THE INTEGRALS THAT     Virgin

MUST BE EVALUATED ARE OF THE FORM...                               Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

(INTEGRAL EK TO EK+1) (S(E)* EXP(-XCT(E)*Z)*DE)                    Virgin

Virgin

REACTIONS                                                          Virgin

---------                                                          Virgin

(INTEGRAL EK TO EK+1) (S(E)*XCR(E)*EXP(-XCT(E)*Z)*DE)              Virgin

Virgin

WHERE..                                                            Virgin

EK TO EK+1 = LONGEST ENERGY INTERVAL OVER WHICH S(E), XCT(E) AND   Virgin

XCR(E) ARE ALL LINEARLY INTERPOLABLE.                 Virgin

S(E)       = ENERGY DEPENDENT WEIGHTING SPECTRUM                   Virgin

XCR(E)     = REACTION CROSS SECTION                                Virgin

XCT(E)     = OPTICAL PATH LENGTH (BASED ON TOTAL CROSS SECTION)    Virgin

Z          = MATERIAL THICKNESS                                    Virgin

Virgin

S(E), XCR(E) AND XCT(E) ARE ALL ASSUMED TO BE GIVEN IN TABULAR     Virgin

FORM WITH LINEAR INTERPOLATION USED BETWEEN TABULATED POINTS.      Virgin

IN OTHER WORDS BETWEEN TABULATED POINTS EACH OF THESE THREE IS     Virgin

DEFINED BY A FUNCTION OF THE FORM...                               Virgin

Virgin

F(E)=((E - EK)*FK+1 + (EK+1 - E)*FK)/(EK+1 - EK)                   Virgin

Virgin

EACH OF THESE THREE CAN BE CONVERTED TO NORMAL FORM BY THE         Virgin

CHANGE OF VARIABLES....                                            Virgin

Virgin

X=(E - 0.5*(EK+1 + EK))/(EK+1 - EK)                                Virgin

Virgin

IN WHICH CASE X WILL VARY FROM -1 (AT EK) TO +1 (AT EK+1) AND      Virgin

EACH FUNCTION REDUCES TO THE NORMAL FORM...                        Virgin

Virgin

F(X)=0.5*(FK*(1 - X) + FK+1*(1 + X))                               Virgin

=0.5*(FK+1 + FK) + 0.5*(FK+1 - FK)*X                           Virgin

Virgin

BY DEFINING THE AVERAGE VALUE AND 1/2 THE CHANGE ACROSS THE        Virgin

INTERVAL.                                                          Virgin

Virgin

AVF=0.5*(FK+1 + FK)                                                Virgin

DF= 0.5*(FK+1 - FK)                                                Virgin

DE= 0.5*(EK+1 - EK)                                                Virgin

Virgin

EACH OF THE THREE FUNCTIONS REDUCES TO THE SIMPLE FORM...          Virgin

Virgin

F(X)=AVF+DF*X                                                      Virgin

Virgin

AND THE TWO REQUIRED INTEGRALS REDUCE TO...                        Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                             Virgin

((AVS+DS*X)*EXP(-DXCT*Z*X)*DX)                                     Virgin

Virgin

REACTION                                                           Virgin

--------                                                           Virgin

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                             Virgin

((AVS*AVXCR+(AVS*DXCR+AVXCR*DS)*X+DS*DXCR*X*X)*EXP(-DXCT*Z*X)*DX)  Virgin

Virgin

WHERE                                                              Virgin

Virgin

AVXCT   = AVERAGE VALUE OF THE TOTAL CROSS SECTION                 Virgin

AVXCR   = AVERAGE VALUE OF THE REACTION CROSS SECTION              Virgin

AVS     = AVERAGE VALUE OF THE SOURCE                              Virgin

DXCT    = 1/2 THE CHANGE IN THE TOTAL CROSS SECTION                Virgin

DXCR    = 1/2 THE CHANGE IN THE REACTION CROSS SECTION             Virgin

DS      = 1/2 THE CHANGE IN THE SOURCE                             Virgin

DE      = 1/2 THE CHANGE IN THE ENERGY                             Virgin

Virgin

NOTE THAT IN THIS FORM THE ENERGY ONLY APPEARS IN FRONT OF THE     Virgin

INTEGRALS AND THE INTEGRALS ARE EXPRESSED ONLY IN TERMS OF THE     Virgin

TABULATED VALUES OF S(E), XCT(E) AND XCR(E). IN PARTICULAR NO      Virgin

DERIVATIVES ARE USED, SO THAT THERE ARE NO NUMERICAL INSTABILITY   Virgin

PROBLEMS IN THE VACINITY OF DISCONTINUITIES IN S(E), XCT(E) OR     Virgin

XCR(E). INDEED, SINCE (EK+1 - EK) APPEARS IN FRONT OF THE INTEGRAL Virgin

POINTS OF DISCONTINUITY AUTOMATICALLY MAKE ZERO CONTRIBUTION TO    Virgin

THE INTEGRALS.                                                     Virgin

Virgin

THE REQUIRED INTEGRALS CAN BE EXPRESSED IN TERMS OF THE THREE      Virgin

INTEGRALS IN NORMAL FORM....                                       Virgin

Virgin

F(A,N) = (INTEGRAL -1 TO 1) (X**N*EXP(-A*X)*DX), N=0,1 AND 2.      Virgin

Virgin

THESE THREE INTEGRALS CAN BE EVALUATED TO FIND...                  Virgin

Virgin

N=0                                                                Virgin

---                                                                Virgin

F(A,0) = (EXP(A)-EXP(-A))/A                                        Virgin

Virgin

N=1                                                                Virgin

---                                                                Virgin

F(A,1) = ((1-A)*EXP(A)-(1+A)*EXP(-A))/(A*A)                        Virgin

Virgin

N=2                                                                Virgin

---                                                                Virgin

F(A,2) = ((2-2*A+A*A)*EXP(A)-(2+2*A+A*A)*EXP(-A))/(A*A*A)          Virgin

Virgin

HOWEVER THESE EXPRESSIONS ARE NUMERICALLY UNSTABLE FOR SMALL       Virgin

VALUES OF A. THEREFORE FOR SMALL A THE EXPONENTIAL IN THE          Virgin

INTEGRALS ARE EXPANDED IN A POWER SERIES...                        Virgin

Virgin

EXP(-AX)=1.0-(AX)+(AX)**2/2-(AX)**3/6+(AX)**4/24-........          Virgin

=(SUM K=0 TO INFINITY) (-AX)**K/(K FACTORIAL)              Virgin

Virgin

AND THE INTEGRAL REDUCES TO THE FORM....                           Virgin

Virgin

(SUM K=0 TO INFINITY) ((-A)**K/(K FACTORIAL)) *                    Virgin

(INTEGRAL -1 TO 1) (X**(N+K))*DX                                   Virgin

Virgin

WHICH CAN BE ANALYTICALLY EVAULATED TO FIND....                    Virgin

(K(N) = K FACTORIAL)                                               Virgin

Virgin

N=0                                                                Virgin

---                                                                Virgin

F(A,0) = 2*(1+(A**2)/K(3)+(A**4)/K(5)+(A**6)/K(7)+....             Virgin

Virgin

N=1                                                                Virgin

---                                                                Virgin

F(A,1) = -2*A*(2/K(3)+4*(A**2)/K(5)+6*(A**4)/K(7)+8*(A**6)/K(9)+.. Virgin

Virgin

N=2                                                                Virgin

---                                                                Virgin

F(A,2) = 2*(2/K(3)+3*4*(A**2)/K(5)+5*6*(A**4)/K(7)+                Virgin

7*8*(A**6)/K(9)+....                                      Virgin

Virgin

THESE EXPANSIONS ARE USED WHEN THE ABSOLUTE VALUE OF A IS LESS     Virgin

THAN 0.1. BY TRUNCATING THE ABOVE SERIES BEFORE A**8 THE ERROR     Virgin

RELATIVE TO THE LEADING TERM OF THE SERIES WILL BE 10**(-10),      Virgin

YIELDING 10 DIGIT ACCURACY.                                        Virgin

Virgin

AFTER EVALUATING THE ABOVE FUNCTIONS, EITHER DIRECTLY OR BY USING  Virgin

THE EXPANSION THE TWO REQUIRED INTEGRALS CAN BE WRITTEN AS...      Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

DE*EXP(-AVXCT*Z)*(AVS*F(A,0) + DS*F(A,1))                          Virgin

Virgin

REACTIONS                                                          Virgin

---------                                                          Virgin

DE*EXP(-AVXCT*Z)*                                                  Virgin

(AVS*AVXCR*F(A,0) + (AVS*DXCR+AVXCR*DS)*F(A,1) + DS*DXCR*F(A,2))   Virgin

Virgin

INPUT FILES                                                        Virgin

-----------                                                        Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

INPUT       2   INPUT LINES                                        Virgin

ENDFIN     10   EVALUATED DATA IN ENDF/B FORMAT                    Virgin

Virgin

OUTPUT FILES                                                       Virgin

------------                                                       Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

OUTPUT      3   OUTPUT LISTING                                     Virgin

Virgin

SCRATCH FILES                                                      Virgin

-------------                                                      Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

SCR1       12   REACTION, FLUX AND CROSS SECTION RESULTS (BCD)     Virgin

(SORTED AT END OF RUN AND OUTPUT SEPARATELY)       Virgin

SCR2       13   TALLY GROUP ENERGY BOUNDARIES (BINARY)             Virgin

SCR3       14   SOURCE SPECTRUM (BINARY)                           Virgin

SCR4       15   TOTAL CROSS SECTION (BINARY)                       Virgin

SCR5       16   REACTION CROSS SECTION (BINARY)                    Virgin

Virgin

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILEIO2)   Virgin

----------------------------------------------------------------   Virgin

UNIT  FILE NAME   FORMAT                                           Virgin

----  ----------  ------                                           Virgin

2   VIRGIN.INP   BCD                                             Virgin

3   VIRGIN.LST   BCD                                             Virgin

10   ENDFB.IN     BCD                                             Virgin

11-15  (SCRATCH)   BINARY                                           Virgin

16   PLOTTAB.CUR PLOTTAB OUTPUT FORMAT DATA                       Virgin

Virgin

INPUT LINES                                                        Virgin

-----------                                                        Virgin

ANY NUMBER OF CASES MAY BE RUN ONE AFTER THE OTHER. AFTER THE      Virgin

FIRST CASE HAS BEEN RUN THE FOLLOWING CASES MAY USE THE SAME       Virgin

THICKNESSES, GROUP STRUCTURE AND SPECTRUM AS THE PRECEDING CASE.   Virgin

IN ADDITION THE TRANSMITTED SPECTRUM FROM ONE CASE MAY BE USED     Virgin

AS THE INCIDENT SPECTRUM IN THE NEXT CASE, TO ALLOW MULTIPLE       Virgin

LAYERS OF DIFFERENT MATERIALS.                                     Virgin

Virgin

LINE  COLS.  FORMAT  DESCRIPTION                                   Virgin

----  -----  ------  ----------                                    Virgin

1   1-60   ENDF/B INPUT DATA FILENAME                           Virgin

(STANDARD OPTION = ENDFB.IN)                       Virgin

Virgin

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Virgin

THEN USE STANDARD FILENAMES.                                       Virgin

Virgin

2-3   1-72   18A4   TWO LINE TITLE DESCRIBING PROBLEM             Virgin

4   1- 6    I6    ZA (1000*Z+A) OF TARGET FOR TOTAL             Virgin

7-11    I5    MT OF TOTAL                                   Virgin

12-22   E11.4  DENSITY FOR TOTAL                             Virgin

23-28    I6    ZA (1000*Z+A) OF TARGET FOR REACTION          Virgin

29-33    I5    MT OF REACTION                                Virgin

= 0 - NO REACTION CALCULATION (ONLY FLUX).    Virgin

= GREATER THAN 0 - CALCULATE REACTIONS.       Virgin

34-44   E11.4  DENSITY FOR REACTION                          Virgin

45-50    I6    NUMBER OF TARGET THICKNESSES                  Virgin

= GREATER THAN 0 = READ FROM INPUT            Virgin

(1 TO 2000 ALLOWED)                         Virgin

=  0 = SAME AS LAST CASE                      Virgin

51-55    I5    NUMBER OF TALLY GROUPS                        Virgin

(REMEMBER NUMBER OF GROUP BOUNDARIES          Virgin

IS ONE MORE THAN THE NUMBER OF GROUPS)        Virgin

UP TO 2000 GROUPS ARE ALLOWED                 Virgin

BUILT-IN GROUP STRUCTURES.                    Virgin

= GREATER THAN 0 = READ FROM INPUT            Virgin

=  0 SAME AS LAST CASE                        Virgin

= -1 TART 175 GROUPS                          Virgin

= -2 ORNL  50 GROUPS                          Virgin

= -3 ORNL 126 GROUPS                          Virgin

= -4 ORNL 171 GROUPS                          Virgin

= -5 SAND-II 620 GROUPS..UP TO 18 MEV.        Virgin

= -6 SAND-II 640 GROUPS..UP TO 20 MEV.        Virgin

= -7 WIMS 69 GROUPS                           Virgin

= -8 GAM-I 68 GROUPS                          Virgin

= -9 GAM-II 99 GROUPS                         Virgin

=-10 MUFT 54 GROUPS                           Virgin

=-11 ABBN 28 GROUPS                           Virgin

56-60    I5    NUMBER OF POINTS IN SOURCE SPECTRUM           Virgin

(MUST BE AT LEAST TWO POINTS)                 Virgin

= GREATER THAN 1 = READ FROM INPUT            Virgin

=  0 = SAME AS LAST CASE                      Virgin

= -1 = CONSTANT (ENERGY INDEPENDENT)          Virgin

= -2 = 1/E                                    Virgin

= -3 = BLACKBODY - PHOTON SPECTRUM            Virgin

= -4 = BLACKBODY - ENERGY SPECTRUM            Virgin

= -5 = TRANSMITTED SPECTRUM FROM LAST CASE    Virgin

NOTE, ALL SPECTRA, EXCEPT THE TRANSMITTED     Virgin

SPECTRUM FROM THE LAST CASE, WILL BE          Virgin

NORMALIZED SUCH THAT ITS INTEGRAL OVER        Virgin

ENERGY WILL BE UNITY.                         Virgin

61-64  1X,3I1  SPATIALLY DEPENDENT OUTOUT                    Virgin

= 0 = NO                                      Virgin

= 1 = YES                                     Virgin

FOR THE 3 QUANTITIES                          Virgin

COLUMN 67 FLUX                                Virgin

68 REACTIONS                           Virgin

69 AVERAGE CROSS SECTION               Virgin

65-65    I1    ENERGY DEPENDENT OUTOUT                       Virgin

= 0 = NONE                                    Virgin

= 1 = INCIDENT SPECTRUM                       Virgin

= 2 = TRANSMITTED SPECTRUM                    Virgin

= 3 = INCIDENT REACTIONS                      Virgin

= 4 = TRANSMIITED REACTIONS                   Virgin

= 5 = TOTAL CROSS SECTION                     Virgin

= 6 = REACTION CROSS SECTION                  Virgin

5   1-11   E11.4  BLACKBODY TEMPERATURE IN EV                   Virgin

12-22   E11.4  FLUX NORMALIZATION                            Virgin

23-33   E11.4  REACTION NORMALIZATION                        Virgin

CALCULATIONS WILL BE BASED ON THE SPECTRUM    Virgin

AND CROSS SECTIONS AS READ. AT OUTPUT THE     Virgin

RESULTS WILL BE MULTIPLIED BY THESE           Virgin

NORMALIZATION FACTORS.                        Virgin

34-44    I11   DENSITY PROFILE                               Virgin

= 0 - UNIFORM - BASED ON TOTAL DENSITY        Virgin

= 1 - UNIFORM - TOTAL + REACTION DENSITY      Virgin

= 2 - TOTAL + LINEAR REACTION                 Virgin

= 3 - LINEAR (TOTAL + REACTION)               Virgin

= 4 - TOTAL + SQUARE REACTION                 Virgin

= 5 - SQUARE (TOTAL + REACTION)               Virgin

= 6 - TOTAL + CUBIC REACTION                  Virgin

= 7 - CUBIC (TOTAL + REACTION)                Virgin

6-N   1-66  6E11.4  TARGET THICKNESSES IN CM                      Virgin

IF SAME AS LAST CASE THIS SECTION IS NOT      Virgin

INCLUDED IN THE INPUT.                        Virgin

VARY   1-66  6E11.4  TALLY GROUP ENERGY BOUNDARIES                 Virgin

(NUMBER OF BOUNDARIES IS ONE MORE THAN        Virgin

THE NUMBER OF TALLY GROUPS)                   Virgin

IF THE STANDARD OPTION (-11 TO 0) IS          Virgin

SELECTED THIS SECTION IS NOT INCLUDED         Virgin

IN THE INPUT                                  Virgin

VARY   1-66  6E11.4  SOURCE SPECTRUM IN ENERGY (EV)-SOURCE PAIRS   Virgin

(MUST BE AT LEAST TWO POINTS)                 Virgin

IF STANDARD OPTION (-5 TO 0) IS SELECTED THIS Virgin

SECTION IS NOT INCLUDED IN THE INPUT          Virgin

Virgin

ANY NUMBER OF CASES MAY BE RUN ONE AFTER ANOTHER.                  Virgin

Virgin

EXAMPLE INPUT NO. 1                                                Virgin

-------------------                                                Virgin

CALCULATE THE UNCOLLIDED FLUX AND CAPTURE (MT=102) THROUGH         Virgin

30 CM OF IRON (DENSITY 7.87 G/CC). TALLY THE RESULTS USING         Virgin

THE TART 175 GROUP STRUCTURE. THE SOURCE WILL BE CONSTANT          Virgin

FROM 1 KEV TO 20 MEV. USE THE STANDARD ENDF/B INPUT DATA           Virgin

FILENAME.                                                          Virgin

Virgin

ENDFB.IN                                                           Virgin

IRON 0 TO 30 CM THICK.                                             Virgin

CONSTANT SOURCE FROM 1 KEV TO 20 MEV.                              Virgin

26000    1 7.87000+ 0 26000  102 7.87000+ 0     2    0    2 1100  Virgin

0.00000+ 0 1.00000+ 0 1.00000+ 0          0 0.00000+00            Virgin

0.00000+00 3.00000+01                                             Virgin

1.0000E+03 1.0000E+00 2.0000E+07 1.0000E+00                       Virgin

Virgin

EXAMPLE INPUT NO. 2                                                Virgin

-------------------                                                Virgin

CALCULATE THE UNCOLLIDED PHOTON FLUX THROUGH A MIXTURE OF SILICON  Virgin

AND IRON FOR 100 MEV PHOTONS INCIDENT. THE TRANSMISSION WILL BE    Virgin

CALCULATED FOR 21 THICKNESSES VARYING BETWEEN 0 AND 1 CM. THERE    Virgin

WILL BE ONLY 1 TALLY GROUP SPANNING A VERY NARROW ENERGY RANGE     Virgin

NEAR 100 MEV, AND THE SOURCE SPECTRUM WILL BE CONSTANT OVER THE    Virgin

SAME ENERGY RANGE. USE THE STANDARD ENDF/B INPUT DATA FILENAME     Virgin

BY LEAVING THE FIRST INPUT LINE BLANK.                             Virgin

Virgin

(THIS IS A BLANK LINE TO USE THE STANDARD INPUT FILENAME)          Virgin

100 MEV PHOTONS                                                    Virgin

SILICON + 5 % IRON                                                 Virgin

14000  521 2.30000+ 0 26000  521 1.15000- 1    21    1    2 1000  Virgin

0.00000+ 0 1.00000+ 0 1.00000+ 0          1 0.00000+00            Virgin

0.00000+00 5.00000-01 1.00000+00 1.50000+00 2.00000+00 2.50000+00 Virgin

3.00000+00 3.50000+00 4.00000+00 4.50000+00 5.00000+00 5.50000+00 Virgin

6.00000+00 6.50000+00 7.00000+00 7.50000+00 8.00000+00 8.50000+00 Virgin

9.00000+00 9.50000+00 1.00000+01                                  Virgin

9.99000+ 7 1.00100+ 8                                             Virgin

9.99000+ 7 1.00000+ 4 1.00100+ 8 1.00000+ 4                       Virgin

Virgin

======================================================================= Virgin