======================================================================= Groupie

Groupie

PROGRAM GROUPIE                                                    Groupie

VERSION 76-1 (NOVEMBER 1976)                                       Groupie

VERSION 79-1 (OCTOBER 1979) CDC-7600 AND CRAY-1 VERSION.           Groupie

VERSION 80-1 (MAY 1980) IBM, CDC AND CRAY VERSION                  Groupie

VERSION 81-1 (JANUARY 1981) EXTENSION TO 3000 GROUPS               Groupie

VERSION 81-2 (MARCH 1981) IMPROVED SPEED                           Groupie

VERSION 81-3 (AUGUST 1981) BUILT-IN 1/E WEIGHTING SPECTRUM         Groupie

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY        Groupie

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Groupie

*ELIMINATED COMPUTER DEPENDENT CODING.  Groupie

*NEW, MORE COMPATIBLE I/O UNIT NUMBERS. Groupie

*NEW MULTI-BAND LIBRARY BINARY FORMAT.  Groupie

VERSION 83-2 (OCTOBER 1983) ADDED OPTION TO ALLOW SIGMA-0 TO BE    Groupie

DEFINED EITHER AS MULTIPLES OF         Groupie

UNSHIELDED TOTAL CROSS SECTION IN EACH Groupie

GROUP, OR POWERS OF 10 IN ALL GROUPS.  Groupie

VERSION 84-1 (APRIL 1984)   ADDED MORE BUILT IN MULTIGROUP ENERGY  Groupie

STRUCTURES.                            Groupie

VERSION 85-1 (APRIL 1985)  *UPDATED FOR ENDF/B-VI FORMATS.         Groupie

*SPECIAL I/O ROUTINES TO GUARANTEE      Groupie

ACCURACY OF ENERGY.                    Groupie

*DOUBLE PRECISION TREATMENT OF ENERGY   Groupie

(REQUIRED FOR NARROW RESONANCES).      Groupie

*MINIMUM TOTAL CROSS SECTION TREATMENT  Groupie

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Groupie

VERSION 86-1 (JANUARY 1986)*ENDF/B-VI FORMAT                       Groupie

VERSION 86-2 (JUNE 1986)   *BUILT-IN MAXWELLIAN, 1/E AND FISSION   Groupie

WEIGHTING SPECTRUM.                    Groupie

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Groupie

FILE NAMES (SEE, SUBROUTINES FILIO1    Groupie

FILIO2 FOR DETAILS).                   Groupie

*IMPROVED BASED ON USER COMMENTS.       Groupie

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Groupie

INSURE PROGRAM WILL NOT DO ANYTHING    Groupie

CRAZY.                                 Groupie

*UPDATED TO USE NEW PROGRAM CONVERT     Groupie

KEYWORDS.                              Groupie

*ADDED LIVERMORE CIVIC COMPILER         Groupie

CONVENTIONS.                           Groupie

VERSION 91-1 (JUNE 1991)   *INCREASED PAGE SIZE FROM 1002 TO 5010  Groupie

POINTS                                 Groupie

*UPDATED BASED ON USER COMMENTS         Groupie

*ADDED FORTRAN SAVE OPTION              Groupie

*COMPLETELY CONSISTENT ROUTINE TO READ  Groupie

FLOATING POINT NUMBERS.                Groupie

VERSION 92-1 (JANUARY 1992)*ADDED RESONANCE INTEGRAL CALCULATION - Groupie

UNSHIELDED AND/OR SHIELDED - FOR       Groupie

DETAILS SEE BELOW                      Groupie

*INCREASED NUMBER OF ENERGY POINTS      Groupie

IN BUILT-IN SPECTRA - TO IMPROVE       Groupie

ACCURACY.                              Groupie

*ALLOW SELECTION OF ZA/MF/MT OR         Groupie

MAT/MF/MT RANGES - ALL DATA NOT        Groupie

SELECTED IS SKIPPED ON INPUT AND       Groupie

NOT WRITTEN AS OUTPUT.                 Groupie

*COMPLETELY CONSISTENT I/O ROUTINES -   Groupie

TO MINIMIZE COMPUTER DEPENDENCE.       Groupie

*NOTE, CHANGES IN INPUT PARAMETER       Groupie

FORMAT - FOR ZA/MF/MT OR MAT/MF/MT     Groupie

RANGES.                                Groupie

VERSION 92-2 (JUNE 1992)   *MULTIBAND PARAMETERS OUTOUT AS         Groupie

CHARACTER (RATHER THAN BINARY) FILE.   Groupie

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 5010 TO       Groupie

30000 POINTS                           Groupie

*ELIMINATED COMPUTER DEPENDENCE.        Groupie

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Groupie

TO ALLOW ACCESS TO FILE STRUCTURES     Groupie

(WARNING - INPUT PARAMETER FORMAT      Groupie

HAS BEEN CHANGED)                      Groupie

*CLOSE ALL FILES BEFORE TERMINATING     Groupie

(SEE, SUBROUTINE ENDIT)                Groupie

VERSION 95-1 (JANUARY 1994)*CORRECTED MAXWELLIAN WEIGHTING         Groupie

*CHANGING WEIGHTING SPECTRUM FROM       Groupie

0.1 TO 0.001 % UNCERTAINTY             Groupie

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Groupie

*IMPROVED COMPUTER INDEPENDENCE        Groupie

*ALL DOUBLE PRECISION                  Groupie

*ON SCREEN OUTPUT                      Groupie

*UNIFORM TREATMENT OF ENDF/B I/O       Groupie

*IMPROVED OUTPUT PRECISION             Groupie

*DEFINED SCRATCH FILE NAMES            Groupie

*UP TO 1000 GROUP MULTI-BAND           Groupie

CALCULATION (PREVIOUSLY 175)          Groupie

*MAXIMUM NUMBER OF GROUPS REDUCED      Groupie

FROM 3,000 TO 1,000                   Groupie

*UP TO 1000 MATERIALS                  Groupie

(PREVIOUSLY 100)                      Groupie

*CORRECTED USE OF MAXWELLIAN +         Groupie

1/E + FISSION SPECTRUM                Groupie

*ONLY 2 BAND VERSION DISTRIBUTED       Groupie

(CONTACT AUTHOR FOR DETAILS)          Groupie

*DEFINED SCRATCH FILE NAMES            Groupie

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Groupie

POINT READ FOR MORE DIGITS            Groupie

*UPDATED TEST FOR ENDF/B FORMAT        Groupie

VERSION BASED ON RECENT FORMAT CHANGE Groupie

*GENERAL IMPROVEMENTS BASED ON         Groupie

USER FEEDBACK                         Groupie

VERSION 99-2 (JUNE 1999)    *ASSUME ENDF/B-VI, NOT V, IF MISSING   Groupie

MF=1, MT-451.                         Groupie

VERS. 2000-1 (FEBRUARY 2000)*ADDED MF=10, ACTIVATION CROSS SECTION Groupie

PROCESSING.                           Groupie

*GENERAL IMPROVEMENTS BASED ON         Groupie

USER FEEDBACK                         Groupie

VERS. 2002-1 (FEBRUARY 2002)*ADDED TART 700 GROUP STRUCTURE        Groupie

*ADDED VARIABLE SIGMA0 INPUT OPTION    Groupie

(MAY 2002)     *OPTIONAL INPUT PARAMETERS             Groupie

(NOV. 2002)    *ADDED SAND-II EXTENDED DOWN TO        Groupie

1.0E-5 EV.                            Groupie

(JUNE 2003)    *CORRECTED SAND-II 620 AND 640 GROUP   Groupie

ENERGY BOUNDARIES DEFINITIONS.        Groupie

VERS. 2004-1 (SEPT. 2004)  *INCREASED PAGE SIZE FROM 30000 TO      Groupie

120000 POINTS                          Groupie

*ADDED "OTHER" AS ADDITIONAL REACTION   Groupie

TO IMPROVE MULTI-BAND FITTING          Groupie

*ADDED ITERATION FOR "BEST" PARTIAL     Groupie

PARAMETERS.                            Groupie

*DO NOT SKIP LOW TOTAL ENERGY RANGES    Groupie

WHEN DEFINING AVERAGE CROSS SECTIONS - Groupie

THIS MAKES OUTPUT COMPATIBLE WITH      Groupie

ANY STANDARD AVERAGING PROCEDURE       Groupie

VERS. 2005-1 (JAN. 2005)   *ADDED OPTION TO CHANGE TEMPERATURE OF  Groupie

BUILT-IN STANDARD SPECTRUM.            Groupie

VERS. 2007-1 (JAN. 2007)   *CHECKED AGAINST ALL ENDF/B-VII.    OF  Groupie

*INCREASED PAGE SIZE FROM 120,000 TO    Groupie

600,000 POINTS                         Groupie

Groupie

OWNED, MAINTAINED AND DISTRIBUTED BY                               Groupie

------------------------------------                               Groupie

THE NUCLEAR DATA SECTION                                           Groupie

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Groupie

P.O. BOX 100                                                       Groupie

A-1400, VIENNA, AUSTRIA                                            Groupie

EUROPE                                                             Groupie

Groupie

ORIGINALLY WRITTEN BY                                              Groupie

------------------------------------                               Groupie

DERMOTT E. CULLEN                                                  Groupie

UNIVERSITY OF CALIFORNIA                                           Groupie

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Groupie

L-159                                                              Groupie

P.O. BOX 808                                                       Groupie

LIVERMORE, CA 94550                                                Groupie

U.S.A.                                                             Groupie

TELEPHONE  925-423-7359                                            Groupie

E. MAIL    CULLEN1@LLNL.GOV                                        Groupie

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Groupie

Groupie

AUTHORS MESSAGE                                                    Groupie

---------------                                                    Groupie

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Groupie

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Groupie

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Groupie

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Groupie

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Groupie

Groupie

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Groupie

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Groupie

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Groupie

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Groupie

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Groupie

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Groupie

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Groupie

COMPUTER.                                                          Groupie

Groupie

PURPOSE                                                            Groupie

-------                                                            Groupie

THIS PROGRAM IS DESIGNED TO CALCULATE ANY COMBINATION OF           Groupie

THE FOLLOWING QUANTITIES FROM LINEARLY INTERPOLABLE TABULATED      Groupie

CROSS SECTIONS IN THE ENDF/B FORMAT                                Groupie

Groupie

(1) UNSHIELDED GROUP AVERAGED CROSS SECTIONS                       Groupie

(2) BONDARENKO SELF-SHIELDED GROUP AVERAGED CROSS SECTIONS         Groupie

(3) MULTI-BAND PARAMETERS                                          Groupie

Groupie

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Groupie

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Groupie

DISK OR ANY OTHER MEDIUM.                                          Groupie

Groupie

ENDF/B FORMAT                                                      Groupie

-------------                                                      Groupie

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Groupie

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Groupie

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV OR V FORMAT).     Groupie

Groupie

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Groupie

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Groupie

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE  Groupie

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Groupie

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Groupie

AND ALL SECTIONS OF MF= 3 MUST BE CORRECT. THE PROGRAM COPIES ALL  Groupie

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Groupie

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Groupie

Groupie

ALL FILE 3 CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE    Groupie

LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B          Groupie

INTERPOLATION LAW 2). FILE 3 BACKGROUND CROSS SECTIONS MAY BE MADE Groupie

LINEARLY INTERPOLABLE USING PROGRAM LINEAR (UCRL-50400, VOL. 17,   Groupie

PART A). THE RESONANCE CONTRIBUTION MAY BE ADDED TO THE BACKGROUND Groupie

CROSS SECTIONS USING PROGRAM RECENT (UCRL-50400, VOL. 17, PART B). Groupie

IF THIS PROGRAM FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT       Groupie

LINEARLY INTERPOLABLE THIS PROGRAM WILL TERMINATE EXECUTION.       Groupie

Groupie

CONTENTS OF OUTPUT                                                 Groupie

------------------                                                 Groupie

IF ENDF/B FORMATTED OUTPUT IS REQUESTED ENTIRE EVALUATIONS ARE     Groupie

OUTPUT, NOT JUST THE MULTI-GROUPED FILE 3 CROSS SECTIONS, E.G.     Groupie

ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO INCLUDED.                Groupie

Groupie

DOCUMENTATION                                                      Groupie

-------------                                                      Groupie

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Groupie

BY THE ADDITION OF THREE COMMENT CARDS AT THE END OF EACH          Groupie

HOLLERITH SECTION TO DESCRIBE THE GROUP STRUCTURE AND WEIGHTING    Groupie

SPECTRUM, E.G.                                                     Groupie

Groupie

********************** PROGRAM GROUPIE (2007-1) ***************    Groupie

UNSHIELDED GROUP AVERAGES USING   69 GROUPS (WIMS)                 Groupie

MAXWELLIAN, 1/E AND FISSION WEIGHTING SPECTRUM                     Groupie

Groupie

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR, RECENT AND SIGMA1) Groupie

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Groupie

THE DATA.                                                          Groupie

Groupie

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Groupie

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Groupie

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Groupie

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Groupie

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Groupie

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Groupie

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Groupie

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Groupie

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Groupie

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Groupie

Groupie

REACTION INDEX                                                     Groupie

--------------                                                     Groupie

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Groupie

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Groupie

Groupie

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Groupie

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Groupie

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Groupie

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Groupie

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Groupie

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Groupie

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Groupie

Groupie

SECTION SIZE                                                       Groupie

------------                                                       Groupie

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Groupie

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Groupie

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Groupie

Groupie

SELECTION OF DATA                                                  Groupie

-----------------                                                  Groupie

THE PROGRAM SELECTS MATERIALS TO BE PROCESSED BASED EITHER ON      Groupie

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Groupie

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Groupie

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Groupie

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Groupie

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Groupie

Groupie

ENERGY ORDER AND UNITS                                             Groupie

----------------------                                             Groupie

ALL ENERGIES (FOR CROSS SECTIONS, WEIGHTING SPECTRUM OR GROUP      Groupie

BOUNDARIES) MUST BE IN UNITS OF EV AND MUST BE IN ASCENDING        Groupie

NUMERICAL ORDER.                                                   Groupie

Groupie

ENERGY GRID                                                        Groupie

-----------                                                        Groupie

ALTHOUGH ALL REACTIONS MUST TO LINEARLY INTERPOLABLE, THEY DO NOT  Groupie

ALL HAVE TO USE THE SAME ENERGY GRID. EACH REACTION CAN BE GIVEN   Groupie

BY AN INDEPENDENT ENERGY GRID. THIS PROGRAM WILL PROCEED FROM      Groupie

THE LOWEST TO HIGHEST ENERGY SELECTING EACH ENERGY INTERVAL OVER   Groupie

WHICH ALL DATA, FOR ANY GIVEN CALCULATION, ARE ALL LINEARLY        Groupie

INTERPOLABLE.                                                      Groupie

Groupie

GROUP STRUCTURE                                                    Groupie

---------------                                                    Groupie

THIS PROGRAM IS DESIGNED TO USE AN ARBITRARY ENERGY GROUP          Groupie

STRUCTURE WHERE THE ENERGIES ARE IN EV AND ARE IN INCREASING       Groupie

ENERGY ORDER. THE MAXIMUM NUMBER OF GROUPS IS 1000.                Groupie

Groupie

THE USER MAY INPUT AN ARBITRARY GROUP STRUCTURE OR THE USER MAY    Groupie

USE USE ONE OF THE SEVEN BUILT-IN GROUP STRUCTURES.                Groupie

(0) 175 GROUP (TART STRUCTURE)                                     Groupie

(1)  50 GROUP (ORNL STRUCTURE)                                     Groupie

(2) 126 GROUP (ORNL STRUCTURE)                                     Groupie

(3) 171 GROUP (ORNL STRUCTURE)                                     Groupie

(4) 620 GROUP (SAND-II STRUCTURE, UP TO 18 MEV)                    Groupie

(5) 640 GROUP (SAND-II STRUCTURE, UP TO 20 MEV)                    Groupie

(6)  69 GROUP (WIMS STRUCTURE)                                     Groupie

(7)  68 GROUP (GAM-I STRUCTURE)                                    Groupie

(8)  99 GROUP (GAM-II STRUCTURE)                                   Groupie

(9)  54 GROUP (MUFT STRUCTURE)                                     Groupie

(10)  28 GROUP (ABBN STRUCTURE)                                     Groupie

(11) 650 GROUP (TART STRUCTURE)                                     Groupie

(12) 700 GROUP (TART STRUCTURE)                                     Groupie

(13) 665 GROUP (SAND-II STRUCTURE, 1.0e-5 eV, UP TO 18 MEV)         Groupie

(14) 685 GROUP (SAND-II STRUCTURE, 1.0e-5 eV, UP TO 20 MEV)         Groupie

Groupie

GROUP AVERAGES                                                     Groupie

--------------                                                     Groupie

THIS PROGRAM DEFINES GROUP AVERAGED CROSS SECTIONS AS...           Groupie

Groupie

(INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)             Groupie

AVERAGE = -----------------------------------------                Groupie

(INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)                      Groupie

WHERE...                                                           Groupie

Groupie

AVERAGE  = GROUP AVERAGED CROSS SECTION                            Groupie

E1, E2   = ENERGY LIMITS OF THE GROUP                              Groupie

SIGMA(E) = ENERGY DEPENDENT CROSS SECTION FOR ANY GIVEN REACTION   Groupie

S(E)     = ENERGY DEPENDENT WEIGHTING SPECTRUM                     Groupie

WT(E)    = ENERGY DEPENDENT SELF-SHIELDING FACTOR.                 Groupie

Groupie

ENERGY DEPENDENT WEIGHTING SPECTRUM                                Groupie

-----------------------------------                                Groupie

THE ENERGY DEPENDENT WEIGHTING SPECTRUM IS GIVEN BY AN ARBITRARY   Groupie

TABULATED LINERLY INTERPOLABLE FUNCTION WHICH CAN BE DESCRIBED     Groupie

BY AN ARBITRARY NUMBER OF POINTS. THIS ALLOWS THE USER TO          Groupie

SPECIFY ANY DESIRED WEIGHTING SPECTRUM TO ANY GIVEN DEGREE OF      Groupie

ACCURACY. REMEMBER THAT THE PROGRAM WILL ASSUME THAT THE SPECTRUM  Groupie

IS LINEARLY INTERPOLABLE BETWEEN TABULATED POINTS. THEREFORE THE   Groupie

USER SHOULD USE ENOUGH POINTS TO INSURE AN ADEQUATE REPRESENTATION Groupie

OF THE SPECTRUM BETWEEN TABULATED DATA POINTS.                     Groupie

Groupie

THE PRESENT VERSION OF THE CODE HAS THREE BULIT-IN WEIGHTING       Groupie

SPECTRA,                                                           Groupie

Groupie

(1) CONSTANT                                                       Groupie

(2) 1/E                                                            Groupie

(3) MAXWELLIAN = E*EXP(-E/KT)/KT                (0.0 TO 4*KT)      Groupie

1/E        = C1/E                           (4*KT TO 67 KEV)   Groupie

FISSION    = C2*EXP(-E/WA)*SINH(SQRT(E*WB)) (ABOVE 67 KEV)     Groupie

Groupie

KT     = 0.253 EV (293 KELVIN)                                 Groupie

WA     = 9.65E+5                                               Groupie

WB     = 2.29E-6                                               Groupie

C1, C2 = DEFINED TO MAKE SPECTRUM CONTINUOUS                   Groupie

Groupie

FISSION SPECTRUM CONSTANTS FROM                                Groupie

A.F.HENRY, NUCLEAR REACTOR ANALYSIS, P. 11, MIT PRESS (1975)   Groupie

Groupie

UNSHIELDED GROUP AVERAGES                                          Groupie

-------------------------                                          Groupie

FOR UNSHIELDED AVERAGES THE SELF-SHIELDING FACTOR (WT(E)) IS SET   Groupie

TO UNITY. THIS PROGRAM ALLOWS UP TO 1000 GROUPS.                   Groupie

Groupie

SELF-SHIELDED GROUP AVERAGES                                       Groupie

----------------------------                                       Groupie

IF SELF-SHIELDED AVERAGES AND/OR MULTI-BAND PARAMETERS ARE         Groupie

CALCULATED THIS PROGRAM ALLOWS UP TO 1000 GROUPS. SELF-SHIELDED    Groupie

AVERAGES AND/OR MULTI-BAND PARAMETERS ARE CALCULATED FOR THE       Groupie

TOTAL, ELASTIC, CAPTURE AND FISSION.                               Groupie

Groupie

FOR THE TOTAL, ELASTIC, CAPTURE AND FISSION THE PROGRAM USES A     Groupie

WEIGHTING FUNCTION THAT IS A PRODUCT OF THE ENERGY DEPENDENT       Groupie

WEIGHTING SPECTRUM TIMES A BONDERENKO TYPE SELF-SHIELDING FACTOR.  Groupie

Groupie

WT(E) = S(E)/(TOTAL(E)+SIGMA0)**N                                  Groupie

Groupie

WHERE...                                                           Groupie

Groupie

S(E)     - ENERGY DEPENDENT WEIGHTING SPECTRUM (DEFINED BY         Groupie

TABULATED VALUES AND LINEAR INTERPOLATION BETWEEN       Groupie

TABULATED VALUES).                                      Groupie

TOTAL(E) - ENERGY DEPENDENT TOTAL CROSS SECTION FOR ONE MATERIAL   Groupie

(DEFINED BY TABULATED VALUES AND LINEAR INTERPOLATION   Groupie

BETWEEN TABULATED VALUES).                              Groupie

SIGMA0   - CROSS SECTION TO REPRESENT THE EFFECT OF ALL OTHER      Groupie

MATERIALS AND LEAKAGE (DEFINED WITHIN EACH GROUP TO BE  Groupie

A MULTIPLE OF THE UNSHIELDED TOTAL CROSS SECTION WITHIN Groupie

THAT GROUP OR POWERS OF 10 - INPUT OPTION).             Groupie

N        - A POSITIVE INTEGER (0, 1, 2 OR 3).                      Groupie

Groupie

THE PROGRAM WILL USE ONE ENERGY DEPENDENT WEIGHTING SPECTRUM S(E)  Groupie

AND 25 DIFFERENT BONDERENKO TYPE SELF-SHIELDING FACTORS (25 SIGMA0 Groupie

AND N COMBINATIONS) TO DEFINE 25 DIFFERENT AVERAGE CROSS SECTIONS, Groupie

FOR EACH REACTION, WITHIN EACH GROUP.                              Groupie

Groupie

THE 25 WEIGHTING FUNCTIONS USED ARE....                            Groupie

(1)   - UNSHIELDED CROSS SECTIONS (N=0)                            Groupie

(2-22)- PARTIALLY SHIELDED CROSS SECTIONS (N=1 ,VARIOUS SIGMA0)    Groupie

THE VALUES OF SIGMA0 USED WILL BE EITHER,                  Groupie

(A) THE VALUES OF SIGMA0 THAT ARE USED VARY FROM 1024      Groupie

TIMES THE UNSHIELDED TOTAL CROSS SECTIONS IN STEPS OF 1/2  Groupie

DOWN TO 1/1024 TIMES THE UNSHIELDED TOTAL CROSS SECTION    Groupie

(A RANGE OF OVER 1 MILLION, CENTERED ON THE UNSHIELDED     Groupie

TOTAL CROSS SECTION WITHIN EACH GROUP).                    Groupie

(B) THE SAME CONSTANT VALUES OF SIGMA0 IN EACH GROUP. THE  Groupie

VALUES OF SIGMA0 USED INCLUDE 40000, 20000, 10000, 7000,   Groupie

4000, 2000, 1000, 700, 400, 200, 100, 70, 40, 20, 10, 7,   Groupie

4, 2, 1, 0.7, 0.4 (A RANGE OF 100,000 SPANNING MORE THAN   Groupie

THE RANGE OF SIGMA0 VALUES THAT MAY BE ENCOUNTERED IN      Groupie

ACTUAL APPLICATIONS)                                       Groupie

(23)  - TOTALLY SHIELDED FLUX WEIGHTED CROSS SECTION               Groupie

(N=1, SIGMA0=0)                                            Groupie

(24)  - TOTALLY SHIELDED CURRENT WEIGHTED CROSS SECTION            Groupie

(N=2, SIGMA0=0)                                            Groupie

(25)  - TOTALLY SHIELDED COSINE SQUARED WEIGHTED CROSS SECTION     Groupie

(N=3, SIGMA0=0)                                            Groupie

Groupie

FOR ALL OTHER REACTIONS (EXCEPT TOTAL, ELASTIC, CAPTURE AND        Groupie

FISSION) THE PROGRAM WILL USE THE ENERGY DEPENDENT WEIGHTING       Groupie

SPECTRUM S(E) TO DEFINE THE UNSHIELDED (BONDERENKO N=0)            Groupie

AVERAGED CROSS SECTION WITHIN EACH GROUP.                          Groupie

Groupie

CALCULATION OF RESONANCE INTEGRALS                                 Groupie

----------------------------------                                 Groupie

IN A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH A         Groupie

CONSTANT CROSS SECTION THE SPECTRUM WILL BE 1/E AND THERE WILL     Groupie

BE NO SELF-SHIELDING.                                              Groupie

Groupie

IN THIS CASE IF THE CROSS SECTION VARIES WITH ENERGY THE           Groupie

SPECTRUM WILL STILL BE 1/E AND THE SELF-SHIELDING FACTOR WILL      Groupie

BE EXACTLY 1/SIG-TOT(E) - WHERE SIG-TOT(E) = SIG-EL(E), SINCE      Groupie

THERE IS ONLY SCATTERING.                                          Groupie

Groupie

IF WE HAVE AN INFINITELY DILUTE AMOUNT OF A MATERIAL UNIFORMLY     Groupie

MIXED WITH A PURE ELASTIC ISOTROPICALLY SCATTERING MATERIAL WITH   Groupie

A CONSTANT CROSS SECTION THE STANDARD DEFINITION OF THE RESONANCE  Groupie

INTEGRAL CAN BE USED TO DEFINE REACTION RATES FOR EACH REACTION.   Groupie

Groupie

THE RESONANCE INTEGRAL IS DEFINED AS,                              Groupie

Groupie

RI      = (INTEGRAL E1 TO E2) (SIGMA(E)*S(E)*WT(E)*DE)             Groupie

Groupie

WHERE NORMALLY,                                                    Groupie

S(E)    = 1/E                                                      Groupie

WT(E)   = 1    - NO SELF-SHIELDING                                 Groupie

Groupie

FROM THE ABOVE DEFINITION OF GROUP AVERAGED CROSS SECTIONS THE     Groupie

RESONANCE INTEGRAL IS,                                             Groupie

Groupie

RI      = AVERAGE * (INTEGRAL E1 TO E2) (S(E)*WT(E)*DE)            Groupie

Groupie

FOR A 1/E SPECTRUM AND NO SELF-SHIELDING THIS REDUCES TO,          Groupie

Groupie

RI      = AVERAGE* LOG(E2/E1)                                      Groupie

Groupie

IN ANY OTHER SITUATION, INCLUDING ABSORPTION AND/OR ENERGY         Groupie

DEPENDENT CROSS SECTIONS, THE SPECTRUM WILL NOT BE 1/E -           Groupie

ABSORPTION WILL TEND TO DECREASE THE SPECTRUM PROGRESSIVELY        Groupie

MORE AT LOWER ENERGIES - ENERGY DEPENDENCE OF THE CROSS SECTION    Groupie

WILL LEAD TO SELF-SHIELDING.                                       Groupie

Groupie

HERE WE WILL NOT ATTEMPT TO PERFORM A DETAILED SPECTRUM            Groupie

CALCULATION TO ACCOUNT FOR ABSORPTION.                             Groupie

Groupie

HOWEVER, WE WILL EXTEND THE DEFINITION OF THE RESONANCE INTEGRAL   Groupie

TO ACCOUNT FOR SELF-SHIELDING EFFECTS BY ALLOWING FOR INCLUSION    Groupie

OF SELF-SHIELDING EFFECTS IN THE DEFINITION OF GROUP AVERAGES      Groupie

AND THEN DEFINING THE RESONANCE INTEGRAL AS,                       Groupie

Groupie

RI      = AVERAGE* LOG(E2/E1)                                      Groupie

Groupie

IN ORDER TO CALCULATE RESONANCE INTEGRALS YOU MUST FOLLOW THESE    Groupie

STEPS,                                                             Groupie

Groupie

1) SELECT A 1/E SPECTRUM - ON FIRST LINE OF INPUT PARAMETERS.      Groupie

2) SELECT THE ENERGY BOUNDARIES - NORMALLY ONLY 1 GROUP FROM       Groupie

0.5 EV UP TO 20 MEV - HOWEVER, YOU ARE FREE TO SELECT ANY       Groupie

ENERGY RANGE THAT YOU WISH - YOU MAY EVEN SELECT MORE THAN      Groupie

1 GROUP MERELY BY SPECIFYING MORE THAN 1 GROUP AS INPUT -       Groupie

THIS CAN BE USED TO DEFINE THE CONTRIBUTIONS TO THE RESONANCE   Groupie

INTEGRAL FROM INDIVIDUAL ENERGY RANGES.                         Groupie

3) SELECT THIS OPTION FOR THE UNSHIELDED AND/OR SHIELDED OUTPUT    Groupie

LISTING - ON THE SECOND LINE OF INPUT PARAMETERS.               Groupie

Groupie

WHEN THIS OPTION IS USED THE PROGRAM WILL CALCULATE GROUP AVERAGED Groupie

CROSS SECTIONS - AS DEFINED ABOVE - PRIOR TO OUTPUT THE RESULTS    Groupie

WILL MERELY BE MULTIPLIED BY THE WIDTH OF THE GROUP ASSUMING YOU   Groupie

HAVE SELECTED A 1/E SPECTRUM - THERE IS NO CHECK ON THIS - THE     Groupie

PROGRAM MERELY MULTIPLIES THE GROUP AVERAGED CROSS SECTIONS BY,    Groupie

Groupie

LOG(E2/E1) - WHERE E2 AND E1 ARE THE GROUP ENERGY BOUNDARIES.      Groupie

Groupie

WARNING - IT IS UP TO YOU TO INSURE THAT YOU FOLLOW EXACTLY THE    Groupie

STEPS OUTLINED ABOVE IF YOU WISH TO OBTAIN MEANINGFUL    Groupie

RESULTS.                                                 Groupie

Groupie

NOTE - OUTPUT IN THE ENDF/B FORMAT IS ALWAYS GROUP AVERAGED CROSS  Groupie

SECTIONS, REGARDLESS OF WHETHER YOU ASK FOR AVERAGED CROSS  Groupie

SECTIONS OR RESONANCE INTEGRALS - THIS IS BECAUSE DATA IN   Groupie

THE ENDF/B FORMAT IS EXPLICITLY DEFINED TO BE CROSS         Groupie

SECTIONS.                                                   Groupie

Groupie

RESONANCE INTEGRAL OUTPUT CAN ONLY BE OBTAINED IN THE       Groupie

LISTING FORMATS.                                            Groupie

Groupie

MINIMUM TOTAL CROSS SECTION TREATMENT                              Groupie

-------------------------------------                              Groupie

SINCE THE BONDARENKO SELF-SHIELDING DEPENDS ON 1/TOTAL CROSS       Groupie

SECTION, THE ALGORITHM WILL BECOME NUMERICALLY UNSTABLE IF THE     Groupie

TOTAL CROSS SECTION IS NEGATIVE (AS OCCURS IN MANY ENDF/B          Groupie

EVALUATIONS). IF THE TOTAL IS LESS THAN SOME MINIMUM ALLOWABLE     Groupie

VALUE (DEFINE BY OKMIN, PRESENTLY 1 MILLI-BARN) AN ERROR MESSAGE   Groupie

WILL BE PRINTED AND FOR THE SELF-SHIELDING CALCULATION ALL ENERGY  Groupie

INTERVALS IN WHICH THE TOTAL IS LESS THAN THE MINIMUM WILL BE      Groupie

IGNORED.                                                           Groupie

Groupie

NOTE, FOR THE UNSHIELDED CALCULATIONS ALL CROSS SECTIONS WILL BE   Groupie

CONSIDERED WHETHER THEY ARE POSITIVE OR NEGATIVE. THEREFORE IF     Groupie

THE TOTAL CROSS SECTION IS NEGATIVE OR LESS THAN THE MINIMUM       Groupie

VALUE THERE MAY BE AN INCONSISTENCY BETWEEN THE UNSHIELDED AND     Groupie

THE SELF-SHIELDED CROSS SECTIONS. IF THE TOTAL CROSS SECTION IS    Groupie

NEGATIVE AND SELF-SHIELDED CROSS SECTIONS ARE CALCULATED THE       Groupie

PROGRAM WILL PRINT AN ERROR MESSAGE INDICATING THAT THE SELF-      Groupie

SHIELDED RESULTS ARE UNRELIABLE AND SHOULD NOT BE USED. THEREFORE  Groupie

IN THIS CASE THE PROGRAM WILL NOT ATTEMPT TO MODIFY THE UNSHIELDED Groupie

RESULTS TO ELIMINATE THE EFFECT OF NEGATIVE CROSS SECTIONS, SINCE  Groupie

THE UNSHIELDED RESULTS ARE THE ONLY ONES WHICH TRULY REFLECT THE   Groupie

ACTUAL INPUT.                                                      Groupie

Groupie

RESOLVED RESONANCE REGION                                          Groupie

-------------------------                                          Groupie

IN THE RESOLVED RESONANCE REGION (ACTUALLY EVERYWHERE BUT IN THE   Groupie

UNRESOLVED RESONANCE REGION) THE CROSS SECTIONS OUTPUT BY LINEAR-  Groupie

RECENT-SIGMA1 WILL BE ACTUAL ENERGY DEPENDENT CROSS SECTIONS AND   Groupie

THE CALCULATIONS BY THIS PROGRAM WILL YIELD ACTUAL SHIELDED AND    Groupie

UNSHIELDED CROSS SECTIONS.                                         Groupie

Groupie

UNRESOLVED RESONANCE REGION                                        Groupie

---------------------------                                        Groupie

IN THE UNRESOLVED RESONANCE REGION PROGRAM RECENT USES THE         Groupie

UNRESOLVED RESONANCE PARAMETERS TO CALCULATE INFINITELY DILUTE     Groupie

AVERAGE CROSS SECTIONS. THIS PROGRAM WILL MERELY READ THIS         Groupie

INFINITELY DILUTE DATA AS IF IT WERE ENERGY DEPENDENT DATA AND     Groupie

GROUP AVERAGE IT. AS SUCH THIS PROGRAM WILL PRODUCE THE CORRECT    Groupie

UNSHIELDED CROSS SECTION IN THE UNRESOLVED RESONANCE REGION, BUT   Groupie

IT WILL NOT PRODUCE THE CORRECT SELF-SHIELDING EFFECTS.            Groupie

Groupie

ACCURACY OF RESULTS                                                Groupie

-------------------                                                Groupie

ALL INTEGRALS ARE PERFORMED ANALYTICALLY. THEREFORE NO ERROR IS    Groupie

INTRODUCED DUE TO THE USE OF TRAPAZOIDAL OR OTHER INTEGRATION      Groupie

SCHEME. THE TOTAL ERROR THAT CAN BE ASSIGNED TO THE RESULTING      Groupie

AVERAGES IS JUST THAT DUE TO THE ERROR IN THE CROSS SECTIONS       Groupie

AND ENERGY DEPENDENT WEIGHTING SPECTRUM. GENERALLY SINCE THE       Groupie

THE ENERGY DEPENDENT WEIGHTING SPECTRUM APPEARS IN BOTH THE        Groupie

NUMERATOR AND THE DENOMINATOR THE AVERAGES RAPIDLY BECOME          Groupie

INSENSITIVE TO THE WEIGHTING SPECTRUM AS MORE GROUPS ARE USED.     Groupie

SINCE THE WEIGHTING SPECTRUM IS LOADED IN THE PAGING SYSTEM THE    Groupie

USER CAN DESCRIBE THE SPECTRUM TO ANY REQUIRED ACCURACY USING      Groupie

ANY NUMBER OF ENERGY VS. SPECTRUM PAIRS.                           Groupie

Groupie

MULTI-BAND PARAMETERS                                              Groupie

---------------------                                              Groupie

MULTI-BAND PARAMETERS ARE CALCULATED FOR THE TOTAL, ELASTIC,       Groupie

CAPTURE AND FISSION REACTIONS. WITH THE NUMBER OF GROUPS THAT      Groupie

ARE NORMALLY USED (SEE BUILT IN GROUP STRUCTURES) ALL OTHER        Groupie

REACTIONS RESULT IN A NEGLIGABLE AMOUNT OF SELF-SHIELDING. AS      Groupie

SUCH THEIR EQUIVALENT BAND CROSS SECTION WILL MERELY BE THEIR      Groupie

UNSHIELDED VALUE WITHIN EACH BAND.                                 Groupie

Groupie

FOR ANY GIVEN EVALUATION, WITHIN ANY GIVEN GROUP THIS PROGRAM      Groupie

WILL GENERATE THE MINIMUM NUMBER OF BANDS REQUIRED WITHIN THAT     Groupie

GROUP. AS OUTPUT TO THE COMPUTER READABLE DISK FILE THE BAND       Groupie

PARAMETERS FOR EACH EVALUATION WILL BE FORMATTED TO HAVE THE       Groupie

SAME NUMBER OF BANDS IN ALL GROUPS (WITH ZERO WEIGHT FOR SOME      Groupie

BANDS WITHIN ANY GROUP). THE USER MAY DECIDE TO HAVE OUTPUT        Groupie

EITHER WITH THE MINIMUM NUMBER OF BANDS REQUIRED FOR EACH          Groupie

EVALUATION (E.G. 2 BANDS FOR HYDROGEN AND 4 BANDS FOR U-233) OR    Groupie

THE SAME NUMBER OF BANDS FOR ALL EVALUATIONS (E.G. 4 BANDS FOR     Groupie

BOTH HYDROGEN AND U-233).                                          Groupie

Groupie

FOR 2 OR FEWER BANDS THE PROGRAM USES AN ANALYTIC EXPRESSION       Groupie

TO DEFINE ALL MULTI-BAND PARAMETERS. FOR MORE THAN 2 BANDS THE     Groupie

PROGRAM PERFORMS A NON-LINEAR FIT TO SELECT THE MULTI-BAND         Groupie

PARAMETERS THAT MINIMIZE THE MAXIMUM FRACTIONAL ERROR AT ANY       Groupie

POINT ALONG THE ENTIRE SELF-SHIELDING CURVE. THE NUMBER OF BANDS   Groupie

REQUIRED WITHIN ANY GIVEN GROUP IS DEFINED BY INSURING THAT THE    Groupie

MULTI-BAND PARAMETERS CAN BE USED TO ACCURATELY DEFINE SELF-       Groupie

SHIELDED CROSS SECTIONS ALONG THE ENTIRE SELF-SHIELDING CURVE      Groupie

FROM SIGMA0 = 0 TO INFINITY. THE USER MAY DEFINE THE ACCURACY      Groupie

REQUIRED.                                                          Groupie

Groupie

ENDF/B FORMATTED UNSHIELDED AVERAGES                               Groupie

------------------------------------                               Groupie

UNSHIELDED MULTI-GROUP AVERAGED CROSS SECTIONS FOR ALL REACTIONS   Groupie

MAY BE OBTAINED IN THE ENDF/B FORTRAN IN EITHER HISTOGRAM          Groupie

(INTERPOLATION LAW 1) OR LINEARLY INTERPOLABLE (INTERPOLATION      Groupie

LAW 2) FORM. SEE INPUT BELOW FOR DETAILS.                          Groupie

Groupie

MIXTURES OF MATERIALS AND RESONANCE OVERLAP                        Groupie

-------------------------------------------                        Groupie

THE SELF-SHIELDED CROSS SECTIONS FOR THE INDIVIDUAL CONSTITUENTS   Groupie

OF ANY MIXTURE CAN BE CALCULATED BY THIS PROGRAM BY REALIZING THAT Groupie

THIS PROGRAM ESSENTIALLY ONLY USES THE TOTAL CROSS SECTION AS A    Groupie

WEIGHTING FUNCTION TO ACCOUNT FOR SELF-SHIELDING EFFECTS. FOR A    Groupie

MIXTURE IT IS THEREFORE ONLY NECESSARY TO USE THE TOTAL CROSS      Groupie

SECTION FOR THE MIXTURE IN PLACE OF THE ACTUAL TOTAL CROSS SECTION Groupie

FOR EACH CONSTITUENT AND TO RUN THIS PROGRAM. THIS CAN BE DONE BY  Groupie

FIRST RUNNING PROGRAM MIXER TO CALCULATE THE ENERGY DEPENDENT      Groupie

TOTAL CROSS SECTION FOR ANY COMPOSITE MIXTURE. NEXT, SUBSTITUTE    Groupie

THIS COMPOSITE TOTAL CROSS SECTION FOR THE ACTUAL TOTAL CROSS      Groupie

SECTION OF EACH CONSTITUENT (IN EACH ENDF/B FORMATTED EVALUATION). Groupie

FINALLY, RUN THIS PROGRAM TO CALCULATE THE SELF-SHIELDED CROSS     Groupie

SECTION FOR EACH CONSTITUENT, PROPERLY ACCOUNTING FOR RESONANCE    Groupie

OVERLAP BETWEEN THE RESONANCES OF ALL OF THE CONSTITUENTS OF THE   Groupie

MIXTURE. DURING THE SAME RUN THESE SELF-SHIELDED CROSS SECTIONS    Groupie

CAN IN TURN BE USED TO CALCULATE FULLY CORRELATED MULT-BAND        Groupie

Groupie

MULTI-BAND PARAMETER OUTPUT FORMAT                                 Groupie

----------------------------------                                 Groupie

FOR VERSIONS 92-2 AND LATER VERSIONS THE MULTI-BAND PARAMETERS     Groupie

ARE OUTPUT IN A SIMPLE CHARACTER FORMAT, THAT CAN BE TRANSFERRED   Groupie

AND USED ON VIRTUALLY ANY COMPUTER.                                Groupie

Groupie

THE BINARY FORMAT USED IN EARLIER VERSIONS OF THIS CODE IS NO      Groupie

LONGER USED.                                                       Groupie

Groupie

CONTACT THE AUTHOR IF YOU WOULD LIKE TO RECEIVE A SIMPLE PROGRAM   Groupie

TO READ THE CHARACTER FORMATTED MULTI-BAND PARAMETER FILE AND      Groupie

CREATE A BINARY, RANDOM ACCESS FILE FOR USE ON VIRTUALLY ANY       Groupie

COMPUTER.                                                          Groupie

Groupie

THE FORMAT OF THE CHARACTER FILE IS,                               Groupie

Groupie

RECORD   COLUMNS   FORMAT   DESCRIPTION                            Groupie

1       1-72     18A4    LIBRARY DESCRIPTION (AS READ)          Groupie

2       1-11      I11    MATERIAL ZA                            Groupie

12-22      I11    NUMBER GROUPS                          Groupie

23-33      I11    NUMBER OF BANDS                        Groupie

34-44     D11.4   TEMPERATURE (KELVIN)                   Groupie

45-55    1X,10A1  HOLLERITH DESCRIPTION OF ZA            Groupie

3       1-11     D11.4   ENERGY (EV) - GROUP BOUNDARY.          Groupie

12-22     D11.4   TOTAL      (FIRST BAND)                Groupie

23-33     D11.4   ELASTIC                                Groupie

34-44     D11.4   CAPTURE                                Groupie

35-55     D11.4   FISSION                                Groupie

4       1-11     -----   BLANK                                  Groupie

12-22     D11.4   TOTAL      (SECOND BAND)               Groupie

23-33     D11.4   ELASTIC                                Groupie

34-44     D11.4   CAPTURE                                Groupie

35-55     D11.4   FISSION                                Groupie

Groupie

LINES 3 AND 4 ARE REPEATED FOR EACH GROUP. THE LAST LINE FOR EACH  Groupie

MATERIAL (ZA) IS,                                                  Groupie

Groupie

N       1-11     D11.4   ENERGY (EV) - UPPER ENERGY LIMIT OF    Groupie

LAST GROUP.              Groupie

Groupie

FOR EXAMPLE, A 175 GROUP, 2 BAND FILE, FOR EACH MATERIAL WILL      Groupie

CONTAIN 352 LINES = 1 HEADER LINE, 175 * 2 LINES OF PARAMETERS,    Groupie

AND 1 FINAL LINE WITH THE UPPER ENERGY LIMIT   Groupie

OF THE LAST GROUP.                             Groupie

Groupie

INPUT FILES                                                        Groupie

-----------                                                        Groupie

UNIT  DESCRIPTION                                                  Groupie

----  -----------                                                  Groupie

2   INPUT DATA (BCD - 80 CHARACTERS/RECORD)                      Groupie

10   ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Groupie

Groupie

OUTPUT FILES                                                       Groupie

------------                                                       Groupie

UNIT  DESCRIPTION                                                  Groupie

----  -----------                                                  Groupie

31   MULTI-BAND PARAMETERS CHARACTER FILE - OPTIONAL              Groupie

(BCD - 80 CHARACTERS/RECORD)                                 Groupie

32   SELF-SHIELDED CROSS SECTION LISTING - OPTIONAL               Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

33   MULTI-BAND PARAMETER LISTING - OPTIONAL                      Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

34   UNSHIELDED CROSS SECTION LISTING - OPTION                    Groupie

(BCD - 120 CHARACTERS/RECORD)                                Groupie

3   OUTPUT REPORT (BCD - 80 CHARACTERS/RECORD)                   Groupie

11   MULTI-GROUP ENDF/B DATA - OPTIONAL                           Groupie

(BCD - 80 CHARACTERS/RECORD)                                 Groupie

Groupie

SCRATCH FILES                                                      Groupie

-------------                                                      Groupie

UNIT  FILENAME  DESCRIPTION                                        Groupie

----  --------  -----------                                        Groupie

8   ENERGY DEPENDENT WEIGHTING SPECTRUM                          Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

9   TOTAL CROSS SECTION                                          Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

12   ELASTIC CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

13   CAPTURE CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

14   FISSION CROSS SECTION - ONLY FOR SELF-SHIELDING CALCULATION  Groupie

(BINARY - 40080 WORDS/BLOCK)                                 Groupie

Groupie

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINES FILIO1 AND FILIO2)   Groupie

----------------------------------------------------------------   Groupie

UNIT  FILE NAME                                                    Groupie

----  ----------                                                   Groupie

2   GROUPIE.INP                                                  Groupie

3   GROUPIE.LST                                                  Groupie

8   (SCRATCH)                                                    Groupie

9   (SCRATCH)                                                    Groupie

10   ENDFB.IN                                                     Groupie

11   ENDFB.OUT                                                    Groupie

12   (SCRATCH)                                                    Groupie

13   (SCRATCH)                                                    Groupie

14   (SCRATCH)                                                    Groupie

31   MULTBAND.TAB                                                 Groupie

32   SHIELD.LST                                                   Groupie

33   MULTBAND.LST                                                 Groupie

34   UNSHIELD.LST                                                 Groupie

Groupie

I/O UNITS USED                                                    Groupie

--------------                                                    Groupie

UNITS 2, 3 8, 9 AND 10 WILL ALWAYS BE USED.                       Groupie

UNITS 31 THROUGH 34 AND 11 ARE OPTIONALLY USED DEPENDING ON THE   Groupie

OUTPUT REQUESTED.                                                 Groupie

UNITS 12, 13 AND 14 WILL ONLY BE USED IF SELF-SHIELDED OR         Groupie

MULTIBAND OUTPUT IS REQUESTED.                                    Groupie

Groupie

INPUT CARDS                                                        Groupie

-----------                                                        Groupie

CARD  COLS.  FORMAT  DESCRIPTION                                   Groupie

----  -----  ------  -----------                                   Groupie

1    1-11    I11   SELECTION CRITERIA (0=MAT, 1=ZA)              Groupie

1   12-22    I11   NUMBER OF GROUPS.                             Groupie

=.GT.0 - ARBITRARY GROUP BOUNDARIES ARE READ  Groupie

FROM INPUT FILE (N GROUPS REQUIRE    Groupie

N+1 GROUP BOUNDARIES). CURRENT       Groupie

PROGRAM MAXIMUM IS 1000 GROUPS.      Groupie

BUILT-IN OPTIONS INCLUDE....         Groupie

=  0   - TART    175 GROUPS                   Groupie

= -1   - ORNL     50 GROUPS                   Groupie

= -2   - ORNL    126 GROUPS                   Groupie

= -3   - ORNL    171 GROUPS                   Groupie

= -4   - SAND-II 620 (665) GROUPS TO 18 MEV   Groupie

= -5   - SAND-II 640 (685) GROUPS TO 20 MEV   Groupie

= -6   - WIMS     69 GROUPS                   Groupie

= -7   - GAM-I    68 GROUPS                   Groupie

= -8   - GAM-II   99 GROUPS                   Groupie

= -9   - MUFT     54 GROUPS                   Groupie

=-10   - ABBN     28 GROUPS                   Groupie

=-11   - TART    650 GROUPS                   Groupie

=-12   - TART    700 GROUPS                   Groupie

=-13   - SAND-II 665 GROUPS TO 18 MEV         Groupie

=-14   - SAND-II 685 GROUPS TO 20 MEV         Groupie

1   23-33    I11   MULTI-BAND SELECTOR                           Groupie

=  0 - NO MULTI-BAND CALCULATIONS             Groupie

=  1 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)    Groupie

AND AV(1/TOT**2)                       Groupie

=  2 - 2 BAND. CONSERVE AV(TOT), AV(1/TOT)    Groupie

AND AV(1/(TOT+SIGMA0)) WHERE           Groupie

SIGMA0 = AV(TOT) IN EACH GROUP         Groupie

= 3-5- MULTI-BAND FIT. CONSERVE AV(TOT) AND   Groupie

MINIMIZE FRACTIONAL ERROR FOR ENTIRE   Groupie

SELF-SHIELDING CURVE (SIGMA0 = 0 TO    Groupie

INFINITY)                              Groupie

IF THE SELECTOR IS POSITIVE (1 TO 5) THE      Groupie

MINIMUM NUMBER OF BANDS WILL BE OUTPUT FOR    Groupie

EACH ISOTOPE INDEPENDENTLY. IF THE SELECTOR   Groupie

IS NEGATIVE (-1 TO -5) THE SAME NUMBER OF     Groupie

BANDS (ABS(SELECTOR)) WILL BE OUTPUT FOR      Groupie

ALL ISOTOPES.                                 Groupie

1   34-44    I11   NUMBER OF POINTS USED TO DESCRIBE ENERGY      Groupie

DEPENDENT WEIGHTING SPECTRUM S(E).            Groupie

= -2    - MAXWELLIAN - UP TO 0.1 EV           Groupie

1/E        - 0.1 EV TO 67 KEV       Groupie

FISSION    - ABOVE 67 KEV           Groupie

05/01/20-----------------ADDED OPTION TO ALLOW TEMPERATURE OF THE      Groupie

MAXWELLIAN TO BE CHANGED - SEE INPUT LINE 4,  Groupie

COLUMNS 55 - 66.                              Groupie

= -1    - 1/E                                 Groupie

= 0 OR 1- ENERGY INDEPENDENT (SO CALLED FLAT  Groupie

WEIGHTING SPECTRUM).                Groupie

= .GT.1 - READ THIS MANY POINTS FROM INPUT    Groupie

TO DESCRIBE WEIGHTING SPECTRUM.     Groupie

NO LIMIT TO THE NUMBER OF POINTS    Groupie

USED TO DESCRIBE WEIGHTING.         Groupie

1   45-55   D11.4  MULTI-BAND CONVERGENCE CRITERIA.              Groupie

ONLY USED FOR 3 OR MORE BANDS. THE NUMBER OF  Groupie

BANDS IN EACH GROUPS IS SELECTED TO INSURE    Groupie

THAT THE ENTIRE SELF-SHIELDING CURVE CAN BE   Groupie

REPRODUCED TO WITHIN THIS FRACTIONAL ERROR.   Groupie

= .LT. 0.0001 - USE STANDARD 0.001            Groupie

(0.1 PER-CENT)                Groupie

= .GE. 0.0001 - USE AS CONVERGENCE CRITERIA   Groupie

1   56-66    I11   SIGMA-0 DEFINITION SELECTOR.                  Groupie

< 0 - 21 VALUES OF SIGMA0 ARE READ INPUT AND  Groupie

INTERPRETED AS FIXED VALUES = SAME AS   Groupie

= 1 DESCRIPTION BELOW                   Groupie

INPUT VALUES MUST ALL BE,               Groupie

1) GREATER THAN 0                       Groupie

2) IN DESCENDING VALUE ORDER            Groupie

= 0 - SIGMA-0 WILL BE DEFINED AS A MULTIPLE   Groupie

OF THE UNSHIELDED TOTAL CROSS SECTION   Groupie

IN EACH GROUP (VALUES OF 1/1024 TO      Groupie

1024 IN STEPS OF A FACTOR OF 2 WILL     Groupie

BE USED AS THE MULTIPLIER).             Groupie

= 1 - SIGMA-0 WILL BE DEFINED AS THE SAME     Groupie

NUMBER OF BARNS IN EACH GROUP (VALUES   Groupie

40000 TO 0.4 BARNS WILL BE USED. WITHIN Groupie

EACH DECADE VALUES OF 10, 7, 4, 2, 1    Groupie

BARNS WILL BE USED).                    Groupie

2-4    1-66 6D11.4   IF SIGMA-0 DEFINITION SELECTOR < 0, THE NEXT  Groupie

4 LINES OF INPUT ARE THE 22 VALUES OF SIGMA0, Groupie

6 PER LINE.                                   Groupie

2    1-60    A60   ENDF/B INPUT DATA FILENAME                    Groupie

(STANDARD OPTION = ENDFB.IN)                  Groupie

3    1-60    A60   ENDF/B OUTPUT DATA FILENAME                   Groupie

(STANDARD OPTION = ENDFB.OUT)                 Groupie

Groupie

THE FOURTH INPUT CARD IS USED TO SELECT ALL DESIRED OUTPUT MODES.  Groupie

EACH OUTPUT DEVICE MAY BE TURNED OFF (0) OR ON (1). THEREFORE      Groupie

THEREFORE EACH OF THE FOLLOWING INPUT PARAMETERS MAY BE EITHER     Groupie

ZERO TO INDICATE NO OUTPUT OR NON-ZERO TO INDICATE OUTPUT.         Groupie

Groupie

4     1-11   I11   SELF-SHIELDED CROSS SECTION LISTING           Groupie

= 1 - CROSS SECTIONS                          Groupie

= 2 - RESONANCE INTEGRALS                     Groupie

4    12-22   I11   MULTI-BAND PARAMETER LISTING                  Groupie

4    23-33   I11   MULTI-BAND PARAMETERS COMPUTER READABLE       Groupie

4    34-44   I11   UNSHIELDED CROSS SECTIONS IN ENDF/B FORMAT    Groupie

= 1 - HISTOGRAM FORMAT (INTERPOLATION LAW 1)  Groupie

= 2 - LINEAR-LINEAR (INTERPOLATION LAW 2)     Groupie

4    45-55   I11   UNSHIELDED CROSS SECTIONS LISTING             Groupie

= 1 - CROSS SECTIONS                          Groupie

= 2 - RESONANCE INTEGRALS                     Groupie

05/01/20 - ADDED THE BELOW OPTION                                      Groupie

4    56-66   D11.4 IF THE STANDARD BUILT-IN SPECTRA IS USED,     Groupie

INPUT LINE 1, COLUMNS 34-44 = 2, THIS FIELD   Groupie

CAN BE USED TO OPTIONALLY CHANGE TEMPERATURE  Groupie

OF THE MAXWELLIAN.                            Groupie

INPUT IS IN EV (0.0253 EV = ROOM TEMPERATURE) Groupie

= 0 - USE DEFAULT 0.0253 EV, ROOM TEMPERATURE Groupie

> 0 - USE THIS AS THE TEMPERATURE             Groupie

RESTRICTION - TEMPERATURE CANNOT EXCEED       Groupie

1000 EV.                                      Groupie

Groupie

5     1-80   18A4  LIBRARY IDENTIFICATION. ANY TEXT THAT THE     Groupie

USER WISHES TO IDENTIFY THE MULTI-BAND        Groupie

PARAMETERS. THIS LIBRARY IDENTIFICATION IS    Groupie

WRITTEN INTO THE COMPUTER READABLE MULTI-BAND Groupie

DATA FILE.                                    Groupie

Groupie

6-N    1- 6    I6   LOWER MAT OR ZA LIMIT                         Groupie

7- 8    I2   LOWER MF LIMIT                                Groupie

9-11    I3   LOWER MT LIMIT                                Groupie

12-17   I11   UPPER MAT OR ZA LIMIT                         Groupie

18-19    I2   UPPER MF LIMIT                                Groupie

20-22    I3   UPPER MT LIMIT                                Groupie

UP TO 100 RANGES MAY BE SPECIFIED, ONE RANGE  Groupie

PER LINE. THE LIST OF RANGES IS TERMINATED    Groupie

BY A BLANK CARD. IF THE UPPER MAT OR ZA       Groupie

LIMIT IS LESS THAN THE LOWER LIMIT THE UPPER  Groupie

IS SET EQUAL TO THE LOWER LIMIT. IF THE UPPER Groupie

MF OR MT LIMIT IS ZERO IT WILL BE SET EQUAL   Groupie

TO ITS MAXIMUM VALUE, 99 OR 999, RESPECTIVELY Groupie

IF THE FIRST REQUEST LINE IS BLANK IT WILL    Groupie

TERMINATE THE LIST OF REQUESTS AND CAUSE ALL  Groupie

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).     Groupie

Groupie

VARY   1-66  6D11.4 ENERGY GROUP BOUNDARIES. ONLY REQUIRED IF     Groupie

THE NUMBER OF GROUPS INDICATED ON THE FIRST   Groupie

INPUT CARD IS POSITIVE. ALL ENERGIES MUST     Groupie

BE IN ASCENDING ENERGY IN EV. THE PRESENT     Groupie

LIMITS ARE 1 TO 1000 GROUPS. FOR N GROUPS     Groupie

N+1 BOUNDARIES WILL BE READ FROM THE          Groupie

INPUT FILE, E.G. IF THE FIRST INPUT CARD      Groupie

INDICATES 20 GROUPS, 21 ENERGY BOUNDARIES     Groupie

WILL BE READ FROM THE INPUT FILE.             Groupie

Groupie

VARY   1-66  6D11.4 ENERGY DEPENDENT WEIGHTING SPECTRUM. ONLY     Groupie

REQUIRED IF THE NUMBER OF POINTS INDICATED    Groupie

ON FIRST CARD IS MORE THAN ONE. DATA IS       Groupie

GIVEN IN (ENERGY, WEIGHT) PAIRS, UP TO 3      Groupie

PAIRS PER CARD, USING ANY NUMBER OF CARDS     Groupie

REQUIRED. ENERGIES MUST BE IN ASCENDING       Groupie

ORDER IN EV. THE SPECTRUM VALUES MUST BE      Groupie

NON-NEGATIVE. THE ENERGY RANGE OF SPECTRUM    Groupie

MUST AT LEAST SPAN THE ENERGY RANGE OF THE    Groupie

ENERGY GROUPS. SINCE SPECTRUM IS STORED IN    Groupie

PAGING SYSTEM THERE IS NO LIMIT TO NUMBER     Groupie

OF POINTS THAT CAN BE USED TO DESCRIBE THE    Groupie

WEIGHTING SPECTRUM.                           Groupie

Groupie

EXAMPLE INPUT NO. 1                                                Groupie

-------------------                                                Groupie

REQUEST DATA BY MAT AND PROCESS ALL DATA (ALL MAT BETWEEN 1 AND    Groupie

9999). USE THE TART 175 GROUP STRUCTURE, GENERATE 2 BAND           Groupie

PARAMETERS (THE FOR ALL ISOTOPES) TO 0.1 PER-CENT ACCURACY         Groupie

IN THE SELF-SHIELDING CURVE. OUTPUT ALL  LISTING, COMPUTER         Groupie

READABLE AND ENDF/B FORMAT GROUP AVERAGES.                         Groupie

Groupie

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Groupie

Groupie

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                          Groupie

Groupie

0          0         -2          0 1.00000-03          0      Groupie

ENDFB.IN                                                               Groupie

ENDFB.OUT                                                              Groupie

1          1          1          1          1                 Groupie

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY                Groupie

1 1  1  9999 0  0                                                  Groupie

(BLANK CARD TERMINATES REQUEST LIST)             Groupie

Groupie

EXAMPLE INPUT NO. 2                                                Groupie

-------------------                                                Groupie

THE SAME EXAMPLE 1, AS ABOVE, ONLY THE ENDF/B DATA WILL BE READ    Groupie

FROM \ENDFB6\SIGMA1\K300\ZA092238 (U-238 AT 300 KELVIN) AND        Groupie

WRITTEN TO \ENDFB6\GROUPIE\K300\ZA092238                           Groupie

Groupie

THE FOLLOWING 7 INPUT LINES ARE REQUIRED.                          Groupie

Groupie

0          0         -2          0 1.00000-03          0      Groupie

\ENDFB6\SIGMA1\K300\ZA092238                                           Groupie

\ENDFB6\GROUPIE\K300\ZA092238                                          Groupie

1          1          1          1          1                 Groupie

TART 175 GROUP, 2 BAND LIBRARY TO 0.1 PER-CENT ACCURACY                Groupie

1 1  1  9999 0  0                                                  Groupie

(BLANK CARD TERMINATES REQUEST LIST)             Groupie

Groupie

EXAMPLE INPUT NO. 3                                                Groupie

-------------------                                                Groupie

PROCESS ALL DATA. USE 1/V WEIGHTING IN ORDER TO CALCULATE          Groupie

UNSHIELDED ONE GROUP CROSS SECTIONS OVER THE ENERGY RANGE 0.5 EV   Groupie

TO 1 MEV (NOTE THAT THE RESULTS ARE SIMPLY PROPORTIONAL TO THE     Groupie

RESONANCE INTEGRAL FOR EACH REACTION). OUTPUT UNSHIELDED LISTING.  Groupie

Groupie

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Groupie

THEN USE STANDARD FILENAMES.                                       Groupie

Groupie

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Groupie

Groupie

0          0          1         -1                     0      Groupie

(USE STANDARD FILENAME = ENDFB.IN)               Groupie

(USE STANDARD FILENAME = ENDFB.OUT)              Groupie

0          0          0          0          1                 Groupie

RESONANCE INTEGRAL CALCULATION (FROM 0.5 EV TO 1 MEV)                  Groupie

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Groupie

5.00000-01 1.00000+06                                                  Groupie

Groupie

======================================================================= Groupie