======================================================================= Recent

Recent

PROGRAM RECENT                                                     Recent

VERSION 79-1 (OCTOBER 1979)  CDC-7600                              Recent

VERSION 80-1 (MAY 1980)      IBM, CDC AND CRAY VERSION             Recent

VERSION 80-2 (DECEMBER 1980) IMPROVED TREATMENT OF UNRESOLVED      Recent

REGION TO COMPUTE ALL REACTIONS AT    Recent

THE SAME TIME.                        Recent

VERSION 81-1 (MARCH 1981)  IMPROVED BASED ON USER COMMENTS.        Recent

VERSION 81-2 (AUGUST 1981) ADDED MONITOR MODE. ADDED SPEED OPTION  Recent

TO BYPASS BACKWARDS THINNING IF FILE 3  Recent

ALLOWABLE ERROR = 0.0 (NOTE THIS OPTION Recent

WILL RESULT IN ALL TABULATED POINTS     Recent

FROM THE EVALUATION BEING KEPT IN THE   Recent

OUTPUT FROM THIS PROGRAM).              Recent

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY.       Recent

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Recent

*PAGE SIZES INCREASED.                  Recent

*ELIMINATED COMPUTER DEPENDENT CODING.  Recent

*NEW, MORE COMPATIBLE I/O UNIT NUMBERS. Recent

*ADDED OPTION TO KEEP ALL RECONSTRUCTED Recent

AND BACKGROUND ENERGY POINTS.          Recent

*ADDED STANDARD ALLOWABLE ERROR OPTIONS Recent

(CURRENTLY 0.1 PER-CENT RECONSTRUCTION Recent

AND 0.0 PER-CENT THINNING).            Recent

VERSION 83-2 (OCTOBER 1983) IMPROVED BASED ON USER COMMENTS.       Recent

VERSION 84-1 (JANUARY 1984) IMPROVED INTERVAL HALFING CONVERGENCE. Recent

VERSION 85-1 (APRIL 1985)  *A BRAND NEW PROGRAM WHICH COMPLETELY   Recent

SUPERCEDES ALL PREVIOUS VERSIONS OF    Recent

THIS PROGRAM.                          Recent

*UPDATED FOR ENDF/B-VI FORMATS.         Recent

*ADDED GENERAL REICH-MOORE FORMALISM    Recent

(WITH TWO FISSION CHANNELS).           Recent

*DECREASED RUNNING TIME.                Recent

*SPECIAL I/O ROUTINES TO GUARANTEE      Recent

ACCURACY OF ENERGY.                    Recent

*DOUBLE PRECISION TREATMENT OF ENERGY   Recent

(REQUIRED FOR NARROW RESONANCES).      Recent

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Recent

VERSION 86-1 (JANUARY 1986)*ENERGY DEPENDENT SCATTERING RADIUS     Recent

VERSION 86-2 (JUNE 1986)   *IF FIRST CHANCE FISSION (MT=19)        Recent

BACKGROUND IS PRESENT ADD RESONANCE    Recent

CONTRIBUTION OF FISSION TO IT.         Recent

VERSION 86-3 (OCTOBER 1986)*MULTI-LEVEL OR REICH-MOORE..CORRECT    Recent

POTENTIAL SCATTERING CROSS SECTION FOR Recent

MISSING AND/OR FICTICIOUS (L,J)        Recent

SEQUENCES.                             Recent

VERSION 87-1 (JANUARY 1987)*IMPROVED COMBINING FILE 2+3            Recent

VERSION 87-2 (MARCH 1987)  *CORRECTED ADLER-ADLER CALCULATIONS.    Recent

VERSION 88-1 (JULY 1988)   *UPDATED REICH-MOORE ENDF/B-VI FORMAT   Recent

TO BE THE SAME AS REICH-MOORE FORMAT   Recent

IN EARLIER VERSIONS OF ENDF/B FORMAT.  Recent

*CHECK FOR PRELIMINARY ENDF/B-VI        Recent

REICH-MOORE FORMAT (NOW ABANDONED)     Recent

AND TERMINATE EXECUTION IF DATA IS     Recent

IN THIS FORMAT.                        Recent

*CALCULATE CHANNEL RADIUS OR SET IT     Recent

EQUAL TO THE SCATTERING RADIUS.        Recent

*IMPLEMENTED HYBRID R-FUNCTION WITH THE Recent

FOLLOWING RESTRICTIONS                 Recent

- ONLY INELASTIC COMPETITION (NO       Recent

CHARGED PARTICLES)                   Recent

- NO TABULATED FILE 2 BACKGROUND       Recent

- NO TABULATED OPTICAL MODEL PHASE     Recent

SHIFT                                Recent

*PROGRAM EXIT IF GENERAL R-MATRIX IN    Recent

THE EVALUATION (THIS FORMALISM WILL    Recent

BE IMPLEMENTED ONLY AFTER THE AUTHOR   Recent

RECEIVES REAL EVALUATIONS WHICH USE    Recent

THIS FORMALISM...UNTIL THEN IT IS      Recent

IMPOSSIBLE TO ADEQUATELY TEST THAT     Recent

THE CODING FOR THIS FORMALISM IS       Recent

CORRECT).                              Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 1002 TO 4008.                     Recent

*DOUBLE PRECISION RESONANCE REGION      Recent

LIMITS.                                Recent

*FILE 2 AND FILE 3 ENERGIES WHICH ARE   Recent

NEARLY EQUAL ARE TREATED AS EQUAL      Recent

(I.E., SAME TO ABOUT 9 DIGITS).        Recent

*CHECK FILE 3 BACKGROUND CROSS SECTIONS Recent

IN EDIT MODE.                          Recent

*OPTION...INTERNALLY DEFINE FILENAMES   Recent

(SEE SUBROUTINE FILEIO FOR DETAILS).   Recent

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Recent

INSURE PROGRAM WILL NOT DO ANYTHING    Recent

CRAZY.                                 Recent

*UPDATED TO USE NEW PROGRAM CONVERT     Recent

KEYWORDS.                              Recent

*CORRECTED MULTILEVEL, REICH-MOORE AND  Recent

HYBRID R-FUNCTION POTENTIAL SCATTER    Recent

TO ACCOUNT FOR REPEATED J-VALUES FOR   Recent

THE SAME TARGET SPIN AND L-VALUE.      Recent

*ADDED LIVERMORE CIVIC COMPILER         Recent

CONVENTIONS.                           Recent

*UPDATED TO USE NEW ENDF/B-VI           Recent

CONVENTION TO ALLOW UNRESOLVED         Recent

RESONANCE CONTRIBUTION TO ALREADY      Recent

BE INCLUDED IN THE FILE 3 CROSS        Recent

SECTIONS (INFINITELY DIULUTE           Recent

CONTRIBUTION).                         Recent

VERSION 90-1 (JUNE 1990)   *UPDATED BASED ON USER COMMENTS         Recent

*ADDED FORTRAN SAVE OPTION              Recent

*NEW MORE CONSISTENT ENERGY OUTPUT      Recent

ROUTINE                                Recent

VERSION 91-1 (JULY 1991)   *NEW UNIFORM TREATMENT OF ALL RESONANCE Recent

FORMALISMS (SEE, COMMENTS BELOW)       Recent

*NEW REICH-MOORE ALGORITHM              Recent

*MORE EXTENSIVE ERROR CHECKING AND      Recent

ERROR MESSAGE EXPLANATIONS             Recent

VERSION 92-1 (JANUARY 1992)*MAJOR RESTRUCTING TO IMPROVE ACCURACY  Recent

AND COMPUTER INDEPENDENCE.             Recent

*INCREASED ENERGY POINT PAGE SIZE FROM  Recent

1002 TO 4008.                          Recent

*NO MORE THAN 2 ENERGY POINTS WHERE     Recent

CROSS SECTION IS ZERO AT BEGINNING     Recent

OF A SECTION FOR EACH REACTION,E.G.,   Recent

THRESHOLD FISSION.                     Recent

*PROCESS ONLY A PORTION OF RESONANCE    Recent

REGION - SEE EXPLANATION BELOW         Recent

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Recent

TO CALCULATIONS.                       Recent

*COMPLETELY CONSISTENT I/O AND ROUNDING Recent

ROUTINES - TO MINIMIZE COMPUTER        Recent

DEPENDENCE.                            Recent

VERSION 93-1 (MARCH 1993)  *UPDATED REICH-MOORE TREATMENT TO USE   Recent

L DEPENDENT SCATTERING RADIUS (APL)    Recent

RATHER THAN SCATTERING RADIUS (AP)     Recent

(SEE, ENDF/B-VI FORMATS AND            Recent

PROCEDURES MANUAL, PAGE 2.6)          Recent

*INCREASED PAGE SIZE FROM 4008 TO       Recent

20040 DATA POINTS.                     Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 4008 TO 20040.                    Recent

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Recent

TO ALLOW ACCESS TO FILE STRUCTURES     Recent

(WARNING - INPUT PARAMETER FORMAT      Recent

HAS BEEN CHANGED).                     Recent

*CLOSE ALL FILES BEFORE TERMINATING     Recent

(SEE, SUBROUTINE ENDIT)                Recent

VERSION 94-2 (AUGUST 1994) *CORRECTED ADDJ FOR ENERGY DEPENDENT    Recent

(TABULATED) SCATTERING RADIUS CASE.    Recent

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Recent

*IMPROVED COMPUTER INDEPENDENCE        Recent

*ALL DOUBLE PRECISION                  Recent

*ON SCREEN OUTPUT                      Recent

*UNIFORM TREATMENT OF ENDF/B I/O       Recent

*IMPROVED OUTPUT PRECISION             Recent

*ALWAYS INCLUDE THERMAL VALUE          Recent

*DEFINED SCRATCH FILE NAMES            Recent

VERSION 97-1 (APRIL 1997)   *OPTIONAL MAKE NEGATIVE CROSS          Recent

SECTION = 0 FOR OUTPUT                Recent

*INCREASED PAGE SIZE FROM 20040 TO      Recent

120000 DATA POINTS.                    Recent

*INCREASED MAXIMUM NUMBER OF RESONANCES Recent

FROM 20040 TO 120000.                  Recent

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Recent

POINT READ FOR MORE DIGITS            Recent

*UPDATED TEST FOR ENDF/B FORMAT        Recent

VERSION BASED ON RECENT FORMAT CHANGE Recent

*UPDATED CONSTANTS BASED ON CSEWG      Recent

SUBCOMMITTEE RECOMMENDATIONS          Recent

*GENERAL IMPROVEMENTS BASED ON         Recent

USER FEEDBACK                         Recent

VERSION 99-2 (JUNE 1999)    *IMPLEMENTED NEW REICH-MOORE FORMALISM Recent

TO ALLOW DEFINITION OF (L,J,S) FOR    Recent

EACH SEQUENCE.                        Recent

*ASSUME ENDF/B-VI, NOT V, IF MISSING   Recent

MF=1, MT-451.                         Recent

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Recent

USER FEEDBACK                         Recent

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Recent

(SEPT. 2002)   *OUTPUT RESONANCE WITH 9 DIGITS        Recent

*TO BE C AND C++ COMPATIBLE OUTPUT     Recent

VERS. 2004-1 (JAN. 2004)    *ADDED INCLUDE 'recent.h'              Recent

*MADE ENDF/B-VII READY                 Recent

*UPDATED FOR NEW REICH-MOORE LRF=7     Recent

PARAMETERS WITH COMPETITION           Recent

*ADDED COULOMB PENETRATION FACTORS FOR Recent

LRF=7 COMPETITIVE CHANNELS.           Recent

*EXTENDED DEFINITIONS OF PENETRATION   Recent

FACTOR, LEVEL SHIFT FACTOR, AND       Recent

POTENTIAL SCATTERING PHASE SHIFT      Recent

ABOVE L = 5 TO INFINITY.              Recent

*ADDED QUICK CALCULATION - IF THE      Recent

INPUT ALLOWABLE ERROR IS 1.0 OR MORE  Recent

(100 % OR MORE) THERE IS NO ITERATION Recent

TO CONVERGENCE - CROSS SECTION ARE    Recent

QUICKLY CALCULATED ONLY AT A FIXED    Recent

SET OF ENERGY POINTS, BASED ON THE    Recent

ENERGY AND WIDTH OF ALL RESONANCES.   Recent

THIS CAN BE USED TO QUICKLY "SEE"     Recent

NEW EVALUATIONS THAT MAY CONTAIN      Recent

ERRORS, THAT WOULD OTHERWISE CAUSE    Recent

THIS CODE TO RUN FOR AN EXCESSIVELY   Recent

LONG TIME.                            Recent

VERS. 2005-1 (JUNE 2005)    *ADDED ENERGY DEPENDENT SCATTERING     Recent

RADIUS FOR ALL RESONANCE TYPES        Recent

(EARLIER ONLY BREIT-WIGNER ALLOWED).  Recent

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Recent

*DECOUPLED PAGE SIZE FROM MAX. # OF    Recent

RESONANCES.                           Recent

*INCREASED PAGE SIZE FROM 120,000 TO   Recent

750,000 DATA POINTS.                  Recent

*KEPT MAX. # OF RESONANCE AT 120,000.  Recent

*CORRECTED ALL BACKGROUND = 0 CASE     Recent

Recent

OWNED, MAINTAINED AND DISTRIBUTED BY                               Recent

------------------------------------                               Recent

THE NUCLEAR DATA SECTION                                           Recent

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Recent

P.O. BOX 100                                                       Recent

A-1400, VIENNA, AUSTRIA                                            Recent

EUROPE                                                             Recent

Recent

ORIGINALLY WRITTEN BY                                              Recent

------------------------------------                               Recent

DERMOTT E. CULLEN                                                  Recent

UNIVERSITY OF CALIFORNIA                                           Recent

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Recent

L-159                                                              Recent

P.O. BOX 808                                                       Recent

LIVERMORE, CA 94550                                                Recent

U.S.A.                                                             Recent

TELEPHONE  925-423-7359                                            Recent

E. MAIL    CULLEN1@LLNL.GOV                                        Recent

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Recent

Recent

Acknowledgement (Version 2004-1)                                   Recent

================================================================== Recent

The author thanks Nancy Larson, ORNL, for providing her SAMRML     Recent

code for comparison to RECENT output for Reich-Moore evaluations,  Recent

in particular to verify results for the new LFR=7 evaluations. I   Recent

also thank her for providing guidance to help me understand and    Recent

implement this new teatment for Reich-Moore parameters.            Recent

Recent

ACKNOWLEDGEMENT (VERSION 92-1)                                     Recent

================================================================== Recent

THE AUTHOR THANKS SOL PEARLSTEIN (BROOKHAVEN NATIONAL LAB) FOR     Recent

SIGNIFICANTLY CONTRIBUTING TOWARD IMPROVING THE ACCURACY AND       Recent

COMPUTER INDEPENDENCE OF THIS CODE - THANKS, SOL                   Recent

================================================================== Recent

Recent

AUTHORS MESSAGE                                                    Recent

================================================================== Recent

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Recent

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Recent

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Recent

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Recent

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Recent

Recent

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Recent

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Recent

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Recent

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Recent

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Recent

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Recent

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Recent

COMPUTER.                                                          Recent

Recent

PURPOSE                                                            Recent

================================================================== Recent

THIS PROGRAM IS DESIGNED TO RECONSTRUCT THE RESONANCE CONTRIBUTION Recent

TO THE CROSS SECTION IN LINEARLY INTERPOLABLE FORM, ADD IN ANY     Recent

LINEARLY INTERPOLABLE BACKGROUND CROSS SECTION AND OUTPUT THE      Recent

RESULT IN THE ENDF/B FORMAT. THE CROSS SECTIONS OUTPUT BY THIS     Recent

PROGRAM WILL BE LINEARLY INTERPOLABLE OVER THE ENTIRE ENERGY RANGE Recent

Recent

THE RESONANCE CONTRIBUTION IS CALCULATED FOR TOTAL (MT=1),         Recent

ELASTIC (MT=2), CAPTURE (MT=102) AND FISSION (MT=18), ADDED        Recent

TO THE BACKGROUND (IF ANY) AND OUTPUT. IN ADDITION, IF THERE       Recent

IS A FIRST CHANCE FISSION (MT=19) BACKGROUND PRESENT THE RESONANCE Recent

CONTRIBUTION OF FISSION WILL BE ADDED TO THE BACKGROUND AND        Recent

OUTPUT. IF THERE IS NO FIRST CHANCE FISSION (MT=19) BACKGROUND     Recent

PRESENT THE PROGRAM WILL NOT OUTPUT MT=19.                         Recent

Recent

IN THE FOLLOWING FOR SIMPLICITY THE ENDF/B TERMINOLOGY--ENDF/B     Recent

TAPE--WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE TAPE, CARDS,  Recent

DISK OR ANY OTHER MEDIUM.                                          Recent

Recent

PROCESSING DATA IN THE ENDF/B-VI FORMAT                            Recent

================================================================== Recent

IT HAS NOW BEEN CONFIRMED (PRIVATE COMMUNICATION, CHARLES DUNFORD, Recent

APRIL, 1991) THAT THE PROPER PROCEDURE TO FOLLOW WHEN THERE ARE    Recent

MISSING OR DUPLICATE J VALUES IS TO IN ALL CASES ADD A SEQUENCE    Recent

WITH NO RESONANCES TO ACCOUNT FOR THE CONTRIBUTION OF THE SEQUENCE Recent

TO THE POTENTIAL SCATTERING CROSS SECTION.                         Recent

Recent

THIS IS THE PROCEDURE WHICH WAS FOLLOWED BY ALL VERSIONS OF RECENT Recent

SINCE 86-3 AND WILL CONTINUE TO BE THE PROCEDURE.                  Recent

Recent

INPUT ENDF/B FORMAT AND CONVENTIONS                                Recent

================================================================== Recent

ENDF/B FORMAT                                                      Recent

-------------                                                      Recent

THIS PROGRAM ONLY USES THE ENDF/B BCD OR LINE IMAGE FORMAT (AS     Recent

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Recent

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II,III, IV, V OR VI FORMAT). Recent

Recent

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Recent

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Recent

ASSUMED THAT THE MAT, MF AND MT ON EACH LINE IS CORRECT. SEQUENCE  Recent

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Recent

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Recent

AND ALL SECTIONS OF MF=2 AND 3 MUST BE CORRECT. THE PROGRAM COPIES Recent

ALL OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE  Recent

TO THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.         Recent

Recent

ENDF/B FORMAT VERSION                                              Recent

---------------------                                              Recent

THE FORMATS AND CONVENTIONS FOR READING AND INTERPRETING THE DATA  Recent

VARIES FROM ONE VERSION OF ENDF/B TO THE NEXT. HOWEVER, IF THE     Recent

HOLLERITH SECTION (MF=1, MT=451) IS PRESENT IT IS POSSIBLE FOR     Recent

THIS PROGRAM TO DISTINGUISH BETWEEN DATA IN THE ENDF/B-IV, V AND   Recent

VI FORMATS AND TO USE THE APPROPRIATE CONVENTIONS FOR EACH         Recent

ENDF/B VERSION (SEE, SUBROUTINE FILE1 FOR A DESCRIPTION OF HOW     Recent

THIS IS DONE). IF THE HOLLERITH SECTION IS NOT PRESENT THE         Recent

PROGRAM WILL ASSUME THE DATA IS IN THE ENDF/B-VI FORMAT AND USE    Recent

ALL CONVENTIONS APPROPRIATE TO ENDF/B-V. USERS ARE ENCOURAGED TO   Recent

INSURE THAT THE HOLLERITH SECTION (MF=1, MT=451) IS PRESENT IN     Recent

ALL EVALUATIONS.                                                   Recent

Recent

INPUT OF ENERGIES                                                  Recent

-----------------                                                  Recent

ALL ENERGIES ARE READ IN DOUBLE PRECISION (BY SPECIAL FORTRAN I/O  Recent

ROUTINES) AND ARE TREATED IN DOUBLE PRECISION IN ALL CALCULATIONS. Recent

Recent

OUTPUT ENDF/B FORMAT AND CONVENTIONS                               Recent

================================================================== Recent

CONTENTS OF OUTPUT                                                 Recent

------------------                                                 Recent

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE RECONSTRUCTED FILE     Recent

3 CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE        Recent

ALSO INCLUDED.                                                     Recent

Recent

DOCUMENTATION                                                      Recent

-------------                                                      Recent

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Recent

BY THE ADDITION OF COMMENT CARDS AT THE END OF EACH HOLLERITH      Recent

SECTION IN THE FORM                                                Recent

Recent

***************** RECENT (VERSION 2007-1) ***************          Recent

RESONANCE CONTRIBUTION RECONSTRUCTED TO WITHIN   0.100 PER-CENT    Recent

COMBINED DATA NOT THINNED (ALL RESONANCE + BACKGROUND DATA KEPT)   Recent

Recent

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR, SIGMA1 AND GROUPY) Recent

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Recent

THE DATA, INCLUDING WHICH VERSION OF EACH PROGRAM WAS USED.        Recent

Recent

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Recent

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Recent

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Recent

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Recent

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Recent

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Recent

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Recent

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Recent

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Recent

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Recent

Recent

REACTION INDEX                                                     Recent

--------------                                                     Recent

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Recent

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Recent

Recent

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Recent

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Recent

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Recent

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Recent

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Recent

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Recent

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Recent

Recent

OUTPUT FORMAT OF ENERGIES                                          Recent

-------------------------                                          Recent

IN THIS VERSION OF RECENT ALL FILE 3 ENERGIES WILL BE OUTPUT IN    Recent

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Recent

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Recent

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Recent

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Recent

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Recent

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Recent

JUST DUE TO TRANSLATION OF ENERGIES FROM THEIR INTERNAL (BINARY)   Recent

REPRESENTATION TO THE ENDF/B FORMAT.                               Recent

Recent

ACCURACY OF ENERGY                                                 Recent

------------------                                                 Recent

IN ORDER TO ALLOW ENERGIES TO BE ACCURATELY OUTPUT TO 9 DIGITS     Recent

ON SHORT WORD LENGTH COMPUTERS (E.G. IBM) ALL ENERGIES AND         Recent

ENERGY DEPENDENT TERMS ARE READ AND TREATED IN DOUBLE PRECISION.   Recent

Recent

OUTPUT OF RESONANCE PARAMETERS                                     Recent

------------------------------                                     Recent

A SPECIAL CONVENTION HAS BEEN INTRODUCED REGARDING RESONANCE       Recent

PARAMETERS. IN ORDER TO ALLOW THE USER TO DOPPLER BROADEN AND/OR   Recent

SELF-SHIELD CROSS SECTIONS THE RESONANCE PARAMETERS ARE ALSO       Recent

INCLUDED IN THE OUTPUT WITH THE EVALUATION. IN ORDER TO AVOID THE  Recent

POSSIBILITY OF ADDING THE RESONANCE CONTRIBUTION A SECOND TIME     Recent

TWO CONVENTIONS HAVE BEEN ADOPTED TO INDICATE THAT THE RESONANCE   Recent

CONTRIBUTION HAS ALREADY BEEN ADDED TO THE FILE 3 CROSS SECTIONS,  Recent

Recent

(1) WHEN THE DATA IS PROCESSED BY THIS PROGRAM LRP (IN MF=1,       Recent

MT=451) IS SET EQUAL TO 2. THIS IS A CONVENTION WHICH HAS BEEN     Recent

ADOPTED AS A STANDARD CONVENTION IN ENDF/B-VI, BUT IS ONLY TO BE   Recent

USED FOR PROCESSED DATA, AS OPPOSED TO THE ORIGINAL EVALUATIONS.   Recent

IN EVALUATIONS WHICH CONTAIN MF=1, MT=451 LRP CAN BE USED TO       Recent

DETERMINE IF THE MATERIAL HAS BEEN PROCESSED.                      Recent

Recent

(2) THE LRU FLAG IN EACH SECTION OF FILE 2 DATA IS CHANGED TO      Recent

LRU=LRU+3. FOR EXAMPLE WHEN READING AN ENDF/B EVALUATION LRU=0     Recent

(NO RESONANCES), =1 (RESOLVED) OR =2 (UNRESOLVED) INDICATES THAT   Recent

THE DATA IS IN THE ORIGINAL ENDF/B FORM. LRU=3 (NO RESONANCES),    Recent

=4 (RESOLVED) OR =5 (UNRESOLVED) INDICATES THAT THE RESONANCE      Recent

CONTRIBUTION HAS ALREADY BEEN ADDED TO THE FILE 3 DATA. THIS       Recent

SECOND CONVENTION HAS BEEN ADOPTED AS INSURANCE THAT THE RESONANCE Recent

CONTRIBUTION WILL NOT BE ADDED TWICE, EVEN FOR EVALUATIONS WHICH   Recent

DO NOT CONTAIN MF=1, MT=451 (EVALUATIONS WHICH CONTAIN MF=1,       Recent

MT=451 ARE COVERED BY CONVENTION (1), DESCRIBED ABOVE).            Recent

Recent

UNIFORM TREATMENT OF RESONANCE FORMALISMS                          Recent

================================================================== Recent

NORMALIZATION                                                      Recent

=============                                                      Recent

ALL OF THE RESONANCE FORMALISMS INCLUDE A FACTOR OF,               Recent

Recent

PI*(FRACTIONAL ABUNDANCE)/(K**2)                                   Recent

Recent

THIS FACTOR HAS BEEN REMOVED FROM THE CALCULATION OF EACH TYPE     Recent

OF RESONANCE FORMALISM AND IS APPLIED AS A FINAL NORMALIZATION     Recent

AFTER THE CALCULATION, ONLY ONE PLACE IN THIS PROGRAM.             Recent

Recent

FOR SIMPLICITY THIS TERM IS NOT INCLUDED IN THE FOLLOWING          Recent

DERIVATIONS - IN ALL CASES THE ACTUAL CROSS SECTION IS A PRODUCT   Recent

OF THE ABOVE FACTOR TIMES THE RESULTS PRESENTED BELOW.             Recent

Recent

SIMILARITIES                                                       Recent

============                                                       Recent

FOR THE RESOLVED RESONANCE REGION, EXCEPT FOR SINGLE LEVEL BREIT   Recent

WIGNER, PARAMETERS ALL OF THE FORMALISMS DEFINE THE CROSS SECTIONS Recent

IN AN EQUIVALENT FORM,                                             Recent

Recent

TOTAL    = 2*GJ*REAL(1 - U)                                        Recent

= 2*GJ*(1 - REAL(U))                                      Recent

ELASTIC  =   GJ*(1 - U)**2                                         Recent

=   GJ*((1 - 2*REAL(U)) + (REAL(U)**2 + IM(U)**2))        Recent

= 2*GJ*(1 - REAL(U)) - GJ*(1 - (REAL(U)**2 + IM(U)**2))   Recent

Recent

SINCE THE FIRST TERM IS THE TOTAL, THE SECOND TERM MUST BE         Recent

ABSORPTION. SO WE FIND,                                            Recent

Recent

ABSORPTION = GJ*(1 - (REAL(U)**2 + IM(U)**2))                      Recent

Recent

IN ALL CASES U IS DEFINED IN THE FORM,                             Recent

Recent

U        = EXP(-I*2*PS)*((1-X) - I*Y)                              Recent

Recent

WHERE (X) AND (Y) ARE RELATED TO THE SYMMETRIC AND ANTI-SYMMETRIC  Recent

CONTRIBUTIONS OF THE RESONANCES, RESPECTIVELY. ONLY THE DEFINITION Recent

OF (X) AND (Y) WILL BE DIFFERENT FOR EACH RESONANCE FORMALISM.     Recent

BELOW WE WILL SHOW THAT WHAT MIGHT APPEAR TO BE A STRANGE CHOICE   Recent

OF DEFINITION OF THE SIGN OF (X) AND(Y) HAS BEEN SELECTED SO THAT  Recent

FOR BREIT-WIGNER PARAMETERS (X) AND (Y) CORRESPOND EXACTLY TO THE  Recent

SYMMETRIC AND ANTI-SYMMETRIC CONTRIBUTION OF THE RESONANCES.       Recent

Recent

U        = (COS(2*PS) - I*SIN(2*PS))*((1-X) - I*Y)                 Recent

=   ((1-X)*COS(2*PS) - Y*SIN(2*PS))                       Recent

=-I*((1-X)*SIN(2*PS) + Y*COS(2*PS))                       Recent

Recent

REAL(U)  = ((1-X)*COS(2*PS) - Y*SIN(2*PS))                         Recent

IM(U)    =-((1-X)*SIN(2*PS) + Y*COS(2*PS))                         Recent

Recent

R(U)**2  =((1-X)*COS(2*PS))**2 + (Y*SIN(2*PS))**2                  Recent

-2*(1-X)*Y*COS(2*PS)*SIN(2*PS)                           Recent

I(U)**2  =((1-X)*SIN(2*PS))**2 + (Y*COS(2*PS))**2                  Recent

+2*(1-X)*Y*COS(2*PS)*SIN(2*PS)                           Recent

Recent

THE TERMS 2*(1-X)*Y*COS(2*PS)*SIN(2*PS) CANCEL AND UPON USING      Recent

THE IDENTITY COS(2*PS)**2 + SIN(2*PS)**2 = 1,                      Recent

Recent

SUM      = (1-X)**2 + (Y)**2                                       Recent

Recent

WE NOW HAVE ALL THE QUANTITIES THAT WE NEED TO DEFINE THE CROSS    Recent

SECTIONS,                                                          Recent

Recent

ELASTIC                                                            Recent

=======                                                            Recent

ELASTIC  =GJ*(1 - 2*REAL(U) + (REAL(U)**2 + IM(U)**2))             Recent

=GJ*(1 - 2*((1-X)*COS(2*PS)-Y*SIN(2*PS))+(1-X)**2+(Y)**2) Recent

Recent

THIS CAN BE WRITTEN AS A SUM OF 2 SQUARES,                         Recent

Recent

ELASTIC  =GJ*(COS(2*PS) - (1-X))**2 + (SIN(2*PS) + Y)**2)          Recent

Recent

=GJ*((COS(2*PS))**2 - 2*(1-X)*COS(2*PS) + (1-X)**2) +     Recent

(SIN(2*PS))**2 + 2*Y*SIN(2*PS)    + (Y)**2)          Recent

Recent

AGAIN USING THE IDENTITY COS(2*PS)**2 + SIN(2*PS)**2 = 1, WE CAN   Recent

SEE THAT THE DEFINITION AS THE SUM OF 2 SQUARES IS IDENTICAL TO    Recent

THE PRECEDING DEFINITION OF THE ELASTIC.                           Recent

Recent

ELASTIC  =GJ*(COS(2*PS) - (1-X))**2 + (SIN(2*PS) + Y)**2)          Recent

=GJ*((COS(2*PS)-1) + X)**2 + (SIN(2*PS) + Y)**2)          Recent

Recent

USING THE IDENTITY (1 - COS(2*PS))) = 2*SIN(PS)**2, WE OBTAIN      Recent

THE FINAL FORM FOR THE ELASTIC,                                    Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 - X)**2 + (SIN(2*PS) + Y)**2)            Recent

Recent

ABSORPTION                                                         Recent

==========                                                         Recent

ABSORPTION = GJ*(1 - (REAL(U)**2 + IM(U)**2))                      Recent

= GJ*(1 - ((1-X)**2   + (Y)**2)                         Recent

= GJ*(1 - (1 - 2*X + (X)**2 + (Y)**2)                   Recent

= GJ*(2*X - (X)**2 + (Y)**2)                            Recent

Recent

SINCE PHYSICALLY THE ABSORPTION CANNOT BE NEGATIVE WE CAN SEE      Recent

THAT (X) MUST BE POSITIVE AND 2*X MUST BE GREATER THAN             Recent

(X)**2 + (Y)**2, FOR ALL OF THE FORMALISMS.                        Recent

Recent

TOTAL                                                              Recent

=====                                                              Recent

IN THIS PROGRAM THE TOTAL CROSS SECTION IS ALWAYS DEFINED TO BE    Recent

THE SUM OF ITS PARTS - SO THE ABOVE DEFINITION IS NEVER EXPLICITLY Recent

USED. HOWEVER, WE CAN LEARN SOMETHING BY EXAMINING THE DEFINITION, Recent

Recent

TOTAL    = 2*GJ*REAL(1 - U)                                        Recent

= 2*GJ*(1 - (((1-X)*COS(2*PS) - Y*SIN(2*PS)))             Recent

= 2*GJ*((1 - COS(2*PS))*(1-X) - (1-X) + Y*SIN(2*PS))      Recent

= 2*GJ*(2*SIN(PS)**2*(1-X)    - (1-X) + Y*SIN(2*PS))      Recent

Recent

= 4*GJ*SIN(PS)**2 +                                       Recent

2*GJ*((X-1) - 2*X*SIN(PS)**2 +  Y*SIN(2*PS))            Recent

Recent

THE IMPORTANT POINT TO NOTE IS THAT THE DEFINITION OF THE TOTAL    Recent

DOES NOT EXPLICITLY CONTAIN ANY DEPENDENCE ON X**2 AND Y**2 -      Recent

THE LEVEL-LEVEL INTERFERENCE TERMS.                                Recent

Recent

THIS IMPLIES THAT IF A GIVEN SET OF RESONANCE PARAMETERS ARE USED  Recent

WITH THIS DEFINITION THEY WILL PRODUCE EXACTLY THE SAME TOTAL      Recent

CROSS SECTION - WHETHER WE CLAIM THE PARAMETERS HAVE BEEN          Recent

PRODUCED USING A SINGLE OR MULTI-LEVEL FIT. THIS RESULT COULD      Recent

BE VERY MISLEADING, IF THIS RESULT FOR THE TOTAL IS IMPLIED TO     Recent

MEAN THAT ONE INTERPRETATION OR THE OTHER WILL NOT HAVE ANY        Recent

EFFECT ON THE INDIVIDUAL CROSS SECTIONS.                           Recent

Recent

STARTING FROM EXACTLY THE SAME RESONANCE PARAMETERS, RELATIVE TO   Recent

THE RESULTS OBTAINED USING THE SINGLE LEVEL FORMULA, MULTI-LEVEL   Recent

RESULTS WILL TEND TO ALWAYS DECREASE THE ABSORPTION AND INCREASE   Recent

THE ELASTIC. THIS CAN BE IMMEDIATELY SEEN FROM OUR GENERAL         Recent

MULTI-LEVEL DEFINITION OF ABSORPTION,                              Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

Recent

THE SINGLE LEVEL ABSORPTION IS,                                    Recent

Recent

ABSORPTION =GJ*(2*X)                                               Recent

Recent

THE DIFFERENCE BETWEEN THE TWO IS -2*GJ*(X**2 + Y**2), SO THAT     Recent

REGARDLESS OF HOW WE DEFINE (X) AND (Y) THE INCLUSION OF THIS      Recent

TERM WILL ALWAYS DECREASE ABSORPTION. SINCE THE TOTAL CROSS        Recent

SECTION IS THE SAME IN BOTH CASE, THIS MEANS THAT THE ELASTIC      Recent

HAS BEEN INCREASED BY THIS AMOUNT.                                 Recent

Recent

AGAIN, THESE RESULTS ARE BASED ON STARTING FROM EXACTLY THE SAME   Recent

PARAMETERS - IN ANY ACTUAL CASE THE PARAMETERS BASED ON A SINGLE   Recent

OR MULTI-LEVEL FIT WILL BE QUITE DIFFERENT - THE POINT THAT WE     Recent

WANT TO STRESS HERE IS THAT YOU SHOULD NEVER USE PARAMETERS        Recent

WHICH HAVE BEEN DEFINED BY A FIT USING ONE FORMALISM - IN THE      Recent

EQUATIONS FOR A DIFFERENT FORMALISM - AND ASSUME THAT THE RESULTS  Recent

WILL BE CONSISTENT - AND NEVER USE THE TOTAL CROSS SECTION TO      Recent

SEE WHETHER OR NOT A SET OF SINGLE LEVEL PARAMETERS CAN BE USED    Recent

WITH A MULTI-LEVEL FORMALISM.                                      Recent

Recent

POTENTIAL CROSS SECTION                                            Recent

=======================                                            Recent

FAR FROM RESONANCES (X) AND (Y) WILL BE SMALL AND THE ELASTIC      Recent

CROSS SECTION REDUCES TO,                                          Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2)**2     + (SIN(2*PS))**2                 Recent

=GJ*4*(SIN(PS)**4         + SIN(2*PS)**2                   Recent

Recent

USING THE IDENTITY SIN(2*PS) = 2*SIN(PS)*COS(PS)                   Recent

Recent

=4*GJ*(SIN(PS)**4         + (SIN(PS)*COS(PS))**2)          Recent

=4*GJ*SIN(PS)**2*(SIN(PS)**2 + COS(PS)**2)                 Recent

=4*GJ*SIN(PS)**2                                           Recent

Recent

WHICH IS THE POTENTIAL CROSS SECTION. NOTE THAT THIS RESULT IS     Recent

INDEPENDENT OF THE FORMALISM USED, AS IT MUST PHYSICALLY BE,       Recent

AND AS SUCH ALTHOUGH AS YET WE HAVE NOT DEFINED IT, WE CAN         Recent

NOW SEE THAT IN ALL CASES (PS) MUST BE THE PHASE SHIFT AND FOR     Recent

CONSISTENCY IT MUST BE DEFINED USING EXACTLY THE SAME DEFINITION   Recent

IN ALL CASES.                                                      Recent

Recent

IN ADDITION SINCE PHYSICALLY FOR EACH L VALUE WE EXPECT TO OBTAIN  Recent

A POTENTIAL CROSS SECTION,                                         Recent

Recent

4*(2*L+1)*SIN(PS)**2                                               Recent

Recent

OBVIOUSLY FOR CONSISTENCY WE MUST HAVE,                            Recent

Recent

(2*L+1) = (SUM OVER J) GJ                                          Recent

Recent

ONLY IN THIS CASE WILL THE RESULTS BE CONSISTENT - THIS POINT WILL Recent

BE DISCUSSED IN DETAIL BELOW.                                      Recent

Recent

WHAT ARE THIS TERMS (X) AND (Y)                                    Recent

===============================                                    Recent

(X) AND (Y) CAN BE EASILY IDENTIFIED BY CONSIDERING THE SINGLE     Recent

AND MULTI-LEVEL BREIT WIGNER FORMALISMS. IN THESE CASES WE WILL    Recent

FIND THAT,                                                         Recent

Recent

X        = GAM(N)*GAM(T)/2/DEN                                     Recent

Y        = GAM(N)*(E-ER)/DEN                                       Recent

DEN      = ((E-ER)**2 + (GAM(T)/2)**2)                             Recent

Recent

EXTREME CARE HAS TO BE USED TO PROPERLY DEFINE (Y) SUCH THAT IT    Recent

IS NEGATIVE FOR E LESS THAN ER AND POSITIVE FOR E GREATER THAN     Recent

ER. I WILL MERELY MENTION THAT THE EQUATIONS FOR ALL FORMALISMS    Recent

IN ENDF-102 DO NOT CONSISTENTLY USE (E - ER) - IN SOME CASES       Recent

THIS IS WRITTEN AS (ER - E), WHICH CAN LEAD TO AN INCORRECT        Recent

SIGN IN THE DEFINITION OF THE (Y) THAT WE REQUIRE.                 Recent

Recent

THE INTERFERENCE TERMS CAN BE WRITTEN IN TERMS OF,                 Recent

1) LEVEL-SELF INTERFERENCE  = THE CONTRIBUTION OF EACH LEVEL       Recent

INTERFERRING WITH ITSELF             Recent

2) LEVEL-LEVEL INTERFERENCE = THE CONTRIBUTION OF EACH LEVEL       Recent

INTERFERRRING WITH ALL OTHER LEVELS  Recent

Recent

WE WILL REFER TO THESE TWO AS (L-S) AND (L-L),                     Recent

Recent

X**2     = (GAM(N)*(GAM(T)/2)**2/(DEN)**2      + (L-L)             Recent

= (GAM(N)**2*((GAM(T)/2)**2)/(DEN)**2 + (L-L)             Recent

Y**2     = (GAM(N))**2*((E-ER))**2/(DEN)**2    + (L-L)             Recent

Recent

X**2+Y**2= GAM(N)**2*DEN/(DEN)**2 = GAM(N)**2/DEN + (L-L)          Recent

Recent

TO SEE THE EFFECT OF INCLUDING MULTI-LEVEL INTERFERENCE WE CAN     Recent

CONSIDER OUR GENERAL EXPRESSION FOR ABSORPTION,                    Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

Recent

AND NOTE THAT FOR BOTH SINGLE AND MULTI-LEVEL BREIT WIGNER THE     Recent

ENDF-102 SAYS TO TREAT ABSORPTION IN A SINGLE LEVEL APPROXIMATION  Recent

I.E., IGNORE LEVEL-LEVEL INTERFERENCE. IF ALL INTERFERENCE IS      Recent

IGNORED THIS IS EQUIVALENT TO COMPLETELY IGNORING X**2 + Y**2 AND  Recent

DEFINING,                                                          Recent

Recent

ABSORPTION =GJ*2*X                                                 Recent

=2*GJ*GAM(N)*GAM(T)/DEN                                 Recent

Recent

WHICH IS INCORRECT - SINCE THIS SEEMS TO INDICATE EVERYTHING IS    Recent

ABSORBED. IN ORDER TO OBTAIN THE CORRECT EXPRESSION WE CANNOT      Recent

COMPLETELY IGNORE INTERFERENCE - WE CAN IGNORE LEVEL-LEVEL         Recent

INTERFERENCE, BUT WE MUST INCLUDE LEVEL-SELF INTERFERENCE,         Recent

Recent

X**2+Y**2= GAM(N)**2/DEN                                           Recent

Recent

ABSORPTION =GJ*(2*X - ((X)**2 + (Y)**2))                           Recent

=GJ*GAM(N)*(GAM(T)-GAM(N))/DEN                          Recent

=GJ*GAM(N)*GAM(A)/DEN                                   Recent

Recent

SUMMARY                                                            Recent

=======                                                            Recent

AN IMPORTANT POINT TO NOTE IS THE DEFINITION OF (X) AND (Y)        Recent

WHICH IN ALL CASES WILL CORRESPOND TO THE SYMMETRIC AND            Recent

ANTI-SYMMETRIC CONTRIBUTION OF THE RESONANCES. IN PARTICULAR       Recent

DEFINING (U) IN TERMS OF (1-X) INSTEAD OF (X) IS EXTREMELY         Recent

IMPORTANT. NOTE, THAT THE DEFINITION OF THE ELASTIC AND            Recent

ABSORPTION ONLY INVOLVE (X), NOT (1-X). FAR FROM RESONANCES        Recent

(X) CAN BE EXTREMELY SMALL, THEREFORE (1-X) WILL BE VERY CLOSE     Recent

TO (1). IF THE CALCULATION PROCEEDS BY FIRST CALCULATING (1-X)     Recent

AND THEN DEFINING (X) BY SUBTRACTING (1), EXTREME ROUND-OFF        Recent

PROBLEMS CAN RESULT. THESE PROBLEMS CAN BE AVOIDED BY IN ALL       Recent

CASES DEFINING (X) DIRECTLY, WITHOUT ANY DIFFERENCES.              Recent

Recent

IN EACH FORMALISM THE DEFINITION OF (X) AND (Y) MAY BE DIFFERENT   Recent

BUT ONCE WE HAVE DEFINED (X) AND (Y) WE CAN IMMEDIATELY WRITE      Recent

THE CROSS SECTIONS USING A UNIFORM DEFINITION,                     Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 - X)**2 + (SIN(2*PS) + Y)**2)            Recent

Recent

ABSORPTION =-GJ*(2*X + (X)**2 + (Y)**2)                            Recent

Recent

AND DEFINE THE TOTAL AS THE SUM OF THESE 2 PARTS.                  Recent

Recent

RELATIONSHIP TO SINGLE LEVEL                                       Recent

============================                                       Recent

HOW DO THE SINGLE AND MULTI-LEVEL FORMALISMS COMPARE. TO SEE,      Recent

STARTING FROM OUR GENERAL DEFINITION OF THE ELASTIC IN THE FORM,   Recent

Recent

ELASTIC =GJ*(2*SIN(PS)**2 + X)**2 + (SIN(2*PS) + Y)**2)            Recent

=GJ*(4*SIN(PS)**4 - 4*X*SIN(PS)**2 + X**2                  Recent

+ SIN(2*PS)**2 + 2*Y*SIN(2*PS)  + Y**2)                 Recent

Recent

=4*GJ*SIN(PS)**2 +                                         Recent

GJ*(X**2 + Y**2                                         Recent

-4*X*SIN(PS)**2                                      Recent

+2*Y*SIN(2*PS))                                      Recent

Recent

AND OUR SPECIFIC DEFINITIONS OF (X) AND (Y) FOR MULTI-LEVEL BREIT- Recent

WIGNER PARAMETERS,                                                 Recent

Recent

X        = GAM(N)*GAM(T)/2/DEN                                     Recent

Y        = GAM(N)*(E-ER)/DEN                                       Recent

DEN      = ((E-ER)**2 + (GAM(T)/2)**2)                             Recent

Recent

X**2+Y**2= GAM(N)**2/DEN + (L-L)                                   Recent

Recent

WE CAN RECOGNIZE X**2 AND Y**2 AS THE INTERFERENCE - (L-S) + (L-L) Recent

TERMS IN THE MULTI-LEVEL FORMALISM. IN ORDER TO OBTAIN THE SINGLE  Recent

LEVEL EQUATION WE CAN ASSUME THAT EACH LEVEL DOES NOT INTERFERE    Recent

WITH ANY OTHER LEVEL - THEREFORE THE (L-L) CONTRIBUTION IS ZERO.   Recent

Recent

ELASTIC =4*GJ*SIN(PS)**2 +                                         Recent

GJ*GAM(N)*(GAM(N)                                       Recent

-2*GAM(T)*SIN(PS)**2                         Recent

+2*(E-ER)*SIN(2*PS))/DEN                     Recent

Recent

WHICH IS THE FORM THAT IT APPEARS IN ENDF-102, EXCEPT FOR TWO      Recent

TYPOGRAPHICAL ERRORS IN THE SECOND TERM,                           Recent

Recent

-2*GAM(T)*SIN(PS)**2                                               Recent

Recent

WHICH IN ENDF-102 IS WRITTEN,                                      Recent

Recent

-2*(GAM(T)-GAM(N))*SIN(2*PS)**2                                    Recent

Recent

PROGRAM CONVENTIONS                                                Recent

================================================================== Recent

MINIMUM INPUT DATA                                                 Recent

------------------                                                 Recent

FOR EACH MATERIAL TO BE PROCESSED THE MINIMUM INPUT DATA ARE THE   Recent

RESONANCE PARAMETERS IN FILE 2. IF THERE ARE NO FILE 2 PARAMETERS  Recent

IN A GIVEN MATERIAL THE ENTIRE MATERIAL WILL SIMPLY BE COPIED.     Recent

NEITHER THE HOLLERITH SECTION (MF=1, MT=451) NOR THE BACKGROUND    Recent

CROSS SECTION (SECTIONS OF MF=3) NEED BE PRESENT FOR THIS PROGRAM  Recent

TO EXECUTE PROPERLY. HOWEVER, SINCE THE CONVENTIONS USED IN        Recent

INTERPRETING THE RESONANCE PARAMETERS DEPENDS ON ENDF/B VERSION    Recent

USERS ARE STRONGLY RECOMMENDED TO INSURE THAT MF=1, MT=451 IS      Recent

PRESENT IN EACH MATERIAL TO ALLOW THE PROGRAM TO DETERMINE THE     Recent

ENDF/B FORMAT VERSION.                                             Recent

Recent

RESONANCE PARAMETERS                                               Recent

--------------------                                               Recent

RESONANCE PARAMETERS MAY BE REPRESENTED USING ANY COMBINATION      Recent

OF THE REPRESENTATIONS ALLOWED IN ENDF/B,                          Recent

(1) RESOLVED DATA                                                  Recent

(A) SINGLE LEVEL BREIT-WIGNER                                  Recent

(B) MULTI-LEVEL BREIT-WIGNER                                   Recent

(C) ADLER-ADLER                                                Recent

(D) REICH-MOORE                                                Recent

(E) HYBRID R-FUNCTION                                          Recent

(2) UNRESOLVED DATA                                                Recent

(A) ALL PARAMETERS ENERGY INDEPENDENT                          Recent

(B) FISSION PARAMETERS ENERGY DEPENDENT                        Recent

(C) ALL PARAMETERS ENERGY DEPENDENT                            Recent

Recent

THE FOLLOWING RESOLVED DATA FORMALISMS ARE NOT TREATED BY THIS     Recent

VERSION OF THE CODE AND WILL ONLY BE IMPLEMENTED AFTER EVALUATIONS Recent

USING THESE FORMALISMS ARE AVAILABLE TO THE AUTHOR OF THIS CODE    Recent

FOR TESTING IN ORDER TO INSURE THAT THEY CAN BE HANDLED PROPERLY   Recent

(A) GENERAL R-MATRIX                                           Recent

Recent

CALCULATED CROSS SECTIONS                                          Recent

-------------------------                                          Recent

THIS PROGRAM WILL USE THE RESONANCE PARAMETERS TO CALCULATE THE    Recent

TOTAL, ELASTIC, CAPTURE AND POSSIBLY FISSION CROSS SECTIONS. THE   Recent

COMPETITIVE WIDTH WILL BE USED IN THESE CALCULATIONS, BUT THE      Recent

COMPETITIVE CROSS SECTION ITSELF WILL NOT BE CALCULATED. THE       Recent

ENDF/B CONVENTION IS THAT ALTHOUGH A COMPETITIVE WIDTH MAY BE      Recent

GIVEN, THE COMPETITIVE CROSS SECTION MUST BE SEPARATELY TABULATED  Recent

AS A SECTION OF FILE 3 DATA.                                       Recent

Recent

RESOLVED REGION                                                    Recent

---------------                                                    Recent

IN THE RESOLVED REGION THE RESOLVED PARAMETERS ARE USED TO         Recent

CALCULATE COLD (0 KELVIN), LINEARLY INTERPOLABLE, ENERGY DEPENDENT Recent

CROSS SECTIONS.                                                    Recent

Recent

SCATTERING RADIUS                                                  Recent

-----------------                                                  Recent

FOR SINGLE OR MULTI LEVEL BREIT-WIGNER PARAMETERS THE SCATTERING   Recent

RADIUS MAY BE SPECIFIED IN EITHER ENERGY INDEPENDENT (CONSTANT)    Recent

OR ENERGY DEPENDENT FORM (A TABLE OF ENERGY VS. RADIUS AND AN      Recent

ASSOCIATED INTERPOLATION LAW). IN ALL OTHER CASE ONLY AN ENERGY    Recent

INDEPENDENT SCATTERING RADIUS IS ALLOWED.                          Recent

Recent

FOR ANY ONE MATERIAL (I.E. MAT) IF ENERGY DEPENDENT SCATTERING     Recent

RADII ARE GIVEN THE TOTAL NUMBER OF INTERPOLATION REGIONS AND      Recent

TABULATED VALUES FOR THE ENTIRE MATERIAL CANNOT EXCEED,            Recent

200 - INTERPOLATION REGIONS                                        Recent

500 - TABULATED VALUES                                             Recent

IF THESE LIMITS ARE EXCEEDED THE PROGRAM WILL PRINT AN ERROR       Recent

MESSAGE AND TERMINATE.                                             Recent

Recent

IF YOU REQUIRE A LARGER NUMBER OF INTERPOLATION REGION AND/OR      Recent

TABULATED VALUES,                                                  Recent

(1) INTERPOLATION REGIONS - INCREASE THE DIMENSION OF NBTRHO AND   Recent

INTRHO IN COMMON/TABRHO/ THROUGHOUT THE PROGRAM AND CHANGE MAXSEC  Recent

IN SUBROUTINE RDAP (MAXSEC = MAXIMUM NUMBER OF INTERPOLATION       Recent

REGIONS).                                                          Recent

(2) TABULATED VALUES - INCREASE THE DIMENSION OF ERHOTB, RHOTAB    Recent

AND APTAB IN COMMON/TABRHO/ THROUGHOUT THE PROGRAM AND CHANGE      Recent

MAXRHO IN SUBROUTINE RDAP (MAXRHO = MAXIMUM NUMBER OF TABULATED    Recent

VALUES).                                                           Recent

Recent

RESOLVED REICH-MOORE AND MULTI-LEVEL BREIT-WIGNER PARAMETERS       Recent

------------------------------------------------------------       Recent

CROSS SECTIONS FOR REICH-MOORE PARAMETERS ARE CALCULATED ACCORDING Recent

TO THE EQUATION (1) - (8) OF SECTION D.1.3 OF ENDF-102. IN ORDER   Recent

TO CALCULATE CROSS SECTIONS FROM MULTI-LEVEL PARAMETERS IN A       Recent

REASONABLE AMOUNT OF TIME THIS PROGRAM EXPRESSES THE CROSS SECTION Recent

IN TERMS OF A SINGLE SUM OVER RESONANCES (SEE, ENDF-102, SECTION   Recent

D.1.2, EQUATIONS 6-7), RATHER THAN AS A DOUBLE SUM (SEE, ENDF-102  Recent

SECTION D.1.2, EQUATION 1-2). IN ORDER FOR THE ENDF-102 EQUATIONS  Recent

TO BE CORRECT THE PARAMETERS MUST MEET THE FOLLOWING CONDITIONS,   Recent

Recent

(1) FOR EACH L STATE ALL PHYSICALLY POSSIBLE J SEQUENCES MUST BE   Recent

PRESENT. ONLY IN THIS CASE WILL THE CONTRIBUTIONS OF THE       Recent

INDIVIDUAL J SEQUENCES ADD UP TO PRODUCE THE CORRECT POTENTIAL Recent

SCATTERING CONTRIBUTION FOR THE L STATE (SEE, ENDF-102,        Recent

SECTION D.1.2, EQUATIONS 6-7). IF ANY J SEQUENCE IS MISSING    Recent

THE PROGRAM WILL PRINT A WARNING AND ADD THE J SEQUENCE WITH   Recent

NO RESONANCE PARAMETERS IN ORDER TO ALLOW THE POTENTIAL        Recent

SCATTERING TO BE CALCULATED CORRECTLY (THIS IS EQUIVALENT TO   Recent

ASSUMING THAT THE EVALUATOR REALIZES THAT ALL J SEQUENCES MUST Recent

BE AND ARE PRESENT AND THAT THE EVALUATION STATES THAT THERE   Recent

ARE NO RESONANCES WITH CERTAIN PHYSICALLY POSSIBLE J VALUES... Recent

IN THIS CASE POTENTIAL CONTRIBUTION MUST STILL BE CONSIDERED). Recent

Recent

EXAMPLE                                                        Recent

=======                                                        Recent

AN EXAMPLE OF WHERE THIS OCCURS AND IS IMPORTANT TO CONSIDER   Recent

IS U-238 IN ENDF/B-IV AND V LIBRARIES WHERE FOR L=1 THERE IS   Recent

ONLY A J=1/2 SEQUENCE. NOT INCLUDING THE J=3/2 SEQUENCE LEADS  Recent

TO UNDERESTIMATING THE POTENTIAL SCATTERING AND PRODUCES       Recent

MINIMA IN THE ELASTIC CROSS SECTION WHICH ARE AN ORDER OF      Recent

MAGNITUDE LOWER THAN THE CROSS SECTIONS OBTAINED BE INCLUDING  Recent

THE J=3/2 SEQUENCE.                                            Recent

Recent

(2) FOR A GIVEN TARGET SPIN AND L VALUE THERE MAY BE 2 POSSIBLE    Recent

MEANS OF OBTAINING THE SAME J VALUE. WHEN THIS OCCURS IN       Recent

ORDER TO CALCULATE THE CORRECT POTENTIAL SCATTERING CROSS      Recent

SECTION IT IS IMPORTANT TO INCLUDE THE EFFECT OF BOTH          Recent

POSSIBLE J SEQUENCES, EVEN THOUGH FROM THE ENDF/B DATA IT IS   Recent

NOT POSSIBLE TO DETERMINE WHICH OF THE 2 POSSIBLE SEQUENCES    Recent

ANY GIVEN RESONANCE BELONGS TO. IN THIS CASE THIS PROGRAM      Recent

TREAT ALL RESONANCES WITH THE SAME J VALUE AS BELONGING TO     Recent

THE SAME J SEQUENCE (TO ALLOW INTERFERENCE) AND WILL ADD AN    Recent

ADDITIONAL J SEQUENCE WITH NO RESONANCES IN ORDER TO ALLOW     Recent

THE POTENTIAL CROSS SECTION TO BE CALCULATED CORRECTLY. WHEN   Recent

THIS OCCURS A WARNING MESSAGE IS PRINTED, BUT BASED ON THE     Recent

ENDF/B DATA THERE IS NOTHING WRONG WITH THE DATA AND THERE IS  Recent

NOTHING THAT THE USER CAN DO TO CORRECT OR IN ANY WAY MODIFY   Recent

THE DATA TO ELIMINATE THE PROBLEM.                             Recent

Recent

EXAMPLE                                                        Recent

=======                                                        Recent

FOR A TARGET SPIN =1 AND L=1 THE 2 RANGES OF PHYSICALLY        Recent

POSSIBLE J ARE 1/2, 3/2, 5/2 AND 1/2, 3/2. BY CHECKING THE     Recent

ENDF/B DATA IT IS POSSIBLE TO INSURE THAT THE 3 POSSIBLE       Recent

J VALUES (1/2, 3/2, 5/2) ARE PRESENT AND TO INCLUDE ALL 3      Recent

J SEQUENCES IN THE CALCULATIONS. HOWEVER, UNLESS ALL 5         Recent

POSSIBLE J SEQUENCES ARE INCLUDED THE STATISTICAL WEIGHTS      Recent

OF THE J SEQUENCES WILL NOT SUM UP TO 2*L+1 AND THE            Recent

POTENTIAL CROSS SECTION WILL BE UNDERESTIMATED. IN THIS        Recent

EXAMPLE THE SUM OF THE 3 J SEQUENCES 1/2, 3/2, 5/2 IS 2,       Recent

RATHER THAN 3 AS IT SHOULD BE FOR L=1, AND THE CONTRIBUTION    Recent

OF THE L=1 RESONANCES TO THE POTENTIAL SCATTERING CROSS        Recent

SECTION WILL ONLY BE 2/3 OF WHAT IT SHOULD BE, UNLESS THE      Recent

OTHER 2 J SEQUENCES (WITH DUPLICATE J VALUES) ARE INCLUDED     Recent

IN THE CALCULATION.                                            Recent

Recent

(3) EACH RESONANCE MUST HAVE AN ASSIGNED, PHYSICALLY POSSIBLE      Recent

J VALUE. PHYSICALLY IMPOSSIBLE OR AVERAGE J VALUES CANNOT BE   Recent

UNIQUELY INTERPRETED USING THE EQUATIONS IN ENDF-102 AND       Recent

THEIR USE WILL USUALLY RESULT IN PHYSICALLY UNRELIABLE CROSS   Recent

SECTIONS. THIS PROGRAM WILL CHECK ALL J VALUES AND IF ANY ARE  Recent

ARE FOUND TO BE PHYSICALLY IMPOSSIBLE (BASED ON TARGET SPIN    Recent

AND L VALUE) AN ERROR MESSAGE WILL BE PRINTED TO INDICATE THAT Recent

THE RECONSTRUCTED CROSS SECTIONS WILL BE UNRELIABLE AND THE    Recent

PROGRAM WILL CONTINUE. IN AN ATTEMPT TO CALCULATE THE CORRECT  Recent

POTENTIAL SCATTERING CROSS SECTION THIS PROGRAM WILL SUBTRACT  Recent

THE POTENTIAL SCATTERING CONTRIBUTION DUE TO ALL FICTICIOUS J  Recent

SEQUENCES AND ADD THE CONTRIBUTION OF ALL PHYSICALLY POSSIBLE  Recent

J SEQUENCES (AS DESCRIBED ABOVE).                              Recent

Recent

WARNING (LET THE USER BEWARE)                                  Recent

=============================                                  Recent

(A) IT CANNOT BE STRESSED ENOUGH THAT CROSS SECTIONS OBTAINED  Recent

USING PHYSICALLY IMPOSSIBLE J VALUES FOR REICH-MOORE AND   Recent

MULTI-LEVEL BREIT-WIGNER RESONANCE PARAMETERS WILL RESULT  Recent

IN UNRELIABLE CROSS SECTIONS. THE DECISION TO HAVE THIS    Recent

PROGRAM CONTINUE TO PROCESS WHEN THIS CONDITION IS FOUND   Recent

IS BASED ON AN ATTEMPT TO ALLOW THE USER TO AT LEAST HAVE  Recent

SOME RESULTS (HOWEVER BAD THEY MAY BE) IF THERE IS NO      Recent

OTHER EVALUATED DATA AVAILABLE.                            Recent

(B) EVEN THOUGH THE REICH-MOORE AND MULTI-LEVEL EQUATIONS ARE  Recent

DEFINED AS ABSOLUTE OR SQUARED CONTRIBUTIONS WHICH MUST    Recent

ALL BE PHYSICALLY POSSIBLE, ATTEMPTING TO CORRECT THE      Recent

POTENTIAL CROSS SECTION (AS DESCRIBED ABOVE) CAN LEAD TO   Recent

NEGATIVE ELASTIC CROSS SECTIONS. THIS IS BECAUSE BASED ON  Recent

THE INFORMATION AVAILABLE IN THE EVALUATION IT IS NOT      Recent

NOT POSSIBLE TO CORRECTLY ACCOUNT FOR THE INTERFERENCE     Recent

BETWEEN THE RESONANCE AND POTENTIAL CONTRIBUTIONS FOR EACH Recent

J SEQUENCE.                                                Recent

Recent

UNRESOLVED RESONANCE REGION                                        Recent

---------------------------                                        Recent

IN THE UNRESOLVED RESONANCE REGION THE UNRESOLVED PARAMETERS       Recent

ARE USED TO CALCULATE INFINITELY DILUTE AVERAGE CROSS SECTIONS.    Recent

NOTE, IT IS IMPORTANT TO UNDERSTAND THAT FROM THE DEFINITION OF    Recent

THE UNRESOLVED PARAMETERS IT IS NOT POSSIBLE TO UNIQUELY CALCULATE Recent

ENERGY DEPENDENT CROSS SECTIONS. ONLY AVERAGES OR DISTRIBUTIONS    Recent

MAY BE CALCULATED.                                                 Recent

Recent

UNRESOLVED INTERPOLATION                                           Recent

------------------------                                           Recent

IN THE UNRESOLVED RESONANCE REGION CROSS SECTIONS AT EACH ENERGY   Recent

ARE CALCULATED BY INTERPOLATING PARAMETERS. THIS IS THE CONVENTION Recent

USED IN ENDF/B-IV AND EARLIER VERSIONS OF ENDF/B. THE ENDF/B-V     Recent

CONVENTION OF INTERPOLATING CROSS SECTIONS, NOT PARAMETERS, HAS    Recent

BEEN ABANDONED AS IMPRACTICAL SINCE IT CAN LEAD TO THE SITUATION   Recent

WHERE EXACTLY THE SAME PHYSICAL DATA CAN LEAD TO DIFFERENT RESULTS Recent

DEPENDING ON WHICH OF THE THREE ENDF/B UNRESOLVED PARAMTER FORMATS Recent

IS USED. FOR EXAMPLE, GIVEN A SET OF ENERGY INDEPENDENT UNRESOLVED Recent

PARAMETERS IT IS POSSIBLE TO CODE THESE PARAMETERS IN EACH OF THE  Recent

THREE ENDF/B UNRESOLVED PARAMETER FORMATS. SINCE PHYSICALLY WE     Recent

ONLY HAVE ONE SET OF PARAMETERS WE WOULD EXPECT THE RESULTS TO BE  Recent

INDEPENDENT OF HOW THEY ARE REPRESENTED IN ENDF/B. UNFORTUNATELY   Recent

USING THE ENDF/B-V CONVENTION TO INTERPOLATE CROSS SECTIONS CAN    Recent

LEAD TO THREE COMPLETELY DIFFERENT RESULTS. IN CONTRAST USING THE  Recent

ENDF/B-IV AND EARLIER CONVENTION OF INTERPOLATING PARAMETERS LEADS Recent

TO COMPLETELY CONSISTENT RESULTS.                                  Recent

Recent

INTERNAL REPRESENTATION OF UNRESOLVED PARAMETERS                   Recent

------------------------------------------------                   Recent

ANY OF THE THREE POSSIBLE REPRESENTATIONS OF UNRESOLVED PARAMETERS Recent

CAN BE UNIQUELY REPRESENTED IN THE ALL PARAMETERS ENERGY DEPENDENT Recent

REPRESENTATIONS WITH THE APPROPRIATE (ENDF/B VERSION DEPENDENT)    Recent

INTERPOLATION LAW. THIS IS DONE BY THE PROGRAM WHILE READING THE   Recent

UNRESOLVED PARAMETERS AND ALL SUBSEQUENT CALCULATIONS NEED ONLY    Recent

CONSIDER THE ALL PARAMETERS ENERGY DEPENDENT REPRESENTATION.       Recent

Recent

RESONANCE RECONSTRUCTION STARTING ENERGY GRID                      Recent

---------------------------------------------                      Recent

AS IN ANY ITERATIVE METHOD THE WAY TO SPEED CONVERGENCE IS TO TRY  Recent

TO START CLOSE TO THE ANSWER. THIS PROGRAM ATTEMPTS TO DO THIS BY  Recent

STARTING FROM AN ENERGY GRID WHICH IS A GOOD APPROXIMATION TO A    Recent

SIMPLE BREIT-WIGNER LINE SHAPE,                                    Recent

Recent

SIGMA(X)=1.0/(1.0+X*X)                                             Recent

Recent

WHERE X IS THE DISTANCE FROM THE PEAK IN HALF-WIDTHS               Recent

Recent

SUBROUTINE SUBINT HAS A BUILT-IN TABLE OF NODES WHICH ARE THE      Recent

HALF-WIDTH MULTIPLES TO APPROXIMATE THE SIMPLE BREIT-LINE SHAPE    Recent

TO WITHIN 1 PER-CENT OVER THE ENTIRE INTERVAL 0 TO 500 HALF-WIDTHS Recent

Recent

BETWEEN ANY TWO RESOLVED RESONANCES THE STARTING GRID IS BASED ON  Recent

THE HALF-WIDTHS OF THE TWO RESONANCES. FROM THE LOWER ENERGY       Recent

RESONANCE UP TO THE MID-POINT BETWEEN THE RESONANCES (MID-POINT    Recent

IS DEFINED HERE AS AN EQUAL NUMBER OF HALF-WIDTHS FROM EACH        Recent

RESONANCE) THE HALF-WIDTH OF THE LOWER ENERGY RESONANCE IS USED.   Recent

FROM THE MID-POINT UP TO THE HIGHER ENERGY RESONANCE THE HALF-     Recent

WIDTH OF THE UPPER ENERGY RESONANCE IS USED.                       Recent

Recent

WITH THIS ALOGORITHM CLOSELY SPACED RESONANCES WILL HAVE ONLY      Recent

A FEW STARTING NODES PER RESONANCE (E.G. U-235). WIDELY SPACED     Recent

RESONANCES WILL HAVE MORE NODES PER RESONANCE (E.G. U-238). FOR    Recent

A MIX OF S, P, D ETC. RESONANCES THIS ALOGORITHM GUARANTEES AN     Recent

ADEQUTE DESCRIPTION OF THE PROFILE OF EVEN EXTREMELY NARROW        Recent

RESONANCES (WHICH MAY IMMEDIATELY CONVERGENCE TO THE ACCURACY      Recent

REQUESTED, THUS MINIMIZING ITERATION).                             Recent

Recent

BACKGROUND CROSS SECTIONS                                          Recent

-------------------------                                          Recent

THE PROGRAM WILL SEARCH FOR BACKGROUND CROSS SECTIONS FOR TOTAL    Recent

(MT=1), ELASTIC (MT=2), FISSION (MT=18), FIRST CHANCE FISSION      Recent

(MT=19) AND CAPTURE (MT=102).                                      Recent

Recent

(1) THE BACKGROUND CROSS SECTIONS (FILE 3) CAN BE PRESENT OR NOT   Recent

PRESENT FOR EACH REACTION.                                     Recent

(2) IF FOR A GIVEN REACTION THE BACKGROUND CROSS SECTION IS        Recent

PRESENT, IT WILL BE ADDED TO THE RESONANCE CONTRIBUTION AND    Recent

THE RESULT WILL BE OUTPUT.                                     Recent

(3) IF FOR A GIVEN REACTION THE BACKGROUND IS NOT PRESENT THE      Recent

PROGRAM WILL,                                                  Recent

(A) IF THE INPUT TO THE PROGRAM SPECIFIES NO OUTPUT FOR        Recent

REACTIONS WITH NO BACKGROUND THERE WILL BE NO OUTPUT.      Recent

(B) IF THE INPUT TO THE PROGRAM SPECIFIES OUTPUT FOR REACTIONS Recent

WITH NO BACKGROUND,                                        Recent

(I) THE RESONANCE CONTRIBUTION TO TOTAL, ELASTIC OR        Recent

CAPTURE WILL BE OUTPUT.                                Recent

(II) IF ALL FISSION RESONANCE PARAMETERS ARE ZERO THE      Recent

FISSION CROSS SECTION (MT=18) WILL NOT BE OUTPUT.     Recent

OTHERWISE THE RESONANCE CONTRIBUTION OF THE FISSION   Recent

(MT=18) WILL BE OUTPUT.                               Recent

(III) THERE WILL BE NO OUTPUT FOR FIRST CHANCE FISSION     Recent

(MT=19).                                             Recent

Recent

COMBINING RESONANCES AND BACKGROUND CROSS SECTIONS                 Recent

--------------------------------------------------                 Recent

IN ORDER TO BE COMBINED WITH THE RESONANCE CONTRIBUTION THE        Recent

BACKGROUND CROSS SECTIONS MUST BE GIVEN AT 0 KELVIN TEMPERATURE    Recent

AND MUST BE LINEARLY INTERPOLABLE. IF THESE CONDITIONS ARE MET     Recent

THE RESONANCE AND BACKGROUND CONTRIBUTIONS WILL BE ADDED TOGETHER  Recent

AND OUTPUT. IF THESE CONDITIONS ARE NOT MET THE BACKGROUND CROSS   Recent

SECTION WILL BE IGNORED AND ONLY THE RESONANCE CONTRIBUTION WILL   Recent

BE OUTPUT. IF THE BACKGROUND HAS NOT BEEN ADDED TO THE RESONANCE   Recent

CONTRIBUTION AFTER THIS PROGRAM FINISHES THE USER CAN MAKE THE     Recent

RESONANCE AND BACKGROUND CONTRIBUTIONS COMPATIBLE BY,              Recent

Recent

(1) IF THE BACKGROUND IS NOT LINEARLY INTERPOABLE, LINEARIZE THE   Recent

BACKGROUND (E.G., USE PROGRAM LINEAR).                         Recent

(2) IF THE BACKGROUND IS NOT GIVEN AT 0 KELVIN, DOPPLER BROADEN    Recent

THE RESONANCE (NOT BACKGROUND) CONTRIBUTION TO THE SAME        Recent

TEMPERATURE AS THE BACKGROUND (E.G., USE PROGRAM SIGMA1).      Recent

Recent

ONCE THE RESONANCE AND BACKGROUND CONTRIBUTIONS HAVE BEEN MADE     Recent

COMPATIBLE THEY CAN BE ADDED TOGETHER (E.G., USE PROGRAM MIXER).   Recent

Recent

THE RECONSTRUCTION OF THE RESONANCE CONTRIBUTION TO THE CROSS      Recent

SECTION CAN BE QUITE EXPENSIVE (IN TERMS OF COMPUTER TIME). SINCE  Recent

THE RECONSTRUCTION IS PERFORMED BEFORE THE BACKGROUND CROSS        Recent

SECTIONS ARE READ, THE ABOVE CONVENTIONS HAVE BEEN ADOPTED IN      Recent

ORDER TO AVOID LOSE OF COMPUTER TIME INVOLVED IN RECONSTRUCTING    Recent

THE RESONANCE CONTRIBUTION.                                        Recent

Recent

COMMON ENERGY GRID                                                 Recent

------------------                                                 Recent

THIS PROGRAM WILL RECONSTRUCT THE RESONANCE CONTRIBUTION TO THE    Recent

TOTAL, ELASTIC, FISSION AND CAPTURE CROSS SECTIONS ALL ON THE      Recent

SAME ENERGY GRID. EACH REACTION WILL THEN BE COMBINED WITH ITS     Recent

BACKGROUND CROSS SECTION (IF ANY) AND OUTPUT WITHOUT ANY FURTHER   Recent

THINNING. IF THERE ARE NO BACKGROUND CROSS SECTIONS, OR IF THE     Recent

BACKGROUND CROSS SECTION FOR ALL FOUR REACTIONS ARE GIVEN ON A     Recent

COMMON ENERGY GRID, THE OUTPUT FROM THIS PROGRAM WILL BE ON A      Recent

COMMON ENERGY GRID FOR ALL FOUR REACTIONS.                         Recent

Recent

THERMAL ENERGY                                                     Recent

--------------                                                     Recent

IF THE RESONANCE REGION SPANS THERMAL ENERGY (0.0253 EV) THIS      Recent

POINT IS ALWAYS INCLUDED IN THE COMMON ENERGY GRID USED FOR ALL    Recent

REACTIONS AND WILL ALWAYS APPEAR IN THE OUTPUT DATA.               Recent

Recent

SECTION SIZE                                                       Recent

------------                                                       Recent

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Recent

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Recent

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Recent

Recent

SELECTION OF DATA                                                  Recent

-----------------                                                  Recent

THE PROGRAM SELECTS MATERIALS TO BE PROCESSED BASED EITHER ON      Recent

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Recent

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Recent

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Recent

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Recent

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Recent

Recent

ALLOWABLE ERROR                                                    Recent

---------------                                                    Recent

THE RECONSTRUCTION OF LINEARLY INTERPOLABLE CROSS SECTIONS FROM    Recent

RESONANCE PARAMETERS CANNOT BE PERFORMED EXACTLY. HOWEVER IT CAN   Recent

BE PERFORMED TO VIRTUALLY ANY REQUIRED ACCURACY AND MOST           Recent

IMPORTANTLY CAN BE PERFORMED TO A TOLERANCE THAT IS SMALL COMPARED Recent

TO THE UNCERTAINTY IN THE CROSS SECTIONS THEMSELVES. AS SUCH THE   Recent

CONVERSION OF CROSS SECTIONS TO LINEARLY INTERPOLABLE FORM CAN BE  Recent

PERFORMED WITH ESSENTIALLY NO LOSS OF INFORMATION.                 Recent

Recent

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Recent

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Recent

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Recent

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Recent

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Recent

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Recent

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Recent

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES,     Recent

E.G., 0.1 PER-CENT FROM 0 UP TO THE LOW EV RANGE AND A LESS        Recent

STRINGENT TOLERANCE AT HIGHER ENERGIES.                            Recent

Recent

DEFAULT ALLOWABLE ERROR                                            Recent

-----------------------                                            Recent

IN ORDER TO INSURE CONVERENCE OF THE RESONANCE RECONSTRUCTION THE  Recent

ALLOWABLE ERROR MUST BE POSITIVE. IF THE USER INPUTS AN ERROR FOR  Recent

RESONANCE RECONSTRUCTION THAT IS NOT POSITIVE IT WILL BE SET TO    Recent

THE DEFAULT VALUE (CURRENTLY 0.1 PER-CENT) AND INDICATED AS SUCH   Recent

IN THE OUTPUT LISTING.                                             Recent

Recent

INTERVAL HALVING ALGORITHM                                         Recent

-------------------------                                          Recent

THIS PROGRAM WILL START BY CALCULATING THE CROSS SECTIONS AT THE   Recent

ENERGIES CORRESPONDING TO THE PEAK OF EACH RESONANCE, AS WELL AS   Recent

A FIXED NUMBER OF HALF-WIDTHS ON EACH SIDE OF EACH RESONANCE.      Recent

STARTING FROM THIS BASIC GRID OF POINTS THE PROGRAM WILL CONTINUE  Recent

TO HALF EACH INTERVAL UNTIL THE CROSS SECTIONS FOR ALL REACTIONS   Recent

AT THE CENTER OF THE INTERVAL CAN BE DEFINED BY LINEAR             Recent

INTERPOLATION FROM THE ENDS OF THE INTERVAL TO WITHIN THE USER     Recent

SPECIFIED ACCURACY CRITERIA.                                       Recent

Recent

DISTANT RESONANCE TREATMENT                                        Recent

---------------------------                                        Recent

THE OPTION TO TREAT DISTANT RESONANCES, WHICH WAS AVAILABLE IN     Recent

EARLIER VERSIONS OF THIS PROGRAM, IS NO LONGER AVAILABLE, BECAUSE  Recent

IT WAS FOUND TO PRODUCE UNRELIABLE RESULTS. IN THIS VERSION OF     Recent

THE PROGRAM ALL RESONANCES ARE TREATED EXACTLY.                    Recent

Recent

PROGRAM OPERATION                                                  Recent

================================================================== Recent

EDIT MODE                                                          Recent

---------                                                          Recent

IT IS SUGGESTED THAT BEFORE RUNNING THIS PROGRAM TO RECONSTRUCT    Recent

CROSS SECTIONS FROM RESONANCE PARAMETERS (WHICH CAN BE QUITE       Recent

EXPENSIVE) THE USER FIRST RUN THE PROGRAM IN THE EDIT MODE (SEE,   Recent

DESCRIPTION OF INPUT PARAMETERS BELOW). IN THE EDIT MODE THE       Recent

PROGRAM WILL READ, LIST AND EXTENSIVELY CHECK THE CONSISTENCY OF   Recent

ALL RESONANCE PARAMETERS AND ENDF/B DEFINED RESONANCE FLAGS. THIS  Recent

IS A VERY INEXPENSIVE MEANS OF CHECKING ALL DATA BEFORE INVESTING  Recent

A LARGE AMOUNT OF MONEY IN RECONSTRUCTING CROSS SECTIONS. ANY AND  Recent

ALL DIGNOSTICS RECEIVED FROM THE EDIT WILL SUGGEST HOW TO CORRECT  Recent

THE EVALUATED DATA TO MAKE IT CONSISTENT BEFORE RECONSTRUCTING     Recent

CROSS SECTIONS. IN ORDER TO OBTAIN MEANINGFUL RESULTS FROM THE     Recent

RECONSTRUCTION ALL SUGGESTED CHANGES TO THE EVALUATION SHOULD BE   Recent

PERFORMED BEFORE TRYING RECONSTRUCTION (OTHERWISE THE RESULT OF    Recent

RECONSTRUCTION WILL NOT BE RELIABLE).                              Recent

Recent

RECONSTRUCTION MODE                                                Recent

-------------------                                                Recent

FOR EACH REQUESTED MATERIAL                                        Recent

---------------------------                                        Recent

IF SECTION MF=1, MT=451 IS PRESENT COMMENTS WILL BE ADD TO         Recent

DOCUMENT THAT THE MATERIAL HAS BEEN PROCESSED. MF=1, MT=451 WILL   Recent

ALSO BE USED TO DETERMINE THE VERSION OF THE ENDF/B FORMAT WHICH   Recent

WILL ALLOW THE PROGRAM TO USE THE APPROPRIATE CONVENTIONS.         Recent

Recent

ALL OF THE FILE 2 RESONANCE PARAMETERS ARE FIRST READ AND THE      Recent

LINEARLY INTERPOLABLE CONTRIBUTION OF THE RESONANCE PARAMETERS     Recent

TO THE TOTAL, ELASTIC, CAPTURE AND FISSION CROSS SECTIONS IS       Recent

CALCULATED SIMULTANEOUSLY USING A COMMON ENERGY GRID FOR ALL       Recent

FOUR REACTIONS.                                                    Recent

Recent

AFTER THE RESONANCE CONTRIBUTION HAS BEEN RECONSTRUCTED EACH OF    Recent

THE FIVE REACTIONS (MT=1, 2, 18, 19, 102) IS CONSIDERED SEPARATELY Recent

FOR COMBINATION WILL THE BACKGROUND CROSS SECTION, IF ANY, AS      Recent

DESCRIBED ABOVE.                                                   Recent

Recent

OUTPUT WILL INCLUDE THE ENTIRE EVALUATION, INCLUDING RESONANCES    Recent

PARAMETERS WITH LRU MODIFIED (AS DESCRIBED ABOVE) TO INDICATE      Recent

THAT THE RESONANCE CONTRIBUTION HAS ALREADY BEEN ADDED TO THE      Recent

FILE 3 CROSS SECTIONS.                                             Recent

Recent

THE CYCLE OF RECONSTRUCTING THE RESONANCE CONTRIBUTION AND ADDING  Recent

THE BACKGROUND WILL BE REPEATED FOR EACH MATERIAL REQUESTED.       Recent

Recent

PROCESS ONLY A PORTION OF RESONANCE REGION                         Recent

================================================================== Recent

MODERN EVALUATIONS MAY BE EXTREMELY LARGE AND IT MAY NOT BE        Recent

POSSIBLE TO PROCESS AN ENTIRE EVALUATION (I.E., ADD THE RESONANCE  Recent

CONTRIBUTION) DURING A SINGLE COMPUTER RUN.                        Recent

Recent

ALSO IN THE CASE WHERE YOU ARE ONLY INTERESTED IN THE CROSS        Recent

SECTIONS OVER A SMALL ENERGY RANGE, YOU MAY NOT WANT TO PROCESS    Recent

AN ENTIRE EVALUATION, E.G., IF YOU ONLY WANT TO KNOW WHAT THE      Recent

CROSS SECTIONS ARE NEAR THERMAL ENERGY, 0.0253 EV.                 Recent

Recent

IN ORDER TO ALLOW AN EVALUATION TO BE PROCESSED USING A NUMBER OF  Recent

SHORTER COMPUTER RUNS AN OPTION HAS BEEN ADDED TO THIS PROGRAM TO  Recent

ALLOW THE USER TO SPECIFY THE ENERGY RANGE TO BE PROCESSED.        Recent

Recent

USING THIS OPTION YOU MAY START AT THE LOWEST ENERGY (ZERO UP TO   Recent

SOME ENERGY) AND USE THE RESULTS OF THIS RUN AS INPUT TO THE       Recent

NEXT RUN, WHERE YOU CAN SPECIFY THE NEXT ENERGY RANGE. THIS        Recent

CYCLE CAN BE REPEATED UNTIL YOU HAVE PROCESSED THE ENTIRE          Recent

EVALUATION.                                                        Recent

Recent

WARNING - THIS OPTION SHOULD BE USED WITH EXTREME CARE - THIS      Recent

OPTION HAS BEEN RELUCTANTLY ADDED - RELUCTANTLY BECAUSE IT CAN     Recent

BE EXTREMELY DANGEROUS TO USE THIS OPTION UNLESS YOU CAREFULLY     Recent

CHECKED WHAT YOU ARE DOING.                                        Recent

Recent

THE OPTION SHOULD ONLY BE USED AS FOLLOWS,                         Recent

1) YOU MUST PROCESS USING ENERGY RANGES STARTING AT LOW ENERGY     Recent

AND WORKING YOUR WAY TOWARD HIGH ENERGY, E.G.,                  Recent

0.0   TO  3.0+3                                                Recent

3.0+3 TO 10.0+3                                                Recent

10.0+3 TO 80.0+3, ETC.                                          Recent

2) FOR THE LAST ENERGY RANGE THE LOWER ENERGY LIMIT MUST BE        Recent

NON-ZERO (WHERE TO START) AND THE UPPER ENERGY LIMIT MUST       Recent

BE ZERO (NO LIMIT)                                              Recent

80.0+3 TO  0.0                                                  Recent

Recent

IF YOU ARE ONLY INTERESTED IN THE CROSS SECTION OVER A NARROW      Recent

ENERGY INTERVAL AND DO NOT INTENT TO MAKE ANY OTHER USE OF THE     Recent

RESULTS, YOU CAN IGNORE THESE WARNINGS AND MERELY SPECIFY ANY      Recent

ENERGY INTERVAL OVER WHICH YOU WISH CALCULATIONS TO BE             Recent

PERFORMED.                                                         Recent

Recent

NORMALLY WHEN THIS PROGRAM PROCESSES AN EVALUATION IT WILL SET     Recent

FLAGS IN THE EVALUATION TO PREVENT THE SAME RESONANCE              Recent

CONTRIBUTION FROM BEING ADDED TO THE CROSS SECTION MORE THAN       Recent

ONCE, SHOULD YOU USE THE OUTPUT FROM THIS PROGRAM AS INPUT TO      Recent

THE PROGRAM.                                                       Recent

Recent

WHEN PROCESSING ONLY PORTIONS OF THE RESONANCE REGION THIS         Recent

PROGRAM CANNOT SET THESE FLAGS TO PROTECT AGAINST ADDING THE       Recent

RESONANCE CONTRIBUTION MORE THAN ONCE - WHICH MAKES USE OF         Recent

THIS OPTION EXTREMELY DANGEROUS.                                   Recent

Recent

ONLY YOU CAN CHECK TO MAKE SURE THAT YOU HAVE CORRECTLY            Recent

INCLUDED EACH ENERGY RANGE ONLY ONCE - SEE THE COMMENT LINES       Recent

AT THE END OF SECTION, MF=1, MT=451, FOR A COMPLETE RECORD         Recent

OF EACH RUN USING THIS PROGRAM. THIS SECTION WILL CONTAIN          Recent

LINES OF THE FORM                                                  Recent

Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  0.00000+ 0 TO  3.00000+ 3 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  3.00000+ 3 TO  1.00000+ 4 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  1.00000+ 4 TO  8.00000+ 4 EV                         Recent

***************** PROGRAM RECENT (VERSION 2007-1) *************    Recent

ONLY PROCESS  8.00000+ 4 TO  2.00000+ 7 EV                         Recent

Recent

YOU SHOULD CHECK TO INSURE THAT THERE ARE NO OVERLAPPING ENERGY    Recent

RANGES OR MISSING ENERGY RANGES.                                   Recent

Recent

WHEN YOU INDICATE BY INPUT THAT YOU ARE ABOUT TO PROCESS THE       Recent

LAST ENERGY RANGE (SEE ABOVE, LOWER ENERGY LIMIT = NON-ZERO,       Recent

UPPER ENERGY LIMIT = ZERO), THIS PROGRAM WILL ASSUME THAT          Recent

YOU HAVE NOW COMPLETED ALL PROCESSING - AND ONLY THEN WILL         Recent

IT SET FLAGS IN THE EVALUATION TO PREVENT THE RESONANCE            Recent

CONTRIBUTION FROM BEING ADDED MORE THAN ONCE. FOR THIS REASON      Recent

YOU CANNOT PROCESS STARTING WITH ENERGY INTERVALS AT HIGH          Recent

ENERGY AND WORKING TOWARD LOW ENERGY - YOU MUST START AT LOW       Recent

ENERGY AND WORK TOWARD HIGH ENERGY.                                Recent

Recent

I/O FILES                                                          Recent

================================================================== Recent

INPUT FILES                                                        Recent

-----------                                                        Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

2   INPUT LINE (BCD - 80 CHARACTERS/RECORD)                      Recent

10   ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Recent

Recent

OUTPUT FILES                                                       Recent

------------                                                       Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

3   OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Recent

11   FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Recent

Recent

SCRATCH FILES                                                      Recent

-------------                                                      Recent

UNIT  DESCRIPTION                                                  Recent

----  -----------                                                  Recent

12   SCRATCH FILE FOR DATA RECONSTRUCTED FROM RESONANCE           Recent

PARAMETERS (BINARY - 100200 WORDS/RECORD)                    Recent

14   SCRATCH FILE FOR COMBINED FILE 2 AND 3 DATA                  Recent

(BINARY - 40080 WORDS/RECORD)                                Recent

Recent

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Recent

================================================================== Recent

UNIT  FILE NAME                                                    Recent

----  ----------                                                   Recent

2   RECENT.INP                                                   Recent

3   RECENT.LST                                                   Recent

10   ENDFB.IN                                                     Recent

11   ENDFB.OUT                                                    Recent

12   (SCRATCH)                                                    Recent

14   (SCRATCH)                                                    Recent

Recent

INPUT CARDS                                                        Recent

================================================================== Recent

LINE  COLS.  FORMAT  DESCRIPTION                                   Recent

----  -----  ------  -----------                                   Recent

1    1-11    I11   RETRIEVAL CRITERIA (0=MAT, 1=ZA)              Recent

THIS OPTION DEFINED WHETHER COLUMNS 1-22 OF   Recent

SUBSEQUENT INPUT CARDS SHOULD BE INTERPRETED  Recent

TO BE MAT OR ZA RANGES.                       Recent

12-22   E11.4  FILE 2 MINIMUM ABSOLUTE CROSS SECTION         Recent

(IF 1.0E-10 OR LESS IS INPUT THE PROGRAM      Recent

WILL USE 1.0E-10)                             Recent

23-33    I11   TREATMENT OF REACTIONS FOR WHICH BACKGROUND   Recent

CROSS SECTION IS NOT GIVEN.                   Recent

= 0 - IGNOR (I.E. NO OUTPUT)                  Recent

= 1 - OUTPUT RESONANCE CONTRIBUTION.          Recent

THIS OPTION IS USEFUL WITH PARTIAL EVALUATION Recent

(E.G. ENDF/B-V DOSIMETRY LIBRARY) WHERE ONLY  Recent

ONE OR MORE OF THE REACTIONS ARE OF ACTUAL    Recent

INTEREST.                                     Recent

WARNING...THE USE OF THIS FIELD HAS BEEN      Recent

CHANGED. THIS FIELD WAS PREVIOUSLY USED TO    Recent

DEFINE THE PRECISION OF THE CALCULATION AND   Recent

OUTPUT. THE FORMER DEFINITION OF THIS FIELD   Recent

WAS...                                        Recent

MINIMUM ENERGY SPACING FLAG                   Recent

= 0 - 6 DIGIT MINIMUM ENERGY SPACING.         Recent

STANDARD 6 DIGIT E11.4 OUTPUT.          Recent

= 1 - 9 DIGIT MINIMUM ENERGY SPACING.         Recent

STANDARD 6 DIGIT E11.4 OUTPUT.          Recent

= 2 - 9 DIGIT MINIMUM ENERGY SPACING.         Recent

VARIABLE 9 DIGIT F FORMAT OUTPUT.       Recent

FROM EXPERIENCE IT HAS BEEN FOUND THAT        Recent

FAILURE TO SET THIS OPTION TO 2 CAN RESULT    Recent

IN LARGE ERRORS IN THE FINAL DATA. THEREFORE  Recent

INTERNALLY THIS OPTION IS SET TO 2.           Recent

34-44    I11   OPERATING MODE                                Recent

= 0 - CACULATE. MINIMUM OUTPUT LISTING        Recent

= 1 - CACULATE. LIST ALL RESONANCE PARAMETERS Recent

= 2 - EDIT MODE. NO CALCULATION. LIST ALL     Recent

RESONANCE PARAMETERS.                   Recent

NOTE, THE EDIT MODE (=2) IS THE SUGGESTED     Recent

MODE TO FIRST TEST THE CONSISTENCY OF THE     Recent

EVALUATED DATA, BEFORE RECONSTRUCTING CROSS   Recent

SECTIONS (SEE, COMMENTS ABOVE).               Recent

45-55    I11   NEGATIVE CROSS SECTIOIN TREATMENT             Recent

= 0 - O.K. - NO CHANGE                        Recent

= 1 - SET = 0                                 Recent

56-66    I11   MONITOR MODE SELECTOR                         Recent

= 0 - NORMAL OPERATION                        Recent

= 1 - MONITOR PROGRESS OF RECONSTRUCTION OF   Recent

FILE 2 DATA AND COMBINING FILE 2 AND    Recent

FILE 3 DATA. EACH TIME A PAGE OF DATA   Recent

POINTS IS WRITTEN TO A SCRATCH FILE     Recent

PRINT OUT THE TOTAL NUMBER OF POINTS    Recent

ON SCRATCH AND THE LOWER AND UPPER      Recent

ENERGY LIMITS OF THE PAGE (THIS OPTION  Recent

MAY BE USED IN ORDER TO MONITOR THE     Recent

EXECUTION SPEED OF LONG RUNNING JOBS).  Recent

2    1-60    A60   ENDF/B INPUT DATA FILENAME                    Recent

(STANDARD OPTION = ENDFB.IN)                  Recent

3    1-60    A60   ENDF/B OUTPUT DATA FILENAME                   Recent

(STANDARD OPTION = ENDFB.OUT)                 Recent

4-N    1-11    I11   MINIMUM MAT OR ZA (SEE COLS. 1-11, LINE 1)    Recent

12-22    I11   MAXIMUM MAT OR ZA (SEE COLS. 1-11, LINE 1)    Recent

UP TO 100 MAT OR ZA RANGES MAY BE SPECIFIED,  Recent

ONE RANGE PER LINE. THE LIST IS TERMINATED    Recent

BY A BLANK LINE. IF THE THE UPPER LIMIT OF    Recent

ANY REQUEST IS LESS THAN THE LOWER LIMIT THE  Recent

UPPER LIMIT WILL BE SET EQUAL TO THE LOWER    Recent

LIMIT. IF THE FIRST REQUEST LINE IS BLANK IT  Recent

WILL TERMINATE THE REQUEST LIST AND CAUSE ALL Recent

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).     Recent

23-33   E11.4  LOWER ENERGY LIMIT FOR PROCESSING.            Recent

34-44   E11.4  UPPER ENERGY LIMIT FOR PROCESSING.            Recent

*THE LOWER AND UPPER ENERGY LIMITS MUST BE     Recent

ZERO, OR BLANK, UNLESS YOU WISH TO ONLY       Recent

PROCESS A PORTION OF RESONANCE REGIONS.       Recent

*THESE ENERGY LIMITS ARE ONLY READ FROM THE    Recent

FIRST MAT/ZA REQUEST LINE                     Recent

*IF BOTH ARE ZERO (OR BLANK) THE ENTIRE        Recent

RESONANCE REGION FOR EACH MATERIAL WILL BE    Recent

PROCESSED                                     Recent

*IF LIMITS ARE INPUT ONLY THAT PORTION OF THE  Recent

RESONANCE REGION FOR EACH MATERIAL WHICH      Recent

LIES BETWEEN THESE LIMITS WILL BE PROCESSED   Recent

*SEE INSTRUCTIONS ABOVE BEFORE USING THIS      Recent

OPTION.                                       Recent

VARY   1-11   E11.4  ENERGY FOR FILE 2 ERROR LAW     (  SEE   )    Recent

12-22   E11.4  ERROR FOR FILE 2 ERROR LAW      (COMMENTS)    Recent

( BELOW  )    Recent

Recent

NOTE, THIS VERSION OF THE PROGRAM DOES NOT THIN THE COMBINED FILE  Recent

FILE 2 + 3 DATA. AS SUCH THE ERROR LAW FOR COMBINING FILE 2 + 3    Recent

WHICH WAS REQUIRED IN EARLIER VERSIONS OF THIS CODE ARE NO LONGER  Recent

REQUIRED.                                                          Recent

Recent

THE FILE 2 ERROR LAW MAY BE ENERGY INDEPENDENT (DEFINED BY A       Recent

SINGLE ERROR) OR ENERGY DEPENDENT (DEFINED BY UP TO 20 ENERGY,     Recent

ERROR PAIRS). FOR THE ENERGY DEPENDENT CASE LINEAR INTERPOLATION   Recent

WILL BE USED TO DEFINE THE ERROR AT ENERGIES BETWEEN THOSE AT      Recent

WHICH THE ERROR IS TABULATED. THE ERROR LAW IS TERMINATED BY A     Recent

BLANK LINE. IF ONLY ONE ENERGY, ERROR PAIR IS GIVEN THE LAW WILL   Recent

BE CONSIDERED TO BE ENERGY INDEPENDENT. IF MORE THAN ONE PAIR      Recent

IS GIVEN IT BE CONSIDERED TO BE ENERGY DEPENDENT (NOTE, THAT       Recent

FOR A CONSTANT ERROR THE ENERGY INDEPENDENT FORM WILL RUN FASTER.  Recent

HOWEVER, FOR SPECIFIC APPLICATIONS AN ENERGY DEPENDENT ERROR MAY   Recent

BY USED TO MAKE THE PROGRAM RUN CONSIDERABLE FASTER).              Recent

Recent

ALL ENERGIES MUST BE IN ASCENDING ENERGY ORDER. FOR CONVERGENCE    Recent

OF THE FILE 2 RECONSTRUCTION ALGORITHM ALL THE ERRORS MUST BE      Recent

POSITIVE. IF ERROR IS NOT POSITIVE IT WILL BE SET EQUAL TO THE     Recent

STANDARD OPTION (CURRENTLY 0.001, CORRRESPONDING TO 0.1 PER-CENT). Recent

IF THE FIRST LINE OF THE ERROR LAW IS BLANK IT WILL TERMINATE THE  Recent

ERROR LAW AND THE ERROR WILL BE TREATED AS ENERGY INDEPENDENT,     Recent

EQUAL TO THE STANDARD OPTION (CURRENTLY, 0.1 PER-CENT). SEE,       Recent

EXAMPLE INPUT 4.                                                   Recent

Recent

EXAMPLE INPUT NO. 1                                                Recent

-------------------                                                Recent

CONSIDER ALL URANIUM ISOTOPES AND TH-232. CONSIDER CROSS SECTIONS  Recent

WHICH ARE LARGER THAN 1.0E-8 BARNS IN ABSOLUTE VALUE. ONLY OUTPUT  Recent

REACTIONS FOR WHICH A BACKGROUND IS GIVEN. LIST ALL PARAMETERS AND Recent

CALCULATE CROSS SECTIONS. MONITOR THE EXECUTION PROGRESS OF THE    Recent

PROGRAM. BETWEEN 0 AND 100 EV USE 0.1 PER-CENT ACCURACY. BETWEEN   Recent

100 EV AND 1 KEV VARY THE ACCURACY FROM 0.1 TO 1 PER-CENT. ABOVE   Recent

1 KEV USE 1 PER-CENT ACCURACY.                                     Recent

Recent

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Recent

Recent

THE FOLLOWING 11 INPUT CARDS ARE REQUIRED.                         Recent

Recent

1 1.00000-08          0          1          0         1       Recent

ENDFB.IN                                                               Recent

ENDFB.OUT                                                              Recent

92000      92999                                                  Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

0.00000+ 0 1.00000-03                                                  Recent

1.00000+02 1.00000-03                                                  Recent

1.00000+03 1.00000-02                                                  Recent

1.00000+09 1.00000-02                                                  Recent

(END FILE 2 ERROR LAW)                     Recent

Recent

EXAMPLE INPUT NO. 2                                                Recent

-------------------                                                Recent

CONSIDER ALL URANIUM ISOTOPES AND TH-232. CONSIDER CROSS SECTIONS  Recent

WHICH ARE LARGER THAN 1.0E-8 BARNS IN ABSOLUTE VALUE. ONLY OUTPUT  Recent

REACTIONS FOR WHICH A BACKGROUND IS GIVEN. CROSS SECTIONS WILL BE  Recent

CALCULATED, BUT PARAMETERS WILL NOT BE LISTED. THE PROGRESS OF THE Recent

PROGRAM WILL NOT BE MONITORED. USE 0.1 PER-CENT ACCURACY FOR ALL   Recent

ENERGIES. SINCE 0.1 PER-CENT IS THE STANDARD OPTION FOR THE ERROR  Recent

LAW THE FIRST ERROR LAW LINE MAY BE LEFT BLANK.                    Recent

Recent

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Recent

THEN USE THE STANDARD FILENAMES.                                   Recent

Recent

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Recent

Recent

1 1.00000-08          0          0          0         0       Recent

Recent

Recent

92000      92999                                                  Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

(USE STANDARD OPTION FOR ERROR LAW)        Recent

Recent

EXAMPLE INPUT NO. 3                                                Recent

-------------------                                                Recent

THE SAME AS EXAMPLE INPUT NO. 2, ONLY IN THIS CASE ONLY CALCULATE  Recent

CROSS SECTIONS OVER THE ENERGY RANGE 0.01 TO 0.1 EV - ACROSS THE   Recent

THERMAL ENERGY RANGE. NOTE, THE ONLY DIFFERENCE BETWEEN THE INPUT  Recent

PARAMETERS IN THIS CASE AND IN EXAMPLE NO. 2, IS THAT ON THE       Recent

SECOND INPUT LINE WE HAVE ADDED THE ENERGY RANGE 0.01 TO 0.1 EV.   Recent

USE \PREPRO94\LINEAR\ENDFB.OUT AS INPUT AND ENDFB.OUT AS OUTPUT -  Recent

SINCE ENDFB.OUT IS THE STANDARD OUTPUT FILENAME THE NAME CAN BE    Recent

EITHER INCLUDED IN THE INPUT OR LEFT BLANK.                        Recent

Recent

THE FOLLOWING 7 INPUT CARDS ARE REQUIRED.                          Recent

Recent

1 1.00000-08          0          0          0         0       Recent

\PREPRO94\LINEAR\ENDFB.OUT                                             Recent

ENDFB.OUT                                                              Recent

92000      92999 1.00000- 2 1.00000- 1                            Recent

90232                  (UPPER LIMIT AUTOMATICALLY SET TO 90232)   Recent

(END REQUEST LIST)                         Recent

(USE STANDARD OPTION FOR ERROR LAW)        Recent

Recent

EXAMPLE INPUT NO. 4                                                Recent

-------------------                                                Recent

RECONSTRUCT ALL DATA. OUTPUT ALL REACTIONS, REGARDING OF WHETHER   Recent

OR NOT THERE IS A BACKGROUND CROSS SECTION. DO NOT MONITOR THE     Recent

PROGRESS OF THE PROGRAM. RECONSTRUCT CROSS SECTIONS TO 1 PER-CENT  Recent

ACCURACY. USE \ENDFB6\LINEAR\ZA092238 AS INPUT AND                 Recent

\ENDFB6\RECENT\ZA092238 AS OUTPUT.                                 Recent

Recent

THE FOLLOWING 6 INPUT CARDS ARE REQUIRED.                          Recent

Recent

0 0.0                 1          0          0         0       Recent

\ENDFB6\ZA092238                                                       Recent

\ENDFB6\RECENT\ZA092238                                                Recent

(RETRIEVE ALL DATA, END REQUEST LIST)            Recent

1.00000- 2                                                  Recent

(END FILE 2 ERROR LAW)                           Recent

Recent

EXAMPLE INPUT NO. 5                                                Recent

-------------------                                                Recent

RECONSTRUCT ALL DATA. ONLY OUTPUT REACTIONS FOR WHICH A BACKGROUND Recent

CROSS SECTION IS GIVEN. DO NOT MONITOR THE PROGRESS OF THE PROGRAM Recent

RECONSTRUCT CROSS SECTIONS TO 0.1 PER-CENT ACCURACY. USE ENDFB.IN  Recent

AS INPUT AND ENDFB.OUT AS OUTPUT.                                  Recent

Recent

THIS CORRESPONDS TO USING ALL OF THE STANDARD OPTONS BUILT-IN TO   Recent

THE PROGRAM AND ALL INPUT CARDS MAY BE BLANK.                      Recent

Recent

IN THIS CASE THE FOLLOWING 5 INPUT CARDS ARE REQUIRED.             Recent

(ZEROES ARE INDICATED ON THE FIRST LINE, BELOW, ONLY TO INDICATE   Recent

WHERE THE LINE IS. THE ACTUAL INPUT LINE CAN BE COMPLETELY BLANK). Recent

Recent

0 0.0                 0          0          0         0       Recent

(USE STANDARD INPUT FILENAME = ENDFB.IN)         Recent

(USE STANDARD OUTPUT FILENAME = ENDFB.OUT)       Recent

(RETRIEVE ALL DATA, END REQUEST LIST)            Recent

(0.1 ERROR, END FILE 2 ERROR LAW)                Recent

Recent

======================================================================= Recent