======================================================================= Sigma1

Sigma1

PROGRAM SIGMA1                                                     Sigma1

==============                                                     Sigma1

VERSION 73-1 (MARCH 1973)                                          Sigma1

VERSION 76-1 (FEBRUARY 1976)                                       Sigma1

VERSION 76-2 (OCTOBER 1976)                                        Sigma1

VERSION 77-1 (JANUARY 1977)                                        Sigma1

VERSION 78-1 (JULY 1978)                                           Sigma1

VERSION 79-1 (JULY 1979)    CDC-7600 AND CRAY-1 VERSION.           Sigma1

VERSION 80-1 (MAY 1980)     IBM, CDC AND CRAY VERSION              Sigma1

VERSION 80-2 (DECEMBER 1980)IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 81-1 (MARCH 1981)   DOUBLE PRECISION IBM VERSION           Sigma1

VERSION 81-2 (AUGUST 1981)  IMPROVED IBM SPEED AND STABILITY       Sigma1

VERSION 82-1 (JANUARY 1982) IMPROVED COMPUTER COMPATIBILITY        Sigma1

VERSION 83-1 (JANUARY 1983)*MAJOR RE-DESIGN.                       Sigma1

*PAGE SIZE INCREASED - 1002 TO 2004.    Sigma1

*ELIMINATED COMPUTER DEPENDENT CODING.  Sigma1

*NEW, MORE COMPATIBLE I/O UNIT NUMBER.  Sigma1

*ADDED STANDARD ALLOWABLE ERROR OPTION  Sigma1

(CURRENTLY 0.1 PER-CENT).              Sigma1

*UNRESOLVED RESONANCE REGION COPIED.    Sigma1

*1/V EXTENSION OF CROSS SECTIONS        Sigma1

OUTSIDE OF TABULATED ENERGY RANGE AND  Sigma1

INTO UNRESOLVED ENERGY RANGE.          Sigma1

VERSION 83-2 (OCTOBER 1983)*IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 84-1 (APRIL 1984)  *IMPROVED NUMERICAL STABILITY.          Sigma1

*PARTIAL EVALUATION TREATMENT.          Sigma1

VERSION 85-1 (APRIL 1985)  *ITERATE TO CONVERGENCE (USING THE SAME Sigma1

ENERGY GRID FOR HOT CROSS SECTION AS   Sigma1

COLD CROSS SECTIONS WAS FOUND TO BE    Sigma1

INACCURATE).                           Sigma1

*NEW FASTER HIGH ENERGY BROADENING.     Sigma1

*UPDATED FOR ENDF/B-VI FORMATS.         Sigma1

*SPECIAL I/O ROUTINES TO GUARANTEE      Sigma1

ACCURACY OF ENERGY.                    Sigma1

*DOUBLE PRECISION TREATMENT OF ENERGY   Sigma1

(REQUIRED FOR NARROW RESONANCES).      Sigma1

VERSION 85-2 (AUGUST 1985) *FORTRAN-77/H VERSION                   Sigma1

VERSION 86-1 (JANUARY 1986)*ENERGY DEPENDENT SCATTERING RADIUS     Sigma1

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Sigma1

FILE NAMES (SEE, SUBROUTINE FILEIO     Sigma1

FOR DETAILS).                          Sigma1

*IMPROVED BASED ON USER COMMENTS.       Sigma1

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Sigma1

INSURE PROGRAM WILL NOT DO ANYTHING    Sigma1

CRAZY.                                 Sigma1

*UPDATED TO USE NEW PROGRAM CONVERT     Sigma1

KEYWORDS.                              Sigma1

*ADDED LIVERMORE CIVIC COMPILER         Sigma1

CONVENTIONS.                           Sigma1

VERSION 90-1 (JUNE 1990)   *UPDATED BASED ON USER COMMENTS         Sigma1

*ADDED FORTRAN SAVE OPTION              Sigma1

*NEW MORE CONSISTENT ENERGY OUTPUT      Sigma1

ROUTINES                               Sigma1

VERSION 91-1 (JULY 1991)   *WARNING...INPUT PARAMETER FORMAT       Sigma1

HAS BEEN CHANGED - SEE BELOW FOR       Sigma1

DETAILS.                               Sigma1

*ADDED CHARGED PARTICLE PROJECTILES     Sigma1

*OUTPUT ENERGY RANGE IS ALWAYS AT       Sigma1

LEAST AS LARGE AS INPUT ENERGY RANGE.  Sigma1

*NO 1/V EXTENSION OF CROSS SECTIONS     Sigma1

FROM UNRESOLVED ENERGY RANGE.          Sigma1

VERSION 92-1 (JANUARY 1992)*INSURE MINIMUM AND MAXIMUM CROSS       Sigma1

SECTIONS ARE ALWAYS KEPT (NOT THINNED) Sigma1

*MT=19 (FIRST CHANCE FISSION) TREATED   Sigma1

THE SAME AS FISSION.                   Sigma1

*VARIABLE MINIMUM CROSS SECTION OF      Sigma1

INTEREST - TO ALLOW SMALL CROSS        Sigma1

SECTIONS NEAR THRESHOLDS TO BE         Sigma1

TREATED PROPERLY.                      Sigma1

*ALL ENERGIES INTERNALLY ROUNDED PRIOR  Sigma1

TO CALCULATIONS.                       Sigma1

*COMPLETELY CONSISTENT I/O AND ROUNDING Sigma1

ROUTINES - TO MINIMIZE COMPUTER        Sigma1

DEPENDENCE.                            Sigma1

VERSION 92-2 (JULY 1992)   *CORRECTED BUG ASSOCIATED WITH          Sigma1

THRESHOLD REACTIONS.                   Sigma1

*UNRESOLVED REGION COPIED WITHOUT       Sigma1

THINNING (IT SHOULD BE EXACTLY THE     Sigma1

SAME AT ALL TEMPERATURES).             Sigma1

*NO THINNING OF REACTIONS (MT) THAT     Sigma1

WERE NOT BROADENED.                    Sigma1

VERSION 93-1 (APRIL 1993)  *INCREASED PAGE SIZE FROM 2004          Sigma1

TO 24000 ENERGY PONTS.                 Sigma1

VERSION 94-1 (JANUARY 1994)*VARIABLE ENDF/B DATA FILENAMES         Sigma1

TO ALLOW ACCESS TO FILE STRUCTURES     Sigma1

(WARNING - INPUT PARAMETER FORMAT      Sigma1

HAS BEEN CHANGED)                      Sigma1

*CLOSE ALL FILES BEFORE TERMINATING     Sigma1

(SEE, SUBROUTINE ENDIT)                Sigma1

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Sigma1

*IMPROVED COMPUTER INDEPENDENCE        Sigma1

*ALL DOUBLE PRECISION                  Sigma1

*ON SCREEN OUTPUT                      Sigma1

*UNIFORM TREATMENT OF ENDF/B I/O       Sigma1

*IMPROVED OUTPUT PRECISION             Sigma1

*DEFINED SCRATCH FILE NAMES            Sigma1

*ALWAYS INCLUDE THERMAL VALUE          Sigma1

VERSION 97-1 (APRIL 1997)   *OPTIONALLY SET NEGATIVE CROSS         Sigma1

SECTIONS = 0 ON INPUT AND             Sigma1

OUTPUT.                               Sigma1

*INCREASED PAGE SIZE FROM 24000        Sigma1

TO 60000 ENERGY POINTS.               Sigma1

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Sigma1

POINT READ FOR MORE DIGITS            Sigma1

*UPDATED TEST FOR ENDF/B FORMAT        Sigma1

VERSION BASED ON RECENT FORMAT CHANGE Sigma1

*TREAT LOW ENERGY INITIAL CROSS        Sigma1

SECTIONS AS LOG-LOG INTERPOLABLE      Sigma1

*CONSTANT (RATHER THAN 1/V) EXTENSION  Sigma1

TO HIGHER ENERGY.                     Sigma1

*UPDATED CONSTANTS BASED ON CSEWG      Sigma1

SUBCOMMITTEE RECOMMENDATIONS          Sigma1

*GENERAL IMPROVEMENTS BASED ON         Sigma1

USER FEEDBACK                         Sigma1

VERSION 99-2 (JUNE 1999)    *EXTENDED RANGE OF INTEGRALS FROM 4    Sigma1

TO 5 UNITS ON EACH SIDE OF ENERGY     Sigma1

POINT TO ALLOW FOR LARGER VARIATION   Sigma1

IN THE LOCAL CROSS SECTION            Sigma1

*ASSUME ENDF/B-VI, NOT V, IF MISSING   Sigma1

MF=1, MT-451.                         Sigma1

VERSION 99-3 (OCTOBER 1999))*IMPROVED ERFC FUNCTION DEFINITION.    Sigma1

I THANK BOB MACFARLANE (LANL) FOR     Sigma1

SUPPLYING A MORE ACCURATE ERFC        Sigma1

FUNCTION.                             Sigma1

VERS. 2000-1 (FEBRUARY 2000)*CORRECTED LOW ENERGY INTERPOLATION    Sigma1

FOR NON-POSITIVE CROSS SECTIONS       Sigma1

*GENERAL IMPROVEMENTS BASED ON         Sigma1

USER FEEDBACK                         Sigma1

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Sigma1

VERS. 2004-1 (JAN. 2004)    *OPTIONALLY IGNORE UNRESOLVED REGION   Sigma1

*CORRECTED PROBLEM AT THE RESOLVED/    Sigma1

UNRESOLVED ENERGY BOUNDARY.           Sigma1

*CORRECTED HIGH ENERGY CONSTANT CROSS  Sigma1

SECTION EXTENSION.                    Sigma1

*TIGHTER CRITERIA FOR INITIAL ENERGY   Sigma1

POINT SPACING                         Sigma1

*TEMPERATURE DEPENDENT ENERGY POINT    Sigma1

SPACING.                              Sigma1

*ADDED NEW REICH-MOORE (LRF=7) TO      Sigma1

FILE2 TO ALLOW COPY TO FIND ANY       Sigma1

FOLLOWING UNRESOLVED PARAMETERS       Sigma1

VERS. 2005-1 (JUNE 2005)    *CORRECTED ERROR IN EHOT3 EQUIVALENCE  Sigma1

TO EHOT - THIS ONLY EFFECTS VERY BIG  Sigma1

OUTPUT FILES.                         Sigma1

VERS. 2007-1 (JAN. 2007)    *CHECKED AGAINST ALL ENDF/B-VII.       Sigma1

*INCREASED PAGE SIZE FROM 60,000       Sigma1

TO 360,000 ENERGY POINTS.             Sigma1

Sigma1

Acknowledgement 2004                                               Sigma1

--------------------                                               Sigma1

Currently almost all improvements to this code are based upon      Sigma1

feedback from code users who report problems. This feedback        Sigma1

benefits ALL users of this code, and ALL users are encouraged      Sigma1

to report problems.                                                Sigma1

Sigma1

Improvements on the 2004 version of this code based on user        Sigma1

feedback including,                                                Sigma1

1) Bret Beck  - reported a problem at the resolved/unresolved      Sigma1

energy boundary.                                   Sigma1

2) S. Ganesan - reported a problem for small temperature changes.  Sigma1

Sigma1

OWNED, MAINTAINED AND DISTRIBUTED BY                               Sigma1

------------------------------------                               Sigma1

THE NUCLEAR DATA SECTION                                           Sigma1

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Sigma1

P.O. BOX 100                                                       Sigma1

A-1400, VIENNA, AUSTRIA                                            Sigma1

EUROPE                                                             Sigma1

Sigma1

ORIGINALLY WRITTEN BY                                              Sigma1

------------------------------------                               Sigma1

DERMOTT E. CULLEN                                                  Sigma1

UNIVERSITY OF CALIFORNIA                                           Sigma1

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Sigma1

L-159                                                              Sigma1

P.O. BOX 808                                                       Sigma1

LIVERMORE, CA 94550                                                Sigma1

U.S.A.                                                             Sigma1

TELEPHONE  925-423-7359                                            Sigma1

E. MAIL    CULLEN1@LLNL.GOV                                        Sigma1

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Sigma1

Sigma1

AUTHORS MESSAGE                                                    Sigma1

---------------                                                    Sigma1

THE REPORT DESCRIBED ABOVE IS THE LATEST PUBLISHED DOCUMENTATION   Sigma1

FOR THIS PROGRAM. HOWEVER, THE COMMENTS BELOW SHOULD BE CONSIDERED Sigma1

THE LATEST DOCUMENTATION INCLUDING ALL RECENT IMPROVEMENTS. PLEASE Sigma1

READ ALL OF THESE COMMENTS BEFORE IMPLEMENTATION, PARTICULARLY     Sigma1

THE COMMENTS CONCERNING MACHINE DEPENDENT CODING.                  Sigma1

Sigma1

AT THE PRESENT TIME WE ARE ATTEMPTING TO DEVELOP A SET OF COMPUTER Sigma1

INDEPENDENT PROGRAMS THAT CAN EASILY BE IMPLEMENTED ON ANY ONE     Sigma1

OF A WIDE VARIETY OF COMPUTERS. IN ORDER TO ASSIST IN THIS PROJECT Sigma1

IT WOULD BE APPECIATED IF YOU WOULD NOTIFY THE AUTHOR OF ANY       Sigma1

COMPILER DIAGNOSTICS, OPERATING PROBLEMS OR SUGGESTIONS ON HOW TO  Sigma1

IMPROVE THIS PROGRAM. HOPEFULLY, IN THIS WAY FUTURE VERSIONS OF    Sigma1

THIS PROGRAM WILL BE COMPLETELY COMPATIBLE FOR USE ON YOUR         Sigma1

COMPUTER.                                                          Sigma1

Sigma1

PURPOSE                                                            Sigma1

-------                                                            Sigma1

THIS PROGRAM IS DESIGNED TO DOPPLER BROADEN NEUTRON INDUCED        Sigma1

CROSS SECTIONS. EACH SECTION OF CROSS SECTIONS (FILE 3) IS READ    Sigma1

FROM THE ENDF/B FORMAT. THE DATA IS DOPPLER BROADENED, THINNED     Sigma1

AND OUTPUT IN THE ENDF/B FORMAT.                                   Sigma1

Sigma1

IN THE FOLLOWING DISCUSSION FOR SIMPLICITY THE ENDF/B TERMINOLOGY  Sigma1

---ENDF/B TAPE---WILL BE USED. IN FACT THE ACTUAL MEDIUM MAY BE    Sigma1

TAPE, CARDS, DISK OR ANY OTHER MEDIUM.                             Sigma1

Sigma1

ENDF/B FORMAT                                                      Sigma1

-------------                                                      Sigma1

THIS PROGRAM ONLY USES THE ENDF/B BCD OR CARD IMAGE FORMAT (AS     Sigma1

OPPOSED TO THE BINARY FORMAT) AND CAN HANDLE DATA IN ANY VERSION   Sigma1

OF THE ENDF/B FORMAT (I.E., ENDF/B-I, II, III, IV OR V FORMAT).    Sigma1

Sigma1

IT IS ASSUMED THAT THE DATA IS CORRECTLY CODED IN THE ENDF/B       Sigma1

FORMAT AND NO ERROR CHECKING IS PERFORMED. IN PARTICULAR IT IS     Sigma1

ASSUMED THAT THE MAT, MF AND MT ON EACH CARD IS CORRECT. SEQUENCE  Sigma1

NUMBERS (COLUMNS 76-80) ARE IGNORED ON INPUT, BUT WILL BE          Sigma1

CORRECTLY OUTPUT ON ALL CARDS. THE FORMAT OF SECTION MF=1, MT=451  Sigma1

AND ALL SECTIONS OF MF=3 MUST BE CORRECT. THE PROGRAM COPIES ALL   Sigma1

OTHER SECTION OF DATA AS HOLLERITH AND AS SUCH IS INSENSITIVE TO   Sigma1

THE CORRECTNESS OR INCORRECTNESS OF ALL OTHER SECTIONS.            Sigma1

Sigma1

ALL CROSS SECTIONS THAT ARE USED BY THIS PROGRAM MUST BE TABULATED Sigma1

AND LINEARLY INTERPOLABLE IN ENERGY AND CROSS SECTION (ENDF/B      Sigma1

INTERPOLATION LAW 2). FILE 3 CROSS SECTIONS MAY BE MADE LINEARLY   Sigma1

INTERPOLABLE BY USING PROGRAM LINEAR (UCRL-50400, VOL.17, PART A). Sigma1

FILE 2 RESONANCE PARAMETERS MAY BE USED TO RECONSTRUCT ENERGY      Sigma1

DEPENDENT CROSS SECTIONS AND ADD IN FILE 3 BACKGROUND CROSS        Sigma1

SECTIONS TO DEFINE LINEARLY INTERPOLABLE CROSS SECTIONS BY USING   Sigma1

PROGRAM RECENT (UCRL-50400, VOL. 17, PART C). IF THIS PROGRAM      Sigma1

FINDS THAT THE FILE 3 CROSS SECTIONS ARE NOT LINEARLY INTERPOLABLE Sigma1

THIS PROGRAM WILL TERMINATE EXECUTION.                             Sigma1

Sigma1

UNRESOLVED RESONANCE REGION                                        Sigma1

---------------------------                                        Sigma1

IN THE UNRESOLVED RESONANCE REGION IT IS NOT POSSIBLE TO EXACTLY   Sigma1

DEFINE THE ENERGY DEPENDENCE OF THE CROSS SECTIONS. THE AVERAGE    Sigma1

WIDTHS AND SPACINGS GIVEN IN ENDF/B ARE ONLY ADEQUATE TO DEFINE    Sigma1

AVERAGE VALUES OF THE CROSS SECTIONS. THEREFORE ALL CROSS SECTIONS Sigma1

IN THE ENDF/B FORMAT FOR THE UNRESOLVED REGION ARE REALLY AVERAGE  Sigma1

VALUES WHICH CANNOT BE DOPPLER BROADENED USING THE SIGMA1 METHOD   Sigma1

(WHICH REQUIRES TABULATED, LINEARLY INTERPOLABLE, ENERGY DEPENDENT Sigma1

CROSS SECTIONS.                                                    Sigma1

Sigma1

THEREFORE,                                                         Sigma1

(1) ALL TABULATED POINTS WITHIN THE UNRESOLVED RESONANCE REGION    Sigma1

WILL BE COPIED, WITHOUT MODIFICATION OR BROADENING. ADOPTION OF    Sigma1

THIS CONVENTION WILL ALLOW SUBSEQUENT PROGRAMS TO PROPERLY DEFINE  Sigma1

SELF-SHIELDED, DOPPLER BROADENED CROSS SECTIONS IN THE UNRESOLVED  Sigma1

RESONANCE REGION.                                                  Sigma1

(2) CROSS SECTIONS WILL BE EXTENDED AS 1/V ABOVE THE UPPER ENERGY  Sigma1

LIMIT OF THE RESOLVED RESONANCE REGION AND BELOW THE LOWER ENERGY  Sigma1

LIMIT OF THE CONTINUUUM REGION (I.E. INTO THE UNRESOLVED           Sigma1

RESONANCE REGION). THIS CONVENTION WILL GUARANTEE A SMOOTH         Sigma1

BEHAVIOR CLOSE TO THE UNRESOLVED RESONANCE REGION BOUNDARIES.      Sigma1

Sigma1

OUTPUT FORMAT                                                      Sigma1

-------------                                                      Sigma1

IN THIS VERSION OF SIGMA1 ALL FILE 3 ENERGIES WILL BE OUTPUT IN    Sigma1

F (INSTEAD OF E) FORMAT IN ORDER TO ALLOW ENERGIES TO BE WRITTEN   Sigma1

WITH UP TO 9 DIGITS OF ACCURACY. IN PREVIOUS VERSIONS THIS WAS AN  Sigma1

OUTPUT OPTION. HOWEVER USE OF THIS OPTION TO COMPARE THE RESULTS   Sigma1

OF ENERGIES WRITTEN IN THE NORMAL ENDF/B CONVENTION OF 6 DIGITS    Sigma1

TO THE 9 DIGIT OUTPUT FROM THIS PROGRAM DEMONSTRATED THAT FAILURE  Sigma1

TO USE THE 9 DIGIT OUTPUT CAN LEAD TO LARGE ERRORS IN THE DATA     Sigma1

JUST DUE TO TRANSLATION OF THE ENERGIES TO THE ENDF/B FORMAT.      Sigma1

Sigma1

CONTENTS OF OUTPUT                                                 Sigma1

------------------                                                 Sigma1

ENTIRE EVALUATIONS ARE OUTPUT, NOT JUST THE BROADENED FILE 3       Sigma1

CROSS SECTIONS, E.G. ANGULAR AND ENERGY DISTRIBUTIONS ARE ALSO     Sigma1

INCLUDED.                                                          Sigma1

Sigma1

DOCUMENTATION                                                      Sigma1

-------------                                                      Sigma1

THE FACT THAT THIS PROGRAM HAS OPERATED ON THE DATA IS DOCUMENTED  Sigma1

BY THE ADDITION OF THREE COMMENTS CARDS AT THE END OF EACH         Sigma1

HOLLERITH SECTION IN THE FORM                                      Sigma1

Sigma1

***************** PROGRAM SIGMA1 (2007-1) ***************          Sigma1

DATA DOPPLER BROADENED TO 300.0   KELVIN AND                       Sigma1

DATA THINNED TO WITHIN AN ACCURACY OF  0.1 PER-CENT                Sigma1

Sigma1

THE ORDER OF ALL SIMILAR COMMENTS (FROM LINEAR,RECENT AND GROUPY)  Sigma1

REPRESENTS A COMPLETE HISTORY OF ALL OPERATIONS PERFORMED ON       Sigma1

THE DATA.                                                          Sigma1

Sigma1

THESE COMMENT CARDS ARE ONLY ADDED TO EXISTING HOLLERITH SECTIONS, Sigma1

I.E., THIS PROGRAM WILL NOT CREATE A HOLLERITH SECTION. THE FORMAT Sigma1

OF THE HOLLERITH SECTION IN ENDF/B-V DIFFERS FROM THE THAT OF      Sigma1

EARLIER VERSIONS OF ENDF/B. BY READING AN EXISTING MF=1, MT=451    Sigma1

IT IS POSSIBLE FOR THIS PROGRAM TO DETERMINE WHICH VERSION OF      Sigma1

THE ENDF/B FORMAT THE DATA IS IN. WITHOUT HAVING A SECTION OF      Sigma1

MF=1, MT=451 PRESENT IT IS IMPOSSIBLE FOR THIS PROGRAM TO          Sigma1

DETERMINE WHICH VERSION OF THE ENDF/B FORMAT THE DATA IS IN, AND   Sigma1

AS SUCH IT IS IMPOSSIBLE FOR THE PROGRAM TO DETERMINE WHAT FORMAT  Sigma1

SHOULD BE USED TO CREATE A HOLLERITH SECTION.                      Sigma1

Sigma1

REACTION INDEX                                                     Sigma1

--------------                                                     Sigma1

THIS PROGRAM DOES NOT USE THE REACTION INDEX WHICH IS GIVEN IN     Sigma1

SECTION MF=1, MT=451 OF EACH EVALUATION.                           Sigma1

Sigma1

THIS PROGRAM DOES NOT UPDATE THE REACTION INDEX IN MF=1, MT=451.   Sigma1

THIS CONVENTION HAS BEEN ADOPTED BECAUSE MOST USERS DO NOT         Sigma1

REQUIRE A CORRECT REACTION INDEX FOR THEIR APPLICATIONS AND IT WAS Sigma1

NOT CONSIDERED WORTHWHILE TO INCLUDE THE OVERHEAD OF CONSTRUCTING  Sigma1

A CORRECT REACTION INDEX IN THIS PROGRAM. HOWEVER, IF YOU REQUIRE  Sigma1

A REACTION INDEX FOR YOUR APPLICATIONS, AFTER RUNNING THIS PROGRAM Sigma1

YOU MAY USE PROGRAM DICTIN TO CREATE A CORRECT REACTION INDEX.     Sigma1

Sigma1

SECTION SIZE                                                       Sigma1

------------                                                       Sigma1

SINCE THIS PROGRAM USES A LOGICAL PAGING SYSTEM THERE IS NO LIMIT  Sigma1

TO THE NUMBER OF POINTS IN ANY SECTION, E.G., THE TOTAL CROSS      Sigma1

SECTION MAY BE REPRESENTED BY 200,000 DATA POINTS.                 Sigma1

Sigma1

SELECTION OF DATA                                                  Sigma1

-----------------                                                  Sigma1

THE PROGRAM SELECTS MATERIALS TO BE BROADENED BASED EITHER ON      Sigma1

MAT (ENDF/B MAT NO.) OR ZA. THE PROGRAM ALLOWS UP TO 100 MAT OR    Sigma1

ZA RANGES TO BE SPECIFIED. THE PROGRAM WILL ASSUME THAT THE        Sigma1

ENDF/B TAPE IS IN EITHER MAT OR ZA ORDER, WHICHEVER CRITERIA IS    Sigma1

USED TO SELECT MATERIALS, AND WILL TERMINATE WHEN A MAT OR ZA      Sigma1

IS FOUND THAT IS ABOVE THE RANGE OF ALL REQUESTS.                  Sigma1

Sigma1

ENERGY GRID OF BROADENED DATA                                      Sigma1

-----------------------------                                      Sigma1

THE ENERGY GRID FOR THE DOPPLER BROADENED CROSS SECTIONS IS        Sigma1

SELECTED TO INSURE THAT THE BROADENED DATA IS LINEAR-LINEAR        Sigma1

INTERPOLABLE. AS SUCH THE ENERGY GRID FOR THE BROADENED DATA       Sigma1

MAY NOT BE THE SAME AS THE ENERGY GRID FOR THE ORIGINAL            Sigma1

UNBROADENED DATA. GENERALLY AFTER BROADENING THERE WILL BE         Sigma1

FEWER DATA POINTS IN THE RESONANCE REGION, BUT AT LOW ENERGY       Sigma1

THERE MAY BE MORE POINTS, DUE TO THE 1/V LOW ENERGY EFFECT         Sigma1

CREATED BY DOPPLER BROADENING.                                     Sigma1

Sigma1

EFFECTIVE TEMERATURE INCREASE                                      Sigma1

-----------------------------                                      Sigma1

IF THE ORIGINAL DATA IS NOT AT ZERO KELVIN THE PROGRAM WILL        Sigma1

BROADEN THE DATA BY THE EFFECTIVE TEMPERATURE DIFFENCE TO THE      Sigma1

FINAL TEMPERATURE. IF THE DATA IS ALREADY AT A TEMPERATURE THAT    Sigma1

IS HIGHER THAN THE FINAL TEMPERATURE DOPPLER BROADENING IS         Sigma1

NATURALLY NOT PERFORMED AND THE TEMPERATURE IN THE SECTION IS LEFT Sigma1

AT ITS ORIGINAL VALUE.                                             Sigma1

Sigma1

MULTIPLE FINAL TEMPERATURES                                        Sigma1

---------------------------                                        Sigma1

THE PRESENT VERSION ONLY DOPPLER BROADENS TO ONE FINAL TEMPERATURE Sigma1

(IF THERE IS SUFFICIENT INTEREST EXPRESSED BY USERS FUTURE         Sigma1

VERSION MAY BROADEN TO MULTIPLE TEMPERATURES. PLEASE               Sigma1

CONTACT THE AUTHOR IF YOU ARE INTERESTED IN A MULTIPLE             Sigma1

TEMPERATURE OPTION).                                               Sigma1

Sigma1

PROGRAM OPERATION                                                  Sigma1

-----------------                                                  Sigma1

EACH SECTION OF FILE 3 DATA IS CONSIDERED SEPERATELY. THE DATA     Sigma1

IS READ AND DOPPLER BROADENED A PAGE AT A TIME (ONE PAGE IS        Sigma1

60000 DATA POINTS). UP TO THREE PAGES OF DATA MAY BE IN THE CORE   Sigma1

AT ANY GIVEN TIME, THE PAGE BEING BROADENED, THE PAGE BELOW IT     Sigma1

IN ENERGY AND THE PAGE ABOVE IT IN ENERGY. AFTER A PAGE HAS BEEN   Sigma1

BROADENED IT IS THINNED, IF THE ENTIRE SECTION CONTAINS ONLY       Sigma1

ONE PAGE OR LESS, IT WILL STILL BE CORE RESIDENT AND WILL BE       Sigma1

WRITTEN DIRECTLY FROM CORE TO THE OUTPUT TAPE. IF THE BROADENED,   Sigma1

THINNED SECTION IS LARGER THAN A PAGE, AFTER A PAGE HAS BEEN       Sigma1

BROADENED AND THINNED IT IS WRITTEN TO A SCRATCH FILE. AFTER THE   Sigma1

ENTIRE SECTION HAS BEEN BROADENED AND THINNED THE DATA IS READ     Sigma1

FROM SCRATCH TO CORE, ONE PAGE AT A TIME, THE OUTPUT TO THE OUTPUT Sigma1

TAPE.                                                              Sigma1

Sigma1

ALLOWABLE ERROR                                                    Sigma1

---------------                                                    Sigma1

AFTER DOPPLER BROADENING THE CROSS SECTION IN THE RESONANCE REGION Sigma1

WILL GENERALLY BE MUCH SMOOTHER THAN THE UNBROADENED DATA AND CAN  Sigma1

BE REPRESENTED TO THE SAME ACCURACY BY A SMALLER NUMBER OF ENERGY  Sigma1

POINTS. THEREFORE AFTER DOPPLER BROADENING THE DATA CAN BE THINNED Sigma1

WITH ESSENTIALLY NO LOSE OF INFORMATION.                           Sigma1

Sigma1

THE ALLOWABLE ERROR MAY BE ENERGY INDEPENDENT (CONSTANT) OR ENERGY Sigma1

DEPENDENT. THE ALLOWABLE ERROR IS DESCRIBED BY A TABULATED         Sigma1

FUNCTION OF UP TO 20 (ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION Sigma1

BETWEEN TABULATED POINTS. IF ONLY ONE TABULATED POINT IS GIVEN THE Sigma1

ERROR WILL BE CONSIDERED CONSTANT OVER THE ENTIRE ENERGY RANGE.    Sigma1

WITH THIS ENERGY DEPENDENT ERROR ONE MAY OPTIMIZE THE OUTPUT FOR   Sigma1

ANY GIVEN APPLICATION BY USING A SMALL ERROR IN THE ENERGY RANGE   Sigma1

OF INTEREST AND A LESS STRINGENT ERROR IN OTHER ENERGY RANGES.     Sigma1

Sigma1

INPUT FILES                                                        Sigma1

-----------                                                        Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

2  INPUT CARDS (BCD - 80 CHARACTERS/RECORD)                     Sigma1

10  ORIGINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)            Sigma1

Sigma1

OUTPUT FILES                                                       Sigma1

------------                                                       Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

3  OUTPUT REPORT (BCD - 120 CHARACTERS/RECORD)                  Sigma1

11  FINAL ENDF/B DATA (BCD - 80 CHARACTERS/RECORD)               Sigma1

Sigma1

SCRATCH FILES                                                      Sigma1

-------------                                                      Sigma1

UNIT  DESCRIPTION                                                  Sigma1

----  -----------                                                  Sigma1

12  SCRATCH FILE FOR BROADENED DATA                              Sigma1

(BINARY - 180000 WORDS/RECORD - DOUBLE PRECISION/            Sigma1

42000 WORDS/RECORD - SINLGE PRECISION)            Sigma1

Sigma1

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILEIO)               Sigma1

----------------------------------------------------               Sigma1

UNIT  FILE NAME                                                    Sigma1

----  ----------                                                   Sigma1

2   SIGMA1.INP                                                   Sigma1

3   SIGMA1.LST                                                   Sigma1

10   ENDFB.IN                                                     Sigma1

11   ENDFB.OUT                                                    Sigma1

12   (SCRATCH)                                                    Sigma1

Sigma1

INPUT CARDS                                                        Sigma1

-----------                                                        Sigma1

CARD  COLS.  DESCRIPTION                                           Sigma1

----  -----  -----------                                           Sigma1

1   1-11  SELECTION CRITERIA (0=MAT, 1=ZA)                      Sigma1

12-22  MONITOR MODE SELECTOR                                 Sigma1

= 0 - NORMAL OPERATION                                Sigma1

= 1 - MONITOR PROGRESS OF DOPPLER BROADENING OF DATA. Sigma1

EACH TIME A PAGE OF DATA POINTS IS WRITTEN TO   Sigma1

THE SCRATCH FILE PRINT OUT THE TOTAL NUMBER OF  Sigma1

POINTS ON SCRATCH AND THE LOWER AND UPPER       Sigma1

ENERGY LIMITS OF THE PAGE (THIS OPTION MAY BE   Sigma1

USED IN ORDER TO MONITOR THE EXECUTION SPEED    Sigma1

OF LONG RUNNING JOBS).                          Sigma1

23-33  KELVIN TEMPERATURE                                    Sigma1

34-44  MINIMUM CROSS SECTION OF INTEREST                     Sigma1

(DEFAULT VALUE = 1.0E-10 BARNS).                      Sigma1

45-55  NEGATIVE CROSS SECTION TREATMENT                      Sigma1

= 0 - O.K. - NO CHANGE                                Sigma1

= 1 - SET = 0                                         Sigma1

56-66  UNRESOLVED RESONANCE REGION TREATMENT                 Sigma1

= 0 - COPY (NO BROADENING)                            Sigma1

= 1 - IGNORE (BROADEN)                                Sigma1

2   1-60  ENDF/B INPUT DATA FILENAME                            Sigma1

(STANDARD OPTION = ENDFB.IN)                          Sigma1

3   1-60  ENDF/B OUTPUT DATA FILENAME                           Sigma1

(STANDARD OPTION = ENDFB.OUT)                         Sigma1

4-N   1-11  LOWER MAT OR ZA LIMIT                                 Sigma1

12-22  UPPER MAT OR ZA LIMIT                                 Sigma1

UP TO 100 MAT OR ZA RANGES MAY BE SPECIFIED, ONE      Sigma1

RANGE PER CARD. THE LIST OF RANGES IS TERMINATED BY   Sigma1

A BLANK CARD. IF THE UPPER LIMIT IS LESS THAN THE     Sigma1

LOWER LIMIT THE UPPER LIMIT WILL BE SET EQUAL TO THE  Sigma1

LOWER LIMIT. IF THE FIRST REQUEST CARD IS BLANK IT    Sigma1

WILL TERMINATE THE LIST OF REQUESTS AND CAUSE ALL     Sigma1

DATA TO BE RETRIEVED (SEE EXAMPLE INPUT).             Sigma1

VARY  1-11  ENERGY FOR ERROR LAW                                  Sigma1

12-22  ERROR FOR ERROR LAW                                   Sigma1

THE ACCEPTABLE LINEARIZING ERROR CAN BE GIVEN AS AN   Sigma1

ENERGY DEPENDENT FUNCTION SPECIFIED BY UP TO 20       Sigma1

(ENERGY,ERROR) PAIRS AND LINEAR INTERPOLATION         Sigma1

TABULATE POINTS. ENERGIES MUST BE IN ASCENDING ORDER. Sigma1

THE ERROR LAW IS TERMINATED BY A BLANK CARD. IF THE   Sigma1

FIRST ERROR LAW CARD IS BLANK IT WILL TERMINATE THE   Sigma1

ERROR LAW AND THE ERROR WILL BE TREATED AS ENERGY     Sigma1

INDEPENDENT, EQUAL TO ZERO, WHICH INDICATES THAT THE  Sigma1

BROADENED DATA SHOULD NOT BE THINNED.                 Sigma1

Sigma1

EXAMPLE INPUT NO. 1                                                Sigma1

-------------------                                                Sigma1

BROADEN ALL URANIUM ISOTOPES AND THORIUM-232 TO 300 KELVIN. FROM   Sigma1

0 TO 100 EV THIN OUTPUT DATA TO 0.1 PER-CENT ACCURACY. FROM 100 EV Sigma1

TO 1 KEV VARY THE ERROR BETWEEN 0.1 AND 1 PER-CENT. ABOVE 1 KEV    Sigma1

USE 1 PER-CENT ACCURACY.                                           Sigma1

Sigma1

EXPLICITLY SPECIFY THE STANDARD FILENAMES.                         Sigma1

Sigma1

THE FOLLOWING 11 CARDS ARE REQUIRED                                Sigma1

Sigma1

1          0 3.00000+ 2                                       Sigma1

ENDFB.IN                                                               Sigma1

ENDFB.OUT                                                              Sigma1

92000      92999                                                  Sigma1

90232               (UPPER LIMIT WILL AUTOMATICALLY BE DEFINED)   Sigma1

(BLANK CARD INDICATES END OF REQUEST LIST)    Sigma1

0.00000+ 0 1.00000-03                                                  Sigma1

1.00000+ 2 1.00000-03                                                  Sigma1

1.00000+ 3 1.00000-02                                                  Sigma1

1.00000+ 9 1.00000-02                                                  Sigma1

(BLANK CARD INDICATES END OF ERROR LAW)       Sigma1

Sigma1

EXAMPLE INPUT NO. 2                                                Sigma1

-------------------                                                Sigma1

BROADEN ALL DATA TO 300 KELVIN AND DO NOT THIN THE BROADEN DATA.   Sigma1

ALL OF THE STANDARD OPTION MAY BE INVOKED MERELY BY SPECIFYING     Sigma1

THE KELVIN TEMPERATURE ON THE FIRST CARD. ALL OTHER FIELDS MAY     Sigma1

BE LEFT BLANK.                                                     Sigma1

Sigma1

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Sigma1

THEN USE STANDARD FILENAMES.                                       Sigma1

Sigma1

THE FOLLOWING 5 CARDS ARE REQUIRED                                 Sigma1

Sigma1

3.00000+ 2                                       Sigma1

(USE STANDARD FILENAME = ENDFB.IN)               Sigma1

(USE STANDARD FILENAME = ENDFB.OUT)              Sigma1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Sigma1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)       Sigma1

Sigma1

EXAMPLE INPUT NO. 3                                                Sigma1

-------------------                                                Sigma1

THE SAME AS ABOVE, ONLY DEFINE THE MINIMUM CROSS SECTION OF        Sigma1

INTEREST TO BE 1.0E-30 BARNS (INSTEAD OF THE DEFAULT VALUE OF      Sigma1

1.0E-10).                                                          Sigma1

Sigma1

READ ENDF/B DATA FROM \ENDFB6\RECENT\ZA092238 AND WRITE ENDF/B     Sigma1

DATA TO \ENDFB\SIGMA1\ZA092238                                     Sigma1

Sigma1

THE FOLLOWING 5 CARDS ARE REQUIRED                                 Sigma1

Sigma1

3.00000+ 2 1.00000-30                            Sigma1

\ENDFB6\RECENT\ZA092238                                                Sigma1

\ENDFB6\SIGMA1\ZA092238                                                Sigma1

(RETRIEVE ALL DATA, TERMINATE REQUEST LIST)      Sigma1

(0.0 ALLOWABLE ERROR, TERMINATE ERROR LAW)       Sigma1

Sigma1

======================================================================= Sigma1