======================================================================= Virgin

Virgin

PROGRAM VIRGIN                                                     Virgin

VERSION 76-1 (NOVEMBER 1976)                                       Virgin

VERSION 84-1 (JUNE 1984)   *DOUBLE PRECISION ENERGY                Virgin

VERSION 86-1 (JANUARY 1986)*FORTRAN-77/H VERSION                   Virgin

VERSION 88-1 (JULY 1988)   *OPTION...INTERNALLY DEFINE ALL I/O     Virgin

FILE NAMES (SEE, SUBROUTINE FILEIO     Virgin

FOR DETAILS).                          Virgin

*IMPROVED BASED ON USER COMMENTS.       Virgin

VERSION 89-1 (JANUARY 1989)*PSYCHOANALYZED BY PROGRAM FREUD TO     Virgin

INSURE PROGRAM WILL NOT DO ANYTHING    Virgin

CRAZY.                                 Virgin

*UPDATED TO USE NEW PROGRAM CONVERT     Virgin

KEYWORDS.                              Virgin

*ADDED LIVERMORE CIVIC COMPILER         Virgin

CONVENTIONS.                           Virgin

VERSION 92-1 (JANUARY 1992)*COMPLETE RE-WRITE                      Virgin

*OUTPUT IN PLOTTAB FORMAT               Virgin

*UP TO 2000 THICKNESSES                 Virgin

*INCREASED INCORE PAGE SIZE TO 6000     Virgin

CROSS SECTION POINTS                   Virgin

*ADDED PHOTON CALCULATIONS              Virgin

*ADDED BLACKBODY SPECTRUM               Virgin

*ADDED MULTIPLE LAYERS                  Virgin

*ADDED SPATIALLY DEPENDENT DENSITY      Virgin

*ADDED FORTRAN SAVE OPTION              Virgin

*COMPLETELY CONSISTENT I/O ROUTINES -   Virgin

TO MINIMIZE COMPUTER DEPENDENCE.       Virgin

VERSION 92-2 (MAY 1992)    *CORRECTED TO HANDLE MULTIGROUP CROSS   Virgin

SECTIONS AS INPUT IN ENDF/B FORMAT.    Virgin

VERSION 96-1 (JANUARY 1996) *COMPLETE RE-WRITE                     Virgin

*IMPROVED COMPUTER INDEPENDENCE        Virgin

*ALL DOUBLE PRECISION                  Virgin

*ON SCREEN OUTPUT                      Virgin

*UNIFORM TREATMENT OF ENDF/B I/O       Virgin

*IMPROVED OUTPUT PRECISION             Virgin

*DEFINED SCRATCH FILE NAMES            Virgin

VERSION 99-1 (MARCH 1999)   *CORRECTED CHARACTER TO FLOATING       Virgin

POINT READ FOR MORE DIGITS            Virgin

*UPDATED TEST FOR ENDF/B FORMAT        Virgin

VERSION BASED ON RECENT FORMAT CHANGE Virgin

*GENERAL IMPROVEMENTS BASED ON         Virgin

USER FEEDBACK                         Virgin

VERS. 2000-1 (FEBRUARY 2000)*GENERAL IMPROVEMENTS BASED ON         Virgin

USER FEEDBACK                         Virgin

VERS. 2002-1 (MAY 2002)     *OPTIONAL INPUT PARAMETERS             Virgin

VERS. 2004-1 (MARCH 2004)   *ADDED INCLUDE FOR COMMON              Virgin

*UP TO 2000 THICKNESSES                Virgin

*INCREASED INCORE PAGE SIZE TO 60,000  Virgin

VERS. 2007-1 (JAN.  2007)   *CHECKED AGAINST ALL ENDF/B-VII.       Virgin

*INCREASED INCORE PAGE SIZE TO         Virgin

240,000 FROM 60,000.                  Virgin

Virgin

OWNED, MAINTAINED AND DISTRIBUTED BY                               Virgin

------------------------------------                               Virgin

THE NUCLEAR DATA SECTION                                           Virgin

INTERNATIONAL ATOMIC ENERGY AGENCY                                 Virgin

P.O. BOX 100                                                       Virgin

A-1400, VIENNA, AUSTRIA                                            Virgin

EUROPE                                                             Virgin

Virgin

ORIGINALLY WRITTEN BY                                              Virgin

------------------------------------                               Virgin

DERMOTT E. CULLEN                                                  Virgin

UNIVERSITY OF CALIFORNIA                                           Virgin

LAWRENCE LIVERMORE NATIONAL LABORATORY                             Virgin

L-159                                                              Virgin

P.O. BOX 808                                                       Virgin

LIVERMORE, CA 94550                                                Virgin

U.S.A.                                                             Virgin

TELEPHONE  925-423-7359                                            Virgin

E. MAIL    CULLEN1@LLNL.GOV                                        Virgin

WEBSITE    HTTP://WWW.LLNL.GOV/CULLEN1                             Virgin

Virgin

PURPOSE                                                            Virgin

-------                                                            Virgin

THIS PROGRAM IS DESIGNED TO CALCULATE UNCOLLIDED (I.E. VIRGIN)     Virgin

FLUX AND REACTIONS DUE TO TRANSMISSION OF A MONODIRECTIONAL        Virgin

BEAM OF NEUTRONS THROUGH ANY THICKNESS OF MATERIAL. IN ORDER       Virgin

TO SIMULATE AN EXPERIMENTAL MEASUREMENT THE RESULTS ARE GIVEN      Virgin

AS INTEGRALS OVER ENERGY TALLY GROUPS (AS OPPOSED TO POINTWISE     Virgin

IN ENERGY). BY TAKING THE RATIO OF REACTIONS TO FLUX IN EACH       Virgin

GROUP AN EQUIVALENT SPATIALLY DEPENDENT GROUP AVERAGED CROSS       Virgin

SECTION IS CALCULATED BY THE PROGRAM.                              Virgin

Virgin

EVALUATED DATA                                                     Virgin

--------------                                                     Virgin

THE EVALUATED DATA MUST BE IN THE ENDF/B FORMAT. HOWEVER IT        Virgin

MUST BE LINEAR-LINEAR INTERPOLABLE IN ENERGY-CROSS SECTION         Virgin

BETWEEN TABULATED POINTS. SINCE ONLY CROSS SECTIONS (FILE 3 OR 23) Virgin

ARE USED, THIS PROGRAM WILL WORK ON ANY VERSION OF ENDF/B          Virgin

(I.E. ENDF/B-I, II, III, IV, V OR VI).                             Virgin

Virgin

RELATED COMPUTER CODES                                             Virgin

----------------------                                             Virgin

IN ORDER TO CONVERT ENDF/B DATA TO THE FORM REQUIRED BY THIS CODE  Virgin

THE FOLLOWING COMPUTER CODES MAY BE USED,                          Virgin

Virgin

LINEAR  - CONVERT FROM GENERAL ENDF/B INTERPOLATION TO LINEAR-     Virgin

LINEAR INTERPOLATION.                                    Virgin

RECENT  - ADD THE RESONANCE CONTRIBUTION TO TABULATED BACKGROUND   Virgin

CROSS SECTIONS TO OBTAIN LINEAR-LINEAR INTERPOLABLE      Virgin

RESULTS.                                                 Virgin

SIGMA1  - DOPPLER BROADEN CROSS SECTION TO OBTAIN LINEAR-LINEAR    Virgin

INTERPOLABLE RESULTS.                                    Virgin

MIXER   - MIX INDIVIDUAL MATERIALS TOGETHER TO DEFINE COMPOSITE    Virgin

MIXTURES, E.G., COMBINE MATERIALS TO DEFINE STAINLESS    Virgin

STELL.                                                   Virgin

Virgin

IN ORDER TO PLOT THE OUTPUT RESULTS OF THIS CODE USE PROGRAM       Virgin

PLOTTAB.                                                           Virgin

Virgin

COPIES OF ANY OR ALL OF THESE CODES MAY BE OBTAINED FROM D.E.      Virgin

CULLEN AT THE ABOVE ADDRESS.                                       Virgin

Virgin

OUTPUT FORMAT                                                      Virgin

-------------                                                      Virgin

FOR ALL VERSIONS OF THIS PROGRAM PRIOR TO VERSION 92-1 OUTPUT WAS  Virgin

IN TABULAR FORM.                                                   Virgin

Virgin

FOR VERSION 92-1 AND LATER VERSIONS OF THIS CODE ALL OUTPUT IS IN  Virgin

THE PROGRAM PLOTTAB FORMAT TO ALLOW RESULTS TO BE EASILY PLOTTED.  Virgin

FOR A COPY OF PROGRAM PLOTTAB CONTACT D.E. CULLEN AT THE ABOVE     Virgin

ADDRESS.                                                           Virgin

Virgin

TALLY GROUPS                                                       Virgin

------------                                                       Virgin

THE TALLY GROUP STRUCTURE MAY BE ANY SET OF MONONTONICALLY         Virgin

INCREASING ENERGY BOUNDARIES. THERE MAY BE UP TO 2000 TALLY        Virgin

GROUPS. BY USING THE INPUT PARAMETERS THE USER MAY SPECIFY ANY     Virgin

ARBITRARY TALLY GROUP STRUCTURE OR SELECT ONE OF THE FOLLOWING     Virgin

BUILT-IN GROUP STRUCTURES.                                         Virgin

Virgin

(1) TART 175 GROUPS (INPUT  0)                                     Virgin

(2) ORNL  50 GROUPS (INPUT -1)                                     Virgin

(3) ORNL 126 GROUPS (INPUT -2)                                     Virgin

(4) ORNL 171 GROUPS (INPUT -3)                                     Virgin

(5) SAND-II 620 GROUPS...UP TO 18 MEV (INPUT -4)                   Virgin

(6) SAND-II 640 GROUPS...UP TO 20 MEV (INPUT -5)                   Virgin

(7) WIMS 69 GROUPS (INPUT -6)                                      Virgin

(8) GAM-I 68 GROUPS (INPUT -7)                                     Virgin

(9) GAM-II 99 GROUPS (INPUT -8)                                    Virgin

(10) MUFT 54 GROUPS (INPUT -9)                                      Virgin

(11) ABBN 28 GROUPS (INPUT -10)                                     Virgin

Virgin

INCIDENT SPECTRUM                                                  Virgin

-----------------                                                  Virgin

THE INCIDENT SPECTRUM MAY BE ANY TABULATED FUNCTION THAT IS        Virgin

GIVEN BY A SET OF POINTS THAT IS MONOTONICALLY INCREASING IN       Virgin

ENERGY AND LINEAR-LINEAR INTERPOLABLE IN ENERGY-SPECTRUM           Virgin

BETWEEN TABULATED POINTS. THERE IS NO LIMIT TO THE NUMBER OF       Virgin

POINTS USED TO DESCRIBE THE SPECTRUM. THERE ARE FIVE BUILT-IN      Virgin

OPTIONS FOR THE SPECTRUM.                                          Virgin

Virgin

(1) CONSTANT...ENERGY INDEPENDENT (INPUT 0)                        Virgin

(2) 1/E (INPUT 1)                                                  Virgin

(3) BLACKBODY - PHOTON SPECTRUM                                    Virgin

(4) BLACKBODY - ENERGY SPECTRUM (E TIMES THE PHOTON SPECTRUM)      Virgin

(5) TRANSMITTED SPECTRUM FROM PREVIOUS CASE                        Virgin

Virgin

NORMALIZATION OF SPECTRUM                                          Virgin

-------------------------                                          Virgin

ANY INCIDENT SPECTRUM, EITHER READ AS INPUT OR ONE OF THE          Virgin

BUILT-IN SPECTRA, WILL BE NORMALIZED TO UNITY WHEN INTEGRATED      Virgin

OVER THEIR ENTIRE ENERGY RANGE.                                    Virgin

Virgin

TRANSMITTED SPECTRA WILL NOT BE RE-NORMALIZED, SINCE IT ALREADY    Virgin

INCLUDES THE NORMALIZATION OF THE INCIDENT SPECTRUM.               Virgin

Virgin

NOTE, INCIDENT SPECTRA IS NORMALIZED TO UNITY OVER THEIR ENTIRE    Virgin

ENERGY RANGE - NOT OVER THE ENERGY RANGE OF THE GROUPS. IF THE     Virgin

ENERGY RANGE OF THE GROUPS IS LESS THAN THAT OF THE SPECTRUM       Virgin

ONLY THAT PORTION OF THE SPECTRUM WILL BE USED AND THIS WILL       Virgin

NOT BE RE-NORMALIZED TO UNITY.                                     Virgin

Virgin

COMPOSITION OF A LAYER                                             Virgin

----------------------                                             Virgin

YOU MAY RUN PROBLEMS INVOLVING                                     Virgin

1) A LAYER OF UNIFORM DENSITY - DENSITY FOR ATTENUATION IS THAT    Virgin

OF THE TOTAL. DENSITY FOR REACTIONS IS THAT OF THE REACTION.    Virgin

2) A LAYER OF UNIFORM DENSITY - DENSITY IS THE SUM OF THE TOTAL    Virgin

AND REACTION DENSITIES - THE SUM OF THE CROSS SECTIONS IS       Virgin

USED FOR ATTENUATION AND REACTIONS.                             Virgin

3) A LAYER OF VARYING DENSITY BASED ON A UNIFORM TOTAL DENSITY     Virgin

PLUS A VARIATION BETWEEN 0 AND A MAXIMUM BASED ON THE           Virgin

REACTION DENSITY - 0 AT 0 THICKNESS AND MAXIMUM AT MAXIMUM      Virgin

THICKNESS. IN THIS CASE THE AVERAGE REACTION DENSITY IS EQUAL   Virgin

TO THE INPUT REACTION DENSITY. THE VARIATION IN REACTION        Virgin

DENSITY CAN BE LINEAR, SQUARE OR CUBIC.                         Virgin

4) A LAYER OF VARYING DENSITY BASED ON A TOTAL DENSITY WHICH       Virgin

VARYING FROM MAXIMUM AT 0 THICKNESS TO 0 AT MAXIMUM THICKNESS   Virgin

PLUS A REACTION DENSITY WHICH VARIES FROM 0 AT 0 THICKNESS      Virgin

TO MAXIMUM AT MAXIMUM THICKNESS. IN THIS CASE THE AVERAGE       Virgin

DENSITY OF THE TOTAL AND REACTION WILL BOTH BE EQUAL TO THE     Virgin

INPUT TOTAL AND REACTION DENSITIES. THE VARIATION IN TOTAL      Virgin

AND REACTION DENSITY CAN BE LINEAR, SQUARE OR CUBIC.            Virgin

Virgin

IN THE FIRST CASE THE TWO REQUESTED CROSS SECTIONS ARE CONSIDERED  Virgin

TO BE INDEPENDENT - THE TOTAL CROSS SECTION IS USED TO CALCULATE   Virgin

ATTENUATION AND THE REACTION CROSS SECTION IS USED TO CALCULATE    Virgin

REACTIONS, E.G., TRANSMISSION THROUGH NATURAL URANIUM (THE TOTAL   Virgin

CROSS SECTION SHOULD BE THAT OF NATURAL URANIUM) AND REACTIONS     Virgin

IN A U-235 DETECTOR (THE REACTION CROSS SECTION SHOULD BE THAT OF  Virgin

U-235).                                                            Virgin

Virgin

IN THE OTHER THREE CASES THE TWO REQUESTED CROSS SECTIONS ARE      Virgin

TREATED AS TWO CONSTITUENTS OF A MIXTURE OF TWO MATERIALS AND      Virgin

THE TWO CROSS SECTIONS ARE USED BOTH TO DEFINE A TOTAL CROSS       Virgin

SECTION FOR ATTENUATION AND A REACTION CROSS SECTION TO DEFINE     Virgin

REACTIONS. IN THESE CASES THE MIXTURE WILL VARY CONTINUOUSLY,      Virgin

E.G., IN CASE 4) HALF WAY THROUGH THE LAYER THE COMPOSITION WILL   Virgin

BE 1/2 THE MATERIAL DEFINED BY THE TOTAL AND 1/2 THE MATERIAL      Virgin

BASED ON THE REACTION. IN THESE CASES RATHER THAN THINKING OF      Virgin

THE TWO CROSS SECTIONS AS A TOTAL AND REACTION CROSS SECTION,      Virgin

IT IS BETTER TO THINK OF THEM AS THE TOTAL CROSS SECTIONS FOR      Virgin

MATERIALS A AND B AND THE CALCULATED REACTIONS WILL BE BASED       Virgin

ON THESE TWO TOTAL CROSS SECTIONS.                                 Virgin

Virgin

MULTIPLE LAYERS                                                    Virgin

---------------                                                    Virgin

THIS CODE MAY BE USED TO RUN EITHER A NUMBER OF INDEPENDENT        Virgin

PROBLEMS, EACH INVOLVING TRANSMISSION THROUGH A SINGLE LAYER OF    Virgin

MATERIAL, OR TRANSMISSION THROUGH A NUMBER OF LAYERS ONE AFTER     Virgin

THE OTHER.                                                         Virgin

Virgin

IN THE CASE OF MULTIPLE LAYERS, ONE LAYER AFTER ANOTHER, THE       Virgin

TRANSMITTED ENERGY DEPENDENT SPECTRUM IS USED AS THE INCIDENT      Virgin

SPECTRUM FOR THE NEXT LAYER. THERE IS NO LIMIT TO THE NUMBER       Virgin

OF LAYERS WHICH MAY BE USED - EACH LAYER IS TREATED AS A           Virgin

COMPLETELY INDEPENDENT PROBLEM WITH A DEFINED INCIDENT SOURCE,     Virgin

AND AS SUCH THE CYCLE OF TRANSMISSION THROUGH EACH LAYER AND       Virgin

USING THE TRANSMITTED SPECTRUM AS THE INCIDENT SPECTRUM FOR THE    Virgin

NEXT LAYER MAY BE REPEATED ANY NUMBER OF TIMES.                    Virgin

Virgin

REMEMBER - THE INCIDENT SPECTRUM IS ASSUMED TO BE LINEARLY         Virgin

INTERPOLABLE IN ENERGY AND SPECTRUM BETWEEN THE ENERGIES AT        Virgin

WHICH IT IS TABULATED. THE TRANSMITTED SPECTRUM WILL BE TABULATED  Virgin

AT THE UNION OF ALL ENERGIES OF THE INCIDENT SPECTRUM AND CROSS    Virgin

SECTIONS (TOTAL AND REACTION). IN ORDER TO INSURE THE ACCURACY     Virgin

OF THE RESULT WHEN PERFORMING MULTIPLE LAYER CALCULATION BE SURE   Virgin

TO SPECIFY THE INCIDENT SPECTRUM ON THE FIRST LAYER TO SUFFICIENT  Virgin

DETAIL (ENOUGH ENERGY POINTS CLOSELY SPACED TOGETHER) IN ORDER TO  Virgin

ALLOW THE TRANSMITTED SPECTRUM TO BE ACCURATELY REPRESENTED BY     Virgin

LINEAR INTERPOLATION BETWEEN SUCCESSIVE ENERGY POINTS - THERE IS   Virgin

NO LIMIT TO THE NUMBER OF POINTS ALLOWED IN THE INCIDENT SPECTRUM, Virgin

SO IF YOU ARE IN DOUBT, SIMPLY USE MORE ENERGY POINTS TO SPECIFY   Virgin

THE INCIDENT SPECTRUM.                                             Virgin

Virgin

RESULT OUTPUT UNITS                                                Virgin

-------------------                                                Virgin

FLUX      = EXACTLY AS CALCULATED                                  Virgin

REACTIONS = 1/CM OR 1/GRAM                                         Virgin

AVERAGE   = 1/CM - MACROSCOPIC UNITS                               Virgin

CROSS                                                              Virgin

SECTION                                                            Virgin

Virgin

THICKNESS AND DENSITY                                              Virgin

---------------------                                              Virgin

THE UNCOLLIDED CALCULATION ONLY DEPENDS ON THE PRODUCT OF          Virgin

THICKNESS AND DENSITY (I.E. GRAMS PER CM SQUARED). THIS FACT       Virgin

MAY BE USED TO SIMPLIFY INPUT BY ALLOWING THE THICKNESS AND        Virgin

DENSITY TO BE GIVEN EITHER AS CM AND GRAMS/CC RESPECTIVELY         Virgin

OR ELSE TO GIVE THICKNESS IN GRAMS/(CM*CM) AND INPUT A             Virgin

DENSITY OF 1.0 - OR IN ANY OTHER CONVENIENT UNITS AS LONG AS       Virgin

THE PRODUCT OF THICKNESS AND DENSITY IS IN THE CORRECT GRAMS       Virgin

PER CENTIMETER SQUARED.                                            Virgin

Virgin

GRAMS/(CM*CM) ARE RELATED TO ATOMS/BARN THROUGH THE RELATIONSHIP   Virgin

Virgin

GRAMS/(CM*CM)=(ATOMS/BARN)*(GRAMS/MOLE)*(MOLE/ATOM)                Virgin

Virgin

OR...                                                              Virgin

Virgin

GRAMS/(CM*CM)=(ATOMS/BARN)*(ATOMIC WEIGHT)/0.602                   Virgin

Virgin

CROSS SECTIONS AT A SPACE POINT AND OPTICAL THICKNESS              Virgin

-----------------------------------------------------              Virgin

THIS PROGRAM ALLOWS LAYERS OF EITHER UNIFORM DENSITY OR            Virgin

CONTINUOUSLY VARYING DENSITY. THE DENSITY CAN BE ONE OF THE        Virgin

FOLLOWING FORMS,                                                   Virgin

1) C                   = UNIFORM DENSITY                           Virgin

2) C*2*(X/T)           = LINEAR VARIATION FROM 0 TO C              Virgin

3) C*(2-2*(X/T))       = LINEAR VARIATION FROM C TO 0              Virgin

4) C*3*(X/T)**2        = SQUARE VARIATION FROM 0 TO C              Virgin

5) C*(3-3*(X/T)**2)/2  = SQUARE VARIATION FROM C TO 0              Virgin

6) C*4*(X/T)**3        = CUBIC VARIATION FROM 0 TO C               Virgin

7) C*(4-4*(X/T)**3)/3  = CUBIC VARIATION FROM C TO 0               Virgin

Virgin

IN ORDER TO CALCULATE REACTIONS AT A POINT THE MICROSCOPIC         Virgin

REACTION CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES.   Virgin

Virgin

IN ORDER TO CALCULATE TRANSMISSION WE MUST DEFINE THE OPTICAL      Virgin

PATH LENGTH WHICH MAY BE DEFINED BY INTEGRATING EACH OF THE        Virgin

ABOVE DENSITY FORMS TO FIND,                                       Virgin

1) C*X                                                             Virgin

2) C*X*(X/T)                                                       Virgin

3) C*X*(2-(X/T))                                                   Virgin

4) C*X*(X/T)**2                                                    Virgin

5) C*X*(3-(X/T)**2)/2                                              Virgin

6) C*X*(X/T)**3                                                    Virgin

7) C*X*(4-(X/T)**3))/3                                             Virgin

Virgin

IN ORDER TO CALCULATE TRANSMISSION TO A POINT THE MICROSCOPIC      Virgin

TOTAL CROSS SECTION NEED MERELY BE SCALED BY THESE DENSITIES       Virgin

TO DEFINE THE OPTICAL PATH LENGTH.                                 Virgin

Virgin

THE VARIATION OF THE DENSITY THROUGH THE LAYER MAY BE DEFINED      Virgin

BY SETTING X = 0 OR X = T TO FIND,                                 Virgin

X = 0      X = T                                                Virgin

-----      -----                                                Virgin

1) C          C                                                    Virgin

2) 0          2*C                                                  Virgin

3) 2*C        0                                                    Virgin

4) 0          3*C                                                  Virgin

5) 3*C/2      0                                                    Virgin

6) 0          4*C                                                  Virgin

7) 4*C/3      0                                                    Virgin

Virgin

THE OPTICAL PATH THROUGH A LAYER OF THICKNESS T MAY BE DEFINED     Virgin

FROM THE ABOVE EXPRESSIONS BY SETTING X=T TO FIND THAT IN ALL      Virgin

CASES THE ANSWER WILL BY C*T. THE CONSTANTS IN THE ABOVE           Virgin

EXPRESSIONS HAVE BEEN INTRODUCED IN ORDER TO FORCE THIS RESULT.    Virgin

WITH THESE FACTORS THE OPTICAL PATH LENGTH THROUGH THE LAYER       Virgin

WILL EXACTLY CORRESPOND TO AN AVERAGE DENSITY CORRESPONDING TO     Virgin

THAT INPUT FOR THE TOTAL AND/OR REACTION, I.E., C CORRESPONDS      Virgin

TO THE INPUT DENSITY.                                              Virgin

Virgin

NOTE - FOR THE SAME OPTICAL PATH LENGTHS THROUGH THE LAYER THE     Virgin

TRANSMISSION WILL BE EXACTLY THE SAME. HOWEVER, VARYING THE        Virgin

DENSITY WILL ALLOW YOU TO MODIFY THE REACTION RATES AT SPECIFIC    Virgin

DEPTHS INTO THE LAYER.                                             Virgin

Virgin

COMPUTATION OF INTEGRALS                                           Virgin

------------------------                                           Virgin

STARTING FROM TOTAL CROSS SECTIONS, REACTION CROSS SECTIONS AND    Virgin

A SOURCE SPECTRUM ALL OF WHICH ARE GIVEN IN TABULAR FORM WITH      Virgin

LINEAR INTERPOLATION BETWEEN TABULATED POINTS ALL REQUIRED         Virgin

INTEGRALS CAN BE DEFINED BY ANALYTICAL EXPRESSIONS INVOLVING       Virgin

NOTHING MORE COMPLICATED THAN EXPONENTIALS. THE INTEGRALS THAT     Virgin

MUST BE EVALUATED ARE OF THE FORM...                               Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

(INTEGRAL EK TO EK+1) (S(E)* EXP(-XCT(E)*Z)*DE)                    Virgin

Virgin

REACTIONS                                                          Virgin

---------                                                          Virgin

(INTEGRAL EK TO EK+1) (S(E)*XCR(E)*EXP(-XCT(E)*Z)*DE)              Virgin

Virgin

WHERE..                                                            Virgin

EK TO EK+1 = LONGEST ENERGY INTERVAL OVER WHICH S(E), XCT(E) AND   Virgin

XCR(E) ARE ALL LINEARLY INTERPOLABLE.                 Virgin

S(E)       = ENERGY DEPENDENT WEIGHTING SPECTRUM                   Virgin

XCR(E)     = REACTION CROSS SECTION                                Virgin

XCT(E)     = OPTICAL PATH LENGTH (BASED ON TOTAL CROSS SECTION)    Virgin

Z          = MATERIAL THICKNESS                                    Virgin

Virgin

S(E), XCR(E) AND XCT(E) ARE ALL ASSUMED TO BE GIVEN IN TABULAR     Virgin

FORM WITH LINEAR INTERPOLATION USED BETWEEN TABULATED POINTS.      Virgin

IN OTHER WORDS BETWEEN TABULATED POINTS EACH OF THESE THREE IS     Virgin

DEFINED BY A FUNCTION OF THE FORM...                               Virgin

Virgin

F(E)=((E - EK)*FK+1 + (EK+1 - E)*FK)/(EK+1 - EK)                   Virgin

Virgin

EACH OF THESE THREE CAN BE CONVERTED TO NORMAL FORM BY THE         Virgin

CHANGE OF VARIABLES....                                            Virgin

Virgin

X=(E - 0.5*(EK+1 + EK))/(EK+1 - EK)                                Virgin

Virgin

IN WHICH CASE X WILL VARY FROM -1 (AT EK) TO +1 (AT EK+1) AND      Virgin

EACH FUNCTION REDUCES TO THE NORMAL FORM...                        Virgin

Virgin

F(X)=0.5*(FK*(1 - X) + FK+1*(1 + X))                               Virgin

=0.5*(FK+1 + FK) + 0.5*(FK+1 - FK)*X                           Virgin

Virgin

BY DEFINING THE AVERAGE VALUE AND 1/2 THE CHANGE ACROSS THE        Virgin

INTERVAL.                                                          Virgin

Virgin

AVF=0.5*(FK+1 + FK)                                                Virgin

DF= 0.5*(FK+1 - FK)                                                Virgin

DE= 0.5*(EK+1 - EK)                                                Virgin

Virgin

EACH OF THE THREE FUNCTIONS REDUCES TO THE SIMPLE FORM...          Virgin

Virgin

F(X)=AVF+DF*X                                                      Virgin

Virgin

AND THE TWO REQUIRED INTEGRALS REDUCE TO...                        Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                             Virgin

((AVS+DS*X)*EXP(-DXCT*Z*X)*DX)                                     Virgin

Virgin

REACTION                                                           Virgin

--------                                                           Virgin

DE*EXP(-AVXCT*Z) * (INTEGRAL -1 TO +1)                             Virgin

((AVS*AVXCR+(AVS*DXCR+AVXCR*DS)*X+DS*DXCR*X*X)*EXP(-DXCT*Z*X)*DX)  Virgin

Virgin

WHERE                                                              Virgin

Virgin

AVXCT   = AVERAGE VALUE OF THE TOTAL CROSS SECTION                 Virgin

AVXCR   = AVERAGE VALUE OF THE REACTION CROSS SECTION              Virgin

AVS     = AVERAGE VALUE OF THE SOURCE                              Virgin

DXCT    = 1/2 THE CHANGE IN THE TOTAL CROSS SECTION                Virgin

DXCR    = 1/2 THE CHANGE IN THE REACTION CROSS SECTION             Virgin

DS      = 1/2 THE CHANGE IN THE SOURCE                             Virgin

DE      = 1/2 THE CHANGE IN THE ENERGY                             Virgin

Virgin

NOTE THAT IN THIS FORM THE ENERGY ONLY APPEARS IN FRONT OF THE     Virgin

INTEGRALS AND THE INTEGRALS ARE EXPRESSED ONLY IN TERMS OF THE     Virgin

TABULATED VALUES OF S(E), XCT(E) AND XCR(E). IN PARTICULAR NO      Virgin

DERIVATIVES ARE USED, SO THAT THERE ARE NO NUMERICAL INSTABILITY   Virgin

PROBLEMS IN THE VACINITY OF DISCONTINUITIES IN S(E), XCT(E) OR     Virgin

XCR(E). INDEED, SINCE (EK+1 - EK) APPEARS IN FRONT OF THE INTEGRAL Virgin

POINTS OF DISCONTINUITY AUTOMATICALLY MAKE ZERO CONTRIBUTION TO    Virgin

THE INTEGRALS.                                                     Virgin

Virgin

THE REQUIRED INTEGRALS CAN BE EXPRESSED IN TERMS OF THE THREE      Virgin

INTEGRALS IN NORMAL FORM....                                       Virgin

Virgin

F(A,N) = (INTEGRAL -1 TO 1) (X**N*EXP(-A*X)*DX), N=0,1 AND 2.      Virgin

Virgin

THESE THREE INTEGRALS CAN BE EVALUATED TO FIND...                  Virgin

Virgin

N=0                                                                Virgin

---                                                                Virgin

F(A,0) = (EXP(A)-EXP(-A))/A                                        Virgin

Virgin

N=1                                                                Virgin

---                                                                Virgin

F(A,1) = ((1-A)*EXP(A)-(1+A)*EXP(-A))/(A*A)                        Virgin

Virgin

N=2                                                                Virgin

---                                                                Virgin

F(A,2) = ((2-2*A+A*A)*EXP(A)-(2+2*A+A*A)*EXP(-A))/(A*A*A)          Virgin

Virgin

HOWEVER THESE EXPRESSIONS ARE NUMERICALLY UNSTABLE FOR SMALL       Virgin

VALUES OF A. THEREFORE FOR SMALL A THE EXPONENTIAL IN THE          Virgin

INTEGRALS ARE EXPANDED IN A POWER SERIES...                        Virgin

Virgin

EXP(-AX)=1.0-(AX)+(AX)**2/2-(AX)**3/6+(AX)**4/24-........          Virgin

=(SUM K=0 TO INFINITY) (-AX)**K/(K FACTORIAL)              Virgin

Virgin

AND THE INTEGRAL REDUCES TO THE FORM....                           Virgin

Virgin

(SUM K=0 TO INFINITY) ((-A)**K/(K FACTORIAL)) *                    Virgin

(INTEGRAL -1 TO 1) (X**(N+K))*DX                                   Virgin

Virgin

WHICH CAN BE ANALYTICALLY EVAULATED TO FIND....                    Virgin

(K(N) = K FACTORIAL)                                               Virgin

Virgin

N=0                                                                Virgin

---                                                                Virgin

F(A,0) = 2*(1+(A**2)/K(3)+(A**4)/K(5)+(A**6)/K(7)+....             Virgin

Virgin

N=1                                                                Virgin

---                                                                Virgin

F(A,1) = -2*A*(2/K(3)+4*(A**2)/K(5)+6*(A**4)/K(7)+8*(A**6)/K(9)+.. Virgin

Virgin

N=2                                                                Virgin

---                                                                Virgin

F(A,2) = 2*(2/K(3)+3*4*(A**2)/K(5)+5*6*(A**4)/K(7)+                Virgin

7*8*(A**6)/K(9)+....                                      Virgin

Virgin

THESE EXPANSIONS ARE USED WHEN THE ABSOLUTE VALUE OF A IS LESS     Virgin

THAN 0.1. BY TRUNCATING THE ABOVE SERIES BEFORE A**8 THE ERROR     Virgin

RELATIVE TO THE LEADING TERM OF THE SERIES WILL BE 10**(-10),      Virgin

YIELDING 10 DIGIT ACCURACY.                                        Virgin

Virgin

AFTER EVALUATING THE ABOVE FUNCTIONS, EITHER DIRECTLY OR BY USING  Virgin

THE EXPANSION THE TWO REQUIRED INTEGRALS CAN BE WRITTEN AS...      Virgin

Virgin

FLUX                                                               Virgin

----                                                               Virgin

DE*EXP(-AVXCT*Z)*(AVS*F(A,0) + DS*F(A,1))                          Virgin

Virgin

REACTIONS                                                          Virgin

---------                                                          Virgin

DE*EXP(-AVXCT*Z)*                                                  Virgin

(AVS*AVXCR*F(A,0) + (AVS*DXCR+AVXCR*DS)*F(A,1) + DS*DXCR*F(A,2))   Virgin

Virgin

INPUT FILES                                                        Virgin

-----------                                                        Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

INPUT       2   INPUT LINES                                        Virgin

ENDFIN     10   EVALUATED DATA IN ENDF/B FORMAT                    Virgin

Virgin

OUTPUT FILES                                                       Virgin

------------                                                       Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

OUTPUT      3   OUTPUT LISTING                                     Virgin

Virgin

SCRATCH FILES                                                      Virgin

-------------                                                      Virgin

FILENAME  UNIT  DESCRIPTION                                        Virgin

--------  ----  -----------                                        Virgin

SCR1       12   REACTION, FLUX AND CROSS SECTION RESULTS (BCD)     Virgin

(SORTED AT END OF RUN AND OUTPUT SEPARATELY)       Virgin

SCR2       13   TALLY GROUP ENERGY BOUNDARIES (BINARY)             Virgin

SCR3       14   SOURCE SPECTRUM (BINARY)                           Virgin

SCR4       15   TOTAL CROSS SECTION (BINARY)                       Virgin

SCR5       16   REACTION CROSS SECTION (BINARY)                    Virgin

Virgin

OPTIONAL STANDARD FILE NAMES (SEE SUBROUTINE FILIO1 AND FILEIO2)   Virgin

----------------------------------------------------------------   Virgin

UNIT  FILE NAME   FORMAT                                           Virgin

----  ----------  ------                                           Virgin

2   VIRGIN.INP   BCD                                             Virgin

3   VIRGIN.LST   BCD                                             Virgin

10   ENDFB.IN     BCD                                             Virgin

11-15  (SCRATCH)   BINARY                                           Virgin

16   PLOTTAB.CUR PLOTTAB OUTPUT FORMAT DATA                       Virgin

Virgin

INPUT LINES                                                        Virgin

-----------                                                        Virgin

ANY NUMBER OF CASES MAY BE RUN ONE AFTER THE OTHER. AFTER THE      Virgin

FIRST CASE HAS BEEN RUN THE FOLLOWING CASES MAY USE THE SAME       Virgin

THICKNESSES, GROUP STRUCTURE AND SPECTRUM AS THE PRECEDING CASE.   Virgin

IN ADDITION THE TRANSMITTED SPECTRUM FROM ONE CASE MAY BE USED     Virgin

AS THE INCIDENT SPECTRUM IN THE NEXT CASE, TO ALLOW MULTIPLE       Virgin

LAYERS OF DIFFERENT MATERIALS.                                     Virgin

Virgin

LINE  COLS.  FORMAT  DESCRIPTION                                   Virgin

----  -----  ------  ----------                                    Virgin

1   1-60   ENDF/B INPUT DATA FILENAME                           Virgin

(STANDARD OPTION = ENDFB.IN)                       Virgin

Virgin

LEAVE THE DEFINITION OF THE FILENAMES BLANK - THE PROGRAM WILL     Virgin

THEN USE STANDARD FILENAMES.                                       Virgin

Virgin

2-3   1-72   18A4   TWO LINE TITLE DESCRIBING PROBLEM             Virgin

4   1- 6    I6    ZA (1000*Z+A) OF TARGET FOR TOTAL             Virgin

7-11    I5    MT OF TOTAL                                   Virgin

12-22   E11.4  DENSITY FOR TOTAL                             Virgin

23-28    I6    ZA (1000*Z+A) OF TARGET FOR REACTION          Virgin

29-33    I5    MT OF REACTION                                Virgin

= 0 - NO REACTION CALCULATION (ONLY FLUX).    Virgin

= GREATER THAN 0 - CALCULATE REACTIONS.       Virgin

34-44   E11.4  DENSITY FOR REACTION                          Virgin

45-50    I6    NUMBER OF TARGET THICKNESSES                  Virgin

= GREATER THAN 0 = READ FROM INPUT            Virgin

(1 TO 2000 ALLOWED)                         Virgin

=  0 = SAME AS LAST CASE                      Virgin

51-55    I5    NUMBER OF TALLY GROUPS                        Virgin

(REMEMBER NUMBER OF GROUP BOUNDARIES          Virgin

IS ONE MORE THAN THE NUMBER OF GROUPS)        Virgin

UP TO 2000 GROUPS ARE ALLOWED                 Virgin

BUILT-IN GROUP STRUCTURES.                    Virgin

= GREATER THAN 0 = READ FROM INPUT            Virgin

=  0 SAME AS LAST CASE                        Virgin

= -1 TART 175 GROUPS                          Virgin

= -2 ORNL  50 GROUPS                          Virgin

= -3 ORNL 126 GROUPS                          Virgin

= -4 ORNL 171 GROUPS                          Virgin

= -5 SAND-II 620 GROUPS..UP TO 18 MEV.        Virgin

= -6 SAND-II 640 GROUPS..UP TO 20 MEV.        Virgin

= -7 WIMS 69 GROUPS                           Virgin

= -8 GAM-I 68 GROUPS                          Virgin

= -9 GAM-II 99 GROUPS                         Virgin

=-10 MUFT 54 GROUPS                           Virgin

=-11 ABBN 28 GROUPS                           Virgin

56-60    I5    NUMBER OF POINTS IN SOURCE SPECTRUM           Virgin

(MUST BE AT LEAST TWO POINTS)                 Virgin

= GREATER THAN 1 = READ FROM INPUT            Virgin

=  0 = SAME AS LAST CASE                      Virgin

= -1 = CONSTANT (ENERGY INDEPENDENT)          Virgin

= -2 = 1/E                                    Virgin

= -3 = BLACKBODY - PHOTON SPECTRUM            Virgin

= -4 = BLACKBODY - ENERGY SPECTRUM            Virgin

= -5 = TRANSMITTED SPECTRUM FROM LAST CASE    Virgin

NOTE, ALL SPECTRA, EXCEPT THE TRANSMITTED     Virgin

SPECTRUM FROM THE LAST CASE, WILL BE          Virgin

NORMALIZED SUCH THAT ITS INTEGRAL OVER        Virgin

ENERGY WILL BE UNITY.                         Virgin

61-64  1X,3I1  SPATIALLY DEPENDENT OUTOUT                    Virgin

= 0 = NO                                      Virgin

= 1 = YES                                     Virgin

FOR THE 3 QUANTITIES                          Virgin

COLUMN 67 FLUX                                Virgin

68 REACTIONS                           Virgin

69 AVERAGE CROSS SECTION               Virgin

65-65    I1    ENERGY DEPENDENT OUTOUT                       Virgin

= 0 = NONE                                    Virgin

= 1 = INCIDENT SPECTRUM                       Virgin

= 2 = TRANSMITTED SPECTRUM                    Virgin

= 3 = INCIDENT REACTIONS                      Virgin

= 4 = TRANSMIITED REACTIONS                   Virgin

= 5 = TOTAL CROSS SECTION                     Virgin

= 6 = REACTION CROSS SECTION                  Virgin

5   1-11   E11.4  BLACKBODY TEMPERATURE IN EV                   Virgin

12-22   E11.4  FLUX NORMALIZATION                            Virgin

23-33   E11.4  REACTION NORMALIZATION                        Virgin

CALCULATIONS WILL BE BASED ON THE SPECTRUM    Virgin

AND CROSS SECTIONS AS READ. AT OUTPUT THE     Virgin

RESULTS WILL BE MULTIPLIED BY THESE           Virgin

NORMALIZATION FACTORS.                        Virgin

34-44    I11   DENSITY PROFILE                               Virgin

= 0 - UNIFORM - BASED ON TOTAL DENSITY        Virgin

= 1 - UNIFORM - TOTAL + REACTION DENSITY      Virgin

= 2 - TOTAL + LINEAR REACTION                 Virgin

= 3 - LINEAR (TOTAL + REACTION)               Virgin

= 4 - TOTAL + SQUARE REACTION                 Virgin

= 5 - SQUARE (TOTAL + REACTION)               Virgin

= 6 - TOTAL + CUBIC REACTION                  Virgin

= 7 - CUBIC (TOTAL + REACTION)                Virgin

6-N   1-66  6E11.4  TARGET THICKNESSES IN CM                      Virgin

IF SAME AS LAST CASE THIS SECTION IS NOT      Virgin

INCLUDED IN THE INPUT.                        Virgin

VARY   1-66  6E11.4  TALLY GROUP ENERGY BOUNDARIES                 Virgin

(NUMBER OF BOUNDARIES IS ONE MORE THAN        Virgin

THE NUMBER OF TALLY GROUPS)                   Virgin

IF THE STANDARD OPTION (-11 TO 0) IS          Virgin

SELECTED THIS SECTION IS NOT INCLUDED         Virgin

IN THE INPUT                                  Virgin

VARY   1-66  6E11.4  SOURCE SPECTRUM IN ENERGY (EV)-SOURCE PAIRS   Virgin

(MUST BE AT LEAST TWO POINTS)                 Virgin

IF STANDARD OPTION (-5 TO 0) IS SELECTED THIS Virgin

SECTION IS NOT INCLUDED IN THE INPUT          Virgin

Virgin

ANY NUMBER OF CASES MAY BE RUN ONE AFTER ANOTHER.                  Virgin

Virgin

EXAMPLE INPUT NO. 1                                                Virgin

-------------------                                                Virgin

CALCULATE THE UNCOLLIDED FLUX AND CAPTURE (MT=102) THROUGH         Virgin

30 CM OF IRON (DENSITY 7.87 G/CC). TALLY THE RESULTS USING         Virgin

THE TART 175 GROUP STRUCTURE. THE SOURCE WILL BE CONSTANT          Virgin

FROM 1 KEV TO 20 MEV. USE THE STANDARD ENDF/B INPUT DATA           Virgin

FILENAME.                                                          Virgin

Virgin

ENDFB.IN                                                           Virgin

IRON 0 TO 30 CM THICK.                                             Virgin

CONSTANT SOURCE FROM 1 KEV TO 20 MEV.                              Virgin

26000    1 7.87000+ 0 26000  102 7.87000+ 0     2    0    2 1100  Virgin

0.00000+ 0 1.00000+ 0 1.00000+ 0          0 0.00000+00            Virgin

0.00000+00 3.00000+01                                             Virgin

1.0000E+03 1.0000E+00 2.0000E+07 1.0000E+00                       Virgin

Virgin

EXAMPLE INPUT NO. 2                                                Virgin

-------------------                                                Virgin

CALCULATE THE UNCOLLIDED PHOTON FLUX THROUGH A MIXTURE OF SILICON  Virgin

AND IRON FOR 100 MEV PHOTONS INCIDENT. THE TRANSMISSION WILL BE    Virgin

CALCULATED FOR 21 THICKNESSES VARYING BETWEEN 0 AND 1 CM. THERE    Virgin

WILL BE ONLY 1 TALLY GROUP SPANNING A VERY NARROW ENERGY RANGE     Virgin

NEAR 100 MEV, AND THE SOURCE SPECTRUM WILL BE CONSTANT OVER THE    Virgin

SAME ENERGY RANGE. USE THE STANDARD ENDF/B INPUT DATA FILENAME     Virgin

BY LEAVING THE FIRST INPUT LINE BLANK.                             Virgin

Virgin

(THIS IS A BLANK LINE TO USE THE STANDARD INPUT FILENAME)          Virgin

100 MEV PHOTONS                                                    Virgin

SILICON + 5 % IRON                                                 Virgin

14000  521 2.30000+ 0 26000  521 1.15000- 1    21    1    2 1000  Virgin

0.00000+ 0 1.00000+ 0 1.00000+ 0          1 0.00000+00            Virgin

0.00000+00 5.00000-01 1.00000+00 1.50000+00 2.00000+00 2.50000+00 Virgin

3.00000+00 3.50000+00 4.00000+00 4.50000+00 5.00000+00 5.50000+00 Virgin

6.00000+00 6.50000+00 7.00000+00 7.50000+00 8.00000+00 8.50000+00 Virgin

9.00000+00 9.50000+00 1.00000+01                                  Virgin

9.99000+ 7 1.00100+ 8                                             Virgin

9.99000+ 7 1.00000+ 4 1.00100+ 8 1.00000+ 4                       Virgin

Virgin

======================================================================= Virgin