Search History   13 March 2002 (from 1969 on)

*  #7 #3 and #6 (570 records)

#6 #4 or #5 (34034 records)

#5 energy loss (27642 records)

#4 stopping (9780 records)

#3 #1 or #2 (14917 records)

#2 alpha particle (14089 records)

#1 Helium ion (856 records)

Record 1 of 43 in INSPEC 1996

TI:  Stopping powers of 200-3200 keV /sup 4/He and 550-1559 keV /sup 1/H ions in polyimide

AU:  Plompen-AJM; Munnik-F; Raisanen-J; Watjen-U

SO:  Journal-of-Applied-Physics. vol.80, no.6; 15 Sept. 1996; p.3147-54

PY:  1996

LA:  English

AB:  Accurate stopping powers of polyimide have been determined for mean energies (E) in the range of 200 keV<or=E<or=3200 keV for /sup 4/He ions and 550 keV<or=E<or=1550 keV for /sup 1/H ions, using the transmission method for foils with areal densities of 30-225 mu g/cm/sup 2/. The overall uncertainties of ~2% are mainly due to the foil thickness determination. The present data have been parametrized and compared to results obtained by previous authors in the upper energy range and to various parametrizations found in the literature. The best description of the data is given by Bragg's rule calculations using parametrized elemental stopping powers that were derived from large data sets (deviations <2%). No support is found for the cores-and-bonds model showing deviations of 3%-5%, that may be due to the neglect of phase state effects.

AN:  5402157

*LHM:  ZB f. Physik Wien, Signatur: 02628.001

Bestand: 8.1937=>

Record 2 of 43 in INSPEC 1969-1979

TI:  Mean excitation energies of Al, Ne, Se, Ag and Au extracted from alpha particle stopping power measurements

AU:  Porter-LE

SO:  Nuclear-Instruments-and-Methods. vol.157, no.2; 1 Dec. 1978; p.333-8

PY:  1978

LA:  English

AB:  Stopping power measurements for alpha particles traversing targets of Al, Ni, Se, Ag and Au have been analyzed both with and without inclusion of a low velocity projectile-z/sup 3/ correction in the Bethe-Bloch formula in order to extract mean excitation energies. In the former case, the values obtained for Al and Ni are consistent with expectation, whereas those obtained for Se, Ag and Au lie 34%, 28% and 12%, respectively, above generally accepted values. Also, the method of analysis was modified by incorporation of an effective charge parameter while fixing mean excitation energies at the generally accepted values in order to include some stopping power measurements at energies below the limit of strict applicability of the Bethe-Bloch formula; the quality of fit to the data deteriorated for all targets except Ni.

AN:  1298116

Record 3 of 43 in INSPEC 1969-1979

TI:  Energy loss and straggling of protons and helium ions traversing some thin solid foils

AU:  Eckardt-JC             Ec78

SO:  Physical-Review-A-(General-Physics). vol.18, no.2; Aug. 1978; p.426-33

PY:  1978

LA:  English

AB:  Absolute measurements of stopping powers and energy straggling for hydrogen- and helium-ion beams in Ge, Se, Pd, Ag, Sb, and Bi in the energy range 20<or=E<or=260 keV were performed by the method of determining the energy loss suffered by an ion beam which has traversed a thin film. The measured stopping powers fit reasonably well with those of other authors obtained at higher and lower ion energies. Agreement with theoretical predictions for energies >100 keV/amu is found to be better for hydrogen than it is for helium beams. The Z/sub 2/ dependence of the stopping power for hydrogen projectiles is discussed including measurements of other authors performed on other elements, and is found to agree in shape with that for helium for ion velocities nu >2 a.u., but there is evidence that it begins to differ with decreasing nu . It is deduced from the present measurements that for hydrogen as well as helium ions the amplitude of oscillations of the Z/sub 2/ dependence increases with decreasing ion velocity, reaching a maximum at nu =1.4 a.u. The Lindhard-Scharff theory, (1953), is found to be in satisfactory agreement with the measured energy straggling.

Record 4 of 43 in INSPEC 1969-1979

TI:  Re-evaluation of stopping powers of Be, Al, Ti, V, Fe, Co, Ni, Cu, Mo, Rh, Ag, Ta, and Au for 28 MeV alpha particles

AU:  Ishiwari-R; Shiomi-N; Sakamoto-N     Is78 neu

SO:  Bulletin-of-the-Institute-for-Chemical-Research,-Kyoto-University. vol.56, no.1; Jan. 1978; p.47-8

PY:  1978

LA:  English

AB:  The results of a previous paper, Ishiwari et al. (1977), in which the stopping powers for various metals were measured for 28.8 MeV alpha -particles, have been reevaluated.

AN:  1250567

Record 5 of 43 in INSPEC 1969-1979

TI:  Mean excitation energies of Si, Ni, Ge, Y, Ag and Au based on alpha particle stopping power measurements

AU:  Porter-LE

SO:  Nuclear-Instruments-and-Methods. vol.148, no.1; 1 Jan. 1978; p.119-23

PY:  1978

LA:  English

AB:  Mean excitation energies for Si, Ni, Ge, Y, Ag and Au have been extracted from previous stopping power measurements with alpha particle projectiles. Data were analyzed both with and without inclusion of the low-velocity projectile-z/sup 3/ effect. Values of mean excitation energy obtained by the former method proved consistent with generally accepted values for Si, Ge and Y but lay higher than expected for Ni, Ag and Au by about 11%, 20% and 20% respectively.

AN:  1174801

Record 6 of 43 in INSPEC 1969-1979

TI:  Stopping powers of Al, Ni, Cu, Ag, and Ta for 8.78 MeV alpha particles

AU:  Ishiwari-R; Shiomi-N; Kinoshita-T; Yasue-F

SO:  Bulletin-of-the-Institute-for-Chemical-Research,-Kyoto-University. vol.55, no.1; Jan. 1977; p.68-73

PY:  1977

LA:  English

AB:  The stopping powers of Al, Ni, Cu, Ag, and Ta have been measured by using the absorber wheel technique and a silicon detector. In order to examine the possible nonuniformity of the sample foil, the alpha particle beam was made to scan in two directions, i.e. parallel and perpendicular to the rolling direction of the foil. The agreement between the measurements of two directions was proved to be satisfactory. The present results were divided by 4 and reduced to 2.0 MeV and compared with the proton data from the table of Bichsel. It turned out that the data for alpha particles are 1.4-3.4 percent higher than those for protons.

AN:  1110818

Record 7 of 43 in INSPEC 1969-1979

TI:  Stopping powers of Be, Al, Ti, V, Fe, Co, Ni, Cu, Mo, Rh, Ag, Ta, and Au for 28.8 MeV alpha particles

AU:  Ishiwari-R; Shiomi-N; Shirai-S

SO:  Bulletin-of-the-Institute-for-Chemical-Research,-Kyoto-University. vol.55, no.1; Jan. 1977; p.60-7

PY:  1977

LA:  English

AB:  Stopping powers of Be, Al, Ti, V, Fe, Co, Ni, Cu, Mo, Rh, Ag, Ta, and Au for 28.8 MeV alpha particles have been measured with a silicon detector and associated electronic equipment. It has been confirmed that the stopping power for alpha particles divided by 4 is higher than that for protons of the same velocity. The deviations are from 2.4 to 4 percent.

AN:  1110817

Record 8 of 43 in INSPEC 1969-1979

TI:  Stopping powers of copper, silver and gold for protons and helium ions of low energy

AU:  Nomura-A; Kiyono-S             No75

SO:  Journal-of-Physics-D-(Applied-Physics). vol.8, no.13; 11 Sept. 1975; p.1551-9

PY:  1975

LA:  English

AB:  Protons and helium ions in the energy range 4-16 keV have impinged on free films of copper, silver and gold of thicknesses from 300 to 700 AA. The stopping powers have been determined by measuring the energy spectra of the ions transmitted within a cone of 3 degrees half-angle. As for the stopping power for helium ions, the nuclear stopping power is very small compared with the electronic stopping power in this method. The experimental results show that the dependence of the electronic stopping power on the ion velocity is almost in quantitative agreement with the Lindhard, Scharff and Schiott (LSS) theory.

AN:  831438

*LHM:  ZB f. Physik Wien, Signatur: 04130.001

Bestand: 3.1970=>

Record 9 of 43 in INSPEC 1969-1979

TI:  Z/sub 1//sup 3/ effect on the stopping powers of several metallic elements for 28.8 MeV alpha particles: Deviations of experimental data from theories

AU:  Ishiwari-R; Shiomi-N; Shirai-S   Is75a

SO:  Physics-Letters-A. 27 Jan. 1975; p.53-4

PY:  1975

LA:  English

AB:  Stopping powers of several metallic elements for 28.8 MeV alpha particles have been measured. One fourth of the stopping power for alpha particles is larger than the stopping power for protons of the same velocity. But the differences do not agree with Z/sub 1//sup 3/ correction theories.

AN:  752032

*LHM:  ZB f. Physik Wien, Signatur: 08614.010

Bestand: 24.1967=>   L:63?

Record 10 of 43 in INSPEC 1969-1979

TI:  alpha -particle stopping cross section of gold and silver as measured from thick targets

AU:  Lin-WK; Matteson-S; Powers-D            Li74

SO:  Physical-Review-B-(Solid-State). vol.10, no.9; 1 Nov. 1974; p.3746-55

PY:  1974

LA:  English

AB:  The advantages of thick targets over thin for the measurement of stopping cross sections are discussed. A method is described for measuring the energy spectrum of alpha -particles elastically scattered from thick targets. The theory of calculating stopping cross sections from the energy spectra is developed. Stopping cross sections obtained from data on thick targets of Ag and Au are compared with thin-target data over the energy range 0.3-2 MeV and good agreement is found.

AN:  737916

Record 11 of 43 in INSPEC 1969-1979

TI:  Helium ion stopping cross sections in bismuth, lead and tungsten

AU:  Borders-JA     Bo74

SO:  Radiation-Effects. vol.21, no.3; 1974; p.165-9

PY:  1974

LA:  English

AB:  The stopping cross sections of He ions in bismuth, lead and tungsten have been measured at incident energies from 0.4 MeV-1.9 MeV. The energy losses of helium ions in vacuum evaporated films of bismuth and lead and sputtered films of tungsten were determined using a backscattering technique. Atomic areal densities were measured by a microbalance technique for the bismuth and lead films. The thicknesses of the tungsten films were determined by multiple-beam interferometry and a density of 0.93X atomic density was assumed in order to evaluate the areal density. Brice's three parameter formula (1972) has been fit to the data. Comparison of the results has also been made with the semi-empirical calculations and the experimental results of Chu, et al. (1973), and those of Lin, et al.

AN:  685074

Record 12 of 43 in INSPEC 1969-1979

TI:  Stopping power of cadmium sulphide for helium ions

AU:  Engels-PF; Borders-JA; Chernow-F

SO:  Journal-of-Applied-Physics. vol.45, no.1; Jan. 1974; p.38-42

PY:  1974

LA:  English

AB:  A new experimental technique is described involving the measurement of the energy spectrum of either He ions backscattered from implanted Bi or of alpha -particles given out by an implanted radioactive isotope such as /sup 210/Po. A thin layer is then removed by etching and the energy spectrum re-measured. The stopping power is then readily estimated, calculations being made for energies ranging from 0.963 to 5.3 MeV with accuracies between 12 and 16%.

AN:  635442

*LHM:  ZB f. Physik Wien, Signatur: 02628.001

Bestand: 8.1937=>

Record 13 of 43 in INSPEC 1969-1979

TI:  Alpha-particle stopping cross section of solids from 0.3 to 2.0 MeV

AU:  Lin-WK; Olson-HG; Powers-D        Li73b

SO:  Physical-Review-B-(Solid-State). vol.8, no.5; 1 Sept. 1973; p.1881-8

PY:  1973

LA:  English

AB:  The stopping cross sections of alpha particles for the solid elements Se, Y, Zr, Nb, Mo, Sb, Te, La, Dy, Ta, W and Au have been measured in the energy range from 0.3 to 2.0 MeV and oscillatory structure has been observed in the dependence of the stopping cross section on the atomic number of the stopping element in addition to that previously reported. The energy dependence of the measured particle stopping power is fitted to a theoretical expression derived by Brice (1972) and it was found that the parameters of that fit were interdependent. Extrapolation of these results on this basies was acceptable up to 1o MeV energies for those elements for which there are values available for comparison.

AN:  619953

Record 14 of 43 in INSPEC 1969-1979

TI:  Alpha-particle stopping cross section of silicon and germanium

AU:  Lin-WK; Olson-HG; Powers-D        Li73a

SO:  Journal-of-Applied-Physics. vol.44, no.8; Aug. 1973; p.3631-4

PY:  1973

LA:  English

AB:  Stopping cross sections of alpha particles for silicon and germanium have been measured for 0.3<or=E/sub alpha /<or=2.0 MeV. The experimental method consists of the elastic scattering of alpha particles either from a thick Ta backing onto which the stopping element has been evaporated, or directly from the film of stopping element evaporated onto a thick Al backing. The results were analyzed using a three-parameter formula for stopping cross section. With the parameters obtained from a least-squares fit, the formula was found to be useful in calculating the stopping cross sections for energies outside the measurements, at least up to 10 MeV.

AN:  559067

*LHM:  ZB f. Physik Wien, Signatur: 02628.001

Bestand: 8.1937=>

Record 15 of 43 in INSPEC 1969-1979

TI:  Stopping power of energetic helium ions transmitted through thin silicon crystals in channelling and random directions

AU:  Eisen-FH; Clark-GJ; Bottiger-J; Poate-JM

SO:  Radiation-Effects. vol.13, no.1-2; March 1972; p.93-100

PY:  1972

LA:  English

AB:  The stopping powers of helium ions transmitted through silicon in channelling directions ((110) and (111) axes and (111) plane) and in random directions for the energy range from about 0.1 MeV to 18 MeV have been determined. For the channelled cases, stopping cross sections have been calculated using both the position of the leading edge and the peak of the channelled energy spectra. Evidence is presented to support the view that the leading edge of the energy spectrum corresponds to the energy loss of the best channelled particles. It was found that the ratio of the channelled-to-random stopping, using either of the above mentioned channelled stopping powers, is a function of the helium- ion energy having a maximum value at the same energy at which the maximum in the stopping cross section is observed. For (110) channelling, this ratio of the best channelled ions varied from about 0.3 at 0.1 MeV to a maximum of about 0.55 and decreased to a limit of about 0.3 at high energies. The relation between the values of stopping powers determined in the transmission experiments and those applicable to depth calculations in Rutherford-scattering experiments is discussed.

AN:  391453

Record 16 of 43 in INSPEC 1969-1979

TI:  Stopping cross sections for 0.3- to 1.7-MeV helium ions in silicon and silicon dioxide

AU:  Thompson-DA

SO:  Journal-of-Applied-Physics. vol.42, no.10; Sept. 1971; p.3969-76

PY:  1971

LA:  English

AB:  Stopping cross sections of helium ions with energy 0.3-1.7 MeV have been measured with a systematic error of +or-1.6% in silicon and +or-1.3% in amorphous silicon dioxide. The probable random error of the measurements is estimated to be 2.3%. Experimentally, a backscattering technique is used in which the ions are scattered through 150 degrees from a heavy mass atom on the back surface of the target film. The stopping cross-section data were obtained using a 1114- AA-thick silicon target film and a 1030-AA silicon dioxide film. The results for SiO/sub 2/ indicate a deviation from the Bragg-Kleeman rule of additivity of atomic stopping cross sections.

AN:  315406

*LHM:  ZB f. Physik Wien, Signatur: 02628.001

Bestand: 8.1937=>

Record 17 of 43 in INSPEC 1969-1979

TI:  A Bragg-rule study of alpha-particle stopping power in gaseous compounds

AU:  Bourland-PD      Thesis????

SO:  Baylor Univ. Waco, TX, USA, 143 pp.

PY:  1970

LA:  English

AB:  Stopping powers for alpha particles have been measured in a differentially-pumped gas-cell system from 300 keV to 2 MeV in fifteen gaseous compounds: H/sub 2/, N/sub 2/, O/sub 2/, NH/sub 3/, N/sub 2/O, CO, CO/sub 2/, CH/sub 4/, C/sub 2/H/sub 2/, C/sub 2/H/sub 4/, C/sub 2/H/sub 6/, C/sub 3/H/sub 4/ (methyl acetylene), C/sub 3/H/sub 4/ (allene), C/sub 3/H/sub 6/, and (CH/sub 2/)/sub 3/. The probable random error of the measurements varies from 1 to 3%, and the systematic error is estimated to be 1.4%. Independent measurements have also been made with a sealed gas cell in a scattering chamber for N/sub 2/ and H/sub 2/, and are found to agree to the differentially-pumped gas-cell measurements to within 2.7%. All measurements are compared to the independent measurements of Palmer and of Rotondi, and agreement varying from 0 to 21% is found.

AN:  298117

Record 18 of 43 in INSPEC 1969-1979

TI:  Alpha-particle stopping cross section in solids from 400 keV to 2 MeV

AU:  Chu-WK; Powers-D            Ch69

SO:  Physical-Review. vol.187, no.2; Nov. 1969; p.478-90

PY:  1969

LA:  English

AB:  Stopping cross sections of alpha particles from 400 keV to 2 MeV have been measured to an accuracy of +or-3.6 to +or-4.9% in 17 elements (Be, C, Mg, Al, Ti, V, Cr, Mn, Fe, Co, Ni, Cu, Ge, Pd, Ag, In, and Sn). The experimental methods consists of the elastic scattering of alpha particles from a thick Ta backing onto which a thin layer of target element has been evaporated. The energy loss of alpha particles in the target film is determined by the difference in energy between alpha particles scattered from clean Ta and alpha particles scattered from Ta after having gone through the thin layer of target element. Structure in and decrease of the alpha -particle stopping cross section epsilon /sub alpha / with stopping element atomic number Z/sub 2/ are noticed in the region Z/sub 2/=22-29. The oscillatory structure of epsilon /sub alpha / on Z/sub 2/ is discussed qualitatively by comparing epsilon /sub alpha /(E/sub alpha /) versus Z/sub 2/ with a Hartree-Fock-Slater potential pi (r) versus Z, with E/sub alpha / related to the radius r by a velocity comparison.

AN:  116015

Record 19 of 43 in INSPEC 1980-1984

TI:  Stopping power measurement using thick alpha sources

AU:  Hosoe-M; Takami-Y; Shiraishi-F; Tomura-K

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-A-(Accelerators,-Spectrometers,-Detectors-and-Associated-Equipment). vol.223, no.2-3; 15 June 1984; p.377-81

PY:  1984

LA:  English

AB:  For alpha particles emerging from a plane surface of a thick medium in which alpha activity is distributed homogeneously, the energy spectrum is inversely proportional to the stopping power of the medium. The surface barrier type SSDs made of p type silicon were used to measure the spectrum and the stopping power was calculated. Although this method can be applied only to alpha emitting sources at present, it has the merit of simplicity in the experimental procedure and gives the stopping power as a continuous function of energy by measuring only one spectrum. Powder with less than 1 mu m particle size is a suitable medium to mix with alpha activity in this method. The results are compared with stopping powers calculated from the Northcliffe-Schilling and Ziegler data tables.

AN:  2329128

*LHM:  ZB f. Physik Wien, Signatur: 08574.010

Bestand: A219.1984=>

Record 20 of 43 in INSPEC 1980-1984

TI:  Stopping power measurements of C, Al, Si, Ti, Ni, Ag, Au and mylar using radioactive alpha sources

AU:  Santry-DC; Werner-RD    Sa84

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.229, no.1; Jan. 1984; p.13-15

PY:  1984

LA:  English

AB:  Stopping power values and their energy dependence for alpha energies near 5.5 MeV have been measured for mylar and self-supported thin films of selected elements of known thickness. The measured values are compared with those of a semiempirical tabulation.

AN:  2233058

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 21 of 43 in INSPEC 1980-1984

TI:  alpha -particle stopping power for titanium and vanadium

AU:  Haight-RC; Vonach-HK    Ht84

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.229, no.1; Jan. 1984; p.9-12

PY:  1984

LA:  English

AB:  A method for accurate measurement of the specific energy loss dE/dx of alpha -particles is described and results are given for titanium in the alpha -energy range 5.25-13 MeV and for vanadium in the range 5.25-12 MeV with uncertainties of about 3%. The results are in excellent agreement with the stopping power predictions of Ziegler (1977) and earlier precision measurements of the stopping power of protons and deuterons in Ti and V.

AN:  2233057

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 22 of 43 in INSPEC 1980-1984

TI:  Stopping cross-sections of rare gases in amorphous silicon for MeV energy helium ions

AU:  Qotuka-A; Komaki-K; Fujimoto-F; Kawatsura-K; Ozawa-K; Shimada-T; Katayama-Y

SO:  Japanese-Journal-of-Applied-Physics,-Part-1-(Regular-Papers-&-Short-Notes). vol.22, no.8; Aug. 1983; p.1306-9

PY:  1983

LA:  English

AB:  Using the Rutherford backscattering method, the stopping cross-sections of amorphous silicon doped with rare gases to a concentration of a few percent were measured for helium ions. The effective stopping cross-sections of the rare gases were deduced by assuming Bragg's rule and by using Ziegler's cross-section values for silicon. The results were systematically about 30 percent lower than Ziegler's values for the gaseous state, in the energy range near 1 MeV. The incident energies were 1.0-2.6 MeV for samples containing 8% argon, and 1.0-1.6 MeV for those containing 7% krypton or 4% xenon.

AN:  2195046

Record 23 of 43 in INSPEC 1980-1984

TI:  Energy-loss and straggling of hydrogen and helium ions in silver

AU:  Lombaard-J; Conradie-J; Friedland-E      Lo83

SO:  Nuclear-Instruments-and-Methods-in-Physics-Research. vol.216, no.1-2; 15 Oct. 1983; p.293-8

PY:  1983

LA:  English

AB:  Energy-loss and straggling of protons, deuterons and /sup 4/He ions in silver were measured in the energy range between 0.14 MeV and 2.6 MeV. For energies between 0.25 and 0.60 MeV/amu the stopping powers of the hydrogen ions in silver are up to 10% lower than the semi-empirical values of Andersen and Ziegler. Better agreement is obtained with the shell-correction fit for energies above 0.6 MeV/amu. In the case of the helium ions the measured values are between 8% to 13% lower than the semi-empirical values of Ziegler. The reduced straggling for the /sup 4/He and hydrogen ions is energy independent above 0.3 MeV/amu with the He values the same as predicted by the Bohr estimate but the hydrogen values are about 15% lower.

AN:  2164579

Record 24 of 43 in INSPEC 1980-1984

TI:  Stopping power of Ni, Au, and Pb for approximately 7-MeV/nucleon alpha particles and carbon ions: Z/sub 1//sup 3/ deviation from the Bethe formula

AU:  Takahashi-T; Awaya-Y; Tonuma-T; Kumagai-H; Izumo-K; Nishida-M; Hitachi-A; Hashizume-A; Uchiyama-S; Doke-T         Ta83

SO:  Physical-Review-A-(General-Physics). vol.27, no.3; March 1983; p.1360-4

PY:  1983

LA:  English

AB:  The stopping powers of Ni, Ag, Au, and Pb for approximately 7-MeV/N (where N means nucleon) C ions and alpha particles have been measured. For the 7.00-MeV/N C ion, the stopping powers of Ni, Ag, Au, and Pb are found to be 1.353+or-0.016, 1.080+or-0.010, 0.832+or-0.006, and 0.825+or-0.006 MeV/mg cm/sup 2/, respectively. From these results, the Z/sub 1//sup 3/ deviation from the Bethe-Bloch formula is evaluated comparing with the authors' experimental stopping power for alpha particles. In the course of analysis they employed appropriate effective-charge corrections. Pierce and Blann's effective-charge correction (1968), e.g. gives a reasonable value (1.38) of the b which appears in the Ritchie-Brandt theory (1978), if a chi value of 1.29 in their theory is assumed.

AN:  2053794

*LHM:  ZB f. Physik Wien, Signatur: 01072.040

Bestand: 3.ser.1.1970=>

Record 25 of 43 in INSPEC 1980-1984

TI:  Energy-loss and straggling of hydrogen and helium ions in selenium

AU:  Conradie-J; Lombaard-J; Friedland-E

SO:  Nuclear-Instruments-and-Methods-in-Physics-Research. vol.205, no.1-2; 15 Jan. 1983; p.359-63

PY:  1983

LA:  English

AB:  Using a transmission technique, the energy-loss and straggling of protons, deuterons and helium ions in Se were measured in the energy range between 0.3 and 2.5 MeV. The experimental stopping powers of the various ions are the same as the semiempirical values of Andersen and Ziegler (1977), except in the vicinity of the stopping power maximum. The reduced straggling for the helium and hydrogen ions is energy independent above 0.25 MeV/amu with the helium values the same as predicted by Bohr but the hydrogen values are somewhat lower.

AN:  2025943

Record 26 of 43 in INSPEC 1980-1984

TI:  Alpha particle stopping cross sections of Cu and Au at energies below 3 MeV

AU:  Stoquert-JP; Abdesselam-A; Beaumevieille-H; Boudouma-Y; Oberlin-JC    Sq82

SO:  Nuclear-Instruments-and-Methods-in-Physics-Research. vol.194, no.1-3; 15 March 1982; p.51-5

PY:  1982

LA:  English

AB:  Alpha particle cross sections have been measured in thin films of Cu and Au deposited onto light Si substrates. A method is described for the analysis of peaks obtained in the backscattering configuration with natural multi-isotopic Cu targets. Comparisons are made with the best fit values from the compilation by Ziegler (1977) and recent published data. Theoretical calculations based on the Gertner et al. (1978) model derived from the Lindhard-Winther (1964) theory have been made, using modified Hartree-Fock-Slater wave functions in order to introduce a realistic charge density distribution in solids. The predicted stopping powers are in good agreement with experimental data at energies above 1 MeV. Discrepancies between different results at lower energies are discussed with regard to experimental and theoretical difficulties.

AN:  1875396

Record 27 of 43 in INSPEC 1980-1984

TI:  Stopping power and straggling of 0.2-2.0 MeV protons and 0.3-3.1 MeV /sup 4/He ions in erbium

AU:  Oberlin-JC; Amokrane-A; Beaumevieille-H; Stoquert-JP; Perrier-de-la-Bathie-R          Ob82

SO:  Journal-de-Physique. vol.43, no.3; March 1982; p.485-91

PY:  1982

LA:  English

AB:  The stopping power cross section and energy straggling of protons and /sup 4/He ions in erbium films evaporated in vacuum were determined using a backscattering technique in the energy range from 0.2 to 2.0 MeV for protons, and 0.3 to 3.1 MeV for /sup 4/He ions. The accuracy of the measurements was estimated to be +or-3%. Disagreement between the experimental and theoretical points for energy straggling of /sup 4/He ions is explained by the influence of the thickness nonuniformity of the target.

AN:  1858181

*LHM:  ZB f. Physik Wien, Signatur: 04082.001

Bestand: 24.1963-51.1990   L:27,37

Record 28 of 43 in INSPEC 1980-1984

TI:  Stopping power of solid argon for helium ions

AU:  Besenbacher-F; Bottinger-J; Graversen-O; Hansen-JL; Sorensen-H

SO:  Nuclear-Instruments-and-Methods-in-Physics-Research. vol.188, no.3; 15 Oct. 1981; p.657-67

PY:  1981

LA:  English

AB:  The stopping cross section of solid argon has been measured for 0.5-3 MeV helium ions to an accuracy of approximately 3% using Rutherford-backscattering method. The results agree within the experimental accuracies with the authors' earlier measurements for gaseous argon over the energy region studied. This proves that the gas/solid difference for argon predicted in recent stopping-power tabulation is significantly overestimated. With high-order Z/sub 1/ correction terms included in the theoretical description, calculated shell corrections based on the Lindhard-Scharff model (1953) are in good agreement with the experimental results even down to energies close to 0.6 MeV.

AN:  1796254

Record 29 of 43 in INSPEC 1980-1984

TI:  Stopping power of swift hydrogen and helium ions in gases

AU:  Besenbacher-F; Andersen-HH; Hvelplund-P; Knudsen-H         Bs79

SO:  Matematisk-Fysiske-Meddelelser-Konglige-Danske-Videnskabernes-Selskab. vol.40, no.3; 1979; p.3-39

PY:  1979

LA:  English

AB:  Stopping powers of H/sub 2/, He, N/sub 2/, O/sub 2/, CO/sub 2/, Ne, Ar, Kr, and Xe for 40-keV to 1-MeV hydrogen ions and 100-keV to 2.4-MeV helium ions have been measured to an accuracy of +or-2.5% (2 sigma ). The stopping powers for hydrogen show good agreement with most other published results and with the Andersen and Ziegler tabulations (1977), while those for helium ions are up to 6% lower than the helium data of Chu and Powers (1971). With higher-order Z/sub 1/ correction terms included in the theoretical description, Bonderup's calculated shell-corrections based on the Linhard-Scharff model (1953), are in good agreement with the experimental proton data for E/sub p/>or=100 keV, and experimental I values may be deduced. Within the velocity region 4.4<or approximately=v/v/sub 0/<or approximately=4.9 and for target materials with 1<or=Z/sub 2/<or=54, the experimental findings support Lindhard's and Esbensen's (1977) value for the Barkas correction. The stopping-power ratios S/sub He//S/sub H/ depend strongly on Z/sub 2/ and deviate significantly from the mean-square charge state obtained from experimental equilibrium charge state distributions.

AN:  1453521

Record 30 of 43 in INSPEC 1985-1989

TI:  Energy losses of alpha -particles in Al and Cu in the energy region of 1-7 MeV

AU:  Mayatskii-VD; Pucherov-NN

SO:  Ukrayins'kyi-Fizychnyi-Zhurnal. vol.33, no.9; Sept. 1988; p.1285-8

PY:  1988

LA:  Russian

AB:  The stopping cross-sections of alpha -particles in Al and Cu have been experimentally determined in the energy region of 1-7 MeV. Semiempirical expressions describing the stopping cross-section, energy losses and straggling of alpha -particles in the indicated energy region are suggested.

AN:  3295814

Record 31 of 43 in INSPEC 1985-1989

TI:  Stopping power ratios for protons and helium ions in water and tissue equivalent media

AU:  Sugiyama-H

SO:  Bulletin-of-the-Electrotechnical-Laboratory. vol.51, no.7; 1987; p.520-4

PY:  1987

LA:  English

AB:  Phase effect in stopping powers has been estimated for protons and helium ions incident on water and tissue equivalent plastic and some filling gases. Calculated stopping power ratios of these materials are in good agreement with the recent experimental results. Uncertainty in the fast neutron dosimetry by the use of cavity chambers can be reduced by adopting the calculated relative stopping power ratios.

AN:  3010723

Record 32 of 43 in INSPEC 1985-1989

TI:  Stopping power and straggling of energetic light ion beams in yttrium

AU:  Alberts-HW

SO:  Radiation-Effects. vol.102, no.1-4; 1987; p.23-9

PY:  1987

LA:  English

AB:  Energy loss and straggling for protons, deuterons and alpha -particles in yttrium were measured in the energy region between 0.15 MeV and 2.5 MeV. The measured stopping powers of the hydrogen ions agree reasonably well with the semi-empirical values of Andersen and Ziegler (1977), although better agreement was found with the values of Janni (1982). For helium ions there are a marked difference with the values of Ziegler (1978). Below 200 keV/amu the stopping ratios of the helium and hydrogen ions are lower than Ziegler's master curve. The straggling values of the hydrogen ions are about 15% lower than the Bohr estimate, while the straggling of the helium ions reach the Bohr value at about 0.2 MeV/amu.

AN:  2975006

Record 33 of 43 in INSPEC 1985-1989

TI:  Stopping power measurements for megaelectronvolt /sup 4/He/sup +/ ions via thin films of metals and binary metal alloys

AU:  Harith-MA; Osman-WH; Gaafar-NS; El-Nadi-LM             Hh87

SO:  Thin-Solid-Films. vol.149, no.2; 25 May 1987; p.219-24

PY:  1987

LA:  English

AB:  The stopping power values of successive thin films of copper, silver and Cu-Ag mixtures were determined using alpha particles with energies in the range 1200-1800 keV in steps of 50 keV. A reasonable agreement between the values of dE/dx where E is the energy and x is a measure of the film thickness for copper and silver reported in this work and previously published data was obtained. Comparison of the experimental values for dE/dx for binary mixed films of Cu/sub 0.75/Ag/sub 0.25/, Cu/sub 0.5/Ag/sub 0.5/ and Cu/sub 0.25/Ag/sub 0.75/ with the sum of the dE/dx values obtained experimentally in the present study for the individual elements, taking into consideration the weight ratio according to Bragg's rule, indicates the validity of this rule.

AN:  2943297

*LHM:  ZB f. Physik Wien, Signatur: 26046.000

Bestand: 1.1968=>   L:92

Record 34 of 43 in INSPEC 1985-1989

TI:  Range and damage distributions of low energy /sup 4/He ions in alpha -Fe and Mo

AU:  Hautala-M; Anttila-A; Hirvonen-J

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.B19-20, pt.1; Feb. 1987; p.50-4

PY:  1987

LA:  English

AB:  A new method, which is based on nitrogen trapping with He atoms, has been used for the measurement of /sup 4/He range and damage distributions for 5-60 keV /sup 4/He in alpha -Fe and for 10-100 keV /sup 4/He in Mo. The computer simulation program COSIPO was used to calculate the distributions. The experimental and calculated modal ranges are in excellent agreement. The comparison of the measured and calculated distributions yields information on the electronic stopping power of /sup 4/He. The effect of polycrystallinity is found to be noticeable.

AN:  2879995

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 35 of 43 in INSPEC 1985-1989

TI:  Energy straggling and stopping power of 4 and 5.486 MeV alpha particles in tissue equivalent materials

AU:  Al-Bedri-MB

SO:  Iraqi-Journal-of-Science. vol.27, no.1-2; 1986; p.21-36

PY:  1986

LA:  English

AB:  Energy straggling and stopping power for 4 and 5.486 MeV alpha particles have been measured in commercial tissue equivalent gas mixtures and the plastic solid Melinex. A region of increasing energy straggling to a maximum and then followed by sharp reduction of straggling has been observed towards the end of the range. An explanation of the reduction of straggling is given by reference to the reduction of stopping power at low energies. Expected differences in energy straggling have been observed for the two alpha particles energies.

AN:  2825688

Record 36 of 43 in INSPEC 1985-1989

TI:  1-3 MeV alpha and triton stopping power in LiF and Li alloys

AU:  Biersack-JP; Fink-D; Miekeley-W; Tjan-K

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.B15, no.1-6; April 1986; p.96-100

PY:  1986

LA:  English

AB:  The energy dependence of stopping powers for 1-2 MeV alpha particles and 1-2.7 MeV tritons was determined by using the /sup 6/Li(n, alpha )T reaction in LiF and several Li alloys. The results are analysed to obtain both absolute stopping powers and the effective charge of He projectiles. These values are compared to the generalized ZBL predictions, and to new fully analytic calculations for one- and two-electron ions/atoms.

AN:  2689467

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 37 of 43 in INSPEC 1985-1989

TI:  Study of the stopping power and straggling for alpha particles and protons in organic solids, liquids and gases

AU:  Haque-AKMM; Mohammadi-A; Nikjoo-H

SO:  Radiation-Protection-Dosimetry. vol.13, no.1-4; 1985; p.71-4

PY:  1985

LA:  English

AB:  The stopping power and straggling for 5.5 MeV alpha particles in liquid and vapour phases of water, methanol, ethanol, propanol, n-hexane, n-octane and cyclohexane, and those for low energy protons in ethylene, styrene and propylene and their polymers, have been measured. Range-energy have been adjusted to obtain the best fit. The value of chi-squared per degree of freedom has been calculated, together with the parameters. The theoretical stopping cross section has been considered employing the Bethe-Bloch expression together with various corrections (shell correction using Walske and Bichsel procedure, Z/sub 1//sup 3/ contribution according to Ashley and Bloch correction based on Lindhard formalism). The existence of a phase effect has been clearly demonstrated for the stopping of both alpha particles and protons.

AN:  2659857

*LHM:  ZB f. Physik Wien, Signatur: 26869.000

Bestand: 1.1981=>

Record 38 of 43 in INSPEC 1990-1992

TI:  Stopping power for alpha particles in organic liquids and vapours

AU:  Haque-AKMM; Nikjoo-H

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.B53, no.1; Jan. 1991; p.15-23

PY:  1991

LA:  English

AB:  The stopping power for 5.5 MeV alpha particles has been studied in liquid and vapour phases of water, methanol, ethanol, propanol, pentane, hexane, cyclohexane, heptane, and octane. Experimental values for stopping powers in the range 5.5 MeV to 0.5 MeV have been extracted by fitting the range-energy data with polynomial and inverse stopping power functions, and compared with existing results. The effect of phase of the absorber on the stopping power has been clearly demonstrated, the stopping power being higher in the vapour phase. Mean values of the excitation potential have been deduced from the experimental stopping powers after allowing for shell, Barkas and Bloch corrections, which are then compared with the recommended values of Berger and Seltzer.

AN:  3857411

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 39 of 43 in INSPEC 1995

TI:  Stopping powers for alpha-particles and Barkas correction

AU:  Novkovic-D; Subotic-K; Stojanovic-M; Milosevic-Z; Manic-S; Paligoric-D

SO:  Journal-of-the-Moscow-Physical-Society. vol.3, no.3; Aug. 1993; p.209-14

PY:  1993

LA:  English

AB:  The precise measurement of alpha particle stopping powers in noble gases, and in nitrogen and oxygen has been performed with the maximal uncertainty of 0.7%. The accurate values of stopping powers obtained in these measurements enable extraction of Barkas correction term and completing of general stopping power formula. The obtained formula was used for comparison of stopping powers in different stopping media. Fairly good agreement was obtained with precise alpha stopping powers in solids measured by Andersen et al.

AN:  5055368

Record 40 of 43 in INSPEC 1/97-6/97

TI:  Stopping power determinations by the transmission technique

AU:  Raeisaenen-J; Waetjen-U; Plompen-AJM; Munnik-F

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.118, no.1-4; Sept. 1996; p.1-6

PY:  1996

LA:  English

AB:  Details of the experimental procedures related to the transmission technique are described. Parameters affecting the stopping power measurements by this technique are pointed out and discussed. For example, the importance of an accurate foil areal density determination is emphasized, and methods for the determination are introduced. The accuracy and possible error sources of the stopping power values are reviewed. Advantages as well as limitations of the method are discussed. As an example the stopping powers of 600-3200 keV /sup 4/He ions in polyimide (Kapton) have been determined. The experimental results are compared with the predictions of widely used semiempirical models.

AN:  5459739

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 41 of 43 in INSPEC 1998/07-1998/12

TI:  Stopping cross section measurements of Ti thin films

AU:  Climent-Font-A; Raeisaenen-J; Rauhala-E

SO:  Nuclear-Instruments-&-Methods-in-Physics-Research,-Section-B-(Beam-Interactions-with-Materials-and-Atoms). vol.136-138; March 1998; p.109-13

PY:  1998

LA:  English

AB:  The stopping cross section for /sup 4/He projectiles of Ti thin films have been measured with the backscattering method using a multi-compound marker layer deposited between the test film and the substrate. Problems derived from the non-uniformity of the marker layer as well as corrugations of the films under test have been solved with the help of a computer code to synthesize Rutherford backscattering spectrometry (RBS) spectra. Stopping cross section values are obtained with an estimated uncertainty of 4%.

AN:  5987866

*LHM:  ZB f. Physik Wien, Signatur: 08574.020

Bestand: 229=B1.1984=>

Record 42 of 43 in INSPEC 1999/01-1999/10

TI:  Range, energy loss, energy straggling and damage production for alpha -particles in uranium dioxide

AU:  Matzke-Hj

SO:  Journal-of-Nuclear-Materials. vol.270, no.1-2; 20 April 1999; p.49-54

PY:  1999

LA:  English

AB:  Energy loss and energy straggling of alpha -particles in UO/sub 2/ were measured for alpha -energies up to 8.78 MeV. alpha -sources of Pu-239 and a `multi-energy source' emitting alpha -particles of five different energies obtained by recoil implantation from a Th-228 source were coated with UO/sub 2/ layers of 17 different thicknesses. Confirmation of these results was obtained from Rutherford backscattering spectroscopy, RBS/channeling experiments with UO/sub 2/ single crystals preimplanted with He-ions. The measured energy loss values were used to deduce the range-energy relation. The results are in good agreement with calculations using the code TRIM 96. The energy straggling results are discussed in the frame of Bohr's theory. Finally, defects and damage produced by alpha -particles and He-ions in UO/sub 2/ are briefly treated.

AN:  6298167

*LHM:  ZB f. Physik Wien, Signatur: 08545.000

Bestand: 1.1959=>

Record 43 of 43 in INSPEC 2000/07-2000/12

TI:  Stopping power of solid targets for slow helium ions

AU:  Nagy-I; Eppacher-Ch; Semrad-D

SO:  Physical-Review-B-(Condensed-Matter). vol.62, no.9; 1 Sept. 2000; p.5270-2

PY:  2000

LA:  English

AB:  The energy loss of slow helium ions interacting with different solid targets is determined experimentally. By using a phase-shift sum rule for the amplitude of the dipolar backflow current, a consistent screened potential of these projectiles in an electron gas is constructed and applied to the kinetic model of stopping. The theoretical description reproduces well the experimental data over a broad range of the effective electron density including the accessible dilute limit. The combined theoretical and experimental analysis results in a consistent physical picture for the stopping of slow helium ions in solids.

AN:  6695579

*LHM:  ZB f. Physik Wien, Signatur: 01072.050

Bestand: 3.ser.1.1970=>